3GPP TSG RAN WG1 #109-e	                                             R1-2203465

e-Meeting, May 9th – 20th, 2022

Source:	CATT, GOHIGH

Title:	Discussion on SL positioning scenarios and requirements

Agenda item:	9.5.1.1

Document for:	Discussion and Decision

Introduction

At RAN#94-E meeting, a new study item expanded and improved NR positioning was approved in RP-213588 [1].

The following table lists the part of objectives related to SL positioning.

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

This contribution discusses sidelink positioning scenarios and requirements. Section 2 discusses motivations of SL positioning. Section 3 discusses scenarios of SL positioning. Section 4 discusses target performance requirements of SL positioning. Section 5 summarizes the proposals with conclusions.

Motivations of SL positioning

In Release 17, 3GPP RAN conducted studies on "NR Positioning Enhancements" and "Scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases".

The study on "Scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases" focussed on V2X and public safety use cases with the outcome being captured in TR38.845 [2]. Additionally, SA1 has developed requirements in TS22.261 for "Ranging based services", and has developed positioning accuracy requirements in TS22.104 for IIoT uses cases in out-of-coverage scenarios. There is a need for 3GPP to study and develop sidelink positioning solutions that can support the use cases, scenarios and requirements identified during these activities.

This requirement was also mentioned as part of the input from 5G Automotive Association (5GAA) to the Rel-18 RAN Workshop, and it was listed as the highest priority among the 8 requirement proposals by 5GAA [3], as shown in the table below.

Table 1: 5GAA list of proposals according to priority ranking

| No. | 5GAA features and requirements proposals | Priority ranking |
| --- | --- | --- |
| 1 | Positioning enhancements | 1 |
| 2 | LTE/NR-V2X sidelink co-channel coexistence | 2 |
| 3 | Sidelink carrier aggregation (CA) | 3 |
| 4 | Enhancements to sidelink power saving | 4 |
| 5 | Predictive QoS | 5 |
| 6 | UE-to-UE relay | 6 |
| 7 | NR-V2X sidelink adjacent-channel coexistence with non-3GPP technologies | 7 |
| 8 | Enhancements for vehicular distributed antenna system (DAS) UE transmission | 8 |

According to the requirements from 5GAA, the motivations for sidelink positioning include [3]:

- Advanced use cases require high level positioning accuracy and precision;

- Advanced use cases include basic and advanced VRU protection, automated driving, teleoperated driving, dynamic intersection management, group start, etc.

Scenarios of SL positioning

In the SID of Rel-18 positioning evolution [1], it is proposed that all the three coverage scenarios should be cover, including in-coverage, partial-coverage and out-of-coverage. According to TR 38.845, three network coverage scenarios can be considered when at least two UEs are involved in positioning for V2X and public safety use cases as follows. And a UE may transit between in-coverage, partial coverage and out-of-coverage scenarios.

- In-coverage scenario (IC): refers to the case where all the UEs envolved in the sidelink positioning are inside the network, as shown in Figure 1.

- Partial coverage scenario (PC): refers to the case where at least one UE envolved in the sidelink positioning remains inside the network coverage and at least one UE envolved in the sidelink positioning is outside the network coverage, as shown in Figure 2.

- Out-of-coverage scenario (OOC): refers to the case where all the UEs envolved in the sidelink positioning are outside the network coverage, as shown in Figure 3.

Figure 1: In-coverage scenario

Figure 2: Partial coverage scenario

Figure 3: Out-of-coverage scenario

Regarding the three coverage scenarios, for the in-coverage scenario, the positioning methods defined in Rel-16, such as DL-TDOA/Multi-RTT/DL-AoD, can be used to obtain the location of UEs. For the partial coverage scenario, it seems the combination of in-coverage scenario and out-of-coverage. In our point of view, the main task of Rel-18 sidelink positioning should be finding the positioning solution for UEs in out-of-coverage and evaluating its positioning performance. Therefore, out-of-coverage scenario has higher priority than the other two scenrios. The performance evaluation and potential solution investigation should focus on out-of-coverage scenario in Rel-18.

Proposal 1: Out-of-coverage scenario has higher priority than the other two scenrios. The performance evaluation and potential solutions investigation should focus on out-of-coverage scenario in Rel-18.

Target performance requirements of SL positioning

In this section, the target performance requirements of SL positioning are discussed, for V2X use cases, public safetly use cases, commercial use cases and IIoT use cases, respectively.

V2X use cases

In the TR 38.845 [2], the V2X positioning requirements are basically defined according to TS 22.261 [4] and TS 22.186 [5]. Three sets of 2D positioning metrics are defined corresponding to the three use cases groups as follows, which are applicable for both absolute positioning and relative positioning:

- Set 1: 10 – 50 m with 68 – 95 % confidence level.

  - This includes Group 1 in [6] and Service level 1 in [4].

- Set 2: 1 – 3 m with 95 – 99 % confidence level.

  - This includes Group 2 in [6], Service level 2, 3, 4 in [4].

- Set 3: 0.1 – 0.5 m with 95 – 99 % confidence level.

  - This includes Group 3 in [6], Service level 5, 6, 7 in [4], the requirements in [5].

Other performance metrics are also defined in a range depending on the positioning service level in [4] as follows,

- 2 – 3 m (absolute) or 0.2 m (relative) vertical accuracy

- 95 – 99.9% positioning service availability

- 10 ms – 1 s positioning service latency

In addition, V2X positioning requirements are applicable to:

- Inside the network coverage and outside the network coverage

- With or without GNSS-based positioning

Considering the positioning requirements in the V2X use cases, V2X positioning service should be provided for indoor, outdoor, under tunnel with the UE velocity up to 250 km/h.

For V2X use cases, we prefer adopting Set 2 (1 – 3 m) as the target performance requirements of horizontal accuracy for both absolute positiong and relative positioning in Rel-18, since the first version of sidelink positioning will be developed in  Rel-18, and V2X positioning service should be provided with the UE velocity up to 250 km/h, the higher accuracy of Set 3 (0.1 – 0.5 m) may not be reached for sidelink positioning. About vertical accuracy, we prefer adopting 2 – 3 m as the target performance requirements for both absolute positiong and relative positioning in Rel-18.

Proposal 2: For V2X use cases, the following accuracy requirement should be adopted as the target performance requirements of sidelink positioning in Rel-18:

- Horizontal accuracy: Set 2 (1–3 m) for both absolute positioning and relative positioning for 90% of UEs.

- Vertical accuracy: 2–3 m  for both absolute positioning and relative positioning for 90% of UEs.

Public safety use cases

In the TR 38.845 [2], the “1st responders” use case in TS 22.261 [4] is referenced as the public safety use case. Public safety positioning requirements are defined according to TS 22.261 [4] and TS 22.280 [7] as follows, which are applicable for both absolute positioning and relative positioning:

- TS 22.261 [4]: 1 m horizontal accuracy, 2 m (absolute) or 0.3 m (relative) vertical accuracy, 95 – 98 % positioning service availability

- TS 22.280 [7]  Qualitative positioning requirements in its Clause 5.11, 6.12, and 7.8.

In addition, public safety positioning requirements are applicable to:

- Inside the network coverage and outside the network coverage

- With or without GNSS-based positioning

Considering the positioning requirements in the public safety use cases, public safety positioning should be provided both in indoor and outdoor areas.

For public safety use cases, we prefer adopting 1 m as the target performance requirements of horizontal accuracy for both absolute positiong and relative positioning in Rel-18. About vertical accuracy, we prefer adopting 2 m as the target performance requirements for both absolute positiong and relative positioning in Rel-18.

Proposal 3: For public safety use cases, the following accuracy requirement should be adopted as the target performance requirements of sidelink positioning in Rel-18:

- Horizontal accuracy: 1 m for both absolute positioning and relative positioning for 90% of UEs.

- Vertical accuracy: 2 m for both absolute positioning and relative positioning for 90% of UEs.

Commerial use case

In the TR 38.857 [8], the target positioning requirements for commercial use cases for Rel-17 positionng enhancements (Uu) are defined as follows:

-	Horizontal position accuracy (< 1 m) for 90% of UEs

-	Vertical position accuracy (< 3 m) for 90% of UEs

-	End-to-end latency for position estimation of UE (< 100 ms)

-	Physical layer latency for position estimation of UE (< 10 ms)

We prefer to reuse the above target positioning requirements as the target performance requirements of sidelink positioning for commercial use cases in Rel-18.

Proposal 4: For commerial use cases, the following accuracy requirement should be adopted as the target performance requirements of sidelink positioning in Rel-18:

- Horizontal accuracy: 1 m for both absolute positioning and relative positioning for 90% of UEs.

- Vertical accuracy: 3 m for both absolute positioning and relative positioning for 90% of UEs.

IIoT use case

In the TR 38.857 [8], the target positioning requirements for IIoT use cases for Rel-17 positionng enhancements (Uu) are defined as follows:

-	Horizontal position accuracy (< 0.2 m) for 90% of UEs

-	Vertical position accuracy (< 1 m) for 90% of UEs

-	End-to-end latency for position estimation of UE (< 100ms, in the order of 10 ms is desired)

-	Physical layer latency for position estimation of UE (<10ms)

For IIoT use cases, we prefer adopting 1 m as the target performance requirements of horizontal accuracy for both absolute positiong and relative positioning in Rel-18, since the first version of sidelink positioning will be developed in  Rel-18, the higher accuracy of 0.2 m may not be reached for sidelink positioning. About vertical accuracy, we prefer adopting 1 m as the target performance requirements for both absolute positiong and relative positioning in Rel-18.

Proposal 5: For IIoT use cases, the following accuracy requirement should be adopted as the target performance requirements of sidelink positioning in Rel-18:

- Horizontal accuracy: 1 m for both absolute positioning and relative positioning for 90% of UEs.

- Vertical accuracy: 1 m for both absolute positioning and relative positioning for 90% of UEs.

In addition, considering that the scope of Rel-18 positioning evolution is relatively large, in order to reduce the work load of the groups, we prefer to focus on 1 ~ 2 more important kinds of use cases rather than all of the four kinds of use cases. Since the positioning requirements of V2X use cases from 5GAA and IIoT use cases from verticals are more urgent, V2X use cases and IIoT use cases should have higher priority than the other two kinds of use cases. The performance evaluation and potential solutions investigation should focus on V2X use cases and IIoT use cases in Rel-18.

Proposal 6: V2X use cases and IIoT use cases should have higher priority than the other two kinds of use cases. The performance evaluation and potential solution investigation should focus on V2X use cases and IIoT use cases in Rel-18.

Conclusions

In this contribution, we discuss sidelink positioning scenarios and requirements, and give the following proposals:

Proposal 1: Out-of-coverage scenario has higher priority than the other two scenrios. The performance evaluation and potential solutions investigation should focus on out-of-coverage scenario in Rel-18.

Proposal 2: For V2X use cases, the following accuracy requirement should be adopted as the target performance requirements of sidelink positioning in Rel-18:

- Horizontal accuracy: Set 2 (1–3 m) for both absolute positioning and relative positioning for 90% of UEs.

- Vertical accuracy: 2–3 m  for both absolute positioning and relative positioning for 90% of UEs.

Proposal 3: For public safety use cases, the following accuracy requirement should be adopted as the target performance requirements of sidelink positioning in Rel-18:

- Horizontal accuracy: 1 m for both absolute positioning and relative positioning for 90% of UEs.

- Vertical accuracy: 2 m for both absolute positioning and relative positioning for 90% of UEs.

Proposal 4: For commerial use cases, the following accuracy requirement should be adopted as the target performance requirements of sidelink positioning in Rel-18:

- Horizontal accuracy: 1 m for both absolute positioning and relative positioning for 90% of UEs.

- Vertical accuracy: 3 m for both absolute positioning and relative positioning for 90% of UEs.

Proposal 5: For IIoT use cases, the following accuracy requirement should be adopted as the target performance requirements of sidelink positioning in Rel-18:

- Horizontal accuracy: 1 m for both absolute positioning and relative positioning for 90% of UEs.

- Vertical accuracy: 1 m for both absolute positioning and relative positioning for 90% of UEs.

Proposal 6: V2X use cases and IIoT use cases should have higher priority than the other two kinds of use cases. The performance evaluation and potential solution investigation should focus on V2X use cases and IIoT use cases in Rel-18.

References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”, RAN#94-E meeting.

- 3GPP TR 38.845, “Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases (Release 17)”, V17.0.0 (2021-09)

- RWS-210360, “5GAA input to 3GPP Rel.18 Workshop”, RAN#92-E meeting.

- 3GPP TS 22.261: "Service requirements for the 5G system".

- 3GPP TS 22.186 v16.2.0: "Enhancement of 3GPP support for V2X scenarios".

- 3GPP RP-210040: "Reply LS to RP-201390 on requirements of in-coverage, partial coverage, and out-of-coverage positioning use cases," (source: 5GAA).

- 3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe)".

- 3GPP TR 38.857, “Study on NR Positioning Enhancements (Release 17)”, V17.0.0 (2021-03)