3GPP TSG RAN WG1 #109-e	                          R1-2203467

e-Meeting, May 9th – 20th, 2022

Source:	CATT, GOHIGH

Title:	Discussion on potential solutions for SL positioning

Agenda item:	9.5.1.3

Document for:	Discussion and Decision

Introduction

At RAN#94-E meeting, a new study item expanded and improved NR positioning was approved in RP-213588 [1].

The following table lists the part of objectives related to SL positioning.

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

This contribution discusses potential solutions for sidelink positioning. Section 2 discusses the principles of  sidelink positioning. Section 3 discusses sidelink positioning methods. Section 4 discusses sidelink positioning reference signal. Section 5 discusses sidelink positioning measurements and configuration. Section 6 discusses sidelink positioning physical-layer procedures. Section 7 summarizes the proposals with conclusions.

Principles of sidelink positioning

In the SID of Rel-18 positioning evolution [1], it is proposed that all the three coverage scenarios should be cover, including in-coverage, partial-coverage and out-of-coverage. According to TR 38.845 [2], three network coverage scenarios can be considered when at least two UEs are involved in positioning for V2X and public safety use cases as follows. And a UE may transit between in-coverage, partial coverage and out-of-coverage scenarios.

- In-coverage scenario (IC): refers to the case where all the UEs envolved in the sidelink positioning are inside the network, as shown in Figure 1.

- Partial coverage scenario (PC): refers to the case where at least one UE envolved in the sidelink positioning remains inside the network coverage and at least one UE envolved in the sidelink positioning is outside the network coverage, as shown in Figure 2.

- Out-of-coverage scenario (OOC): refers to the case where all the UEs envolved in the sidelink positioning are outside the network coverage, as shown in Figure 3.

Figure 1: In-coverage scenario

Figure 2: Partial coverage scenario

Figure 3: Out-of-coverage scenario

For the in-coverage scenario, the downlink or uplink positioning methods defined in Rel-16, such as DL-TDOA/UL-TDOA/RTT/DL-AoD/UL-AoA via Uu, can be used to obtain the absolute location of UEs. At the same time, the sidelink positioning methods which will be discussed in Rel-18, such as SL-TDOA/SL-RTT/SL-AoA via PC5, can also be used to obtain the relative/absolute location of UEs.

For the out-of-coverage scenario, the downlink or uplink positioning methods defined in Rel-16 cannot work since there is no the involvement of base stations. Only the sidelink positioning methods which will be discussed in Rel-18, such as SL-TDOA/SL-RTT/SL-AoA via PC5, can also be used to obtain the relative/absolute location of UEs.

For the partial coverage scenario, it seems to be the combination of in-coverage scenario and out-of-coverage scenario.

In order to support to use sidelink positioning methods to obtain the location of UEs in the above three coverage scenarios, UE-1 should transmit the sidelink positioning reference signal (S-PRS), and UE-2 should receive the S-PRS and calculate the sidelink positioning measurements based on the received S-PRS. The sidelink positioning measurements may be reported to UE-1 for UE-based positioning or reported to base stations/LMF for network-based positioning. For the transmission and receiption of S-PRS, the sidelink physical-layer procedures may be enhanced to support sidelink positioning, e.g. the power control of S-PRS.

In a word, the solutions for sidelink positioning should include the following aspects:

- Sidelink positioning methods

- Sidelink positioning reference signal

- Sidelink positioning measurement and configuration

- Sidelink positioning physical-layer procedures

Proposal 1: the solutions for sidelink positioning should include the following aspects:.

- Sidelink positioning methods

- Sidelink positioning reference signal

- Sidelink positioning measurement and configuration

- Sidelink positioning physical-layer procedures

Sidelink positioning methods

During the RAN#94 meeting, the following positioning methods were discussed [1]:

- Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1]

The potential positioning methods supported for SL positioning are presented as follows:

- Time Difference of Arrival (TDOA)

TDOA is based on the time difference among the different radio signals’ arrivals originated from or received at different anchors. The target UE position can be obtained with the intersection of the hyperbolic curves from at least three anchor nodes (e.g. RSU), which should be synchronized with each others. Considering the fast-varying vehicular enviorments, it may be hard to achieve positioning when the number of anchor nodes is not enough. Meanwhile, the distribution of anchor nodes may also affect the positioning accuracy. The robustness and the availability of the SL positioning with TDOA need further study.

- Angle-of-Departure (AoD)/Angle-of-Arrival (AoA)

In order to achieve SL positioning with AoA and AoD, OBU/RSU should be equipped with the distributed antenna. Compared with multiple-antenna mechanism in the cellular communications, with the less supporting antenna number of SL communications, the accuracy of SL positioning with AoA and AoD may be insufficient for specific V2X applications of stringent positioning requirements.

- Round Trip Time Positioning (RTT)

RTT can mitigate the impact of synchronization delay to improve the SL positioning accuracy. The disadvantage is that the ranging result may be impacted by the drift of local oscillator.

Proposal 2: The potential positioning methods of TDOA, AOD/AOA, RTT for sidelink positioning should be further studied.

In TR 38.845 [2], Uu interface is available for UEs of in-coverage scenario, while PC5 interface is available for UEs of in-coverage, partial coverage, and out-of-coverage scenarios. The necessary information and the measurements for SL positioning can be transmitted via Uu and/or PC5 interface.

The SL positioning solutions can be categorized by the involved entities performing the positioning estimation and calculation as follows.

- Network-based positioning: UE’s position is calculated by a network node. For the network-based positioning, the UE may report necessary information to the network for the calculation. This solution increases the process of notifying the calculation results to the UE, and may introduce the additional delay.

- UE-based positioning: UE’s position is calculated by the UE itself. From the perspective of time delay and anti-tracking, the UE-based positioning solution should be the focus of SL positioning, especially for the relative positioning and ranging.

For the design of SL positioning procedures, there are differences between SL positioning and Uu positioning, such as the diverse V2X application requirements, low-latency and high-reliability communications requirements in high-speed vehicular scenarios.

An example of UE The positioning operation to supporting SL positioning is shown in Figure 4.

Figure 4:The procedure of SL positioning operation

- Step 1a: Either: the assisted UEs provide the assistance for positioning operation (e.g. positioning or delivery of assistance data) to a target UE.

- Step 1b: Or: a target UE requests the assistance for positioning operation (e.g. positioning or delivery of assistance data) from other UE.

- Step 2: A target UE or the assisted UEs instigate positioning procedures with each other – e.g. to obtain a location estimate or positioning measurements.

- Step 3: The assisted UE returns a positioning operation response to the target UE and includes any required results (e.g. a location estimatation for target UE).

For the operation of SL positioning without LMF, the following issues are suggested to be addressed:

- In the relative positioning, ranging and absolute positioning scenarios of SL positioning, the target UE can directly send a request information to other assisted UEs to initiate the positioning. This may be defined as device to device location service(request) which will be discussed in SA2.

- In the absolute positioning scenarios of SL positioning, the assisted UEs may be the positioning anchor nodes (e.g. RSUs), and the assisted UEs can broadcast positioning indication information to assist the target UE to trigger the positioning procedure. This may be defined as group location service in SA2.

For the operations of SL positioning with LMF, the legacy Uu positioning system architecture and signalling procedures with LMF can be reused as defined in TR 38.305 [3].

Proposal 3: The sidelink positioning procedure can be triggered by the positioning operation indication signaling or the positioning operation request signaling.

Sidelink positioning reference signal

Sequence design of S-PRS

The sequence design of S-PRS should reuse the existing sidelink/positioning reference signals design as much as possible to reduce the workload of standardization. The sequence design of S-PRS can refer to DL-PRS, UL SRS-Pos, SL CSI-RS and SL PT-RS. For these reference signals, except that UL SRS-Pos uses ZC sequence, other reference signals use Gold sequence. The advantage of ZC sequence is low PAPR, which is conducive to the power saving of UE. However, considering that the reference signals defined in SL, such as SL CSI-RS and SLPT-RS, all use Gold sequence, we prefer S-PRS to also use Gold sequence.

Proposal 4: Gold sequence should be adopted as the sequence of S-PRS.

Resource allocation of S-PRS

If the S-PRS is multiplexed with PSCCH/PSSCH of Rel-16, the sidelink physical layer structure needs to be redesigned. The issues as TBS determination, S-PRS mapping rules, multiplexing method and compatibility should be discussed. Besides, the transmission band of S-PRS will be limited with PSCCH/PSSCH, which will affect the positioning accuracy.

Proposal 5: Considering the compatibility for Rel-16/17, the dedicated resource pool for sidelink positioning should be introduced.

In order to meet the positioning requirements, UEs need to transmit S-PRS frequently to complete sidelink positioning. In order to improve the positioning accuracy, the bandwidth occupied by S-PRS is often large. In this way, the resources occupied by S-PRS transmitted by different UEs are prone to collide with each other. In order to solve this issue, it is necessary to introduce an effective S-PRS resource allocation mechanism.

For in-coverage scenario, the gNB can be responsible for the time-frequency resources used for S-PRS, similar to the Mode 1 resource allocation mechanism in NR V2X. By allocating different time-frequency resources used by S-PRS to different UEs, the gNB can effectively avoid the collision of resources occupied by S-PRS sent by different UEs.

For out-of-coverage scenario, since there is no unified scheduling by the gNB, the UE can only select the resources autonomously. We prefer to adopt the way of resource sensing and resource exclusion for the UE to select the time-frequency resources used by S-PRS, which is similar to the Mode 2 resource allocation mechanism in NR V2X. First, UE-1 indicate the time-frequency resources used by its S-PRS via the SCI, and other UEs obtain the time-frequency resources used by the S-PRS of UE-1 by decoding the SCI. Therefore, the time-frequency resources used by the S-PRS of UE-1 can be avoided by means of resource exclusion of other UEs. In addition, random resource selection is also a possible solution for S-PRS resource allocation for out-of-coverage scenario. This scheme can reduce the positioning latency and can be applied to sidelink positioning scenarios with latency sensitivity.

For partial-coverage scenario, as some UEs are located outside the network coverage, the gNB cannot allocate the resources used by S-PRS to the UEs outside the coverage. Therefore, for partial coverage scenario, we prefer to adopt the resource allocation scheme of S-PRS in the out-of-coverage scenario.

Proposal 6: For the resource allocation of S-PRS in resource pool:

- Under the in-coverage scenario, gNB should schedule the resources of S-PRS for all UEs, similar to the Mode 1 mechanism in Rel-16 NR V2X.

- Under the out-of-coverage scenario or partial coverage scenario, the resources of S-PRS should be allocated by resource sensing and exclusion, similar to the Mode 2 mechanism in Rel-16 NR V2X. Random resource selection mechanism can also be considered to reduce the sidelink positioning latency.

Considering the sidelink resource allocation mode 2 in Rel-16, the resource selection for S-PRS transmission will follow the same approach as much as possible. The sidelink positioning control information(SPCI) for scheduling the S-PRS transmission should be introduced for the resource allocation. Which sidelink physical channels the SPCI will be carried on, or the SPCI will be transmitted on the dedicated resource pool for sidelink positioning , both the above options are possible, which needs more detail discussion.

Proposal 7: The SPCI used for the scheduling of S-PRS, can be transmitted in the dedicated resource pool for sidelink positioning or the legacy resource pool for Rel-16/17 sidelink.

Resource types of S-PRS

For S-PRS resource types, there are three candidate options for NR sidelink positioning:

- Periodic S-PRS

- Semi-Persistent S-PRS

- Aperiodic S-PRS

Obviously, at least semi-Persistent S-PRS and aperiodic S-PRS should be supported, these two kinds of resource type are just like the periodic and aperiodic transmission in Rel-16. Besides, they are more suitable for relative positioning and ranging between sidelink UEs (like OBU). Because the procedure for relative positioning and ranging is more likely triggered based on demand, there is no need to transmit the periodic S-PRS persistently in this scenario.

About periodic S-PRS, there are more details should be discussed first. For sidelink positioning, the sidelink UEs maybe move fast, and different location update period may be required for different positioning scenarios. How to choose an appropriate S-PRS transmission period will be an issue. If a short period is configured to meet the different location update requirement, it will occupy too many limited physical resources. However, periodic S-PRS can be used for absolute positioning. The anchor node UE (like RSU) should be better to transmit the periodic S-PRS persistently to assist other UE in positioning.

Proposal 8: Periodic, semi-persistent and aperiodic S-PRS should be supported in Rel-18.

For semi-persistent S-PRS and aperiodic S-PRS transmission procedure, the signaling procedure to trigger S-PRS transmission should be introduced in sidelink positioning. When a UE receives the related signaling, the UE should select the resources for the S-PRS transmission based on S-PRS resource allocation.

For periodic S-PRS transmission procedure, the UE can be (pre-)configured with one or more set of S-PRS configurations. The UE may transmit the S-PRS periodically on the (pre-)configured resources. If the S-PRS configurations cannot be achieved, the UE may select the resources for the S-PRS transmission based on S-PRS resource allocation mechanism.

Proposal 9: The semi-persistent S-PRS and aperiodic S-PRS transmission procedure should be triggered explicitly by the related signaling procedure.

Sidelink positioning measurements and configuration

New sidelink positioning measurements

In Rel-16/17 positioning, time measurement, angle measurement and power measurement are all supported for different positioning methods via Uu interface. Sidelink measurement aims to define the positioning measurement for PC5 interface. For sidelink, both absolute positioning and relative positioning could be supported.

For absolute positioning via PC5 interface, RSU and UE would be envolved. Similar to gNB, the locations of RSUs are assumed to be known. From the positioning accuracy perspective, the requirement of synchronization among UE/RSU and its impact on positioning accuracy should be evaluated first. If synchronization among RSUs could be guaranteed, all the positioning methods supported in Rel-16/17 (e.g. DL-TDOA, DL-AOD, UL-TDOA, UL-AOA and multi-RTT) could be used for sidelink. Correspondingly, SL RSTD, SL RSRP, SL RTOA, SL AOA and SL Rx-Tx time difference should be supported for SL positioning. Otherwise, it is only necessary to support those positioning methods without synchronization requirement (i.e. DL-AOD, UL-AOA and multi-RTT). In this way, only SL RSRP/RSRPP, SL AOA and SL Rx-Tx time difference needs to be considered in Rel-18.

For relative positioning via PC5 interface, positioning between two UEs is assumed, where both distance and angle value are required for location calculation. Referring to multi-RTT, by transmitting and receving S-PRS, distance between two UEs could be obtained using Rx-Tx time difference. The angle value could be estimated by measuring AOA of S-PRS. In addition, SL RSRP/RSRPP is also beneficial for ranging. Therefore, SL Rx-Tx time difference, SL AOA and SL RSRP/RSRPP should be supported for SL relative positioning.

As a summary, to support both absolute and relative positioning, at least SL Rx-Tx time difference, SL AOA and SL RSRP/RSRPP should be supported for sidelink.

Proposal 10: Support at least the following measurements for sidelink positioning:

- SL Rx-Tx time difference

- SL AOA

- SL RSRP/RSRPP

Configuration of sidelink positioning measurements

In the case of in coverage, LMF and gNB would be included in the positioning procedure, where both UE-assisted and UE-based positioning should be supported. Basically, the positioning procedure in Rel-16/17 could be reused. Some enhancements on the assisted data from the LMF would be required. For example, LMF would indicate the S-PRS resources allocation and SL measurements to be reported.

In the case of out of coverage, only UE-based positioning is feasible. As mentioned above, to cover most of the scenarios, periodic S-PRS, semi-persistent S-PRS and aperiodic S-PRS should all be supported in Rel-18, and S-PRS transmission is triggered by the target UE. For all types of S-PRS, the required measurements are configured through SLPP (Sidelink Positioning Protocol, the definition is up to RAN2). And corresponding SL measurements are also reported through SLPP. Taking the relative positioning as an example (shown in Figure 5), UE-1 triggers aperiodic S-PRS transmission to UE-2. And then triggers aperiodic S-PRS transmission from UE-2. According to the required measurements in the assisted data, after measuring S-PRS, UE-2 reports SL Rx-Tx time difference and SL AOA to UE-1 through SLPP for location calculation.

Figure 5: Relative positioning for sidelink

Proposal 11: For all types of S-PRS (periodic, semi-persistent and aperiodic S-PRS), the required measurements are configured through SLPP (the definition is up to RAN2). And corresponding SL measurements are also reported through SLPP.

Sidelink positioning physical-layer procedures

S-PRS power control

In order to mitigate the interference among S-PRS transmitted by different UEs and more effectively utilize the transmission power of UE, S-PRS power control needs to be introduced in Rel-18. The power control mechanism of PSSCH in NR V2X in Rel-16 can be reused for S-PRS , i.e., the minimum pathloss between downlink pathloss and sidelink pathloss can be adopted as the compensated pathloss for the S-PRS power control.

Proposal 12: S-PRS power control should be introduced in Rel-18.

- The minimum pathloss between DL pathloss and SL pathloss can be adopted as the compensated pathloss for the S-PRS power control.

S-PRS beam management

For S-PRS in FR2 band, S-PRS beam management need to be considered. However, considering the sidelink beam management mechanism has not been introduced in NR V2X, and it may be discussed in Rel-18 sidelink evolution work item, we prefer the beam management of S-PRS will not be specified in Rel-18 and this feature can be studied in future release, e.g, Rel-19. The potential solutions investigation and performance evaluation should focus on ITS band and FR1 band in Rel-18.

Proposal 13: There is no need to specify S-PRS beam management in Rel-18.

Conclusions

In this contribution, we discuss potential solutions for SL positioning, and give the following proposals:

Proposal 1: the solutions for sidelink positioning should include the following aspects:.

- Sidelink positioning methods

- Sidelink positioning reference signal

- Sidelink positioning measurement and configuration

- Sidelink positioning physical-layer procedures

Proposal 2: The potential positioning methods of TDOA, AOD/AOA, RTT for sidelink positioning should be further studied.

Proposal 3: The sidelink positioning procedure can be triggered by the positioning operation indication signaling or the positioning operation request signaling.

Proposal 4: Gold sequence should be adopted as the sequence of S-PRS.

Proposal 5: Considering the compatibility for Rel-16/17, the dedicated resource pool for sidelink positioning should be introduced.

Proposal 6: For the resource allocation of S-PRS in resource pool:

- Under the in-coverage scenario, gNB should schedule the resources of S-PRS for all UEs, similar to the Mode 1 mechanism in Rel-16 NR V2X.

- Under the out-of-coverage scenario or partial coverage scenario, the resources of S-PRS should be allocated by resource sensing and exclusion, similar to the Mode 2 mechanism in Rel-16 NR V2X. Random resource selection mechanism can also be considered to reduce the sidelink positioning latency.

Proposal 7: The SPCI used for the scheduling of S-PRS, can be transmitted in the dedicated resource pool for sidelink positioning or the legacy resource pool for Rel-16/17 sidelink.

Proposal 8: Periodic, semi-persistent and aperiodic S-PRS should be supported in Rel-18.

Proposal 9: The semi-persistent S-PRS and aperiodic S-PRS transmission procedure should be triggered explicitly by the related signaling procedure.

Proposal 10: Support at least the following measurements for sidelink positioning:

- SL Rx-Tx time difference

- SL AOA

- SL RSRP/RSRPP

Proposal 11: For all types of S-PRS (periodic, semi-persistent and aperiodic S-PRS), the required measurements are configured through SLPP (the definition is up to RAN2). And corresponding SL measurements are also reported through SLPP.

Proposal 12: S-PRS power control should be introduced in Rel-18.

- The minimum pathloss between DL pathloss and SL pathloss can be adopted as the compensated pathloss for the S-PRS power control.

Proposal 13: There is no need to specify S-PRS beam management in Rel-18.

References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”, RAN#94-E meeting.

- 3GPP TR 38.845, “Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases (Release 17)”, V17.0.0 (2021-09).

- 3GPP TR 38.305, “Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN (Release 17)”, V17.0.0 (2022-03).