3GPP TSG RAN WG1 #109-e                            		R1-2203468

e-Meeting, May 9th – 20th, 2022

Source:	CATT

Title:	Discussion on solutions for the integrity of RAT dependent positioning techniques

Agenda item:	9.5.2.1

Document for:	Discussion and Decision

Introduction

Positioning integrity is a measure of the trust in the accuracy of the position-related data and the ability to provide timely warnings based on assistance data provided by the network. Integrity enables applications to make the correct decisions based on the reported position, e.g., when monitoring a robotic arm to decide whether its arm movement is within allowed limits to ensure safety distances to humans and other objects. In Rel-17, 3GPP has developed the methodologies, procedures, and signaling support for GNSS integrity. For Rel-18, 3GPP is going to  extend it to RAT-dependent positioning techniques, as defined in the following objective in the SI of “Study on expanded and improved NR positioning”[1]:

- Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]:

  - Identify the error sources, [RAN1, RAN2].

  - Study methodologies, procedures, signalling, etc for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2]

  - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible.

In this contribution, we present our views on solutions for the integrity of RAT dependent positioning techniques.

Discussion

Principle of GNSS Integrity Operation

Positioning integrity represents the trust in the accuracy of the position-related data and the ability to provide associated alerts. Integrity risk (IR) represents the probability of hazardously misleading information (PHMI) when the error exceeds the Alert Limit (AL) and there is no timely alert within the Time-to-Alert (TTA). The basic requirement of the integrity operation is to ensure the PHMI should be below a predefined value. In TS 38.305 [2], it defines the network should ensure the following condition needs to be met for the parameters in the GNSS Real-Time Integrity assistance:

P(Error > Bound for longer than TTA | NOT DNU) <= Residual Risk + IRallocation			(1)

where

Error: the difference between the true value of a GNSS parameter, and its value provided in the assistance data;

irMinimum, irMaximum: the minimum and maximum IR provided in the Integrity Service Parameters.

IRallocation: the target IR requested by the client based on the desired error bound.

DNU (Do Not Use): A DNU flag indicates that the corresponding assistance data is not suitable for the purpose of computing integrity.

Residual Risk: the Probability of Onset which is defined per unit of time and represents the probability that the feared event begins. Each Residual Risk is accompanied by a Mean Duration which represents the expected mean duration of the corresponding feared event and is used to convert the Probability of Onset to a probability that the feared event is present at any given time, i.e.

- P(Feared Event is Present) = Mean Duration * Probability of Onset of Feared Event		(2)

Bound: the statistical distribution of the residual errors associated with the GNSS positioning corrections (e.g. RTK, SSR etc). Integrity bounds are used to statistically bound the residual errors after the positioning corrections have been applied.

Based on the information provided by the GNSS Real-Time Integrity assistance, the UE (or LMF) may provide the integrity result for the estimated UE location. The integrity requirement is defined as follows [2]:

Prob per unit of time [((PE>AL) & (PL<=AL)) for longer than TTA] < required target IR(TIR)	(3)

where PL (protection level) is defined as the statistical upper bound of the Positioning Error (PE). The following can be a different form of integrity requirement, which is commonly used for GNSS integrity [3], where the PL should satisfy the following inequality

Prob per unit of time [(PE>PL) for longer than TTA] < required TIR				(4)

The integrity requirements (3) and (4) have slightly different meanings. The requirement (3) defines that the probability of “PE>AL” should be smaller than the target IR, while the requirement (4) requires the probability of “PE>PL” to be smaller than the target IR, where the PL is derived with the consideration of the required TIR.

The concepts of integrity and the relations of  the requirements (3) and (4) can be illustrated with the Stanford diagram [4] as shown in Fig 1, which shows two types of integrity events:

A misleading information (MI) event occurs when, being the system declared available, the position error exceeds the protection level but not the alert limit.

A hazardously misleading information (HMI) event occurs when, being the system declared available, the position error exceeds the alert limit.

- Integrity requirement (3) can be seen as requiring the HMI probability to be smaller than the TIR, while integrity requirement (4) requires the HMI+MI probability to be smaller than the TIR. Obviously, if the integrity requirement (4) is satisfied, the integrity requirement (3) is also satisfied. But, integrity requirement (3) does not use the condition that the PL (protection level) is the statistical upper bound of the Positioning Error (PE).

Figure 1: The Standford diagram [4]

Integrity of RAT-dependent positioning

For RAT- dependent positioning integrity in Rel-18, in our view there is a need to consider UE-assistant and UE-based positioning.

- For the UE-based positioning, the UE determines and report the integrity results of the calculated location. In this case, the UE can use the integrity requirements and assistance data obtained via LMF/gNB, together with its own measurements, to determine the integrity results of the calculated location.

- For the UE-assisted positioning, the LMF determines and report the integrity results of the calculated location. In this case, the LMF can use the integrity requirements and assistance data obtained via gNB and UE, together with the measurements reported from the gNB and UE, to determine the integrity results of the calculated location.

Proposal 1: For the integrity of RAT-dependent positioning in Rel-18, there is a need to consider the integrity for both UE-assistant and UE-based positioning.

For RAT- dependent positioning integrity in Rel-18, one of main issues is the identification of the error sources (or the fault events) that could affect the assessment of the integrity requirements discussed in the previous section. The error sources to be considered can be divided into two categories: normal error sources and unexpected faults. The normal error sources include those that we may expect related to the normal operation of the RAT-dependent positioning, related to the general, propagation, and reception of positioning reference signals. The normal errors related to the generation and reception of the positioning reference signals may be modeled as Gaussian distribution with unknown but bounded bias and deviation. The values of the error bounds can be defined with the consideration of the minimum performance requirements defined by RAN4 (e.g., TS 38.101 and TS 38.104 for the generation of the DL/UL positioning reference signals, and TS 38.133 for the reception of the DL/UL positioning reference signals in the positioning measurements). The modelling of the error sources related to signal propagation can be more complicated, which depends on the positioning scenarios and surrounding RF environment. The RF channel models defined in TR 38.901 may be used for modeling these errors.

Unexpected faults are those event that may happen rarely, but could result in extremely large errors, such as the malfunction of the TRP or UE in the generation and reception of the positioning reference signals, or sudden changes of the surrounding RF environment. We may need to have further discussion on whether and how to handle unexpected faults in the integrity for RAT-dependent positioning.

Proposal 2: For the integrity of RAT-dependent positioning, there is a need to consider at least the normal error sources related to the generation, propagation and reception of DL/UL positioning reference signals. These errors may be modelled as Gaussian distribution with unknown but bounded bias and deviation. The values of the error bounds can be defined with the consideration of the minimum performance requirements defined by RAN4 (e.g., TS 38.101 and TS 38.104 for the generation of the DL/UL positioning reference signals, and TS 38.133 for the positioning measurement errors).

Different RAT-dependent positioning methods may be impacted by different error sources. For example, DL positioning may be impacted by TRP signal transmission and UE measurement errors, UL positioning may be impacted by UE signal transmission and TRP measurement errors, while DL+UL positioning may be impacted by the UE and TRP transmission and reception errors. Thus, there is need to identify which error sources have impact on which of the RAT-dependent positioning methods for supporting the determination of the determining the integrity.

Proposal 3: For the integrity of RAT-dependent positioning, there is a need to consider the error sources related to each of the positioning methods.

Proposal 4: The following error sources may be considered for the Integrity of RAT-dependent positioning.

- gNB (TRP)  error sources:

  - self-clock errors

  - synchronization errors (between TRPs)a

  - TRP antenna reference point (ARP) errors (or antenna phase center errors)

  - TRP Rx/Tx/RxTx TEG margins

  - Differences of Rx/Tx/RxTx TEG margins of the same TRP

  - TRP measurement errors (RTOA/UL-AOA/UL-RSRP/UL-RSRPP/gNB Rx-Tx time difference measurements)

  - Multipath indication for TRP measurements

- UE error sources:

  - UE antenna reference point (ARP) errors (or antenna phase center errors)

  - Rx/Tx/RxTx TEG margins

  - Difference of Rx/Tx/RxTx TEG margins of the same UE

  - UE measurement errors (RSTD/ DL-RSRP/DL-RSRPP/UE Rx-Tx time measurements)

  - Multipath indication for UE measurements

- PRU error sources (when PRU is a TRP):

  - self-clock errors

  - PRU antenna reference point (ARP) errors (or antenna phase center errors)

  - PRU Rx/Tx/RxTx TEG margins

  - Differences of Rx/Tx/RxTx TEG margins of the same PRU

  - PRU measurement errors (RTOA/UL-AOA/UL-RSRP/UL-RSRPP/gNB Rx-Tx time difference measurements)

  - Multipath indication for PRU measurements

- PRU error sources (when PRU is a UE):

  - PRU antenna reference point (ARP) errors (or antenna phase center errors)

  - PRU Rx/Tx/RxTx TEG margins

  - Difference of Rx/Tx/RxTx TEG margins of the same PRU

  - PRU measurement errors (RSTD/ DL-RSRP/DL-RSRPP/UE Rx-Tx time measurements)

  - Multipath indication for PRU measurements

The parameters of the error sources may be signaled from gNB, UE and/or positioning reference unit (PRU) to the network entity that performs the integrity assessment depending on the RAT-dependent positioning methods.

Conclusions

In this contribution, we discussed the solutions for the integrity of RAT dependent positioning techniques. Based on the discussion, we have the following proposals:

Proposal 1: For the integrity of RAT-dependent positioning in Rel-18, there is a need to consider the integrity for both UE-assistant and UE-based positioning.

Proposal 2: For the integrity of RAT-dependent positioning, there is a need to consider at least the normal error sources related to the generation, propagation and reception of DL/UL positioning reference signals. These errors may be modelled as Gaussian distribution with unknown but bounded bias and deviation. The values of the error bounds can be defined with the consideration of the minimum performance requirements defined by RAN4 (e.g., TS 38.101 and TS 38.104 for the generation of the DL/UL positioning reference signals, and TS 38.133 for the positioning measurement errors).

Proposal 3: For the integrity of RAT-dependent positioning, there is a need to consider the error sources related to each of the positioning methods.

Proposal 4: The following error sources may be considered for the Integrity of RAT-dependent positioning.

- gNB (TRP)  error sources:

  - self-clock errors

  - synchronization errors (between TRPs)a

  - TRP antenna reference point (ARP) errors (or antenna phase center errors)

  - TRP Rx/Tx/RxTx TEG margins

  - Differences of Rx/Tx/RxTx TEG margins of the same TRP

  - TRP measurement errors (RTOA/UL-AOA/UL-RSRP/UL-RSRPP/gNB Rx-Tx time difference measurements)

  - Multipath indication for TRP measurements

- UE error sources:

  - UE antenna reference point (ARP) errors (or antenna phase center errors)

  - Rx/Tx/RxTx TEG margins

  - Difference of Rx/Tx/RxTx TEG margins of the same UE

  - UE measurement errors (RSTD/ DL-RSRP/DL-RSRPP/UE Rx-Tx time measurements)

  - Multipath indication for UE measurements

- PRU error sources (when PRU is a TRP):

  - self-clock errors

  - PRU antenna reference point (ARP) errors (or antenna phase center errors)

  - PRU Rx/Tx/RxTx TEG margins

  - Differences of Rx/Tx/RxTx TEG margins of the same PRU

  - PRU measurement errors (RTOA/UL-AOA/UL-RSRP/UL-RSRPP/gNB Rx-Tx time difference measurements)

  - Multipath indication for PRU measurements

- PRU error sources (when PRU is a UE):

  - PRU antenna reference point (ARP) errors (or antenna phase center errors)

  - PRU Rx/Tx/RxTx TEG margins

  - Difference of Rx/Tx/RxTx TEG margins of the same PRU

  - PRU measurement errors (RSTD/ DL-RSRP/DL-RSRPP/UE Rx-Tx time measurements)

  - Multipath indication for PRU measurements

References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning.”

- TS 38.305, “NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment”.

- “Phase II of the GNSS Evolutionary Architecture Study”, February 2010

- M. Tossaint  et al, “The Stanford – ESA Integrity Diagram: Focusing on SBAS Integrity”, https://gssc.esa.int/navipedia/index.php/The_Stanford_ESA_Integrity_Diagram:_Focusing_on_SBAS_Integrity

- TS 37.355,  “LTE Positioning Protocol”.

- TS 38.455, “NG-RAN; NR Positioning Protocol A (NRPPa)”.