3GPP TSG RAN WG1 #109-e                            		R1-2203472

e-Meeting, May 9th – 20th, 2022

Source:	CATT

Title:	Discussion on  solutions of carrier phase positioning in multipath scenarios

Agenda item:	9.5.4

Document for:	Discussion and Decision

Introduction

GNSS carrier phase positioning has been used very successfully for centimeter-level positioning but is limited to outdoor applications. NR carrier phase positioning has the potential for significant performance improvements for indoor and outdoor deployments in comparison with the existing NR positioning methods, as well as shorter latency and lower UE power consumption in comparison with RTK-GNSS outdoors. In Rel-18, the solutions for accuracy improvement based on NR carrier phase measurements will be investigated, including the following tasks [1]:

  - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning

  - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary

Carrier phase positioning is normally used for supporting the location applications that centimeter-level positioning accuracy is required. Various error sources prevent high accuracy positioning, including TRP and UE clock errors, and multipath errors, which occur when a signal reflected from surrounding objects reaches the antenna in addition to the line-of-sight (LOS) signal. TRP and UE clock errors may be removed by differential techniques.  The measurement error due to multipath, however, may not be removed by differential techniques, before the measurement errors due to multipath are typically uncorrelated between the receivers.

Different approaches have been proposed to mitigate the multipath error in timing-based positioning techniques. Multiple mitigation is also included in Rel-17 as one of the main enhancements in NR positioning. Although the study of the multipath mitigation for carrier phase positioning is not specifically mentioned in Rel-18 SI objective, we consider it is important to discuss the potential impact of the multipath on the NR carrier phase measurements, and the potential solutions for mitigating the impact, due to the fact that the impact of the multipath may not be avoided in practical applications.

Discussion

Impact of the multipath on carrier phase measurements

Assume a receiver a measures the positioning reference signals from a transmitter i  to obtain the time of arrival (TOA) measurement,  Tai , and the carrier phase (CP) measurement, Pai , which can be expressed as follows

Tai=rai+cbr-bt+mai,T+wai,T ,			(1)

λPai=rai+cbr-bt+λNai+mai,P+wa,Pi,		(2)

where rai  is the geometric distance between the transmitter and the receiver, c is the speed of light, br and bt are respectively the receiver and transmitter clock offsets, λ is the wavelength of carrier frequency, Nai is the unknown integer ambiguity, mai,T and mai,P are the TOA and CP multipath errors, wai,T  and wa,Pi  are the TOA and CP measurement errors, respectively.

In this paper, we focus on the impact of multipath on the carrier phase positioning under the assumption that LOS signals are always available (Note: This assumption is reasonable for the carrier phase positioning, since it is in general used in the environment that have a good reception of LOS signals for the purpose of very high accuracy positioning). Multipath signals occur when the positioning reference signals are reflected off the surface of the objects in the vicinity of the antenna and received together with the direct signals.

Assume the LOS signal is Asin2πfct= Asinωct, where A is the signal amplitude, fc is the carrier frequency, the multipath signal is αAsinωct+Δ=αAsinωct+ϕ0, where α the reflection (attenuation) coefficient, and Δ the propagation delay of multipath signal due to the reflection, and ϕ0=Δ/λ is the phase shift due to the multipath in cycles. A simple example is shown in Figure 1, where we have Δ=dcosθ/c, where d is the distance between the antenna location and its image point, c is the speed of the light and θ is the elevation angle.

The received signal can be seen as the superposition of the direct and reflected signals:

yt=Asinωct+αAsinωct+Δ=Asinωct+αAsinωct+ϕ0		(3)

The superposed signal can also be written as follows:

yt=α'Asin⁡(ωct+ϕerr)							(4)

where

- α'=1+2αcosϕ0+α2

- ϕerr=tan-1(αsin⁡(ϕ0)1+αcos⁡(ϕ0))

ϕerr is the carrier phase error caused by the multipath signal in the received signal. The maximum value of the ϕerr for any given value of reflection coefficient α can be obtained by letting the derivative of ϕerr with respect to ϕ0 to be zero. The theoretical maximum of the carrier-multipath error is ϕerr=π2  radius for the maximum reflection coefficient α=1 ([2][3][4]), which corresponds to max⁡(ϕerr)*λ/(2π)=0.25λ of carrier-phase measurement error in range. The reflection coefficients in practice are in general much smaller than the maximum reflection coefficient. Thus, CP multipath errors are in general much smaller than the maximum theoretical values.

Observation 1: The theoretical maximum of the carrier-multipath error is about 0.25λ in range. In practice, the impact can be much smaller when the reflection (or attenuation) coefficient is smaller than 1.

Figure 1: Impact of multipath on carrier phase measurements

Multi-path mitigation for carrier phase positioning

As discussed in the previous section, the impact of the carrier-multipath error on the carrier phase measurement in range can be as large as 0.25λ. The impact of the carrier-multipath error will be enlarged when the single/double differential operations are used,  and/or the linear combinations of the carrier phase measurements (observables) are used for fast resolution of the integer ambiguity. As discussed in [5], when the carrier phase measurements (observables) obtained from multiple carrier frequencies are linearly combined properly, the combined (virtual) carrier phase measurements will have a longer (virtual) wavelength, which helps fast resolution of the integer ambiguity. The potential impact is the measurement errors of the combined carrier phase measurements can be enlarged due to the operation of the combination.

If we denote Xkm, k∈{0, 1, …, } as the data symbols of the DL/UL positioning reference signals transmitted in the m-th OFDM symbol in a slot, the received data symbols, Rkm, k∈{0, 1, …, } in the frequency domain can be expressed as:

|  | Rkm= HkXkm+ Wk; k∈{0, · · · , N-1} | (5) |
| --- | --- | --- |

where Hk is the channel frequency response (CFR), and Wk represents the measurement errors in the frequency domain.

If we assume the channel is quasi-static channel, i.e., assume the channel does not change during the transmission of one OFDM symbol, i.e., the channel impulse response h does not vary when the transmission length of one OFDM symbol is short as compared to the coherence time of the channel, the quasi-static impulse response of the multipath channel (CIR) can then be described by a time-discrete channel impulse response   h0, h1, . . . , hL-1T , where L is the number of signal paths.

|  | ht=l=0L-1hle-jϕlδt-τl | (6) |
| --- | --- | --- |

where hl, ϕl and τl are the attenuation, phase shift and delay components, respectively, of the l-th path. The channel frequency response (CFR) at the subcarrier k will be:

|  | Hk=l=0L-1hle-jϕle-2πkτl/T=l=0L-1hle-j2π(fc+kΔf)τle-jϕl* | (7) |
| --- | --- | --- |

For the purpose of positioning, our main interest is to get the signal propagation delay of the first path τ0. For carrier phase positioning, here we assume the first path is the LOS path.

There can be different approaches for the estimation of the τ0. For example, a direct approach is to use the phase difference between the CFR of the subcarriers. For example, τ0 may be estimated the average difference between the phases of Hk and Hk+Δ:

|  | τ0≈averge of (∠Hk+Δ-∠Hk)/Δ | (8) |
| --- | --- | --- |

The main limitation of the above approach is the estimation accuracy of τ0 is inversely proportional to the channel bandwidth.

For carrier phase positioning, the carrier phase information is used instead of the phase difference of the subcarriers, in which the centimeter accuracy positioning is possible with a very small sandwich as shown in GNSS  carrier phase positioning due to the small error of carrier phase measurements.

Different approaches have been proposed to mitigate the multipath error in carrier phase positioning. These can be broadly classified into the following groups:

Mitigating the impact of multipath on the measurements by the improvement of hardware and software design of the antenna and receiver. For example, the effect of carrier phase multipath in static mode is investigated and a system is developed to reduce the effect using multiple closely-spaced antennas [6]. The correlated nature of multipath, along with the known geometry among the antennas are used to estimate and mitigate carrier phase multipath. Two-step algorithm was proposed in [7] to reduce the error introduced by multipath, where the propagation delay of the LOS path is first coarsely determined, and then the carrier phase is estimated using the earlier determined coarse time delays.

Mitigating the impact of multipath on the positioning solution by advanced data processing and positioning algorithms. The impact of the multipath on the carrier phase measurements can be mitigated by using advanced algorithms to process the obtained measurements. For example, the signal-to-noise (SNR) [8] and the estimated amplitudes of the composed signal and direct signal [9] are used for multipath mitigation. MHM (multipath hemispherical map) algorithm and Variational Mode Decomposition (VMD) and MHM are used to extract the multipath from single difference (SD) residuals for mitigating multipath in [10].

Observation 2: Different approaches have been developed for multipath mitigation in carrier phase positioning, especially GNSS carrier phase positioning.

Most of the existing techniques for the multipath mitigation of carrier phase positioning are developed for GNSS carrier phase positioning, which may or may not be suitable for NR carrier phase positioning. Thus, in our view, there is a need to study whether the existing multipath mitigation techniques for GNSS carrier phase positioning can be directly adopted for NR carrier phase positioning, and whether to develop new multipath mitigation techniques for NR carrier phase positioning.

Proposal 1: The impact of multipath errors on NR carrier phase positioning as well as the multipath mitigation techniques for NR carrier phase positioning should be studied in Rel-18 SI.

Conclusions

In this contribution, we have discussed the impact of multipath errors on NR carrier phase positioning as well as the multipath mitigation techniques for NR carrier phase positioning. Based on the discussion, we have the following observations and proposals.

Observation 1: The theoretical maximum of the carrier-multipath error is about 0.25λ. In practice, the impact can be much smaller than it.

Observation 2: Different approaches have been developed for multipath mitigation in carrier phase positioning, especially GNSS carrier phase positioning.

Proposal 1: The impact of multipath errors on NR carrier phase positioning as well as the multipath mitigation techniques for NR carrier phase positioning should be studied in Rel-18 SI.

References

- 3GPP, "RP-213588, Revised SID on Study on expanded and improved NR positioning," 3GPP TSG RAN Meeting #94e, Dec. 6 - 17, 2021.

- R. Moradi, W. Schuster, S. Feng, A. Jokinen, and W. Ochieng, “The carrier-multipath observable: a new carrier phase multipath mitigation technique”, GPS Solutions, Vol. 19, Issue 1, January 2015 pp 73–82.

- J.K. Ray, and M.E. Cannon, “Characterization of GPS carrier phase multipath”,  Proceedings of ION-NTM-1999, January 25–27, pp 343–352.

- W. Dai, Q. Shi and C. Cai, “Characteristics of the BDS Carrier Phase Multipath and Its Mitigation Methods in Relative Positioning”, Sensors 2017.

- R1-2003642, “Discussion of NR positioning enhancements”, CATT

- J. Ray, M. Cannon, and P. Fenton, “Mitigation of Static Carrier Phase Multipath Effects Using Multiple Closely-Spaced Antennas”, NAVIGATION: Journal of the Institute of Navigation, Vol. 46, No. 3, 1999.

- H. Dun, C. Tiberius, and G. Hanssen, “Positioning in a Multipath Channel Using OFDM Signals With Carrier Phase Tracking,” IEEE Access, Vol. 8, 2020.

- A. Bilich, K. Larson, and P. Axelrad, “Modeling GPS phase multipath with SNR: Case study from the Salar de Uyuni, Boliva,” J. Geophysical Research, Vol. 113, 2008.

- C. Rost and L. Wanninger, “Carrier phase multipath mitigation based on GNSS signal quality measurements,” J. of Applied Geodesy, 2009.

- R. Zhang, et al., “An Improved Multipath Mitigation Method and Its Application in Real-Time Bridge Deformation Monitoring”, Remote Sensing, 2021.