3GPP TSG RAN WG1 #109-e                                                                               R1-2203475

e-Meeting, May 9th – 20th, 2022

Source:	CATT

Title:	Views on coexistence between Rel-17 and Rel-18 RedCap UE

Agenda Item:	9.6.3

Document for:	Discussion and Decision

Introduction

In Rel-17, NR supports a special type of NR devices with reduced capability, namely RedCap UE. Typically, in FR1, 20 MHz is supported as the maximum bandwidth. Besides, other capabilities are also introduced to further reduce UE complexity, e.g. reduced number of Rx branches.

In RAN#94-e, a new SID was approved to further reduce UE complexity in Rel-18 [1]. The objectives include:

- Study further UE complexity reduction techniques based on Rel-17 evaluation methodology in TR 38.875 [RAN1] - Consider network impact, coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs in a cell, UE impact, specification impact - Potential solutions, which may complement each other, for reducing device complexity are focusing on: - UE bandwidth reduction to 5MHz in FR1, - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - reduced UE peak data rate in FR1, - Possibly including restricted bandwidth for PDSCH and/or PUSCH - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - Notes: - Rel-15 SSB should be reused and L1 changes minimized. - Operation in BWP with/without SSB and without/with RF retuning should be considered. - It is not precluded that some solutions for FR1 can be applied to FR2 in WI stage. - Aim to define a single Rel-18 RedCap UE type for further UE complexity reduction.

Based on the strong interest, RAN has agreed to study further UE complexity/cost reduction in Rel-18. Potential solutions were identified, i.e. UE bandwidth reduction to 5 MHz, reduced UE peak data rate, and relaxed UE processing time. In this contribution, we provide our views on the boundary of Rel-18 RedCap UEs and other UEs, and also analyze the coexistence between Rel-17 and Rel-18 RedCap UEs. Last but not least, we express our concern in Rel-17 RedCap UE and point out that some issues may need to be addressed in Rel-18.

Discussion

Boundary between Rel-18 RedCap UE and other UEs

In the SID, it is required that ‘Rel-18 RedCap should provide NR support for low-tier devices between existing LPWA UEs and the capabilities of Rel-17 RedCap UEs’ [1]. However, the definition of ‘existing LPWA’ is a little ambiguous:

- Since Rel-14, a typical kind of LPWA UE, i.e. eMTC UE, can already support a bandwidth of 5 MHz in UL and 20MHz in DL.

- LTE Cat 1/1bis module is widely applied for IoT purpose around the world.

This implies that nowadays there may be no clear boundary for LPWA UE or module for IoT. It is hard to judge that ‘A RedCap UE with 5 MHz does not step into LPWA area’. This should make everyone rethink whether cost reduction is the only important characteristics for RedCap UE. Indeed, cost reduction is important, but it should be very clear that the compatibility for NR air interface is another important advantage of RedCap UE. This greatly affects the difficulty of network update, interest of operator, and subsequently the speed of deployment.

Observation 1: A bandwidth of 5 MHz is not a clear boundary of LPWA UE or module for IoT.

Observation 2: While cost reduction is important, the compatibility for NR air interface is also important for RedCap UEs.

In early phase of Rel-17, there was one view that two kinds of RedCap UE types should be defined. One kind is ‘high-end’ RedCap UE, which is with 20 MHz and 2 Rx branches. Another kind is ‘low-end’ RedCap UEs, which is possibly with narrower bandwidth than 20 MHz and/or 1 Rx branches. After exhaustive study, only one kind of RedCap UE was introduced. This is mainly because defining two kinds of RedCap UEs will divide the market and reduce the economy of scale.

It is understood that further cost reduction is targeted in Rel-18, but we should not pursue defining a completely new UE type. Also note that, the cost reduction of Rel-17 RedCap UE can reach ~70% with allowable combination (e.g. 20 MHz, 1 Rx branches, 1 MIMO layer, 64 QAM) [2]. If this is considered as the baseline, whether there is much room to further reduce the cost is questionable. Anyway, in Rel-18, we should avoid similar situation, i.e. making Rel-18 RedCap UE another kind of ‘Rel-17 low-end RedCap UE’, to guarantee the successful commercial deployment.

Proposal 1: We should avoid making Rel-18 RedCap UE another kind of ‘Rel-17 low-end RedCap UE’ to segment the market and reduce the economy of scale.

Co-existence of Rel-17 and Rel-18 RedCap UEs

So far, there is no clear definition for ‘RedCap UE type’ in Rel-17. However, it is almost common understanding that the basic feature group (FG 28-1) for reduced capability is the symbol of RedCap UE. Therefore, from UE capability point of view, FG 28-1 can serve as the prerequisite of other RedCap capability. This is not only applicable to Rel-17 RedCap UE but also to the RedCap UE in future release. A clear, stable relationship should be built for RedCap UEs in different releases.  It is preferred that future RedCap capabilities are ‘add-on’(s) on top of FG 28-1.

Proposal 2: From UE capability point of view, FG 28-1 should serve as the prerequisite of other RedCap capability. Future RedCap capabilities are ‘add-on’(s) on top of FG 28-1.

From view of standard, several key RedCap-specific channels/signals have been defined for Rel-17 RedCap UEs, e.g. RACH resource, separate initial DL/UL BWP, common CORESET. Also, Rel-17 RedCap UE can reuse key cell-common channels/signals as well as legacy normal UEs, e.g. SSB, CORESET#0, PDCCH for paging. In our companion paper, we analyze the co-existence of Rel-17 and Rel-18 RedCap UEs, based on different assumptions of potential solutions for cost reduction [3]. In short, if Rel-18 RedCap UE cannot support the above signals/channels, it will be a disaster to both network complexity and normative workload. We propose that Rel-18 RedCap UE shall support key Rel-17 RedCap-specific channels/signals and key cell-common signal/channels to alleviate the co-existence issue.

Proposal 3: It is desired that Rel-18 RedCap UE can support key Rel-17 RedCap-specific channels/signals and key cell-common signal/channels, including SSB, CORESET#0, PDCCH for paging, RACH resource, separate initial DL/UL BWP and common CORESET.

Conclusion

In this contribution, we analyze the solutions for further complexity/cost reduction for Rel-18 RedCap UE. The observations and proposals are summarized as follows.

Observation 1: A bandwidth of 5 MHz is not a clear boundary of LPWA UE or module for IoT.

Observation 2: While cost reduction is important, the compatibility for NR air interface is also important for RedCap UEs.

Proposal 1: We should avoid making Rel-18 RedCap UE another kind of ‘Rel-17 low-end RedCap UE’ to segment the market and reduce the economy of scale.

Proposal 2: From UE capability point of view, FG 28-1 should serve as the prerequisite of other RedCap capability. Future RedCap capabilities are ‘add-on’(s) on top of FG 28-1.

Proposal 3: It is desired that Rel-18 RedCap UE can support key Rel-17 RedCap-specific channels/signals and key cell-common signal/channels, including SSB, CORESET#0, PDCCH for paging, RACH resource, separate initial DL/UL BWP and common CORESET.

References

- RP-213661, New SID on Study on further NR RedCap UE complexity reduction, Ericsson, RAN#94-e, e-Meeting, December 6th – 17th, 2021.

- TR 38.875, Study on support of reduced capability NR devices, v17.0.0.

- R1-2203473, Discussion on solutions to further reduce UE complexity in Rel-18, CATT, RAN1#109-e, e-Meeting, May 9th – 20th.