3GPP TSG RAN WG1 #109-e	      	R1-2203476

e-Meeting, May 9th – May 20th, 2022

Source:	CATT

Title:	Side control information to enable NR network-controlled repeaters

Agenda Item:	9.8.1

Document for:	Discussion and Decision

- Introduction

The work scope of NR network-controlled repeaters was discussed in RAN#94 meeting [1], and the objectives for the SI were agreed as the following:

- The study on NR smart repeaters are to focus on the following scenarios and assumptions: - Smart repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios. - For only single hop stationary smart repeaters - Smart repeaters are transparent to UEs. - Smart repeater can maintain the gNB-repeater link and repeater-UE link simultaneously. NOTE1: Cost efficiency is a key consideration point for smart repeaters. - Study and identify which side control information below is necessary for smart repeaters including assumption of max transmission power [RAN1]: - Beamforming information - Timing information to align transmission / reception boundaries of smart repeater - Information on UL-DL TDD configuration - ON-OFF information for efficient interference management and improved energy efficiency - Power control information for efficient interference management (as the 2nd priority). - Study and identify L1/L2 signalling (including its configuration) to carry the side control information [RAN1] - Study the following aspects of smart repeater management - Identification and authorization of smart repeaters [RAN2, RAN3] NOTE2: Coordination with SA3 may be needed.

In this document, we review some typical side control information (SCI) and discuss which is/are the enabling SCI for NR network-controlled repeaters.

- Discussion

  - NR network-controlled repeaters

Coverage is an essential metric of cellular network deployments. RF repeaters have been used in 2G, 3G and 4G deployments to improve coverage. They constitute the simplest and most cost-effective way to improve network coverage.

During the discussion in Rel-17 NR repeater WI, there were discussions and several proposals for introducing assisted information for repeaters, e.g. to support dynamic TDD via timing information or to support configurable repeater bandwidth. In order to better support the intelligent amplify-and-forward operation for NR repeaters, the NR network-controlled repeaters (NCR) can be controlled by side control information (SCI).

The NR network-controlled repeater outperforms conventional RF repeaters due to the capability to receive and process SCI from the network. The SCI is transmitted by donor gNB, and potentially includes timing information, beam information, ON-OFF information, etc. For example, Figure 1 shows the difference in beamforming between RF Repeater (R17) and network-controlled repeater (R18):

- For RF Repeater, only one broadcast beam can be used because no control information for the beam direction is available.

- For network-controlled Repeater, four narrow beams can be used because the donor gNB could obtain the best beam direction through the report from UE and then send the control information about beam direction to the repeater. Because of the narrower beam width, the UE can obtain higher received SNR.

Figure 1: RF Repeaters beamforming vs. NR network-controlled repeaters beamforming.

SCI could allow the repeater to perform its amplify-and-forward operation in a more efficient manner. Considering the potential benefits and possible cost increases that are associated with control information, it is important to study which information is necessary for the NR network-controlled repeaters and how to transmit it.

For different scenarios, network-controlled repeater can provide various degrees of benefits with different required SCI. Potential benefits could include mitigation of unnecessary noise amplification, transmissions and receptions with better spatial diversity, and power saving for repeater. The parameters of SCI should be configurable for different scenarios. Take the number of beams for example, for scenario in FR2 and if all the UEs in the coverage are static or at low speed, the NCR can be configured with several narrow beams to improve the received SNR. But for FR1 deployment scenario or for high speed UE scenario in FR2, the NCR should be configured to one broadcast beam to ensure the stability of system performance.

The NR network-controlled repeater outperforms conventional RF repeaters due to the capability to receive and process SCI from the network. The SCI leads to possible cost increases as well as potential benefits.

Study which information should be included in the SCI and if these parameters are configurable for different scenarios.

  - Side Control Information

Beamforming information

From the analysis in section of 2.1, the beamforming information is necessary for potential benefits. The requirements to beamforming information are different for different scenarios, which are shown in Figure 2.

For FR1 (around 6GHz), one broadcast beam is enough for the NR network-controlled repeaters, so the beaming information is not needed. For FR2 (above 24GHz), propagation conditions which degrade fast compared with lower frequencies exacerbate the coverage challenges, so the NR network-controlled repeaters should be able to be configured with several narrow beams. Additional beamforming gain can be obtained, if the repeaters could know the beam which targeting specific UEs. Hence, beamforming information in the SCI is necessary for FR2. Considering the overhead of SCI, the maximum number of beams should be restricted. Because the degradation of propagation conditions is about 6dB, the number of beam should be up to 4.

The granularity of beamforming information is another parameter that affects the overhead of SCI. It can be per symbol, per slot or per slot group. The granularity depends on the speed of the UEs within the coverage area. If the granularity of beamforming information per symbol beam indication is introduced, considering the associated information overhead, the beamforming information can be configured per slot, and updated in a semi-static manner (e.g. 200ms granularity).

Figure 2: Beamforming information for FR1 and FR2.

For FR1, beamforming information is not necessary information. For FR2, beamforming information is necessary for potential beamforming gain.

To reduce the information overhead, the following solutions can be considered:

- Limit the number of beams, such as no more than 4.

- Time-domain granularity can be slot-based. Symbol-based granularity can be further studied.

- Update the beamforming information in a semi-static way. Dynamic indication can be further studied.

Timing information

The NR network-controlled repeater needs timing information to determine the time boundary for transmission/reception. The timing information could be derived in the same way as legacy UEs. For downlink, NCR can determine OFDM symbol boundary by detecting PSS and SSS, and read the SFN information from MIB by detecting PBCH to derive the boundary of sub-frame information. For amplify-and-forward operation, coarse timing synchronization seems sufficient. RAN1 can further study whether finer synchronization is required for SCI transmission and reception.

For uplink transmission to gNB, NCR can obtain the TA from RACH process. After TA adjustment, the uplink timing synchronization error can be within the length of CP. For amplify-and-forward operation, this seems sufficient too.

Therefore, timing information to align transmission/reception boundaries of NCR can be obtained by NCR implementation, and is not required to be carried by SCI. It is expected that the standardization effort for DL and UL timing determination of NCR can be minimized.

Timing information can be obtained by NCR implementation, and is not required to be carried by SCI.

Information on UL-DL TDD configuration

UL-DL TDD configuration is important information for NR network-controlled repeaters. According to the UL-DL TDD configuration, the NR network-controlled repeater determines the transmission and reception directions of each symbol. It also can be derived in a similar way used by legacy UEs. In NR, there are two methods to transmit information on UL-DL TDD configuration to UE:

- Configuration through SIB1

- Indicated by DCI format 2-0

SIB1 can be used to derive the configuration if full protocol stack is implemented in NCR.

For NR network-controlled repeaters, how to deal with flexible symbols is another issue. To simplify NCR operation, the flexible symbols may not be used for signal amplify-and-forward operation, which means these symbols may be muted by NCR. On other hand, flexible symbols can be used for transmission or reception of SCI on C-link following legacy rules.

As another alternative, the donor gNB can transmit information about UL-DL TDD configuration to NCR by SCI. The advantage of this method is that NCR will not read SIB-1 or RRC message and NCR is not necessary to support full stack.

The OAM configuration can also be used to indicate the UL-DL TDD configuration to NCR, but the configuration flexibility may be an issue.

UL-DL TDD configuration is important information for NCR, and it can be obtained from at least one of:

- SIB1 of donor gNB

- Side control information

- OAM configuration

If UL-DL TDD configuration can be obtained in the same way as UEs, further study is needed on how to treat with flexible symbols.

ON-OFF information and Power-Control information

The ON-OFF information is beneficial for gNB to flexibly control the NR network-controlled repeaters. Generally speaking, only part of time/frequency resources are occupied by the UEs connected to one repeater. There are many blank physical resources which could be power-off for the repeater. Efficient interference management and improved energy efficiency are expected via power on/off. At least for outdoor scenarios, the benefits of ON-OFF information are very clear. In our view, power ON-OFF operation is only for Forward-link (F-link), while the Control-link (C-link) is not affected and should perform as regular UE. Procedures such as DRX configuration can be used to reduce C-link power consumption. The concept of C-link and F-link is as follows:

- C-link: Refers to the link between the BS and NCR-controller with indexes 1 and 2 indicating DL and UL directions, respectively. Potential interaction between NCR and BS includes reception of control information, which is used for F-link.

- F-link: Refers to the link between the BS and NCR Radio Unit (NCR-RU) and the UE with indexes 1 to 4 indicating the DL and UL direction, respectively. The NCR only conducts the amplify-and-forward operation according to the control information via C-link by NCR-controller.

The illustration of F-link and C-link are shown in Figure 3.

Figure 3: C-link and F-link.

There are two possible alternatives that the ON-OFF information could be transmitted：

- RRC/MAC-CE configuration (semi-static)

- Side control information (dynamic)

The Power-Control information is a further enhancement in addition to ON-OFF information. In legacy Rel-17 repeater, the F-link power control is performed by gNB/UE, and it is transparent to repeater. For NCR, at least, no obvious motivation can be found to support dynamic power control. Whether the semi-static power control information is needed or not can be studied.

For C-link between donor gNB and repeater, the power control can be performed as regular UE operation. When the C-link and F-link share the same transmission panel, to avoid effecting F-link transmission of uplink, only digital adjustment can be implemented by NCR.

The ON-OFF information is necessary for NR network-controlled repeaters. Power ON-OFF operation is only needed for Forward-link (F-link).

Study of power-Control related information of NCR can be de-prioritized.

Power control shall follow legacy rule. To avoid effecting F-link transmission of uplink, only digital adjustment can be implemented by NCR if the C-link and F-link share the same transmission panel.

Identification information (IDI) for the donor gNB and NCR

Before a NCR (network-controlled repeater) starts to perform amplify-and-forward operation, the NCR will try to access a donor gNB which is upgraded to Rel-18. The gNB can transmit SCI，because SCI will be enable NCR work more efficiency.

There are two possible schemes to enable NCR access to the donor gNB:

Scheme 1：IDI of the donor gNB is indicated after RACH procedure

- In this scheme, NCR does cell search/selection and RACH procedure as regular UE. After radio link is established, gNB will indicate whether SCI transmission is supported or not. If the gNB does not support SCI transmission, NCR will release the connection and repeat the cell search/selection and PRACH, until it finds a gNB that support SCI transmission.

Scheme 2：IDI of the donor gNB is indicated by broadcast

- In this scheme, after finding a cell with satisfactory signal quality, NCR can obtain IDI of the donor gNB, and select the gNB that supports SCS transmission to perform access procedure.

According to the analysis above, scheme 2 is preferred. Therefore, before the NCR achieve initial access, the IDI for the donor gNB should be broadcasted to all the neighbouring NCRs. The NCR could launch random access after receiving the information. During the random access process, the identification information of the NCR can be transmitted to donor gNB, and then the donor gNB could transmit control information to the NCR by SCI. (Figure 4)

Figure 4: IDI for the donor gNB and NCR

The IDI of the donor gNB and NCR are necessary and can be transmitted as following:

- The IDI of the donor gNB can be broadcasted before NCR starts initial access.

- The IDI of the NCR can be indicated by RRC or MAC-CE.

- Conclusion

Observation 1: The NR network-controlled repeater outperforms conventional RF repeaters due to the capability to receive and process SCI from the network. The SCI leads to possible cost increases as well as potential benefits.

Proposal 1: Study which information should be included in the SCI and if these parameters are configurable for different scenarios.

Proposal 2: For FR1, beamforming information is not necessary information. For FR2, beamforming information is necessary for potential beamforming gain.

Proposal 3: To reduce the information overhead, the following solutions can be considered:

•	Limit the number of beams, such as not more than 4.

•	Time-domain granularity can be slot-based. Symbol-based granularity can be further studied.

•	Update the beamforming information in a semi-static way. Dynamic indication can be further studied.

Proposal 4: Timing information can be obtained by NCR implementation, and is not required to be carried by SCI.

Proposal 5: UL-DL TDD configuration is important information for NCR, and it can be obtained from at least one of:

•	SIB1 of donor gNB

•	Side control information

•	OAM configuration

Proposal 6: If UL-DL TDD configuration can be obtained in the same way as UEs, further study is needed on how to treat with flexible symbols.

Proposal 7: The ON-OFF information is necessary for NR network-controlled repeaters. Power ON-OFF operation is only needed for Forward-link (F-link).

Proposal 8: Study of power-Control related information of NCR can be de-prioritized.

Proposal 9: Power control shall follow legacy rule. To avoid effecting F-link transmission of uplink, only digital adjustment can be implemented by NCR if the C-link and F-link share the same transmission panel.

Proposal 10: The IDI of the donor gNB and NCR are necessary and can be transmitted as following:

- The IDI of the donor gNB can be broadcasted before NCR starts initial access.

- The IDI of the NCR can be indicated by RRC or MAC-CE.

- Reference

RP-213592, “New SI: Study on NR Smart Repeaters”, ZTE