3GPP TSG RAN WG1 #109-e	      	R1-2203477

e-Meeting, May 9th – May 20th, 2022

Source:	CATT

Title:	Discussion on signaling for side control information for NR network-controlled repeaters

Agenda Item:	9.8.2

Document for:	Discussion and Decision

- Introduction

The work scope of NR network-controlled repeaters was discussed in RAN#94 meeting [1], and the objectives for the SI were agreed as the following:

- The study on NR smart repeaters are to focus on the following scenarios and assumptions: - Smart repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios. - For only single hop stationary smart repeaters - Smart repeaters are transparent to UEs. - Smart repeater can maintain the gNB-repeater link and repeater-UE link simultaneously. NOTE1: Cost efficiency is a key consideration point for smart repeaters. - Study and identify which side control information below is necessary for smart repeaters including assumption of max transmission power [RAN1]: - Beamforming information - Timing information to align transmission / reception boundaries of smart repeater - Information on UL-DL TDD configuration - ON-OFF information for efficient interference management and improved energy efficiency - Power control information for efficient interference management (as the 2nd priority). - Study and identify L1/L2 signalling (including its configuration) to carry the side control information [RAN1] - Study the following aspects of smart repeater management - Identification and authorization of smart repeaters [RAN2, RAN3] NOTE2: Coordination with SA3 may be needed.

In this document, we discuss aspects of L1/L2 signalling for side control information (SCI), including the protocol stack and SCI design.

- Discussion

  - Concept of protocol stack

The typical NR network-controlled repeater (NCR) structure is shown in Figure 1. In Figure 1, two different function module of the NCR are given. These two modules are the control module which is used to transmit and receive the control information and the forward part which is used to forward the receiving signal. For better illustration, the links of NCR are divided into two parts: the control link (C-link) and the forward link (F-link). The concept of C-link and F-link is as follows:

- C-link: Refers to the link between the BS and NCR-controller with indexes 1 and 2 indicating DL and UL directions, respectively. Potential interaction between NCR and BS is expected including reception of control information, which is used for F-link.

- F-link: Refers to the link between the BS and NCR Radio Unit (NCR-RU) and the UE with indexes 1 to 4 indicating the DL and UL direction, respectively. The NCR only conducts the amplify-and-forward operation according to the control information from C-link by NCR-controller.

Figure 1: C-link and F-link of NR network-controlled repeaters

  - Different design principle for NCR protocol stack

In R17, repeaters only have F-link, because legacy repeater only performs amplify-and-forward operation. So the protocol stack of repeater only has a RF layer which is shown in Figure 2.

Figure 2: Protocol stack of RF repeaters (R17)

For F-link, the repeater does not detect data in OFDM symbols, so it cannot obtain the control information or decode the signal for UE transmitted in the F-link.

In Rel-18, the NR network-controlled repeater outperforms conventional RF repeaters due to the capability to receive and process SCI from the network. The SCI is transmitted in the C-link through PHY/MAC/RRC. So the protocol stack of repeater is more than just RF layer. The F-link is same as in Rel-17, and the NR network-controlled repeater cannot obtain the control information or decode the signal for UE either.

Different design of NCR protocol stack will affect the design of control signalling for SCI. Two alternatives, i.e. simple protocol stack model and full protocol stack model can be used as a candidate solution. The two models are described in detail as below.

Different design of NCR protocol stack will affect the design of control signaling for SCI.

Simple protocol stack

The simple protocol stack model is shown in Figure 3. The NCR has three layers: simple MAC, PHY and RF. The simple MAC layer can detect the C-link1 to obtain the SCI1 via DCI/MAC-CE, and can send the C-link2 to report the SCI2 by UCI/MAC-CE.

The SCI1 transmitted by gNB carries the control information which potentially includes timing information, beam information and ON-OFF information, etc. And the SCI2 transmitted by NCR carries the feedback and capability information, etc.

In this model, SCI can be only obtained from L1/L2, so it will most likely increase standardization effort associated with new L1/L2 procedures. Because the model does not have higher layers, it could reduce the cost of repeater.

Figure 3: Simple protocol stack of NCR

Full protocol stack

The full protocol stack model is shown in Figure 4. The NCR has five layers: RRC, RLC, MAC, PHY and RF.

Figure 4: Full protocol stack of NCR

The C-link1 can be divided to two parts: the SCI1 transmits the semi-static or periodic control information to the NCR and the SCI3 transmits the dynamic or aperiodic control information to the NCR. The SCI1 and SCI3 can transmit the same or different types of information including timing information, beam information and ON-OFF information, etc. For the same type of information transmitted in SCI1 and SCI3, the information in SCI3 can override the information in SCI1.

The C-link2 can also be divided to two parts: The SCI2 transmitted by NCR carries the capability information and the SCI4 transmitted by NCR carries the feedback information.

In this model, some SCI can be obtained by legacy procedures (e.g. TDD configuration) as this could reduce standardization effort. The existence of higher layers in the model will lead to some increased cost (associated with implementation of L2/L3 protocol). One possible way to alleviate the difficulty is to reuse the UE chipset in Rel-17 and modify the RF module. But this will depends on the characteristics of the chipset implementation and is not easy to evaluate.

The work assumption of protocol stack in NCR shall be discussed and down selected as following two candidates:

- Candidate 1：Simple protocol stack model;

- Candidate 2：Full protocol stack model.

For full protocol stack as work assumption, the C-link1 can be divided to two parts: the SCI1 transmits the semi-static or periodic control information to the NCR and the SCI3 transmits the dynamic or aperiodic control information to the NCR.

For full protocol stack as work assumption, the C-link2 can be divided to two parts: The SCI2 transmitted by NCR carries the ability level information and the SCI4 transmitted by NCR carries the feedback information.

- Conclusion

Observation 1: Different design of NCR protocol stack will affect the design of control signalling for SCI.

Proposal 1: The work assumption of protocol stack in NCR shall be discussed and down selected as following two candidates:

-	Candidate 1：Simple protocol stack model;

-	Candidate 2：Full protocol stack model.

Proposal 2: For full protocol stack as work assumption, the C-link1 can be divided to two parts: the SCI1 transmits the semi-static or periodic control information to the NCR and the SCI3 transmits the dynamic or aperiodic control information to the NCR.

Proposal 3: For full protocol stack as work assumption, the C-link2 can be divided to two parts: The SCI2 transmitted by NCR carries the ability level information and the SCI4 transmitted by NCR carries the feedback information.

- Reference

RP-213592, “New SI: Study on NR Smart Repeaters”, ZTE