3GPP TSG RAN WG1 #109-e		R1-2203479

e-Meeting, May 9th – 20th, 2022

# Source:             CATT

Title:                  Remaining issues of PDCCH monitoring adaptation

Agenda Item:    8.7.2

Document for:  Discussion and Decision

Introduction

Most of the remaining issues had been discussed and agreed below with text proposals captured in the specification in RAN1#108bis-e. Some remaining issues need to be further discussed in RAN1#109-e.

Working Assumption (Excerpted from RAN#107bis-e) - The bit length of the candidate skipping values and SSSG switching initial timer values in slots for 480kHz and 960kHz SCS are assumed to be the same as that for 120KHz SCS Agreement - The 480kHz and 960kHz SCS are also supported for Rel-17 PDCCH monitoring adaptation. Agreement - It is up to B52.6GHz to deicide the values for candidate skipping values and SSSG switching initial timer values. Agreement For DCI format 0_1, DCI format 0_2, DCI format 1_1 and DCI format 1_2, - 1 or 2 bits, if PDCCHSkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured - 1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2; - 2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0, search space set(s) with group index 1 and search space set(s) with group index 2; Note: Same as in existing specifications, specifications allow gNB to configure a search-space set with zero PDCCH candidates. Agreement Answer 1 for R1-2200884 (R2-2201960): Yes, PDCCH skipping can be applied to C-RNTI(s) and MCS-C-RNTI(s) in at least Type3 PDCCH CSS and USS monitored during RAR/MsgB window for RAR/MsgB reception based on RAN1 agreements till RAN1#107bis-e. Agreement Answer 2 for R1-2200884 (R2-2201960): RAN1 would discuss and conclude how to capture the above RAN2 agreements in RAN1 specification. o It is RAN1 understanding PDCCH skipping is not applied to perform PDCCH monitoring during RAR window/MsgB window/contention resolution timer or when SR is pending. - FFS: If the UE is skipping PDCCH monitoring for a duration and at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, the UE stops PDCCH skipping (i.e., PDCCH skipping is not activated ). Agreement Answer 4 for R1-2200884 (R2-2201960):  It is RAN1’s understanding that transmission of CSI/SRS is not affected by PDCCH skipping i.e. CSI/SRS is still transmitted during the skipping duration. Agreement  It is RAN1 understanding that for DRX operation, - As DCP is monitored only outside active time, it cannot be missed due to PDCCH skipping as skipping applies only in active time. Note: RAN1 is discussing whether PDCCH skipping duration can apply to outside active time. Agreement If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE - Resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by - Working Assumption: C-RNTI/CS-RNTI/MCS-C-RNTI - Otherwise, decrease the timer value by one after each slot. - Note: There is no timer for SSSG switching is running for UE in SSSG#0 Agreement [Draft] LS reply on PDCCH skipping, R1-2202896 is endorsed in principle. Agreement Final LS reply on PDCCH skipping, R1-2202905 is endorsed Working Assumption - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B), - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - Note: Pswitch is defined in Table 10.4-1 in TS38.213 Agreement - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating PDCCH skipping (i.e., Beh 1A), select one of the following schemes - Alt 1a: the UE applies Beh 1A on an active BWP of the serving cell at the first slot after the last OFDM symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - FFS: whether the UE will monitor PDCCH when DRX Retransmission timer is running Agreement - When the timer expires, the UE monitors PDCCH on the serving cell according to search space sets with group index 0 in a slot - no earlier than [application delay] after the slot where timer expires and - if PDCCH skipping is being applied, no earlier than in slot where PDCCH skipping duration expires. Working assumptions: - When the UE receives DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication' field in slot n, the UE does not expect to receive DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay. - If SSSG timer expires in slot n, the UE dos not expect to receive DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

In this contribution, we first discuss the remaining issues of PDCCH skipping procedure in Section 2. The discussion of the remaining issues on SSSG switching timer and application delay are in Section 3 and Section 4, respectively. The working assumptions would be discussed whether it should be confirmed in Section 5.

The remaining issues of the PDCCH skipping procedure

The UE PDCCH skipping behavior before the new adaptation indication received

It was agreed in RAN1#106bis-e that the UE behavior of receiving indication of PDCCH monitoring adaptation includes the PDCCH skipping and/or SSSG switching. The joint indication of the PDCCH skipping and switching between 2 SSSGs had been agreed in RAN1#107-e. The application delay for PDCCH skipping and SSSG switching had been agreed in RAN1#108-e. However, the UE behavior before receiving the new PDCCH monitoring adaptation indication has not been further discussed in previous meeting, which is mainly discussed in this section.

When SSSG and SSSG switching are not configured, the PDCCH skipping is a straightforward UE behavior of skipping monitoring PDCCH for the indicated duration. UE is configured with PDCCH monitoring adaptation indication in scheduling DCI formats 1_1, 1_2, 0_1, or 0_2. The number of adaptation indication for PDCCH skipping is up to 2 bits. Each code point of adaptation indication map to a value in the set of skipping durations configured in PDCCHSkippingDurationList. The PDCCH monitoring adaptation indication will always be included in the scheduling DCI. When there is no data transmission, the UE behavior whether to skip the same interval of skipping PDCCH monitoring or to recover PDCCH monitoring as the configured monitoring occasion in SearchSpace after the PDCCH skipping duration completion. The detailed analysis is provided as following:

If UE resumes the normal PDCCH monitoring at the configured monitoring occasion from SearchSpace indication after the end of PDCCH skipping duration, there are some drawbacks for this UE behavior:

- The one-shot PDCCH skipping cannot achieve  the  power saving gain when there is no subsequent data transmission since the non-scheduling DCI is supported to be used for indicating the PDCCH monitoring adaptation when there is no DL data to be scheduled.

- Even if the dummy grant DCI is used to indicate the new PDCCH monitoring adaptation triggered by MAC, UE would not go to sleep because the inactive timer is reset again and again for the PDCCH indicates a new transmission (DL or UL) of a dummy grant.

The other alternative is that UE will continue the same behavior as that of SSSB switching by   skipping PDCCH monitoring until UE receive a new DCI to indicate new interval of skipping PDCCH monitoring with the advantages  as follows,

- gNB knows the interval in terms of number of slots that UE will skip monitoring PDCCH and will send any DCI at the next UE PDCCH monitoring occasion.

- If UE miss-detects the DCI with the adaptation indication, UE will not skip any slot of the PDCCH monitoring. gNB would not send any DCI during the interval where gNB expects UE to skip PDCCH monitoring. There is no impact to the UE for not receiving the new indication of skipping interval with any DCI misdetection.

- In order to avoid the miss-alignment between the gNB and UE, the shorter skipping duration would be chosen for the uncertain PDCCH receiving, such as the PDCCH for retransmission, the miss-detected PDCCH, etc, since the PDCCH skipping is used to reduce PDCCH monitoring during the period of data arrival uncertainty when drx-InactivityTimer is running. The small skipping duration would ensure the reduction of PDCCH monitoring but also minimizing the potential latency of DL data scheduling when subsequent data arrives.

- When there is no scheduling DCI to indicate UE, UE will keep the power saving state to continue skipping the same interval of PDCCH monitoring until UE receive a new DCI to indicate new interval of skipping PDCCH monitoring.

Thus, UE would keep skipping the interval of PDCCH monitoring after receiving the PDCCH skipping indication with a skipping interval until UE receives a new PDCCH skipping indication.

Proposal 1：If UE is indicated in skipping an interval of the PDCCH monitoring, UE would keep skipping the interval of PDCCH monitoring and monitor the PDCCH at the next PDCCH monitoring occasion after the skipping interval until UE receives a new PDCCH skipping indication.

Proposal 2: The Text Proposal 1 below is adopted.

--------------------------------- Start of Text Proposal 1 for TS 38.213------------------------------------ < Unchanged parts are omitted > 10.4 Search space set group switching and skipping of PDCCH monitoring < Unchanged parts are omitted > A UE can be provided a set of durations by PDCCHSkippingDurationList for PDCCH monitoring on an active DL BWP of a serving cell and, if the UE is not provided searchSpaceGroupIdList-r17 on the active DL BWP of the serving cell, a DCI format 0_1 and a DCI format 0_2 that schedule PUSCH transmission, and a DCI format 1_1 and a DCI format 1_2 that schedule PDSCH receptions can include a PDCCH monitoring adaptation field of 1 bit or of 2 bits. If the field has 1 bit and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell - a '0' value for the bit indicates no skipping in PDCCH monitoring - a '1' value for the bit indicates skipping PDCCH monitoring for a duration provided by the first value in the set of durations If the field has 2 bits and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell - a '00' value for the bits indicates no skipping in PDCCH monitoring - a '01' value for the bits indicates skipping PDCCH monitoring for a duration provided by the first value in the set of durations - a '10' value for the bits indicates skipping PDCCH monitoring for a duration provided by the second value in the set of durations - a '11' value for the bits indicates skipping PDCCH monitoring for a duration provided by the third value in the set of durations, if any; otherwise, if the set of durations includes two values, a use of the '11' value is reserved When UE is indicated skipping PDCCH monitoring for a duration in a DCI format 0_1, DCI format 1_1, DCI format 0_2, and/or DCI format 1_2, UE would keep skipping the indicated interval until UE receives a new PDCCH skipping indication. < Unchanged parts are omitted > ----------------------------------- End of Text Proposal 1 for TS 38.213--------------------------------------------------

The PDCCH skipping duration outside C-DRX active time

Agreement - It is RAN1 understanding that for DRX operation, - As DCP is monitored only outside active time, it cannot be missed due to PDCCH skipping as skipping applies only in active time. Note: RAN1 is discussing whether PDCCH skipping duration can apply to outside active time.

According to the scope of the UE power saving item enhancement work item in the following in RP-212630[2], the PDCCH monitoring adaptation would be studied and specified during DRX Active time. In particular, DRX functionality is configured by RRC as the MAC entity to control the UE's PDCCH monitoring activity. If DRX is configured, UE monitors the PDCCH discontinuously and only have PDCCH monitoring at the DRX ON duration. The PDCCH monitoring adaptation includes the PDCCH skipping or SSSG switching should be only applied in DRX active time.

- Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4] - Study and specify, if agreed, extension(s) to Rel-16 DCI-based power saving adaptation during DRX Active Time for an active BWP, including PDCCH monitoring reduction when C-DRX is configured [RAN1]

Furthermore, when UE is indicated to skip the PDCCH monitoring, UE would reduce PDCCH monitoring during the skipping duration. If the PDCCH skipping duration is applied outside active time, UE would skip the PDCCH monitoring in Type3 CSS and USS during the PDCCH skipping duration, where the PDCCH monitoring including DCI format 1_0 for SPS retransmission and BFR and DCI format 2_6, the retransmission of SPS PDSCH, etc. would be impacted. The exception of PDCCH monitoring at the DRX-OFF duration is designed for the service-specific operation, such as SPS, or critical system operation aspects, such as beam failure recovery beyond the MAC control of UE PDCCH monitoring. The PDCCH skipping is for gNB to indicate the skipping the unnecessary PDCCH monitoring. Skipping the exceptional PDCCH monitoring occasion for special service or critical mission is against the principle of PDCCH skipping. Thus, PDCCH skipping duration would not be applies to outside active time, i.e. the PDCCH skipping duration would be stopped outside the DRX active time.

Proposal 3: PDCCH skipping duration would not be applies to outside active time, i.e. the PDCCH skipping duration would be stopped outside the DRX active time.

The remaining issues of resetting the SSSG switching timer

Agreement If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on a serving cell and the timer is running, the UE - Resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by - Working Assumption: C-RNTI/CS-RNTI/MCS-C-RNTI - Otherwise, decrease the timer value by one after each slot. - Note: There is no timer for SSSG switching is running for UE in SSSG#0.

Unlike the SSSG switching timer in Rel-16 NR-U, the SSSG switching timer was introduced in SSSG switching by scheduling DCI to avoid ambiguous in understanding between UE and gNB as the fall back mechanism to switching back to the default SSSG (SSSG#0) after timer expired. Since the UE is only provided the group indexes searchSpaceGroupIdList-r17 for the Type3-PDCCH CSS set or USS set, the PDCCH with the CRC scrambled by the following RNTI would impact the SSSG timer to reset including

- a Type3-PDCCH CSS set for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, CI-RNTI , C-RNTI, MCS-C-RNTI, CS-RNTI(s), AI-RNTI, PS-RNTI, G-RNTI, or G-CS-RNTI

- a USS set for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, CS-RNTI(s), AI-RNTI, SL-RNTI, SL-CS-RNTI, SL Semi-Persistent Scheduling V-RNTI.

For the above RNTI, as specified in TS38.321[4]: “If Sidelink resource allocation mode 1 is configured by RRC, a DRX functionality is not configured.”, the PDCCH with CRC scrambled by SL-RNTI, SL-CS-RNTI or SL Semi-Persistent Scheduling V-RNTI would not be affected by PDCCH skipping. Furthermore, the multicast DCI formats that are with CRC scrambled by G-RNTI or G-CS-RNTI and the DCI formats would also not be applied by the PDCCH monitoring adaptation.

According to the UE power saving enhancement WID in RP-212630, the enhanced PDCCH monitoring adaptation is studied and specified during DRX Active timethe, therefore the PDCCH with CRC scrambled by PS-RNTI monitored outside the DRX active time would not be impacted by PDCCH skipping as described in Section 2.2.

However, the PDCCH with others RNTI in Type3 CSS and USS, such as INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, CI-RNTI, C-RNTI, CS-RNTI(s), SP-CSI-RNTI and AI-RNTI are controlled by DRX functionality, which could be affected by PDCCH skipping.

Since the AI-RNTI is used by IAB-MT for detection of DCI format 2_5 indicating AvailabilityCombinationId for an IAB-DU's cells [5], the UE power saving for the IAB-MT should be separately discussed. Thus, the PDCCH with CRC scrambled by AI-RNTI would not be considered by PDCCH skipping as described in Section 2.2.

Thus, UE resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI/INT-RNTI/SFI-RNTI/TPC-PUSCH-RNTI/TPC-PUCCH-RNTI/TPC-SRS-RNTI/CI-RNTI/SP-CSI-RNTI-.

Proposal 4: UE resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI/INT-RNTI/SFI-RNTI/TPC-PUSCH-RNTI/TPC-PUCCH-RNTI/TPC-SRS-RNTI/CI-RNTI/SP-CSI-RNTI.

Proposal 5: The TP 2 below is adopted.

--------------------------------- Start of Text Proposal 2 for TS 38.213------------------------------------ < Unchanged parts are omitted > 10.4 Search space set group switching and skipping of PDCCH monitoring < Unchanged parts are omitted > If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on an active DL BWP of a serving cell and the timer is running, the UE - decrements the timer after a slot of the active DL BWP of the serving cell when the UE does not detect a DCI format in a PDCCH reception in the slot - resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI/INT-RNTI/SFI-RNTI/TPC-PUSCH-RNTI/TPC-PUCCH-RNTI/TPC-SRS-RNTI/CI-RNTI/SP-CSI-RNTI. < Unchanged parts are omitted > ----------------------------------- End of Text Proposal 2 for TS 38.213--------------------------------------------------

The remaining issues for application delay for PDCCH monitoring adaptation

Working Assumption - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B), - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - Note: Pswitch is defined in Table 10.4-1 in TS38.213 Agreement - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating PDCCH skipping (i.e., Beh 1A), select one of the following schemes - Alt 1a: the UE applies Beh 1A on an active BWP of the serving cell at the first slot after the last OFDM symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - FFS: whether the UE will monitor PDCCH when DRX Retransmission timer is running

The application delay of the PDCCH skipping and the SSSG switching had been reached consensus in RAN1#108-e, which has no interaction with retransmission. However, there are still some remaining issues needed to be further discussed, such as whether the minimum applicable scheduling offset would be considered in application delay or not and whether the PDCCH skipping is impacted by the DRX retransmission timer. In this section, these issues would be further discussed.

Whether the minimum applicable scheduling offset is considered in the application delay

In Rel-16 AI-Power Saving, the minimum applicable scheduling offset was introduced to inform UE the minimum time interval between the PDCCH and its scheduled PDSCH, in which UE would buffer the PDSCH after the minimum applicable scheduling offset and UE go to micro sleep state. During the specification discussion procedure, Proposal in [3] believes that the relaxing PDCCH processing time could obtain the power saving gain when the minimum applicable scheduling offset is configured. Similarly, the minimum applicable scheduling offset would be considered in the application delay for either the SSSG switching or the PDCCH skipping after the scheduling DCI with the monitoring adaptation indication received.

However, UE would not achieve power saving gain with long micro sleep while PDCCH processing is relaxed and extended with the configured the minimum scheduling offset.

Thus, it is clear that the minimum  scheduling offset should not have impact to the PDCCH decoding time and would not be considered as  the application delay for either the SSSG switching or the PDCCH skipping after the monitoring adaptation indication received.

Proposal 6: Confirm the working assumption:

- Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B),

  - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception

  - Note: Pswitch is defined in Table 10.4-1 in TS38.213

Proposal 7: The minimum applicable scheduling offset would not be considered in the application delay for either the SSSG switching or the PDCCH skipping after the monitoring adaptation indication received.

Whether the PDCCH skipping is impacted by the DRX retransmission timer

As specified in TS 38.321 as follows, when the DRX retransmission timer is running, UE would monitoring the PDCCH on the Serving Cells as the SearchSpace set configuration. However, if the UE had been indicated to skip the PDCCH monitoring for a skipping duration, which is partial or full overlapped with the DRX retransmission timer, the UE behavior would be further discussed and specified.

1> if a drx-HARQ-RTT-TimerDL expires: 2> if the data of the corresponding HARQ process was not successfully decoded: 3> start the drx-RetransmissionTimerDL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerDL. 1> if a drx-HARQ-RTT-TimerUL expires: 2> start the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerUL. …… 1> if a DRX group is in Active Time: 2> monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6]; ……

As the PDCCH skipping procedure described in Section 2, when UE is indicated to stop the PDCCH monitoring for a duration, UE would keep skipping the same interval of PDCCH monitoring and monitor the PDCCH at the next PDCCH monitoring occasion after the skipping interval until UE receives a new PDCCH skipping indication. Even if the DRX Retransmission timer is running, UE could also monitor the PDCCH for the indicated skipping interval and gNB could send the DCI with the retransmission information at the next UE PDCCH monitoring occasion. The retransmission would not be impacted by the PDCCH skipping.

Thus, the PDCCH skipping would not be impacted when DRX Retransmission timer is running.

Proposal 8: The PDCCH skipping would not be impacted when DRX Retransmission timer is running.

Others

The values of the candidate SSSG switching timer and the skipping values for the different SCSs, including 15kHz, 30kHz, 60kHz and 120kHz, had been finalized and agreed in RAN1#107bis-e. The working assumption was made in RAN1#107bis-e that the bit length of the candidate skipping values and SSSG switching initial timer values in slots for 480kHz and 960kHz SCS are assumed to be the same as those for 120KHz SCS. The following wording assumptions can be confirmed.

Proposal 9: Confirm the Working Assumption:

The bit length of the candidate skipping values and SSSG switching initial timer values in slots for 480kHz and 960kHz SCS are assumed to be the same as that for 120KHz SCS.

In order to maintain the single PDCCH monitoring adaptation behaviour, when UE is indicated SSSG switching, or PDCCH skipping and SSSG switching timer expires, UE would not expected to be indicated the different monitoring adaptation behaviour before the application delay. The following wording assumptions can be confirmed.

Proposal 10: Confirm the Working Assumptions:

- When the UE receives DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication' field in slot n, the UE does not expect to receive DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

- If SSSG timer expires in slot n, the UE dos not expect to receive DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

Conclusion

The proposals are summarized as follows:

Proposal 1：If UE is indicated in skipping an interval of the PDCCH monitoring, UE would keep skipping the interval of PDCCH monitoring and monitor the PDCCH at the next PDCCH monitoring occasion after the skipping interval until UE receives a new PDCCH skipping indication.

Proposal 2: The Text Proposal 1 below is adopted.

--------------------------------- Start of Text Proposal 1 for TS 38.213------------------------------------ < Unchanged parts are omitted > 10.4 Search space set group switching and skipping of PDCCH monitoring < Unchanged parts are omitted > A UE can be provided a set of durations by PDCCHSkippingDurationList for PDCCH monitoring on an active DL BWP of a serving cell and, if the UE is not provided searchSpaceGroupIdList-r17 on the active DL BWP of the serving cell, a DCI format 0_1 and a DCI format 0_2 that schedule PUSCH transmission, and a DCI format 1_1 and a DCI format 1_2 that schedule PDSCH receptions can include a PDCCH monitoring adaptation field of 1 bit or of 2 bits. If the field has 1 bit and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell - a '0' value for the bit indicates no skipping in PDCCH monitoring - a '1' value for the bit indicates skipping PDCCH monitoring for a duration provided by the first value in the set of durations If the field has 2 bits and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell - a '00' value for the bits indicates no skipping in PDCCH monitoring - a '01' value for the bits indicates skipping PDCCH monitoring for a duration provided by the first value in the set of durations - a '10' value for the bits indicates skipping PDCCH monitoring for a duration provided by the second value in the set of durations - a '11' value for the bits indicates skipping PDCCH monitoring for a duration provided by the third value in the set of durations, if any; otherwise, if the set of durations includes two values, a use of the '11' value is reserved When UE is indicated skipping PDCCH monitoring for a duration in a DCI format 0_1, DCI format 1_1, DCI format 0_2, and/or DCI format 1_2, UE would keep skipping the indicated interval until UE receives a new PDCCH skipping indication. < Unchanged parts are omitted > ----------------------------------- End of Text Proposal 1 for TS 38.213--------------------------------------------------

Proposal 3: PDCCH skipping duration would not be applies to outside active time, i.e. the PDCCH skipping duration would be stopped outside the DRX active time.

Proposal 4: UE resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI/INT-RNTI/SFI-RNTI/TPC-PUSCH-RNTI/TPC-PUCCH-RNTI/TPC-SRS-RNTI/CI-RNTI/SP-CSI-RNTI.

Proposal 5: The TP 2 below is adopted.

--------------------------------- Start of Text Proposal 2 for TS 38.213------------------------------------ < Unchanged parts are omitted > 10.4 Search space set group switching and skipping of PDCCH monitoring < Unchanged parts are omitted > If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring on an active DL BWP of a serving cell and the timer is running, the UE - decrements the timer after a slot of the active DL BWP of the serving cell when the UE does not detect a DCI format in a PDCCH reception in the slot - resets the timer after a slot of the active DL BWP of the serving cell when the UE detects a DCI format in a PDCCH reception in the slot with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI/INT-RNTI/SFI-RNTI/TPC-PUSCH-RNTI/TPC-PUCCH-RNTI/TPC-SRS-RNTI/CI-RNTI/SP-CSI-RNTI. < Unchanged parts are omitted > ----------------------------------- End of Text Proposal 2 for TS 38.213--------------------------------------------------

Proposal 6: Confirm the working assumption:

- Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B),

  - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception

  - Note: Pswitch is defined in Table 10.4-1 in TS38.213

Proposal 7: The minimum applicable scheduling offset would not be considered in the application delay for either the SSSG switching or the PDCCH skipping after the monitoring adaptation indication received.

Proposal 8: The PDCCH skipping would not be impacted when DRX Retransmission timer is running.

Proposal 9: Confirm the Working Assumption:

The bit length of the candidate skipping values and SSSG switching initial timer values in slots for 480kHz and 960kHz SCS are assumed to be the same as that for 120KHz SCS.

Proposal 10: Confirm the Working Assumptions:

- When the UE receives DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication' field in slot n, the UE does not expect to receive DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

- If SSSG timer expires in slot n, the UE dos not expect to receive DCI format 0_1 /1_1/0_2/1_2 with a ‘PDCCH monitoring adaptation indication’ field indicating change to another SSSG or skipping for the same active BWP of the scheduling cell before slot n+X of the scheduling cell, where X is the value of the application delay.

References

  - Chairman’s Notes, RAN1#108-e, Agreements for UE Power Saving Enhancements.

  - WID RP-212630, Revised WID UE Power Saving Enhancements for NR.

  - R1-1906981, On cross-slot scheduling power saving techniques, Sumsung.

  - 3GPP TS 38.321,v16.7.0, (2021-12): “NR; Medium Access Control (MAC) protocol specification”.

  - 3GPP TS 38.331, v16.7.0 (2021-12): “Radio Resource Control (RRC) protocol specification”.