3GPP TSG RAN WG1 #109-e                                                   R1-2203480

e-Meeting, May 9th – 20th, 2022

Source:              CATT

Title:                   Discussion on UE feature of UE Power saving enhancements for NR

Agenda Item:     8.16.7

Document for:   Discussion and Decision

# Introduction

The UE feature of UE power saving enhancements for NR [1] was discussed in RAN1#108-e.  Feature Groups 29-1 Paging Enhancement, 29-2 TRS resources for IDLE/Inactive UEs, and 29-3 PDCCH monitoring adaptation were agreed as the place holder for further discussion.

Agreement

- FG 29-2 is updated as follows

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability | FFS | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode UE cannot receive TRS resources for idle/inactive mode | Per UE N/A | N/A | N/A | N/A | Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7 | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- The type of FG 29-1 is per band

Agreement

- The type of FGs 29-3a to 29-3d is per band

Agreement

FG 29-1 is updated as follows

| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support receiving paging early indication in DCI format 2_7 2. Support receiving UE subgroup indication in DCI format 2_7 [3. For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot] |  | Yes |  | UE does not support paging enhancement | Per band | N/A | N/A | N/A | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- Revise component of 29-3a as: Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured

- Revise component of FG 29-3b as: Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based SSSG switching, without PDCCH skipping if PDCCHSkippingDurationList is not configured

- Revise component of FG 29-3c as: Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based SSSG switching, without PDCCH skipping if PDCCHSkippingDurationList is not configured

- Revise component of FG 29-3d as: Support of 2-bit indication of SSSG switching between 2 SSSGs, with PDCCH skipping by scheduling DCI, and timer based SSSG switching

This paper further discusses the remain aspects of FG29 of UE power saving enhancements for NR.

# UE Features of UE Power Saving Enhancements for NR

In [1], the objectives of UE power saving enhancements are as follows,

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1]

  - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1]

- NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #102-e meeting

  - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1]

- NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required

- Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4]

  - Study and specify, if agreed, extension(s) to Rel-16 DCI-based power saving adaptation during DRX Active Time for an active BWP, including PDCCH monitoring reduction when C-DRX is configured [RAN1]

- NOTE: Rel-15 and Rel-16 available power saving solutions should be supported by the UE and included in the evaluation. RAN1 will ask the confirmation from RAN2 that Rel-15 and Rel-16 available power saving solutions are properly utilized.

  - Study the feasibility and performance impact of relaxing UE measurements for RLM and/or BFD, particularly for low mobility UE with short DRX periodicity/cycle, and specify, if agreed, relaxation in the corresponding requirements [RAN4]

- NOTE: Supplementary RAN2 work, if needed, can be triggered by RAN4 LS

## Paging Enhancement

For UE in RRC_IDLE state and not yet having AS security activation in RRC_CONNECTED state, UE has not reported its UE capability to the network.   The network does not know the number of UEs not reporting its capability in supporting UE ID-based subgrouping.  If RAN would like to capture UE capability of UE ID-based subgrouping for all IDLE/Inactive UEs, the IDLE/Inactive UE procedures and the UE capability transfer need to be enhanced to support the UE capability report of UE ID-based subgrouping when UE camping at each cell.   RAN2 agreed to have number of subgroups set to 1 if network does not support paging subgrouping.  The number of paging subgroups is always present if the PEI is configured.  The SearchSpace configured for PEI is generally a normal search space configuration without any specific restriction if SearchSpaceID is greater than 0.    Thus, the UE capability of paging enhancement FG29-1 does not need to capture the specific configuration for Type 2 PDCCH-CSS.

Proposal 1:  UE capability of paging enhancement FG29-1 does not need to capture the specific configuration for Type 2 PDCCH-CSS

| NR_UE_ pow_sav_enh | 29-1 | Paging enhancement | 1. Support receiving paging early indication in DCI format 2_7 2. Support receiving UE subgroup indication in DCI format 2_7 3. For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot] |  | N |  | UE does not support paging enhancement | Per band | N | N | N |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## TRS Configuration for IDLE/Inactive UEs

For IDLE/Inactive UE power saving by the additional TRS/CSI-RS configuration, SIB-X is used to broadcast the configuration of TRS/CSI-RS resource with L1 signaling for the indication of TRS availability dynamically.  The UE capability of TRS should be the UE obtaining the TRS configuration from the SIB and the L1 signaling from DCI formats in Paging DCI and/or PDCCH-based PEI.   Since TRS/CSI-RS availability would be indicated by L1 signaling dynamically, it is UE implementation whether to use TRS/CSI-RS for AGC or channel tracking even it is configured and available.  It was agreed in RAN1#107b-e that TRS availability indication field is present in DCI format 2_7 (if configured) with CRC scrambled by PEI-RNTI and DCI format 1_0 with CRC scrambled by P-RNTI.   UE could elect to retrieve L1 signaling of TRS availability indication from either PEI, paging DCI or both whenever CRC check passes at a DCI monitoring occasion.

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability |  | N |  | UE cannot receive TRS resources for idle/inactive mode | N/A | N | N | N | Receiving L1 indication via DCI format 2_7 is supported only if the UE supports FG 29-1 | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3:  The UE capability of TRS should be the UE obtaining the TRS configuration from the SIB and the L1 signaling from DCI format 2_7 for PEI if configured and DCI format 1_0 for Paging DCI.

## PDCCH Adaptation

The objective of CONNECTED UE power saving with reducing PDCCH monitoring reduction is achieved by dynamic adaptation of PDCCH monitoring interval.  It was agreed in RAN1#106-e that up to 2 bits are include in the scheduling DCI format 1_1, 1_2, 0_1, and 0_2 to indicate PDCCH monitoring adaptation.     The UE feature of PDCCH adaptation discussion in RAN1#108-e had agreed to have UE feature groups of FG29-3a and FG29-3d operation.   FG29-3c has the prerequisite of UE support of 2 SSSG switching in FG29-3b.  FG29-3d has the prerequisite of both FG29-3a and FG29-3b.    The UE feature for PDCCH monitoring adaptation would also include the indication from scheduling DCI for SSSG switching per UE.

Proposal 4:  The UE capability of PDCCH monitoring adaptation for CONNECTED mode UE FG29-3b and FG29-3c should be per UE.

| NR_UE_ pow_sav_enh | 29-3a | PDCCH skipping | Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |  | Y |  | UE could not reduce PDCCH monitoring configured by the given search space. | Per band | N | N | N |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_UE_ pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching if PDCCHSkippingDurationList is not configured |  | Y |  | UE could not reduce the PDCCH monitoring with the SSSG switching | Per UE | N | N | N |  | Optional with capability signaling |
| NR_UE_ pow_sav_enh | 29-3c | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching if PDCCHSkippingDurationList is not configured | 29-3b | Y |  | UE could not reduce the PDCCH monitoring with the SSSG switching | Per UE | N | N | N |  | Optional with capability signaling |
| NR_UE_ pow_sav_enh | 29-3d | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs PDCCH skipping by scheduling DCI and timer based SSSG switching | 29-3a, 29-3b | Y |  | UE could not reduce the PDCCH monitoring by PDCCH skipping and/or SSSG switching | Per band | N | N | N |  | Optional with capability signaling |

# Conclusion

In this contribution, we have discussed the UE features for UE power saving enhancements for NR.  We have the following proposals

- Proposal 1:  UE features of power saving enhancement for IDLE/Inactive UEs should be optional with capability signaling

- Proposal 2: UE capability of FG29-1 paging enhancement for IDLE/Inactive UE power saving should be based on the support of both PDCCH-based PEI with new DCI format and paging subgroup indication.  The paging enhancement should be per UE

- Proposal 3:  The UE capability of TRS should be the UE obtaining the TRS configuration from the SIB and the L1 signaling from DCI format 2_7 for PEI if configured and DCI format 1_0 for Paging DCI.    There is no prerequisite of UE feature support of FG29-1.

- Proposal 4:  The UE capability of PDCCH monitoring adaptation for CONNECTED mode UE should be per UE.

We proposed to use Table 1for the UE feature for UE power saving enhancements for NR.

Table 1:  UE capability for UE power saving enhancements

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory / Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_UE_ pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication . For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot] |  | N |  | UE does not support paging enhancement | Per band | N | N | N |  | Optional with capability signaling |
| NR_UE_ pow_sav_enh | 29-2 | TRS for IDLE/Inactive UEs | TRS resource configuration for IDLE/Inactive UEs - Support of SIB decoding for the configuration of TRS resource and L1 signaling for availability indication - Support paging DCI and new DCI format for PEI with additional bit for TRS availability indication |  | N |  | UE cannot receive TRS resources for idle/inactive mode | N/A | N | N | N | Receiving L1 indication via DCI format 2_7 is supported only if the UE supports FG 29-1 | Optional without capability signaling |
| NR_UE_ pow_sav_enh | 29-3a | PDCCH skipping | Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |  | Y |  | UE could not reduce PDCCH monitoring configured by the given search space. | Per band | N | N | N |  | Optional with capability signaling |
| NR_UE_ pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching if PDCCHSkippingDurationList is not configured |  | Y |  | UE could not reduce the PDCCH monitoring with the SSSG switching | Per UE | N | N | N |  | Optional with capability signaling |
| NR_UE_ pow_sav_enh | 29-3c | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching if PDCCHSkippingDurationList is not configured | 29-3b | Y |  | UE could not reduce the PDCCH monitoring with the SSSG switching | Per UE | N | N | N |  | Optional with capability signaling |
| NR_UE_ pow_sav_enh | 29-3d | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs PDCCH skipping by scheduling DCI and timer based SSSG switching | 29-3a, 29-3b | Y |  | UE could not reduce the PDCCH monitoring by PDCCH skipping and/or SSSG switching | Per band | N | N | N |  | Optional with capability signaling |

# References

- RP-200938, “UE Power Saving Enhancements for NR”, MediaTek

- TS38.331v16.7.0, “Radio Resource Control (RRC) protocol specification”.

- R1-2201516, “Summary on UE features for UE power saving enhancements”, Moderator (NTT DOCOMO)