3GPP TSG RAN WG1 #109-e	                                           R1-2203487

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Discussion on UE Power Saving for XR and Media Services

Agenda Item:	5

Document for:	Discussion and Decision

- Introduction

In SA2 #150e, an LS [1] was sent to RAN1 on UE power saving for XR and media services with the following details:

1. Overall Description: TR38.838 evaluates the impact of power saving schemes on the performance of XR applications and identify potential enhancements under RAN2 responsibility. Specifically, the following power saving features have been identified: - CDRX enhancements - PDCCH monitoring enhancements It understood that these enhancements require assistance information from the network whose specification falls under SA2 responsibility. In 3GPP SA WG2 Meeting #S2-149E, new key issue was agreed for TR 23.700-60 to study potential enhancements for power consumption considering traffic pattern of media services in the scope of FS_XRM. It was agreed as follows: - Which information (e.g., XR/media traffic characteristics, traffic pattern and statistics), if any, is needed by the RAN from the CN and/or the UE to enhance power management (i.e., CDRX)? If needed, how such information is collected? NOTE: This Key issue requires collaboration with RAN WGs. SA2 wants to request RAN1, RAN2 which type of information will be useful for the RAN for power saving enhancements for XR applications. 2. Actions: To RAN WG1, RAN WG2: - ACTION: SA2 kindly asks RAN1, RAN2 to take the above information into account and clarify which type of information should be provided to the RAN for power saving enhancements for XR applications.

In this contribution, we provide our views on detailed RAN required assistance information for power saving enhancements for XR applications.

- Discussion

According to Rel-17 RAN1 XR SI output in [2], XR traffic in DL mainly consists of video traffic, with certain PDB and PER requirements to ensure the user experience. Regarding video traffic, the video frames are encoded by the codec with a given rate. The frame rates include 30 fps, 60 fps, 90 fps, 120 fps and so on. These frame rates determine the periodicity of video traffic arrival, which would affect the configuration of CDRX or PDCCH monitoring. Hence, to appropriate configure CDRX cycle or PDDCH monitoring to match the XR DL video traffic, the frame rate, which can also be called PDU set periodicity, are expected to be provided to RAN for CDRX/PDCCH monitoring configuration for power saving.

Secondly, the size of a DL video frame may vary from time to time. This various frame size (which can also be called PDU set level packets size) would require non-uniform monitoring behavior to ensure the data scheduling and transmission. However, due to the periodical PDCCH monitoring, the various frame size could lead to unnecessary PDCCH monitoring. Therefore, the boundary of a frame, or the indication of the last packet of a PDU set or a frame, are expected to be provided for PDCCH monitoring adaptation, e.g. skipping PDCCH monitoring once the transmissions of the PDU set for a frame are completed.

Thirdly, the jitter may result in waste of XR device power, due to the mismatch of actual traffic arrival time and the PDCCH monitoring. The jitter means the time offset between the expected PDU set arrival time and the actual PDU set arrival time at RAN. Although jitter is random and unpredictable, the statistics characteristics of jitter can be collected, e.g. jitter range. It would be beneficial for the PDCCH monitoring adaptation, e.g. avoiding unnecessary PDCCH monitoring before the actual PDU set arrival.

- Proposal 1: For XR DL applications, the following information is expected to be provided to RAN for power saving enhancements:

- Frame rate (PDU set periodicity)

- Frame boundary, or indication of the last packet indication within a PDU set

- Statistics characteristics of jitter, e.g. jitter range

Unlike DL, UL XR traffic is generated by the XR device itself, e.g. grabing a frame from the camera, rendering a video frame. According to [2], for AR, UL traffic could be different from DL traffic in terms of frame rate, frame size, jitter, etc. For example, jitter in UL tends to be less severe.

Most of the UL XR traffic information can be obtained through UE internal implementation, such that these information can be used by RAN for XR-specific handling. From RAN1 perspective, UL XR traffic frame rate and frame size are useful for power saving enhancement for XR applications. If UL XR traffic frame rate is acquired by network, it would aid gNB to provide accurate and efficient CDRX configuration or PDCCH monitoring adaption indication, which is beneficial for UE power saving. If the UE can report frame size to gNB, it could help gNB to schedule more efficiently, e.g., allocate sufficient frequency domain resources to UE for fast transmission, which is also beneficial for UE power saving. In another example, for UL pose/control transmission, if gNB can acquire frame rate and frame size from UE, it would properly schedule aggregated UL transmission without affecting QoS requirements, which would greatly increase UE power saving performance.

- Proposal 2: For XR UL applications, the following information is expected to be provided to RAN for power saving enhancements:

- Frame rate (or PDU set periodicity)

- Frame size (or PDU set level packets size)

- Conclusion

In this contribution, we provide our views on potential RAN required assistance information for power saving enhancements for XR applications with the following proposals:

- Proposal 1: For XR DL applications, the following information is expected to be provided to RAN for power saving enhancements:

- Frame rate (PDU set periodicity)

- Frame boundary, or indication of the last packet indication within a PDU set

- Statistics characteristics of jitter, e.g. jitter range

- Proposal 2: For XR UL applications, the following information is expected to be provided to RAN for power saving enhancements:

- Frame rate (or PDU set periodicity)

- Frame size (or PDU set level packets size)

# References

- S2-2203418, LS on UE Power Saving for XR and Media Services, SA2 #150e, Apr. 6-12, 2022

- TR 38.838 V17.0.0 (2021-12)