3GPP TSG-RAN WG1 Meeting #109-e     	                        R1-2203492

E-Meeting, May 9th – May 20th, 2022

Title:	[Draft] Reply LS on HARQ process for MCCH and Broadcast MTCH(s)

Response to:	R2-2204017

Release:	Rel-17

Work Item:	NR_MBS-Core

Source:	RAN1

To:	RAN2

Contact Person:

Name:	Qu Xin

E-mail Address:	quxin@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	No

1. Overall Description:

RAN1 would like to thank RAN2 for sending their questions in LS R2-2204017 on HARQ process for MCCH and Broadcast MTCH(s).

Please find RAN1 answers on the question below:

Question: Currently, RAN2 has agreed that there are no dedicated HARQ process IDs for MCCH and Broadcast MTCH with an assumption on single HARQ process for MCCH and single HARQ process for Broadcast MTCH(s). RAN2 would like to confirm this understanding with RAN1.

- Answer: Regarding to HARQ process for MCCH and Broadcast MTCH(s), RAN1 has agreed that HARQ process ID is not indicated in DCI format 4_0 for both MCCH and MTCH. There is also a conclusion that additional HARQ process(es) is(are) not introduced for Rel-17 MBS broadcast reception on serving cell. Additionally, RAN 1 has also agreed that a UE is not required to support reception of FDMed MCCH PDSCH and MTCH PDSCH in PCell as well as a UE is not required to support reception of FDMed multiple MTCH PDSCHs in PCell for RRC RRC_IDLE/INACTIVE UEs. Thus, single HARQ process is enough for MCCH and broadcast MTCH(s) and it is up to UE implementation to perform HARQ combination for the MCCH or broadcast MTCH. RAN 1 confirms RAN2’s assumption on single HARQ process for MCCH and single HARQ process for broadcast MTCH(s) and it is up to UE implementation on whether they share the same HARQ process.

- 2. Actions:

To RAN2

ACTION: 	RAN1 respectfully ask RAN2 to take above information into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #110                               22 August – 26 August 2022

TSG-RAN WG1 Meeting #110-e-Bis              10 October – 19 October 2022 E-Meeting