3GPP TSG RAN WG1 #109-e	R1-2203506

e-Meeting, May 9th – 20th, 2022

Source: 	vivo

Title:	Maintenance on enhancements for multi-TRP deployment

Agenda Item:	8.1.2

Document for:	Discussion and Decision

Introductions

Several issues on multi-TRP operations are discussed with corresponding text proposals in this contribution.

Maintenance on MTRP PDCCH

Clarification on PDCCH repetition configured with SSGS

Based on the current spec of TS 38.213, the determination of PDCCH reception is only based on searchSpaceLinking by RRC configured for two linked search space sets, also for some special case, if one of PDCCH repetition candidate is dropped due to overlap with some high priority channels/RSs, UE always interprets the DCI based on Rel. 17 PDCCH repetition rules.

In RAN1 #107-e, the following conclusion about the combination of PDCCH repetition and search space group switching (SSGS) was achieved.

Conclusion For two linked SS sets are configured with respective searchSpaceGroupIdList, - If both SS sets are monitored in a search space set group, then PDCCH repetition mechanism is applied for the linked SS sets - If only one of the two SS sets is monitored in a search space set group, then the legacy PDCCH mechanism is applied for the monitored one (i.e., no PDCCH repetition)

As per conclusion, whether PDCCH repetition mechanism is applied or not also depends on whether both SS sets are monitored in same search space group even though these two SS sets have been configured with searchSpaceLinking, which doesn’t make sense as this is against the essence of PDCCH repetition behavior in entire Rel-17.

Support the following TP.

Reason for change:

In Rel-16, the procedure of SSGS can be executed with explicit switching based on the indication carried by DCI format 2_0 or implicit switching based on some trigger condition, for instance, once the UE detects any of DCI formats by monitoring PDCCH according to a search space set with group index 0, the UE starts monitoring PDCCH according to search space sets with group index 1 and stops monitoring PDCCH related to group index 0.

However, given UE does not decode format 2_0 successfully in case of explicit switching or dose not decode any DCI format successfully in group index 0 in case of implicit switching, the UE would not perform SSGS while gNB always assumes the procedure of SSGS takes effect, so there is misunderstanding about whether PDCCH repetition mechanism is applied.

Summary of change:

In section 10.4 of 38.213, clarification on configuration restriction is added: ‘If a PDCCH reception includes two PDCCH candidates from two linked search space sets based on searchSpaceLinking, UE expects to be provided with a same configuration for searchSpaceGroupIdList in each of search space set.’

Consequences if not approved:

Determination of PDCCH repetition depending on the condition of UE decoding will cause serious negative impact to communication, since some scheduling timeline or reference candidate rule for channels/RSs of DL and UL is different between legacy PDCCH mechanism and PDCCH repetition mechanism in Rel-17, if misunderstanding is unresolved.

TS 38.213 10.4 Search space set group switching and skipping of PDCCH monitoring A UE can be provided a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList for PDCCH monitoring on a serving cell. If the UE is not provided searchSpaceGroupIdList for a search space set, the following procedures are not applicable for PDCCH monitoring according to the search space set. If a PDCCH reception includes two PDCCH candidates from two linked search space sets based on searchSpaceLinking, UE expects to be provided with a same configuration for searchSpaceGroupIdList in each of search space set. If a UE is provided cellGroupsForSwitchList, indicating one or more groups of serving cells, the following procedures apply to all serving cells within each group; otherwise, the following procedures apply only to a serving cell for which the UE is provided searchSpaceGroupIdList. When a UE is provided searchSpaceGroupIdList, the UE resets PDCCH monitoring according to search space sets with group index 0, if provided by searchSpaceGroupIdList. < Unchanged text omitted >

Maintenance on MTRP PUSCH

PT-RS related issues

Agreement For the indication of PTRS-DMRS association for maxRank > 2 in mTRP PUSCH repetition type B, select Option 1 - Option 1 (4 bits): with a second PTRS-DMRS association field (similar to the existing field), and each field separately indicating the association between PTRS port and DMRS port for two TRPs.

Support the following TP.

Reason for change:

Based on the above agreement, the second PTRS-DMRS association field is introduced to indicate PTRS-DMRS association for maxRank > 2 in mTRP PUSCH. When the second PTRS-DMRS association field is configured, the association between UL PT-RS port(s) and DM-RS port(s) for PUSCH associated to two SRS resource sets should be signaled by the first and the second PTRS-DMRS association field in DCI format 0_1 and DCI format 0_2 respectively. This situation seems to be not included in current version.

Summary of change:

Add ‘s’ for the PTRS-DMRS association field.

Consequences if not approved:

UE’s behaviour on the association between UL PT-RS port(s) and DM-RS port(s) for the PUSCH corresponding to the second SRS resource set will be unclear.

TS 38.214 6.2.3.1 UE PT-RS transmission procedure when transform precoding is not enabled When transform precoding is not enabled and if a UE is configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig, - the higher layer parameters timeDensity and frequencyDensity in PTRS-UplinkConfig indicate the threshold values ptrs-MCSi, i=1,2,3 and NRB,i , i=0,1, as shown in Table 6.2.3.1-1 and Table 6.2.3.1-2, respectively. - if either or both higher layer parameters timeDensity and/or frequencyDensity in PTRS-UplinkConfig are configured, the UE shall assume the PT-RS antenna ports' presence and pattern are a function of the corresponding scheduled MCS and scheduled bandwidth in a corresponding bandwidth part as shown in Table 6.2.3.1-1 and Table 6.2.3.1-2, respectively, - if the higher layer parameter timeDensity is not configured, the UE shall assume LPT-RS = 1. - if the higher layer parameter frequencyDensity is not configured, the UE shall assume KPT-RS = 2. - if none of the higher layer parameters timeDensity and frequencyDensity in PTRS-UplinkConfig are configured, the UE shall assume LPT-RS = 1 and KPT-RS = 2. Table 6.2.3.1-1: Time density of PT-RS as a function of scheduled MCS Scheduled MCS; Time density(). IMCS < ptrs-MCS1; PT-RS is not present. ptrs-MCS1 IMCS < ptrs-MCS2; 4. ptrs-MCS2 IMCS < ptrs-MCS3; 2. ptrs-MCS3 IMCS < ptrs-MCS4; 1. Table 6.2.3.1-2: Frequency density of PT-RS as a function of scheduled bandwidth Scheduled bandwidth; Frequency density (). NRB < NRB0; PT-RS is not present. NRB0 NRB < NRB1; 2. NRB1 NRB; 4. The higher layer parameter PTRS-UplinkConfig provides the parameters ptrs-MCSi, i=1,2,3 and with values in 0-29 when MCS Table 5.1.3.1-1 or Table 5.1.3.1-3 is used and 0-28 when MCS Table 5.1.3.1-2 is used, respectively. ptrs-MCS4 is not explicitly configured by higher layers but assumed 29 when MCS Table 5.1.3.1-1 or Table 5.1.3.1-3 is used and 28 when MCS Table 5.1.3.1-2 is used. The higher layer parameter PTRS-UplinkConfig provides the parameters NRBi i=0,1 with values in range 1-276. If the higher layer parameter PTRS-UplinkConfig indicates that the time density thresholds ptrs-MCSi = ptrs-MCSi+1, then the time density LPTRS of the associated row where both these thresholds appear in Table 6.2.3.1-1 is disabled. If the higher layer parameter frequencyDensity in PTRS-UplinkConfig indicates that the frequency density thresholds NRB,i = NRB,i+1, then the frequency density KPTRS of the associated row where both these thresholds appear in Table 6.2.3.1-2 is disabled. If either or both of the parameters PT-RS time density (LPT-RS) and PT-RS frequency density (KPT-RS), shown in Table 6.2.3.1-1 and Table 6.2.3.1-2, indicates that are configured as 'PT-RS not present', the UE shall assume that PT-RS is not present. When a UE is scheduled to transmit PUSCH with allocation duration of 2 symbols or less, and if LPT-RS is set to 2 or 4, the UE shall not transmit PT-RS. When a UE is scheduled to transmit PUSCH with allocation duration of 4 symbols or less, and if LPT-RS is set to 4, the UE shall not transmit PT-RS. When a UE is scheduled to transmit PUSCH for retransmission, if the UE is scheduled with IMCS > V, where V = 28 for MCS Table 5.1.3.1-1 and MCS Table 5.1.3.1-3 and V = 27 for MCS Table 5.1.3.1-2, respectively, the MCS for PT-RS time-density determination is obtained from the DCI for the same transport block in the initial transmission, which is smaller than or equal to V. The maximum number of configured PT-RS ports is given by the higher layer parameter maxNrofPorts in PTRS-UplinkConfig. The UE is not expected to be configured with a larger number of UL PT-RS ports than it has reported need for. If a UE has reported the capability of supporting full-coherent UL transmission, the UE shall expect the number of UL PT-RS ports to be configured as one if UL-PTRS is configured. For codebook or non-codebook based UL transmission, the association between UL PT-RS port(s) and DM-RS port(s) is signalled by PTRS-DMRS association field(s) in DCI format 0_1 and DCI format 0_2. For a PUSCH corresponding to a configured grant Type 1 transmission, the UE may assume the association between UL PT-RS port(s) and DM-RS port(s) defined by value 0 in Table 7.3.1.1.2-25 or value "00" in Table 7.3.1.1.1.2-26 described in Clause 7.3.1 of [5, TS38.212]. For PUSCH scheduled by DCI format 0_0 or by activation DCI format 0_0, the UL PT-RS port is associated to DM-RS port 0. For non-codebook based UL transmission, the actual number of UL PT-RS port(s) to transmit is determined based on SRI(s) in DCI format 0_1 and DCI format 0_2 or higher layer parameter sri-ResourceIndicator in rrc-ConfiguredUplinkGrant. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'noncodebook', the actual number of UL PT-RS port(s) to transmit corresponding to each SRS resource set is determined based on SRI(s) corresponding to the associated SRS resource set or higher layer parameter sri-ResourceIndicator or sri-ResourceIndicator2 corresponding to the associated SRS resource set in rrc-ConfiguredUplinkGrant. A UE is configured with the PT-RS port index for each configured SRS resource by the higher layer parameter ptrs-PortIndex configured by SRS-Config if the UE is configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig. If the PT-RS port index associated with different SRIs are the same, the corresponding UL DM-RS ports are associated to the one UL PT-RS port.

Support the following TP.

Reason for change:

Power boosting factor of PT-RS is determined by the number of scheduled PT-RS ports, the rank and coherence of the indicated precoder. For MTRP PUSCH, power scaling factor of PT-RS for PUSCH associated two SRS resource sets should be determined separately. The actual number of scheduled PT-RS ports can be different, which is already captured in current spec. The number of layers scheduled for PUSCH associated two SRS resource sets is same, which is also constrained by indicated two TPMIs/SRIs. However, the coherence properties of the two indicated TPMIs may be different. For instance, one TPMI may indicate a full-coherent precoder while the second TPMI may indicate a non-coherent precoder when full-partial-non codebook subset is configured. So, the scaling factors for PT-RS transmitted on PUSCH repetitions towards 2TRPs should be determined according to the first and second TPMI indication respectively. Current spec does not capture this case.

Summary of change:

Description on power boosting factor determination in mTRP transmission is added.

Consequences if not approved:

UE’s behavior on determining power boosting factor of the PTRS transmitted on PUSCH associated with the second SRS resource is unclear. Or, the power boosting factor will be determined by mistake because it is also determined based on the first TPMI field as described in current version.

TS 38.214 6.2.3.1 UE PT-RS transmission procedure when transform precoding is not enabled < Unchanged text omitted > For non-codebook based UL transmission, the actual number of UL PT-RS port(s) to transmit is determined based on SRI(s) in DCI format 0_1 and DCI format 0_2 or higher layer parameter sri-ResourceIndicator in rrc-ConfiguredUplinkGrant. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'noncodebook', the actual number of UL PT-RS port(s) to transmit corresponding to each SRS resource set is determined based on SRI(s) corresponding to the associated SRS resource set or higher layer parameter sri-ResourceIndicator or sri-ResourceIndicator2 corresponding to the associated SRS resource set in rrc-ConfiguredUplinkGrant. A UE is configured with the PT-RS port index for each configured SRS resource by the higher layer parameter ptrs-PortIndex configured by SRS-Config if the UE is configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig. If the PT-RS port index associated with different SRIs are the same, the corresponding UL DM-RS ports are associated to the one UL PT-RS port. For partial-coherent and non-coherent codebook-based UL transmission, the actual number of UL PT-RS port(s) is determined based on TPMI(s) and/or number of layers which are indicated by 'Precoding information and number of layers' field(s) in DCI format 0_1 and DCI format 0_2 or configured by higher layer parameter precodingAndNumberOfLayers: - if the UE is configured with the higher layer parameter maxNrofPorts in PTRS-UplinkConfig set to 'n2', the actual UL PT-RS port(s) and the associated transmission layer(s) are derived from indicated TPMI(s) as: - PUSCH antenna port 1000 and 1002 in indicated TPMI(s) share PT-RS port 0, and PUSCH antenna port 1001 and 1003 in indicated TPMI(s) share PT-RS port 1. - UL PT-RS port 0 is associated with the UL layer 'x' of layers which are transmitted with PUSCH antenna port 1000 and PUSCH antenna port 1002 in indicated TPMI(s), and UL PT-RS port 1 is associated with the UL layer 'y' of layers which are transmitted with PUSCH antenna port 1001 and PUSCH antenna port 1003 in indicated TPMI(s), where 'x' and/or 'y' are given by DCI parameter 'PTRS-DMRS association' as shown in DCI format 0_1 and DCI format 0_2 described in Clause 7.3.1 of [5, TS38.212]. When the UE is scheduled with Qp={1,2} PT-RS port(s) in uplink and the number of scheduled layers is , - If the UE is configured with higher layer parameter ptrs-Power, the PUSCH to PT-RS power ratio per layer per RE is given by , where is shown in the Table 6.2.3.1-3 according to the higher layer parameter ptrs-Power, the PT-RS scaling factor specified in clause 6.4.1.2.2.1 of [4, TS 38.211] is given by and also on the 'Precoding Information and Number of Layers' field in DCI. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', the PUSCH corresponding to each SRS resource set to PT-RS power ratio per layer per RE is given by , where is shown in the Table 6.2.3.1-3 according to the higher layer parameter ptrs-Power, the PT-RS scaling factor specified in clause 6.4.1.2.2.1 of [4, TS 38.211] is given by and also on the 'Precoding Information and Number of Layers' field or 'Second Precoding Information and Number of Layers' field in DCI. - The UE shall assume ptrs-Power in PTRS-UplinkConfig is set to state "00" in Table 6.2.3.1-3 if not configured or in case of non-codebook based PUSCH. Table 6.2.3.1-3: Factor related to PUSCH to PT-RS power ratio per layer per RE UL-PTRS-power /; The number of PUSCH layers ( ). 1; 2; 3; 4. All cases; Full coherent; Partial and non- coherent and non-codebook based; Full coherent; Partial and non- coherent and non-codebook based; Full coherent; Partial coherent; Non-coherent and non-codebook based. 00; 0; 3; 3Qp-3; 4.77; 3Qp-3; 6; 3Qp; 3Qp-3. 01; 0; 3; 3; 4.77; 4.77; 6; 6; 6. 10; Reserved. 11; Reserved. < Unchanged text omitted >

Power control

Agreement For single-DCI based M-TRP PUSCH repetition schemes, when one SRS resource per SRS resource set is configured (i.e., when two SRI fields are absent in DCI formats 0_1 / 0_2), per TRP default P0, alpha, PL-RS, and closed loop index is defined by, - If the UE is provided enablePL-RS-UpdateForPUSCH-SRS, the first set of values {the first value in P0-AlphaSet, the PL-RS corresponding to the first sri-PUSCH-PowerControl associated with the first SRS resource set and closed-loop index l = 0} is used for TRP1, and the second set of values {the second value in P0-AlphaSet, the PL-RS corresponding to the first sri-PUSCH-PowerControl associated with the second SRS resource set and closed-loop index l = 1 if twoPUSCH-PC-AdjustmentStates is configured, l=0 otherwise} is used for TRP2. - Otherwise, the first set of values {the first value in P0-AlphaSet, the PL-RS with PUSCH-PathlossReferenceRS-Id=0 and closed-loop index l = 0} can be used for TRP1, and the second set of values {the second value in P0-AlphaSet, the PL-RS with PUSCH-PathlossReferenceRS-Id = 1 and closed-loop index l = 1 if twoPUSCH-PC-AdjustmentStates is configured, l=0 otherwise } can be used for TRP2. - Note: How to design the signaling link sri-PUSCH-PowerControl with two SRS resource sets is up to RAN2.

Support the following TP.

Reason for change:

In RAN1#106-e, two sets of default power control parameter are defined for PUSCH transmission towards two TRPs. When PUSCH transmission towards the first or the second TRP is indicated by the SRS resource set indication field, the first or second set of default power control parameter is utilized accordingly. However, in current version of spec, two sets of power control parameter are described only when two TRP operation is indicated. So, we propose to further clarify default power control parameter acquisition for single TRP scheduling case.

Summary of change:

Additional illustration on P0/alpha determination when SRS resource set indicator field is ‘00’ and ‘01’ is added.

Consequences if not approved:

If not explicitly captured in spec, one set of parameters determined as Rel-15/16 is shared for the first and second TRP when only the first or second TRP is scheduled which is unreasonable.

7.1.1 UE behaviour If a UE transmits a PUSCH on active UL BWP b of carrier f of serving cell c using parameter set configuration with index j and PUSCH power control adjustment state with index l, the UE determines the PUSCH transmission power PPUSCH,b,f,c(i,j,qd,l) in PUSCH transmission occasion i as [dBm] where, - PCMAX,f,c(i)is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier f of serving cell c in PUSCH transmission occasion i. - PO_PUSCH,b,f,c(j) is a parameter composed of the sum of a component PO_NOMINAL,PUSCH,f,c(j) and a component PO_UE_PUSCH,b,f,c(j) where j∈0,1,…,J-1. - If a UE established dedicated RRC connection using a Type-1 random access procedure, as described in clause 8, and is not provided P0-PUSCH-AlphaSet or for a PUSCH (re)transmission corresponding to a RAR UL grant as described in clause 8.3, j=0, PO_UE_PUSCH,b,f,c0=0, and PO_NOMINAL,PUSCH,f,c0=PO_PRE+∆PREAMBLE,Msg3, where PO_PRE is provided by preambleReceivedTargetPower [11, TS 38.321] and ΔPREAMBLE_Msg3 is provided by msg3-DeltaPreamble, or ∆PREAMBLE,Msg3=0 dB if msg3-DeltaPreamble is not provided, for carrier f of serving cell c - If a UE established dedicated RRC connection using a Type-2 random access procedure, as described in clause 8, and is not provided P0-PUSCH-AlphaSet, or for a PUSCH transmission for Type-2 random access procedure as described in clause 8.1A, j=0, PO_UE_PUSCH,b,f,c(0)=0, and PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH, where PO_PRE is provided by msgA-preambleReceivedTargetPower, or by preambleReceivedTargetPower if msgA-preambleReceivedTargetPower is not provided and ΔMsgA_PUSCH is provided by msgA-DeltaPreamble, or ΔMsgA_PUSCH=ΔPREAMBLE_Msg3 dB if msgA-DeltaPreamble is not provided, for carrier f of serving cell c - For a PUSCH (re)transmission configured by ConfiguredGrantConfig, j=1, PO_NOMINAL,PUSCH,f,c1 is provided by p0-NominalWithoutGrant, or PO_NOMINAL,PUSCH,f,c1=PO_NOMINAL,PUSCH,f,c0 if p0-NominalWithoutGrant is not provided. - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', and is provided p0-PUSCH-Alpha2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format that includes a SRS resource set indicator field, and for active UL BWP b of carrier f of serving cell - If the SRS resource set indicator value is 00, first PO_UE_PUSCH,b,f,c1 value is provided by the value of p0-PUSCH-Alpha in ConfiguredGrantConfig. - If the SRS resource set indicator value is 01, second PO_UE_PUSCH,b,f,c1 value is provided by the value of p0-PUSCH-Alpha2 in ConfiguredGrantConfig. - If the SRS resource set indicator value is 10 or 11, first and second PO_UE_PUSCH,b,f,c1 values are respectively provided by the values of p0-PUSCH-Alpha and by p0-PUSCH-Alpha2 in ConfiguredGrantConfig. - else, PO_NOMINAL,PUSCH,f,c1 is provided by p0 obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig that provides an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet, or by p0-PUSCH for a PUSCH (re)transmission as described in clause 19.1, for active UL BWP b of carrier f of serving cell c - For j∈2,…,J-1=SJ, a PO_NOMINAL,PUSCH,f,cj value, applicable for all j∈SJ, is provided by p0-NominalWithGrant, or PO_NOMINAL,PUSCH,f,cj=PO_NOMINAL,PUSCH,f,c0 if p0-NominalWithGrant is not provided, for each carrier f of serving cell c and a set of PO_UE_PUSCH,b,f,cj values are provided by a set of p0 in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP b of carrier f of serving cell c - If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId and if a DCI format scheduling the PUSCH transmission includes an SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in the DCI format [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values and determines the value of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value. If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId - if the DCI format scheduling the PUSCH transmission includes two SRI fields and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' and if, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the two SRI fields and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines first and second values of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values that are mapped to the values of the first and second SRI fields, respectively. - if the DCI format scheduling the PUSCH transmission includes two SRI fields and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between - a set of values for the first SRI field value and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines the first value of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the first SRI field value, and - a set of values associated with the second SRI field value for a same number of layers as indicated by the first SRI field [5, TS 38.212], and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines the second value of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212]. - If the DCI format also includes an open-loop power control parameter set indication field and a value of the open-loop power control parameter set indication field is '1' and if the DCI format scheduling the PUSCH transmission includes an SRI field, the UE determines a value of PO_UE_PUSCH,b,f,cj from a first value in P0-PUSCH-Set with a p0-PUSCH-SetId value mapped to the SRI field value. - If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId - if a DCI format scheduling the PUSCH transmission includes two SRI fields and an open-loop power control parameter set indication field and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' - if a value of the open-loop power control parameter set indication field is '0', the UE determines two values of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values in SRI-PUSCH-PowerControl that are mapped to the two SRI values corresponding to each SRS resource set with usage set to 'codebook'. - if a value of the open-loop power control parameter set indication field is '1', the UE determines two values of PO_UE_PUSCH,b,f,cj from first values in P0-PUSCH-Set in P0-PUSCH-SetList and P0-PUSCH-Set in P0-PUSCH-SetList2 with p0-PUSCH-SetId values mapped to the two SRI values corresponding to each SRS resource set with usage set to 'codebook', respectively. - if a DCI format scheduling the PUSCH transmission includes two SRI fields and an open-loop power control parameter set indication field and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', - if a value of the open-loop power control parameter set indication field is '0', the UE determines two values of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values in SRI-PUSCH-PowerControl that are mapped to the first SRI field value corresponding to the first SRS resource set with usage set to 'nonCodebook' and to a second value, that is associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, corresponding to the second SRS resource set with usage set to 'nonCodebook'. - if a value of the open-loop power control parameter set indication field is '1', the UE determines two values of PO_UE_PUSCH,b,f,cj from first values in P0-PUSCH-Set in P0-PUSCH-SetList and P0-PUSCH-Set in P0-PUSCH-SetList2 with p0-PUSCH-SetId values mapped to the first SRI field value corresponding to the first SRS resource set with usage set to 'nonCodebook, and a second value, that is associated with the second SRS field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, corresponding to the second SRS resource set with usage set to 'nonCodebook', respectively. - If the UE is not provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and if the PUSCH transmission, except for the PUSCH retransmission corresponding to a RAR UL grant, is scheduled by a DCI format that does not include an SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, j=2, - If P0-PUSCH-Set is provided to the UE and the DCI format includes an open-loop power control parameter set indication field, the UE determines a value of PO_UE_PUSCH,b,f,cj from - a first P0-PUSCH-AlphaSet in p0-AlphaSets if a value of the open-loop power control parameter set indication field is '0' or '00' - a first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value if a value of the open-loop power control parameter set indication field is '1' or '01' - a second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value if a value of the open-loop power control parameter set indication field is '10' - else, the UE determines PO_UE_PUSCH,b,f,cj from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and the PUSCH transmission is scheduled by a DCI format that does not include an SRI field and includes an SRS resource set indicator field with value 10 or 11 - If P0-PUSCH-Set is provided to the UE and the DCI format includes an open-loop power control parameter set indication field, the UE determines first and second values of PO_UE_PUSCH,b,f,cj as - first and second P0-PUSCH-AlphaSet in p0-AlphaSets if the open-loop power control parameter set indication value is '0' or '00' - first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value in p0-PUSCH-SetList and first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value in p0-PUSCH-SetList2, respectively, if the open-loop power control parameter set indication value is '1' or '01' - second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value in p0-PUSCH-SetList and second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID in p0-PUSCH-SetList2, respectively, if the open-loop power control parameter set indication value is '10' or '11' - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and the PUSCH transmission is scheduled by a DCI format that does not include an SRI field and includes an SRS resource set indicator field with value 00 - If P0-PUSCH-Set is provided to the UE and the DCI format includes an open-loop power control parameter set indication field, the UE determines first and second values of PO_UE_PUSCH,b,f,cj as - first P0-PUSCH-AlphaSet in p0-AlphaSets if the open-loop power control parameter set indication value is '0' or '00' - first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value in p0-PUSCH-SetList, if the open-loop power control parameter set indication value is '1' or '01' - second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value in p0-PUSCH-SetList, if the open-loop power control parameter set indication value is '10' or '11' - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and the PUSCH transmission is scheduled by a DCI format that does not include an SRI field and includes an SRS resource set indicator field with value 01 - If P0-PUSCH-Set is provided to the UE and the DCI format includes an open-loop power control parameter set indication field, the UE determines first and second values of PO_UE_PUSCH,b,f,cj as - second P0-PUSCH-AlphaSet in p0-AlphaSets if the open-loop power control parameter set indication value is '0' or '00' - first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value in p0-PUSCH-SetList2, if the open-loop power control parameter set indication value is '1' or '01' - second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID in p0-PUSCH-SetList2, if the open-loop power control parameter set indication value is '10' or '11' - else, the UE determines first and second values PO_UE_PUSCH,b,f,cj from the value of the first and second P0-PUSCH-AlphaSet in p0-AlphaSets, respectively - For αb,f,cj - For j=0, - if PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH and msgA-Alpha is provided, αb,f,c(0) is the value of msgA-Alpha - elseif PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔPREAMBLE_Msg3 or msgA-Alpha is not provided, and msg3-Alpha is provided, αb,f,c(0) is the value of msg3-Alpha - else, αb,f,c0=1 - For j=1, - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', and is provided p0-PUSCH-Alpha2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format that includes an SRS resource set indicator field, and for active UL BWP b of carrier f of serving cell - If the SRS resource set indicator value is 00, first αb,f,c(1) value is provided by p0-PUSCH-Alpha in ConfiguredGrantConfig. - If the SRS resource set indicator value is 01, first αb,f,c(1) value is provided by p0-PUSCH-Alpha2 in ConfiguredGrantConfig. - If the SRS resource set indicator value is 10 or 11, first and second αb,f,c(1) values are respectively provided by p0-PUSCH-Alpha and p0-PUSCH-Alpha2 in ConfiguredGrantConfig. - else αb,f,c(1) is provided by alpha obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig providing an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet, or by alpha for a PUSCH (re)transmission as described in clause 19.1, for active UL BWP b of carrier f of serving cell c - For j∈SJ, a set of αb,f,cj values are provided by a set of alpha in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP b of carrier f of serving cell c - If the UE is provided SRI-PUSCH-PowerControl and more than one values of p0-PUSCH-AlphaSetId in p0-AlphaSets, - if a DCI format scheduling the PUSCH transmission includes two SRI fields and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for - the two SRI fields and a set of indexes provided by P0-PUSCH-AlphaSetId that map to P0-PUSCH-AlphaSet values, and determines first and second values of αb,f,cj from the P0-PUSCH-AlphaSetID values that are mapped to the values of the first and second SRI field values, respectively. - if a DCI format scheduling the PUSCH transmission includes two SRI fields and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for - the first SRI field and a set of indexes provided by P0-PUSCH-AlphaSetId that map to P0-PUSCH-AlphaSet values, and determines first value of αb,f,cj from the P0-PUSCH-AlphaSetID value that is mapped to the first SRI field value, and - the second value, associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines the second value of αb,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the second SRI field value - if a DCI format scheduling the PUSCH transmission includes one SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in the DCI format [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values and determines the values of αb,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value - If the UE is not provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and if the PUSCH transmission except for the PUSCH retransmission corresponding to a RAR UL grant is scheduled by a DCI format that does not include an SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, j=2, and the UE determines αb,f,cj from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and the PUSCH transmission is scheduled by a DCI format that does not include an SRI field and includes an SRS resource set indicator field with value 10 or 11, the UE determines αb,f,cj from first and second P0-PUSCH-AlphaSet in p0-AlphaSets - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and the PUSCH transmission is scheduled by a DCI format that does not include an SRI field and includes an SRS resource set indicator field with value 00 or 01, the UE determines αb,f,cj from first P0-PUSCH-AlphaSet or second P0-PUSCH-AlphaSet in p0-AlphaSets, respectively.

Maintenance on inter-cell mTRP operation

In RAN1#104-e, following agreement was reached on rate matching, which means PDSCH from serving cell PCI is rate matched the SS/PBCH block from serving cell PCI and PDSCH from serving cell with additional PCI is rate matched around the SS/PBCH block from serving cell associated with additional PCI.

Agreement Agree on scheme1 - Scheme1: PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI

However current version of 38.214 [4] doesn’t capture this agreement, hence a text proposal is made for 38.214 section 5.1.4 as below.

Support the following TP.

Reason for change:

It was agreed that PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI, current spec doesn’t reflect the agreement.

Summary of change:

In section 5.1.4, “associated with the same PCI” is added behind SS/PBCH block

Consequences if not approved:

PDSCH rate matching is incorrect for intercell multi-TRP operation.

5.1.4 PDSCH resource mapping When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH. When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MSGB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, and the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block associated with the same PCI is transmitted. A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH. For operation with shared spectrum channel access, SS/PBCH block transmission according to ssb-PositionsInBurst represents all of the candidate SS/PBCH blocks corresponding to SS/PBCH block indices provided by ssb-PositionsInBurst as described in Clause 4.1 of [6, TS 38.213].

Maintenance on MTRP multi-beam enhancement

Beam report for MTRP

Default beam of aperiodic CSI-RS

In Rel-16, the default beam of aperiodic CSI-RS in single-DCI-based MTRP and multi-DCI-based MTRP scenarios had been specified. For multi-DCI-based MTRP, if the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet is smaller than the UE reported threshold beamSwitchTiming/beamSwitchTiming-r16, the UE applies the QCL assumption of the other DL signals satisfying following conditions to receive the aperiodic CSI-RS:

overlap fully in symbols with the aperiodic CSI-RS

associate with the same CORESETPoolindex as the aperiodic CSI-RS

If there is no other DL signal, UE will apply the QCL parameter(s) of the CORSET associated with a monitored search space with the lowest controlResourceSetId among CORESETs, which are configured with the same value of coresetPoolIndex as the PDCCH triggering that aperiodic CSI-RS. However, due to the agreed two CSI-RS resource sets for beam measurement and report in MTRP scenarios, it means for aperiodic CSI-RS resource, a DCI would trigger two resource sets. In this case, directly reusing the default beam mechanism defined in Rel-16 may be non-workable, since both sets would be associated with the same CORESETPoolindex. Therefore, the default beam of aperiodic CSI-RS for beam management in multi-DCI-based MTRP scenarios needs to be clarified. In our view, defining the default beam per TRP is the most straightforward way.

- Default beam of aperiodic CSI-RS resource should be defined per resource set, when a DCI triggers two aperiodic CSI-RS resource sets.

CRI report

In the RAN1#108-e meeting [1], we discussed whether the UE should be mandatory configured with the same value(s) of repetition in different CSI-RS resource sets and made an agreement as follows.

Agreement When the higher layer parameter groupBasedBeamReporting-r17 in CSI -ReportConfig is configured, the UE can be configured with the same or different value(s) of repetition in different CSI Resource Sets. - FFS: UE reports CRI regardless of whether repetition is set to ‘on’ or not - Whether/how to capture the above is up to editor.

Table 5.1.2-1 UCI format when both sets are configured with repetition “on”

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | Resource set indicator |
|  | RSRP of CRI or SSBRI #1 of 1st resource group |
|  | Differential RSRP of CRI or SSBRI #2 of 1st resource group |
|  | Differential RSRP of CRI or SSBRI #1 of 2nd resource group |
|  | Differential RSRP of CRI or SSBRI #2 of 2nd resource group |

Table 5.1.2-2 UCI format when one set is configured with repetition “on”

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | Resource set indicator |
|  | CRI or SSBRI #1 of 1st resource group |
|  | CRI or SSBRI #1 of 2nd resource group |
|  | RSRP of CRI or SSBRI #1 of 1st resource group |
|  | Differential RSRP of CRI or SSBRI #2 of 1st resource group |
|  | Differential RSRP of CRI or SSBRI #1 of 2nd resource group |
|  | Differential RSRP of CRI or SSBRI #2 of 2nd resource group |

For the remaining issue of the CRI report, we think following the legacy rule is enough, which means if a resource set is configured with repetition “on”, the corresponding CRI(s) would not be reported. To be specific, for both resource sets are configured with repetition “on”, only N(N=2) pairs of L1-RSRP measurement results shall be reported, and the corresponding UCI format is shown in Table 5.1.2-1. And the resource set indicator introduced in Rel-17 can still use to indicate the correspondence between the L1-RSRP measurement results and resource sets. For only one resource set is configured with repetition “on”, N CRIs and N pairs of L1-RSRP measurement results shall be reported, and the corresponding UCI format is shown in Table 5.1.2-2. According to the resource set indicator, the network will be aware of the correspondence between the L1-RSRP measurement results and the CRIs. Therefore, the first two rows of the CRIs/SSBRIs in the current UCI format captured in TS38.212[2] should be added with “if reported”.

Support the following TP.

Reason for change:

For mTRP beam report, to reduce UCI payload, the CRI(s) are not needed to report when the corresponding resource set is configured with repetition “on”. However, in the current spec version, at least two CRIS/SSBRIs are reported in any case.

Summary of change:

In TS38.212 section 6.3.1.1.2, add “if reported” in the first two rows of the CRIs/SSBRIs.

Consequences if not approved:

More CRIs will be reported but meaningless, and low resource efficiency.

TS38.212 6.3.1.1.2 CSI only Table 6.3.1.1.2-8B: Mapping order of CSI fields of one report for group-based CRI/RSRP or SSBRI/RSRP reporting CSI report number; CSI fields. CSI report #n; Resource set indicator. CRI or SSBRI #1 of 1st resource group as in Table 6.3.1.1.2-6, if reported. CRI or SSBRI #2 of 1st resource group as in Table 6.3.1.1.2-6, if reported. CRI or SSBRI #1 of 2nd resource group as in Table 6.3.1.1.2-6, if reported. CRI or SSBRI #2 of 2nd resource group as in Table 6.3.1.1.2-6, if reported. CRI or SSBRI #1 of 3rd resource group as in Table 6.3.1.1.2-6, if reported. CRI or SSBRI #2 of 3rd resource group as in Table 6.3.1.1.2-6, if reported. CRI or SSBRI #1 of 4th resource group as in Table 6.3.1.1.2-6, if reported. CRI or SSBRI #2 of 4th resource group as in Table 6.3.1.1.2-6, if reported. RSRP of CRI or SSBRI #1 of 1st resource group as in Table 6.3.1.1.2-6. Differential RSRP of CRI or SSBRI #2 of 1st resource group as in Table 6.3.1.1.2-6. Differential RSRP of CRI or SSBRI #1 of 2nd resource group as in Table 6.3.1.1.2-6, if reported. Differential RSRP of CRI or SSBRI #2 of 2nd resource group as in Table 6.3.1.1.2-6, if reported. Differential RSRP of CRI or SSBRI #1 of 3rd resource group as in Table 6.3.1.1.2-6, if reported. Differential RSRP of CRI or SSBRI #2 of 3rd resource group as in Table 6.3.1.1.2-6, if reported. Differential RSRP of CRI or SSBRI #1 of 4th resource group as in Table 6.3.1.1.2-6, if reported. Differential RSRP of CRI or SSBRI #2 of 4th resource group as in Table 6.3.1.1.2-6, if reported.

Beam failure recovery for intra-cell and inter-cell MTRP

Optionality of NBI-RS

For intra-cell scenarios, both multi-DCI-based MTRP and single-DCI-based MTRP are considered for beam failure recovery enhancement. And the procedure of TRP-specific BFR on multi-DCI-based MTRP had been almost complete with the effort of the previous meetings, while for single-DCI-based MTRP scenarios, other than the explicit configuration of two BFD-RS sets, no valid agreement had been reached. We suggest simplifying the procedure of TRP-specific BFR in single-DCI-based MTRP scenarios. The specification can be designed that UE only reports the beam failure event and does not need to reset beam for the failed TRP and thus reuses the procedure defined for multi-DCI-based MTRP BFR procedure as much as possible. In this case,  the configuration of TRP-specific NBI-RS set(s) for TRP-specific BFR is not a necessity, especially for single-DCI based MTRP. For example, if one TRP fails and its corresponding TRP-specific NBI-RS set is not configured, an aperiodic beam report can be triggered by the TRP in a good radio link to obtain one or more new beams to recover the failed TRP. Compared with periodic measurement of NBI-RS resources, aperiodic beam measurement consumes less resources, which is beneficial for the network to schedule various services of users within the limited UE capability flexibly.

For inter-cell MTRP case, it is also unnecessary to configure TRP-specific NBI-RS set explicitly. When receiving a TCI state for PDCCH, SSB(s) indicated in SSB-MTC-AdditionalPCI-r17 and associated with the same PCI as the indicated TCI state, can be regarded as NBI-RS and used to identify new beam(s).

Send LS to RAN on RAN1 conclusion on optionally configure TRP-specific NBI-RS, rather than always configured.

BFR for inter-cell multi-TRP cases

For inter-cell mTRP scenarios, some agreement on the configuration of BFD-RS set and NBI-RS set was reached in the last meeting as follows.

Agreement For multi-DCI based MTRP inter-cell, if Rel-17 per-TRP BFR is configured, SSB associated with an additional PCI can be configured as NBI-RS in one NBI -RS set, and the NBI-RS set is associated with the BFD-RS set associated with the additional PCI. Agreement For Question 3.3 from RAN2, RAN1 response is as follows: Question 3.3: When a serving cell use inter-cell mTRP, can the UE be configured with two BFD RS sets? If yes, please explain if there is any relation between a BFD RS set and a PCI (e.g. one set associated with RS of this serving cell and another associated with RS associated with an additional PCI). Answer: Yes, when a serving cell is configured with inter-cell mTRP, the UE can also be configured with two BFD RS sets, each associated with one different PCI.

Based on the agreement, it should be clarified in the specification how the BFD-RS/NBI-RS set is associated with the PCI.

For inter-cell TRP-specific BFD-RS configuration and update, support the following TP.

Reason for change:

In current spec, only the association between BFD-RS/NBI-RS set and PCI has been captured and the associated PCI is configured by RRC. Therefore, the PCI configured to be associated with the BFD-RS set/NBI-RS set may be different from the PCI involved in the indicated TCI state.

Summary of change:

In TS38.213 section 6, clarify the association between BFD-RS/NBI-RS set, CORESETPoolindex and PCI when two BFD-RS/NBI-RS set are configured and one BFD-RS/NBI-RS set associates with a PCI different from that of the serving cell.

Consequences if not approved:

It can cause misalignment between the PCI associated with the BFD-RS/NBI-RS set and PCI involved in the indicated TCI state, which may lead to the new beam(s) found by UE cannot use to recover the failed link.

TS38.213 6 Link recovery procedures < Unchanged parts are omitted > If a UE - is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs on an active DL BWP of a serving cell, - is provided coresetPoolIndex with a value of 1 for second CORESETs on the active DL BWP of the serving cells, and - is provided SSB-MTCAdditionalPCI SS/PBCH block indexes other than the one provided by physCellId in ServingCellConfigCommon can be associated with either q1,0 or q1,1 set and the corresponding q0,0 or q0,1 set is associated with the physical cell identity. In this case, the physical cell ID associated with the SS/PBCH block indexes is the same as the physical cell ID associated with the indicated TCI state for the second CORESETs.

- For inter-cell MTRP case, the following case also needs clarification.

when BFR of both failed TRP is triggered in SpCell, no new beam for the TRP associated with serving cell PCI and new beam(s) for another failed TRP is found, after 28 symbols from receiving the BFR response

for the CORESETs associated with the BFD-RS set that is associated with serving cell PCI, reset the QCL assumption by the spatial filter as for the last PRACH transmission simultaneously; while for the CORESETs associated with the same BFD-RS set as the new beam, reset the QCL assumption by the new beam.

when BFR of both failed TRP is triggered in SpCell, no new beam for the both failed TRPs, after 28 symbols from receiving the BFR response

the QCL assumption of CORESETs associated with the serving cell is reset by the spatial filter as for the last PRACH transmission.

For the CORESET associated with PCI different from serving cell PCI, the spatial filer is not necessary to be reset.

For update of QCL assumption after receiving the BFR response for inter-cell MTRP cases,

- Support to update QCL assumption of CORESETs associated with the serving cell PCI and scheduled PDSCH by the spatial filter as for the last PRACH transmission simultaneously.

Maintenance on HST-SFN scheme

Default TCI state for PDSCH

Time offset less than timeDurationForQCL

Agreement If enableTwoDefaultTCI-States is configured and at least one TCI codepoint indicates two TCI states and time offset between the reception of the DL DCI and the PDSCH is less than the threshold timeDurationForQCL, default beam(s) for Rel-17 enhanced SFN PDSCH (scheme 1 or if supported TRP-based pre-compensation) reception: - Alt 1: Reuse rule to determine TCI states as defined for Rel-16 PDSCH scheme-1a This is UE optional feature Agreement The agreement from RAN1#106b-e meeting is updated as follows When SFN PDSCH is not configured by RRC and there is no TCI codepoint which indicates two TCI states activated for the PDSCH (i.e. Rel-16 MTRP PDSCH is not configured), for PDSCH reception scheduled by DCI format 1_0, 1_1, 1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH is smaller than the threshold timeDurationForQCL, - For DCI format 1_1/1_2, support both configurations with and without TCI state field. - For both cases with and without TCI state field, - If enhanced SFN PDCCH transmission scheme 1 is configured and the lowest CORESET ID in the latest slot is indicated with two TCI states, select the 1st TCI state of the two TCI states of the CORESET as default beam for the PDSCH reception - Otherwise, UE applies the one active TCI state of the CORESET with the lowest controlResourceSetId in the latest slot when receiving the PDSCH - It is up to editor how to capture the above agreement

If the time offset between the reception of the DCI and its scheduled PDSCH is less than the threshold timeDurationForQCL, UE would use the default TCI state for PDSCH reception. Potential scheduling situations are shown in Table 1, considering the UE capability of dynamic switching between STRP and SFN schemes.

Default TCI state assumption when the time offset is less than timeDurationForQCL

| Case | SFN PDSCH configured by RRC | enableTwoDefaultTCI-States | TCI codepoint indicates two TCI states | Dynamic switching | Default TCI state |
| --- | --- | --- | --- | --- | --- |
| 1 | Yes | Configured | At least one | Support | Reuse rule to determine TCI states as defined for Rel-16 PDSCH scheme-1a |
|  |  |  | All | Not support |  |
| 2 | Yes | Configured | None | Support | If enhanced SFN PDCCH transmission scheme 1 or TRP-based pre-compensation is configured and the lowest CORESET ID in the latest slot is indicated with two TCI states, UE applies both TCI states of the CORESET Otherwise, UE applies the one active TCI state of the CORESET with the lowest ID in the latest slot |
| 3 | Yes | Not configured | / | Support | Solution 1: If enhanced SFN PDCCH transmission scheme 1 is configured and the CORESET with the lowest ID in the latest slot is indicated with two TCI states, UE applies the 1st TCI state of the two TCI states of the CORESET Otherwise, UE applies the one active TCI state of the CORESET with the lowest ID in the latest slot Solution 2: Error case (UE expects that enableTwoDefaultTCI-States is configured) |
| 4 |  |  | All | Not support | Error case (UE expects that enableTwoDefaultTCI-States is configured) |
| 5 | No | / | None | Support | If enhanced SFN PDCCH transmission scheme 1 is configured and the lowest CORESET ID in the latest slot is indicated with two TCI states, select the 1st TCI state of the two TCI states of the CORESET Otherwise, apply the one active TCI state of the CORESET with the lowest ID in the latest slot(agreed) |
|  |  |  |  | Not support |  |

Case 1&5: These two cases were agreed in the previous e-meetings.

Case 2: When SFN PDSCH is configured by RRC and enableTwoDefaultTCI-States is configured, if UE can support the dynamic switching between STRP and SFN transmission, but there is no TCI codepoint with two TCI states activated by MAC CE, the default TCI state for PDSCH can be determined as follow.

If enhanced SFN PDCCH transmission scheme 1 or TRP-based pre-compensation is configured and the CORESET with the lowest ID in the latest slot is indicated with two TCI states, UE applies both TCI states of the CORESET

Otherwise, UE applies the one active TCI state of the CORESET with the lowest ID in the latest slot

This case is beneficial for saving signal overhead, since UE can work in SFN scheme by default TCI rule without any TCI state indication.

Regarding UE not capable of dynamic switching between STRP and SFN transmission, when SFN PDSCH is configured by RRC, UE would not expect to be indicated by MAC CE with a single TCI state per any of TCI codepoint in the previous agreement. That means all TCI codepoints indicated by MAC CE would be with two TCI states, which has been covered in case 1.

Case 3: When SFN PDSCH is configured by RRC and enableTwoDefaultTCI-States is not configured, UE would only buffer one default TCI state. Therefore, for a UE which supports dynamic switching between STRP and SFN transmission, the default TCI state for PDSCH can be determined as follow.

If enhanced SFN PDCCH transmission scheme 1 is configured and the CORESET with the lowest ID in the latest slot is indicated with two TCI states, UE applies the 1st TCI state of the two TCI states of the CORESET

Otherwise, UE applies the one active TCI state of the CORESET with the lowest ID in the latest slot

Another potential solution is to avoid this case. That is if SFN PDSCH is configured by RRC, UE expects enableTwoDefaultTCI-States is configured.

Case 4: When SFN PDSCH is configured by RRC, if UE does not support dynamic switching between STRP and SFN transmission, all TCI codepoints indicated by MAC CE would be with two TCI states. However, if enableTwoDefaultTCI-States is not configured, UE would only buffer one default TCI state, then dynamic switching would happen between the SFN scheme and STRP scheme. Therefore, to avoid this case, enableTwoDefaultTCI-States should be configured, when SFN PDSCH is configured by RRC and UE does not support dynamic switching between STRP and SFN transmission. Then case 4 would convert to case 1.

It can be observed that case 3 and case 4 are both about the situation when SFN PDSCH is configured by RRC and enableTwoDefaultTCI-States is not configured. Therefore, one easy way to avoid the default beam behavior for these cases is to restrict that when SFN PDSCH is configured, UE expects enableTwoDefaultTCI-States configuration.

When SFN PDSCH is configured by RRC, enableTwoDefaultTCI-States is configured and there is no TCI codepoint with two TCI states activated by MAC CE, if UE is capable of the dynamic switching between STRP and SFN transmission, the time offset between the reception of the DCI and its scheduled PDSCH is less than the threshold timeDurationForQCL, the default TCI state for PDSCH can be determined as follow.

If enhanced SFN PDCCH transmission scheme 1 or TRP-based pre-compensation is configured and the lowest CORESET ID in the latest slot is indicated with two TCI states, UE applies both TCI states of the CORESET

Otherwise, UE applies the one active TCI state of the CORESET with the lowest ID in the latest slot

When SFN PDSCH is configured, and the time offset between the reception of the DCI and its scheduled PDSCH is less than the threshold timeDurationForQCL, UE expects enableTwoDefaultTCI-States is configured.

UE supporting SFN PDSCH is mandated to report FG 16-2b-0

Agreement For PDSCH reception scheduled by DCI format 1_0, [1_1 and 1_2], if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold timeDurationForQCL - Support configuration when there is no TCI field in the DCI scheduling PDSCH - UE applies the state(s) of the scheduling CORESET when receiving the PDSCH - if there are two active TCI states for the CORESET, UE applies the both QCL assumption of the CORESET that schedules the PDSCH when receiving the PDSCH - otherwise, UE applies the one active TCI state of the CORESET when receiving the PDSCH - FFS if the time offset between the reception of the DL DCI and the corresponding PDSCH is smaller than the threshold timeDurationForQCL This is UE optional feature Agreement When SFN PDSCH is not configured by RRC and there is no TCI codepoint which indicates two TCI states activated for the PDSCH (i.e. Rel-16 MTRP PDSCH is not configured) and SFN transmission scheme 1 is configured for PDCCH, for PDSCH reception scheduled by DCI format 1_0, 1_1, 1_2 without TCI field, if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold timeDurationForQCL if applicable and the CORESET which schedules the PDSCH is indicated with two TCI states, the default TCI state is defined as the first TCI state of the CORESET. Agreement When SFN PDSCH and SFN PDCCH are configured by RRC, for PDSCH reception scheduled by DCI formats 1_1 and 1_2, and, if applicable the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold timeDurationForQCL - Support configuration when there is no TCI field in the DCI scheduling PDSCH - UE applies the TCI state(s) of the scheduling CORESET when receiving the PDSCH - If there are two active TCI states for the CORESET, UE applies both QCL assumptions of the CORESET that schedules the PDSCH when receiving the PDSCH - otherwise, if there is one active TCI state for the CORESET, UE applies the one active TCI state of the CORESET when receiving the PDSCH This feature is UE optional capability - If UE doesn’t support this capability, UE is expected to be configured with TCI state field - UEs supporting this feature and are not capable of dynamic switching between single TRP and SFN, the CORESET that schedules PDSCH by DCI formats 1_1 and 1_2 (FFS DCI format 1_0) should be activated with two TCI states. FFS for maintenance: if SFN PDCCH is not configured Agreement When SFN is configured for PDSCH and not configured for PDCCH, TCI field should be always present in DCI formats 1_1 and 1_2 for SFN PDSCH transmission with scheduling offset larger than threshold timeDurationForQCL if applicable - FFS whether the above assumption is applicable for UE not capable of dynamic switching

If the time offset between the reception of the DCI and its scheduled PDSCH is equal to or larger than the threshold timeDurationForQCL and there is no TCI field in the DCI, UE would also use the default beam for PDSCH reception. In this case, potential scheduling situations are shown in Table 2, considering the UE capability of dynamic switching between STRP and SFN transmission.

Default TCI state assumption when the time offset is equal to or larger than timeDurationForQCL and there is no TCI field in DCI 1_0, 1_1 and 1_2

| Case | SFN PDSCH configured by RRC | SFN PDCCH configured by RRC | TCI state(s) of the CORESET | Dynamic switching | Default TCI state |
| --- | --- | --- | --- | --- | --- |
| 1 | Yes | Yes | Two | / | Both TCI states of the CORESET that schedules the PDSCH |
| 2 |  |  | One | Support | The one active TCI state of the CORESET that schedules the PDSCH |
| 3 |  |  |  | Not support | Error case (the CORESET that schedules PDSCH by DCI formats 1_1 and 1_2 (FFS DCI format 1_0) should be activated with two TCI states) |
| 4 |  | No | One | Support | For DCI 1_1 and 1_2: no such default beam case (TCI field should be always present in DCI formats 1_1 and 1_2) For DCI 1_0: The one active TCI state of the CORESET that schedules the PDSCH |
| 5 |  |  |  | Not support | For DCI 1_1 and 1_2: no such default beam case (TCI field should be always present in DCI formats 1_1 and 1_2) For DCI 1_0: Error case (UE expects that SFN PDCCH is configured by RRC) |
| 6 | No | YES | Two | Support | The first TCI state of the CORESET that schedules the PDSCH |
|  |  |  |  | Not support |  |

Case 1&2&3&6: They were agreed in the previous e-meetings, excluding the FFS for DCI format 1_0 in case 3. Regarding the case of DCI 1_0 not capable of dynamic switching between STRP and SFN transmission, it can be discussed with case 5 together.

Case 4: When SFN is configured for PDSCH and not configured for PDCCH, the scheduling offset larger than threshold timeDurationForQCL, it has been agreed that TCI field should always be present in DCI formats 1_1 and 1_2. However, regarding DCI 1_0 without TCI field, when the CORESET which schedules the PDSCH is indicated with one TCI state and UE is capable of dynamic switching between STRP and SFN transmission, UE can apply the one active TCI state of the CORESET that schedules the PDSCH.

Case 5: For DCI 1_1 and 1_2, since TCI field should always be present, there is no default beam behavior required for UE not capable of dynamic switching between STRP and SFN transmission. For DCI 1_0, if UE is not capable of the dynamic switching, one easy way is to restrict that SFN PDCCH should be configured by RRC and the CORESET should be indicated with two TCI states, which is similar to the restriction for DCI 1_1 and 1_2 in case 3 that has been agreed in the previous RAN1 e-meeting.

When SFN PDSCH is configured by RRC, if applicable the time offset between the reception of the DCI format 1_0 and the corresponding PDSCH is equal to or larger than the threshold timeDurationForQCL,

If SFN PDCCH is not configured by RRC and UE is capable of dynamic switching between STRP and SFN transmission, UE applies the one active TCI state of the CORESET that schedules the PDSCH

If UE is not capable of dynamic switching between STRP and SFN transmission, UE expects SFN PDCCH is configured by RRC and the CORESET that schedules PDSCH by DCI format 1_0 should be activated with two TCI states

Beam failure recovery for SFN PDCCH

Regarding beam failure recovery for SFN PDCCH, it has been agreed that RS of CORESETs with both single and two TCI states are used for BFD-RS and one BFD RS pair for SFN CORESET is counted as two BFD RS. The remaining issue is whether to enhance the number of BFD-RS and NBI-RS.

Agreement If enhanced SFN PDCCH transmission scheme (scheme 1 or TRP-based pre-compensation) is configured and two TCI states are activated for at least one CORESET, support the following configuration of RS for BFD - For implicit configuration - Alt 1-2: RS of CORESETs with both single and two TCI states are used FFS: The maximum number of BFD RS and details on RS determination Agreement When CORESET is indicated with two TCI states - One BFD RS pair for SFN CORESET is counted as two BFD RSs - FFS: Increase the maximum number of monitored BFD RSs to X. - X is UE capability - X = 2, 3, 4, FFS other values of X Agreement When two TCI states are activated for a CORESET, NBI RS can be configured as follows - Alt 4-1: Using the existing Rel-15 NBI configuration based on single SSB / CSI-RS resource - FFS addition support of Alt 4-2: two new beam identification CSI-RS resource sets / new beam identification CSI-RS resource pairs or SSB pairs

Configuration of BFD-RS

Beam failure detection is the first step of the beam failure recovery mechanism, in which UE monitors a set of q0  periodic CSI-RS resources to assess whether the beam failure trigger condition has been met or not. The CSI-RS can be explicitly configured by RRC signaling or implicitly associated by using the same QCL assumption with CORESETs. In the current specification, at most three CORESETs are configured for one active BWP and one of them is reserved as special CORESET-BFR, so the UE only supports up to a set q0 of two SSBs or CSI-RS resources for BFD. Since the UE can be configured with multiple CORESETs, the CORESET would be indicated with STRP-based transmission or SFN-based transmission. If the SSB or CSI-RS resources for BFD are implicitly indicated, it has been agreed that the case with one of the CORESETs is configured with two TCI states while the other CORESET is configured with one TCI state is supported. In this case, the maximum number of BFD RS should be further discussed. One way is that UE still monitors up to two SSB or CSI-RS resources. Another way is to increase the number of BFD-RS, which would increase the UE complexity for UE monitoring.

Support to keep up to two BFD RS for implicit BFD RS configuration for both CORESETs with both single and two TCI states.

Configuration of NBI-RS

After the BFD measurement, if all the BFD-RS are failed for SFN transmission, then the beam recovery would be triggered. In this situation, cell-specific BFR can be implemented rather than TRP-specific BFR for SFN transmission. That means the beam recovery can start from one of the TRP based on one NBI-RS reported by UE. After the success of beam recovery, the STRP-based transmission would resume first. Then, based on the regular beam management mechanism, the network would find one new reliable beam for another TRP, then the transmission can switch to SFN-based transmission. In this way, there is no need for UE to report one SSB or CSI-RS pair of NBI-RS. Therefore, regarding the NBI-RS configuration, the existing specified behavior can be reused.

Support reusing the existing NBI-RS configuration without NBI-RS pair when SFN transmission is configured in Rel-17.

SFN-based CORESET associated with Type-0/0A/1/2 PDCCH CSS

When Type-0/0A/1/2 PDCCH CSS is associated with SFN-based CORESET, how does the UE receive the SI, msg2 and paging in the CORESET with the first TCI state or both TCI states should be further discussed, especially when the CORESET is CORESET#0.

Agreement If PDCCH candidates in CSS 3 are associated with CORESET that is activated with two TCI states and configured with enhanced SFN scheme 1 or TRP based pre-compensation, both TCI states can be applied for the CSS reception. - FFS: Whether/How specification change is needed is up to the editor Agreement For the response to RAN2 LS (in R1-2200886), the following is agreed Question: RAN2 would like to ask whether “Enhanced TCI state indication for UE specific PDCCH MAC CE” can be applied to CORESET zero or not.. - RAN1 response: There is no restriction in RAN1 on whether enhanced TCI state indication for UE specific PDCCH MAC CE can be applied to CORESET zero.

The compatibility of system information reception

In general, system information such as SIB1, OSI and paging is broadcast from the network. Therefore, when the CORESET is indicated with two TCI states and the system information is transmitted in the SFN scheme, legacy UEs in the network would not be able to receive the system information correctly, since they can only track one TRS based on the indicated TCI state. The following figures show the received QPSK symbols of the Rel-17 UEs and legacy UEs, when they receive the SFN-based system information.

| (a) Rel-17 UE, 3km/h | (b) Legacy UE, 3km/h |
| --- | --- |
| (c) Rel-17 UE, 500km/h | (d) Legacy UE, 500km/h |

From the above evaluation results, it can be found that the performance of the legacy UE to demodulate the system information is terrible. That means deploying SFN-based system information in a network will sacrifice the legacy UEs.

Considering the performance degradation of legacy UEs when receiving SFN-based system information, it is difficult to deploy SFN-based system information in a network with legacy UEs.

Issue on Type-0/0A/2 PDCCH CSS

In the spec TS 38.213, the following PDCCH monitoring behavior in the search space#0 is specified.

If a UE is provided a zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set as described in Clause 13, and the UE is provided a C-RNTI, the UE monitors PDCCH candidates only at monitoring occasions associated with a SS/PBCH block, where the SS/PBCH block is determined by the most recent of - a MAC CE activation command indicating a TCI state of the active BWP that includes a CORESET with index 0, as described in [6, TS 38.214], where the TCI-state includes a CSI-RS which is quasi-co-located with the SS/PBCH block, or - a random access procedure that is not initiated by a PDCCH order that triggers a contention-free random access procedure

If CORESET#0 is indicated with two TCI states, there would be two CSI-RS resources as the QCL sources in the two TCI states separately, and each CSI-RS resource is QCL-ed with one SS/PBCH block. Therefore, when Type0/0A/2-PDCCH CSS set is configured as search space#0 which is associated with SFN-based CORESET#0, UE needs to monitor PDCCH candidates at monitoring occasions associated with two SS/PBCH blocks if both TCI states is applied for the CSS reception. However, in this case, UE would monitor the PDCCH in two monitoring occasions, that is a TDM-based monitoring rather than SFN-based. Moreover, if only the first TCI state of the CORESET is applied, UE can monitor PDCCH candidates only at monitoring occasions associated with the SS/PBCH block QCL-ed with the CSI-RS resource in the first TCI state.

If both TCI states of the CORESET are applied for PDCCH reception when Type0/0A/2-PDCCH CSS is configured as search space#0, UE would monitor PDCCH candidates at monitoring occasions associated with two SS/PBCH blocks QCL-ed with CSI-RS resources in two TCI states, leading to a TDM-based PDCCH monitoring rather than SFN-based.

Issue on Type-1 PDCCH CSS

In the spec TS 38.213, the following QCL assumption is specified for PDCCH order and PDCCH in msg2.

If the UE attempts to detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI in response to a PRACH transmission initiated by a PDCCH order that triggers a contention-free random access procedure for the SpCell [11, TS 38.321], the UE may assume that the PDCCH that includes the DCI format 1_0 and the PDCCH order have same DM-RS antenna port quasi co-location properties.

From the above description, when receiving the PDCCH initiated by a PDCCH order that triggers a CFRA, UE may assume that the PDCCH in msg2 and the PDCCH order have same DM-RS antenna port quasi co-location properties. Therefore, if the CORESET of PDCCH in msg2 is indicated with two TCI states, UE would receive PDCCH in msg2 and PDCCH order in SFN scheme. However, PDCCH repetition is not agreed in the 8.1.2.1 agenda item. Besides, whether msg2 can be transmitted in SFN scheme hasn’t been discussed yet. It is hard to complete SFN-based transmission in random-access procedure at this stage.

If both TCI states of the CORESET are applied for PDCCH reception in Type1-PDCCH CSS, in the case that the PDCCH with RA-RNTI is transmitted in a CFRA procedure triggered by a PDCCH order, both the PCCCH and PDCCH order would be transmitted in SFN manner.

If Type0/0A/1/2-PDCCH CSS are associated with CORESET activated with two TCI states, the first TCI state is applied for the PDCCH reception.

TP for maintenance

Support the following TP.

Reason for change:

The combination of Rel-17 SFN PDCCH scheme 1 and single-TRP PDSCH is supported as an optional UE feature [nonSfnPdsch-sfnPdcch]. Meanwhile, the combination of Rel-17 SFN PDCCH TRP-based pre-compensation and single-TRP PDSCH is not supported. However, in the current spec TS 38.214, the description of these mentioned combinations has not been captured.

Summary of change:

To describe the combination of SFN PDCCH scheme 1 and single-TRP PDSCH is supported, and the combination of SFN PDCCH TRP-based pre-compensation and single-TRP PDSCH is not supported.

Consequences if not approved:

It would affect UE’s reception process and increase the complexity of UE.

TS 38.214 5.1 UE procedure for receiving the physical downlink shared channel < Unchanged parts are omitted > When a UE is configured with higher layer parameter sfnSchemePdsch set to either 'sfnSchemeA' or 'sfnSchemeB' for a DL BWP and - if the UE reports its capability of [dynamicSFN], the UE is indicated with one or two TCI state(s) in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2, or - otherwise, the UE is not expected to be indicated with one TCI state per any of TCI codepoint by MAC CE, and the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2, and the UE procedure for receiving the PDSCH upon detection of a PDCCH follows clause 5.1 and the QCL assumption for the PDSCH as defined in clause 5.1.5. When a UE is configured with both sfnSchemePdsch and sfnSchemePdcch, the UE shall expect that sfnSchemePdsch and sfnSchemePdcch are set to the same scheme, either 'sfnSchemeA' or 'sfnSchemeB'. If a UE is configured with sfnSchemePdcch set to 'sfnSchemeA' for a DL BWP and activated with two TCI states by MAC CE, and the UE does not report its capability of [nonSfnPdsch-sfnPdcch], the UE does not expect to be indicated with one TCI state in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2. If a UE is configured with sfnSchemePdcch set to 'sfnSchemeB' for a DL BWP and activated with two TCI states by MAC CE, the UE does not expect to be indicated with one TCI state in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2. < Unchanged parts are omitted >

Conclusions

In this contribution, we have the following observations and proposals:

- Considering the performance degradation of legacy UEs when receiving SFN-based system information, it is difficult to deploy SFN-based system information in a network with legacy UEs.

- If both TCI states of the CORESET are applied for PDCCH reception when Type0/0A/2-PDCCH CSS is configured as search space#0, UE would monitor PDCCH candidates at monitoring occasions associated with two SS/PBCH blocks QCL-ed with CSI-RS resources in two TCI states, leading to a TDM-based PDCCH monitoring rather than SFN-based.

- If both TCI states of the CORESET are applied for PDCCH reception in Type1-PDCCH CSS, in the case that the PDCCH with RA-RNTI is transmitted in a CFRA procedure triggered by a PDCCH order, both the PCCCH and PDCCH order would be transmitted in SFN manner.

- Support the following TP.

Reason for change:

In Rel-16, the procedure of SSGS can be executed with explicit switching based on the indication carried by DCI format 2_0 or implicit switching based on some trigger condition, for instance, once the UE detects any of DCI formats by monitoring PDCCH according to a search space set with group index 0, the UE starts monitoring PDCCH according to search space sets with group index 1 and stops monitoring PDCCH related to group index 0.

However, given UE does not decode format 2_0 successfully in case of explicit switching or dose not decode any DCI format successfully in group index 0 in case of implicit switching, the UE would not perform SSGS while gNB always assumes the procedure of SSGS takes effect, so there is misunderstanding about whether PDCCH repetition mechanism is applied.

Summary of change:

In section 10.4 of 38.213, clarification on configuration restriction is added: ‘If a PDCCH reception includes two PDCCH candidates from two linked search space sets based on searchSpaceLinking, UE expects to be provided with a same configuration for searchSpaceGroupIdList in each of search space set.’

Consequences if not approved:

Determination of PDCCH repetition depending on the condition of UE decoding will cause serious negative impact to communication, since some scheduling timeline or reference candidate rule for channels/RSs of DL and UL is different between legacy PDCCH mechanism and PDCCH repetition mechanism in Rel-17, if misunderstanding is unresolved.

TS 38.213 10.4 Search space set group switching and skipping of PDCCH monitoring A UE can be provided a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList for PDCCH monitoring on a serving cell. If the UE is not provided searchSpaceGroupIdList for a search space set, the following procedures are not applicable for PDCCH monitoring according to the search space set. If a PDCCH reception includes two PDCCH candidates from two linked search space sets based on searchSpaceLinking, UE expects to be provided with a same configuration for searchSpaceGroupIdList in each of search space set. If a UE is provided cellGroupsForSwitchList, indicating one or more groups of serving cells, the following procedures apply to all serving cells within each group; otherwise, the following procedures apply only to a serving cell for which the UE is provided searchSpaceGroupIdList. When a UE is provided searchSpaceGroupIdList, the UE resets PDCCH monitoring according to search space sets with group index 0, if provided by searchSpaceGroupIdList. < Unchanged text omitted >

Support the following TP.

Reason for change:

Based on the above agreement, the second PTRS-DMRS association field is introduced to indicate PTRS-DMRS association for maxRank > 2 in mTRP PUSCH. When the second PTRS-DMRS association field is configured, the association between UL PT-RS port(s) and DM-RS port(s) for PUSCH associated to two SRS resource sets should be signaled by the first and the second PTRS-DMRS association field in DCI format 0_1 and DCI format 0_2 respectively. This situation seems to be not included in current version.

Summary of change:

Add ‘s’ for the PTRS-DMRS association field.

Consequences if not approved:

UE’s behaviour on the association between UL PT-RS port(s) and DM-RS port(s) for the PUSCH corresponding to the second SRS resource set will be unclear.

TS 38.214 6.2.3.1 UE PT-RS transmission procedure when transform precoding is not enabled When transform precoding is not enabled and if a UE is configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig, - the higher layer parameters timeDensity and frequencyDensity in PTRS-UplinkConfig indicate the threshold values ptrs-MCSi, i=1,2,3 and NRB,i , i=0,1, as shown in Table 6.2.3.1-1 and Table 6.2.3.1-2, respectively. - if either or both higher layer parameters timeDensity and/or frequencyDensity in PTRS-UplinkConfig are configured, the UE shall assume the PT-RS antenna ports' presence and pattern are a function of the corresponding scheduled MCS and scheduled bandwidth in a corresponding bandwidth part as shown in Table 6.2.3.1-1 and Table 6.2.3.1-2, respectively, - if the higher layer parameter timeDensity is not configured, the UE shall assume LPT-RS = 1. - if the higher layer parameter frequencyDensity is not configured, the UE shall assume KPT-RS = 2. - if none of the higher layer parameters timeDensity and frequencyDensity in PTRS-UplinkConfig are configured, the UE shall assume LPT-RS = 1 and KPT-RS = 2. Table 6.2.3.1-1: Time density of PT-RS as a function of scheduled MCS Scheduled MCS; Time density(). IMCS < ptrs-MCS1; PT-RS is not present. ptrs-MCS1 IMCS < ptrs-MCS2; 4. ptrs-MCS2 IMCS < ptrs-MCS3; 2. ptrs-MCS3 IMCS < ptrs-MCS4; 1. Table 6.2.3.1-2: Frequency density of PT-RS as a function of scheduled bandwidth Scheduled bandwidth; Frequency density (). NRB < NRB0; PT-RS is not present. NRB0 NRB < NRB1; 2. NRB1 NRB; 4. The higher layer parameter PTRS-UplinkConfig provides the parameters ptrs-MCSi, i=1,2,3 and with values in 0-29 when MCS Table 5.1.3.1-1 or Table 5.1.3.1-3 is used and 0-28 when MCS Table 5.1.3.1-2 is used, respectively. ptrs-MCS4 is not explicitly configured by higher layers but assumed 29 when MCS Table 5.1.3.1-1 or Table 5.1.3.1-3 is used and 28 when MCS Table 5.1.3.1-2 is used. The higher layer parameter PTRS-UplinkConfig provides the parameters NRBi i=0,1 with values in range 1-276. If the higher layer parameter PTRS-UplinkConfig indicates that the time density thresholds ptrs-MCSi = ptrs-MCSi+1, then the time density LPTRS of the associated row where both these thresholds appear in Table 6.2.3.1-1 is disabled. If the higher layer parameter frequencyDensity in PTRS-UplinkConfig indicates that the frequency density thresholds NRB,i = NRB,i+1, then the frequency density KPTRS of the associated row where both these thresholds appear in Table 6.2.3.1-2 is disabled. If either or both of the parameters PT-RS time density (LPT-RS) and PT-RS frequency density (KPT-RS), shown in Table 6.2.3.1-1 and Table 6.2.3.1-2, indicates that are configured as 'PT-RS not present', the UE shall assume that PT-RS is not present. When a UE is scheduled to transmit PUSCH with allocation duration of 2 symbols or less, and if LPT-RS is set to 2 or 4, the UE shall not transmit PT-RS. When a UE is scheduled to transmit PUSCH with allocation duration of 4 symbols or less, and if LPT-RS is set to 4, the UE shall not transmit PT-RS. When a UE is scheduled to transmit PUSCH for retransmission, if the UE is scheduled with IMCS > V, where V = 28 for MCS Table 5.1.3.1-1 and MCS Table 5.1.3.1-3 and V = 27 for MCS Table 5.1.3.1-2, respectively, the MCS for PT-RS time-density determination is obtained from the DCI for the same transport block in the initial transmission, which is smaller than or equal to V. The maximum number of configured PT-RS ports is given by the higher layer parameter maxNrofPorts in PTRS-UplinkConfig. The UE is not expected to be configured with a larger number of UL PT-RS ports than it has reported need for. If a UE has reported the capability of supporting full-coherent UL transmission, the UE shall expect the number of UL PT-RS ports to be configured as one if UL-PTRS is configured. For codebook or non-codebook based UL transmission, the association between UL PT-RS port(s) and DM-RS port(s) is signalled by PTRS-DMRS association field(s) in DCI format 0_1 and DCI format 0_2. For a PUSCH corresponding to a configured grant Type 1 transmission, the UE may assume the association between UL PT-RS port(s) and DM-RS port(s) defined by value 0 in Table 7.3.1.1.2-25 or value "00" in Table 7.3.1.1.1.2-26 described in Clause 7.3.1 of [5, TS38.212]. For PUSCH scheduled by DCI format 0_0 or by activation DCI format 0_0, the UL PT-RS port is associated to DM-RS port 0. For non-codebook based UL transmission, the actual number of UL PT-RS port(s) to transmit is determined based on SRI(s) in DCI format 0_1 and DCI format 0_2 or higher layer parameter sri-ResourceIndicator in rrc-ConfiguredUplinkGrant. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'noncodebook', the actual number of UL PT-RS port(s) to transmit corresponding to each SRS resource set is determined based on SRI(s) corresponding to the associated SRS resource set or higher layer parameter sri-ResourceIndicator or sri-ResourceIndicator2 corresponding to the associated SRS resource set in rrc-ConfiguredUplinkGrant. A UE is configured with the PT-RS port index for each configured SRS resource by the higher layer parameter ptrs-PortIndex configured by SRS-Config if the UE is configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig. If the PT-RS port index associated with different SRIs are the same, the corresponding UL DM-RS ports are associated to the one UL PT-RS port.

Support the following TP.

Reason for change:

Power boosting factor of PT-RS is determined by the number of scheduled PT-RS ports, the rank and coherence of the indicated precoder. For MTRP PUSCH, power scaling factor of PT-RS for PUSCH associated two SRS resource sets should be determined separately. The actual number of scheduled PT-RS ports can be different, which is already captured in current spec. The number of layers scheduled for PUSCH associated two SRS resource sets is same, which is also constrained by indicated two TPMIs/SRIs. However, the coherence properties of the two indicated TPMIs may be different. For instance, one TPMI may indicate a full-coherent precoder while the second TPMI may indicate a non-coherent precoder when full-partial-non codebook subset is configured. So, the scaling factors for PT-RS transmitted on PUSCH repetitions towards 2TRPs should be determined according to the first and second TPMI indication respectively. Current spec does not capture this case.

Summary of change:

Description on power boosting factor determination in mTRP transmission is added.

Consequences if not approved:

UE’s behavior on determining power boosting factor of the PTRS transmitted on PUSCH associated with the second SRS resource is unclear. Or, the power boosting factor will be determined by mistake because it is also determined based on the first TPMI field as described in current version.

TS 38.214 6.2.3.1 UE PT-RS transmission procedure when transform precoding is not enabled < Unchanged text omitted > For non-codebook based UL transmission, the actual number of UL PT-RS port(s) to transmit is determined based on SRI(s) in DCI format 0_1 and DCI format 0_2 or higher layer parameter sri-ResourceIndicator in rrc-ConfiguredUplinkGrant. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'noncodebook', the actual number of UL PT-RS port(s) to transmit corresponding to each SRS resource set is determined based on SRI(s) corresponding to the associated SRS resource set or higher layer parameter sri-ResourceIndicator or sri-ResourceIndicator2 corresponding to the associated SRS resource set in rrc-ConfiguredUplinkGrant. A UE is configured with the PT-RS port index for each configured SRS resource by the higher layer parameter ptrs-PortIndex configured by SRS-Config if the UE is configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig. If the PT-RS port index associated with different SRIs are the same, the corresponding UL DM-RS ports are associated to the one UL PT-RS port. For partial-coherent and non-coherent codebook-based UL transmission, the actual number of UL PT-RS port(s) is determined based on TPMI(s) and/or number of layers which are indicated by 'Precoding information and number of layers' field(s) in DCI format 0_1 and DCI format 0_2 or configured by higher layer parameter precodingAndNumberOfLayers: - if the UE is configured with the higher layer parameter maxNrofPorts in PTRS-UplinkConfig set to 'n2', the actual UL PT-RS port(s) and the associated transmission layer(s) are derived from indicated TPMI(s) as: - PUSCH antenna port 1000 and 1002 in indicated TPMI(s) share PT-RS port 0, and PUSCH antenna port 1001 and 1003 in indicated TPMI(s) share PT-RS port 1. - UL PT-RS port 0 is associated with the UL layer 'x' of layers which are transmitted with PUSCH antenna port 1000 and PUSCH antenna port 1002 in indicated TPMI(s), and UL PT-RS port 1 is associated with the UL layer 'y' of layers which are transmitted with PUSCH antenna port 1001 and PUSCH antenna port 1003 in indicated TPMI(s), where 'x' and/or 'y' are given by DCI parameter 'PTRS-DMRS association' as shown in DCI format 0_1 and DCI format 0_2 described in Clause 7.3.1 of [5, TS38.212]. When the UE is scheduled with Qp={1,2} PT-RS port(s) in uplink and the number of scheduled layers is , - If the UE is configured with higher layer parameter ptrs-Power, the PUSCH to PT-RS power ratio per layer per RE is given by , where is shown in the Table 6.2.3.1-3 according to the higher layer parameter ptrs-Power, the PT-RS scaling factor specified in clause 6.4.1.2.2.1 of [4, TS 38.211] is given by and also on the 'Precoding Information and Number of Layers' field in DCI. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', the PUSCH corresponding to each SRS resource set to PT-RS power ratio per layer per RE is given by , where is shown in the Table 6.2.3.1-3 according to the higher layer parameter ptrs-Power, the PT-RS scaling factor specified in clause 6.4.1.2.2.1 of [4, TS 38.211] is given by and also on the 'Precoding Information and Number of Layers' field or 'Second Precoding Information and Number of Layers' field in DCI. - The UE shall assume ptrs-Power in PTRS-UplinkConfig is set to state "00" in Table 6.2.3.1-3 if not configured or in case of non-codebook based PUSCH. Table 6.2.3.1-3: Factor related to PUSCH to PT-RS power ratio per layer per RE UL-PTRS-power /; The number of PUSCH layers ( ). 1; 2; 3; 4. All cases; Full coherent; Partial and non- coherent and non-codebook based; Full coherent; Partial and non- coherent and non-codebook based; Full coherent; Partial coherent; Non-coherent and non-codebook based. 00; 0; 3; 3Qp-3; 4.77; 3Qp-3; 6; 3Qp; 3Qp-3. 01; 0; 3; 3; 4.77; 4.77; 6; 6; 6. 10; Reserved. 11; Reserved. < Unchanged text omitted >

Support the following TP.

Reason for change:

In RAN1#106-e, two sets of default power control parameter are defined for PUSCH transmission towards two TRPs. When PUSCH transmission towards the first or the second TRP is indicated by the SRS resource set indication field, the first or second set of default power control parameter is utilized accordingly. However, in current version of spec, two sets of power control parameter are described only when two TRP operation is indicated. So, we propose to further clarify default power control parameter acquisition for single TRP scheduling case.

Summary of change:

Additional illustration on P0/alpha determination when SRS resource set indicator field is ‘00’ and ‘01’ is added.

Consequences if not approved:

If not explicitly captured in spec, one set of parameters determined as Rel-15/16 is shared for the first and second TRP when only the first or second TRP is scheduled which is unreasonable.

7.1.1 UE behaviour If a UE transmits a PUSCH on active UL BWP b of carrier f of serving cell c using parameter set configuration with index j and PUSCH power control adjustment state with index l, the UE determines the PUSCH transmission power PPUSCH,b,f,c(i,j,qd,l) in PUSCH transmission occasion i as [dBm] where, - PCMAX,f,c(i)is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier f of serving cell c in PUSCH transmission occasion i. - PO_PUSCH,b,f,c(j) is a parameter composed of the sum of a component PO_NOMINAL,PUSCH,f,c(j) and a component PO_UE_PUSCH,b,f,c(j) where j∈0,1,…,J-1. - If a UE established dedicated RRC connection using a Type-1 random access procedure, as described in clause 8, and is not provided P0-PUSCH-AlphaSet or for a PUSCH (re)transmission corresponding to a RAR UL grant as described in clause 8.3, j=0, PO_UE_PUSCH,b,f,c0=0, and PO_NOMINAL,PUSCH,f,c0=PO_PRE+∆PREAMBLE,Msg3, where PO_PRE is provided by preambleReceivedTargetPower [11, TS 38.321] and ΔPREAMBLE_Msg3 is provided by msg3-DeltaPreamble, or ∆PREAMBLE,Msg3=0 dB if msg3-DeltaPreamble is not provided, for carrier f of serving cell c - If a UE established dedicated RRC connection using a Type-2 random access procedure, as described in clause 8, and is not provided P0-PUSCH-AlphaSet, or for a PUSCH transmission for Type-2 random access procedure as described in clause 8.1A, j=0, PO_UE_PUSCH,b,f,c(0)=0, and PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH, where PO_PRE is provided by msgA-preambleReceivedTargetPower, or by preambleReceivedTargetPower if msgA-preambleReceivedTargetPower is not provided and ΔMsgA_PUSCH is provided by msgA-DeltaPreamble, or ΔMsgA_PUSCH=ΔPREAMBLE_Msg3 dB if msgA-DeltaPreamble is not provided, for carrier f of serving cell c - For a PUSCH (re)transmission configured by ConfiguredGrantConfig, j=1, PO_NOMINAL,PUSCH,f,c1 is provided by p0-NominalWithoutGrant, or PO_NOMINAL,PUSCH,f,c1=PO_NOMINAL,PUSCH,f,c0 if p0-NominalWithoutGrant is not provided. - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', and is provided p0-PUSCH-Alpha2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format that includes a SRS resource set indicator field, and for active UL BWP b of carrier f of serving cell - If the SRS resource set indicator value is 00, first PO_UE_PUSCH,b,f,c1 value is provided by the value of p0-PUSCH-Alpha in ConfiguredGrantConfig. - If the SRS resource set indicator value is 01, second PO_UE_PUSCH,b,f,c1 value is provided by the value of p0-PUSCH-Alpha2 in ConfiguredGrantConfig. - If the SRS resource set indicator value is 10 or 11, first and second PO_UE_PUSCH,b,f,c1 values are respectively provided by the values of p0-PUSCH-Alpha and by p0-PUSCH-Alpha2 in ConfiguredGrantConfig. - else, PO_NOMINAL,PUSCH,f,c1 is provided by p0 obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig that provides an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet, or by p0-PUSCH for a PUSCH (re)transmission as described in clause 19.1, for active UL BWP b of carrier f of serving cell c - For j∈2,…,J-1=SJ, a PO_NOMINAL,PUSCH,f,cj value, applicable for all j∈SJ, is provided by p0-NominalWithGrant, or PO_NOMINAL,PUSCH,f,cj=PO_NOMINAL,PUSCH,f,c0 if p0-NominalWithGrant is not provided, for each carrier f of serving cell c and a set of PO_UE_PUSCH,b,f,cj values are provided by a set of p0 in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP b of carrier f of serving cell c - If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId and if a DCI format scheduling the PUSCH transmission includes an SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in the DCI format [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values and determines the value of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value. If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId - if the DCI format scheduling the PUSCH transmission includes two SRI fields and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' and if, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the two SRI fields and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines first and second values of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values that are mapped to the values of the first and second SRI fields, respectively. - if the DCI format scheduling the PUSCH transmission includes two SRI fields and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between - a set of values for the first SRI field value and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines the first value of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the first SRI field value, and - a set of values associated with the second SRI field value for a same number of layers as indicated by the first SRI field [5, TS 38.212], and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines the second value of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212]. - If the DCI format also includes an open-loop power control parameter set indication field and a value of the open-loop power control parameter set indication field is '1' and if the DCI format scheduling the PUSCH transmission includes an SRI field, the UE determines a value of PO_UE_PUSCH,b,f,cj from a first value in P0-PUSCH-Set with a p0-PUSCH-SetId value mapped to the SRI field value. - If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId - if a DCI format scheduling the PUSCH transmission includes two SRI fields and an open-loop power control parameter set indication field and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' - if a value of the open-loop power control parameter set indication field is '0', the UE determines two values of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values in SRI-PUSCH-PowerControl that are mapped to the two SRI values corresponding to each SRS resource set with usage set to 'codebook'. - if a value of the open-loop power control parameter set indication field is '1', the UE determines two values of PO_UE_PUSCH,b,f,cj from first values in P0-PUSCH-Set in P0-PUSCH-SetList and P0-PUSCH-Set in P0-PUSCH-SetList2 with p0-PUSCH-SetId values mapped to the two SRI values corresponding to each SRS resource set with usage set to 'codebook', respectively. - if a DCI format scheduling the PUSCH transmission includes two SRI fields and an open-loop power control parameter set indication field and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', - if a value of the open-loop power control parameter set indication field is '0', the UE determines two values of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values in SRI-PUSCH-PowerControl that are mapped to the first SRI field value corresponding to the first SRS resource set with usage set to 'nonCodebook' and to a second value, that is associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, corresponding to the second SRS resource set with usage set to 'nonCodebook'. - if a value of the open-loop power control parameter set indication field is '1', the UE determines two values of PO_UE_PUSCH,b,f,cj from first values in P0-PUSCH-Set in P0-PUSCH-SetList and P0-PUSCH-Set in P0-PUSCH-SetList2 with p0-PUSCH-SetId values mapped to the first SRI field value corresponding to the first SRS resource set with usage set to 'nonCodebook, and a second value, that is associated with the second SRS field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, corresponding to the second SRS resource set with usage set to 'nonCodebook', respectively. - If the UE is not provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and if the PUSCH transmission, except for the PUSCH retransmission corresponding to a RAR UL grant, is scheduled by a DCI format that does not include an SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, j=2, - If P0-PUSCH-Set is provided to the UE and the DCI format includes an open-loop power control parameter set indication field, the UE determines a value of PO_UE_PUSCH,b,f,cj from - a first P0-PUSCH-AlphaSet in p0-AlphaSets if a value of the open-loop power control parameter set indication field is '0' or '00' - a first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value if a value of the open-loop power control parameter set indication field is '1' or '01' - a second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value if a value of the open-loop power control parameter set indication field is '10' - else, the UE determines PO_UE_PUSCH,b,f,cj from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and the PUSCH transmission is scheduled by a DCI format that does not include an SRI field and includes an SRS resource set indicator field with value 10 or 11 - If P0-PUSCH-Set is provided to the UE and the DCI format includes an open-loop power control parameter set indication field, the UE determines first and second values of PO_UE_PUSCH,b,f,cj as - first and second P0-PUSCH-AlphaSet in p0-AlphaSets if the open-loop power control parameter set indication value is '0' or '00' - first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value in p0-PUSCH-SetList and first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value in p0-PUSCH-SetList2, respectively, if the open-loop power control parameter set indication value is '1' or '01' - second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value in p0-PUSCH-SetList and second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID in p0-PUSCH-SetList2, respectively, if the open-loop power control parameter set indication value is '10' or '11' - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and the PUSCH transmission is scheduled by a DCI format that does not include an SRI field and includes an SRS resource set indicator field with value 00 - If P0-PUSCH-Set is provided to the UE and the DCI format includes an open-loop power control parameter set indication field, the UE determines first and second values of PO_UE_PUSCH,b,f,cj as - first P0-PUSCH-AlphaSet in p0-AlphaSets if the open-loop power control parameter set indication value is '0' or '00' - first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value in p0-PUSCH-SetList, if the open-loop power control parameter set indication value is '1' or '01' - second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value in p0-PUSCH-SetList, if the open-loop power control parameter set indication value is '10' or '11' - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and the PUSCH transmission is scheduled by a DCI format that does not include an SRI field and includes an SRS resource set indicator field with value 01 - If P0-PUSCH-Set is provided to the UE and the DCI format includes an open-loop power control parameter set indication field, the UE determines first and second values of PO_UE_PUSCH,b,f,cj as - second P0-PUSCH-AlphaSet in p0-AlphaSets if the open-loop power control parameter set indication value is '0' or '00' - first value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID value in p0-PUSCH-SetList2, if the open-loop power control parameter set indication value is '1' or '01' - second value in P0-PUSCH-Set with the lowest p0-PUSCH-SetID in p0-PUSCH-SetList2, if the open-loop power control parameter set indication value is '10' or '11' - else, the UE determines first and second values PO_UE_PUSCH,b,f,cj from the value of the first and second P0-PUSCH-AlphaSet in p0-AlphaSets, respectively - For αb,f,cj - For j=0, - if PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH and msgA-Alpha is provided, αb,f,c(0) is the value of msgA-Alpha - elseif PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔPREAMBLE_Msg3 or msgA-Alpha is not provided, and msg3-Alpha is provided, αb,f,c(0) is the value of msg3-Alpha - else, αb,f,c0=1 - For j=1, - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', and is provided p0-PUSCH-Alpha2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format that includes an SRS resource set indicator field, and for active UL BWP b of carrier f of serving cell - If the SRS resource set indicator value is 00, first αb,f,c(1) value is provided by p0-PUSCH-Alpha in ConfiguredGrantConfig. - If the SRS resource set indicator value is 01, first αb,f,c(1) value is provided by p0-PUSCH-Alpha2 in ConfiguredGrantConfig. - If the SRS resource set indicator value is 10 or 11, first and second αb,f,c(1) values are respectively provided by p0-PUSCH-Alpha and p0-PUSCH-Alpha2 in ConfiguredGrantConfig. - else αb,f,c(1) is provided by alpha obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig providing an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet, or by alpha for a PUSCH (re)transmission as described in clause 19.1, for active UL BWP b of carrier f of serving cell c - For j∈SJ, a set of αb,f,cj values are provided by a set of alpha in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP b of carrier f of serving cell c - If the UE is provided SRI-PUSCH-PowerControl and more than one values of p0-PUSCH-AlphaSetId in p0-AlphaSets, - if a DCI format scheduling the PUSCH transmission includes two SRI fields and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for - the two SRI fields and a set of indexes provided by P0-PUSCH-AlphaSetId that map to P0-PUSCH-AlphaSet values, and determines first and second values of αb,f,cj from the P0-PUSCH-AlphaSetID values that are mapped to the values of the first and second SRI field values, respectively. - if a DCI format scheduling the PUSCH transmission includes two SRI fields and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for - the first SRI field and a set of indexes provided by P0-PUSCH-AlphaSetId that map to P0-PUSCH-AlphaSet values, and determines first value of αb,f,cj from the P0-PUSCH-AlphaSetID value that is mapped to the first SRI field value, and - the second value, associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines the second value of αb,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the second SRI field value - if a DCI format scheduling the PUSCH transmission includes one SRI field, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in the DCI format [5, TS 38.212] and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values and determines the values of αb,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the SRI field value - If the UE is not provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and if the PUSCH transmission except for the PUSCH retransmission corresponding to a RAR UL grant is scheduled by a DCI format that does not include an SRI field, or if SRI-PUSCH-PowerControl is not provided to the UE, j=2, and the UE determines αb,f,cj from the value of the first P0-PUSCH-AlphaSet in p0-AlphaSets - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and the PUSCH transmission is scheduled by a DCI format that does not include an SRI field and includes an SRS resource set indicator field with value 10 or 11, the UE determines αb,f,cj from first and second P0-PUSCH-AlphaSet in p0-AlphaSets - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' and the PUSCH transmission is scheduled by a DCI format that does not include an SRI field and includes an SRS resource set indicator field with value 00 or 01, the UE determines αb,f,cj from first P0-PUSCH-AlphaSet or second P0-PUSCH-AlphaSet in p0-AlphaSets, respectively.

Support the following TP.

Reason for change:

It was agreed that PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI, current spec doesn’t reflect the agreement.

Summary of change:

In section 5.1.4, “associated with the same PCI” is added behind SS/PBCH block

Consequences if not approved:

PDSCH rate matching is incorrect for intercell multi-TRP operation.

5.1.4 PDSCH resource mapping When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH. When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MSGB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, and the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block associated with the same PCI is transmitted. A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH. For operation with shared spectrum channel access, SS/PBCH block transmission according to ssb-PositionsInBurst represents all of the candidate SS/PBCH blocks corresponding to SS/PBCH block indices provided by ssb-PositionsInBurst as described in Clause 4.1 of [6, TS 38.213].

- Default beam of aperiodic CSI-RS resource should be defined per resource set, when a DCI triggers two aperiodic CSI-RS resource sets.

Support the following TP.

Reason for change:

For mTRP beam report, to reduce UCI payload, the CRI(s) are not needed to report when the corresponding resource set is configured with repetition “on”. However, in the current spec version, at least two CRIS/SSBRIs are reported in any case.

Summary of change:

In TS38.212 section 6.3.1.1.2, add “if reported” in the first two rows of the CRIs/SSBRIs.

Consequences if not approved:

More CRIs will be reported but meaningless, and low resource efficiency.

TS38.212 6.3.1.1.2 CSI only Table 6.3.1.1.2-8B: Mapping order of CSI fields of one report for group-based CRI/RSRP or SSBRI/RSRP reporting CSI report number; CSI fields. CSI report #n; Resource set indicator. CRI or SSBRI #1 of 1st resource group as in Table 6.3.1.1.2-6, if reported. CRI or SSBRI #2 of 1st resource group as in Table 6.3.1.1.2-6, if reported. CRI or SSBRI #1 of 2nd resource group as in Table 6.3.1.1.2-6, if reported. CRI or SSBRI #2 of 2nd resource group as in Table 6.3.1.1.2-6, if reported. CRI or SSBRI #1 of 3rd resource group as in Table 6.3.1.1.2-6, if reported. CRI or SSBRI #2 of 3rd resource group as in Table 6.3.1.1.2-6, if reported. CRI or SSBRI #1 of 4th resource group as in Table 6.3.1.1.2-6, if reported. CRI or SSBRI #2 of 4th resource group as in Table 6.3.1.1.2-6, if reported. RSRP of CRI or SSBRI #1 of 1st resource group as in Table 6.3.1.1.2-6. Differential RSRP of CRI or SSBRI #2 of 1st resource group as in Table 6.3.1.1.2-6. Differential RSRP of CRI or SSBRI #1 of 2nd resource group as in Table 6.3.1.1.2-6, if reported. Differential RSRP of CRI or SSBRI #2 of 2nd resource group as in Table 6.3.1.1.2-6, if reported. Differential RSRP of CRI or SSBRI #1 of 3rd resource group as in Table 6.3.1.1.2-6, if reported. Differential RSRP of CRI or SSBRI #2 of 3rd resource group as in Table 6.3.1.1.2-6, if reported. Differential RSRP of CRI or SSBRI #1 of 4th resource group as in Table 6.3.1.1.2-6, if reported. Differential RSRP of CRI or SSBRI #2 of 4th resource group as in Table 6.3.1.1.2-6, if reported.

Send LS to RAN on RAN1 conclusion on optionally configure TRP-specific NBI-RS, rather than always configured.

For inter-cell TRP-specific BFD-RS configuration and update, support the following TP.

Reason for change:

In current spec, only the association between BFD-RS/NBI-RS set and PCI has been captured and the associated PCI is configured by RRC. Therefore, the PCI configured to be associated with the BFD-RS set/NBI-RS set may be different from the PCI involved in the indicated TCI state.

Summary of change:

In TS38.213 section 6, clarify the association between BFD-RS/NBI-RS set, CORESETPoolindex and PCI when two BFD-RS/NBI-RS set are configured and one BFD-RS/NBI-RS set associates with a PCI different from that of the serving cell.

Consequences if not approved:

It can cause misalignment between the PCI associated with the BFD-RS/NBI-RS set and PCI involved in the indicated TCI state, which may lead to the new beam(s) found by UE cannot use to recover the failed link.

TS38.213 6 Link recovery procedures < Unchanged parts are omitted > If a UE - is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs on an active DL BWP of a serving cell, - is provided coresetPoolIndex with a value of 1 for second CORESETs on the active DL BWP of the serving cells, and - is provided SSB-MTCAdditionalPCI SS/PBCH block indexes other than the one provided by physCellId in ServingCellConfigCommon can be associated with either q1,0 or q1,1 set and the corresponding q0,0 or q0,1 set is associated with the physical cell identity. In this case, the physical cell ID associated with the SS/PBCH block indexes is the same as the physical cell ID associated with the indicated TCI state for the second CORESETs.

For update of QCL assumption after receiving the BFR response for inter-cell MTRP cases,

- Support to update QCL assumption of CORESETs associated with the serving cell PCI and scheduled PDSCH by the spatial filter as for the last PRACH transmission simultaneously.

When SFN PDSCH is configured by RRC, enableTwoDefaultTCI-States is configured and there is no TCI codepoint with two TCI states activated by MAC CE, if UE is capable of the dynamic switching between STRP and SFN transmission, the time offset between the reception of the DCI and its scheduled PDSCH is less than the threshold timeDurationForQCL, the default TCI state for PDSCH can be determined as follow.

If enhanced SFN PDCCH transmission scheme 1 or TRP-based pre-compensation is configured and the lowest CORESET ID in the latest slot is indicated with two TCI states, UE applies both TCI states of the CORESET

Otherwise, UE applies the one active TCI state of the CORESET with the lowest ID in the latest slot

When SFN PDSCH is configured, and the time offset between the reception of the DCI and its scheduled PDSCH is less than the threshold timeDurationForQCL, UE expects enableTwoDefaultTCI-States is configured.

UE supporting SFN PDSCH is mandated to report FG 16-2b-0

When SFN PDSCH is configured by RRC, if applicable the time offset between the reception of the DCI format 1_0 and the corresponding PDSCH is equal to or larger than the threshold timeDurationForQCL,

If SFN PDCCH is not configured by RRC and UE is capable of dynamic switching between STRP and SFN transmission, UE applies the one active TCI state of the CORESET that schedules the PDSCH

If UE is not capable of dynamic switching between STRP and SFN transmission, UE expects SFN PDCCH is configured by RRC and the CORESET that schedules PDSCH by DCI format 1_0 should be activated with two TCI states

Support to keep up to two BFD RS for implicit BFD RS configuration for both CORESETs with both single and two TCI states.

Support reusing the existing NBI-RS configuration without NBI-RS pair when SFN transmission is configured in Rel-17.

If Type0/0A/1/2-PDCCH CSS are associated with CORESET activated with two TCI states, the first TCI state is applied for the PDCCH reception.

Support the following TP.

Reason for change:

The combination of Rel-17 SFN PDCCH scheme 1 and single-TRP PDSCH is supported as an optional UE feature [nonSfnPdsch-sfnPdcch]. Meanwhile, the combination of Rel-17 SFN PDCCH TRP-based pre-compensation and single-TRP PDSCH is not supported. However, in the current spec TS 38.214, the description of these mentioned combinations has not been captured.

Summary of change:

To describe the combination of SFN PDCCH scheme 1 and single-TRP PDSCH is supported, and the combination of SFN PDCCH TRP-based pre-compensation and single-TRP PDSCH is not supported.

Consequences if not approved:

It would affect UE’s reception process and increase the complexity of UE.

TS 38.214 5.1 UE procedure for receiving the physical downlink shared channel < Unchanged parts are omitted > When a UE is configured with higher layer parameter sfnSchemePdsch set to either 'sfnSchemeA' or 'sfnSchemeB' for a DL BWP and - if the UE reports its capability of [dynamicSFN], the UE is indicated with one or two TCI state(s) in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2, or - otherwise, the UE is not expected to be indicated with one TCI state per any of TCI codepoint by MAC CE, and the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2, and the UE procedure for receiving the PDSCH upon detection of a PDCCH follows clause 5.1 and the QCL assumption for the PDSCH as defined in clause 5.1.5. When a UE is configured with both sfnSchemePdsch and sfnSchemePdcch, the UE shall expect that sfnSchemePdsch and sfnSchemePdcch are set to the same scheme, either 'sfnSchemeA' or 'sfnSchemeB'. If a UE is configured with sfnSchemePdcch set to 'sfnSchemeA' for a DL BWP and activated with two TCI states by MAC CE, and the UE does not report its capability of [nonSfnPdsch-sfnPdcch], the UE does not expect to be indicated with one TCI state in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2. If a UE is configured with sfnSchemePdcch set to 'sfnSchemeB' for a DL BWP and activated with two TCI states by MAC CE, the UE does not expect to be indicated with one TCI state in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2. < Unchanged parts are omitted >

References

Chair's Notes RAN1#108-e.

3GPP TS 38.212, V17.1.0, (Release 17) 2022-3.

3GPP TS 38.213, V17.1.0, (Release 17) 2022-3.

3GPP TS 38.214, V17.1.0, (Release 17) 2022-3.