3GPP TSG RAN WG1 #109-e	R1-2203507

e-Meeting, May 9th – 20th, 2022

Source: 	vivo

Title:	Miscellaneous corrections on Rel-17 MIMO

Agenda Item:	8.1.3

Document for:	Discussion and Decision

Introduction

In this paper, our views on remaining issues and further corrections on CSI enhancements and SRS enhancement are provided.

Remaining issues of CSI enhancements for MTRP

In RAN1#103-e [2], following agreements were reached on reduction of work load.

Agreement For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, [at least for multi-DCI based and single-DCI based schemes (scheme 1a)], NZP CSI-RS resources for channel measurement are associated to different TRPs/TCI states at resource level - CMRs corresponding to different TRPs respectively shall be configured within the same resource set (i.e. scheme 1-2) and have the same number of ports among CMRs. - At least ‘typeI-SinglePanel’ codebook is supported - FFS: Other codebook types - Note that RAN1 shall strive to finalize NCJT CSI enhancement with single reporting setting firstly. - The support of larger than 32 ports across two CMRs is optional for a UE supporting Rel. 17 mTRP CSI

For CMR configuration, we had the following restrictions

R1: CMRs corresponding to different TRPs respectively were configured within the same resource set.

R2: CMRs corresponding to different TRPs respectively had the same number of ports among CMRs.

R3: The support of larger than 32 ports across two CMRs is optional for a UE supporting Rel-17 mTRP CSI.

R1 and R3 had been captured clearly in the specification. However, R2 may have been overlooked and is not captured in this statement.

Therefore, we have following text proposal.

Adopt following text proposal in 5.1.4.2 in TS 38.214.

Reason for change:

The restriction “CMRs corresponding to different TRPs respectively had the same number of ports among CMRs” is not captured.

Summary of change:

Add a bullet “Ks resources in the corresponding NZP-CSI-RS-ResourceSet for channel measurement have the same number of ports.”

Consequences if not approved:

The restriction “CMRs corresponding to different TRPs respectively had the same number of ports among CMRs” is missing.

5.1.4.2 Report Quantity Configurations <Unchanged parts are omitted> If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', or 'cri-RI-LI-PMI-CQI' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is configured with Ks≥2 resources, two Resource Groups with K1≥1 resources in Group 1, K2≥1 resources in Group 2, K1+K2=Ks, and N Resource Pairs: - each resource can contain, subject to UE capability, at most 32 CSI-RS ports. - Ks resources in the NZP-CSI-RS-ResourceSet for channel measurement have the same number of ports. - each of the N Resource Pairs is associated to a CRI value. - The CSI-ReportConfig may be configured with higher layer parameter sharedCMR. M1 and M2 are the numbers of resources associated to a CRI value, other than the N CRIs defined above, in Group 1 and Group 2, respectively, with M=M1+M2, such that the total number of CRI values configured for the CSI-ReportConfig is M+N. - If the higher layer parameter csi-ReportMode is set to 'Mode1' and the higher layer parameter numberOfSingleTRP-CSI-Mode1 is set to X∈{0}, M1=M2=0; otherwise, - if the higher layer parameter csi-ReportMode is set to 'Mode1' and the higher layer parameter numberOfSingleTRP-CSI-Mode1 is set to X∈{1,2}, or if csi-ReportMode is set to 'Mode2', - if sharedCMR is configured: M1=K1 and M2=K2; otherwise - if sharedCMR is not configured, only the resources in Group 1 and Group 2 that are not referred to in any Resource Pair are associated to M CRI values other than the N CRIs defined above. <Unchanged parts are omitted>

- Besides, RAN 1 has never discussed whether n1-n2 for Type I codebook needs to be the same or not for different TRPs. From latest version of RRC configuration, typeI-SinglePanel1-r17 and typeI-SinglePanel2-r17 for two CMRs in a CMR pair are separately configured, which implies no restrictions on  n1-n2 between two codebook configurations for NC-JT. We are fine with current RRC configurations and prefer to draw following conclusion.

There are no restrictions on n1-n2 between two codebook configurations for NC-JT.

Remaining issues of SRS enhancements

SRS switching enhancement for 1T2R/2T4R

Two agreements related to 1T2R/2T4R antenna switching enhancement were agreed as below:

Agreement: For extension of aperiodic antenna switching SRS configurations for <=4Rx, support N=4 for 1T4R and N=2 for 1T2R/2T4R. - The above extension is UE optional Agreement For antenna switching SRS, support maximum one SRS resource set for periodic SRS and maximum 2 SRS resource sets for semi-persistent SRS. - Note: the two SP-SRS resource sets are not activated at the same time - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - Applies for all supported xTyR where y<=8 - For each xTyR antenna switching (except for 4T6R if supported), each periodic or semi-persistent resource set contains y/x resources. This feature is UE optional: For UEs that do not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS Additionally, it was agreed to enhance the number of max SRS sets for periodic SRS and semi-persistent SRS.

Thus, based on the above agreements which indicate two separate features, we have four possible combinations between feature A (extension of aperiodic antenna switching SRS configurations for <=4Rx) and feature B (extension of periodic and semi-persistent SRS),

Combination 1: {not support feature A, not support feature B}

Combination 2: {support feature A, support feature B}

Combination 3: {support feature A, not support feature B}

Combination 4: {not support feature A, support feature B}

And we have following table for combinations of different features:

Combinations of different features for 1T2R and 2T4R

| The number of max sets of 1T2R/2T4R |  | Feature B (SP/P SRS sets) |  |
| --- | --- | --- | --- |
|  |  | Not support | Support |
| Feature A (AP SRS sets) | Not support | 1P+1SP or 1P+1AP or 1SP+1AP Combination 1 | Option 1: 1P+2SP+0AP Option 2: 1P+2SP+1AP Combination 4 |
|  | Support | Option 1: 1P+1SP+2AP Option 2: 1P/1SP+2AP Option 3: 0P/SP+2AP Combination 3 | 2SP + 1P + 2AP Combination 2 |

For combination 1, the SRS configurations should follow Rel-15 on the number of resource sets for different resource types, while the number of max resource sets of aperiodic and periodic/semi-persistent SRS follow feature A and feature B simultaneously for combination 2, i.e. 2 semi-persistent SRS resource sets + 1 periodic SRS resource set + 4 aperiodic SRS resource sets for 1T4R and 2 aperiodic SRS resource sets for 1T2R/2T4R. In other words, there is no ambiguity on supported maximum SRS resource sets and resources configuration for antenna switching usage for both combination 1 and combination 2.

For 1T2R/2T4R, Rel-15/16 specification limits the number of total SRS resource sets to 2 and the number of max SRS resource set per resource type to 1, i.e. 1 periodic + 1 semi-persistent, or 1 periodic + 1 aperiodic, or 1 semi-persistent + 1 aperiodic. However, the max number of SRS resource sets configurable for 1T2R/2T4R when either combination 3 or combination 4 is supported is not clear.

More specifically, if combination 3 is supported, it represents maximum 2 aperiodic SRS resource sets can be configured as feature A supported, and, due to feature B not supported, periodic and semi-persistent SRS resource set configurations should follow Rel-15 rule. These ambiguous statements could have been understood in more than one way as shown in table 1. For example, 1 periodic SRS resource set and/or 1 semi-persistent SRS resource set + 2 aperiodic SRS resource set may be configurated simultaneously for 1T2R/2T4R with supporting combination 3 (option 1 or option 2), or maybe only maximum 2 aperiodic SRS resource sets can be configured (option 3).

Similarly, for combination 4, it supports maximum 1 SRS resource set for periodic SRS and maximum 2 SRS resource sets for semi-persistent SRS, while an SRS resource configuration of 2 semi-persistent + 1 aperiodic is supported there could be following cases: support 1 periodic + 2 semi-persistent + 0 aperiodic (option 1 in table 1) or 1 periodic + 2 semi-persistent + 1 aperiodic (option 2 in table 1).

Below is excerpt from 38.214 v17.1.0 which describes SRS antenna switching for 1T2R:

-	For 1T2R, if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets] and/or a capability for [extension of aperiodic antenna switching SRS configuration]:

-	zero or one or two SRS resource sets configured with different value of resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' if the UE is not indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and up to one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. An SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the second resource in the set is associated with a different UE antenna port than the SRS port of the first resource in the same set, and

-	zero or one SRS resource set with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is not indicating a capability for [extension of aperiodic antenna switching SRS configuration], or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is indicating a capability for [extension of aperiodic antenna switching SRS configuration], where in the case of one resource set, a total of two SRS resources transmitted in different symbols of one slot and where the SRS port of the second resource in the given set is associated with a different UE antenna port than the SRS port of the first resource in the same set. In the case of two resource sets, a total of two SRS resources transmitted in different symbols of two different slots, and where the SRS port of each SRS resource in the given two sets is associated with a different UE antenna port. The two sets are each configured with one SRS resource, or

-	otherwise, for 1T2R, up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet set, where each set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the second resource in the set is associated with a different UE antenna port than the SRS port of the first resource in the same set, or

According to 38.214 v17.1.0, option 1 is supported, i.e. 1P+1SP+2AP, for combination 3 (yellow highlighted part) and option 2, i.e. 1P+2SP+1AP, for combination 4 (green highlighted part). However, there is no agreement in RAN1 on the number of maximum configured SRS resource sets with antenna switching usage enhanced to 4 for 1T2R and 2T4R. Thus, we propose to clarify in the spec that the maximum number of SRS resource sets configurations with different resource types for 1T2R/2T4R is 3 if only one of the two features is supported.

Clarify in the spec that maximum number of 3 SRS resource sets can be configured with different resource types for 1T2R/2T4R if only one antenna switching enhancement feature relative to 1T2R/2T4R is supported.

Adopt the following TP to section 6.2.1.2 of TS 38.214.

Reason for change:

There is no agreement in RAN1 on the number of maximum configured SRS resource sets with antenna switching usage enhanced to 4 for 1T2R and 2T4R.

Summary of change:

Suggest to limit the maximum number of SRS resource sets for 1T2R and 2T4R to 3 if only one antenna switching enhancement feature relative to 1T2R/2T4R is supported.

Consequences if not approved:

Force to support maximum of 4 SRS resource sets which is not approved in any RAN 1 agreement.

6.2.1.2 UE sounding procedure for DL CSI acquisition <Unchanged parts are omitted> - For 1T2R, if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets] and/or a capability for [extension of aperiodic antenna switching SRS configuration]: - up to 3 SRS resource sets configured across different value combinations for the higher layer parameter resourceType in SRS-ResourceSet if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets] or a capability for [extension of aperiodic antenna switching SRS configuration], and - zero or one or two SRS resource sets configured with different value of resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' if the UE is not indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and up to one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. An SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the second resource in the set is associated with a different UE antenna port than the SRS port of the first resource in the same set, and - zero or one SRS resource set with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is not indicating a capability for [extension of aperiodic antenna switching SRS configuration], or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is indicating a capability for [extension of aperiodic antenna switching SRS configuration], where in the case of one resource set, a total of two SRS resources transmitted in different symbols of one slot and where the SRS port of the second resource in the given set is associated with a different UE antenna port than the SRS port of the first resource in the same set. In the case of two resource sets, a total of two SRS resources transmitted in different symbols of two different slots, and where the SRS port of each SRS resource in the given two sets is associated with a different UE antenna port. The two sets are each configured with one SRS resource, or - otherwise, for 1T2R, up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet set, where each set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the second resource in the set is associated with a different UE antenna port than the SRS port of the first resource in the same set, or - For 2T4R, if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets] and/or a capability for [extension of aperiodic antenna switching SRS configuration]: - up to 3 SRS resource sets configured across different value combinations for the higher layer parameter resourceType in SRS-ResourceSet if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets] or a capability for [extension of aperiodic antenna switching SRS configuration], and - zero or one or two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' if the UE is not indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and up to one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource, and, - zero or one SRS resource set configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is not indicating a capability for [extension of aperiodic antenna switching SRS configuration], or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is indicating a capability for [extension of aperiodic antenna switching SRS configuration], where in the case of one resource set, a total of two SRS resources transmitted in different symbols in the same slot, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource. In the case of two resource sets, a total of two SRS resources transmitted in different symbols of two different slots, and where the SRS port pair of each SRS resource in the given two sets is associated with a different UE antenna port pair. The two sets are each configured with one SRS resource, or - otherwise, up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet, where each SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource, or <Unchanged parts are omitted>

SRS switching enhancement for up to 8 antennas

In RAN1#106bis-e meeting, following agreement was made:

Agreement For two SRS resource sets of an xTyR antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, a minimum gap period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot - The value of Y is same as the inter-resource GP defined in Rel-15 - FFS: Whether or not the minimum GP exists can be RRC configurable subject to UE capability - Whether this inter-set GP is needed for 4T6R can be discussed later per the decision on 4T6R configuration. - FFS: How/Whether to handle the case where the interval between SRS resource sets is larger than Y

And in RAN1#108-e meeting, following agreement was made:

Agreement For inter-set guard period, UE does not transmit any other signal on any symbols of the interval if the interval between SRS resource sets is Y symbols. - When both the SRS resource on all of the corresponding symbols prior to the gap and the SRS resource on all of the corresponding symbols after the gap are dropped due to collision handling, the gap period is also dropped with same priority and can be used for UL transmission. - The above is the only collision handling rule to be introduced in Rel-17 for antenna switching guard period

Large symbol gap between two SRS resource in adjacent slots

In this section, we will discuss remaining issues on inter-set guard period for antenna switching. The previous agreement describes a special case of inter-set gap = minimal gap of Y symbols. However, consider another case (inter-set gap > Y symbols) as shown in Figure 1, where one resource in the first set is configured on the last symbol in the first slot and another resource in the second set is configured on the last symbol in the second slot, hence the gap between two SRS sets is equal to 13 symbols. Besides, according priority rule, the gap between two SRS with antenna switching usage has same priority as SRS, which could lead to some scheduling restriction in the gap symbols. Other UL signals may not be transmitted on symbols between two antenna switching SRS resources defined as guard symbols or SRS is dropped in second slot according to priority rule which leads to wastage of resources.

According to 38.214 v17.1.0 which describes SRS antenna switching for guard period:

The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. For two SRS resource sets of an antenna switching located in two consecutive slots, if UE is capable of transmitting SRS in all symbols in one slot, a guard period of Y symbols exists between the last OFDM symbol occupied by the SRS resource set in the first slot and the first OFDM symbol occupied by the SRS resource set in the second slot.

From above yellow highlighted text, it can be found that inter-set guard period is defined as Y symbols where Y is minimal guard period and specified with SCS configuration, while legacy guard period is in-between SRS resources of one set in a slot shown in green highlighted text. Thus, symbol location of inter-set guard period is not specified and it may cause some ambiguities between network data scheduling and UE antenna switching processing in the period of the two SRS resource sets.

From above analysis, symbol position of inter-set guard period should be specified, and we propose the inter-set guard period can be configured by signaling or Y symbols immediately following the first SRS resource set in-between two SRS resource sets. In addition, if antenna switching SRS resource and PUSCH/PUCCH are transmitted on different antennas, it is necessary to configure guard symbols in between. We, thus, propose,

Symbol position of the minimal guard symbol Y can be configured by signaling or Y symbols immediately following the first SRS resource set in-between two SRS resource sets.

Guard period between antenna switching SRS resource and other UL signals is needed.

Collision handling for aperiodic SRS triggering

Collision with different offset indication

If more than one aperiodic SRS sets are triggered at the same time by one DCI with same available slot offset indication, the SRS resources in these sets should be configured in non-overlapping symbol(s) if the SRS resources are transmitted in same slot, which is guaranteed by network configuration. However, when available valid slot offset is indicated for each aperiodic SRS resource set triggered by one DCI, it is possible that transmission of 2 or more SRS resource sets are delayed such that SRS resources in multiple sets fall in one slot. For example, as depicted in Figure 2, legacy slot offset is configured as 0, and available slot offset is equal to 2 and 1, which indicates the 2nd available slot and the 1st available slot after slot n + legacy slot offset (‘0’), for the SRS resource set 1 and SRS resource set 2, respectively. If the set of symbols in slot n+1 are configured as flexible and indicated as DL + UL, by definition slot n+1 and slot n+2 are available slots thus according to available slot offset the UE is supposed to transmit SRS set 1 in slot n+2. By contrast, slot n+2 is the first available slot for SRS set 2 since the set of DL symbols in slot n+1 is overlapped with the symbols for SRS set 1 transmission. Thus, SRS set 1 and SRS set 2 are overlapping on symbol level, even if different available slot offset is configured.

Even with different available slot offset indication for two aperiodic SRS resource sets triggered by one or more DCI, collision between two SRS can happen.

Collision with two SRS on different CCs configured with different offset indication

In UL CA, for SRS for antenna switching + SRS for codebook/non-codebook/BM/antenna switching case, the simultaneous transmission of SRS on different CCs depends on UE capability. Thus, in UL CA, collision between two SRS from different carriers is possible when simultaneous transmission of those two SRS on different CCs is not supported. Taking figure 3 as an example, SRS set 1 is transmitted on slot n+2 on CC1 and SRS set 2 is also transmitted on slot n+2 on CC2 even with different available slot offset indication between two SRS.

SRS transmission collision may happen for two aperiodic SRS resource sets triggered in case of UL CA, even with different available slot offset indication, if simultaneous transmission of SRS on difference CCs is not supported.

To address above scenarios, some principle of priority to handle collision in an available slot should be considered.

Although priority can be based on Usage or the order of triggering DCI, it cannot solve collision if multiple sets configured with same usage or simultaneously triggered by multiple DCI are overlapping in time domain across CCs. Thus, as single dropping rule is not sufficient, we propose a combined dropping rule, including usage, order of triggering DCI, CC ID and set ID, to handle collision.

Consider a dropping rule which considers usage, order of triggering DCI, CC ID and set ID, to resolve SRS collision when more than one SRS resource sets are triggered by one or more DCI on same CC or different CCs.

Conclusions

- In this contribution, we analyze potential enhancements of multi-beam operation in Rel-17. To summarize, we have following conclusion, proposals.

- There are no restrictions on n1-n2 between two codebook configurations for NC-JT.

- Adopt following text proposal in 5.1.4.2 in TS 38.214.

Reason for change:

The restriction “CMRs corresponding to different TRPs respectively had the same number of ports among CMRs” is not captured.

Summary of change:

Add a bullet “Ks resources in the corresponding NZP-CSI-RS-ResourceSet for channel measurement have the same number of ports.”

Consequences if not approved:

The restriction “CMRs corresponding to different TRPs respectively had the same number of ports among CMRs” is missing.

------------------------------------------Start of Text Proposal ---------------------------------- 5.1.4.2 Report Quantity Configurations < Unchanged parts are omitted > If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', or 'cri-RI-LI-PMI-CQI' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is configured with Ks≥2 resources, two Resource Groups with K1≥1 resources in Group 1, K2≥1 resources in Group 2, K1+K2=Ks, and N Resource Pairs: - each resource can contain, subject to UE capability, at most 32 CSI-RS ports. - Ks resources in the NZP-CSI-RS-ResourceSet for channel measurement have the same number of ports. - each of the N Resource Pairs is associated to a CRI value. - The CSI-ReportConfig may be configured with higher layer parameter sharedCMR. M1 and M2 are the numbers of resources associated to a CRI value, other than the N CRIs defined above, in Group 1 and Group 2, respectively, with M=M1+M2, such that the total number of CRI values configured for the CSI-ReportConfig is M+N. - If the higher layer parameter csi-ReportMode is set to 'Mode1' and the higher layer parameter numberOfSingleTRP-CSI-Mode1 is set to X∈{0}, M1=M2=0; otherwise, - if the higher layer parameter csi-ReportMode is set to 'Mode1' and the higher layer parameter numberOfSingleTRP-CSI-Mode1 is set to X∈{1,2}, or if csi-ReportMode is set to 'Mode2', - if sharedCMR is configured: M1=K1 and M2=K2; otherwise - if sharedCMR is not configured, only the resources in Group 1 and Group 2 that are not referred to in any Resource Pair are associated to M CRI values other than the N CRIs defined above. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ------------------------------------

Clarify in the spec that maximum number of 3 SRS resource sets can be configured with different resource types for 1T2R/2T4R if only one antenna switching enhancement feature relative to 1T2R/2T4R is supported.

Adopt the following TP to section 6.2.1.2 of TS 38.214.

Reason for change:

There is no agreement in RAN1 on the number of maximum configured SRS resource sets with antenna switching usage enhanced to 4 for 1T2R and 2T4R.

Summary of change:

Suggest to limit the maximum number of SRS resource sets for 1T2R and 2T4R to 3 if only one antenna switching enhancement feature relative to 1T2R/2T4R is supported.

Consequences if not approved:

Force to support maximum of 4 SRS resource sets which is not approved in any RAN 1 agreement.

6.2.1.2 UE sounding procedure for DL CSI acquisition <Unchanged parts are omitted> - For 1T2R, if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets] and/or a capability for [extension of aperiodic antenna switching SRS configuration]: - up to 3 SRS resource sets configured across different value combinations for the higher layer parameter resourceType in SRS-ResourceSet if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets] or a capability for [extension of aperiodic antenna switching SRS configuration], and - zero or one or two SRS resource sets configured with different value of resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' if the UE is not indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and up to one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. An SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the second resource in the set is associated with a different UE antenna port than the SRS port of the first resource in the same set, and - zero or one SRS resource set with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is not indicating a capability for [extension of aperiodic antenna switching SRS configuration], or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is indicating a capability for [extension of aperiodic antenna switching SRS configuration], where in the case of one resource set, a total of two SRS resources transmitted in different symbols of one slot and where the SRS port of the second resource in the given set is associated with a different UE antenna port than the SRS port of the first resource in the same set. In the case of two resource sets, a total of two SRS resources transmitted in different symbols of two different slots, and where the SRS port of each SRS resource in the given two sets is associated with a different UE antenna port. The two sets are each configured with one SRS resource, or - otherwise, for 1T2R, up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet set, where each set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the second resource in the set is associated with a different UE antenna port than the SRS port of the first resource in the same set, or - For 2T4R, if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets] and/or a capability for [extension of aperiodic antenna switching SRS configuration]: - up to 3 SRS resource sets configured across different value combinations for the higher layer parameter resourceType in SRS-ResourceSet if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets] or a capability for [extension of aperiodic antenna switching SRS configuration], and - zero or one or two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' if the UE is not indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and up to one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' if the UE is indicating a capability for [maximum 2 semi-persistent and maximum 1 periodic SRS resource sets], where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource, and, - zero or one SRS resource set configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is not indicating a capability for [extension of aperiodic antenna switching SRS configuration], or up to two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic' if the UE is indicating a capability for [extension of aperiodic antenna switching SRS configuration], where in the case of one resource set, a total of two SRS resources transmitted in different symbols in the same slot, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource. In the case of two resource sets, a total of two SRS resources transmitted in different symbols of two different slots, and where the SRS port pair of each SRS resource in the given two sets is associated with a different UE antenna port pair. The two sets are each configured with one SRS resource, or - otherwise, up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet, where each SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource, or <Unchanged parts are omitted>

Symbol position of the minimal guard symbol Y can be configured by signaling or Y symbols immediately following the first SRS resource set in-between two SRS resource sets.

Guard period between antenna switching SRS resource and other UL signals is needed.

Consider a dropping rule which considers usage, order of triggering DCI, CC ID and set ID, to resolve SRS collision when more than one SRS resource sets are triggered by one or more DCI on same CC or different CCs.

References

Chairman’s notes for 3GPP TSG RAN WG1 #108-e Meeting.

Chairman’s notes for 3GPP TSG RAN WG1 #103-e Meeting.

3GPP TS 38.214 V17.1.0.