3GPP TSG RAN WG1 #109-e		R1- 2203508

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Maintenance on initial access for NR operation from 52.6GHz to 71GHz

Agenda Item:	8.2.1

Document for:	Discussion and Decision

- Introduction

The following objective is included in the revised work item [1]:

  - In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported.

Note: Except for timing line related aspects, a common design framework shall be adopted for 480kHz to 960kHz

  - Time line related aspects adapted to 480kHz and 960kHz, e.g., BWP and beam switching timeing, HARQ timing, UE processing, preparation and computation timelines for PDSCH, PUSCH/SRS and CSI, respectively.

  - Support of up to 64 SSB beams for licensed and unlicensed operation in this frequency range.

  - Supports 120kHz SCS for SSB and 120kHz SCS for initial access related signals/channels in an initial BWP.

    - Study and specify, if needed, additional SCS (480kHz, 960kHz) for SSB for cases other than initial access.

    - Note: coverage enhancement for SSB is not pursued.

  - In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints:

    - Limited sync raster entry numbers

      - It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n257 is 599). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz.

    - only 480kHz CORESET#0/Type0-PDCCH SCS supported for 480 kHz SSB SCS.

    - Prioritize support SSB-CORESET#0 multiplexing pattern 1. Other patterns discussed on a best effort basis.

    - 960 kHz numerology for the SSB is not supported by the UE for initial access in Rel-17.

    - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible

    - Note: 480 kHz is an optional SSB numerology for initial access for the UE. A UE supporting a band in 52.6-71 GHz must at least support 120 kHz SCS (for initial access and after initial access)

    - Note: Dependency or lack thereof for a UE supporting 480kHz and/or 960kHz numerology for data and control to also support 480kHz SSB numerology for initial access is to be tackled as part of UE capability discussion.

  - Support ANR and PCI confusion detection for 120, 480 and 960kHz SCS based SSB, support CORESET#0/Type0-PDCCH configuration in MIB of 120, 480 and 960kHz SSB

    - FFS: additional method(s) to enable support to obtain neighbour cell SIB1 contents related to CGI reporting

    - Only 1 CORESET#0/Type0-PDCCH SCS supported for each SSB SCS, i.e., (120, 120), (480, 480) and (960, 960).

    - Prioritize support SSB-CORESET#0 multiplexing pattern 1. Other patterns discussed on a best effort basis.

    - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible

    - Note: From UE perspective, ANR detection for 480/960kHz SCS based SSB is not supported if the UE does not support 480/960 SCS for SSB.

    - Note: for ANR, when reading the MIB, the cell containing the SSB is known to the UE, as defined in 38.133 specification.

  - Specify timing associated with beam-based operation to new SCS (i.e., 480kHz and/or 960kHz), study, and specify if needed, potential enhancement for shared spectrum operation

    - Rel-15/16 and any Rel-17 beam management enhancements can be considered for 52.6-71 GHz. Whether particular features should be excluded for 52.6-71 GHz can be further discussed

    - Note: As per usual procedure, duplication of work between work items in Rel-17 should be avoided

  - Specify support for PRACH sequence lengths (i.e. L=139, L=571 and L=1151) and study, if needed, specify support for RO configuration for non-consecutive RACH occasions (RO) in time domain for operation in shared spectrum

In this contribution, we will discuss the remaining issues of initial access aspects which is the RB offsets for Coreset#0.

- RB offsets for Coreset#0

In RAN1#108-e meeting [2], Type0 PDCCH design was discussed and the following agreements were agreed:

Working Assumption

- The following table is used for set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120}, {480, 480}, and {960, 960} kHz for FR2-2.

  - FFS: whether/how to define X, if defined, candidate values {56, 76}

- Text Proposal #4-2D (for 38.213, Section 13) in section 3 of R1-2202503 is endorsed

- Note: this working assumption can be revisited once RAN4 finalizes the further details of the channelization.

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 48 | 1 | 0 |
| 3 | 1 | 48 | 1 | 14 |
| 4 | 1 | 48 | 1 | 28 |
| 5 | 1 | 48 | 2 | 0 |
| 6 | 1 | 48 | 2 | 14 |
| 7 | 1 | 48 | 2 | 28 |
| 8 | 1 | 96 | 1 | 0 |
| 9 | 1 | 96 | 1 | X |
| 10 | 1 | 96 | 2 | 0 |
| 11 | 1 | 96 | 2 | X |
| 12 | 3 | 24 | 2 | -20 if kSSB=0 -21 if kSSB > 0 |
| 13 | 3 | 24 | 2 | 24 |
| 14 | 3 | 48 | 2 | -20 if kSSB=0 -21 if kSSB > 0 |
| 15 | 3 | 48 | 2 | 48 |

For Type 0 PDCCH related design, one remaining issue needs to be discussed, which is the RB offsets for Coreset#0 for different SCS. The RB offset design is highly depending on the sync and channel raster design. According to the agreements in RAN4 [3], different sync raster design will be applied for licensed band and unlicensed band, i.e. fixed sync raster for unlicensed band and floating sync raster for licensed band. However, the exact value of GSCN and ARFCN have not yet been determined. Thus, it is difficult for RAN1 to confirm the exact RB offset value design. Considering RAN 1 spec completeness, we suggest providing exact values for RB offsets instead of using X in spec. Finally, the whole working assumption can be revised later based on the final RAN4 decision.

In the working assumption, almost all the value of RB offsets has been determined, except that for 96 RBs. And two candidate values {56, 76} have been provided. For candidate value 56, which means that the frequency domain locations of SSB are fully overlapping with the center part of Coreset#0. If a SSB and its associated Type 0 PDCCH are in the same slot, the RB allocation for SIB1 is limited (i.e. upper or lower part relevant to SSB) since it needs to avoid overlapping with SSB in frequency domain when the time domain allocation for SIB1 PDSCH is overlapping with SSB. To ensure the reliability of SIB 1, it is better to allocate more frequency domain resources. Therefore, we prefer to use 76 for RB offsets in 96 RBs.

Proposal 1: Support RB offset as 76 for 96-RB coreset#0 case and adopt TP #1 in Section 13 for TS 38.213.

Based on the above proposals for Coreset#0 design, we propose the following TP1:

------------------------------------------------- TP #1 for 38213 section 13 [4]--------------------------------------------

<unchanged part omitted>

Table 13-10A/B/C: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120}, {480, 480}, and {960, 960} kHz for FR2-2

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 48 | 1 | 0 |
| 3 | 1 | 48 | 1 | 14 |
| 4 | 1 | 48 | 1 | 28 |
| 5 | 1 | 48 | 2 | 0 |
| 6 | 1 | 48 | 2 | 14 |
| 7 | 1 | 48 | 2 | 28 |
| 8 | 1 | 96 | 1 | 0 |
| 9 | 1 | 96 | 1 | X76 |
| 10 | 1 | 96 | 2 | 0 |
| 11 | 1 | 96 | 2 | X76 |
| 12 | 3 | 24 | 2 | -20 if kSSB=0 -21 if kSSB > 0 |
| 13 | 3 | 24 | 2 | 24 |
| 14 | 3 | 48 | 2 | -20 if kSSB=0 -21 if kSSB > 0 |
| 15 | 3 | 48 | 2 | 48 |

<unchanged part omitted>

------------------------------------------------------END OF TP #1-------------------------------------------------------------

- Conclusion

In this contribution, we focus on the required changes to NR using existing NR waveform, and have the following observations and proposals:

Proposal 1: Support RB offset as 76 for 96-RB coreset#0 case and adopt TP #1 in Section 13 for TS 38.213.

References

- RP-211584, RAN #92-e, Electronic Meeting, June 14-18, 2021.

- 3GPP RAN1#108-e, Chairman’s notes.

- R4-2200081, LS for the channelization for up to 71 GHz.

- TS 38.213 v17.0.0, NR Physical layer procedures for control, 2021-12.

Appendix 1: Draft Rel-17 CR

| CHANGE REQUEST |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.213 | CR | - | rev | - | Current version: | V17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on frequency hopping for PUSCH with a configured grant |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | TSG RAN WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR-52-71GHz |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Release 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Considering RAN 1 spec completeness, it is better to provide exact values for RB offsets instead of using X in spec. Finally, the whole working assumption can be revised later based on the final RAN4 decision. Two candidate values {56, 76} have been provided. For candidate value 56, which means that the frequency domain locations of SSB are fully overlapping with the center part of Coreset#0. If a SSB and its associated Type 0 PDCCH are in the same slot, the RB allocation for SIB1 is limited (i.e. upper or lower part relevant to SSB) since it needs to avoid overlapping with SSB in frequency domain when the time domain allocation for SIB1 PDSCH is overlapping with SSB. To ensure the reliability of SIB 1, it is better to allocate more frequency domain resources. Therefore, we prefer to use 76 for RB offsets in 96 RBs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification as exact value for RB offsets in the CORESET 0 configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The configuration of the CORESET 0 for {120, 120}, {480, 480}, and {960, 960} is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |