3GPP TSG RAN WG1 #109-e		R1- 2203509

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Maintenance on PDCCH monitoring enhancements for NR operation from 52.6GHz to 71GHz

Agenda Item:	8.2.2

Document for:	Discussion and Decision

- Introduction

The following objective is included in the agreed work item [1]:

- Physical layer aspects including [RAN1]:

  - Support enhancement to PDCCH monitoring, including blind detection/CCE budget, and multi-slot span monitoring, potential limitation to UE PDCCH configuration and capability related to PDCCH monitoring.

In this contribution, maintenance issues on PDCCH monitoring enhancements are discussed.

- Discussion

PDCCH monitoring capability for multiple serving cells

In RAN1#108-e meeting, the following agreement is made on PDCCH monitoring capability for multiple serving cells:

Agreement

- For serving cells configured with 480 or 960 kHz SCS, the serving cells with the same SCS and Xs value are grouped together to determine a total BD/CCE budget for that group and the per-cell BD/CCE budget within the group.

- Support UE capability signalling for 4 additional cases:

  - Case 4: Capability on the number of CCs with Rel-17 monitoring capability only

    - Range of pdcch-BlindDetectionCA-R17: {[4, 5, …, , 16]}

  - Case 5: Capability on the number of CCs with Rel-15 monitoring capability and Rel-17 monitoring capability on different serving cells

    - pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability

    - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability

    - Range of pdcch-BlindDetectionCA-R17 and pdcch-BlindDetectionCA-R15: {[1, 2, …, 15]}

      - Range of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R17: {[4, 5, …, 16]}

  - Case 6: Capability on the number of CCs with Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells

    - pdcch-BlindDetectionCA-R16 for Rel-16 PDCCH monitoring capability

    - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability

    - Range of pdcch-BlindDetectionCA-R17 and pdcch-BlindDetectionCA-R16: {[1, 2, …, 15]}

      - Range of pdcch-BlindDetectionCA-R16 + pdcch-BlindDetectionCA-R17: {[3, 4, …, 16]}

  - Case 7: Capability on the number of CCs with Rel-15 monitoring capability , Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells

    - pdcch-BlindDetectionCA-R15 for Rel-15 PDCCH monitoring capability

    - pdcch-BlindDetectionCA-R16 for Rel-16 PDCCH monitoring capability

    - pdcch-BlindDetectionCA-R17 for Rel-17 PDCCH monitoring capability

    - Range of pdcch-BlindDetectionCA-R17, pdcch-BlindDetectionCA-R16, and pdcch-BlindDetectionCA-R15: {[1, 2, …, 15]}

      - Range of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16 + pdcch-BlindDetectionCA-R17 : {[4, 5, …, 16]}

- For the case with Rel-15 monitoring capability, Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells (case 7) or any combination of 2 of the capabilities (i.e. case 5, and case 6), the UE will report one or more combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16, pdcch-BlindDetectionCA-R17) as UE capability. If UE reports more than one combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16, pdcch-BlindDetectionCA-R17), as in Rel-16, the gNB configures which combination for the UE to use for scaling PDCCH monitoring capability if the number of CCs configured is larger than the reported capability.

- FFS: Extension to NR-DC scenario

For Case 4, in NR-DC scenario, capability signaling on Rel-17 monitoring should be reported for MCG and SCG separately as Rel-15/16 does.

-------------------------------------------------------TP1 in TS 38.213 v17.1.0 [3]-------------------------------------------

Reason for change: Rel-17 PDCCH monitoring capability signaling in NR-DC scenario is missing in current spec.

Summary of change: Define Rel-17 PDCCH monitoring capability signaling in NR-DC scenario.

Consequences if not approved: Rel-17 PDCCH monitoring capability signaling in NR-DC scenario is not defined.

*** Unchanged text omitted ***

10	UE procedure for receiving control information

*** Unchanged text omitted ***

If a UE indicates in UE-NR-Capability a carrier aggregation capability larger than four downlink cells, the UE includes in UE-NR-Capability an indication for a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs that the UE can monitor per group of Xs slots when the UE is configured for carrier aggregation operation over more than four downlink cells with SCS configuration μ∈5, 6. When a UE is not configured for NR-DC operation for all downlink cells where the UE monitors PDCCH, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per group of Xs slots that corresponds to Ncellscap-r17 downlink cells, where

-	Ncellscap-r17 is the number of configured downlink cells if the UE does not provide pdcch-MonitoringCA-r17

-	otherwise, Ncellscap-r17 is the value of pdcch-MonitoringCA-r17

When the UE is configured for carrier aggregation operation over more than 4 cells, the UE does not expect to monitor per group of Xs slots a number of PDCCH candidates or a number of non-overlapped CCEs that is larger than the maximum number as derived from the corresponding value of Ncellscap-r17.

When a UE is configured for NR-DC operation and the UE is provided monitoringCapabilityConfig = r17monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per span that corresponds to

-	Ncellscap-r17= Ncells, r17MCG downlink cells for the MCG where Ncells, r17MCG is provided by pdcch-BlindDetection3 for the MCG, and

-	Ncellscap-r16= Ncells, r16SCG downlink cells for the SCG where Ncells, r16SCG is provided by pdcch-BlindDetection3 for the SCG

When the UE is configured for carrier aggregation operation over more than 2 cells, or for a cell group when the UE is configured for NR-DC operation, the UE does not expect to monitor per group of  Xs slots a number of PDCCH candidates or a number of non-overlapped CCEs that is larger than the maximum number as derived from the corresponding value of Ncellscap-r16.

When a UE is configured for NR-DC operation with a total of NNR-DCDL,cells downlink cells on both the MCG and the SCG and the UE is provided monitoringCapabilityConfig = r17monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE expects to be provided pdcch-BlindDetection3 for the MCG and pdcch-BlindDetection3 for the SCG with values that satisfy

-	pdcch-BlindDetection3 for the MCG + pdcch-BlindDetection3 for the SCG <= pdcch-MonitoringCA-r17, if the UE reports pdcch-MonitoringCA, or

-	pdcch-BlindDetection3 for the MCG + pdcch-BlindDetection3 for the SCG <= NNR-DCDL,cells, if the UE does not report pdcch-MonitoringCA-r17

When a UE is configured for NR-DC operation and the UE is provided monitoringCapabilityConfig = r16monitoringcapability for all downlink cells where the UE monitors PDCCH, the UE may indicate, through pdcch-BlindDetectionMCG-UE-r16 and pdcch-BlindDetectionSCG-UE-r16, respective maximum values for pdcch-BlindDetection for the MCG and pdcch-BlindDetection for the SCG.

If the UE reports pdcch-MonitoringCA-r17,

-	the value range of pdcch-BlindDetectionMCG-UE-r17 or of pdcch-BlindDetectionSCG-UE-r17 is [1, …, pdcch-MonitoringCA-r17-1], and

-	pdcch-BlindDetectionMCG-UE-r17 + pdcch-BlindDetectionSCG-UE-r17>= pdcch-MonitoringCA.

Otherwise, if NNR-DC,  max, r17DL,cells is a maximum total number of downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability and the UE is configured on both the MCG and the SCG for NR-DC as indicated in UE-NR-Capability

-	the value of pdcch-BlindDetectionMCG-UE-r17 or of pdcch-BlindDetectionSCG-UE-r17 is 1,

-		pdcch-BlindDetectionMCG-UE-r17 + pdcch-BlindDetectionSCG-UE-r17 >= NNR-DC,  max, r17DL,cells.

-------------------------------------------------------TP1 in TS 38.213 v17.1.0 [3]-------------------------------------------

Proposal 1: For multi-cell operation, support NR-DC scenario with Rel-17 monitoring capability only and adopt TP1 in TS 38.213.

- Conclusion

In this contribution, we focus on PDCCH monitoring enhancements for NR from 52.6GHz to 71GHz, and have the following proposals:

Proposal 1: For multi-cell operation, support NR-DC scenario with Rel-17 monitoring capability only and adopt TP1 in TS 38.213.

- References

- RP-202925, RAN #90-e, Electronic Meeting, 7-11 December 2020.

- Chairman Notes, RAN1 108 e-meeting, February 2022.

- TS 38.213, v17.0.0, April 2022.