3GPP TSG RAN WG1 #109-e		R1-2203510

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Maintenance on PDSCH/PUSCH enhancements for NR operation from 52.6GHz to 71GHz

Agenda Item:	8.2.3

Document for:	Discussion and Decision

- Introduction

In this contribution, some remaining issues for multi-PDSCH/PUSCH scheduling and HARQ-ACK feedback will be discussed.

- Multi-PDSCH/PUSCH scheduling

  - DCI design for multi-PDSCH/PUSCH scheduling

3.1.1 Clarification on “scheduled PXSCH”

During RAN1#107b-e meeting, discussion on the issue whether “scheduled PXSCH” in previous agreements implies valid PXSCH or not was divided into the following 7 cases.

Case 1: For NDI/RV, are NDI/RV fields for invalid PXSCHs present in multi-PXSCH scheduling DCI?

Case 2: For RV field, is the bit-width between 1 bit and 2 bits determined based on the number of configured SLIVs or valid SLIVs?

Case 3: For CSI-request, is the number M determined based on the number of configured SLIVs or valid SLIVs?

Case 4: For CBGTI field, is the presence of CBGTI field determined based on the number of configured SLIVs or valid SLIVs?

Case 5: For out-of-order scheduling, is the rule for OOO scheduling determined based on configured SLIVs or valid SLIVs?

Case 6: For a first DCI scheduling multiple PDSCHs and providing an inapplicable value of k1 in its PDSCH-to-HARQ_feedback timing indicator filed, to multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH in a slot indicated by the PDSCH-to-HARQ_feedback timing indicator filed in a second DCI, only the valid PDSCHs scheduled by the first DCI are considered for definition of the corresponding timeline requirements.

Case 7: In the case of multi-PDSCH scheduling via a single DCI with ‘tdmSchemeA’, cancel both of two repeated PDSCHs if at least one of repeated PDSCHs collides with semi-static UL symbols or determine the validity rule for each of repeated PDSCHs

Case 1/2/4 were resolved during RAN1#107b-e meeting with corresponding agreement/conclusion listed as below.

Agreement

For a DCI that can schedule multiple PDSCHs or multiple PUSCHs,

- It is clarified that NDI/RV fields in the following previous agreements correspond to scheduled PDSCHs indicated by the TDRA information field.

Agreement: (RAN1#104-bis) For a DCI that can schedule multiple PDSCHs, - NDI for the 1st TB: This is signaled per PDSCH and applies to the first TB of each PDSCH - RV for the 1st TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the first TB of each PDSCH Agreement: (RAN1#106bis-e) For a DCI that can schedule multiple PDSCHs, and if RRC parameter configures that two codeword transmission is enabled, - NDI for the 2nd TB: This is signalled per PDSCH and applies to the 2nd TB of each PDSCH - RV for the 2nd TB: This is signalled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the 2nd TB of each PDSCH

- Above clarification also applies to the DCI scheduling multiple PUSCHs, i.e., NDI/RV fields in the DCI correspond to scheduled PUSCHs indicated by the TDRA information field.

- The following example change to 38.214 Sections 5.1.3 and 6.1.4 can be recommended to the editor of 38.214 to use at the editor’s discretion

Conclusion

It is clarified that the absence or presence of CBGTI field in the following previous agreement is determined based on scheduled PUSCHs indicated by the TDRA information field (i.e. irrespective of whether this is a valid PUSCH).

Agreement: (RAN1#105-e) - At least for 120 kHz SCS, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, - If CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16.

Case 3/7 were resolved during RAN1#108-e meeting with corresponding agreement/conclusion listed as below.

Conclusion

For a DCI that can schedule multiple PUSCHs, it is clarified that the following M is counted based on scheduled PUSCHs indicated by the TDRA information field.

- CSI-request: When the DCI schedules M PUSCHs, the PUSCH that carries the aperiodic CSI feedback is M-th scheduled PUSCH for M <= 2, or (M-1)-th scheduled PUSCH for M > 2.

Note: Aperiodic CSI feedback will be dropped if M-th scheduled PUSCH for M <= 2 (or (M-1)-th scheduled PUSCH for M > 2) is collided with semi-static DL or SSB symbols.

Agreement

For multi-PDSCH scheduling via a single DCI with 'tdmSchemeA' for single DCI based multi-TRP mechanism,

- If at least one of the repetitions of the PDSCH collides with semi-static UL symbols, the corresponding PDSCH (i.e., both repetitions) is considered as invalid.

  - Note: No specification impact on Type-1 HARQ-ACK codebook construction is expected, as a consequence of this agreement.

  - Note: This is not applied for the case when the multi-PDSCH DCI schedules only a single PDSCH.

Here our views on Case 5/6 are provided as below.

- Case 5: OoO

In our understanding, an invalid PDSCH/PUSCH can be regarded as not scheduled actually by the gNB, and OoO rules are applied only to cases where PDSCH(s)/PUSCH(s) is(are) scheduled actually. Otherwise, many cases will be ruled out unnecessarily, resulting in rather restrictive scheduling.

Proposal 1: For multi-PDSCH/PUSCH scheduling, OoO scheduling rules are applied only to valid SLIV(s) in the TDRA row indicated by a scheduling DCI.

- Case 6: NNK1

As mentioned above, an invalid PDSCH can be regarded as not scheduled by the gNB, therefore it is natural to not consider invalid PDSCH(s) scheduled by the first DCI when PDSCH(s) scheduled by the first DCI is(are) used to decide if an SPS PDSCH reception is received after the PDSCH(s) or not.

Proposal 2: For multi-PDSCH scheduling, and for case(s) where Type-2 codebook/enhanced Type-2 codebook is configured and an inapplicable value is provided by a PDSCH-to-HARQ_feedback timing indicator field included in a first DCI scheduling more than one PDSCH, only valid PDSCH(s) scheduled by the first DCI is considered to decide if an SPS PDSCH reception is received after the PDSCH(s) or not.

3.1.2 TDRA information for a DCI indicating SCell dormancy

During RAN1#108-e meeting, TDRA information for a DCI indicating SCell dormancy or TCI state update without scheduling PDSCH reception was discussed, with the corresponding agreement as below.

Agreement

When a DCI format indicates TCI state update without scheduling PDSCH reception, the PDCCH indicates a TDRA row index including only one SLIV.

So far it is pending whether the same restriction, i.e., the DCI indicating a TDRA row index including only one SLIV, should be applied for SCell dormancy or not.

From our perspective, it is simpler to apply the same restriction also for SCell dormancy. Otherwise, the UE has to determine the bit-widths of NDI field and RV field based on the number of SLIV entries included in the indicated row, i.e., if the indicated row includes a single SLIV entry, 1-bit NDI field and 2-bit RV field are assumed, otherwise NPDSCHmax-bit NDI field and NPDSCHmax-bit RV field are assumed, and re-purpose the involved bits accordingly to obtain SCell dormancy information, resulting in more complexity.

Proposal 3: For multi-PDSCH scheduling, when a DCI indicates SCell dormancy without scheduling PDSCH reception, the DCI indicates a TDRA row index including only one SLIV.

  - Type-2 HARQ-ACK codebook for multi-PDSCH scheduling

3.2.1 Time domain bundling operation

In [1], a new concept of transport block group (TBG) is introduced to deal with time domain bundling for Type-2 codebook, and some descriptions about TBG are referred as below.

9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

…

…

-	Nm,creceived or Nm,c,greceived, for G-RNTI g or G-CS-RNTI g, is

-	if harq-ACK-SpatialBundlingPUCCH is not provided, the number of transport blocks the UE receives in a PDSCH, or the number of transport block groups in PDSCHs if numberOfHARQ-BundlingGroups with NHARQ-ACKTBG,max=1 is provided, scheduled by a DCI format that the UE detects in PDCCH monitoring occasion m for serving cell c

…

If a UE is provided numberOfHARQ-BundlingGroups and is not provided harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over transport block groups (TBGs) for PDSCH receptions where, for a maximum number of NPDSCHmax PDSCH receptions scheduled by a DCI format on the serving cell, a maximum number of TBGs NHARQ-ACK,cTBG,max is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for first TBs and, if applicable, generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for second TBs in the NPDSCH,c PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c. For a TBG associated with at least one PDSCH that does not overlap with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE assumes that TB(s) provided by a PDSCH that overlaps with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, are correctly received. For a TBG associated only with PDSCHs that overlap with UL symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE generates a NACK value for the TBG.

…

…

-	if harq-ACK-SpatialBundlingPUCCH is not provided,

-	if numberOfHARQ-BundlingGroups is provided, Nm,creceived,TBG is the number of TBGs including at least one PDSCH not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, that the UE receives in serving cell c from the NcellsDL,TBG serving cells in PDCCH monitoring occasion m and the UE reports corresponding HARQ-ACK information in the PUCCH

…

Regarding the concept of TBG, as well as the above referred descriptions, some questions can be raised as below.

Q1: Is TBGs only applicable for an enabled TB, or also applicable for a disabled TB?

When two-codeword transmission is configured, a DL scheduling DCI can indicate two TBs, one of which may be disabled. When both TBs are enabled, the same division of TBGs can be assumed due to the same value setting of NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c. However, when one TB is disabled, is the division of TBGs applied to the disable TB? In our opinion, division of TBGs should not be applied to a disabled TB, since for each scheduled PDSCH by the DCI, no actual TB is transmitted corresponding to the disabled TB.

Besides, the answer to Q1 will impact the understanding or calculation of the number of TBGs. For example, when calculating the value of Nm,creceived or Nm,c,greceived in nHARQ-ACK,TB calculation, to determine the number of transport block groups in PDSCHs if numberOfHARQ-BundlingGroups with NHARQ-ACKTBG,max=1 is provided, assuming a DL scheduling DCI can indicate two TBs and one of them is disabled, if TBGs only applicable for the enable TB, the value is 1, otherwise the value is 2. Naturally, the value of 1 is desirable to indicate the number of meaningful HARQ-ACK bits.

Proposal 4: For time domain bundling of Type-2 codebook, division of TBGs is applicable only for an enabled TB of a DL scheduling DCI.

Q2: How to understand the description “…and, if applicable, generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for second TBs in the NPDSCH,c PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c”?

From our perspective, when two-codeword transmission is configured, each TB of a DL scheduling DCI will correspond to NHARQ-ACK,cTBG,max HARQ-ACK information bits. For an enabled TB, the UE can refer to the description in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c to generate NHARQ-ACK,cTBG,max HARQ-ACK information bits for the enabled TB. But for a disabled TB, the UE cannot apply the above operation since based on Q1 no division of TBGs is applicable for the disabled TB. Instead, the UE directly generate NHARQ-ACK,cTBG,max HARQ-ACK information bits each of which is set to NACK for the disabled TB.

Besides, the description “For a TBG associated with at least one PDSCH that does not overlap with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE assumes that TB(s) provided by a PDSCH that overlaps with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, are correctly received. For a TBG associated only with PDSCHs that overlap with UL symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE generates a NACK value for the TBG.” only applies to an enabled TB of a DL scheduling DCI. Furthermore, for a TBG of an enabled TB, an invalid PDSCH can provide only one TB, so the description “…TB(s) provided by a PDSCH that overlaps…are correctly received” can be updated to “…the TB(s) provided by a PDSCH that overlaps…isare correctly received” for accuracy.

Proposal 5: For time domain bundling of Type-2 codebook, the UE directly generate NHARQ-ACK,cTBG,max HARQ-ACK information bits each of which is set to NACK for a disabled TB of a DL scheduling DCI.

Based on the above two proposals, the following TP1 can be considered, with the revised text in red.

------------------------------------------------------------Start TP1---------------------------------------------------------------

Reason for change: There is ambiguity on the division of TBGs for a TB disabled by a DCI format, as well as the number of TBGs corresponding to the DCI format.

Summary of change: Clarify that for time domain bundling of Type-2 codebook, division of TBGs is applicable only for an enabled TB of a DCI format scheduling more than one PDSCH. For a TB disabled by the DCI format, the UE directly generate NHARQ-ACK,cTBG,max HARQ-ACK information bits each of which is set to NACK.

Consequences if not approved: The UE may have no idea on how to determine the number of TBGs corresponding to a DCI format when one TB is disabled by the DCI format.

9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

…

If a UE is provided numberOfHARQ-BundlingGroups and is not provided harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over transport block groups (TBGs) for PDSCH receptions where, for a maximum number of NPDSCHmax PDSCH receptions scheduled by a DCI format on the serving cell, a maximum number of TBGs NHARQ-ACK,cTBG,max for a TB enabled by the DCI format is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for first TBs if the first TB is enabled by the DCI format and, if applicable, generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for second TBs if the second TB is enabled by the DCI format, in the NPDSCH,c PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c. For a TBG associated with at least one PDSCH that does not overlap with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE assumes that the TB(s) provided by a PDSCH that overlaps with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, are is correctly received. For a TBG associated only with PDSCHs that overlap with UL symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE generates a NACK value for the TBG. For any TB disabled by the DCI format, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits, each of which is set to NACK.

…

------------------------------------------------------------End TP1----------------------------------------------------------------

Proposal 6: For time domain bundling of Type-2 codebook, consider the TP1 in TS38.213 for the division of TBGs when spatial bundling is not configured.

In [1], time domain bundling operation for Type-2 codebook is divided further into two cases, i.e. the case when harq-ACK-SpatialBundlingPUCCH is not provided and the case when harq-ACK-SpatialBundlingPUCCH is provided. For the latter case, the corresponding description is referred as below.

9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

…

If a UE is provided numberOfHARQ-BundlingGroups and harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over PDSCH reception groups for PDSCH receptions scheduled by a DCI format on the serving cell c where a maximum number of PDSCH reception groups, NHARQ-ACK,cTBG,max, is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for the NPDSCH,c PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c. For a PDSCH reception group associated with at least one PDSCH that does not overlap with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE assumes that TBs provided by a PDSCH that overlaps with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, are correctly received. For a PDSCH reception group associated only with PDSCHs that overlap with UL symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE generates a NACK value for the PDSCH reception group.

…

Based on the above description, the UE refers to the description in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c to generate NHARQ-ACK,cTBG,max HARQ-ACK information bits for the DL scheduling DCI. However, in clause 9.1.1 there is no description about spatial bundling, e.g. binary AND operation of the HARQ-ACK information bits corresponding to first and second transport blocks of all PDSCH receptions in a PDSCH reception group. Therefore, there may be ambiguity on how to perform spatial bundling when time domain bundling is configured for Type-2 codebook.

Proposal 7: For time domain bundling of Type-2 codebook, clarify the corresponding UE behaviour to generate NHARQ-ACK,cTBG,max HARQ-ACK information bits for a DL scheduling DCI when spatial bundling is also configured.

3.2.2 Calculation of nHARQ-ACK

In [1], a new nHARQ-ACK calculation formula is introduced for multi-PDSCH scheduling, and the corresponding description is referred as below.

9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

…

If OACK+OSR+OCSI≤11 and NcellsDL,TBG>0, the UE also determines nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,TBG for obtaining a PUCCH transmission power, as described in clause 7.2.1, with

nHARQ-ACK,TBG=VDAI,mlastDL-c=0NcellsDL,TBG-1UDAI,cTBGmodTDNHARQ-ACK,maxTBG,max+c=0NcellsDL,TBG-1m=0M-1Nm,creceived,TBG

where

…

The variable NcellsDL,TBG, as well as NHARQ-ACK,maxTBG,max, is defined in the description of [1] for HARQ-ACK sub-codebook setting when a UE is provided numberOfHARQ-BundlingGroups. Therefore, it may be thought that the calculation formula of nHARQ-ACK,TBG is only applied for the case when numberOfHARQ-BundlingGroups is provided and two sub-codebooks are required, since separate descriptions for HARQ-ACK sub-codebook setting exist in [1] for the case when the UE is not provided numberOfHARQ-BundlingGroups and the case when the UE is provided numberOfHARQ-BundlingGroups. However, there is no other calculation formula of nHARQ-ACK defined in [1] for the case when the UE is not provided numberOfHARQ-BundlingGroups and two sub-codebooks are required.

In our opinion, regarding the calculation formula of nHARQ-ACK for the case when the UE is not provided numberOfHARQ-BundlingGroups and two sub-codebooks are required, the term “TBG” or related variable(s) should not be used to avoid confusion. For example, the variable NcellsDL,TBG should not be used. Besides, the variable NHARQ-ACKmax should be used other than NHARQ-ACK,maxTBG,max.

In summary, it is proposed to introduce an independent description for calculation formula of nHARQ-ACK defined for the case when the UE is not provided numberOfHARQ-BundlingGroups and two sub-codebooks are required, where the corresponding variables should be used. The following TP2 can be considered as a starting point.

It should be noted that in the TP2, the description for calculation formula of nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,multi is newly introduced and tagged in gray background as a whole. Besides, the description for calculation formula of nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,multi in the TP2 is revised from the description for calculation formula of nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,TBG in [1], with the revised text in red.

------------------------------------------------------------Start TP2---------------------------------------------------------------

Reason for change: The defined calculation formula nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,TBG is not applicable for the case when the UE is not provided numberOfHARQ-BundlingGroups and two sub-codebooks are required.

Summary of change: Define a calculation formula nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,multi for the case when the UE is not provided numberOfHARQ-BundlingGroups and two sub-codebooks are required.

Consequences if not approved: There is no calculation formula of nHARQ-ACK applicable for the case when the UE is not provided numberOfHARQ-BundlingGroups and two sub-codebooks are required.

9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

…

If a UE is not provided numberOfHARQ-BundlingGroups, detects a first DCI format scheduling one PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, if any, and a second DCI format scheduling more than one PDSCH reception on a serving cell from the NcellsDL serving cells, if any, and the UE would provide corresponding HARQ-ACK information in a same PUCCH, the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications

-	the UE determines a first HARQ-ACK sub-codebook based on each detected DCI format scheduling one PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, or SPS PDSCH receptions, if any, and

-	the UE determines a second HARQ-ACK sub-codebook based on each detected DCI format scheduling more than one PDSCH reception, and

-	instead of generating one HARQ-ACK information bit per transport block for serving cell c, the UE generates NHARQ-ACKmax HARQ-ACK information bits where NHARQ-ACKmax is the maximum value of NTB,cDL⋅NPDSCH,cmax across all NcellsDL serving cells, NPDSCH,cmax is a maximum number of PDSCH receptions that can be scheduled by a DCI format on serving cell c as described in [6, TS 38.214], and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided; else, NTB,cDL=1. The UE generates the NHARQ-ACKmax HARQ-ACK information bits in ascending order of the NPDSCH,cmax PDSCHs, including any PDSCH that overlaps with UL symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided. If, for serving cell c, the UE detects a DCI format that schedules NPDSCH,c PDSCH receptions and NTB,cDL⋅NPDSCH,c<NHARQ-ACKmax , the UE generates NACK for the last NHARQ-ACKmax -NTB,cDL⋅NPDSCH,c HARQ-ACK information bits

-	The pseudo-code operation when PDSCH-CodeBlockGroupTransmission is provided is not applicable.

-	The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook.

-	The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook.

If the UE is not provided numberOfHARQ-BundlingGroups and OACK+OSR+OCSI≤11 and NcellsDL,TBG>0, the UE also determines nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,TBGmulti for obtaining a PUCCH transmission power, as described in clause 7.2.1, with

nHARQ-ACK,TBGmulti=VDAI,mlastDL-c=0NcellsDL,TBGmulti-1UDAI,cmultiTBGmodTDNHARQ-ACK,maxTBG,max+c=0NcellsDL,multiTBG-1m=0M-1Nm,creceived,multiTBG

where

-	NcellsDL,multi is the number of serving cells each of which is provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH in NcellsDL serving cells

-	if NcellsDL=1, VDAI,mlastDL is the value of the counter DAI in the last DCI format scheduling more than one PDSCH receptions for any serving cell c from the NcellsDL,multiTBG serving cells with TBG-based HARQ-ACK information or with TB-based HARQ-ACK information that the UE detects within the M PDCCH monitoring occasions

-	if NcellsDL>1, VDAI,mlastDL is the value of the total DAI in the last DCI format scheduling more than one PDSCH receptions with TBG-based HARQ-ACK information or with TB-based HARQ-ACK information for any serving cell c from the NcellsDL,multiTBG serving cells that the UE detects within the M PDCCH monitoring occasions

-	VDAI,mlastDL=0, if the UE does not detect any DCI format scheduling more than one PDSCH receptions with TBG-based HARQ-ACK information or with TB-based HARQ-ACK information for any serving cell c from the NcellsDL,multiTBG serving cells in any of the M PDCCH monitoring occasions

-	UDAI,cmultiTBG is the total number of DCI formats scheduling more than one PDSCH receptions with TBG-based HARQ-ACK information or with TB-based HARQ-ACK information for any serving cell c from the NcellsDL,multiTBG serving cells that the UE detects within the M PDCCH monitoring occasions for serving cell c. UDAI,cTBG=0 if the UE does not detect any DCI format scheduling more than one PDSCH receptions for serving cell c in any of the M PDCCH monitoring occasions

-	if harq-ACK-SpatialBundlingPUCCH is provided,

-	if numberOfHARQ-BundlingGroups is provided, Nm,creceived,TBG is the number of PDSCH groups that include at least one PDSCH not overlapping with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated if provided, that the UE receives in serving cell c from the NcellsDL,TBG serving cells in PDCCH monitoring occasion m and the UE reports corresponding HARQ-ACK information in the PUCCH

-	if numberOfHARQ-BundlingGroups is not provided, Nm,creceived,multiTBG is the number of PDSCHs that the UE receives in serving cell c from the NcellsDL,multiTBG serving cells in PDCCH monitoring occasion m and the UE reports corresponding HARQ-ACK information in the PUCCH

-	if harq-ACK-SpatialBundlingPUCCH is not provided,

-	if numberOfHARQ-BundlingGroups is provided, Nm,creceived,TBG is the number of TBGs including at least one PDSCH not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, that the UE receives in serving cell c from the NcellsDL,TBG serving cells in PDCCH monitoring occasion m and the UE reports corresponding HARQ-ACK information in the PUCCH

-	if numberOfHARQ-BundlingGroups is not provided, Nm,creceived,multiTBG is the number of transport blocks in PDSCHs that the UE receives in serving cell c from the NcellsDL,multiTBG serving cells in PDCCH monitoring occasion m and the UE reports corresponding HARQ-ACK information in the PUCCH.

If a UE is provided numberOfHARQ-BundlingGroups and is not provided harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over transport block groups (TBGs) for PDSCH receptions where, for a maximum number of NPDSCHmax PDSCH receptions scheduled by a DCI format on the serving cell, a maximum number of TBGs NHARQ-ACK,cTBG,max is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for first TBs and, if applicable, generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for second TBs in the NPDSCH,c PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c. For a TBG associated with at least one PDSCH that does not overlap with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE assumes that TB(s) provided by a PDSCH that overlaps with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, are correctly received. For a TBG associated only with PDSCHs that overlap with UL symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, the UE generates a NACK value for the TBG.

…

------------------------------------------------------------End TP2----------------------------------------------------------------

Proposal 8: For Type-2 codebook of multi-PDSCH scheduling, introduce an independent description for calculation formula of nHARQ-ACK defined for the case when the UE is not provided numberOfHARQ-BundlingGroups and two sub-codebooks are required, where the corresponding variables are used; the TP2 can be considered in TS38.213 as a starting point.

In addition, in [1], the description for the legacy calculation formula nHARQ-ACK=nHARQ-ACK,TB, applicable for the case when only one sub-codebook is required, is extended to accommodate the case when multi-PDSCH scheduling is configured and the number of bundling groups is 1, which is referred as below. In the following description, three cases in total are listed to apply the calculation formula nHARQ-ACK=nHARQ-ACK,TB.

9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

…

If a UE is

-	not provided PDSCH-CodeBlockGroupTransmission for any serving cell, or

-	not provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH for any serving cell, or

-	provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and numberOfHARQ-BundlingGroups with NHARQ-ACKTBG,max=1 for a serving cell

for PDSCH receptions scheduled by a DCI format that does not support CBG-based PDSCH receptions, or for SPS PDSCH reception, or for a DCI format having associated HARQ-ACK information without scheduling PDSCH reception, and if OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in clause 7.2.1, as

nHARQ-ACK=nHARQ-ACK,TB=VDAI,mlastDL-c=0NcellsDL-1UDAI,cmodTDNTB,maxDL+c=0NcellsDL-1m=0M-1Nm,creceived+NSPS,c+g=0G-1VDAI,mlast,gDL-c=0Ncells,gDL-1UDAI,c,gmodTD,gNTB,max,gDL+c=0Ncells,gDL-1m=0Mg-1Nm,c,greceived+NSPS,c,g

where

…

However, the above calculation formula nHARQ-ACK,TB is also used as a component to calculate other nHARQ-ACK, where the above three cases (which apply only for the equation nHARQ-ACK=nHARQ-ACK,TB) may not be applicable anymore. Taking nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,TBG in [1] as an example, nHARQ-ACK,TB is used to calculate the nHARQ-ACK corresponding to the first sub-codebook, which contains the corresponding HARQ-ACK information described as below in [1], where the above three cases are not applicable anymore.

9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

…

If a UE

-	is provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1 for NcellsDL,TBG serving cells; and

-	is not provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH or is provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max=1, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,TBG=NcellsDL

the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications

-	NcellsDL is used for the determination of a first HARQ-ACK sub-codebook for

-	SPS PDSCH reception,

-	any DCI format having associated HARQ-ACK information without scheduling PDSCH reception, and

-	PDSCH reception scheduled by a DCI format scheduling one PDSCH

-	PDSCH reception with NHARQ-ACKTBG,max=1 for TBG-based HARQ-ACK information on the NcellsDL,TB serving cells,

…

To avoid the aforementioned inconsistent or ambiguity, when nHARQ-ACK,TB is used as a component to calculate other nHARQ-ACK (than nHARQ-ACK=nHARQ-ACK,TB), additional description for nHARQ-ACK,TB can be added to describe the purpose of nHARQ-ACK,TB or HARQ-ACK case(s) covered by nHARQ-ACK,TB to differentiate from the three cases for nHARQ-ACK=nHARQ-ACK,TB. For nHARQ-ACK,TB in the calculation formula nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,TBG, the following TP3 can be considered, with the revised text in red.

------------------------------------------------------------Start TP3---------------------------------------------------------------

Reason for change: When nHARQ-ACK,TB is used as a component to calculate other nHARQ-ACK (than nHARQ-ACK=nHARQ-ACK,TB), identified cases applied only for the equation nHARQ-ACK=nHARQ-ACK,TB may not be applicable anymore.

Summary of change: Clarify the purpose or covered HARQ-ACK case(s) by nHARQ-ACK,TB when it is used as a component to calculate other nHARQ-ACK (than nHARQ-ACK=nHARQ-ACK,TB).

Consequences if not approved: There may be ambiguity on whether the identified cases applied for the equation nHARQ-ACK=nHARQ-ACK,TB is also applicable for nHARQ-ACK,TB when it is used as a component to calculate other nHARQ-ACK (than nHARQ-ACK=nHARQ-ACK,TB).

9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

…

If OACK+OSR+OCSI≤11 and NcellsDL,TBG>0, the UE also determines nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,TBG for obtaining a PUCCH transmission power, as described in clause 7.2.1, where nHARQ-ACK,TB corresponds to HARQ-ACK information contained in the first HARQ-ACK sub-codebook, and nHARQ-ACK,TBG corresponds to HARQ-ACK information contained in the second HARQ-ACK sub-codebook, with

nHARQ-ACK,TBG=VDAI,mlastDL-c=0NcellsDL,TBG-1UDAI,cTBGmodTDNHARQ-ACK,maxTBG,max+c=0NcellsDL,TBG-1m=0M-1Nm,creceived,TBG

where

…

------------------------------------------------------------End TP3----------------------------------------------------------------

Proposal 9: Clarify the purpose or applicable case(s) of nHARQ-ACK,TB when it is used as a component to calculate other nHARQ-ACK (than nHARQ-ACK=nHARQ-ACK,TB) to avid ambiguity, and the TP3 can be considered in TS38.213.

- Conclusion

In this contribution, some remaining issues for multi-PDSCH/PUSCH scheduling and HARQ-ACK feedback are discussed. We have the following proposals.

Proposal 1: For multi-PDSCH/PUSCH scheduling, OoO scheduling rules are applied only to valid SLIV(s) in the TDRA row indicated by a scheduling DCI.

Proposal 2: For multi-PDSCH scheduling, and for case(s) where Type-2 codebook/enhanced Type-2 codebook is configured and an inapplicable value is provided by a PDSCH-to-HARQ_feedback timing indicator field included in a first DCI scheduling more than one PDSCH, only valid PDSCH(s) scheduled by the first DCI is considered to decide if an SPS PDSCH reception is received after the PDSCH(s) or not.

Proposal 3: For multi-PDSCH scheduling, when a DCI indicates SCell dormancy without scheduling PDSCH reception, the DCI indicates a TDRA row index including only one SLIV.

Proposal 4: For time domain bundling of Type-2 codebook, division of TBGs is applicable only for an enabled TB of a DL scheduling DCI.

Proposal 5: For time domain bundling of Type-2 codebook, the UE directly generate NHARQ-ACK,cTBG,max HARQ-ACK information bits each of which is set to NACK for a disabled TB of a DL scheduling DCI.

Proposal 6: For time domain bundling of Type-2 codebook, consider the TP1 in TS38.213 for the division of TBGs when spatial bundling is not configured.

Proposal 7: For time domain bundling of Type-2 codebook, clarify the corresponding UE behaviour to generate NHARQ-ACK,cTBG,max HARQ-ACK information bits for a DL scheduling DCI when spatial bundling is also configured.

Proposal 8: For Type-2 codebook of multi-PDSCH scheduling, introduce an independent description for calculation formula of nHARQ-ACK defined for the case when the UE is not provided numberOfHARQ-BundlingGroups and two sub-codebooks are required, where the corresponding variables are used; the TP2 can be considered in TS38.213 as a starting point.

Proposal 9: Clarify the purpose or applicable case(s) of nHARQ-ACK,TB when it is used as a component to calculate other nHARQ-ACK (than nHARQ-ACK=nHARQ-ACK,TB) to avid ambiguity, and the TP3 can be considered in TS38.213.

References

- TS 38.213, Physical layer procedures for control, v17.1.0, Mar. 2022.