3GPP TSG RAN WG1 #109-e		R1-2203512

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Maintenance on beam management

Agenda Item:	8.2.5

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the remaining issue of UE behavior whether activated TCI states in case of unified beam framework can be updated or not during multi-PDSCH/PUSCH scheduling.

- Discussion

As per the most recent TS 38.214, when UE is configured with multi-PDSCH scheduling, the UE shall expect the activated TCI state(s) are the same across the slots of these PDSCHs. In other words, UE does not expect the updated TCI state(s) informed by MAC CE to be changed during the span of multi-PDSCH.

In Rel-17, unified beam feature has been introduced in MIMO WI which can be applied for multi-PDSCH and multi-PUSCH scheduling as well. However, whether unified TCI states can be updated or not during the span of multi-PDSCH/PUSCH has not been decided. In RAN1 #108-e, the following agreement related to this issue is achieved.

Agreement TP#1 below is endorsed for TS 38.214 - RAN1 continues discussion to down-select between the following Alt1 and Alt2. - Alt1) The applied TCI states can be updated using unified TCI framework within the span of multi-PDSCH - Alt2) The applied TCI states cannot be updated using unified TCI framework within the span of multi-PDSCH - Note: coordination with Rel-17 MIMO WI is allowed as necessary

First and foremost, how to define the applied TCI states in unified TCI framework should be clarified. In Rel-17, unified TCI states can take effect at least BeamAppTime_r17 symbols after the last symbol of one PUCCH which responds to one DCI indicating a new common TCI state, or unified TCI states can take effect at least 3ms after the slot of one PUCCH which is corresponding to the MAC CE carrying a new common TCI state. In our opinion, both TCI states of these two ways should be included for this discussion.

The applied common TCI states includes the TCI states indicated by DCI with/without data scheduling and the TCI states indicated by MAC CE.

Regarding the updated common TCI states behavior, two main use cases should be considered.

- In case of multi-CA scheduling, the parameter of simultaneousTCI-UpdateList indicates TCI States Activation/Deactivation for UE-specific PDSCH MAC CE applied for these CCs in same list simultaneously. An example of single slot PDSCH scheduling on CC1 and multi-PDSCH scheduling on CC2 is illustrated in Figure 1.

multi-CC scheduling configured with simultaneousTCI-UpdateList

Given UE executes the indicated common TCI states by DCI in CC1 marked after red dashed line but the applied TCI states cannot be updated within the span of multi-PDSCH, then there is a confusion that which beam for simultaneous reception for two CCs should be adopted after the timeline. Although this contradiction can be avoided by gNB’s implementation, there is serious restriction to network in real deployment especially a large number of CCs are configured to UE, since gNB should find a clear timeline for all CCs to ensure the rule of Alt2, which it is too difficult in case of dynamic beam change and random traffic scheduling.

- In case of unified beam applied to multi-PDSCH scheduling and other channels or RSs including SRS, PUSCH, PDCCH and CSI-RS for the UE allocated in the span as illustrated in Figure 2, if gNB determines the old TCI states is unsuitable anymore, e.g. due to blockage, and expected new TCI states take effect promptly in green dashed line. However, if applied TCI states cannot be updated using unified TCI framework within the span of multi-PDSCH, then UE would receive these channels and RSs with old beam and the performance of reception will be worse.

multi-PDSCH scheduling and unified beam

In conclusion, if the applied TCI states cannot be updated using unified TCI framework within the span of multi-PDSCH/PUSCH, it will reduce the scheduling flexibility in gNB side and reduce the reception performance of channels/RSs in UE side. Therefore, it is better to support Alt1.

The applied TCI states can be updated using unified TCI framework within the span of multi-PDSCH.

- Conclusion

In this contribution, we have the following proposals.

- The applied common TCI states includes the TCI states indicated by DCI with/without data scheduling and the TCI states indicated by MAC CE.

- The applied TCI states can be updated using unified TCI framework within the span of multi-PDSCH.

References

- R1-2201036, Discussion Summary #1 for Beam Management for new SCSs	Moderator, RAN1#108-e

- 3GPP TS 38.214 V17.1.0