3GPP TSG RAN WG1 #109-e	 	  R1-2203514

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Remaining issues on intra-UE Multiplexing/Prioritization for Rel-17 URLLC

Agenda Item:	8.3.2

Document for:	Discussion and Decision

- Introduction

In RAN1#108 e-meeting, intra-UE multiplexing/prioritization are discussed and some agreements are achieved as following[1]:

Conclusion

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, reuse the same power control formula as in Rel-15.

  - No specification impacts.

In this contribution, we share our view on the remaining issues about intra-UE multiplexing and prioritization for URLLC.

- Intra-UE multiplexing for UCI enhancements

  - PUCCH carrying HP SR and HP HARQ-ACK with PF0/1 overlapping with a PUCCH carrying LP HARQ-ACK

The issue on HP SR and HP HARQ-ACK with PF0/1 overlapping with LP HARQ-ACK has been discussed in several meetings. In the last RAN1 meeting, a compromise proposal is proposed.

Proposal: When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs which are all negative or include at least one positive, - In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource. - Note: The description of Step 1 here is only for information purpose, which has no spec impact. - In Step 2, down-select from the two options: - Option 0: LP HARQ-ACK is dropped. - Option 2: 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1 - The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4.

It is a typical case for URLLC service using PUCCH format 0/1 carrying SR and HARQ-ACK with short format. If option 0 is applied, LP HARQ-ACK would be frequently dropped and LP service performance would be degraded.

According to existing agreement when a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 2/3/4 overlaps with a PUCCH carrying LP HARQ-ACK, LP HARQ-ACK is multiplexed with HP UCI. It is reasonable to support multiplexing for PUCCH format 0/1 case from spec integrity perspective. Otherwise, the multiplexing LP HARQ-ACK would be only allowed for partial cases.

The resultant PUCCH resource for multiplexing HP SR, HP HARQ-ACK and LP HARQ-ACK should include PUCCH format 0. When HP SR and 1bit HP HARQ-ACK with PUCCH format 0 overlaps with 1bit LP HARQ-ACK, the PUCCH format 0 can be used to carry HP SR, HP HARQ-ACK and LP HARQ-ACK as Rel-15 mechanism. gNB implementation can avoid the potential performance degradation, for example, a higher transmitting power at UE side can be indicated by gNB.

For option 0, the potential ambiguity may occur between gNB and UE. For example, when 1 bit HP HARQ-ACK and HP SR with PUCCH format 0 overlaps with 1 bit LP HARQ-ACK and all SRs is negative, UE would treat the case as 1 bit HP HARQ-ACK and 1 bit LP HARQ-ACK. The 2 bits HARQ-ACKs (HP and LP) will use CS 0/3/6/9. If a SR is positive and LP HARQ-ACK is dropped, 1bit HP HARQ-ACK and 1bit HP SR would be transmitted based on HP PUCCH format 0 with CS 3/9. The ambiguity occurs for CS 3/9.

For option 2, SPS HARQ-ACK configured with n1PUCCH-AN in SPS-config should be excluded because the payload size in n1PUCCH-AN would not exceed two bits. Thus, option 2 can be updated as option 2a:

  - Option 2a:

- If HP HARQ-ACK is dynamic HARQ-ACK or SPS HARQ-ACK with SPS-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1

- If HP HARQ-ACK is SPS HARQ-ACK with n1PUCCH-AN, LP HARQ-ACK is dropped.

Based on analysis above mentioned, option 2a is preferred.

Proposal 1: When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs which are all negative or include at least one positive,

- In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource.

  - Note: The description of Step 1 here is only for information purpose, which has no spec impact.

- In Step 2,

  - Option 2a:

    - If HP HARQ-ACK is dynamic HARQ-ACK or SPS HARQ-ACK with SPS-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1

    - If HP HARQ-ACK is SPS HARQ-ACK with n1PUCCH-AN, LP HARQ-ACK is dropped.

  - Prioritizations for transmission power reductions

In Rel-16, when transmission power for PUSCH or PUCCH in a frequency range in a transmission occasion exceeds , UE allocates power to PUSCH/PUCCH according to priority order firstly. For PUCCH or PUSCH transmissions with same priority index, the power allocation would base on the following order

- PUCCH/PUSCH with HARQ-ACK information, and/or SR,

- PUCCH/PUSCH with CSI information

- PUSCH only

For Rel-17 intra-UE multiplexing, UCI multiplexing on a PUSCH with different priority is supported, for example, HP HARQ-ACK on a LP PUSCH and LP HARQ-ACK on a HP PUSCH. The UE power allocation order has not been defined for these cases.

For HP HARQ-ACK on a LP PUSCH, HP HARQ-ACK reliability should be guaranteed. Therefore, a reasonable power allocation order should take the case as the same as HP PUSCH with HP HARQ-ACK, i.e., the higher priority than HP PUSCH with CSI as well as HP PUSCH only.

Secondly, for LP HARQ-ACK on HP PUSCH, PUSCH has the higher priority than LP HARQ-ACK. The power allocation should have the same order as HP PUSCH only, i.e., lower than HP PUSCH with HP HARQ as well as HP PUSCH with HP CSI.

Proposal 2: For HP HARQ-ACK on a LP PUSCH, power allocation order should be the same as HP PUSCH with HP HARQ-ACK. For LP HARQ-ACK on HP PUSCH, power allocation order should be the same as HP PUSCH only.

- Conclusion

In this contribution, we discuss the enhancements to intra UE multiplexing, and the following observation and proposals are made.

Proposal 1: When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs which are all negative or include at least one positive,

- In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource.

  - Note: The description of Step 1 here is only for information purpose, which has no spec impact.

- In Step 2,

  - Option 2a:

    - If HP HARQ-ACK is dynamic HARQ-ACK or SPS HARQ-ACK with SPS-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1

    - If HP HARQ-ACK is SPS HARQ-ACK with n1PUCCH-AN, LP HARQ-ACK is dropped.

Proposal 2: For HP HARQ-ACK on a LP PUSCH, power allocation order should be the same as HP PUSCH with HP HARQ-ACK. For LP HARQ-ACK on HP PUSCH, power allocation order should be the same as HP PUSCH only.

References

- Chairman notes RAN1#108 e-meeting.