3GPP TSG RAN WG1 #109-e		R1-2203515

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Maintenance on accuracy improvements for NR positioning enhancements

Agenda Item:	8.5.1

Document for:	Discussion and Decision

Introduction

In RAN1#108-e meeting, some remaining issues on accuracy enhancements were discussed and some agreements were achieved in [1].

In this contribution, we present our views on remaining issues on accuracy enhancements.

Remaining issues on Rx/Tx timing delay mitigating

Mismatch issue for Multi-RTT

For Multi-RTT positioning, when the UE report Rx-Tx time difference measurements, it may report associated RxTx TEG ID. If the RxTx TEG ID is reported, the UE may also report a Tx TEG ID as the following. That is, for each Rx-Tx time difference measurement, UE may report a pair of {RxTx TEG, Tx TEG}.

37355 v17.0.0 NR-UE-RxTx-TEG-Info-r17 ::= CHOICE { -- FFS if the CHOICE structure is needed case1-r17 SEQUENCE { nr-UE-RxTx-TEG-ID-r17 INTEGER (0..maxNumOfRxTxTEGs-1-r17) }, case2-r17 SEQUENCE { nr-UE-RxTx-TEG-ID-r17 INTEGER (0..maxNumOfRxTxTEGs-1-r17), nr-UE-Tx-TEG-Index-r17 INTEGER (1..maxTxTEG-Sets-r17) }, case3-r17 SEQUENCE { nr-UE-Rx-TEG-ID-r17 INTEGER (0..maxNumOfRxTEGs-1-r17), nr-UE-Tx-TEG-Index-r17 INTEGER (1..maxTxTEG-Sets-r17) }, ...

In addition, UE may also report SRS-TxTEG association in ‘NR-Multi-RTT-SignalMeasurementInformation’ with SRS-TxTEG association set as the following.

NR-Multi-RTT-SignalMeasurementInformation-r16 ::= SEQUENCE { nr-Multi-RTT-MeasList-r16 NR-Multi-RTT-MeasList-r16, nr-NTA-Offset-r16 ENUMERATED { nTA1, nTA2, nTA3, nTA4, ... } OPTIONAL, ..., [[ nr-SRS-TxTEG-Set-r17 SEQUENCE (SIZE(1..maxTxTEG-Sets-r17)) OF NR-SRS-TxTEG-Element-r17 OPTIONAL -- Cond Case2-3 ]] }

It is possible that the Tx TEGs in SRS-TxTEG-set will include the Tx TEG of the above {RxTx TEG, Tx TEG} pair, and other Tx TEGs different from the above Tx TEG. Then, when gNBs measures SRS resource associated with the other Tx TEGs and LMF combine measurements from UE and gNB, there will be no information of RxTx TEG. So, for each Rx-Tx time difference measurement for a PRS resource, the UE may report one or more pairs of {RxTx TEG, Tx TEG}, which are used to represent the mapping relationship with all Tx TEGs of all SRS resources(e.g. all Tx TEGs in nr-SRS-TxTEG-Set-r17).

- For each Rx-Tx time difference measurement for a PRS resource, the UE may report one or more pairs of {RxTx TEG, Tx TEG}, which are used to represent the mapping relationship with all Tx TEGs of all SRS resources (e.g. all Tx TEGs in nr-SRS-TxTEG-Set-r17).

TEG timing error margin

In RAN4 LS [3], the following agreement regarding TEG timing error margin was achieved.

- The framework of UE/TRP Rx TEG: - Define multiple candidate timing error margin values {TE1, TE2, …, TEN} in the spec. - The number of candidate values (i.e. N) and the exact values of {TE1, TE2, …, TEN} will be decided in Perf part. - UE/TRP selects one value M from {TE1, TE2, …, TEN} based on its implementation and indicate to LMF. - For UE that supports multiple Rx TEGs (TEG#1, TEG#2, …), the associated timing error margin value of each Rx TEG is M, which means the timing error difference between the measurements within the same Rx TEG is within the margin M. - The applicability of reported UE Rx TEG is limited to the measurements contained within the measurement report in which the Rx TEG information is provided, and only to measurements that are tagged with the corresponding TEG ID. - The RRM accuracy requirements corresponding to the candidate timing error margin values {TE1, TE2, …, TEN} will be defined in Perf part. - The framework of UE/TRP Rx TEG can be also applied for UE/TRP RxTx TEG - Note: if additional issues are identified based on RAN1/2 progress, then this agreement can be revised.

It should be noted that UE/TRP selects one value M from {TE1, TE2, …, TEN} based on its implementation and indicate to LMF. So, from RAN1 perspective, for a UE, how to indicate the selected time error margin value to LMF should be discussed.

From our point of view, UE can select Rx timing error margin before UE capability report. Based on the margin, the Rx errors can be grouped and the maximum number of Rx TEGs for each band can be determined. Then, UE can report the supported number of Rx TEGs along with Rx timing error margin via UE capability signaling.

In addition, regarding the type of Rx timing error margin, ‘per UE’ type can be supported according to the following RAN4 agreement.

Agreements: - The same timing margin is used for all Rx TEGs per UE/TRP

Similarly, per UE type capability of RxTx timing error margin for RxTx TEG can be supported.

For timing error margin of Tx TEG, we can wait for further RAN4 conclusion.

- Support the UE capability of timing error margin for Rx TEG and RxTx TEG.

- The capability type is per UE.

Remaining issues on DL-AOD enhancements

The maximum number of PRS resources in a subset

Proposal 3.8.1: The maximum number of PRS resources in a PRS subset the following values: {2,4,8,16,24} Proposal 3.8-2 Each PRS resource in the PRS subset is identified with : - PRS resource set ID - PRS resource ID

The maximum number of PRS resources in a PRS subset has been discussed in RAN1#108-e, but no consensus has been made. Some companies have some concerns about multiple values being defined and suggest only 24 for the maximum number. In our view, the subset is supported for the following cases (adjacent beams and two-stage beams) for beam selection for AoD reporting. Based on the following figure, we think the adjacent beams for the target resource don’t need to be larger than 8 and the two-stage beam is also similar. In this case, 8 is a relatively reasonable choice for the maximum number of PRS resources in a PRS subset

In addition, we prefer the number of resources within a subset transmitted to UE isn’t larger than the maximum number of resources that UE reporting.

(a) use case 1(adjacent beam)                                                     (b) use case 2(two-stage beam)

Figure 1 The example use case for a subset

So, we propose:

- The maximum number of PRS resources in a PRS subset is 8.

- The number of resources within a subset transmitted to UE should not be larger than the maximum number of resources that UE reporting

TRP beam information

LS in R3-222721[4] Based on the assistance information for DL-AoD specified by RAN1/RAN2, the corresponding NRPPa signaling could require, in the case where maximum granularity is used uniformly in azimuth and elevation, an excess of 6 million relative powers per TRP / resource to be signaled over NRPPa and, as consequence, via F1-AP and per NG-RAN design, over NG transport. RAN3 assumes (and would like to confirm) that realistic implementations would not require this high level of data volume traffic and would also use this function sparingly. RAN3 has agreed to include some mitigations over NRPPa/F1AP, e.g. allowing for the indication of “no change” if a previous TRP beam antenna configuration is still valid. Note that, as with other TRP configuration items, RAN3 has agreed that OAM is also a possible option for providing such information to LMF.

Based on the assistance information for DL-AoD, the assistance information for one resource may excess of 6 million(ie, 3600*1800) relative powers per TRP / resource to be signaled over NRPPa.

NR-TRP-BeamAntennaAngles-r17 ::= SEQUENCE (SIZE(1..3600)) OF NR-TRP-BeamAntennaInfoAzimuthElevation-r17 NR-TRP-BeamAntennaInfoAzimuthElevation-r17 ::= SEQUENCE { azimuth-r17 INTEGER (0..359) OPTIONAL, -- Cond Az azimuth-fine-r16 INTEGER (0..9) OPTIONAL, -- Need ON elevationList-r17 SEQUENCE (SIZE(1..1801)) OF ElevationElement-R17, ... } ElevationElement-R17 ::= SEQUENCE { elevation-r17 INTEGER (0..180) OPTIONAL, -- Cond El elevation-fine-r17 INTEGER (0..9) OPTIONAL, -- Need ON beamPowerList-r17 SEQUENCE (SIZE (2..maxNumResourcesPerAngle-r17)) OF BeamPowerElement-r17, ... } BeamPowerElement-r17 ::= SEQUENCE { nr-dl-prs-ResourceSetID-r17 NR-DL-PRS-ResourceSetID-r16 OPTIONAL, -- Need OP nr-dl-prs-ResourceID-r17 NR-DL-PRS-ResourceID-r16, nr-dl-prs-RelativePower-r17 INTEGER (0..500), -- FFS ... }

Firstly, we agree with RAN3's concerns and agree with RAN3-related enhancements. Furthermore, in the perspective of RAN1, the angle range for beam antenna information and the granularity of angle can be updated for the reduction of reporting. For example, the maximum angle space of a resource/TRP can be 180 degrees

given there is no signal in the direction of the backplane of the TRP. So, we prefer the angle range can be [-90, 90] for omnidirectional antenna and [-60, 60] for a directional antenna.

Figure 2 The positioning performance with different angle granularity

In addition, the angle granularity can be increased, we evaluate the performance gain for different angle granularities (such as 0.5, 1, 2, 4 degrees) as figure 2. Based on the evaluation, it is observed that the 0.5/1 degree can be achieved sub-meter accuracy. So, in our view, to balance the performance and overhead, 1 degree granularity is enough for high accuracy positioning,

- Support the following angle range and angle granularity for relative Power/Angle response

- [-90, 90] for omnidirectional antenna and [-60, 60] for directional antenna

- 0 degree is represented as the boresight of the panel of TRP

- The granularity angle can be  1 degree so that the maximum element can be reduced by 100 times

Remaining issues on multi-path/NLOS enhancements

LoS/NLoS indicators

For LoS/NLoS indicators, it has been discussed in a couple of meetings that a single-indicator can be reported and the supported values are a discrete set in the interval [0, 1], which does not preclude using binary values only which is up to UE/TRP implementation.

Agreement: For LoS/NLoS indicators, a single-indicator can be reported and the supported values are a discrete set in the interval [0, 1]. - FFS: the number of discrete values to be supported - Note: This does not preclude using binary values only which is up to UE/TRP implementation Note: Single-indicator means that one value in the interval [0, 1] is used for the LoS/NLoS indication

In RAN1 #108-e meeting, the UE feature for LOS/NLOS indicator was agreed as the following:

We have presented our concern in previous contribution [5] for ambiguousness of LoS/NLoS indicator reporting and have given two options to solve this problem.

- Option1: Support the additional UE capability of which type of LoS/NLoS indicators the UE is supportive. When gNB or LMF receives different UE capabilities, it can identify the meaning of 0 and 1 according to the reported UE capability.

- Option2: Support to differentiate the type of LOS/NLOS indicators by the LoS-NLos-Indicator IE, which can include hard value and soft value. Then LMF can have the knowledge of whether 0 and 1 corresponds to hard value or soft value.

In last meeting, it has been agreed that the UE feature for LOS/NLOS indicator includes reporting LoS/NLoS indicator type to LMF, while the component of candidate values is still being discussed. We think if a UE supports LOS/NLOS indicator of soft value type, it will of course support the hard value 0 and 1. Therefore, for Option1, we think only supporting soft value LOS/NLOS indicator is not needed and the capability of both hard and soft value should be supported. And for Option2, we think the hard value type is a baseline which should be supported as a default value.

But for Option1, when a UE is supportive of both hard value and soft value, LMF still cannot distinguish whether the [0, 1] is reported as a hard value which only means the link is detected as LoS/NLoS but the confidence is unknown or a soft value which includes LoS/NLoS confidence information. Therefore, we suggest to differentiate the type of LOS/NLOS Indicator by the LoS-NLos-Indicator IE, where the hard value type should be supported as a default value.

- For UE’s capability to support reporting LoS/NLoS indicator

- Support component 1 candidate value {hard value, both hard value and soft value}

- Support to differentiate the type of LOS/NLOS Indicator by the LoS-NLos-Indicator IE, where the hard value type should be supported as a default value

Conclusion

In this contribution, we discuss remaining issues on accuracy enhancements with the following proposals.

- For each Rx-Tx time difference measurement for a PRS resource, the UE may report one or more pairs of {RxTx TEG, Tx TEG}, which are used to represent the mapping relationship with all Tx TEGs of all SRS resources (e.g. all Tx TEGs in nr-SRS-TxTEG-Set-r17).

- Support the UE capability of timing error margin for Rx TEG and RxTx TEG.

- The capability type is per UE.

- The maximum number of PRS resources in a PRS subset is 8.

- The number of resources within a subset transmitted to UE should not be larger than the maximum number of resources that UE reporting

- Support the following angle range and angle granularity for relative Power/Angle response

- [-90, 90] for omnidirectional antenna and [-60, 60] for directional antenna

- 0 degree is represented as the boresight of the panel of TRP

- The granularity angle can be  1 degree so that the maximum element can be reduced by 100 time

- For UE’s capability to support reporting LoS/NLoS indicator

- Support component 1 candidate value {hard value, both hard value and soft value}

- Support to differentiate the type of LOS/NLOS Indicator by the LoS-NLos-Indicator IE, where the hard value type should be supported as a default value

- References

- Chairman’s notes for 3GPP TSG RAN WG1 Meeting 108 e-Meeting, February 21st – March 3rd, 2022

- 3GPP TS 37.355, “LTE Positioning protocol (LPP)”, v17.0.0

- R4-2206998, LS on the UE/TRP TEG framework, TSG RAN WG4 Meeting #102-e, February 21st – March 3rd, 2022

- R3-222721, Questions concerning the implementation of RAN1 agreements in NRPPa, RAN3#115-e, February 21st – March 3rd, 2022

- R1-2111017, Discussion on potential enhancements for multipath/NLoS mitigation, vivo