3GPP TSG RAN WG1 #109-e		R1-2203518

e-Meeting, May 9th – May 20th, 2022

Source:	vivo, Guangdong Genius

Title:                     Remaining issues on RedCap half-duplex operation

Agenda Item:	8.6.1.2

Document for:	Discussion and Decision

- Introduction

In the RAN1#108-e meeting, following working assumption and conclusion were made for FDD half-duplex RedCap UEs [1]:

Working Assumption - For Case 5 of SSB overlapping with Msg3 (re)transmission or PUCCH for Msg4/MsgB, reuse the same handling as for other dynamically scheduled UL transmission and prioritize the SSB - Note: Whether the above collision rule is reused for Msg3 PUSCH repetition is up to the agreement in the CE WI Conclusion: - For collision handling, HD-FDD RedCap UEs do not support [partialCancellation] in Rel-17 - Note: No specification impact is expected

In this contribution, we provide our views on the remaining issues on half duplex FDD operation for RedCap.

- Case 5 of Msg3 (re)transmission and PUCCH for Msg4 vs. SSB

About to confirm the WA to reuse the same handling as for other dynamically scheduled UL transmission and prioritize the SSB for case 5 of SSB overlapping with Msg3 (re)transmission or PUCCH for Msg4/MsgB. One discussion point is about which collision cases are covered by the WA, more specifically, whether the WA is also applied to the case of separate initial DL BWP without CD-SSB.

Based on the discussion in the AI 8.6.1.1, in case the separate initial DL BWP without CD-SSB is configured for a RedCap UE, the RedCap UE in idle/inactive state should NOT assume there is any NCD-SSB is configured within the separate initial DL BWP and the separate initial DL BWP is only used for random access. In addition, RAN1 expect that during the RACH procedure, e.g. from the transmission of the Msg.1 to the transmission of the PUCCH for Msg.4, the RedCap UE in idle/inactive mode does not need to monitor paging or CD-SSB. About RAN2’s agreement, see below, it is for the case before Msg.1 transmission, not applied to the case of during the RACH procedure for Msg.3 and PUCCH for Msg.4 transmission.

- From RAN2 perspective, if a RedCap UE in idle/inactive mode is configured with a separate initial BWP associated with no SSB (CD or NCD) for RACH, it is up to UE implementation to perform new RSRP measurement in a DL BWP associated with CD-SSB before Msg1/A retransmission.

For the case that the separate initial DL BWP without CD-SSB is configured for a RedCap UE in RRC connected state, at least for BWP#0 configuration option 2, NCD-SSB needs to be configured in the separate initial DL BWP. It should be emphasized that although the NCD-SSB is configured by UE dedicated RRC signaling i.e., NonCellDefiningSSB in BWP-DownlinkDedicated, to reduce the NW overhead and simplify the operation, the same NCD-SSB should be configured for all RedCap UEs in the separate initial DL BWP. In short, the NCD-SSB is actually cell specific, just configured by UE specific signaling. Therefore, there may be collision between the Msg.3 transmission and NCD-SSB, and for such collision case, the NCD-SSB should be prioritized. For RedCap UEs in connected mode without supporting the optional FG6-1a like feature, to avoid the BWP switching for CD-SSB reception, NCD-SSB in the separate initial DL BWP should be configured. In summary, the collision cases involving SSB should only consider the SSB in the active BWP. Therefore, we think above WA should be confirmed with clarifying that the “SSB” in the previous agreement for collision cases should be the SSB that present in the active BWP.

Proposal 1: Confirm the following WA:

Working Assumption

- For Case 5 of SSB overlapping with Msg3 (re)transmission or PUCCH for Msg4/MsgB, reuse the same handling as for other dynamically scheduled UL transmission and prioritize the SSB

  - Note: Whether the above collision rule is reused for Msg3 PUSCH repetition is up to the agreement in the CE WI

Proposal 2: Clarify that for the collision handling between SSB and UL transmissions i.e., dynamically scheduled or configured UL transmission, the SSB refers to the one that present in the active BWP.

Proposal 3: Adopt the following TP if Proposal 1 and 2 are agreed.

Reason for change: A RedCap UE can be configured with CD-SSB or NCD-SSB in a BWP. For HD-FDD RedCap UE to handle the collision cases involving SSB, it is not clear which SSB is covered by the collision cases. Summary of change: - Clarify that the collision handling between SSB and UL transmissions i.e., dynamically scheduled or configured UL transmission, the SSB refers to the one that present in the active BWP. - Clarify that SSB can also be configured by NonCellDefiningSSB in addition to ssb-PositionsInBurst. Consequences if not approved: HD-FDD RedCap UE behavior is not clear for handling the collision cases involving the SSB. 17.2 Half-Duplex UE in paired spectrum [Omit Irrelevant Text] If a HD-UE would transmit a PUSCH, or PUCCH, or SRS in an active UL BWP based on a configuration by higher layers and the HD-UE is indicated presence of SS/PBCH blocks in the active DL BWP by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or by NonCellDefiningSSB, the HD-UE does not transmit - PUSCH or PUCCH if a last symbol of the PUSCH or PUCCH transmission would not be at least NTx-Rx⋅Tc [4, TS 38.211] prior to a first symbol of the next earliest SS/PBCH block - PUSCH or PUCCH if a first symbol of the PUSCH or PUCCH transmission would not be at least NRx-Tx⋅Tc [4, TS 38.211] after a last symbol of the previous latest SS/PBCH block - SRS in symbols that would not be at least NTx-Rx⋅Tc prior to a first symbol of the next earliest SS/PBCH block - SRS in symbols that would not be at least NRx-Tx⋅Tc after a last symbol of the previous latest SS/PBCH block If a HD-UE would transmit a PRACH based on a detected DCI format, or PUSCH, or PUCCH, or SRS in an active UL BWP and the HD-UE is indicated presence of SS/PBCH blocks in the active DL BWP by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or by NonCellDefiningSSB in a set of symbols, the HD-UE does not transmit PUSCH or PUCCH or PRACH if a transmission would overlap with any symbol from the set of symbols and the HD-UE does not transmit SRS in the set of symbols. If a HD-UE would transmit a PRACH or MsgA PUSCH triggered by higher layers in a set of symbols and would receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS, or is indicated presence of SS/PBCH blocks in the active BWP by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or by NonCellDefiningSSB in symbols that include any symbol from the set of symbols, the HD-UE can select based on its implementation whether to either transmit the PRACH or the MsgA PUSCH or receive the PDSCH, or the CSI-RS, or the PL RS, or the PDCCH, or the SS/PBCH blocks. If a HD-UE would receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS based on a configuration by higher layers or is indicated presence of SS/PBCH blocks in the active DL BWP by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or by NonCellDefiningSSB in a set of symbols, and the HD-UE would transmit in the active UL BWP PRACH or MsgA PUSCH triggered by higher layers starting or ending at a symbol that is earlier or later than NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol in the set of symbols, the HD-UE can select based on its implementation whether to either transmit the PRACH or the MsgA PUSCH or receive the PDSCH, or the CSI-RS, or the DL PRS, or the PDCCH, or the SS/PBCH blocks. [Omit Irrelevant Text]

- Avalible slot determination for PUCCH and PUSCH

In the last meeting, there was some discussions about HD-FDD UE behaviour on determining the available slots for PUCCH and PUSCH repetition, especially for the case that when a PUCCH/PUSCH repetition does not overlap with a symbol with SSB, but the switching gap between the back-to-back PUCCH/PUSCH repetition and SSB is not sufficient. Related to this issue, the agreements made in RedCap WI and Coverage enhancements WI are listed in the following:

Agreement made in RedCap WI:  The “back-to-back” non-overlapping UL/DL without sufficient gap between cell-specific configured DL and dedicated configured UL may happen, i.e., allowed for HD-FDD UEs - E.g., SSB vs. CG PUSCH, PUCCH or SRS - Configured UL transmission is cancelled (as in the overlapping case) Agreements made in Coverage enh. WI: - For DG-PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, the legacy counting method applies when K=1, regardless of whether AvailableSlotCounting is enabled or not - Note: The legacy assumption on the K2 offset is applied, i.e., no RAN1 spec impact on the K2 offset is expected. - For CG-PUSCH repetition Type A, the legacy counting method applies when K=1, regardless of whether AvailableSlotCounting is enabled or not. - Note: No RAN1 spec impact is expected. - Note: No additional restriction on CG periodicity configurations, compared to Rel-16, is considered for CG-PUSCH with K=1. - For CG-PUSCH repetition Type A, when AvailableSlotCounting is enabled, and for K>1, a UE assumes that the slot (i.e., the first configured slot in a CG period) determined in 38.321 Section 5.8.2 can be the slot which is not counted in K available slots. - Note: No RAN1 spec impact is expected.

Based on above agreements, for HD-FDD RedCap UE, to determine the available slot/UE behavior for PUCCH and PUSCH repetition when there is no sufficient switching gap with respect to the SSB, following conditions/points need to be taken into account:

- A: AvailableSlotCounting enabled or disabled

- B: Resource overlapping between SSB and UL transmission or Back-to-back SSB/UL transmission without sufficient switching gap

- C: Repetition factor K=1 or K>1

- D: Configured UL transmission (e.g. CG, Configured PUCCH) or Dynamic scheduled UL transmission (e.g. DG, Dynamic PUCCH)

Without combining the Rel-17 Cov_enh. feature of AvailableSlotCounting or when AvailableSlotCounting is disabled, Table 1 summarizes the current HD-FDD UE behavior for above condition B, C and D.

Table 1:  HD-FDD UE behavior for UL transmission vs. SSB without/disabling AvailableSlotCounting

|  | CG/Configured PUCCH for Condition C (K=1 and K>1) | DG/Dynamic PUCCH for Condition C (K=1 and K>1) |
| --- | --- | --- |
| Condition B: Resource overlapping between SSB and UL Tx | Drop CG/configured PUCCH | Drop DG/Dynamic PUCCH |
| Condition B: Back-to-back SSB/UL Tx w/o sufficient switching gap | Drop CG/configured PUCCH | Error case (if no agreement made for other back-to-back case) |

It is observed that without combining the AvailableSlotCounting feature or disabling AvailableSlotCounting, HD-FDD UE behavior is aligned for condition B and C for configured UL transmission. For dynamic UL transmission, HD-FDD UE behavior is aligned for C, but NOT aligned for B if no new agreement to allow the “back-to-back” non-overlapping without sufficient gap between SSB and dynamic scheduled UL is made. In general, HD-FDD UE behavior is still simple without considering the AvailableSlotCounting.

Observation 1: Without considering AvailableSlotCounting feature (e.g., not configure or disable such feature), the HD-FDD RedCap UE has two different behaviors for handling the case of “back-to-back” non-overlapping dynamically scheduled UL transmission and SSB without sufficient gap and other cases if the case of “back-to-back” non-overlapping dynamically scheduled UL transmission and SSB is considered as error case.

However, when AvailableSlotCounting is enabled, if we follow Cov_enh. agreements for condition C, i.e, K=1 and K>1 to define the HD-FDD UE behavior, then UE behavior and specification will become quite complex especially for dynamic UL transmission. Since different handling is required for condition B that resource overlapping vs insufficient switching gap and C that K=1 vs. K>1 as shown in Table 2.

Table 2:  HD-FDD UE behavior for Dynamic UL transmission vs. SSB with enabling AvailableSlotCounting

|  | K=1 (Apply legacy counting method regardless of AvailableSlotCounting) | K>1 |
| --- | --- | --- |
| Condition B: Resource overlapping between SSB and UL Tx | Drop DG/Dynamic PUCCH | Postpone DG/Dynamic PUCCH repetition |
| Condition B: Back-to-back SSB/UL Tx w/o sufficient switching gap | Error case | FFS - Option 1: Postpone DG/Dynamic PUCCH - Option 2: Drop DG/ Dynamic PUCCH - Option 3: error case |

From Table 2, it is observed that when AvailableSlotCounting is enabled, there already are three different UE behaviors. Depending on the decision for the case of back-to-back SSB/UL transmission without sufficient switching gap when K>1, there could be 4 different UE behaviors if option 2 that drop DG/ Dynamic PUCCH is adopted.

Observation 2: For dynamic scheduled UL transmission vs. SSB, when AvailableSlotCounting is enabled, at least three different UE behaviors are required for the cases with resource overlapping or without sufficient switching gap and with K=1 or K>1.

Similarly, Table 3 summarizes the UE behavior for Configured UL transmission vs. SSB with resource overlapping or without sufficient switching gap when AvailableSlotCounting is enabled.

Table 3:  HD-FDD UE behavior for Configured UL transmission vs. SSB with enabling AvailableSlotCounting

|  | K=1 (Apply legacy counting method regardless of AvailableSlotCounting) | K>1 |
| --- | --- | --- |
| Resource overlapping between SSB and UL Tx | Drop CG/configured PUCCH | Postpone CG/configured PUCCH transmission |
| Back-to-back SSB/UL Tx w/o sufficient switching gap | Drop CG/configured PUCCH | FFS - Option 1: Postpone CG/configured PUCCH - Option 2: Drop CG/configured PUCCH |

Observation 3: For configured UL transmission vs. SSB, when AvailableSlotCounting is enabled, two different UE behaviors are required for the cases with resource overlapping or without sufficient switching gap and with K=1 or K>1.

As observed, based on AvailableSlotCounting is enabled or disabled, Repetition factor K=1 or K>1, the UL transmission is CG or DG, different UE behaviors may be required to handle the SSB vs. UL transmission with resource overlapping or without sufficient switching gap, which involves quite complexity from both UE and specification perspective. To simplify it, following Alternatives are worthy to be discussed.

- Alt.1: UE behvaior is not differentiate for K=1 and K>1.

Table 4:  HD-FDD UE behavior for UL transmission vs. SSB with enabling AvailableSlotCounting

|  | CG/Configured PUCCH for both K=1 and K>1 | DG/Dynamic PUCCH for both K=1 and K>1 |
| --- | --- | --- |
| Resource overlapping between SSB and UL Tx | Postpone CG/configured PUCCH transmission | Postpone DG/Dynamic PUCCH repetition |
| Back-to-back SSB/UL Tx w/o sufficient switching gap | Postpone CG/configured PUCCH transmission | - Option 1: Postpone DG/Dynamic PUCCH |

For Alt.1, it is the simplest if option 1 is adopted or collision handling, unified UE behaviour can be achieved for all conditions/cases. But it does not align with agreement made in Cov_enh. for K=1 and K>1. In addition, it needs to make a new agreement to allow the “back-to-back” non-overlapping dynamic scheduled UL transmission and SSB without sufficient gap may happen.

- Alt.2: UE behvaior is not differentiate for resource overlapping vs insufficient switching gap

Table 5:  HD-FDD UE behavior for UL transmission vs. SSB with/enabling AvailableSlotCounting

|  | CG/Configured PUCCH for Resource overlapping/insufficient switching gap with SSB | DG/Dynamic PUCCH for Resource overlapping/insufficient switching gap with SSB |
| --- | --- | --- |
| K=1 | Drop CG/configured PUCCH | - Option 1: Drop DG/ Dynamic PUCCH |
| K>1 | Postpone CG/configured PUCCH | Postpone DG/Dynamic PUCCH |

For Alt.2, it is aligned with agreement made in Cov_enh. for K=1 and K>1. For the case of “back-to-back” non-overlapping dynamic scheduled UL transmission and SSB without sufficient gap, if Option 1 is adopted, UE behaviour is still simple and only differentiate for K=1 and K>1; Otherwise, three UE behaviours are required if option 2 is selected. Similar as Alt.1, it needs to make a new agreement to allow the “back-to-back” non-overlapping dynamic scheduled UL transmission and SSB without sufficient gap happen.

Proposal 4: To simplify UE behaviour and specification efforts, the “back-to-back” non-overlapping UL/DL without sufficient gap between SSB and dynamic scheduled UL may happen, i.e., allowed for HD-FDD UEs.

Proposal 5: Down-select between following Alternatives when AvailableSlotCounting is disabled/enabled for HD-FDD UE:

Alt. 1:

- For PUSCH repetition with a configured grant or scheduled by a PDCCH and for PUCCH repetition semi-statically configured or scheduled by a PDCCH,

  - When AvailableSlotCounting is enabled and for both K=1 and K>1, a slot is not counted in the number of K slots for PUSCH repetitions when a PUSCH/PUCCH repetition starts or ends not at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol in the set of symbols with SSB transmission.

  - When AvailableSlotCounting is NOT enabled and for both K=1 and K>1, HD-FDD UE does not transmit the PUSCH repetition when a PUSCH repetition starts or ends not at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol in the set of symbols with SSB transmission.

Alt. 2:

- For PUSCH repetition with a configured grant or scheduled by a PDCCH and for PUCCH repetition semi-statically configured or scheduled by a PDCCH,

  - When AvailableSlotCounting is enabled,

    - for K>1, a slot is not counted in the number of K slots for PUSCH repetitions when a PUSCH repetition starts or ends or not at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol in the set of symbols with SSB transmission.

    - for K=1, HD-FDD UE does not transmit the PUSCH repetition when a PUSCH repetition starts or ends not at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol in the set of symbols with SSB transmission.

  - When AvailableSlotCounting is NOT enabled, for both K=1 and K>1, HD-FDD UE does not transmit the PUSCH repetition when a PUSCH repetition starts or ends not at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol in the set of symbols with SSB transmission.

- Conclusion

This contribution discusses remaining issues for HD-FDD operation for RedCap. The observations and proposals are summarized as following:

Proposal 1: Confirm the following WA:

Working Assumption

- For Case 5 of SSB overlapping with Msg3 (re)transmission or PUCCH for Msg4/MsgB, reuse the same handling as for other dynamically scheduled UL transmission and prioritize the SSB

  - Note: Whether the above collision rule is reused for Msg3 PUSCH repetition is up to the agreement in the CE WI

Proposal 2: Clarify that for the collision handling between SSB and UL transmissions i.e., dynamically scheduled or configured UL transmission, the SSB refers to the one that present in the active BWP.

Proposal 3: Adopt the following TP if Proposal 1 and 2 are agreed.

Reason for change: A RedCap UE can be configured with CD-SSB or NCD-SSB in a BWP. For HD-FDD RedCap UE to handle the collision cases involving SSB, it is not clear which SSB is covered by the collision cases. Summary of change: - Clarify that the collision handling between SSB and UL transmissions i.e., dynamically scheduled or configured UL transmission, the SSB refers to the one that present in the active BWP. - Clarify that SSB can also be configured by NonCellDefiningSSB in addition to ssb-PositionsInBurst. Consequences if not approved: HD-FDD RedCap UE behavior is not clear for handling the collision cases involving the SSB. 17.2 Half-Duplex UE in paired spectrum [Omit Irrelevant Text] If a HD-UE would transmit a PUSCH, or PUCCH, or SRS in an active UL BWP based on a configuration by higher layers and the HD-UE is indicated presence of SS/PBCH blocks in the active DL BWP by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or by NonCellDefiningSSB, the HD-UE does not transmit - PUSCH or PUCCH if a last symbol of the PUSCH or PUCCH transmission would not be at least NTx-Rx⋅Tc [4, TS 38.211] prior to a first symbol of the next earliest SS/PBCH block - PUSCH or PUCCH if a first symbol of the PUSCH or PUCCH transmission would not be at least NRx-Tx⋅Tc [4, TS 38.211] after a last symbol of the previous latest SS/PBCH block - SRS in symbols that would not be at least NTx-Rx⋅Tc prior to a first symbol of the next earliest SS/PBCH block - SRS in symbols that would not be at least NRx-Tx⋅Tc after a last symbol of the previous latest SS/PBCH block If a HD-UE would transmit a PRACH based on a detected DCI format, or PUSCH, or PUCCH, or SRS in an active UL BWP and the HD-UE is indicated presence of SS/PBCH blocks in the active DL BWP by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or by NonCellDefiningSSB in a set of symbols, the HD-UE does not transmit PUSCH or PUCCH or PRACH if a transmission would overlap with any symbol from the set of symbols and the HD-UE does not transmit SRS in the set of symbols. If a HD-UE would transmit a PRACH or MsgA PUSCH triggered by higher layers in a set of symbols and would receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS, or is indicated presence of SS/PBCH blocks in the active BWP by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or by NonCellDefiningSSB in symbols that include any symbol from the set of symbols, the HD-UE can select based on its implementation whether to either transmit the PRACH or the MsgA PUSCH or receive the PDSCH, or the CSI-RS, or the PL RS, or the PDCCH, or the SS/PBCH blocks. If a HD-UE would receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS based on a configuration by higher layers or is indicated presence of SS/PBCH blocks in the active DL BWP by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or by NonCellDefiningSSB in a set of symbols, and the HD-UE would transmit in the active UL BWP PRACH or MsgA PUSCH triggered by higher layers starting or ending at a symbol that is earlier or later than NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol in the set of symbols, the HD-UE can select based on its implementation whether to either transmit the PRACH or the MsgA PUSCH or receive the PDSCH, or the CSI-RS, or the DL PRS, or the PDCCH, or the SS/PBCH blocks. [Omit Irrelevant Text]

Proposal 4: To simplify UE behaviour and specification efforts, the “back-to-back” non-overlapping UL/DL without sufficient gap between SSB and dynamic scheduled UL may happen, i.e., allowed for HD-FDD UEs.

Proposal 5: Down-select between following Alternatives when AvailableSlotCounting is disabled/enabled for HD-FDD UE:

Alt. 1:

- For PUSCH repetition with a configured grant or scheduled by a PDCCH and for PUCCH repetition semi-statically configured or scheduled by a PDCCH,

  - When AvailableSlotCounting is enabled and for both K=1 and K>1, a slot is not counted in the number of K slots for PUSCH repetitions when a PUSCH/PUCCH repetition starts or ends not at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol in the set of symbols with SSB transmission.

  - When AvailableSlotCounting is NOT enabled and for both K=1 and K>1, HD-FDD UE does not transmit the PUSCH repetition when a PUSCH repetition starts or ends not at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol in the set of symbols with SSB transmission.

Alt. 2:

- For PUSCH repetition with a configured grant or scheduled by a PDCCH and for PUCCH repetition semi-statically configured or scheduled by a PDCCH,

  - When AvailableSlotCounting is enabled,

    - for K>1, a slot is not counted in the number of K slots for PUSCH repetitions when a PUSCH repetition starts or ends or not at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol in the set of symbols with SSB transmission.

    - for K=1, HD-FDD UE does not transmit the PUSCH repetition when a PUSCH repetition starts or ends not at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol in the set of symbols with SSB transmission.

  - When AvailableSlotCounting is NOT enabled, for both K=1 and K>1, HD-FDD UE does not transmit the PUSCH repetition when a PUSCH repetition starts or ends not at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol in the set of symbols with SSB transmission.

5. Reference

- 3GPP RAN1#108-e meeting, Chairman’s notes.