3GPP TSG RAN WG1 #109-e	                                                                                   R1-2203522

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Remaining issues on joint channel estimation

Agenda Item:	8.8.2

Document for:	Discussion and Decision

- Introduction

In this contribution, some remaining issues are further discussed in this contribution for support of joint channel estimation, including

- joint channel estimation for PUSCH,

- joint channel estimation for PUCCH.

- Joint channel estimation for PUSCH

2.1 UE capability of restarting DMRS bundling

In RAN1#106 e-meeting, the working assumption of TDW mechanism is achieved and UE capability of restarting DMRS bundling is included, which is confirmed in RAN1#107 e-meeting as followed.

Agreement The following working assumption is confirmed Working assumption: For joint channel estimation for PUSCH repetition type A of PUSCH repetitions of the same TB, all the repetitions are covered by one or multiple consecutive/non-consecutive configured TDWs. …… - Within one configured TDW, one or multiple actual TDWs can be implicitly determined: …… - If the power consistency and phase continuity are violated due to an event, whether a new actual TDW is created is subject to UE capability of supporting restarting DMRS bundling. - If UE is capable of restarting DM-RS bundling, one new actual TDW is created after the event, - FFS: The start of the new actual TDW is the first available slot/symbol for PUSCH transmission after the event. - If UE is not capable of restarting DM-RS bundling, no new actual TDW is created until the end of the configured TDW. - FFS: UE capability of restarting DMRS bundling is applied only to dynamic event or not

Following two cases require further clarification, considering that if UE is not capable of restarting DM-RS bundling, no new actual TDW is created until the end of the configured TDW.

- Case 1: A semi-static event is triggered after one or multiple dynamic events. Whether a new actual TDW is created after the semi-static event?

- Case 2: A semi-static event overlaps with a dynamic event. Whether a new actual TDW is created after the semi-static event?

However, it is worth noting that the FFS sub-bullet indicating whether the UE capability is applied only to dynamic event or not should be further studied. And in RAN1#107-e meeting, the following agreement was also made:

Agreement - If DM-RS bundling is supported, UE is mandatory to support restarting DM-RS bundling due to semi-static events. UE capability of restarting DMRS bundling is applied only to dynamic events. - An event is regarded as a dynamic event if it is triggered by a DCI or MAC-CE, otherwise it is regarded as a semi-static event. - Note: At least frequency hopping event is considered as semi-static event.

It is clear that UE capability of restarting DMRS bundling is applied only to dynamic events, and UE is mandatory to support restarting DM-RS bundling due to semi-static events even if UE is not capable of restarting DM-RS bundling. Furthermore, once an event that violates phase continuity and power consistency is defined, the event should be also categorized as a semi-static event or a dynamic event. Thus, for both Case 1 and Case 2, if UE is not capable of restarting DM-RS bundling, a new actual TDW should be created after the semi-static event.

Furthermore, according to TS38.214 V17.1.0 section 6.1.7 as listed below, UE capability of restarting an actual TDW is applied only to dynamic events. No specification change is necessary. However, considering the misalignment on Case 1 and Case 2, a conclude should be needed.

The UE shall maintain power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to frequency hopping, or in response to the use of a different SRS resource set association for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, or in response to the use of different spatial relations or different power control parameters for the two PUCCH transmissions of PUCCH repetition, or in response to any event not triggered by DCI or MAC-CE. The UE maintains power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to an event triggered by DCI other than frequency hopping or by MAC-CE, subject to UE capability.

Proposal 1: RAN1 to conclude that if UE is not capable of restarting DM-RS bundling, when a semi-static event occurs after one or multiple dynamic events or overlaps with a dynamic event, a new actual window should be created after the semi-static event.

2.2 TPC command handling for PUSCH

In RAN1 #107e, the following working assumption was achieved for group common TPC commands.

Working assumption: - The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity. - If UE is configured to accumulate TPC commands, - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW. - If UE is not configured to accumulate TPC commands - the last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW. - FFS: no more than 1 TPC command is expected to take effect during a configured TDW.

In previous e-meetings, several options of how to capture the WA has been deeply discussed and no convergent option arises. And there is still no further clarification on R15/R16 legacy power control. In our contribution [1], we provided TPs for TPC command handling when DMRS bundling is enabled. To make UE behavior clear, we suggest the following proposal as a new option for UE behavior of accumulating TPC commands for CG-PUSCH.

Proposal 2: For TPC command handling when DMRS bundling is enabled, adopt the following proposal.

-	For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands and configured with DMRS bundling,

-	for the first transmission occasion i1 within a nominal time domain window, both KPUSCHi1  and Di1 reuse the same definitions in Rel-15/16 TS 38.213.

-    for a transmission occasion i occurs within a nominal time domain window determined as described in [6, TS 38.214], fb,f,ci,l=fb,f,c(i1,l) ;

-	for the first transmission occasion j occurs after the previous nominal time domain window, fb,f,cj,l=fb,f,ci1,l+m=0CDj-1δPUSCH,b,f,c(m,l), where m=0CDj-1δPUSCH,b,f,c(m,l) is a sum of TPC command values in a set Dj of TPC command values with cardinality CDj that the UE receives between KPUSCHi1-1 symbols before PUSCH transmission occasion i1 and KPUSCH(j) symbols before PUSCH transmission occasion j.

According to above proposal, we provide TP#1 which is proposed to be applied to section 7.1.1 of 38.213 V17.1.0.

Proposal 3: Adopt TP#1 for section 7.1.1 in 38.213 for TPC command handling when DMRS bundling is enabled.

| Reason for change: | Completion of TPC command handling when DMRS bundling is enabled for coverage enhancement in NR. |
| --- | --- |
| Summary of change: | Add descriptions for application of TPC commands for PUSCH repetitions transmission within time domain window for DM-RS bundling. |
| Consequences if not approved: | Incomplete support for DM-RS bundling in NR. |
| ------------------------------------------------Start of TP#1 for section 7.1.1 of 38.213 V17.1.0---------------------------------- 7.1.1 UE behaviour If a UE transmits a PUSCH on active UL BWP b of carrier f of serving cell c using parameter set configuration with index j and PUSCH power control adjustment state with index l, the UE determines the PUSCH transmission power PPUSCH,b,f,c(i,j,qd,l) in PUSCH transmission occasion i as [dBm] where, - PCMAX,f,c(i)is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier f of serving cell c in PUSCH transmission occasion i. <<unchanged text omitted>> - fb,f,ci,l=fb,f,ci-i0,l+m=0CDi-1δPUSCH,b,f,c(m,l) is the PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion i if the UE is not provided tpc-Accumulation, where - The δPUSCH,b,f,c values are given in Table 7.1.1-1 - m=0CDi-1δPUSCH,b,f,c(m,l) is a sum of TPC command values in a set Di of TPC command values with cardinality CDi that the UE receives between KPUSCHi-i0-1 symbols before PUSCH transmission occasion i-i0 and KPUSCH(i) symbols before PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c for PUSCH power control adjustment state l, where i0>0 is the smallest integer for which KPUSCH(i-i0) symbols before PUSCH transmission occasion i-i0 is earlier than KPUSCH(i) symbols before PUSCH transmission occasion i - If the UE is provided PUSCH-DMRS-Bundling = ‘enabled’ and not provided tpc-Accumulation, - for a transmission occasion i occurs within a nominal time domain window determined as described in [6, TS 38.214], fb,f,ci,l=fb,f,c(i1,l), where transmission occasion i1 is a first transmission occasion within the nominal time domain window; - for the first transmission occasion j occurs after the previous nominal time domain window, fb,f,cj,l=fb,f,ci1,l+m=0CDj-1δPUSCH,b,f,c(m,l), where m=0CDj-1δPUSCH,b,f,c(m,l) is a sum of TPC command values in a set Dj of TPC command values with cardinality CDj that the UE receives between KPUSCHi1-1 symbols before PUSCH transmission occasion i1 and KPUSCH(j) symbols before PUSCH transmission occasion j. - If a PUSCH transmission is scheduled by a DCI format, KPUSCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission - If a PUSCH transmission is configured by ConfiguredGrantConfig, KPUSCH(i) is a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c - If the UE has reached maximum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≥0, then fb,f,ci,l=fb,f,ci-i0,l - If UE has reached minimum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≤0, then fb,f,ci,l=fb,f,ci-i0,l - A UE resets accumulation of a PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c to fb,f,ck,l=0, k=0,1,…,i - If a configuration for a corresponding PO_UE_PUSCH,b,f,cj value is provided by higher layers - If a configuration for a corresponding αb,f,cj value is provided by higher layers where l is determined from the value of j as - If j>1 and the UE is provided higher SRI-PUSCH-PowerControl, l is the sri-PUSCH-ClosedLoopIndex value(s) configured in any SRI-PUSCH-PowerControl with the sri-P0-PUSCH-AlphaSetId value corresponding to j - If j>1 and the UE is not provided SRI-PUSCH-PowerControl or j=0, l=1 if PO_UE_PUSCH,b,f,cj and αb,f,cj are provided by the second P0-PUSCH-AlphaSet in p0-AlphaSets; otherwise, l=0 - If j=1, - l is provided by the value of powerControlLoopToUse if PO_UE_PUSCH,b,f,c1 and αb,f,c1 are provided by p0-PUSCH-Alpha in ConfiguredGrantConfig - l is provided by the value of powerControlLoopToUse2 if PO_UE_PUSCH,b,f,c1 and αb,f,c1 are provided by p0-PUSCH-Alpha2 in ConfiguredGrantConfig - fb,f,ci,l=δPUSCH,b,f,ci,l is the PUSCH power control adjustment state for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion if the UE is provided tpc-Accumulation, where - δPUSCH,b,f,c absolute values are given in Table 7.1.1-1 - If the UE is provided PUSCH-DMRS-Bundling = ‘enabled’ and provided tpc-Accumulation, - for a transmission occasion i occurs within a nominal time domain window determined as described in [6, TS 38.214], then fb,f,ci,l=fb,f,c(i1,l), where transmission occasion i1 is a first transmission occasion within the nominal time domain window, - for the first transmission occasion j occurs after the previous nominal time domain window, fb,f,cj,l=δPUSCH,b,f,c(i2,l), where δPUSCH,b,f,ci2,l is the last TPC command value that the UE receives between KPUSCHi1-1 symbols before transmission occasion i1 and KPUSCH(j) symbols before transmission occasion j. If the UE transmits a PUSCH associated with the first RS resource index qd, the UE applies the first PO_UE_PUSCH,b,f,cj value, the first αb,f,cj value, and fb,f,ci,l for determining PPUSCH,b,f,c(i,j,qd,l). If the UE transmits a PUSCH associated with the second RS resource index qd, the UE applies the second PO_UE_PUSCH,b,f,cj value, the second αb,f,cj value, and fb,f,ci,l or fb,f,ci,0 if twoPUSCH-PC-AdjustmentStates is provided or not provided, respectively, for determining PPUSCH,b,f,c(i,j,qd,l). <<unchanged text omitted>> ------------------------------------------------End of TP#1 for section 7.1.1 of 38.213 V17.1.0---------------------------------- |  |

If there would still be no consensus that can be reached on how to capture the WA, the simplest way that we can compromise to is to treat the group common TPC command with format 2_2 as an event that violates phase continuity and power consistency.

Proposal 4: If there’s no consensus on how to capture the WA of TPC commands handling, regard the action of group common TPC command with format 2_2 indicating different values of power control adjustment as a dynamic event.

- Joint channel estimation for PUCCH

3.1 Frequency hopping pattern description

Agreement: For the interaction between inter-slot frequency hopping and DMRS bundling for PUCCH/PUSCH repetitions, a UE performs the “hopping intervals determination”, “configured TDW determination”, and “actual TDW determination” in a sequential ordering, based on the following option 1. - Option 1: “hopping intervals determination” -> “configured TDW determination” -> “actual TDW determination” - DMRS bundling shall be restarted at the beginning of each frequency hop - DMRS bunding is per actual TDW - FFS: Frequency hopping pattern is determined by physical slot indices. - FFS: different FH pattern determination for PUCCH and PUSCH - FFS: details of FH pattern design - Support separate RRC configuration(s) for hopping interval and configured TDW length. - if hopping interval is not configured, the default hopping interval is the same as the configured TDW length - FFS: if both hopping interval and TDW length are not configured Note: hopping interval is only determined by the configuration of hopping interval if hopping interval is configured Agreement Inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling is determined based on physical slot index. Agreement Inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling is determined based on down-selection (in RAN1#108e) option 2 - Option 2: relative slot index – option A Note: Option A/B for relative slot index are defined as the following - Option A: frequency hopping pattern is determined based on relative slot index. Relative slot 0 is the slot where PUCCH/PUSCH repetition starts. Each of the subsequent slots are counted and relative slot index increase by one regardless the slot is used to transmit PUCCH/PUSCH or not. - Option B: frequency hopping pattern is determined based on relative slot index. Relative slot 0 is the slot where PUCCH/PUSCH repetition starts. One of the subsequent slots are counted and the slot index increase by one only if the slot is used to transmit PUCCH/PUSCH.

In RAN1 #107-e meeting, it was agreed that DMRS bundling is determined by hopping interval, while the hopping pattern determination and design are still FFS. In RAN1 #107bis-e and RAN1 #108-e meetings, it has been confirmed that for hopping pattern determination, physical slot index is used for PUSCH, and relative slot index determination is used for PUCCH. However, the detailed hopping pattern design needs to be further discussed.

Potential solutions have been provided in the previous meetings, such as introducing a formula with/without considering SFN. However, it is not agreed in RAN1 to consider SFN for start PRB determination in case of frequency hopping. The physical slot index for hop determination can be within one frame, as can be shown in following formula, where NFH is the value of the higher layer parameter PUSCH-Frequencyhopping-Interval.

RBstartns,fμ=RBstart RBstart+RBoffsetmodNBWPsize   ns,fμ+nf∙Nslotframe,μNFH mod 2=0ns,fμ+nf∙Nslotframe,μNFH mod 2=1

Proposal 5: For frequency hopping pattern determination of PUSCH, consider the physical slot index within one frame.

3.2 TPC command handing for DMRS bundling of PUCCH

As discussed in TPC command handling for PUSCH, similar handling should be considered if DMRS bundling for PUCCH is enabled.

Proposal 6: Adopt TP#2 for section 7.2.1 in 38.213 for TPC command handling when DMRS bundling of PUCCH is enabled.

| Reason for change: | Completion of TPC command handling when DMRS bundling is enabled for coverage enhancement in NR. |
| --- | --- |
| Summary of change: | Add descriptions for application of TPC commands for PUCCH repetitions transmission within time domain window for DM-RS bundling. |
| Consequences if not approved: | Incomplete support for DM-RS bundling in NR. |
| ------------------------------------------------Start of TP#2 for section 7.2.1 of 38.213 V17.1.0---------------------------------- 7.2.1 UE behaviour If a UE transmits a PUCCH on active UL BWP b of carrier f in the primary cell c using PUCCH power control adjustment state with index l, the UE determines the PUCCH transmission power PPUCCH,b,f,c(i,qu,qd,l) in PUCCH transmission occasion i as [dBm] where - PCMAX,f,c(i) is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier f of primary cell c in PUCCH transmission occasion i <<unchanged text omitted>> - gb,f,ci,l=gb,f,ci-i0,l+m=0CCi-1δPUCCH,b,f,c(m,l) is the current PUCCH power control adjustment state l for active UL BWP b of carrier f of primary cell c and PUCCH transmission occasion i, where - The δPUCCH,b,f,c values are given in Table 7.1.2-1 - m=0CCi-1δPUCCH,b,f,c(m,l) is a sum of TPC command values in a set Ci of TPC command values with cardinality CCi that the UE receives between KPUCCHi-i0-1 symbols before PUCCH transmission occasion i-i0 and symbols before PUCCH transmission occasion i on active UL BWP b of carrier f of primary cell c for PUCCH power control adjustment state, where i0>0 is the smallest integer for which KPUCCHi-i0 symbols before PUCCH transmission occasion i-i0 is earlier than KPUCCHi symbols before PUCCH transmission occasion i - If the UE is provided PUCCH-DMRS-Bundling = ‘enabled’, - for a transmission occasion i occurs within a nominal time domain window determined as described in [6, TS 38.214], gb,f,ci,l=gb,f,c(i1,l), where transmission occasion i1 is a first transmission occasion within the nominal time domain window; - for the first transmission occasion j occurs after the previous nominal time domain window, gb,f,cj,l=gb,f,ci1,l+m=0CCj-1δPUCCH,b,f,c(m,l), where m=0CCj-1δPUCCH,b,f,c(m,l) is a sum of TPC command values in a set Cj of TPC command values with cardinality CCj that the UE receives between KPUCCHi1-1 symbols before transmission occasion i1 and KPUCCH(j) symbols before transmission occasion j. - If the PUCCH transmission is in response to a detection by the UE of a DCI format, KPUCCHi is a number of symbols for active UL BWP b of carrier f of primary cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUCCH transmission <<unchanged text omitted>> ------------------------------------------------End of TP#2 for section 7.2.1 of 38.213 V17.1.0---------------------------------- |  |

- Conclusion

In this contribution, we discuss the remaining issues of supporting joint channel estimation of PUSCH and PUCCH, and have following proposals:

Proposal 1: RAN1 to conclude that if UE is not capable of restarting DM-RS bundling, when a semi-static event occurs after one or multiple dynamic events or overlaps with a dynamic event, a new actual window should be created after the semi-static event.

Proposal 2: For TPC command handling when DMRS bundling is enabled, adopt the following proposal.

-	For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands and configured with DMRS bundling,

-	for the first transmission occasion i1 within a nominal time domain window, both KPUSCHi1  and Di1 reuse the same definitions in Rel-15/16 TS 38.213.

-    for a transmission occasion i occurs within a nominal time domain window determined as described in [6, TS 38.214], fb,f,ci,l=fb,f,c(i1,l) ;

-	for the first transmission occasion j occurs after the previous nominal time domain window, fb,f,cj,l=fb,f,ci1,l+m=0CDj-1δPUSCH,b,f,c(m,l), where m=0CDj-1δPUSCH,b,f,c(m,l) is a sum of TPC command values in a set Dj of TPC command values with cardinality CDj that the UE receives between KPUSCHi1-1 symbols before PUSCH transmission occasion i1 and KPUSCH(j) symbols before PUSCH transmission occasion j.

Proposal 3: Adopt TP#1 for section 7.1.1 in 38.213 for TPC command handling when DMRS bundling is enabled.

Proposal 4: If there’s no consensus on how to capture the WA of TPC commands handling, regard the action of group common TPC command with format 2_2 indicating different values of power control adjustment as a dynamic event.

Proposal 5: For frequency hopping pattern determination of PUSCH, consider the physical slot index within one frame.

Proposal 6: Adopt TP#2 for section 7.2.1 in 38.213 for TPC command handling when DMRS bundling of PUCCH is enabled.

References

- R1-2201106, Remaining issues on joint channel estimation for PUSCH, vivo, February, 2022.