3GPP TSG RAN WG1 #109-e		R1- 2203523

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Maintenance on Enhancements to Integrated Access and Backhaul

Agenda Item:	8.10

Document for:	Discussion and Decision

- Introduction

New WID [1] on Enhancements to Integrated Access and Backhaul has been approved as following.

Duplexing enhancements [RAN1-led, RAN2, RAN3, RAN4]: - Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including: - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx). - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing. - Specification of IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed, to support simultaneous operation (transmission and/or reception) by IAB-node’s child and parent links.

In this contribution, we provide our view on resource multiplexing between child and parent links of an IAB node to support simultaneous transmission and/or reception of collocated MT and DU.

- Discussion

  - Frequency domain resource management

In the RAN1 107e e-meeting, it has been discussed that when to apply the R17 frequency domain H/S/NA configuration, and the following WA was made.

Working Assumption If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, one of the following is selected: - Alt. 1: An IAB node applies the frequency domain H/S/NA only if the IAB node is currently operating in a non-TDM multiplexing mode in the slot, otherwise the Rel-16 time domain H/S/NA configuration is applied.

The solution in the WA allows more efficient resource utilization due to supporting dynamic switching between Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration, e.g., the Rel-17 frequency NA resource can still be used by IAB node, if the IAB node decides to dynamically fallback to TDMed multiplexing mode and the corresponding Rel-16 time resource type is H.

There were two major concerns to confirm the WA, i.e., 1) the dynamic switching between Rel-16 and Rel-17 H/S/NA would impact the inter-cell interference or CLI handling; 2) the non-TDM multiplexing mode is not defined. For the first concern, it should be noted that the inter-cell interference and CLI handling can also relay on TDD configuration coordination. To address the second concern, RAN1 should further discuss how to decide the non-TDM multiplexing mode.

Proposal 1: Confirm the WA that An IAB node applies the frequency domain H/S/NA only if the IAB node is currently operating in a non-TDM multiplexing mode in the slot, otherwise the Rel-16 time domain H/S/NA configuration is applied.

In our opinion, the multiplexing mode of an IAB node can be achieved by indicating the allowance of a given multiplexing mode on a set of resource(s).  One straightforward method is to indicate a time window in which the IAB node is allowed to perform a given Rel-17 multiplexing modes, and IAB node falls back to Rel-16 multiplexing mode (i.e., TDMed multiplexing mode) when out of the corresponding windows.

Proposal 2: Support to indicate the allowance of a given multiplexing mode (e.g., simultaneous operation or TDM operation) on a set of time/frequency resources by parent node.

Moreover, IAB node implementation can decide whether to enable a Rel-17 multiplexing mode on a given set of time/frequency resources and report the expectation to parent node, thus parent node will perform resource scheduling accordingly. For example, when IAB node would not follow the constraint of a given multiplexing mode on a given set of time resources (e.g., IAB does not follow case 7 timing mode constraint due to scheduling of access UE), IAB node can report the fallback to TDMed multiplexing mode on the associated time resources to parent node.

Proposal 3: Support IAB node to report the expected multiplexing mode (e.g., simultaneous operation or TDM operation) on a set of time/frequency resources to parent node.

- Conclusion

This contribution focus on resource multiplexing between child and parent links and the following are proposed,

Proposal 1: Confirm the WA that An IAB node applies the frequency domain H/S/NA only if the IAB node is currently operating in a non-TDM multiplexing mode in the slot, otherwise the Rel-16 time domain H/S/NA configuration is applied.

Proposal 2: Support to indicate the allowance of a given multiplexing mode (e.g., simultaneous operation or TDM operation) on a set of time/frequency resources by parent node.

Proposal 3: Support IAB node to report the expected multiplexing mode (e.g., simultaneous operation or TDM operation) on a set of time/frequency resources to parent node.

- Reference

- RP-193251, “New WID on Enhancements to Integrated Access and Backhaul”, RAN#86, Spain, December 2019.