3GPP TSG RAN WG1 #109-e                                            R1-2203524

e-Meeting, May 9th – 20th,  2022

Source:	vivo

Title:	Remaining issues on resource allocation for power saving

Agenda Item:	8.11.1

Document for:	Discussion and Decision

- Introduction

Although some good progresses were achieved in the previous meetings, there are still some open issues to be resolved on resource allocation for sidelink power saving. In this contribution, we provide our views, as well as the TPs, on some remaining issues on resource allocation for power saving, including:

- Partial sensing mechanism

- Impact of sidelink DRX

- Partial sensing mechanism

In this section, some remaining issues on the partial sensing mechanism are addressed.

- Issue#1 CPS window

The sensing window [n+TA, n+TB] for CPS is specified in TS 28.214, as following:

[first paragraph] When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled, the sensing window is defined by the range of slots [n+TA, n+TB]. n+TA is M consecutive logical slots earlier than slot ty0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. If Prsvp_TX≠0 the value of M is (pre-)configured with the contiguousSensingWindowPeriodic. If contiguousSensingWindowPeriodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection. [second paragraph] When the UE performs contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) disabled and if Prsvp_TX=0, the sensing window is defined by the range of slots [n+TA, n+TB]. TA and TB are both selected such that the UE has sensing results starting at M consecutive logical slots before ty0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than ty0SL. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

For aperiodic transmission, M(indicated by high layer parameter contiguousSensingWindowAperiodic) is defined as a minimum size for the CPS window, which is different from the (pre-)configured size of CPS window M(indicated by high layer parameter contiguousSensingWindowPeriodic) for the periodic transmission case. To avoid confusion, a new notation M’ is suggested to replace M.

Besides, the first paragraph in the cited text includes two cases in a pool with periodic reservation for another TB (sl-MultiReserveResource) enabled:

Case 1) periodic transmission, and

Case 2) aperiodic transmission;

Additionally, the second paragraph is for another case 3): aperiodic transmission in a pool with periodic reservation for another TB (sl-MultiReserveResource) disabled.

In the above-mentioned three cases, the determination of sensing window [n+TA, n+TB] for CPS are related to ty0SL, where ty0SL is defined as the first slot of the selected Y candidate slots of PBPS.

However, this is not compliant with the agreements. In our understanding, only for periodic transmission, n+TA and n+TB should be defined related to the first slot of the selected Y candidate slots of PBPS; but for aperiodic transmission, no matter the periodic reservation for another TB (sl-MultiReserveResource) is enabled or disabled, ty0 used to deriving CPS window should be the first slot of the selected Y’ candidate slots especially considering that there may be no Y candidate slots of PBPS in some cases. Hence, the text for the determination of sensing window [n+TA, n+TB] for CPS should be revised.

Additionally, to avoid confusion, we suggest using a different notation ty0'SL for aperiodic transmission case.

Agreement When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, - For a resource (re)selection procedure triggered by periodic transmission (Prsvp_TX≠0) in slot n, TA and TB for the CPS monitoring window is defined according to one of the followings: - n+TA is M logical slots earlier than slot ty0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. - By default, M is 31 unless (pre-)configured with another value. Agreement When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, - The UE selects a set of Y’ candidate slots with corresponding PBPS and/or CPS results (if available) within the RSW. - If the total number of Y’ candidate slots is less than a (pre-)configured threshold Y’min, - How UE includes other candidate slots is up to UE implementation - Candidate resource set (SA) is initialized to the set of all single-slot candidate resources in the selected Y’ candidate slots. - For the CPS monitoring window [n+TA, n+TB]: - TA and TB are both selected such that UE has sensing results starting at M consecutive logical slots before ty0 and ending at Tproc,0 + Tproc,1 slots earlier than ty0. - FFS: By default, M is 31 unless (pre-)configured with another value, or where M is (pre-)configured based on transmission priority - FFS: The range of (pre-)configured M from a TBD lowest value up to 30 - When the minimum M slots for CPS cannot be guaranteed, support both - Option A, the UE ensures the Y’min criterion is fulfilled - Option B: UE performs random resource selection - When the UE performs Option A or Option B is up to UE implementation

Proposal 1: Relace the notation of the minimum size for CPS to M’.

Proposal 2: TP#1 should be adopted.

- TP#1

When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled, the sensing window is defined by the range of slots n+TA, n+TB. If Prsvp_TX≠0, n+TA is M consecutive logical slots earlier than slot ty0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots, If Prsvp_TX≠0 and the value of M is (pre-)configured with the contiguousSensingWindowPeriodic. If contiguousSensingWindowPeriodic is not (pre-)configured, M equals to 31. If Prsvp_TX=0, TA and TB are both selected such that the UE has sensing results starting at M’ consecutive logical slots before ty0'SL and ending at Tproc,0SL+Tproc,1SL slots earlier than ty0'SL, where ty0'SL is the first slot of the selected Y’ candidate slots and the value of M’ is (pre-)configured with the contiguousSensingWindowAperiodic. If contiguousSensingWindowAperiodic is not (pre-)configured, M’ equals to 31. When the minimum M’ slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection. When the UE performs contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) disabled and if Prsvp_TX=0, the sensing window is defined by the range of slots [n+TA, n+TB]. TA and TB are both selected such that the UE has sensing results starting at M’ consecutive logical slots before ty0'SL and ending at Tproc,0SL+Tproc,1SL slots earlier than ty0'SL, where ty0'SL is the first slot of the selected Y’ candidate slots and the value of M’ is (pre-)configured with the contiguousSensingWindowAperiodic. If contiguousSensingWindowAperiodic is not (pre-)configured, M’ equals to 31. When the minimum M’ slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

According to the agreement [2], the RRC parameter contiguousSensingWindowAperiodic indicates the minimum size of CPS window. Hence, the following TP is proposed.

- TP#2

===omitted=== - Optionally, indication of the size in logical slots of contiguous partial sensing window as contiguousSensingWindowPeriodic. - Optionally, indication of the minimum size in logical slots of contiguous partial sensing window as contiguousSensingWindowAperiodic.

Proposal 3: Adopt the TP#2.

In current spec for determination of resource selection mechanism’s sensing window, i.e. Step 2) of 8.1.4, when the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB enabled, the sensing window is defined by the range of slots [n+TA, n+TB]. Unfortunately, the current text may be incorrectly understood to cover PBPS window either, while actually it is intended for CPS window only. To avoid this confusion, the following TP is proposed.

- TP#3

When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled, the CPS sensing window is defined by the range of slots [n+TA, n+TB]. n+TA is M consecutive logical slots earlier than slot ty0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. If Prsvp_TX≠0 the value of M is (pre-)configured with the contiguousSensingWindowPeriodic. If contiguousSensingWindowPeriodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection.

Proposal 4: Adopt the above TP#3.

When a UE performs periodic-based partial sensing, the UE monitors slots of at least one PSO, where a PSO is a set of slots according to

The value of k indicates the index of PSO to be monitored. The UE monitors one or two PSOs. The behavior of the UE monitoring PSO is described in TS 38.214 of 8.1.4, as following:

When the UE performs periodic-based partial sensing, the UE shall monitor slots at ty-k×PreserveSL, where tySL is a slot of the selected candidate slots. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. =====omitted===== The UE monitors k sensing occasions determined by additionalPeriodicSensingOccasion, as previously described, and not earlier than n –T0. For a given periodicity Preserve, the values of k correspond to the most recent sensing occasion earlier than ty0SL-(Tproc,0SL+Tproc,1SL ) if additionalPeriodicSensingOccasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if additionalPeriodicSensingOccasion is (pre-)configured. ty0SL is the first slot of the selected Y candidate slots of PBPS.

It can be observed that the definition of k in the two paragraphs are different, which is confusing and should be clarified. Thus, the following TP is proposed.

- TP#4

The UE monitors k’ k sensing occasions determined by additionalPeriodicSensingOccasion, as previously described, and not earlier than n –T0. For a given periodicity Preserve, the values of k correspond to the most recent sensing occasion earlier than ty0SL-(Tproc,0SL+Tproc,1SL ) and k’=1 if additionalPeriodicSensingOccasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one and k’=2 if additionalPeriodicSensingOccasion is (pre-)configured. ty0SL is the first slot of the selected Y candidate slots of PBPS.

Proposal 5：Adopt the above TP#4.

- Impact of sidelink DRX

In this section, we discuss the remaining issues related to sidelink DRX.

  - Impact of SL DRX on sensing

Agreement Whether UE performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by (pre-)configuration per resource pool when partial sensing is configured in the UE by a higher layer. - When it is enabled, - When UE performs periodic-based partial sensing for a given Preserve, UE monitors only the default periodic sensing occasion. - When UE performs contiguous partial sensing, UE monitors a minimum of M slots for CPS. - Note, when it is disabled, the UE is not required to perform SL reception of PSCCH and RSRP measurement in SL DRX inactive time. - Note: no further optimization on the resource (re)selection procedure with regard to SL DRX operation is specified in Rel.17. - FFS the case when full sensing is configured in the UE by a higher layer

It the last meeting, it was discussed if full sensing should be restricted by SL DRX. As power saving is not the main consideration for a UE using full sensing, there is no motivation for the full sensing UE to enable SL DRX. Thus, it seems not necessary to consider or enhance the combination of {full sensing + SL DRX}. However, since resource allocation scheme is per TX pool provided, while DRX is per QoS profile maintained, configuration-wise, this combination is allowed. To avoid this combination, one possible way is to restrict the UE behavior that if UE is performing SL DRX in a pool, it should not use full sensing for transmission in the link.

Proposal 6: A UE configured with SL DRX for power saving for its own reception is not expected to use full sensing for SL transmission for this link.

- Conclusion

This contribution focus on power saving mechanism for NR sidelink with the following proposals:

Proposal 1: Relace the notation of the minimum size for CPS to M’.

Proposal 2: TP#1 should be adopted.

Proposal 3: Adopt the TP#2.

Proposal 4: Adopt the above TP#3.

Proposal 5：Adopt the above TP#4.

Proposal 6: A UE configured with SL DRX for power saving for its own reception is not expected to use full sensing for SL transmission for this link.

- Reference

- RP-193257, “New WID on NR sidelink enhancement”.

- R1-2200699, “ Consolidated higher layers parameter list for Rel-17 NR”