3GPP TSG RAN WG1 #109-e                                     R1-2203526

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Remaining issues on mechanisms to improve reliability for RRC_CONNECTED UEs

Agenda Item:	8.12.1

Document for:  Discussion and Decision

- Introduction

In RAN1 #108 e-meeting, regarding the mechanisms to improve reliability for RRC_CONNECTED UEs, good process was achieved and the following conclusion and agreements were made [1].

Conclusion

No TP is needed to reflect the RAN1 agreement of the FDMed Type-1 HARQ-ACK codebook for more than one configured G-RNTI,

- Note: it means the following RAN1 agreement is updated as follows, to align with further agreements made in RAN1:

Agreement For the Type-1 codebook construction for FDM-ed unicast and multicast via Opt 4 (from the previous agreement), when UE is configured with multiple G-RNTIs and UE is configured with fdmed-Reception-Multicast, the sub-codebook for multicast consists of the HARQ-ACK bits for all configured G-RNTIs.

Agreement

For supporting more than one NACK-only feedback in the same PUCCH transmission, define RRC configuration to configure between Alt1 and Alt4 (from previous agreements):

- Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

  - FFS: how to determine PUCCH resource

- Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

  - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback,

    - FFS: The PUCCH slot for the transmission is based on the K1 in the “last DCI” scheduling multicast.

    - FFS: The PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the mapping between number of TBs with PUCCH resource ID.

      - FFS mapping details.

      - How to determine the number of TBs is discussed separately, e.g., Type-1-like and/or Type-2-like codebook.

    - FFS: whether this applies to a single G-RNTI or multiple G-RNTIs

  - Alt4 is not supported for more than 4 TBs

- FFS: whether RRC configuration between Alt1 and Alt4 is per G-RNTI or per CFR

- FFS: UE capability

Agreement

When HARQ-ACK for unicast SPS PDSCHs and multicast dynamic grant PDSCHs with ACK/NACK based feedback are multiplexed on the same PUCCH for the same priority case, the following option 1 (from the previous agreement) is adopted:

- Option 1: the PUCCH carrying the multiplexed HARQ-ACK is determined from the SPS-PUCCH-AN-List configured for unicast.

- Option 2: the PUCCH carrying the multiplexed HARQ-ACK is determined from PUCCH-Config/PUCCH-ConfigurationList configured for multicast.

Agreement

When NACK-only based HARQ-ACK feedback is used for multicast SPS PDSCH without PDCCH scheduling, the UE determines a priority index from the HARQ-ACK codebook index in the configuration for SPS multicast, using the same method with the one for ACK/NACK based feedback.

Agreement

When UE is configured with unicast SPS and multicast SPS with NACK-only based feedback for multiplexing on the same PUCCH for the same priority case, NACK only based HARQ-ACK is transformed to ACK/NACK based HARQ-ACK.

- For NACK only based HARQ-ACK transformed to ACK/NACK based HARQ-ACK, the HARQ-ACK codebook is constructed as for multiple SPS PDSCHs regardless of unicast SPS PDSCH or multicast SPS PDSCH and the PUCCH carrying the multiplexed HARQ-ACK is determined from the SPS-PUCCH-AN-List configured for unicast, as agreed for ACK/NACK based feedback.

Agreement

Regarding RRC configuring Alt1 or Alt4 (from the previous agreement) for multiplexing more than one NACK-only in the same PUCCH transmission, the configuration is per CFR.

Agreement

If Type-1 codebook is configured for both multicast and unicast, at least for single cell case for both unicast and multicast:

- If the UE is configured to construct the HARQ-ACK codebooks for unicast and multicast jointly, a single UL DAI bit applies for unicast and multicast

- Otherwise, 1 additional bit UL DAI is included for multicast in DCI format 0_1/0_2, in addition to the UL DAI for unicast. The 1-bit UL DAI for multicast is applied to all configured G-RNTIs.

  - FFS: additional restrictions

Agreement

When PUCCH transmission for the NACK-only based feedback for one G-RNTI collides with PUCCH transmission for ACK/NACK feedback for another G-RNTI with the same priority, support UE multiplexing the NACK-only based feedback with the ACK/NACK feedback onto the same PUCCH by transforming NACK-only into the ACK/NACK based HARQ-ACK bit.

- Note: When the TB configured with NACK-only feedback is correctly decoded, the ACK will be transmitted and multiplexed with others.

Agreement

For multiplexing NACK-only feedback for the first G-RNTI with ACK/NACK based feedback for the second G-RNTI or for unicast, down-select from:

- Alt1: the converted NACK-only bits are concatenated to the ACK/NACK feedback.

- Alt2: the codebook construction/concatenation is the same as when the feedback mode is ‘ACK/NACK’ for both G-RNTIs.

Agreement

When UE is configured with enhanced Type-2 codebook for unicast and when the UE is scheduled to multiplex enhanced Type-2 HARQ-ACK for unicast with Type-1 or Type-2 HARQ-ACK codebook for multicast in the same PUCCH slot,

- UE generates separate sub-codebooks for unicast and multicast respectively and appends the multicast HARQ-ACK sub-codebook to the unicast HARQ-ACK sub-codebook.

Agreement

- For the following two cases, the fallback operation for the Type-1 HARQ-ACK codebook for multicast is applied to G-RNTI/G-CS-RNTI configured with HARQ-ACK enabled only,

  - a SPS PDSCH release indicated by DCI format 4_1 with counter DAI field value of 1,

  - a PDSCH reception scheduled by DCI format 4_1 with counter DAI field value of 1 on the PCell,

  - SPS PDSCH reception(s) associated with G-CS-RNTIs.

- Note: If the UE receives two SPS PDSCH releases for two respective G-CS-RNTIs with DAI=1, or two PDSCHs by DCI 4_1 with DAI=1 for two respective G-RNTIs on the PCell, there is no fallback.

Agreement

For Type-1 codebook generation,

- if all configured G-RNTIs are with HARQ-ACK disabled by RRC signalling, UE does not report any HARQ-ACK information in the PUCCH slot;

- For other cases, FFS between the 3 alternatives below:

  - Alt1: if at least one configured G-RNTI is with HARQ-ACK enabled, UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

  - Alt2: if at least one configured G-RNTI is with HARQ-ACK enabled, UE reports actual HARQ-ACK result for all G-RNTIs.

  - Alt3: UE is not expected to be configured with some G-RNTI with HARQ-ACK disabled by RRC signalling and some other G-RNTI with HARQ-ACK enabled by RRC signalling for all configured G-RNTI

  - Other alternatives are not precluded

Agreement

If Type-2 codebook is configured for unicast and multicast, the following option2-2 (from the previous agreement) is adopted:

- Option2-2: 2-bit UL DAI(s) are included in DCI for multicast, in addition to the 2-bit UL DAI for unicast.

  - The 2-bit UL DAI for multicast is applied for all configured G-RNTIs.

    - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs.

          - Alt1-1: UL-DAI indicates the sum of DL-DAIs

          - Alt1-2: DL-DAIs have the same value.

          - Alt1-3: largest DL DAI value among the configured G-RNTIs.

          - Other alternatives are not precluded

Agreement

When Type-1 codebook is configured for unicast and Type-2 codebook is configured for multicast, or when Type-2 codebook is configured for unicast and Type-1 codebook is configured for multicast, the UL-DAI for multicast is included in DCI format 0_1/0_2, in addition to the UL-DAI field for unicast.

- The UL-DAI for multicast is 1-bit for Type-1 codebook

  - The 1-bit UL-DAI for multicast is applied to all configured G-RNTIs.

- The UL-DAI for multicast is 2-bit for Type-2 codebook applied for all configured G-RNTIs.

  - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs.

      - Alt1-1: UL-DAI indicates the sum of DL-DAIs

      - Alt1-2: DL-DAIs have the same value.

      - Alt1-3: largest DL DAI value among the configured G-RNTIs.

      - Other alternatives are not precluded

- FFS: details of the UL DAI field

Agreement

For NACK-only feedback for PUCCH format 0 or format 1, only 1 cyclic shift is used.

- The sequence cyclic shift for NACK-only is mcs=0.

- For PF1 NACK-only, set b0=0.

- Note: the initialCyclicShift is configured for PUCCH-format0 and PUCCH-format1 as legacy.

Agreement

For multiplexing NACK-only with HARQ-ACK feedback/CSI for unicast for the same priority or PUSCH transmission for the same priority or ACK/NACK based HARQ-ACK feedback for multicast with another G-RNTI for the same priority, the multiplexing does not depend on the decoding result.

- Note: when multiplexing the NACK-only is transformed into ACK/NACK as agreed.

Agreement

For Type-2 codebook generation, UE reports HARQ-ACK bits only for TBs with enabled HARQ-ACK by RRC or DCI.

Agreement

For multiplexing NACK-only feedback for the first G-RNTI with ACK/NACK based feedback for the second G-RNTI or for unicast, the following Alt2 (from the previous agreement) is adopted:

- Alt1: the converted NACK-only bits are concatenated to the ACK/NACK feedback.

- Alt2: the codebook construction/concatenation is the same as when the feedback mode is ‘ACK/NACK’ for both G-RNTIs.

Agreement

If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, and if the UE is configured with ACK/NACK based feedback for multicast dynamic scheduling and is configured with disabled HARQ feedback for multicast SPS scheduling, for Type-1 codebook generation,

- FFS between the 4 alternatives below:

  - Alt1: UE will report NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

  - Alt2: UE will report ACK/NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

  - Alt3: UE is not expected to be configured with G-RNTI with HARQ-ACK enabled by RRC signalling and G-CS-RNTI with HARQ-ACK enabled by RRC signalling.

  - Alt4: UE does not report any HARQ-ACK information for G-CS-RNTI with HARQ-ACK feedback disabled by RRC.

  - Other alternatives are not precluded.

In this contribution, we will make discussions on the remaining issues on mechanisms to improve reliability for RRC_CONNECTED UEs.

- Discussion

Different codebook types for HARQ-ACK for multicast and unicast

It was agreed that when UE is configured with different codebook types for unicast and multicast and when UE is scheduled to multiplex HARQ-ACK for unicast and HARQ-ACK for multicast with the same priority in the same PUCCH slot, UE generates two separate sub-codebooks for unicast and multicast respectively and then concatenates them by appending sub-codebook for multicast to the sub-codebook for unicast. This issue is still open when ype-3 HARQ-ACK codebook or enhanced type-3 codebook is used for unicast.

Currently, once UE is triggered to transmit type-3 HARQ-ACK codebook in a slot, UE will generate the HARQ-ACK codebook based on all configured HARQ processes and serving cells. Considering that the HARQ-ACK processes are shared between unicast PDSCH and multicast PDSCH, one type-3 HARQ-ACK codebook would be enough when UE is scheduled to multiplex HARQ-ACK for unicast and HARQ-ACK for multicast with the same priority in the same PUCCH slot. That is, UE only generates type 3 codebook which includes both unicast HARQ-ACK and multicast HARQ-ACK.

In addition, enhanced type 3 codebook is supported in NR R17 URLLC item to reduce the HARQ-ACK codebook payload, where UE only reports HARQ-ACK information for a subset of all configured HARQ processes and serving cells which is configured by RRC. In this case the HARQ processes in enhanced type 3 codebook may or may not include the HARQ processes of multicast PDSCHs. In URLLC WI, it was agreed that for the enhanced type 3 HARQ-ACK codebook of smaller size triggered in a PUCCH slot, the UE is not expecting HARQ-ACK information in a type 1 or type 2 HARQ-ACK codebook to be transmitted that cannot be mapped to the enhanced type 3 HARQ-ACK codebook of smaller size as the HARQ process is not part of the codebook. For simplicity, same option can be reused.

Proposal 1: When UE is configured with different codebook types for unicast and multicast and when UE is scheduled to multiplex HARQ-ACK for unicast and HARQ-ACK for multicast with the same priority in the same PUCCH slot,

- If type-3 HARQ-ACK codebook is used for unicast, UE only generates one type-3 codebook including both unicast HARQ-ACK and multicast information.

  - Note: The PUCCH resource for transmitting the codebook is based on the last unicast DCI.

- If enhanced type-3 codebook is used for unicast, for the enhanced type 3 HARQ-ACK codebook of smaller size triggered in a PUCCH slot, the UE is not expecting HARQ-ACK information in a type 1 or type 2 HARQ-ACK codebook to be transmitted that cannot be mapped to the enhanced type 3 HARQ-ACK codebook of smaller size as the HARQ process is not part of the codebook.

- Multiplexing multicast HARQ-ACK on PUSCH

To multiplex HARQ-ACK for multicast and PUSCH, it was agreed in the last meeting to support UL-DAI indicating the number of HARQ-ACK bits for multicast and 2 additional bits are used as UL-DAI for all configured G-RNTIs when UE is configured type 2 codebook for multicast. One FFS is how to count the total number of HARQ-ACK bits for all configured G-RNTIs and the following options are listed.

  - Alt1-1: UL-DAI indicates the sum of DL-DAIs

  - Alt1-2: DL-DAIs have the same value.

  - Alt1-3: largest DL DAI value among the configured G-RNTIs.

Among these three alternatives, Alt 1-1 is preferred for its simplicity and potential smaller size comparing with the other two alternatives. This alternative can guarantee total size of HARQ-ACK codebook can be correctly understood at the UE side so that gNB can correctly decode UL-SCH.

Proposal 2: For multiplexing unicast and multicast HARQ-ACK onto the same PUSCH with the same priority for the case multicast HARQ-ACK is configured with type 2 codebook:

  - The 2-bit UL-DAI is applied for all configured G-RNTIs and indicates the sum of DL-DAIs.

- Multiple NACK-only are available in the same PUCCH slot

For NACK only feedback, it is possible that there are multiple PDSCHs to be fed back in the same slot in TDD downlink heavy case. In the previous meetings, this issue was extensively discussed and it was agreed to be RRC configured between Alt 1 and Alt 4, and some remaining issues are left.

- PUCCH resource determination when NACK-only is transformed into ACK/NACK mode

When NACK-only is transformed into ACK/NACK mode, it is simple and straightforward to reuse the same mechanism to determine a PUCCH resource as ACK/NACK mode. That is, the PUCCH is determined by the PRI indication in the last DCI from the PUCCH-config for the ACK/NACK feedback mode for multicast. If the PUCCH-config for the ACK/NACK feedback mode for multicast is not configured, PUCCH-config for unicast with the same priority is used.

- The PUCCH slot for the transmission for PUCCH selection mode

Per NR specification, the slot for HARQ-ACK feedback is indicated by K1 in each scheduling DCI or activation/deactivation DCI. For NACK-only feedback, same mechanism is reused. The right procedure is that UE first determines the slot for NACK-only feedback, and if there are multiple NACK-only indicated in the same PUCCH slot, Alt1 or Alt4 is used based on RRC configuration.

- Mapping between number of TBs with PUCCH resource ID

The mapping between of TBs with PUCCH resource ID can be predefined. For example, one TB is mapped to the first PUCCH resource configured in the PUCCH-config for NACK-only. 4TBs is mapped to the 16th PUCCH resource configured in the PUCCH-config for NACK-only.

- Whether alt4 applies to a single G-RNTI or multiple G-RNTIs

As discussed in previous meetings, it would be questionable for alt 4 in multiple G-RNTIs case. Since there would be more than 16 combinations for up to 4 TBs. For example, if UE is configured with two G-RNTIs, there are 32 TB and G-RNTI combinations for up to 4TBs. Then, the 15 PUCCH resources are not enough to cover all combinations. The TB and G-RNTI combinations will increase explosively when UE is configured with more G-RNTIs. Thus, we prefer to apply alt4 to a single G-RNTI case.

- UE capability

Regarding to supporting more than one NACK-only feedback in the same PUCCH transmission, for alt1, UE needs to transform NACK-only feedback to ACK/NACK mode. For alt4, UE needs to maintain the table defining mapping between TBs and PUCCH resources. Therefore, two separate UE capabilities should be defined correspondingly.

Proposal 3: When more than one NACK-only based feedback is available for transmission in the same PUCCH slot,

  - If Alt 1 is used, the PUCCH for transmission is determined in the same way as ACK/NACK mode.

  - If Alt 4 is used,

    - The PUCCH slot is determined based on the K1 indication in each corresponding scheduling DCI

    - The mapping between TBs and PUCCH resources are predefined.

    - This applies to a single G-RNTI.

  - Two separate UE capabilities should be defined for alt 1 and alt 4 correspondingly.

- PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR

It was agreed when PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for HARQ-ACK feedback/CSI for unicast for the same priority or PUSCH transmission for the same priority, support UE multiplexing the NACK-only based feedback with the HARQ-ACK feedback/CSI on PUCCH or on to PUSCH by transforming NACK-only into the ACK/NACK HARQ bit. One FFS is for the case that PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR with the same priority. In NR Rel-15/16, for the overlapping of HARQ-ACK and SR with the same priority, HARQ-ACK and SR are multiplexed in different ways according to the PUCCH formats used for HARQ-ACK and SR, and the details are shown in the following Table 1.

Table 1 NR R15/R16 multiplexing rule for HARQ-ACK and SR

| HARQ-ACK PUCCH SR PUCCH | Format 0 | Format 1 |
| --- | --- | --- |
| Format 0 | Case 1: If SR is positive, SR is multiplexed on the HARQ-ACK PUCCH resource with different CSs from that for HARQ-ACK only. |  |
| Format 1 | Case 2: If SR is positive, drop SR and transmit HARQ-ACK | If SR is positive，transmit HARQ-ACK on SR PUCCH resource |
| Format 2/3/4 | Append K=log2（1+K）bits SR information to HARQ-ACK bits and transmit the multiplexed UCI on HARQ-ACK PUCCH resource. |  |

For NACK-only for multicast and SR collision handling, if the same multiplexing rule is reused, there will be some issues for the above Case 1 and Case 2. In Case 1, SR is transmitted on the NACK-only resource, considering there are multiple UEs in the same MBS group and if multiple SRs are transmitted in the same PUCCH resource, the same cyclic shift will be used for different UEs, then gNB would not be able to distinguish the SR is transmitted from which UE. For Case 2, it would be better to drop NACK rather than SR, as there may be more than one UE sending NACK and gNB can retransmit the PDSCH once it detects NACK. To handle the overlapping between NACK-only and SR, the following options can be considered.

When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmission for SR for the same priority, for the case with one NACK-only.

- Option 1: If SR is positive, NACK-only is dropped. If SR is negative, transmit NACK-only PUCCH

- Option 2: If SR is positive, transmit NACK-only on SR PUCCH resource. If SR is negative, transmit NACK-only PUCCH.

  - Option 2-1: only NACK-only is transmitted on SR PUCCH resource. If it is ACK, transmit SR only.

  - Option 2-2: both NACK and ACK can be transmitted on SR PUCCH resource, if it is ACK, transmit ACK on SR PUCCH resource; if it is NACK, transmit NACK on SR PUCCH resource. Different cyclic shifts are needed for ACK/NACK transmission on SR resource for PUCCH format 0.

- Option 3: When SR is positive, for the case that both SR PUCCH and NACK-only are PUCCH format 1, transmit NACK on SR PUCCH resource using the same rule as NR R15/R16. Otherwise, NACK-only/SR is dropped. When SR is negative, transmit NACK-only PUCCH.

In our view, option 1 is the simplest but may bring unnecessary PDSCH retransmission and option 2 have the best performance but change the current multiplexing rule for HARQ-ACK and SR. Option 3 may be a compromise solution.

For the case with more than one NACK-only, if NACK-only is transformed into ACK/NACK mode, the legacy multiplexing rule should be reused. If PUCCH selection is used, same option as one NACK-only can be used.

Proposal 4: When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, the following solutions can be considered.

- Option 1: If SR is positive, NACK-only is dropped. If SR is negative, transmit NACK-only PUCCH

- Option 2: If SR is positive, transmit NACK-only on SR PUCCH resource. If SR is negative, transmit NACK-only PUCCH.

  - Option 2-1: only NACK-only is transmitted on SR PUCCH resource. If it is ACK, transmit SR only.

- Option 3: When SR is positive, for the case that both SR PUCCH and NACK-only are PUCCH format 1, transmit NACK on SR using the same rule as NR R15/R16. Otherwise, NACK-only/SR is dropped. When SR is negative, transmit NACK-only PUCCH.

- Disabled HARQ-ACK in type 1 codebook

In the last meeting, it was discussed that if at least one G-RNTI is configured with enabled HARQ-ACK, for type 1 codebook, how to report HARQ-ACK information for the G-RNTI or multicast SPS with disabled HARQ-ACK. Considering that the codebook size is already there and has no related with the disabling of HARQ-ACK of some G-RNTIs or G-CS-RNTI, it is better to report the actual HARQ-ACK result for all G-RNTIs than always set NACK for the G-RNTI with HARQ-ACK disabled. This information is beneficial for gNB to do link adaptation.

Proposal 5: For Type-1 codebook generation,

- If at least one configured G-RNTI is with HARQ-ACK enabled, UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

- If the UE is configured with disabled HARQ feedback for multicast SPS scheduling, UE will report NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

- Conclusion

In this contribution, we make discussions on the mechanisms to improve reliability for RRC_CONNECTED UEs, and have the following proposals.

Proposal 1: When UE is configured with different codebook types for unicast and multicast and when UE is scheduled to multiplex HARQ-ACK for unicast and HARQ-ACK for multicast with the same priority in the same PUCCH slot,

- If type-3 HARQ-ACK codebook is used for unicast, UE only generates one type-3 codebook including both unicast HARQ-ACK and multicast information.

  - Note: The PUCCH resource for transmitting the codebook is based on the last unicast DCI.

- If enhanced type-3 codebook is used for unicast, for the enhanced type 3 HARQ-ACK codebook of smaller size triggered in a PUCCH slot, the UE is not expecting HARQ-ACK information in a type 1 or type 2 HARQ-ACK codebook to be transmitted that cannot be mapped to the enhanced type 3 HARQ-ACK codebook of smaller size as the HARQ process is not part of the codebook.

Proposal 2: For multiplexing unicast and multicast HARQ-ACK onto the same PUSCH with the same priority for the case multicast HARQ-ACK is configured with type 2 codebook:

- The 2-bit UL-DAI is applied for all configured G-RNTIs and indicates the sum of DL-DAIs.

Proposal 3: When more than one NACK-only based feedback is available for transmission in the same PUCCH slot,

- If Alt 1 is used, the PUCCH for transmission is determined in the same way as ACK/NACK mode.

- If Alt 4 is used,

    - The PUCCH slot is determined based on the K1 indication in each corresponding scheduling DCI

    - The mapping between TBs and PUCCH resources are predefined.

    - This applies to a single G-RNTI.

- Two separate UE capabilities should be defined for alt 1 and alt 4 correspondingly.

Proposal 4: When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, the following solutions can be considered.

- Option 1: If SR is positive, NACK-only is dropped. If SR is negative, transmit NACK-only PUCCH

- Option 2: If SR is positive, transmit NACK-only on SR PUCCH resource. If SR is negative, transmit NACK-only PUCCH.

  - Option 2-1: only NACK-only is transmitted on SR PUCCH resource. If it is ACK, transmit SR only.

- Option 3: When SR is positive, for the case that both SR PUCCH and NACK-only are PUCCH format 1, transmit NACK on SR using the same rule as NR R15/R16. Otherwise, NACK-only/SR is dropped. When SR is negative, transmit NACK-only PUCCH.

Proposal 5: For Type-1 codebook generation,

- If at least one configured G-RNTI is with HARQ-ACK enabled, UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

- If the UE is configured with disabled HARQ feedback for multicast SPS scheduling, UE will report NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

- References

- RAN WG1 #108 e-meeting Chairman’s Notes