3GPP TSG RAN WG1 #109-e                                     R1-2203527

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Maintenance on NR Multicast and Broadcast Services

Agenda Item:	8.12.2

Document for:  Discussion and Decision

- Introduction

In this contribution, we will make discussions on NR Multicast and Broadcast Services in terms of SPS PDSCH overlapping handling, PDSCH capability, corrections on type 1 HARQ-ACK codebook in PUSCH, HARQ process for MCCH and Broadcast MTCH(s) and MCCH/MTCH search space.

- Discussion

SPS PDSCH overlapping handling in FDM case

The issue of SPS PDSCH overlapping in FDM case was discussed several times in previous meetings but no consensus was achieved. Based on the discussion, if UE is capable of receiving FDMed unicast PDSCH and multicast PDSCH, in a slot, for the following listed cases which are also shown in Figure 1, it seems that the first three cases are acceptable for most companies. For the last two cases, companies seem have different views.

- Case 1: Only TDMed unicast PDSCHs

- Case 2: Only TDMed multicast PDSCHs

- Case 3: One unicast PDSCH and one multicast PDSCH, where the two PDSCHs are overlapped in time domain but non-overlapped frequency domain

- Case 4: One unicast PDSCH and one multicast PDSCH, where the two PDSCHs are non-overlapped in both time domain and frequency domain

- Case 5: One unicast PDSCH and one multicast PDSCH, where the two PDSCHs are non-overlapped in time domain but overlapped in frequency domain

Figure 1 (potential) supported cases when a UE is capable of receiving FDMed unicast PDSCH and multicast PDSCH

In our understanding, FDM between unicast PDSCH and multicast PDSCH only requires that unicast PDSCH and multicast PDSCH are transmitted in different frequency domain, regardless whether they are overlapped in time domain or not. Thus, Case 4 should also be supported. For case 5, considering that case 1 and case 2 are supported, there is no additional complexity for UE to support case 5. Thus, supporting of case 5 is also fine to us.

In addition, when making solution to handle the overlapping between SPS PDSCHs for a UE capable of receiving FDMed unicast PDSCH and multicast PDSCH, the principle should be selecting SPS PDSCHs with the lowest SPS PDSCH configuration index(es) as many as possible based on UE’s capability.

During the discussion in the last meeting, the following proposal was given.

Updated proposal 5-2a: If a UE supports FDM reception between unicast SPS PDSCH and multicast SPS PDSCH in a slot, and if more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, - if the PDSCHs include both unicast SPS PDSCH(s) and multicast SPS PDSCH(s), the UE resolves collisions among unicast SPS PDSCHs resulting in one unicast SPS PDSCH and collisions among multicast SPS PDSCHs resulting in one multicast SPS PDSCH as in Rel-16, respectively. If the resulting unicast SPS PDSCH and multicast SPS PDSCH overlap in both time and frequency, the UE receives the one with lower configured sps-ConfigIndex; else, the UE receives both PDSCHs. - if the PDSCHs only include unicast SPS PDSCH(s) or only include multicast SPS PDSCH(s), the legacy procedure is applied. - FFS: whether a separate UE capability is needed for FDM reception between unicast SPS PDSCH and multicast SPS PDSCH in a slot. - FFS: how to resolve the collision when further considering DG PDSCH(s).

There are some considerations for the above proposal.

- For the case that the PDSCHs include both unicast SPS PDSCH(s) and multicast SPS PDSCH(s), the proposed solution will result in unnecessary dropping which is explained in Figure 2, where only unicast SPS 1 is received based on the proposal while UE is capable to receive SPS 1 and SPS 2 or SPS 1 and SPS 4.

- For the UE capability on FDM reception between unicast PDSCH and multicast PDSCH in a slot, the same principle should be applied no matter it is SPS PDSCH or DG SPS. Additional constraints on SPS case should be avoided.

- For the overlapping between DG PDSCHs and SPS PDSCHs, in NR R15/16, DG PDSCH will overwrite SPS PDSCH if their time domain resources are overlapped. In NR R17, if UE is capable of receiving FDMed unicast PDSCH and multicast PDSCH, the overwriting should happen only when DG PDSCH and SPS PDSCH are overlapped in both time domain and frequency domain.

Figure 2 Example for SPS PDSCHs overlapping

For the handling of SPS PDSCHs overlapping, we have the following proposal where the content in [] is based on the assumption that case 5 is supported.

Proposal 1:

If a UE supports FDM reception between unicast SPS PDSCH and multicast SPS PDSCH in a slot, and if more than one PDSCH without a corresponding PDCCH transmission are in a slot on a serving cell,

- if the PDSCHs include both unicast SPS PDSCH(s) and multicast SPS PDSCH(s), the UE resolves collision as following

  - Step 0: Q is the set of activated PDSCHs without corresponding PDCCH transmissions within the slot.

  - Step 1: A UE receives one PDSCH with the lowest configured sps-ConfigIndex within Q. Designate the received PDSCH as survivor PDSCH.

  - Step 2: If the survivor PDSCH in step 1 is unicast PDSCH, a UE receives one multicast PDSCH with the lowest configured sps-ConfigIndex within Q (if any), where the multicast PDSCH and the survivor PDSCH in step 1 are FDMed in frequency [or TDMed in time]. If the survivor PDSCH in step 1 is multicast PDSCH, a UE receives one unicast PDSCH with the lowest configured sps-ConfigIndex within Q (if any), where the unicast PDSCH and the survivor PDSCH in step 1 are FDMed in frequency [or TDMed in time].

- if the PDSCHs only include unicast SPS PDSCH(s) or only include multicast SPS PDSCH(s), the legacy procedure is applied.

- FFS: how to resolve the collision when further considering DG PDSCH(s).

The corresponding TP to 38.214 based on proposal 1 is shown below:

*************************************Start of TP********************************************

5	Physical downlink shared channel related procedures

5.1	UE procedure for receiving the physical downlink shared channel

< Unchanged parts are omitted >

If more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, after resolving overlapping with symbols in the slot indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated, a UE receives one or more PDSCHs without corresponding PDCCH transmissions in the slot as specified below.

‒	Step 0: set j=0, where j is the number of selected PDSCH(s) for decoding. Q is the set of activated PDSCHs without corresponding PDCCH transmissions within the slot

‒	Step 1: A UE receives one PDSCH with the lowest configured sps-ConfigIndex within Q, set j=j+1. Designate the received PDSCH as survivor PDSCH.

‒	Step 2:

- If the UE is capable of receiving FDMed unicast and multicast PDSCH per slot per carrier and Q includes both unicast SPS PDSCH(s) and multicast SPS PDSCH(s)

  - If the survivor PDSCH in step 1 is unicast PDSCH, the UE receives one multicast PDSCH with the lowest configured sps-ConfigIndex within Q (if any), where the multicast PDSCH and the survivor PDSCH in step 1 are FDMed in frequency domain [or TDMed in time domain].

  - If the survivor PDSCH in step 1 is multicast PDSCH, UE receives one unicast PDSCH with the lowest configured sps-ConfigIndex within Q (if any), where the unicast PDSCH and the survivor PDSCH in step 1 are FDMed in frequency domain [or TDMed in time domain].

  - The UE stops the pseudo code.

- Otherwise, Tthe survivor PDSCH in step 1 and any other PDSCH(s) overlapping (even partially) with the survivor PDSCH in step 1 are excluded from Q.

‒	Step 3: Repeat step 1 and 2 until Q is empty or j is equal to the number of unicast/multicast PDSCHs in a slot supported by the UE

*************************************End of TP********************************************

Corrections on type 1 HARQ-ACK codebook in PUSCH

For type 1 codebook generation, fallback operation was agreed for multicast in the last meeting.

Agreement - For the following two cases, the fallback operation for the Type-1 HARQ-ACK codebook for multicast is applied to G-RNTI/G-CS-RNTI configured with HARQ-ACK enabled only, - a SPS PDSCH release indicated by DCI format 4_1 with counter DAI field value of 1, - a PDSCH reception scheduled by DCI format 4_1 with counter DAI field value of 1 on the PCell, - SPS PDSCH reception(s) associated with G-CS-RNTIs. - Note: If the UE receives two SPS PDSCH releases for two respective G-CS-RNTIs with DAI=1, or two PDSCHs by DCI 4_1 with DAI=1 for two respective G-RNTIs on the PCell, there is no fallback.

However, this was not correctly captured in TS 38.213 9.1.2.2 [1]. The correction is proposed as following.

*************************************Start of TP********************************************

#### 9.1.2.2	Type-1 HARQ-ACK codebook in physical uplink shared channel

If a UE is not provided pdsch-HARQ-ACK-Codebook = ‘semi-static’ for unicast or multicast HARQ-ACK information, the UE does not multiplex the unicast or multicast HARQ-ACK information in the PUSCH transmission, respectively.

If a UE is provided pdsch-HARQ-ACK-Codebook = ‘semi-static’ for unicast and/or multicast HARQ-ACK information, and would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format that does not include a DAI field, then

-	if the UE has not received any PDSCH or SPS PDSCH release or TCI state update that the UE multiplexes corresponding HARQ-ACK information in the PUSCH, based on a value of a respective PDSCH-to-HARQ_feedback timing indicator field in a DCI format scheduling the PDSCH reception or the SPS PDSCH release or the TCI state update, or on the value of dl-DataToUL-ACK or dl-DataToUL-ACK-r16 if the PDSCH-to-HARQ_feedback timing indicator field is not present in DCI format 1_1 or on the value of dl-DataToUL-ACK-ForDCI-Format1-2 if the PDSCH-to-HARQ_feedback timing indicator field is not present in DCI format 1_2 and the UE is provided pdsch-HARQ-ACK-Codebook = ‘semi-static’ for unicast HARQ-ACK information, or on the value of dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in DCI format 4_2 and the UE is provided pdsch-HARQ-ACK-Codebook = ‘semi-static’ for multicast HARQ-ACK information, in any of the Mc occasions for candidate PDSCH receptions by a DCI format or SPS PDSCH on any serving cell c, as described in clause 9.1.2.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission

-	else the UE generates the HARQ-ACK codebook as described in clause 9.1.2.1, except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH, unless the UE receives only a SPS PDSCH release, or only a SPS PDSCH reception(s), or only a TCI state update, or only a PDSCH that is scheduled by DCI format 1_0 with a counter DAI field value of 1 if the UE is provided pdsch-HARQ-ACK-Codebook = ‘semi-static’ for unicast HARQ-ACK information, or is scheduled by DCI format 4_1 with a counter DAI field value of 1 if the UE is provided pdsch-HARQ-ACK-Codebook = ‘semi-static’ for multicast HARQ-ACK information, on the PCell in the Mc occasions for candidate PDSCH receptions in which case the UE generates HARQ-ACK information only for the SPS PDSCH release or only for the PDSCH reception or only for the TCI state update as described in clause 9.1.2

*************************************End of TP********************************************

HARQ process for MCCH and Broadcast MTCH(s)

In RAN2’s LS [1], it states that RAN2 has agreed there are no dedicated HARQ process IDs for MCCH and Broadcast MTCH with an assumption on single HARQ process for MCCH and single HARQ process for Broadcast MTCH(s). RAN2 would like to confirm this understanding with RAN1. Regarding to HARQ process for MCCH and Broadcast MTCH(s), it has agreed that HARQ process ID is not indicated in DCI format 4_0 for both MCCH and MTCH. There is also a conclusion that additional HARQ process(es) is(are) not introduced for Rel-17 MBS broadcast reception on serving cell. Additionally, RAN 1 has also agreed that a UE is not required to support reception of FDMed MCCH PDSCH and MTCH PDSCH in PCell as well as a UE is not required to support reception of FDMed multiple MTCH PDSCHs in PCell for RRC_IDLE/INACTIVE UEs. Thus, single HARQ process is enough for MCCH or broadcast MTCH(s) and it is up to UE implementation to perform HARQ combination for the MCCH or broadcast MTCH. Furthermore, it is up to UE implementation on whether MCCH and broadcast MTCH(s) share the same HARQ process or not.

Proposal 2: Single HARQ process is used for MCCH.

Proposal 3:Single HARQ process is used for broadcast MTCH.

MCCH/MTCH search space

RAN2 has decided to include MCCH/MTCH search space configuration of MBS broadcast as part of PDCCH-ConfigCommon [2], and thus, the corresponding changes are needed to replace pdcch-Config-MCCH and pdcch-Config-MTCH in 38.213 [3].

*************************************Start of TP********************************************

10.1	UE procedure for determining physical downlink control channel assignment

< Unchanged parts are omitted >

A set of PDCCH candidates for a UE to monitor is defined in terms of PDCCH search space sets. A search space set can be a CSS set or a USS set. A UE monitors PDCCH candidates in one or more of the following search spaces sets

-	a Type0-PDCCH CSS set on the primary cell of the MCG configured by

-	pdcch-ConfigSIB1 in MIB or by searchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a SI-RNTI, or

-	searchSpaceZero in PDCCH-ConfigCommon, when searchSpaceBroadcast is not provided in PDCCH-ConfigCommon, for a DCI format 4_0 with CRC scrambled by a MCCH-RNTI or a G-RNTI

-	a Type0A-PDCCH CSS set configured by searchSpaceOtherSystemInformation in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type0B-PDCCH CSS set configured by searchSpaceBroadcast in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a MCCH-RNTI or a G-RNTI

-	a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a RA-RNTI, a MsgB-RNTI, or a TC-RNTI on the primary cell

-	a Type1A-PDCCH CSS set configured by sdt-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a C-RNTI or a CS-RNTI on the primary cell as described in clause 19.1

-	a Type2-PDCCH CSS set configured by pagingSearchSpace in PDCCH-ConfigCommon for a DCI format 1_0 with CRC scrambled by a P-RNTI on the primary cell of the MCG

-	a Type2A-PDCCH CSS set configured by peiSearchSpace in DownlinkConfigCommonSIB for a DCI format 2_7 with CRC scrambled by a RNTI on the primary cell of the MCG

-	a Type3-PDCCH CSS set configured by

-	SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, or CI-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, CS-RNTI(s), or PS-RNTI, or

-	SearchSpace-Multicast in PDCCH-Config-Multicast for DCI formats with CRC scrambled by G-RNTI, or G-CS-RNTI, or

-

< Unchanged parts are omitted >

If a UE monitors PDCCH candidates for DCI formats with CRC scrambled by a C-RNTI and the UE is provided a non-zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, or monitors PDCCH candidates for DCI formats with CRC scrambled by a MCCH-RNTI or a G-RNTI and the UE is provided a non-zero value for searchSpaceBroadcast in PDCCH-ConfigCommon for a Type0/0B-PDCCH CSS set, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set, or of the Type0/0B-PDCCH set, respectively, based on the search space set associated with the value of searchSpaceID.

*************************************End of TP********************************************

- Conclusion

In this contribution, we make discussions on NR Multicast and Broadcast Services and have the following proposals.

Proposal 1:

If a UE supports FDM reception between unicast SPS PDSCH and multicast SPS PDSCH in a slot, and if more than one PDSCH without a corresponding PDCCH transmission are in a slot on a serving cell,

- if the PDSCHs include both unicast SPS PDSCH(s) and multicast SPS PDSCH(s), the UE resolves collisions among unicast SPS PDSCHs resulting in one unicast SPS PDSCH and collisions among multicast SPS PDSCHs resulting in one multicast SPS PDSCH as following

  - Step 0: Q is the set of activated PDSCHs without corresponding PDCCH transmissions within the slot.

  - Step 1: A UE receives one PDSCH with the lowest configured sps-ConfigIndex within Q. Designate the received PDSCH as survivor PDSCH.

  - Step 2: If the survivor PDSCH in step 1 is unicast PDSCH, a UE receives one multicast PDSCH with the lowest configured sps-ConfigIndex within Q (if any), where the multicast PDSCH and the survivor PDSCH in step 1 are FDMed in frequency. If the survivor PDSCH in step 1 is multicast PDSCH, a UE receives one unicast PDSCH with the lowest configured sps-ConfigIndex within Q (if any), where the unicast PDSCH and the survivor PDSCH in step 1 are FDMed in frequency.

- if the PDSCHs only include unicast SPS PDSCH(s) or only include multicast SPS PDSCH(s), the legacy procedure is applied.

- FFS: how to resolve the collision when further considering DG PDSCH(s).

Proposal 2: Single HARQ process is used for MCCH.

Proposal 3:Single HARQ process is used for broadcast MTCH.

- References

- R2-2204017 LS on HARQ process for MCCH and Broadcast MTCH(s)

- R1-2200882 LS on MBS issues

- R1-2202951 CR_38.213-MBS