3GPP TSG RAN WG1 #109-e		R1-2203528

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Maintenance on Scell scheduling P(S)cell

Agenda Item:	8.13

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the maintenance issues on Scell scheduling P(S)cell. For simplicity, the ‘SCell used for scheduling P(S)Cell’ is denoted as sScell (Special Scell) and other Scell as N-Scell (Normal Scell) in the later part.

- Discussion

  - Clarification on the determination of PDCCH candidate

In TS 38.213 [1], PDCCH candidate is determined according to the following spec:

For a search space set s associated with CORESET p, the CCE indexes for aggregation level L corresponding to PDCCH candidate ms,nCI(L) of the search space set in slot ns,fμ for an active DL BWP of a serving cell corresponding to carrier indicator field value nCI are given by L⋅Yp,ns,fμ+ms,nCI(L)⋅NCCE,pL⋅Ms,maxL+nCImodNCCE,pL+i where for any CSS, Yp,ns,fμ=0; for a USS, Yp,ns,fμ=Ap⋅Yp,ns,fμ-1modD, Yp,-1=nRNTI≠0, Ap=39827 for pmod3=0, Ap=39829 for pmod3=1, Ap=39839 for pmod3=2, and D=65537; i=0,⋯,L-1; NCCE,p is the number of CCEs, numbered from 0 to NCCE,p-1, in CORESET p and, if any, per RB set; nCI is the carrier indicator field value if the UE is configured with a carrier indicator field by CrossCarrierSchedulingConfig for the serving cell on which PDCCH is monitored; otherwise, including for any CSS, nCI=0; ms,nCI(L)=0,⋯,Ms,nCIL-1, where Ms,nCIL is the number of PDCCH candidates the UE is configured to monitor for aggregation level L of a search space set s for a serving cell corresponding to nCI;

At RAN1#107-e meeting [1], the following agreement is made:

Agreement Confirm the WA from RAN1#106bis-e with addition of below Note (shown in blue) Working Assumption - When CIF for sSCell to Pcell cross-carrier scheduling is configured, non-fallback DCI formats on P(S)Cell include same number of CIF bits as the corresponding non-fallback DCI formats on sSCell that are used for sSCell to P(S)Cell scheduling - Note: per RAN1#102-e agreement, when sSCell to P(S)Cell scheduling is configured for the UE, cross-carrier scheduling from P(S)Cell to another cell is not allowed. The CIF bits included in non-fallback DCI formats on P(S)Cell are considered reserved.

In R15/R16, CrossCarrierSchedulingConfig and the above procedures are specified for a scheduled cell that only has one scheduling cell. In the CCS case, the scheduled cell must be configured with CrossCarrierSchedulingConfig, and nCI corresponding to a DCI scheduling this cell from the scheduling cell should be set to the value of cif-InSchedulingCell. For the scheduling cell, it may or may not be configured with CrossCarrierSchedulingConfig.

When sScell scheduling P(S)cell is configured, there are two scheduling cells for P(S)cell. For CCS part, the above text can be directly applied, and thus  nCI  corresponding to the DCI scheduling P(S)cell from sScell is set to the value of the cif-InSchedulingCell value configured for the P(S)cell. However, if we simply reuse the highlighted part to determine the nCI for P(S)cell self-scheduling, nCI would be set to CIF value in DCI for self-scheduling which is considered reserved as per agreement. The most straightforward way is to set the nCI to 0, thus the following TP is proposed.

- TP#1 (TS 38.213 v17.1.0 [1], Section 10.1)

For a search space set s associated with CORESET p, the CCE indexes for aggregation level L corresponding to PDCCH candidate ms,nCI(L) of the search space set in slot ns,fμ for an active DL BWP of a serving cell corresponding to carrier indicator field value nCI are given by L⋅Yp,ns,fμ+ms,nCI(L)⋅NCCE,pL⋅Ms,maxL+nCImodNCCE,pL+i where for any CSS, Yp,ns,fμ=0; for a USS, Yp,ns,fμ=Ap⋅Yp,ns,fμ-1modD, Yp,-1=nRNTI≠0, Ap=39827 for pmod3=0, Ap=39829 for pmod3=1, Ap=39839 for pmod3=2, and D=65537; i=0,⋯,L-1; NCCE,p is the number of CCEs, numbered from 0 to NCCE,p-1, in CORESET p and, if any, per RB set; If the serving cell is a primary cell and if it is configured to be scheduled from both the primary cell and a secondary cell: - for the scheduling on the primary cell from the primary cell, nCI=0 - for the scheduling on the primary cell from the secondary cell, nCI is the carrier indicator field value by CrossCarrierSchedulingConfig for the serving cell; Else, - nCI is the carrier indicator field value if the UE is configured with a carrier indicator field by CrossCarrierSchedulingConfig for the serving cell on which PDCCH is monitored; otherwise, including for any CSS, nCI=0; ms,nCI(L)=0,⋯,Ms,nCIL-1, where Ms,nCIL is the number of PDCCH candidates the UE is configured to monitor for aggregation level L of a search space set s for a serving cell corresponding to nCI;

Observation 1. When sScell scheduling P(S)cell is configured, nCI for P(S)cell self-scheduling would be set to a reserved field if directly following the legacy spec as the CIF in DCI for P(S)cell self-scheduling is considered reserved.

Proposal 1: When sScell scheduling P(S)cell is configured, nCI for P(S)cell self-scheduling is 0, and adopt TP#1 for 38.213.

  - Handling of sScell deactivation/dormancy

Agreement

- When CCS from sSCell to PCell/PSCell is configured, CA activation/deactivation operation for the sSCell is supported

Agreement

- Specification supports dormant BWP operation on sSCell for a UE is configured CCS from sSCell to P(S)Cell.

Regarding sScell deactivation/dormancy, the following agreements are made in RAN1#104e and RAN1#106e respectively:

Agreement

- For a UE configured for CCS from sSCell to P(S)Cell, scaling factor  is not applied for PDCCH overbooking/BD/CCE limit computation when sSCell is deactivated, or when an activated sSCell is switched to dormant BWP; otherwise scaling factor  is applied

  - Timing for disabling scaling factor  when sSCell is deactivated follows sSCell deactivation timing in current specifications, i.e., no later than the minimum requirement defined in TS 38.133 as captured in 38.213 subclause 4.3

  - Timing for disabling scaling factor  follows the non-dormant to dormant BWP switching delay in current specifications ( TS 38.133).

  - Introduce separate FG to indicate UE support for disabling scaling factor  when sSCell is deactivated

  - Introduce separate FG to indicate UE support for disabling scaling factor  when activated sSCell is switched to dormant BWP

  - Note: It is up to UE implementation whether/when to apply the scaling factor α during sSCell activation and during sSCell BWP switch

Regarding BD/CCE budget calculation, the following agreement is made in RAN1#108e [2]:

According to the above agreements, sScell could become deactivated or dormant when it is configured to schedule P(S)Cell. Besides, the BD/CCE budget limit computation returns to the normal state (i.e. scaling factor is not applied) when sScell becomes deactivated or dormant.

When P(S)cell is configured to be scheduled by a sScell, a typical configuration for P(S)cell self-scheduling is CSS only with no USSs or very sparse USSs on P(S)cell. Consequently, after the sScell becomes deactivated or dormant, there would be very limited scheduling opportunities for P(S)cell self-scheduling. In this case, the BD/CCE budget for P(S)cell self-scheduling becomes larger due to that scaling factor is not applied. Then how to use the increased BD/CCE budget should be considered. A possible solution is to configure more USS to the UE by RRC in P(S)cell. However, it increases the PDCCH overhead in P(S)cell, which is problematic in DSS case due to limited PDCCH resources in P(S)cell. Moreover, it essentially makes the feature of SCell deactivation/dormancy useless if every deactivation MAC CE or dormancy DCI command requires a subsequent RRC reconfiguration. To avoid this, fallback USSs could be configured when sScell scheduling P(S)cell is configured. When sScell is active or non-dormant, these USSs are not monitored by the UE. After sScell becomes deactivated or dormant, UE starts to monitor the fallback USSs.

Proposal 2: Support configuring fallback USSs on P(S)cell when sScell scheduling P(S)cell is configured:

- If sScell is active or non-dormant, UE doesn’t monitor the fallback USSs on P(S)cell;

- If sScell becomes deactivated or dormant, UE starts to monitor the fallback USSs on P(S)cell.

  - Behavior for UEs supporting both FG 34-1 and 34-2

In RAN1#108e [2], FG 34-1 and 34-2 are defined separately for UEs supporting CCS from Scell to Pcell.

For Type A UEs supporting 34-1 only, there are several search space restrictions, i.e. sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and at least the following search space sets on PCell/PSCell can only be configured such that UE does not monitor them in an overlapping slot of PCell/PSCell and sSCell: USS sets for DCI formats 0_0, 1_0, 0_1,1_1,0_2,1_2 and Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI. On the other hand, sSCell USS set(s) (for CCS from sSCell to Pcell/PSCell) and Type0/0A/1/2 CSS sets on Pcell/PSCell can be configured so that the UE monitors them in an overlapping slot of Pcell/PSCell and sSCell:

- no simultaneous monitoring between ‘USS sets (for P(S)Cell scheduling) on sSCell’ and ‘Type 0/0A/1/2/CSS sets on P(S)Cell for DCI formats with CRC scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI’;

- simultaneous monitoring of ‘USS sets (for P(S)Cell scheduling) on sSCell’ and ‘Type 0/0A/1/2/CSS sets on P(S)Cell for DCI formats with CRC not scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI’.

For Type B UEs supporting 34-2 only, there is no such search space restriction, i.e. UE monitors PDCCH according to the configured search spaces.

However, for UEs reporting support of both 34-1 and 34-2, the UE behavior may be unclear especially when sSCell USS set(s) (for CCS from sSCell to Pcell/PSCell) and Type0/0A/1/2 CSS sets on Pcell/PSCell are configured with overlapping slots. In this case, there are two different interpretations of the UE behavior in the overlapping slots : 1) UE monitors DCIs from sScell and Pcell/PSCell simultaneously for any RNTI following 34-2; or 2) UE does not monitor DCI formats with CRC scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI from Pcell/PSCell and sScell simultaneously. In other words, once UE has decoded a DCI format with CRC scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI from sScell, it may stop blind decoding for DCI formats with CRC scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI for Pcell/PSCell self-scheduling in the overlapping slots, as described in 34-1. To make it clear, there are the following two options:

- Option 1: Define a default behavior in this case, e.g. 34-2 behavior is the default behavior if UEs support both 34-1 and 34-2.

- Option 2: gNB indicates the UE which behavior to apply by RRC signaling.

Considering the maintenance state at this point, option 1 is slightly preferred due to no RRC signaling impact.

Proposal 3: For UEs signaling support of both FG 34-1 and FG 34-2, the behavior for 34-2 is applied when sScell scheduling Pcell is configured.

- Conclusion

In this contribution, we focus on Scell scheduling P(S)cell, and have the following proposals:

Observation 1. When sScell scheduling P(S)cell is configured, nCI for P(S)cell self-scheduling will be set to a reserved field if directly following the legacy spec as the CIF in DCI for P(S)cell self-scheduling is considered reserved.

Proposal 1: When sScell scheduling P(S)cell is configured, nCI for P(S)cell self-scheduling is 0, and adopt TP#2 for 38.213.

Proposal 2: Support configuring fallback USSs on P(S)cell when sScell scheduling P(S)cell is configured:

- If sScell is active or non-dormant, UE doesn’t monitor these USSs;

- If sScell becomes deactive or dormant, UE starts to monitor these USSs.

Proposal 3: For UEs signaling support of both FG 34-1 and FG 34-2, the behavior for 34-2 is applied when sScell scheduling Pcell is configured.

- References

- TS 38.213 v17.1.0, Physical layer procedures for control, April 2022.

- Chairman notes, RAN1#108-e, February 21st – March 3rd, 2022.