3GPP TSG RAN WG1 #109-e	R1-2203529

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Discussion on UE features for further enhancements on NR MIMO

Agenda Item:	8.16.1

Document for:	Discussion and Decision

Introduction

Significant progress on Rel-17 FeMIMO UE features was achieved in RAN1#108-e, in this contribution we provide further our views on necessary revisions.

Discussion

FG 23-1-1: Unified TCI for intra- and inter-cell beam management

For FG 23-1-1c, in Rel-17 unified TCI framework, PCell and SCell BFR are both supported and specified. The following UE capability for supporting PCell BFR with unified TCI framework should be included in this FG since for FR1 this feature is not mandatory.

Support independent FG for PCell BFR.

| 23. NR_FeMIMO | 23-1-1o | PCell BFR with unified TCI framework | 1. Support of PCell BFR with unified TCI framework |
| --- | --- | --- | --- |

The current UE feature for unified TCI framework only includes joint TCI, the separate TCI mode should be introduced as UE feature groups. At least two FGs are needed, one for the MAC CE based TCI state indication and another for DCI based beam indication. For inter-cell beam management, it is also preferred to have a separate FG.

Introduce separate TCI mode as UE feature groups for both intra-cell beam management and inter-cell beam management.

| 23. NR_FeMIMO | 23-1-1 | Unified TCI with joint DL/UL TCI update for intra- [and inter-cell] beam management | - Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) - WA: The maximum number of configured joint TCI states [per BWP per CC] [in a band] [in a band combination] - One MAC-CE activated joint TCI state per CC [in a band] [in a band combination] - TCI state indication [mode]: update and activation [in case of updates] a) MAC CE based TCI state indication [for one active TCI state] - The maximum number of MAC-CE activated joint TCI states across all CC(s) in a band |
| --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-1b | Unified TCI with joint DL/UL TCI update for intra- [and inter-cell] beam management with more than one MAC-CE activated joint TCI state per CC | - TCI state indication [mode]: update and activation [in case of updates] b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS] - The maximum number of MAC-CE activated joint TCI states per CC [in a band] [in a band combination] - [The minimum time gap between the beam indication PDCCH and first slot where beam is applied] |

| 23. NR_FeMIMO | 23-1-1k | Unified TCI with separate DL/UL TCI update for intra-cell beam management | Support separate DL/UL TCI update with one MAC-CE activated separate TCI state pair per CC [in a band] [in a band combination] | 23-1-1 | Yes |
| --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-1l | Unified TCI with separate DL/UL TCI update for intra-cell beam management with DCI-based TCI state indication per CC | - TCI state indication [mode]: update and activation [in case of updates] 1) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) 2) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - The maximum number of MAC-CE activated separate TCI states per CC [in a band] [in a band combination] | 23-1-1 | Yes |
| 23. NR_FeMIMO | 23-1-1m | Unified TCI with joint DL/UL TCI update for and inter-cell beam management | Support joint DL/UL TCI update with one MAC-CE activated separate TCI state pair per CC [in a band] [in a band combination] | 23-1-1 |  |
| 23. NR_FeMIMO | 23-1-1n | Unified TCI with joint DL/UL TCI update for inter-cell beam management with DCI-based TCI state indication per CC | - TCI state indication [mode]: update and activation [in case of updates] 1) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) 2) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - The maximum number of MAC-CE activated separate TCI states per CC [in a band] [in a band combination] | 23-1-1 |  |
| 23. NR_FeMIMO | 23-1-1o | Unified TCI with separate DL/UL TCI update for inter-cell beam management | Support separate DL/UL TCI update with one MAC-CE activated separate TCI state pair per CC [in a band] [in a band combination] | 23-1-1 | Yes |
| 23. NR_FeMIMO | 23-1-1p | Unified TCI with separate DL/UL TCI update for inter-cell beam management with DCI-based TCI state indication per CC | - TCI state indication [mode]: update and activation [in case of updates] 1) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) 2) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - The maximum number of MAC-CE activated separate TCI states per CC [in a band] [in a band combination] | 23-1-1 | Yes |

Update FG 23-1-1j as follows.

| 23. NR_FeMIMO | 23-1-1j | Indication/configuration of R17 TCI states for CORESET #0 | Support of indication/configuration of R17 TCI states for CORESET #0 [and the respective PDSCH] reception reusing the Rel-15/16 signaling/configuration design(s) |
| --- | --- | --- | --- |

FG 23-1-2: Inter-cell measurement and reporting (for inter-cell BM [and mTRP])

For FG 23-1-2 on the inter-cell beam measurement/report, suggest considering the following changes.

- Suggest removing the bracket on “for both inter-cell BM and mTRP”, since the measurement should be applicable to both scenarios.

- For component 2, suggest removing THE“at least one [pair/beam]”, since beam selection and report rule is up to UE implementation, no need to be specified.

- For component 3, prefer to have different reported numbers of RRC configured PCI(s) for L1-RSRP measurement associated with periodic, aperiodic, and semi-persistent reporting in FR1 and FR2, respectively.

- For component 4, 5, suggest removing the bracket on “across all CC”.

- For component 6, 7, suggest deleting the component since the measurement is always outside SMTC.

- For component 4,5, they should be counted in FG16-1g/16-1g-1, since an inter-cell L1-RSRP measurement is still an L1-RSRP measurement.

- For component 8, 9, suggest combining the two components and keeping 8. The bracket of component 8 can be removed since the non-overlapped measurement resources would be simpler than overlapped cases from the UE perspective.

- Additionally, to improve the DL resource efficiency, simultaneous reception of PDCCH/PDSCH and SSBs for L1-RSRP measurement on the same REs should be supported by UE. Otherwise, in addition to serving cell SSBs, the PDCCH/PDSCH also needs to be rate matched around the SSBs configured for inter-cell L1-RSRP measurement. Therefore, a new feature group is introduced in FG 23-1-2a.

| 23. NR_FeMIMO | 23-1-2 | Inter-cell beam measurement and reporting [(for inter-cell BM [and mTRP])] | 1. Support of L1-RSRP measurement and reporting on SSB(s) with PCI(s) different from serving cell PCI 2. Support of up to K[=4] SSBRI-RSRP [pairs/beams] in one report [where at least one [pair/beam] associated with a PCI different from serving cell PCI can be reported] (FFS: if K is a component candidate value) 3. The maximum number of [RRC-configured] PCI(s) different from serving cell PCI for L1-RSRP measurement] (FFS: whether to split this for FR1 and FR2) (FFS: whether/how to capture different values/behaviors for periodic/aperiodic/semi-persistent L1-RSRP measurement) 4. The max number of SSB resources configured to measure L1-RSRP within a slot with PCI(s) same as or different from serving cell PCI [across all CC] [5. The max number of SSB resources configured to measure L1-RSRP with PCI(s) same as or different from serving cell PCI [across all CC]] [6. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement] [7. Supported mode inter-cell measurement: {inside SMTC, both inside and outside SMTC}] [8. Supported mode of measurement over overlappedSSBs: {non-overlapped, both overlapped and non-overlapped}] [9. Maximum number of overlapped SSBs in one SSB resource for L1-RSRP measurement] | [2-24, 2-29] | Yes |  | Inter-cell beam measurement and reporting [(for inter-cell BM [and mTRP])] is not supported | per band | n/a | n/a | n/a | [Note: Whether component 4 and/or 5 are also counted in FG16-1g/16-1g-1] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-2a | simultaneous reception of PDSCH/PDCCH and SSB on the same RE | 1. support simultaneous reception of PDCCH/PDSCH and SSBs for inter-cell L1-RSRP measurement on the same REs |  |  |  |  | per band | n/a | n/a | n/a |  | Optional with capability signalling |

FG 23-3-1: Multi-TRP PUSCH repetition

For the number of SRS resources configured in one SRS resource set for codebook-based transmission, candidate value 4 for Component 4 is only supported when UL full power transmission fullpowerMode2 is enabled. So, we suggest to remove it. A separate FG should be added for mTRP UL full power transmission fullpowerMode2.

Support separate UE feature group for multi-TRP PUSCH repetition supporting fullpowerMode2.

| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A) -codebook based | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 3. Support of two SRS resource sets with usage set to 'codebook' 4. Supported number of SRS resources in one SRS resource set | FFS | Yes |  | Multi-TRP PUSCH repetition (type A) is not supported for codebook based | per FS | n/a | n/a | n/a | Component 4 candidate values: {1,2 [,4]} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 23. NR_FeMIMO | 23-3-1-1 -codebook based | Multi-TRP PUSCH repetition (type B) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) for codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to ‘codebook’ 3. Supported number of SRS resources in one SRS resource set |  | Yes |  | Codebook based multi-TRP PUSCH repetition (type B) is not supported | Per FSPC | No | No | No | Component 3 candidate values: {1,2[,4]} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 23. NR_FeMIMO | XXX | Multi-TRP PUSCH repetition for fullpowerMode2 | 1. Support multi-TRP PUSCH repetition for fullpowerMode2 2. Supported number of SRS resources in one SRS resource set | 16-5c，23-3-1 | Yes |  |  | per FS | n/a | n/a | n/a | Component 4 candidate values: {1,2 4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 23-4: intercell mTRP

The FG 23-4 was agreed in RAN1#108-e with some candidate values for component 2 and 3 in square brackets.

In RAN1#106b-e, following agreements were made:

Agreement

Support two independent X values (X1, X2) are reported as a UE capability for two different assumptions on additional SSB time domain position and periodicity with respect to serving cell SSB.

- X1 (Case 1) = The maximum number of configured additional PCIs when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI

- X2 (Case 2) = The maximum number of configured additional PCIs when the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to Case 1

- Note: By definition, Case 1 and Case 2 cannot be enabled simultaneously

- Supported values for X1 and X2 include at least 0,1,2,3 and 7. FFS on other values

- This UE capability has FR1 and FR2 differentiation

And, in RAN1#108-e, further agreement on configurable values of additional PCIs was made:

Agreement

From RRC signaling perspective, the number of configured additional PCIs can be {1, 2, 3, 4, 5, 6, 7}

From above agreements, it is clear that the UE reported values for X1 and X2 are limited to 0, 1, 2, 3 and 7; while for the UEs reporting value of 7 the gNB can configure one of the values from 1~7. Furthermore, there is one FFS point on how to count X1 and X2, as there is agreement in RAN1 that Case 1 and Case 2 cannot be enabled simultaneously, that means X1 and X2 are counted independently. And, the note in square bracket regarding restriction on configuration of case 1 and case 2 can be captured in TS 38.306 or in TS 38.331 which is up to RAN2. Hence, we propose following revisions.

Support following updates in FG 23-4.

| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info 2. The maximum number of configured additional PCIs per CC is X1 (Case 1) when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI 3. The maximum number of configured additional PCIs per CC is X2 (Case 2) when the configurations of SSB time domain positions and periodicity of the additional PCIs is different with SSB time domain positions and periodicity of the serving cell PCI | 16-2a | Yes |  | IntCell-mTRP is not supported | Per band | n/a | n/a | n/a | Component 2 candidate values: {[0,]1,2,3,[4,5,6,]7} Component 3 candidate values: {0,1,2,3,[4,5,6,]7} Note: UE indicates a non-zero value for at least one of component 2 or component 3 FFS: how to count X1 and X2 [Note: case1 and case2 cannot be enabled simultaneously] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 23-8: SRS enhancement

In FG 23-8-3, a note was added to avoid Tx-switching report, i.e. component 2 and component 3, when component 1 is reported as xTyR with x = y.

In [3], LS NR from RAN2, there are two interpretations on possible combinations of the component 1 as below:

(a) the new capabilities is populated for a band in the band combination only if at least one xTyR entry for the band supports y>4

(b) the new capabilities is populated for a band in the band combination regardless of whether the band supports y>4

If the interpretation is (a), it means at least one xTyR with y > 4 should be supported in component 1. And, if the interpretation is (b) assumed, there is no restriction on capability reporting. For interpretation (a), the note will never occur as component 1 always supports at least xTyR with y>4. For interpretation (b), the note can be applicable to component 2 and 3 when component 1 only supports xTyR with x=y. If component 1 supports any of xTyR with y > 4 in interpretation (b), it is very similar with interpretation (a) that the note is useless. And, if component 1 only supports legacy capabilities with x < y, component 2 and component 3 should not be reported as tx-switching capabilities is already reported in Rel-15 entry of 2-55. We, thus, propose to revise the note as below:

For FG 23-5-3, revise the note as below:

- Component 2 and component 3 are not valid for the same values of xTyR in component 1 with Rel-15/16 UE capability reporting.

Conclusion

- Support independent FG for PCell BFR.

| 23. NR_FeMIMO | 23-1-1o | PCell BFR with unified TCI framework | 1. Support of PCell BFR with unified TCI framework |
| --- | --- | --- | --- |

The current UE feature for unified TCI framework only includes joint TCI, the separate TCI mode should be introduced as UE feature groups. At least two FGs are needed, one for the MAC CE based TCI state indication and another for DCI based beam indication. For inter-cell beam management, it is also preferred to have a separate FG.

Introduce separate TCI mode as UE feature groups for both intra-cell beam management and inter-cell beam management.

| 23. NR_FeMIMO | 23-1-1 | Unified TCI with joint DL/UL TCI update for intra- [and inter-cell] beam management | - Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) - WA: The maximum number of configured joint TCI states [per BWP per CC] [in a band] [in a band combination] - One MAC-CE activated joint TCI state per CC [in a band] [in a band combination] - TCI state indication [mode]: update and activation [in case of updates] a) MAC CE based TCI state indication [for one active TCI state] - The maximum number of MAC-CE activated joint TCI states across all CC(s) in a band |
| --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-1b | Unified TCI with joint DL/UL TCI update for intra- [and inter-cell] beam management with more than one MAC-CE activated joint TCI state per CC | - TCI state indication [mode]: update and activation [in case of updates] b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS] - The maximum number of MAC-CE activated joint TCI states per CC [in a band] [in a band combination] - [The minimum time gap between the beam indication PDCCH and first slot where beam is applied] |

| 23. NR_FeMIMO | 23-1-1k | Unified TCI with separate DL/UL TCI update for intra-cell beam management | Support separate DL/UL TCI update with one MAC-CE activated separate TCI state pair per CC [in a band] [in a band combination] | 23-1-1 | Yes |
| --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-1l | Unified TCI with separate DL/UL TCI update for intra-cell beam management with DCI-based TCI state indication per CC | - TCI state indication [mode]: update and activation [in case of updates] 1) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) 2) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - The maximum number of MAC-CE activated separate TCI states per CC [in a band] [in a band combination] | 23-1-1 | Yes |
| 23. NR_FeMIMO | 23-1-1m | Unified TCI with joint DL/UL TCI update for and inter-cell beam management | Support joint DL/UL TCI update with one MAC-CE activated separate TCI state pair per CC [in a band] [in a band combination] | 23-1-1 |  |
| 23. NR_FeMIMO | 23-1-1n | Unified TCI with joint DL/UL TCI update for inter-cell beam management with DCI-based TCI state indication per CC | - TCI state indication [mode]: update and activation [in case of updates] 1) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) 2) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - The maximum number of MAC-CE activated separate TCI states per CC [in a band] [in a band combination] | 23-1-1 |  |
| 23. NR_FeMIMO | 23-1-1o | Unified TCI with separate DL/UL TCI update for inter-cell beam management | Support separate DL/UL TCI update with one MAC-CE activated separate TCI state pair per CC [in a band] [in a band combination] | 23-1-1 | Yes |
| 23. NR_FeMIMO | 23-1-1p | Unified TCI with separate DL/UL TCI update for inter-cell beam management with DCI-based TCI state indication per CC | - TCI state indication [mode]: update and activation [in case of updates] 1) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) 2) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - The maximum number of MAC-CE activated separate TCI states per CC [in a band] [in a band combination] | 23-1-1 | Yes |

Update FG 23-1-1j as follows.

| 23. NR_FeMIMO | 23-1-1j | Indication/configuration of R17 TCI states for CORESET #0 | Support of indication/configuration of R17 TCI states for CORESET #0 [and the respective PDSCH] reception reusing the Rel-15/16 signaling/configuration design(s) |
| --- | --- | --- | --- |

For FG 23-1-2 on the inter-cell beam measurement/report, suggest considering the following changes.

- Suggest removing the bracket on “for both inter-cell BM and mTRP”, since the measurement should be applicable to both scenarios.

- For component 2, suggest removing THE“at least one [pair/beam]”, since beam selection and report rule is up to UE implementation, no need to be specified.

- For component 3, prefer to have different reported numbers of RRC configured PCI(s) for L1-RSRP measurement associated with periodic, aperiodic, and semi-persistent reporting in FR1 and FR2, respectively.

- For component 4, 5, suggest removing the bracket on “across all CC”.

- For component 6, 7, suggest deleting the component since the measurement is always outside SMTC.

- For component 4,5, they should be counted in FG16-1g/16-1g-1, since an inter-cell L1-RSRP measurement is still an L1-RSRP measurement.

- For component 8, 9, suggest combining the two component and keeping 8. The bracket of component 8 can be removed since the non-overlapped measurement resources would be simpler than overlapped cases from the UE perspective.

- Additionally, to improve the DL resource efficiency, simultaneous reception of PDCCH/PDSCH and SSBs for L1-RSRP measurement on the same REs should be supported by UE. Otherwise, in addition to serving cell SSBs, the PDCCH/PDSCH also needs to be rate matched around the SSBs configured for inter-cell L1-RSRP measurement. Therefore, a new feature group is introduced in FG 23-1-2a.

| 23. NR_FeMIMO | 23-1-2 | Inter-cell beam measurement and reporting [(for inter-cell BM [and mTRP])] | 1. Support of L1-RSRP measurement and reporting on SSB(s) with PCI(s) different from serving cell PCI 2. Support of up to K[=4] SSBRI-RSRP [pairs/beams] in one report [where at least one [pair/beam] associated with a PCI different from serving cell PCI can be reported] (FFS: if K is a component candidate value) 3. The maximum number of [RRC-configured] PCI(s) different from serving cell PCI for L1-RSRP measurement] (FFS: whether to split this for FR1 and FR2) (FFS: whether/how to capture different values/behaviors for periodic/aperiodic/semi-persistent L1-RSRP measurement) 4. The max number of SSB resources configured to measure L1-RSRP within a slot with PCI(s) same as or different from serving cell PCI [across all CC] [5. The max number of SSB resources configured to measure L1-RSRP with PCI(s) same as or different from serving cell PCI [across all CC]] [6. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement] [7. Supported mode inter-cell measurement: {inside SMTC, both inside and outside SMTC}] [8. Supported mode of measurement over overlappedSSBs: {non-overlapped, both overlapped and non-overlapped}] [9. Maximum number of overlapped SSBs in one SSB resource for L1-RSRP measurement] | [2-24, 2-29] | Yes |  | Inter-cell beam measurement and reporting [(for inter-cell BM [and mTRP])] is not supported | per band | n/a | n/a | n/a | [Note: Whether component 4 and/or 5 are also counted in FG16-1g/16-1g-1] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-2a | simultaneous reception of PDSCH/PDCCH and SSB on the same RE | 1. support simultaneous reception of PDCCH/PDSCH and SSBs for inter-cell L1-RSRP measurement on the same REs |  |  |  |  | per band | n/a | n/a | n/a |  | Optional with capability signalling |

Support separate UE feature group for multi-TRP PUSCH repetition supporting fullpowerMode2.

| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A) -codebook based | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 3. Support of two SRS resource sets with usage set to 'codebook' 4. Supported number of SRS resources in one SRS resource set | FFS | Yes |  | Multi-TRP PUSCH repetition (type A) is not supported for codebook based | per FS | n/a | n/a | n/a | Component 4 candidate values: {1,2 [,4]} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 23. NR_FeMIMO | 23-3-1-1 -codebook based | Multi-TRP PUSCH repetition (type B) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) for codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to ‘codebook’ 3. Supported number of SRS resources in one SRS resource set |  | Yes |  | Codebook based multi-TRP PUSCH repetition (type B) is not supported | Per FSPC | No | No | No | Component 3 candidate values: {1,2[,4]} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 23. NR_FeMIMO | XXX | Multi-TRP PUSCH repetition for fullpowerMode2 | 1. Support multi-TRP PUSCH repetition for fullpowerMode2 2. Supported number of SRS resources in one SRS resource set | 16-5c，23-3-1 | Yes |  |  | per FS | n/a | n/a | n/a | Component 4 candidate values: {1,2 4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Support following updates in FG 23-4.

| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info 2. The maximum number of configured additional PCIs per CC is X1 (Case 1) when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI 3. The maximum number of configured additional PCIs per CC is X2 (Case 2) when the configurations of SSB time domain positions and periodicity of the additional PCIs is different with SSB time domain positions and periodicity of the serving cell PCI | 16-2a | Yes |  | IntCell-mTRP is not supported | Per band | n/a | n/a | n/a | Component 2 candidate values: {[0,]1,2,3,[4,5,6,]7} Component 3 candidate values: {0,1,2,3,[4,5,6,]7} Note: UE indicates a non-zero value for at least one of component 2 or component 3 FFS: how to count X1 and X2 [Note: case1 and case2 cannot be enabled simultaneously] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 23-5-3, revise the note as below:

- Component 2 and component 3 are not valid for the same values of xTyR in component 1 with Rel-15/16 UE capability reporting.

References

RAN1#108-e, RAN1 Chair’s Notes.

R1-2202850, Summary of UE features for further enhancements on NR-MIMO, RAN1#108-e, Moderator (AT&T).

R2-2204360, ‘Reply LS on updated Rel-17 RAN1 UE features list for NR’, April 20th – 22nd, 2022