3GPP TSG RAN WG1 #109-e		R1-2203530

e-Meeting, May 9th – May 20th, 2022

Source:	vivo

Title:	Remaining issues on UE features for NR operation from 52.6-71GHz

Agenda Item:	8.16.2

Document for:	Discussion and Decision

- Introduction

After RAN1 #108-e meeting, the Rel-17 UE features are provided in [1]. In this contribution, our views on NR 52.6GHz part are provided.

- Discussion

Wideband PRACH:

| Index | Feature group | Components | Prerequisite feature groups | Type | Note |
| --- | --- | --- | --- | --- | --- |
| 24-1b | Wideband PRACH for 120 kHz in FR2-2 | Enhanced PRACH design for operation by adopting a single long ZC sequence, with ZC sequence equal to 1151 for 120kHz and ZC sequence equal to 571 for 120kHz | 24-1a | Per band | [A UE that supports FG 24-2 must indicate this FG is supported] [Note: This FG is only supported in bands for shared spectrum operation] |
| 24-4b | Wideband PRACH for 480 kHz in FR2-2 | PRACH with 480KHz and length 571 | 24-4a | Per band | [Note: This FG is only supported in bands for shared spectrum operation] |

After RAN1#108, it is not decided yet applicable spectrum type of wideband PRACH, i.e., unlicensed band only or not. This issue is then discussed in RAN#95-e meeting with the following proposal agreed:

Final Proposal 5 (agreed): replace the notes under FGs 24-1c, 24-4c and 24-5c for multi-RB PUCCH, and replace the bracketed notes under FGs 24-1b and 24-4b for wideband PRACH, with “This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations”

Then 24-1b and 24-4b should be updated to capture RAN agreement.

Proposal 1: For FG 24-1b and 24-4b, replace “[Note: This FG is only supported in bands for shared spectrum operation]” with “This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations”.

On 24-1b, another issue is whether “A UE that supports FG 24-2 must indicate this FG is supported”, the same handling as NRU should be adopted, i.e. 10-27 in [1]. It is clearly that wideband PRACH is not a basic FG for any scenario. Therefore, there is no need to bundle 24-2 and 24-1b together.

Proposal 2: For 24-1b, remove “A UE that supports FG 24-2 must indicate this FG is supported”.

Multi-RB PUCCH:

| Index | Feature group | Components | Prerequisite feature groups | Type | Note |
| --- | --- | --- | --- | --- | --- |
| 24-1c | Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2 | 1. Support multi-RB PUCCH format 4 for 120 kHz 2. Support multi-RB PUCCH format 0/1 for 120 kHz | 24-1a | Per band | [A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported] This FG is only supported in bands under PSD limitation in shared spectrum operation |
| 24-4c | Multi-RB PUCCH format 0/1/4 for 480 kHz in FR2-2 | Multi-RB PUCCH format 0/1/4 for 480 kHz in FR2-2 | 24-4a | Per band | This FG is only supported in bands under PSD limitation in shared spectrum operation |
| 24-5c | Multi-RB PUCCH format 0/1/4 for 960 kHz in FR2-2 | Support multi-RB PUCCH format 0/1/4 for 960 kHz | 24-5a | Per band | This FG is only supported in bands under PSD limitation in shared spectrum operation |

Similar with wideband PRACH, the note should be updated and there is no need to bundle the following FG with any other FG.

Proposal 3: For FG 24-1c, 24-4c and 24-5c, replace “This FG is only supported in bands for shared spectrum operation” with “This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations”.

Proposal 4: For 24-1c, remove “[A UE that supports FG 24-2 must indicate this FG is supported]”.

- Conclusion

In this contribution, we focus on UE feature discussion for operation from 52.6-71GHz, and have the following proposals:

Proposal 1: For FG 24-1b and 24-4b, replace “[Note: This FG is only supported in bands for shared spectrum operation]” with “This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations”.

Proposal 2: For 24-1b, remove “A UE that supports FG 24-2 must indicate this FG is supported”.

Proposal 3: For FG 24-1c, 24-4c and 24-5c, replace “This FG is only supported in bands for shared spectrum operation” with “This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations”.

Proposal 4: For 24-1c, remove “[A UE that supports FG 24-2 must indicate this FG is supported]”.

- References

- R1-2202928, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e, Moderators (AT&T, NTT DOCOMO, INC.)