3GPP TSG RAN WG1 Meeting #109-e					R1-2203535

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Discussion on UE features for UE power saving enhancements

Agenda Item:	8.16.7

Document for:	Discussion and Decision

- Introduction

This contribution addresses our proposals for remaining issues for Rel-17 power saving UE features. The discussion is based on the latest version of UE features summary in [1].

- Discussion on Rel-17 UE Power saving features

29-1 PEI

The current version of PEI from [1] is as follows,

| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support receiving paging early indication in DCI format 2_7 2. Support receiving UE subgroup indication in DCI format 2_7 [3. For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot] |  | Yes |  | UE does not support paging enhancement | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Descriptions of the monitoring occasion

For UE feature 3-1 provides Basic DL control channel, and with regards to the monitoring occasion, the following is described,

- For type 1 with dedicated RRC configuration, type 3, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot

- For type 1 PDCCH CSS without dedicated RRC configuration and for type 0, 0A, and 2 PDCCH CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1-PDCCH CSS without dedicated RRC configuration, or Types 0, 0A, or 2 PDCCH-CSS configurations within a single span of three consecutive OFDM symbols within a slot

However, in Rel-17, a new DCI format 2_7 is specified as paging early indication which is monitored in Type2A-PDCCH CSS. Descriptions of the monitoring occasion of Type2A-PDCCH CSS is not provided in the current specification.

Monitoring Type2A-PDCCH CSS can be similar to Type2-PDCCH CSS (for Paging), and hence the following proposal is made.

Proposal 1:

- Update the descriptions of 29-1 as follows,

| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support receiving paging early indication in DCI format 2_7 2. Support receiving UE subgroup indication in DCI format 2_7 3. For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot |  | Yes |  | UE does not support paging enhancement | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

29-2 TRS

The current version of PEI from [1] is as follows,

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occasions for idle/inactive UEs 1. Support reading TRS configuration from SIB [2. Support receiving L1 indication for TRS availability] |  | N |  | UE cannot receive TRS resources for idle/inactive mode | N/A | N/A | N/A | N/A | [Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7] | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Descriptions of the components

RAN1 has agreed that the L1 indication for TRS availability can be either PEI (DCI 2_7) or paging DCI (DCI 1_0). However, receiving TRS availability by PEI (DCI 2_7) requires UE to support 29-1 first. Hence, it is better to separate it as a independent feature and add 29-1 as its prerequisite .

Proposal 2:

- Update the descriptions of 29-2 as follows,

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability via DCI 1_0 |  | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode |  |  |  |  |  | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-2a | TRS resources for idle/inactive UEs | 3. Support receiving L1 indication for TRS availability via DCI 2_7 if the UE supports reception of DCI 2_7 | 29-1, 29-2 | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode |  |  |  |  |  | Optional without capability signalling |

- Conclusions

This contribution addresses our proposals for Rel-17 power saving UE features.,

Proposal 1:

- Update the descriptions of 29-1 as follows,

| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support receiving paging early indication in DCI format 2_7 2. Support receiving UE subgroup indication in DCI format 2_7 3. For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot |  | Yes |  | UE does not support paging enhancement | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2:

- Update the descriptions of 29-2 as follows,

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability via DCI 1_0 |  | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode |  |  |  |  |  | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-2a | TRS resources for idle/inactive UEs | 3. Support receiving L1 indication for TRS availability via DCI 2_7 if the UE supports reception of DCI 2_7 | 29-1, 29-2 | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode |  |  |  |  |  | Optional without capability signalling |

# Reference

- R1-2202926		Updated RAN1 UE features list for Rel-17 LTE after RAN1 #108-e including remaining RAN1 issues	Moderators (AT&T, NTT DOCOMO, INC.)