3GPP TSG-RAN WG1 Meeting #109-e		R1-2203537

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Remaining issues on UE features for NR sidelink enhancement

Agenda Item:	8.16.11

Document for:	Discussion and Decision

# Introduction

In this contribution, we provide our view on the remaining issues on UE features for Rel-17 NR sidelink enhancement, based on the updated RAN1 UE features lists in [1]. The UE features of Rel-17 NR sidelink enhancement are listed in Annex.

# Discussion

  - Receiving NR sidelink of PSFCH/S-SSB

In RAN#95, it had been agreed to split FG 32-2 into two sub-FGs [2]:

- One sub-FG for support of only S-SSB reception

- Another sub-FG for support of PSFCH reception with pre-requisite of S-SSB reception

Thus, the following capabilities should be added:

| 32. NR_SL_enh | 32-2 | Receiving NR sidelink of S-SSB | 1) UE can receive NR S-SSB. | None | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-2a | Receiving NR sidelink of PSFCH | 1) UE can receive NR PSFCH. | 32-2 | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling. |

These capabilities should be reported to gNB for scheduling. For example, gNB can decide whether to assign PUCCH for sidelink grant based on the UE capability of PSFCH reception.

Proposal 1: Introduce FG 32-2 and FG 32-2a as below according to the RAN#95 agreement.

| 32. NR_SL_enh | 32-2 | Receiving NR sidelink of S-SSB | 1) UE can receive NR S-SSB. | None | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-2a | Receiving NR sidelink of PSFCH | 1) UE can receive NR PSFCH. | 32-2 | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling. |

  - Reporting type

In the previous meeting, there were some discussions on the reporting types for the UE FGs. However, no consensus was achieved for the following FGs:

- FG 32-5a-1

- FG 32-5b-1

- FG 32-6-2

- FG 32-7

The key issue is that, although these FGs can operate in a band agnostic way, they require high baseband processing capacities. If more than one bands are supported for sidelink operation, the “Per band” reporting type for these capabilities would require the UE to implement them assuming the worst case (e.g., simultaneously in all the supported bands), which probably is a rare deployment scenario for the device, or alternatively, under reporting the UE capability (e.g., the number of supported sidelink bands), which reduces the deployment flexibility. On the other hand, “Per FS” reporting type enables the UE to implement the hardware for typical deployment scenario (e.g., supporting transmission of inter-UE coordination message only in a limited number of band), while still maintaining a larger number of bands for sidelink operation. This flexibility is especially important in the case of different bandwidth sizes for different bands. Therefore, “Per FS” reporting type is preferred.

Proposal 2: “Per FS” reporting type is supported for FG 32-5a-1, 32-5b-1, 32-6-2, and 32-7.

  - Pre-requisite

The pre-requisites have not been agreed for many of the FGs, such as 32-4, 32-4a, 32-5a-x, etc. Given the high implementation flexibility and variations of sidelink UE, and no base FGs defined for Rel-17 sidelink UEs, it would be difficult to define reasonable pre-requisite for every sidelink UE FG. Improper pre-requisite definition would undesirably reduce the implementation flexibility and may introduce backward compatibility issues in future releases. Therefore, we suggest not defining further prerequisites for sidelink FGs.

Proposal 3: Not define further prerequisites for Rel-17 sidelink FGs.

# Conclusion

In the contribution, we provide our view on the remaining issues on UE features for NR sidelink enhancement WI, with the following proposals:

Proposal 1: Introduce FG 32-2 and FG 32-2a as below according to the RAN#95 agreement.

| 32. NR_SL_enh | 32-2 | Receiving NR sidelink of S-SSB | 1) UE can receive NR S-SSB. | None | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-2a | Receiving NR sidelink of PSFCH | 1) UE can receive NR PSFCH. | 32-2 | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling. |

Proposal 2: “Per FS” reporting type is supported for FG 32-5a-1, 32-5b-1, 32-6-2, and 32-7.

Proposal 3: Not define further prerequisites for Rel-17 sidelink FGs.

# Reference

- R1-2202928, “Updated RAN1 UE features list for Rel-17 NR after RAN1 108-e”, RAN1#108-e.

- Meeting report for RAN#95-e.

# Annex.

## A1 NR feature list

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-2 | Receiving NR sidelink of PSFCH/S-SSB | 1) UE can receive NR PSFCH/S-SSB. FFS whether other components will be included | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 32. NR_SL_enh | 32-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can perform periodic-based partial sensing and resource allocation operation. 5) UE can perform contiguous partial sensing and resource allocation operation. 6) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 10) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 11) DL pathloss based open loop power control when mode 2 is configured by NR Uu | [TBD] | Yes | No | UE does not support transmission according to the partial sensing and resource allocation | Per FS | N.A. | N.A. | N.A. | Note: Random selection [according to Rel-16] in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for B are {8,16} If UE reports more than one FGs of 15-3, 32-4 and 32-4a, the reported value B in each FG is the total number of SL processes and the same among those FGs. Note: Component 6 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-6 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-6 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-6 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: Component 10 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 11 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-4a | Transmitting NR sidelink mode 2 with random resource selection | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with random resource selection configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 5) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 6) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 7) DL pathloss based open loop power control when mode 2 is configured by NR Uu | [TBD] | Yes | No | UE does not support transmission according to the random resource selection and resource allocation | Per band | N.A. | N.A. | N.A. | [Note: Random selection according to Rel-16 in the exceptional pool is supported.] Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for B are {8,16} If UE reports more than one FGs of 15-3, 32-4 and 32-4a, the reported value B in each FG is the total number of SL processes and the same among those FGs. Note: Component 4 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-4 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-4 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-4 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: Component 6 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 7 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-4b | Synchronization sources for NR sidelink transmission | 1) UE supports GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 2) UE can transmit NR sidelink based on the synchronization to an gNB 3) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb if the UE supports Components 1 and 2 4) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true if the UE supports Components 1 and 2. 5) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3 or 32-4 or 32-4a [6) UE supports SyncRef UE as the synchronization reference and can receive S-SSB in NR sidelink if it supports 15-1.] |  | Yes | No |  | Per band | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 1 is only required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Components 2/3/4 are not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-4c | eNB type synchronization sources for NR sidelink transmission | 1) UE can transmit NR sidelink based on the synchronization to an eNB. 2) If UE supports component 1 in FG 32-4b, UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) If UE supports component 1 in FG 32-4b, UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. | 32-4b | Yes | No |  | Per band | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-5a-1 | Transmitting Inter-UE coordination scheme 1 in NR sidelink mode 2 | 1) UE can transmit inter-UE coordination information of preferred resource set/non-preferred resource set in NR sidelink mode 2. 2) UE can receive an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set. | [At least one of 32-5a-2 and 32-5a-3] | Yes | Yes | UE does not support transmitting inter-UE coordination scheme 1 in NR sidelink mode 2. [UE does not receive an explicit request for inter-UE coordination information] | [Per band] | [N.A.] | [N.A.] | [N.A.] | FFS | Optional with capability signalling. |
| 32. NR_SL_enh | 32-5a-2 | Receiving Inter-UE coordination information of preferred resource set in NR sidelink mode 2 | 1) UE can receive inter-UE coordination information of preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit an explicit request for inter-UE coordination information of preferred resource set only. | [TBD] | Yes | Yes | UE does not support receiving inter-UE coordination of preferred resource set in NR sidelink mode 2. [UE does not transmit an explicit request for inter-UE coordination information of preferred resource set only] | Per band | N.A. | N.A. | N.A. | FFS | Optional with capability signalling. |
| 32. NR_SL_enh | 32-5a-3 | Receiving Inter-UE coordination information of non-preferred resource set in NR sidelink mode 2 | 1) UE can receive inter-UE coordination information of non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit an explicit request for inter-UE coordination information of non-preferred resource set only. | [TBD] | Yes | Yes | UE does not support receiving inter-UE coordination of non-preferred resource set in NR sidelink mode 2. [UE does not transmit an explicit request for inter-UE coordination information of non-preferred resource set only] | Per band | N.A. | N.A. | N.A. | FFS | Optional with capability signalling. |
| 32. NR_SL_enh | 32-5b-1 | Transmitting Inter-UE coordination scheme 2 in NR sidelink mode 2 | 1) UE can transmit inter-UE coordination information of presence of expected/potential resource conflict in NR sidelink mode 2. 2) UE can transmit up to M PSFCH(s) resources in a slot | 32-5b-2, TBD | Yes | Yes | UE does not support transmitting inter-UE coordination scheme 2 in NR sidelink mode 2. | [Per band] | [N.A.] | [N.A.] | [N.A.] | [Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1] Candidate values for M are {4, 8, 16} If UE reports more than one FGs of 15-11 and 32-5b-1, the reported value M in each FG is the total number and the same among those FGs. | Optional with capability signalling. |
| 32. NR_SL_enh | 32-5b-2 | Receiving Inter-UE coordination scheme 2 in NR sidelink mode 2 | 1) UE can receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. 2) UE can receive up to N PSFCH(s) resources in a slot. | [TBD] | Yes | Yes | UE does not support receiving inter-UE coordination scheme 2 in NR sidelink mode 2. | Per band | N.A. | N.A. | N.A. | [Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1] Candidate values for N are {5, 15, 25, 32, 35, 45, 50, 64} If UE reports more than one FGs of 15-11 and 32-5b-2, the reported value N in each FG is the total number and the same among those FGs. | Optional with capability signalling. |
| 32. NR_SL_enh | 32-6-1 | Reception of Scheme 1 inter-UE coordination information over 2nd SCI | 1) UE can receive Scheme 1 inter-UE coordination transmission over 2nd SCI that is used in addition to the MAC-CE carrying the same inter-UE coordination information in the same transmission. | FFS | Yes | Yes | UE is not required to decode SCI 2-C and the associated PSSCH | Per band | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| 32. NR_SL_enh | 32-6-2 | Reception of Scheme 1 explicit request over 2nd SCI | 1) UE can receive an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set over 2nd SCI that is used in addition to the MAC-CE carrying the explicit request in the same transmission | FFS | Yes | Yes | UE is not required to decode SCI 2-C and the associated PSSCH | [Per band] | [N.A.] | [N.A.] | [N.A.] | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| 32. NR_SL_enh | 32-7 | Determination of expected conflict in Scheme 2 based on RSRP difference | 1) UE can determine a conflict for overlapping resource reservation between UE-B and another UE based on RSRP difference of the two reservations | 32-5b-1 | No | Yes |  | [Per band] | [N.A.] | [N.A.] | [N.A.] | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |