3GPP TSG RAN WG1 #109-e                                                                                                                                                                  		                R1-2203538

E-Meeting, May 9th – May 20th, 2022

Source:	vivo

Title:	Discussion on UE features for MBS

Agenda Item:	8.16.12

Document for:	Discussion and Decision

- Introduction

In last meeting, Rel-17 NR UE features list has been discussed and updated [1]. In this contribution, our views on MBS remaining issues are further provided based on newly updated version [1].

- Discussion

  - 33-1: Broadcast

For 33-1, higher layer configured slot-level repetition has been agreed for MTCH, and up to 8 repetitions is supported. Considering it is not necessary to support such a flexible number of repetitions from 1 to 8 for MTCH, it is better to support several fixed values, i.e., {2, 4, 8} times repetitions as defined for unicast or multicast.

Regarding DCI indicated slot-level repetition for MTCH, it has been agreed as another FG in last meeting, and it could be merged with FG 33-3-1.

Furthermore, multiple G-RNTIs facilitate different broadcast services, and thus, support of  G-RNTIs should be included in 33-1.

Proposal 1 For higher layer configured slot-level repetition in FG 33-1, UE supports {2, 4, 8} times repetitions.

Proposal 2 DCI indicated slot-level repletion is merged with FG33-3-1.

Proposal 3 ‘Support of  G-RNTIs’ is included in FG 33-1.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. - Support {2, 4, 8} times semi-static slot-level repetition for MTCH - Support of N>=1 G-RNTIs for broadcast. FFS details of N |  | Up to RAN2 | Up to RAN2 | Up to RAN2 | Up to RAN2 |  |  | Up to RAN2 |

  - 33-3-1: Dynamic Slot-level repetition for group-common PDSCH

DCI indicated slot-level repetition for MTCH could be merged with FG 33-3-1.

| 33. NR_MBS | 33-3-1 | Dynamic Slot-level repetition for group-common PDSCH | - Support up to X times dynamic slot-level repetition for group-common PDSCH with CRC scrambled by G-RNTI. | 33-1 or 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  | Candidate values for X is: {8, 16} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - 33-4 to 33-4-1: NACK-only based HARQ-ACK feedback for multicast

Regarding to supporting more than one NACK-only feedback in the same PUCCH transmission. RAN 1 has agreed two modes: mode 1 is UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. Mode 2 is UE transmitting a specific sequence or a PUCCH transmission corresponding the combination more than one NACK-only HARQ feedback. Therefore, two separate UE capabilities should be added correspondingly.

Proposal 4 Add separate UE capabilities for supporting more than one NACK-only feedback in the same PUCCH transmission.

Regarding to whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-4, from our understanding, separate PUCCH resource configurations from unicast does not require separate UE capability, as the total number of PUCCH resources from all PUCCH-Config/PUCCH-ConfigurationList is not changed which was agreed in RAN1.

Proposal 5 Include separate PUCCH resource configurations from unicast in 33-4.

| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast | - [Support NACK-only based HARQ-ACK feedback.] - Support of shared/separate PUCCH resource configurations with unicast | [33-2] | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-4-1 | DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling | [33-4] | Yes |  |  | [Per UE] | FFS | FFS |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-4-2 | ACK/NACK based multiple NACK-only feedback in the same PUCCH | Support of multiplexing more than one NACK-only feedback bits by transforming NACK-only into ACK/NACK HARQ bits | 33-4 |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-4-3 | Sequence based multiple NACK-only feedback in the same PUCCH | Support of transmitting a specific sequence or a PUCCH transmission corresponding the combination more than one NACK-only HARQ feedback | 33-4 |  |  |  |  |  |  |  |  | Optional with capability signalling |

- Conclusion

In this contribution, we provide our further considerations on MBS remaining part in the following proposals and the updated FGs in tracked changes:

Proposal 1 For higher layer configured slot-level repetition in FG 33-1, UE supports {2, 4, 8} times repetitions.

Proposal 2 DCI indicated slot-level repletion is merged with FG33-3-1.

Proposal 3 ‘Support of  G-RNTIs’ is included in FG 33-1.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. - Support {2, 4, 8} times semi-static slot-level repetition for MTCH - Support of N>=1 G-RNTIs for broadcast. FFS details of N |  | Up to RAN2 | Up to RAN2 | Up to RAN2 | Up to RAN2 |  |  | Up to RAN2 |

| 33. NR_MBS | 33-3-1 | Dynamic Slot-level repetition for group-common PDSCH | - Support up to X times dynamic slot-level repetition for group-common PDSCH with CRC scrambled by G-RNTI. | 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  | Candidate values for X is: {8, 16} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4 Add separate UE capabilities for supporting more than one NACK-only feedback in the same PUCCH transmission.

Proposal 5 Include separate PUCCH resource configurations from unicast in 33-4.

| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast | - [Support NACK-only based HARQ-ACK feedback.] - Support of shared/separate PUCCH resource configurations with unicast | [33-2] | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-4-1 | DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling | [33-4] | Yes |  |  | [Per UE] | FFS | FFS |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-4-2 | ACK/NACK based multiple NACK-only feedback in the same PUCCH | Support of multiplexing more than one NACK-only feedback bits by transforming NACK-only into ACK/NACK HARQ bits | 33-4 |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-4-3 | Sequence based multiple NACK-only feedback in the same PUCCH | Support of transmitting a specific sequence or a PUCCH transmission corresponding the combination more than one NACK-only HARQ feedback | 33-4 |  |  |  |  |  |  |  |  | Optional with capability signalling |

- References

- R1-2202928, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e, Moderators (AT&T, NTT DOCOMO, INC.)