3GPP TSG-RAN WG1 Meeting #109-e                                                                 R1-2203539

e-Meeting, May 9th - May 20th, 2022

Source:	vivo

Title:	Maintenance on UE features for DSS

Agenda Item:	8.16.13

Document for:	Discussion and Decision

# Introduction

In this contribution, we provide our views on the updated UE features for DSS in [1].

# Discussion

- Candidate values of Component 4 in FG34-1

4) FFS: #The number of unicast DCI limits for PCell/PSCell scheduling • Processing one K1 unicast DCI scheduling DL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) • Processing one K2 unicast DCI scheduling UL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) • FFS: N is based on pair of (PCell/PSCell SCS, sSCell SCS): N=1 for(15,15), (30,30), (60,60) and N=2 for (15,30), (30,60) and N=4 for (15, 60) Component 4 candidate values: (K1, K2) = {(1,1) for FDD P(S)Cell; (K1, K2) = (1,2) for TDD P(S)Cell, [(K1, K2) = (2,2) for FDD P(S)Cell; (K1, K2) = (2,4) for TDD P(S)Cell]}

At the last meeting, a 2nd set of (K1, K2) value for the number of processed DCI per slot was proposed by companies. Considering that the monitoring occasions of the Pcell SS may be limited by the sScell SS when the sScell is deactivated/dormant, the new set of values aims to allowe more opportunities for Pcell self-scheduling with the support of more unicast DCIs per time slot. However, there are several issues regarding this new set.

- The deactivation/dormancy of sScell is interpreted as a sign that there is no heavy traffic load on the Pcell and that the SS on the Pcell itself should be sufficient to satisfy the needs of Pcell scheduling. If relying on restricted SS on Pcell alone would be problematic, why would gNB deactivate sScell?

- If the  2nd set of (K1, K2) value is supported and reported, e.g, if (K1, K2) = (2,2) is reported for a FDD Pcell, what’s the expected UE capability it refers to? Does it mean UE has to support all the following combiantions of #unicast DCI from different scheduling cells? If the new set is to allow more Pcell self-scheduling, there seems no reason to support case b) and case c).

  - 2 unicast UL or DL DCI for Pcell self-scheduling+0 unicast UL or DL DCI for sScell scheduling Pcell

  - 0 unicast UL or DL DCI for Pcell self-scheduling+2 unicast UL or DL DCI for sScell scheduling Pcell

  - 1 unicast UL or DL DCI for Pcell self-scheduling+1 unicast UL or DL DCI for sScell scheduling Pcell

- If the 2nd set of (K1, K2) value is supported and reported, is UE expected to decode more unicast DCIs only when sScell is deactivated/dormant? Or does UE need to decode more unicast DCIs in every Pcell slot after reporting the new set of  (K1, K2) value even if sScell scheduling Pcell is not configured yet or sScell is still activated/non-dormant? Note that in any case it requires the UE to increase the hardward capability, i.e., higher than that on non-DSS case, which seems to be out of the WI scope.

In conclusion, we think the motivation and corresponding requirements of the new 2nd set of (K1, K2) value are unclear, thus the 2nd set should be removed.

Proposal 1. For the UE feature on 34-1, the highlighted candidate value of K1/K2 for component 4 should be removed.

- SCS combination and the granularity of FG 34-1/34-2

In the last meeting, it was discussed whether the following candidate value sets were needed.

Candidate value set 1: One or more of supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz}) from following set are indicated by the UE: {15,15}, {15,30}, (15, 60), [{30,30}, {30,60},{60,60})]

[Candidate value set 2: frequency band pair(s) for {PCell/PSCell, sSCell}]

BandCombination ::=                 SEQUENCE {

bandList                            SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters,

featureSetCombination               FeatureSetCombinationId,

ca-ParametersEUTRA                  CA-ParametersEUTRA                          OPTIONAL,

ca-ParametersNR                     CA-ParametersNR                             OPTIONAL,

mrdc-Parameters                     MRDC-Parameters                             OPTIONAL,

supportedBandwidthCombinationSet    BIT STRING (SIZE (1..32))                   OPTIONAL,

powerClass-v1530                    ENUMERATED {pc2}                            OPTIONAL

}

First of all, 30/60kHz should be supported for Pcell SCS as sScell scheduling Pcell is a general feature for Pcell offloading. Sceond, this feature is reported per BC, but UE should not be mandated to support all possible combinations of {PCell/PSCell, sSCell} on the BC considering the practical scenarios for sScell scheduling Pcell/PScell would be very specific and highly demand-driven. For example, for a band combination (A, B, C) for CA, band B and band C are wide bands and band A is a narrow band, thus band B/C can be used as sScell to schedule PCell/PSCell on band A, but there is no need to force to UE to support sScell scheduling Pcell between band B and band C. Therefore, further details such as which band(s) in the reported BC can be used for PCell/PSCell and which one(s) are for sScell should be provided to avoid overcomplicated implementation. Therefore, both candidate value set and candidate value set2 should be included in the UE capability.

One way to move forward is to make the candiate value set2 as a separate FG. If it is not reported by UE together with FG 34-1/34-2 for a BC, it means UE support all possible band pairs of the BC for {PCell/PSCell, sSCell}. If value set2 is reported and set to one or more specific band pair(s), it means that UE only supports the indicated band pair(s) for {PCell/PSCell, sSCell}.

Proposal 2. For the UE feature on 34-1/34-2, the following aspects should be considered.

  - Both candidate value set and candidate value set2 should be supported as part of 34-1 and 34-2

    - Support {30,30}, {30,60}, {60,60}, and remove the highlighting and bracket on {30,30}, {30,60},{60,60} in candidate value set

- Behavior of UE indicating 34-1 and 34-2

As 34-1 and 34-2 are independent FGs, a UE may be able to support both 34-1 and 34-2. But it is not clear if such UE is allowed to indicate both FGs to gNB. If yes, the UE behavior may be unclear especially when sSCell USS set(s) (for CCS from sSCell to Pcell/PSCell) and Type0/0A/1/2 CSS sets on Pcell/PSCell are configured with overlapping slots. In this case, there are two interpretations of the UE behavior in the overlapping slots as analyzed in [2]:

- UE monitors DCIs from sScell and Pcell/PSCell simultaneously for any RNTI following 34-2;

- UE does not monitor DCI formats with CRC scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI from Pcell/PSCell and sScell simultaneously. In other words, once UE has decoded a DCI format with CRC scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI from sScell in a overlapping slot, it may stop blind decoding for DCI formats with CRC scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI for Pcell/PSCell self-scheduling in the same slot, as described in 34-1.

If such reporting is not allowed, it should be specified in the FGs 34-1 and 34-2 that only one of them can be indicated by UE.

Proposal 3. Clarify if UE can report both 34-1 and 34-2 to gNB, and the expected UE behavior if this reporting is possible.

- Disabling of scaling factor α when sScell is deactivated/dormant

Agreement - For a UE configured for CCS from sSCell to P(S)Cell, scaling factor α is not applied for PDCCH overbooking/BD/CCE limit computation when sSCell is deactivated, or when an activated sSCell is switched to dormant BWP; otherwise scaling factor α is applied - Timing for disabling scaling factor α when sSCell is deactivated follows sSCell deactivation timing in current specifications, i.e., no later than the minimum requirement defined in TS 38.133 as captured in 38.213 subclause 4.3 - Timing for disabling scaling factor α follows the non-dormant to dormant BWP switching delay in current specifications ( TS 38.133). - Introduce separate FG to indicate UE support for disabling scaling factor α when sSCell is deactivated - Introduce separate FG to indicate UE support for disabling scaling factor α when activated sSCell is switched to dormant BWP - Note: It is up to UE implementation whether/when to apply the scaling factor α during sSCell activation and during sSCell BWP switch

It has been agreed when sScell is dormant or deactivated, the scaling factor α is no longer applied to Pcell self-scheduling, and new features should be introduced for sScell deactivation/dormant cases. Examples of the component description of the new FGs are as below:

| Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- |
| 34-3 | Disabling scaling factor α when sSCell is deactivated | Support of disabling scaling factor α for Cross-carrier scheduling (CCS) from sSCell to PCell/PSCell (Type A or Type B) when sSCell is deactivated - scaling factor α is not applied for PDCCH overbooking/BD/CCE limit computation when sSCell is deactivated | 34-1 or 34-2 |
| 34-4 | Disabling scaling factor α when sSCell is dormant | Support of disabling scaling factor α for Cross-carrier scheduling (CCS) from sSCell to PCell/PSCell (Type A or Type B) when sSCell is switched to dormant BWP scaling factor α is not applied for PDCCH overbooking/BD/CCE limit computation when sSCell is switched to dormant BWP | 34-1 or 34-2 |

Proposal 4. Introduce new UE features 34-3 and 34-4 for disabling scaling factor α when sScell is deactivated/dormant.

- Additional bandwidths for aperiodic CSI-RS for tracking for fast SCell activation

Agreement - Introduce new FG35-2 additional bandwidth for fast SCell activation

In the last meeting, it was agreed to introduce FG35-2 for additional bandwidths for aperiodic CSI-RS for tracking for fast SCell activation. Following is an example of the new FG based on trs-AdditionalBandwidth-r16.

Proposal 5. Introduce new UE feature 35-2 for additional bandwidths for aperiodic CSI-RS for tracking for fast SCell activation.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35. LTE_NR_DC_enh2 | 35-2 | Additional bandwidths for aperiodic CSI-RS for tracking for fast SCell activation | Indicates the UE supported bandwidths for aperiodic CSI-RS for tracking for fast SCell activation, in addition to 52 RBs, for a 10MHz UE channel bandwidth. This field only applies to the BWPs configured with 52 RBs size and 15kHz SCS, in FDD bands. Value of set 1 indicates 28, 32, 36, 40, 44, or 48 RBs. Value of set 2 indicates 32, 36, 40, 44, 48 or RBs. | 35-1 | Yes | N/A | Additional bandwidths for aperiodic CSI-RS for tracking for fast SCell activation are not supported | Per band | N/A | N/A | N/A |

# Conclusion

In the contribution, we provide our views on the UE feature for DSS with the following proposals.

Proposal 1. For the UE feature on 34-1, the highlighted candidate value of K1/K2 for component 4 should be removed.

Proposal 2. For the UE feature on 34-1/34-2, the following aspects should be considered.

  - Both candidate value set and candidate value set2 should be supported as part of 34-1 and 34-2

    - Support {30,30}, {30,60}, {60,60}, and remove the highlighting and bracket on {30,30}, {30,60},{60,60} in candidate value set

Proposal 3. Clarify if UE can report both 34-1 and 34-2 to gNB, and the expected UE behavior if this reporting is possible.

Proposal 4. Introduce new UE features 34-3 and 34-4 for disabling scaling factor α when sScell is deactivated/dormant.

Proposal 5. Introduce new UE feature 35-2 for additional bandwidths for aperiodic CSI-RS for tracking for fast SCell activation.

# Reference

- R1-2202928, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e

- R1-2203528, Maintenance on Scell scheduling P(S)cell, vivo