3GPP TSG RAN WG1 #109-e		R1-2203540

e-Meeting, May 9th - May 20th, 2022

Source:	vivo

Title:	Remaining issues for NR small data transmissions in RRC INACTIVE state and Tx switching enhancements

Agenda Item:	8.17

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss the remaining RAN1 related issues for small data transmission in RRC inactive state and TX switching enhancements.

# Discussion

  - Remaining issues for SDT in RRC inactive state

In this section, we discuss following aspects related for SDT in RRC inactive state:

- SSB to CG PUSCH resource mapping for CG-SDT,

- CG PUSCH resource allocation,

- CG PUSCH resource validation.

    - Mapping between SSBs and CG PUSCH resources

In RAN1 #106b-e meeting, RAN1 aspects on mapping between CG PUSCH resources and SSBs were discussed. Following agreements on RAN1 aspects for CG-SDT were made in RAN1 [1].

Agreement - Mapping ratio of SSB to CG PUSCH is configured per CG configuration. - FFS whether to restrict the same value for all CG configuration and/or allow different value for different CG configurations. - For the candidate value set of SSB to CG PUSCH mapping ratio, support at least {1, 2, 4, 8, 16} - FFS {1/8,1/4,1/2}

As can be seen, it was agreed that the mapping ratio of SSB to CG PUSCH is configured per CG configuration. Then in RAN1 #107-e meeting, it has been further concluded that different mapping ratios can be configured for different CG configurations.

Conclusion No need to restrict the same value of mapping ratio for all CG configurations.

Later in RAN1#108-e meeting, following agreements have been made to support mapping ratio values less than 1 and to define the candidate value set of association period.

Agreement For CG-SDT, support mapping ratio {1/8, 1/4, 1/2} for SSB to CG PUSCH mapping. Agreement - For CG-SDT, the starting time of association period is SFN0. - Regarding the candidate value set of association period, adopt the Table 2.4-1 - FFS CG period smaller than 5ms - Note: It does not mean to RAN2 the maximum CG period is 640ms. The table will be updated if RAN2 introduces other CG period values. - Note: The potential impact on PDCCH monitoring periodicity should be considered if larger CG period value is introduced in RAN2

As is indicated in RAN2 LS [2], candidate values of CG period for SDT have been agreed to be the same as the candidate values of CG period for Type 1 CG PUSCH transmissions in NR Rel-16:

With regards to the RAN1 question whether there is any restriction on the candidate values of CG period, RAN2 agreed that there is no restriction from RAN2 perspective.

According to above, one remaining issue is how to determine the candidate value set of association period for SSB to CG PUSCH mapping in case the CG period is smaller than 5ms. Before discussing the special handling for this case in RAN1, it would be good to send a LS to RAN2 to see whether it possible to not support such small CG period values which would be seldom used for small data transmissions in our understanding, although RAN2 may have to revert their earlier agreements which was made without considering the definition of association period for mapping between SSB and CG PUSCH being discussed in RAN1.

Proposal 1: Send a LS to RAN2 to see whether it possible to not support CG period values smaller than 5ms.

    - CG PUSCH resource allocation

In section 2.5 of [3] , another remaining issue is about the TDRA of CG PUSCH, for which FL made a proposal below.

Proposal #2.5-1 UE specific TDRA table can be configured for CG-SDT.

In RAN1 #108-e meeting, the discussions focus on whether the pusch-Config IE should be included in RRC or not. No consensus has been reached in RAN1 except following agreements were made which means that this should be further discussed in RAN2.

Agreement - UE specific parameter pucch-Config-r17 is not needed for SDT. - RAN1 cannot reach consensus on the support of UE specific CORESET, UE specific parameters pusch-Config-r17 and pdsch-Config-r17.

In our understanding, it could work to just use the SIB1 configured TDRA table or default table when the SIB1 configured TDRA table is not present for scheduling CG PUSCH transmission. However, it would also be good to configure a UE specific TDRA table which can be more flexible so that the collision between CG PUSCH and other UL signals can be avoided given the limited TDRA options provided in SIB1 configured TDRA table intended for all UEs in a serving cell.

Furthermore, such UE specific TDRA table can be used by subsequent SDT (in either RA-SDT or CG-SDT) or retransmission of SDT scheduled by DCI.

With that, we have following proposal which should be discussed in RAN1 though whether such UE specific TDRA list should be provided in a pusch-Config IE in RRC release message should be up to RAN2 to discuss.

Proposal 2: UE specific TDRA table is configured in RRC release message for scheduling a PUSCH for CG-SDT or subsequent SDT or retransmission of SDT.

    - CG PUSCH resource validation

RAN1 has made the following agreements on validation of CG PUSCH resource in SDT topic and validation of MsgA PUSCH for RedCap UEs in RedCap topic in RAN1 #106b-e meeting [1].

Agreement (SDT) - The following PUSCH occasion validation rule is applied for CG-SDT - for unpaired spectrum and for SS/PBCH blocks with indexes provided by ssb-PositionsInBurst in SIB1 or by ServingCellConfigCommon - if a UE is provided tdd-UL-DL-ConfigurationCommon, the valid PO is the PO in UL part in a slot, or at least Ngap symbols after the end of the DL part in a slot or after the end of the SSB in a slot - if a UE is not provided tdd-UL-DL-ConfigurationCommon, the valid PO does not precede a SS/PBCH block in the PUSCH slot, starts at least Ngap symbols after a last SS/PBCH block symbol - Ngap is provided in Table 8.1-2 in TS 38.213 - FFS if any validation rule following the CG-PUSCH in RRC connected state is applicable, and whether and how to handle the overlapping between CG-PUSCH occasions for CG-SDT and any valid PRACH occasion or MsgA PUSCH occasion. - FFS the rule for paired spectrum, and whether/how to support CG-SDT for UEs operating in Type-A HD-FDD. Agreement (SDT) A CG PUSCH occasion is not valid if it overlaps with any valid PRACH occasion. - FFS overlapping between CG PUSCH occasions and MsgA PUSCH occasion Conclusion (SDT) No need to define UL/DL pattern type of validation rule specific for paired spectrum at least for non-RedCap UEs. - FFS the case for RedCap UEs Agreement(RedCap) - The same validation rules of MsgA PUSCH occasions and RO/Preamble-to-PRU mapping rules for FDD can be reused for HD-FDD Agreement(RedCap) - For MsgA PUSCH occasion overlapping with dynamic or semi-static DL reception, leave it to UE implementation to prioritize the DL reception or MsgA PUSCH transmission

In RAN1 #108-e meeting, following agreements have been made for CG-SDT in HD-FDD mode.

Agreement The validation rule defined for CG-SDT in FD-FDD mode can be reused for RedCap UE performing CG-SDT in HD-FDD mode.

For CG PUSCH overlapping with MsgA PUSCH, this has also been discussed in RAN1 #107-e and RAN1 #108-e meeting though there’s no conclusion as different companies have different views on this. According to the discussions summarized in [3], for a UE that supports both CG-SDT and 2-step RACH, it could be fine to say CG PUSCH is not valid when overlapping happens. Furthermore, when a UE supporting both SDT and 2-step RACH performs MsgA transmission in CBRA, the MsgA PUSCH occasions overlapping with CG PUSCH must be treated as valid and mapped to some MsgA PRACH preambles since gNB doesn’t know whether such UE (with its MsgA received by gNB) may have a CG PUSCH transmission overlapping with MsgA PUSCH. Therefore, it would be good to invalidate the CG PUSCH instead of having such collision handling up to UE implementation so that MsgA PUSCH reception is prioritized when the UE supports both 2-step RACH and SDT.

Proposal 3: For a UE that supports both CG-SDT and 2-step RACH, CG PUSCH  occasions for SDT are treated as invalid when overlapping with MsgA PUSCH occasion, i.e. MsgA PUSCH occasion should be prioritized.

For a UE that doesn’t support 2-step RACH, it may not read MsgA PUSCH configurations though it is possible for the UE to read them from SIB1. In such case, the CG PUSCH occasion configured for this UE for CG-SDT can be treated as valid even if it overlaps with a MsgA PUSCH occasion for other UEs supporting 2-step RACH, and the overlapping MsgA PUSCH occasions should also be treated as valid since other UEs have no idea of the CG PUSCH configurations for SDT of the UE. Thus, we have following proposal for this case.

Proposal 4: For a UE that supports CG-SDT but doesn’t support 2-step RACH, CG PUSCH  occasions for SDT can be treated as valid when overlapping with MsgA PUSCH occasions.

  - Remaining issues for TX switching enhancement

In this section we discuss remaining RAN1 related issues for TX switching enhancement, related to Rel-17 Tx switching for SUL. In Rel-17, the 2Tx-2Tx switching and 1Tx-2Tx switching for 3 carriers including 1 SUL are agreed while it is not correctly captured in TS 38214 [4]. Therefore, the following TP #1 should be adopted.

--------------------------------------------------TP #1 for section 6.1.6.3 of TS 38214 ----------------------------------------

6.1.6.3 	Uplink switching for supplementary uplink

For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch or uplinkTxSwitchingPeriod2T2T for a band combination, and if it is for that band combination configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink:

-	If the UE is configured with uplink switching with parameter uplinkTxSwitching,

-	If the UE is to transmit any uplink channel or signal on a different uplink on a different band from the preceding transmission occasion based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s), then the UE assumes that an uplink switching is triggered in a duration of switching gap NTx1-Tx2, where T0 is the start time of the first symbol of the transmission occasion of the uplink channel or signal and Toffset is the preparation procedure time of the transmission occasion of the uplink channel or signal given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213], respectively. During the switching gap NTx1-Tx2, the UE is not expected to transmit on any of the uplinks.

-	In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.

----------------------------------------------------end of TP#1---------------------------------------------------------------------

Proposal 5: Adopt TP #1 for section 6.1.6.3 of TS 38214 to capture the Rel-17 2Tx-2Tx switching and 1Tx-2Tx switching for 3 carriers which include 1 SUL.

Reason for change: Current Tx switching cases specified in Clause 6.3.6.3 does not correctly include the 2Tx-2Tx switching mode and the Tx switching cases for 3 carriers which are reflected in the following agreements in RAN1 #104b-e[5]. Agreements: For Rel-17 2Tx-2Tx switching between two uplink carriers, the mapping between UL transmission ports and Tx chain for SUL and UL CA option 1 is defined as follows. Number of Tx chains in WID (carrier 1 + carrier 2); Number of antenna ports for UL transmission (carrier 1 + carrier 2). Case 2; 0T+2T; 0P+2P, 0P+1P. Case 3; 2T+0T; 2P+0P, 1P+0P. Agreements: For Rel-17 1Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, the mapping between UL transmission ports and Tx chain for SUL and UL CA Option 1 is defined as follows. Number of Tx chains in WID (band A + band B); Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2 + carrier 3)). Case 1; 1T+1T; 1P+(0P+0P). Case 2; 0T+2T; 0P+(2P+0P), 0P+(0P+2P), 0P+(2P+2P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P), 0P+(1P+2P), 0P+(2P+1P). Agreements: For Rel-17 2Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, the mapping between UL transmission ports and Tx chain for SUL and UL CA Option 1 is defined as follows. Number of Tx chains in WID (band A + band B); Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2 + carrier 3)). Case 2; 0T+2T; 0P+(2P+0P), 0P+(0P+2P), 0P+(2P+2P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P), 0P+(1P+2P), 0P+(2P+1P). Case 3; 2T+0T; 2P+(0P+0P), 1P+(0P+0P). Summary of change: Delete the wording “two” to make it clear that the 3 carriers Tx switching cases for SUL are specified. And the parameter “uplinkTxSwitchingPeriod2T2T” to make it clear the 2Tx-2Tx switching mode is supported for SUL. Consequences if not approved: There is mismatch between the specification and agreement. --------------------------------------------------TP #1 for section 6.1.6.3 of TS 38214 ------------------------------------ 6.1.6.3 Uplink switching for supplementary uplink For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch or uplinkTxSwitchingPeriod2T2T for a band combination, and if it is for that band combination configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, - If the UE is to transmit any uplink channel or signal on a different uplink on a different band from the preceding transmission occasion based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s), then the UE assumes that an uplink switching is triggered in a duration of switching gap NTx1-Tx2, where T0 is the start time of the first symbol of the transmission occasion of the uplink channel or signal and Toffset is the preparation procedure time of the transmission occasion of the uplink channel or signal given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213], respectively. During the switching gap NTx1-Tx2, the UE is not expected to transmit on any of the uplinks. - In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions. ----------------------------------------------------end of TP#1------------------------------------------------------------------

# Conclusion

In this contribution, we discuss the remaining issues on the small data transmission and TX switching enhancement from RAN1’s perspective.

We have following proposals according to the discussions.

Proposal 1: Send a LS to RAN2 to see whether it possible to not support CG period values smaller than 5ms.

Proposal 2: UE specific TDRA table is configured in RRC release message for scheduling a PUSCH for CG-SDT or subsequent SDT or retransmission of SDT.

Proposal 3: For a UE that supports both CG-SDT and 2-step RACH, CG PUSCH  occasions for SDT are treated as invalid when overlapping with MsgA PUSCH occasion, i.e. MsgA PUSCH occasion should be prioritized.

Proposal 4: For a UE that supports CG-SDT but doesn’t support 2-step RACH, CG PUSCH  occasions for SDT can be treated as valid when overlapping with MsgA PUSCH occasions.

Proposal 5: Adopt TP #1 for section 6.1.6.3 of TS 38214 to capture the Rel-17 2Tx-2Tx switching and 1Tx-2Tx switching for 3 carriers which include 1 SUL.

Reason for change: Current Tx switching cases specified in Clause 6.3.6.3 does not correctly include the 2Tx-2Tx switching mode and the Tx switching cases for 3 carriers which are reflected in the following agreements in RAN1 #104bis-e. Agreements: For Rel-17 2Tx-2Tx switching between two uplink carriers, the mapping between UL transmission ports and Tx chain for SUL and UL CA option 1 is defined as follows. Number of Tx chains in WID (carrier 1 + carrier 2); Number of antenna ports for UL transmission (carrier 1 + carrier 2). Case 2; 0T+2T; 0P+2P, 0P+1P. Case 3; 2T+0T; 2P+0P, 1P+0P. Agreements: For Rel-17 1Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, the mapping between UL transmission ports and Tx chain for SUL and UL CA Option 1 is defined as follows. Number of Tx chains in WID (band A + band B); Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2 + carrier 3)). Case 1; 1T+1T; 1P+(0P+0P). Case 2; 0T+2T; 0P+(2P+0P), 0P+(0P+2P), 0P+(2P+2P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P), 0P+(1P+2P), 0P+(2P+1P). Agreements: For Rel-17 2Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, the mapping between UL transmission ports and Tx chain for SUL and UL CA Option 1 is defined as follows. Number of Tx chains in WID (band A + band B); Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2 + carrier 3)). Case 2; 0T+2T; 0P+(2P+0P), 0P+(0P+2P), 0P+(2P+2P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P), 0P+(1P+2P), 0P+(2P+1P). Case 3; 2T+0T; 2P+(0P+0P), 1P+(0P+0P). Summary of change: Delete the wording “two” to make it clear that the 3 carriers Tx switching cases for SUL are specified. And the parameter “uplinkTxSwitchingPeriod2T2T” to make it clear the 2Tx-2Tx switching mode is supported for SUL. Consequences if not approved: There is mismatch between the specification and agreement. --------------------------------------------------TP #1 for section 6.1.6.3 of TS 38214 ------------------------------------ 6.1.6.3 Uplink switching for supplementary uplink For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch or uplinkTxSwitchingPeriod2T2T for a band combination, and if it is for that band combination configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, - If the UE is to transmit any uplink channel or signal on a different uplink on a different band from the preceding transmission occasion based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s), then the UE assumes that an uplink switching is triggered in a duration of switching gap NTx1-Tx2, where T0 is the start time of the first symbol of the transmission occasion of the uplink channel or signal and Toffset is the preparation procedure time of the transmission occasion of the uplink channel or signal given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213], respectively. During the switching gap NTx1-Tx2, the UE is not expected to transmit on any of the uplinks. - In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions. ----------------------------------------------------end of TP#1------------------------------------------------------------------

# References

- Chairman’s Notes, 3GPP RAN1 #106b-e.

- R2-2201828, Reply LS on the L1 aspects of small data transmission, 3GPP RAN2 #116bis-e, January 17th -26th, 2022.

- R1-2112779, Summary on the physical layer aspects of small data transmission, 3GPP RAN1 #107-e, November 11th - 19th, 2021.

- 3GPP TS 38.214 V17.1.0 (2022-04), Physical layer procedures for data.

- 3GPP RAN1#104b-e meeting, Chairman’s notes.