3GPP TSG RAN WG1 #109-e	R1-2203542

e-Meeting, May 9th – 20th, 2022

Source: 	vivo

Title:	Views on two TAs for multi-DCI-based multi-TRP operation

Agenda Item:	9.1.1.2

Document for:	Discussion and Decision

Introduction

The following objective on introducing two TAs for UL multi-DCI for multi-TRP operation is approved in [1].

Objective 7: Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

Multi-DCI-based intra-cell UL transmission and inter-cell multi-TRP DL transmission are introduced in Rel-16 and Rel-17 respectively to achieve higher throughput for eMBB. However, such improvement is not sufficient due to the cell-based TA shared by UL transmission towards all TRPs. One shared TA may not be sufficient to cover propagation delay differences, delay spreads, and multi-TRP sync inaccuracy. Figure 1 shows an example of UL timing error due to different propagation delays assuming that the propagation difference between two TRPs is p. Timing offset p or 2p depending on DL timing of which TRP is referred to adjust UL timing will be unfortunately introduced for UL timing of TRP2 if the shared TAC is calculated based on TRP1. Such UL timing inaccuracy may be larger than CP leading to ineffective scheduling from TRP2. To achieve the UL timing accuracy of each TRP, multiple TAs are necessary.

An example of timing advance adjustment procedure in mTRP operation

In this contribution, we focus on the following aspects to facilitate separate TAs for two TRPs:

TRP-specific RACH procedure to achieve initial time adjustment for two TRPs.

TRP-specific relative time adjustment to achieve accurate UL timing for two TRPs.

Other issues to be discussed for the introduction of 2 TAs.

TRP-specific RACH procedure to achieve initial time alignment

Initial time alignment is realized by the absolute TAC carried in RAR or absolute TAC MAC CE during the random access procedure. Random access procedure can be gNB-initiated by sending PDCCH order to UE or UE-initiated by its RRC and MAC entity, for the events including DL or UL data arrival during RRC_CONNECTED when UL synchronization status is non-synchronized and to establish time alignment for a secondary TAG. Whether to enable mTRP scheduling depends on gNB and also whether TRP-specific TA is needed depends on the measurements at gNB. Hence, the gNB-initiated RACH procedure should be supported to acquire UL timing alignment to each TRP if needed. As for the UE-initiated RACH procedure, UE can access any of the TRPs, then the other TRP can send a PDCCH order to achieve timing alignment for the TRP if necessary. Considering the above scenarios, we propose to support TRP-specific PRACH triggered by the PDCCH order for both intra-cell and inter-cell mTRP.

- To achieve initial time alignment for each TRP, support TRP-specific RACH triggered by the PDCCH order for both intra-cell and inter-cell mTRP.

Examples of TRP-specific time alignment by two RACH procedures

Regarding TRP-specific RACH procedure triggered by TRP-specific PDCCH order, the first thing to consider is to clarify the association between the PDCCH order and its triggered RACH procedure. One direct solution is through the CoresetPoolIndex. More specifically, as shown in Figure 2, when UE detects a PDCCH order from the coreset configured with CoresetPoolIndex 0, UE assumes that the RACH procedure associates with the first TRP and time with the first TRP is aligned in this procedure while if UE detects a PDCCH order from the coreset configured with CoresetPoolIndex 1, UE assumes that the RACH procedure associates with the second TRP and time with the second TRP is aligned in this procedure.

In inter-cell mTRP operation, the RACH procedure for the non-serving cell should be introduced to acquire UL time alignment towards non-serving cell TRP. To achieve this, non-serving cell PCI should be informed to UE when UE detects the PDCCH order triggering RACH for the non-serving cell. In Rel-17 inter-cell mTRP, one PCI is associated with one CoresetPoolIndex. As a result, the association between PCI and the PDCCH order can be achieved also by the CoresetPoolIndex. Besides, RACH resource for each TRP should be configured to achieve TRP-specific RACH procedure triggered by PDCCH order in inter-cell mTRP operation.

The association between the RACH procedure and TRP can be implicitly obtained through CoresetpoolIndex in both intra-cell and inter-cell mTRP operations.

      - Per TRP RACH resource configuration also needs to be enhanced for inter-cell MTRP cases.

TRP-specific relative time adjustment

Besides initial time adjustment in RACH procedure, UL timing is maintained also by relative time adjustment by the relative TAC received from gNB. In this clause, we discuss how UE acquires TRP-specific TAC for TRP-specific relative time adjustment. The following two alternatives can be considered.

Alt1: UE receives two relative TACs associated with two TRPs

Alt2: UE receives one relative TAC associated with one TRP, and TAC associated with the other TRP is determined by the received TAC together with DL receiving timing difference

- Regarding Alt1, TRP identity (CoresetPoolIndex) needs to be added for the timing advance command MAC CE in Figure 3, or the TAG ID may be extended to indicate TAG associates with a specific TRP because TAG ID is identical for each TRP.

Timing Advance Command MAC CE

In Alt2, as shown in Figure 4 one TAC received by UE is determined based on the reference TRP for example TRP 1, then UE calculates the TA for the other TRP by utilizing the timing offset between DL receiving timing from two TRPs.

Two TAC acquisitions based on DL receiving timing difference

- Considering two TRPs may not be synchronized, the additional error will be introduced for TRP2, because the timing offset may be unfortunately compensated along with propagation delay. The offset can be calculated by utilizing two TAC received from two RACH procedures and the DL timing difference from two TRPs measured by UE as shown in Figure 5.

An example of time alignment in two RACH procedures

Timing offset p between non-synchronized TRPs can be obtained through two RACH procedures.

- Assuming timing offset p between non-synchronized TRPs is obtained through two RACH procedures, Table 1 provides an analysis of Rx timing offset for each TRP where TAC is calculated by DL timing difference whereas different propagation delays and timing offsets between TRPs are considered. It can be seen that UL timing for the second TRP is aligned well for all cases.

Calculation on Rx timing offset for each TRP where TAC is calculated by DL timing difference

|  | TRP1 | TRP2-a | TRP2-b | TRP2-c |
| --- | --- | --- | --- | --- |
| DL Tx timing | T | T | T+p | T+p |
| Propagation delay | t | t+d | t | t+d |
| UE Rx timing | T+t | T+t+d | T+t+p | T+t+d+p |
| RX timing difference Δ | Δ=0 | Δ=d | Δ=p | Δ=d+p |
| TA offset (s=2t) | s | s+2Δ | s+2Δ-2p | s+2Δ-2p |
| DL timing reference (based on each TRP) | T+t | T+t+d | T+t+p | T+t+d+p |
| UE Tx timing | T+t-s | T+t+d-s-2Δ | T+t+p-s-2Δ+2p | T+t+d+p-s-2Δ+2p |
| TRP Rx timing | T+2t-s | T+2t+2d-s-2Δ | T+2t-s-2Δ+3p | T+2t+2d-s-2Δ+3p |
| TRP Rx timing offset | 2t-s | 2t+2d-s-2Δ | 2t-s-2Δ+2p | 2t+2d-s -2Δ+2p |
| TRP Rx timing offset | 0 | 0 | 0 | 0 |

Alt 2 has minimum spec impact because we only need to clarify the reference TRP in advance and two TACs can be acquired by UE implementation. Besides, considering the asymmetric demand on DL /UL data transmission, DL transmission can be scheduled by two TRPs, while UL data may only be scheduled by one TRP, then UE can acquire UL timing for PUCCH HARQ feedback of the corresponding PDSCH towards the TRP without any SRS or PUSCH transmission based on the TAC from the TRP with SRS transmission and the difference of UE receive timing. Therefore, we propose that the UL timing can be achieved using the TAC from the TRP with SRS/PUSCH transmission and the DL timing difference between the two TRPs.

- For the case without SRS/PUSCH but with PUCCH transmission to the second TRP, support a TA control mode that only one TAC is explicitly indicated and associated with the first TRP, with the other TA determined by the received TAC together with DL receiving timing difference between the TRPs.

Other issues to be discussed

Relation between channels/signals and the applied TAs

- Two TAs are maintained for two TRPs, hence UL channels and signals towards two TRPs should apply two TAs respectively. UL channels and signals should be categorized into two groups by the association with CoresetPoolIndex or the counterpart TRP. Then channels and signals belonging to one group share the associated TA.

- The relation between channels/signals and the applied TAs should be acquired by CoresetPoolindex.

UE capability to maintain two TA

UE capabilities need to be defined for the switch time between two TA loops to guarantee UL transmission towards both TRPs. Due to the RF switch or baseband processing delay when switch TA loop, transmission  from the TRP may be interrupted if UL transmission is scheduled exactly during the switch time. So, UE capability of TA switch time between two TA loops can be reported to avoid such transmission interruption.

- Study transmission interruptions caused by timing switch between two TA loops, including whether additional UE capabilities need to be defined for the switch time between two TA loops.

As shown in Figure 7, two TA loops maintained for two TACs may cause UL transmission overlapping due to two separate Time adjustments. How to handle the overlapped channels should be further discussed. If only a small number of samples are overlapped, UE is capable to deal with such a situation. While if several OFDM symbols overlap, UE may choose one channel/signal to transmit if the simultaneous transmission is not feasible. The determination on the chosen channel/signal to transmit can be based on priority rule as designed in current specification.

Two TA process loops for two TACs

- Study how to handle the overlapped channels and signals due to two TAs applied for different channels/RS towards different TRPs.

TAGs configuration

- Current TA is TAG-based, which means the UL timing of serving cells configured in one TAG is the same. Now for a serving cell, two TAs are maintained, so TAG configuration for serving cells in mTRP operation should be further refined. The following two alternatives can be considered.

Alt1: One serving cell belongs to one TAG which maintains two TAs for two TRPs

Alt2: One serving cell belongs to two TAG associated with two TRPs

Figure 3 a）shows an example of Alt1. Two TAs associated with two TRPs are maintained for one TAG, which requires that the TA for a certain TRP of all serving cells configured within one TAG should be the same. Regarding the case of only the first TRP of each serving cell within the TAG is co-located, the TA of the second TRP cannot be shared for serving cells in the TAG. Besides, to maintain two-timing for one TAG two TimeAlignmentTimer should be configured.

Figure 3 b）shows an example of Alt2. A serving cell can be configured within 2 TAGs, and each TAG maintains a TA for a TRP. Such configuration is flexible for supporting flexible TRP deployment for each serving cell. In addition, Alt2 has minimum spec impact. However, the maximum TAG number may need to be increased for both TRP-specific TAG and carrier-based TAG.

Examples of TAG configuration

- For flexible TRP deployment, 2 TAGs associated with two TRP are configured for a serving cell.

Impact on DL timing

As described in RAN4 spec [2], the DL timing reference for UE to adjust UL timing advance based on the effective TAC is the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. For a cell configured with mTRP, TRP-specific DL timing reference may be required because of the propagation delay differences, delay spreads, and multi-TRP sync inaccuracy.

- Send LS to RAN4 on whether TRP-specific DL timing should be introduced as the reference timing for TRP-specific time adjustment.

Conclusions

In this contribution, we have the following observation and proposals:

- Timing offset p between non-synchronized TRPs can be obtained through two RACH   procedures.

- To achieve initial time alignment for each TRP, support TRP-specific RACH triggered by the PDCCH order for both intra-cell and inter-cell mTRP.

- The association between the RACH procedure and TRP can be implicitly obtained through CoresetpoolIndex in both intra-cell and inter-cell mTRP operations.

      - Per TRP RACH resource configuration also needs to be enhanced for inter-cell MTRP cases.

- For the case without SRS/PUSCH but with PUCCH transmission to the second TRP, support a TA control mode that only one TAC is explicitly indicated and associated with the first TRP, with the other TA determined by the received TAC together with DL receiving timing difference between the TRPs.

- The relation between channels/signals and the applied TAs should be acquired by CoresetPoolindex.

- Study transmission interruptions caused by timing switch between two TA loops, including whether additional UE capabilities need to be defined for the switch time between two TA loops.

- Study how to handle the overlapped channels and signals due to two TAs applied for different channels/RS towards different TRPs.

- For flexible TRP deployment, 2 TAGs associated with two TRP are configured for a serving cell.

- Send LS to RAN4 on whether TRP-specific DL timing should be introduced as the reference timing for TRP-specific time adjustment.

References

RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung.

3GPP TS 38.133,V 17.3.0,2021-09.