3GPP TSG RAN WG1 #109-e	R1-2203543

e-Meeting, May 9th – 20th, 2022

Source: 	vivo

Title:	Views on CSI enhancement for high/medium UE velocities and coherent JT

Agenda Item:	9.1.2

Document for:	Discussion and Decision

Introduction

Two codebook types were specified in Rel-15: Type I codebook employs two-stage codebook structure including wideband spatial information, subband beam selection and co-phasing; Type II codebook supporting up to rank 2 targets finer granularity CSI feedback including spatial bases and corresponding coefficients based on the principle of linear combination, for MU-MIMO, and consequently incurs larger feedback overhead. Type II port selection codebook was also introduced based on Type II codebook.

Rel-16 further specified eType II codebook to reduce the feedback overhead by cultivating frequency-domain correlation, i.e., sparsity in delay domain. Further, another important feature introduced in Rel-16 is multi-TRP (MTRP) operation to enhance both throughput for eMBB and reliability and robustness for URLLC. Multi-DCI-based MTRP allows two TRPs independently schedule PDSCHs, and the UEs supporting this feature may expect the reception of fully/partially-overlapped PDSCHs in time and frequency domain scheduled by two TRPs. While, single-DCI-based MTRP realizes a PDSCH transmitted from two TRPs in SDM, TDM or SDM manners by indicating two TCI states.

Following two objectives related to CSI enhancement are listed in MIMO Evolution for Downlink and Uplink WID [1].

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking - Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

Views on CSI enhancement for high/medium UE velocities

UE velocity impact on performance

Current codebook types only cultivate spatial domain and frequency-delay domain characteristics and feed back the most important components in both domains without considering any Doppler-time domain information. However, performance degrades considerably when the UE is moving in medium/high speed where Doppler effect becomes a crucial factor, as shown by the preliminary simulation results in Figure 1. Detailed simulation assumptions can be found in Appendix.

Performance with different UE velocities

Out-dated CSI due to Doppler effects

The wireless channel is characterized by the scattering environment, the mobility of terminals and some of the scattering objects as given in TR 38.901[2]. The received signal is a mix of multiple paths scattered around in the environment arriving with different directions and delays. In TR 38.901, the fast fading channel is modeled as the sum of a set of clusters, each composed of multiple rays with arrival angles and departure angles for both azimuth and elevation. The channel coefficient for m-th ray between receive antenna u and transmit antenna s is given by following

hu,s,mt=Pu,s,mFtx,u,mTAmFrx,s,mexpj2πλ0rrx,mTdrx,u+rtx,mTdtx,sexpj2πλ0rrx,mTvt,

where Pu,s,m is the power of the m-th ray between receive and transmit antenna pair (u, s); Am is the complex channel gain matrix denoting the cross-polarization effect of the m-th ray; Ftx,u,m and Frx,s,m are the directional field patterns for polarizations of receive antenna element u and transmit antenna s respectively; rrx,m is the unit vector with azimuth arrival angle and elevation arrival angle of the m-th ray, and rtx,m is the unit vector with azimuth departure angle and elevation departure angle of the m-th ray; drx,u is the location vector of receive antenna element u and dtx,s is the location vector of transmit antenna element s. The Doppler frequency component is calculated from the arrival angles, and the UE velocity vector v considering speed and travel azimuth angle and elevation angle and t is the time.

The above formula clearly shows that the phase of the channel coefficient of a single ray is basically proportional to the time. However, the sum of massive number of rays having different Doppler frequencies caused by different arrival angles per ray makes channel prediction more complicated. As a consequence, the coherence time of the channel which is the inverse of the Doppler spread becomes small as the UE moves fast so that the CSI feedback is out-of-date.

Possible solutions

An approach to cope with out-of-date CSI feedback caused by high/medium UE velocity is to further exploit time-domain correlation/Doppler-domain information to assist DL precoding. The following three directions can be considered for this enhancement.

TRS-based Doppler feedback

In NR systems, TRS is used to assist the UE to track and compensate variations in time and frequency domain, including Doppler measurement for channel estimation, but it is not tailored for precise CSI prediction with complicated channel characteristics. Current TRS is a special configuration of CSI-RS resource set(s) consisting of two or four CSI-RS resources in one slot or two consecutive slots with a single port. Periodic TRS can’t be associated with a CSI report, and reportQuantity must be set to ‘none’ if the CSI report is associated with aperiodic TRS, i.e., there is no actual CSI report for a TRS.

TRS-based Doppler information feedback can work together with regular CSI report as shown in Figure 2. Besides regular CSI report, additional Doppler information can be fed back in a separate report. The gNB performs CSI prediction based on these two kinds of feedback. It is a flexible feedback framework in three aspects: 1) the separate Doppler information feedback can be triggered dynamically depending on the UE speed; 2) it can jointly  work with either Type I or Type II codebook; 3) legacy CSI report is not affected.

Illustration of TRS-based Doppler information feedback

If Doppler information for CSI report is based on TRS, TRS enhancement is inevitable to help Doppler measurement for CSI which consists multiple spatial domain bases and frequency-delay domain bases. Assuming the SD-FD bases are relatively constant in time domain, multiple TRS ports, each precoded with an SD-FD base pair, can be transmitted to UE to measure the Doppler information of the SD-FD pair. Thus, the gNB can predict the CSI of future slots by Doppler information feedback.

Thus, we propose to consider separate Doppler measurement and feedback based on TRS.

Support separate Doppler measurement and feedback based on TRS

A TRS port can be precoded with a SD-FD basis

FFS: whether/how TRS is enhanced to carry multiple SD-FD bases

Predicted CSI feedback by UE

Another solution is to let UE perform the CSI prediction and feed back the predicted CSI in a CSI report. As illustrated Figure 3, the UE can predict the CSI in slot n based on multiple previous received CSI-RS estimates. gNB is able to predict the precoder/channel until slot n by interpolation or predict the precoder/channel after slot n by extrapolation. With this approach, there is no change in CSI report format including PMI quantization and feedback rule for the predicted CSI report.

Illustration of predicted CSI feedback

Support UE reporting a predicted CSI in an indicated future slot.

Rel-16/17 Type II codebook by incorporating Doppler domain compression

Enhanced Type II codebook spatial domain and frequency domain characteristics and feedback the most important components in both domains. The precoder of layer r of all precoding granularities, Wr, generated by enhanced Type II codebook in Rel-16 PMI can be given by

(Wr)N×B=W1N×2L(W2,r)2L×M(WfH)M×B

where W1 is the matrix consisting of same L strongest SD bases across two polarizations, Wf is the matrix formed by M significant FD bases, W2,r is the coefficient matrix of layer r corresponding to each SD-FD basis, and N is the number of CSI-RS ports and B is the number of precoding granularities, e.g., subbands.

If multiple snapshots of CSI in time domain can be obtained, UE can possibly extract the time domain features. As the SD bases and FD bases may be kept same for a period of time, the only varying part in the above equation is W2,r. The sequence of K coefficient matrixes (W2,r)0, (W2,r)1, ⋯, (W2,r)K can be compressed by selecting a few strongest TD bases in a similar way as the spatial domain and frequency domain.

Issues to be discussed

According to the above discussion on possible solutions, we think three aspects need to be considered in the scope:

Measurement of Doppler domain information

Whichever the solution is used, multiple RS samples are needed for Doppler measurement. The number of RS samples, the spacing between neighboring RS samples and the density may have impact on the accuracy of the Doppler measurement for CSI prediction. Whether current configuration of TRS pattern or CSI-RS are qualified for Doppler measurement for CSI prediction needs to be studied.

Doppler domain representation and feedback

Form above analysis, Doppler effects stand for channel characteristics, but it is not clear whether it is applicable to precoding matrix which is assumed in PMI feedback in current CSI reporting. In any case, it is desirable not to change precoding information-based CSI feedback as legacy CSI reporting in this Rel-18 CSI study.

DL precoding based on the CSI with Doppler domain information

DL precoding based on the CSI with Doppler domain information is an implementation issue carried out by the gNB but does affect the performance. Potential specification impact needs to be investigated although it seems only limited impact is needed at first glance.

Therefore, we propose to further study the following aspects.

RS configuration for Doppler measurement for CSI prediction.

Detailed feedback form of Doppler information

Potential specification impact on DL precoding prediction based on Doppler information feedback

Further study the following aspects.

RS configuration for Doppler measurement for CSI prediction.

Detailed feedback form of Doppler information

Potential specification impact on DL precoding prediction based on the CSI with Doppler information feedback

Evaluation methodology

For evaluation methodology, most of the simulation assumptions can reuse Rel-16/17 parameters, but several aspects need further discussion and decision for this high/medium-speed CSI enhancement.

The assumed UE speed needs to be aligned. As the WID targets high and medium UE velocities, both require evaluation. We think 30 km/h and 120 km/h can be typical medium and high UE speeds.

Based on the target UE speeds, the deployment scenarios also need to be determined accordingly. In previously release, dense urban is assumed for all the CSI related simulations. This scenario is okay for medium-velocity UEs, e.g., 30 km/h, but not typical for UEs with high moving speed. High velocity UEs usually appear in rural areas like high ways or high-speed trains. Hence further discussion is needed to align the target deployment scenarios for different UE speeds.

Number of UEs per cell for each deployment scenario also needs to be carefully decided. The decision needs to consider both the typical use cases for high-resolution CSI and typical values for the target scenarios. For example, the typical values of rural scenarios should be smaller than dense urban.

Further, UE distribution also requires further discussion as UEs with high and medium speeds can only appear in automobiles or trains. Thus, the assumption used in previous releases, i.e., 80% indoor (3km/h) and 20% outdoor (30km/h), cannot be simply reused.

Most of the legacy simulations use static simulation. That is, after a UE is dropped, the UE’s position remains unchanged, and the small-scale parameters do not change within the simulation time. This static simulation method is not very suitable for CSI enhancement for high/medium UE velocities, and we think that the simulation method for CSI enhancement for high/medium UE velocities should introduce the effect of UE position change over time, which is close to the real-world case. When UE position changes over time, the spatial consistency modeling needs to be considered to model spatial correlation among different locations in UE’s trajectory. In addition, the details on UE trajectory modelling also need to be further discussed.

Assume 30km/h and 120 km/h for typical medium and high UE speeds.

Support introducing the effect of UE position change over time, which is close to the real-world case.

UE trajectory modelling is needed, FFS details

the spatial consistency modelling is needed to model spatial correlation.

Further discuss details of the following aspects for evaluation of high/medium-velocity CSI

Cell deployment scenario, e.g., whether rural scenarios should be assumed for high velocity

Number of UEs per cell and UE distribution, considering UEs with high and medium speeds can only appear in automobiles or trains.

Views on CSI enhancement for coherent JT

In LTE-Advanced Rel-11[3], Coordinated Multi-Point (CoMP) transmission and reception was considered as a tool to improve high data rate coverage, cell edge throughput, and increase system throughput. Coherent JT is one of the various categories studied in CoMP that has proven performance gain over STRP transmission. In NR, more accurately reporting CSI techniques were introduced in previous releases. Therefore, the study may aim at evaluating the performance benefit and the standardization impact of enhanced/refined coherent JT CSI.

The following sections provide our views on the evaluation methodology and potential enhancement of CSI reporting respectively.

Evaluation methodology

For the evaluation of coherent JT, system level simulation should be the main tool. A common set of simulation assumptions should be agreed for better aligning evaluation results among companies. In the following subsections we discuss multiple deployment scenarios and simulation assumptions together with initial evaluation results of coherent JT performance.

Deployment Scenarios

For coherent JT, inter-TRP antennas shall be well calibrated, otherwise the JT performance cannot be guaranteed. Therefore, the Indoor Hotspot with smaller TRP spacing as shown below, i.e. Scenario 1, are more conducive to deploying coherent JT and can be taken for evaluations.

TRP clustering for Indoor Hotspot (Scenario 1)

In addition, several outdoor CoMP scenarios are given in 3GPP TR 36.819[4], as depicted below.

Homogeneous network with intra-site CoMP (Scenario 2)

Homogeneous network with high Tx power RRHs (Scenario 3)

Reference CoMP coordination cell layout for Scenario 3

Network with low power RRHs within the macrocell coverage (Scenario 4 or Scenario 5)

| Deployment scenarios | - Scenario 2: Homogeneous network with intra-site CoMP, as illustrated in Figure 5 - Scenario 3: Homogeneous network with high Tx power RRHs, as illustrated in Figure 6 - The central entity can coordinate 9 cells as a baseline, with the reference layout as in Figure 7 - Scenario 4: Heterogeneous network with low power RRHs within the macrocell coverage as in Figure 8. - transmission/reception points created by the RRHs have different cell IDs as the macro cell - Coordination area includes: - 1 cell with N low-power nodes as starting point - 3 intra-site cells with 3*N low-power nodes - Scenario 5: Network with low power RRHs within the macrocell coverage where the transmission/reception points created by the RRHs have the same cell IDs as the macro cell as in Figure 8. - Coordination area includes: - 1 cell with N low-power nodes as starting point - 3 intra-site cells with 3*N low-power nodes |
| --- | --- |

Considering antenna calibration among TRPs, for outdoor CoMP scenarios, we prefer that intra-site CoMP, i.e. Scenario 2, can be taken for evaluations.

In order to limit the number of scenarios considering the balance between deployment flexibility and simulation workload, it is preferred to pick one typical indoor scenario and one typical outdoor scenario for evaluation purpose. Hence, we have the following proposal.

Take the Indoor Hotspot and intra-site CoMP scenarios for evaluations.

Evaluation Assumptions

Baseline assumption

As data to a single UE is simultaneously transmitted from multiple transmission points coherently, the cell-edge throughput as well as overall system throughput is increased compared to STRP transmission scheme [4]. In addition, as more antenna ports are involved in coherent JT, larger number of UEs can be co-scheduled which significantly improves the system capacity and performance. Therefore, for performance comparison purpose, STRP transmission with MU scheduling can be the baseline for evaluation.

Take STRP transmission with MU scheduling as the baseline for evaluating coherent JT performance.

Traffic model assumption

Based on initial evaluations, compared to non-coherent JT, coherent JT can obtain more performance gain in high load scenario. Therefore, full buffer and FTP1 traffic model with higher RU are more preferred for evaluation purpose.

Take full buffer and FTP1 traffic model with higher RU for evaluation of coherent JT.

Other assumptions

Based on WID description, there could be ambiguity on the total number of CSI-RS ports across multiple TPRs as highlighted below. One could interpret as the maximum number of CSI-RS ports is 32 if one CMR is used for coherent JT CSI measurements, and 128 if up to four CMRs are used for measurements, which could lead to some inconsistencies for simulation evaluation.

Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

Therefore, the maximum number of CSI-RS ports for coherent JT CSI measurement should be clarified. In our view, total number of CSI-RS ports across 4 TRPs should remain same as in Rel-16/Rel-17, i.e. up to 32, due to high UE cost and complexity to handle more than 32 ports.

Up to 32 total CSI-RS ports across 4 TRPs for simulation evaluation of coherent JT CSI measurement..

In addition, the RS overhead, such as CMR, CSI-IM resource, DMRS, etc, for a coherent JT UE may need to be considered. For STRP transmission, the overhead assumption in DL evaluation can be used as shown in table below which is extracted from Table B.3.1.1-2 of 3GPP TR 37.910[5].

The overhead assumption in the NR DL evaluation from Table B.3.1.1-2 of 3GPP TR 37.910

|  | Applied duplexing | FR1 |
| --- | --- | --- |
| OH1 | FDD, TDD (DDDSU) | - PDCCH: CORESET of 24 PRBs (4 CCE) in every slot - 12 RE/PRB/slot - TRS burst of 2 slots with periodicity of 20ms and occupies 52 PRBs - 12 RE/PRB/20 ms - DMRS: Type II, 16 RE/PRB/slot for 8 layers - CSI-RS: 8 CSI-RS ports with periodicity of 20ms - 8 RE/PRB/20 ms - 1 SS/PBCH blocks (SSB) per 20ms; one SSB occupies 960REs = 4 OFDM symbols × 20 PRB × 12 REs/PRB NOTE1: if the channel bandwidth is less than the bandwidth of SSB, then SSB is not transmitted and the overhead of SS/PBCH block is zero. NOTE2: If the channel bandwidth is less than TRS bandwidth, the TRS bandwidth is assumed to be equal to the channel bandwidth. |

- However, for coherent JT, as the number of measured or transmitting TRPs increases, whether the RS overhead and channel overhead are different from the assumption in STRP transmission should be further discussed. For example, as more CSI-RS ports are measured during coherent JT CSI acquisition, whether it causes an increase in the overhead of CSI-RS requires common assumption.

Determine the modelling of the overhead of RS and physical channels in coherent JT evaluation.

- Another modeling needs to be considered is the actual propagation delay difference. Due to the different propagation delays from a UE to different TRPs, the UE may find more delays in the delay domain when UE unites channels of multiple TRPs to acquire a PMI. More delays may result in greater delay spread leading to increased frequency domain sampling density, and will also affect the configuration of the Rel-16/Rel-17 Type II codebook parameters. Moreover, the first delays from different TRPs are not always aligned. Therefore, we think the modeling of actual propagation delay may be considered in CJT evaluation.

The actual propagation delay differences need to be considered in CJT evaluation.

Initial simulation results

With reference to part of the previous assumptions, the initial evaluation parameters and some preliminary simulation results are given below.

The evaluation parameter configuration

| Parameter | Values used for evaluation |
| --- | --- |
| Deployment scenarios | Scenario 2: Homogeneous network with intra-site CoMP, as illustrated in Figure 5 Scenario 1: Indoor hotspot |
| Channel model | Scenario 2: Urban Macro (200m ISD) Scenario 1: InH |
| Frequency Range | Scenario 2: 2.2GHz for downlink Scenario 1: 2.2GHz for downlink |
| Tx power for cell | Scenario 2: 41/44 dBm in a 10/20 MHz carrier Scenario 1: 21/24 dBm in a 10/20 MHz carrier |
| Antenna array configuration for cell | Scenario 2: (M, N, P, Mg, Ng, Mp, Np) = (8,4,2,1,1,1,4), (dH,dV) = (0.5, 0.8)λ Scenario 1: (M, N, P, Mg, Ng, Mp, Np) = (4,4,2,1,1,2,2), (dH,dV) = (0.5, 0.5)λ |
| UE distribution and UE speed | Scenario 2: 10 UEs per TRP, 80% indoor(3km/h), 20% outdoor(30km/h) Scenario 1: 10 UEs per TRP, 100% indoor, 3km/h |
| Simulation bandwidth | 10 MHz/20MHz for 15kHz |
| Network synchronization | Synchronized |
| Backhaul assumptions | Ideal backhaul |
| UE receiver | MMSE-IRC as the baseline receiver |
| Duplex, Waveform | FDD, OFDM |
| Multiple access | OFDMA |
| Antenna array configuration for UE | (M, N, P, Mg, Ng, Mp, Np) = (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| UE antenna height & gain | Follow TR36.873 |
| UE receiver noise figure | 7dB |
| Modulation | Up to 256QAM |
| MIMO scheme | MU-MIMO with rank adaptation is assumed |
| Rank candidate | Rank 1 as a starting point |
| MIMO layers | The maximum MU layers 8 for STRP transmission The maximum MU layers 24 for coherent JT |
| CSI feedback | CSI feedback periodicity: 5 ms Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Codebook configuration | Eigenvector feedback |
| RS overhead | No consideration |
| Absolute delay model | No consideration |

Referring to the above configuration, some preliminary simulation results based on eigenvector feedback are given to see potential gain of coherent JT. According to the simulation results, ideally, more significant gain can be obtained by JT in the Indoor Hotspot and intra-site CoMP scenarios.

Ideally, more significant gain can be obtained by JT in the Indoor Hotspot and intra-site CoMP scenarios.

Performance evaluation for Indoor Hotspot

Performance evaluation for Intra-site CoMP

Views on reporting CSI

In WID[1] description, following assumptions are made for FDD CJT.

Targeting FR1 and up to 4 TRPs

Assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs

Rel-16/17 Type-II codebook refinement

Based on these assumptions, we discuss CSI resource setting, CSI reporting setting, and CSI in following sub-sections.

CSI resource setting

In legacy CSI resource setting, only one CSI resource for channel measurement (CMR) is configured when Rel-16/Rel-17 Type II PMI is configured, and two CMRs are used where each CMR corresponds to a TRP for non-coherent JT CSI acquisition. Therefore, there could be following two ways of CMR configurations for coherent JT.

Method 1: One CMR is used for acquiring the CSI of up to 4 TRPs.

Method 2: Up to 4 CMRs are used for acquiring the CSI of up to 4TRPs.

- For Method 1, the CMR may be divided into multiple port groups, and each port group corresponds to a TRP. It means the definition of port group should be introduced in specs. Besides, as different TRPs may be non-QCLed, multiple TCI states are needed for the CMR, which is contrary to the legacy rule that one CMR is associated with one TCI state.  In particular, the impact may be somewhat greater for SP or AP CSI-RS triggering.

- For Method 2, the problem of different TRPs may have different TCI states can be solved due to multiple CMRs corresponding to multiple TRPs.

- Therefore, the use of multiple CMRs to acquire the coherent JT CSI should be supported where each CMR corresponds to a TRP.

Support a coherent JT measurement hypothesis measured on multiple CMRs where each CMR corresponds to a TRP.

CSI reporting setting

In Rel-17, the following agreement was reached to support flexible CSI reporting.

Agreement For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, support following two options: - Option 1: the UE can be configured to report X CSIs associated with single-TRP measurement hypotheses and one CSI associated with NCJT measurement hypothesis - X = 0, 1, 2 - If X=2, two CSIs are associated with two different single-TRP measurement hypotheses with CMRs from different CMR groups - Support of X=1,2 is UE optional for the UE supporting option 1 - FFS omission of CSI associated with NCJT measurement hypothesis - Option 2: the UE can be configured to report one CSI associated with the best one among NCJT and single-TRP measurement hypotheses - FFS how to report recommended measurement hypothesis associated with that CSI report

For coherent JT CSI, channel information for more than 2 TRPs needs to report. The feedback overhead and signaling overhead will increase compared to non-coherent JT or STRP transmission. Therefore, the flexibility of coherent JT CSI reporting should be considered to reduce the feedback overhead and signaling overhead.

One way to reduce feedback overhead is to allow UE recommending selected TRPs, it can be observed from distribution of UE selected TRPs in Figure 12 and Figure 13 that even if UE does not always reports CJT CSI for 4 TRPs, the performance loss is not severe. In any case, in coherent JT deployment scenario UEs with same measurement set may have different collaboration set. And, the size of the collaboration set is equal to or smaller than the size of the measurement set. For example, the Figure 11 shows some UEs with same measurement set while different collaboration sets. If the UE can report the coherent JT CSI only for collaboration set, the feedback overhead can be reduced compared to reporting of CSI for measurement set. Besides, if TRP recommendation is enabled, the matrix dimension of UE operations also decreases, thus reducing the computational complexity of UE.

Diagram of measurement set and collaboration set

Initial simulation results in Figure 12 and Figure 13 depict TRP recommendation based on CMR RSRP. If the CMR reception power of a TRP is less than the reception power corresponding to the strongest TRP minus a power gap XdB, the CSI of this TRP is not involved in the coherent JT CSI, i.e., UE reports the CSI for TRP(k) based on a power gap as follows

RSRPmax-RSRPTRPk<X dB

Where RSRPmax denotes the largest CMR reception power among TRPs in the measurement set and RSRPTRPk denotes the CMR reception power of TRP(k).

We evaluated three threshold values of RSRP gap, 10 dB, 15 dB, and 20 dB, respectively, for Indoor Hotspot and Intra-site CoMP scenarios. According the following simulation results, TRP recommendation causes marginal performance loss, but it reduces more feedback overhead because more than 50% of UEs do not need to report CSI for all TRPs in the measurement set.

Performance and UE percentage statistics for Indoor Hotspot

Performance and UE percentage statistics for Intra-site CoMP

TRP recommendation causes marginal performance loss, but it reduces feedback overhead and UE complexity significantly because more than 50% of UEs do not need to report CSI for all TRPs in the measurement set.

Therefore, we propose to support UE recommendation of TRPs to reduce the feedback overhead and signaling overhead.

Support UE recommendation of TRPs to reduce the feedback overhead and signaling overhead.

CSI

Regarding the CSI acquisition, the key issue is the codebook to acquire coherent JT PMI. The following potential schemes can be identified.

Scheme 1: A coherent JT CSI contains one PMI that is obtained jointly based on aggregated channels of multiple TRPs.

Scheme 2: A coherent JT CSI contains multiple PMIs and each PMI is obtained based only on the channel of a corresponding TRP.

- The partial DFT vectors in a complete DFT set may not portray the SD information of multiple TRPs well, so a larger number of DFT bases are needed if Rel-16 eType II codebook is used directly for PMI acquisition by Scheme 1. Further, as the number of SD bases increases, the number of the coefficients also increases. Considering the overhead, independent selection of SD bases per TRP may be supported to reduce the valid number of coefficients. In addition, due to the actual propagation delay differences among TRPs, the delay spread of coherent JT may greater than delay spread of STRP transmission, so a finer subband configuration for coherent JT PMI acquisition may be required unless the gNB pre-compensates the delay for the UE.

- Based on independent selection of SD per TRP, the PMI can be denoted as following two expressions,

- W=W1,0…0⋮⋱⋮0…W1,Ntrp-1W2,0⋮W2,Ntrp-1WfH							(1)

- or

- W=W0W1Wp=W1,0,p…0⋮⋱⋮0…W1,Ntrp-1,pW2,0,p⋮W2,Ntrp-1,pWfH,p=0,1									    	(2)

- where W1,i,i=0,1,..,Ntrp-1 denotes the selected SD bases for TRP i which contains beam for two polarization and Ntrp denotes the number of measured TRP.  W2,i,i=0,1,..,Ntrp-1 denotes the coefficients of TRP i which are associated with the SD bases for TRP i. WfH denotes the Hermitian transpose of selected FD bases. Wp,p=0,1 denotes the precoder for polarization p.

- According to the expression (2), it seems that the legacy method can be reused for FD basis acquisition and coefficient acquisition. Regarding the expression (1), it is easy to associate the enhancement of the TRP-specific coefficient quantization to the legacy inter-polarization coefficient quantization.

- Therefore, the acquisition of the FD basses and the coefficients for Scheme 1, the following methods can be considered.

Method 1: Legacy method can be reused for FD basis acquisition and coefficient quantization.

Method 2: Legacy method can be reused for FD basis acquisition and phase coefficient quantization, and quantization of TRP-specific amplitude coefficient can be considered to support more accurate amplitudes.

Method 3: TRP-specific FD basis acquisition and coefficient quantization can be considered.

- With Scheme 2, legacy PMI acquisition based on Rel-16 eType II codebook is used for each TRP. However, similar as in multi-panel Type 1 codebook, the amplitude coefficients and/or the phase coefficients among TRPs may be needed for coherence transmission. In this case, the setting of candidate coefficient set and the application of the coefficient are the key issues. If the number of candidate coefficients is small, the performance may be severely degraded. If the number of candidate coefficients is large, the complexity to search for the optimal coefficient values becomes greater. If the coefficients are applied TRP-specific and layer-specific, the complexity of UE search may be enormous.

- Therefore, for the acquiring of coherent JT PMI, we have the following proposal.

Following aspects should be considered for the acquisition of coherent JT PMI.

UE complexity

feedback overhead

Some initial simulation results based on Rel-16 codebook refinement are given for Scheme 1 and Scheme 2 in Figure 14 and Figure 15. The following assumptions Table 3 are employed in addition to the evaluation parameter configuration shown in Table 2.

The employed assumptions for Rel-16 eType II codebook refinement schemes

|  | - Scheme 1 | - Scheme 2 |
| --- | --- | --- |
| - Codebook configuration | - β=0.5;L=4;pv=0.5 |  |
| - The PMI acquisition | - Independent selection of SD per TRP - Joint selection of FD basis among TRPs following legacy method for FD basis acquisition and coefficient quantization | - Independent PMI acquisition per TRP. - The TRP-specific amplitude coefficients are selected by the eigenvalues associated with each TRP. Based on the selected amplitude, the TRP-specific phase coefficient is further selected based on the channel. - The setting of candidate coefficient set: - Amplitude:12n, n=0,1,…,15 - Phase:8 PSK |

Performance comparison of different coherent JT PMI acquisitions for Indoor Hotspot

Performance comparison of different coherent JT PMI acquisitions for Intra-site CoMP

- According to the simulation results, Scheme 1 performs better than Scheme 2. However, one advantage of Scheme 2 may be is the simplicity at gNB to obtain the optimal precoder for each TRP.

Some potential schemes for PMI acquisition are given below.

Scheme 1: A coherent JT CSI contains one PMI that is obtained based on the aggregated channels of multiple TRPs.

TRP-specific SD acquisition is supported.

Coefficient quantization enhancement can be considered.

It is hard for gNB to obtain the precoder for each TRP.

Scheme 2: A coherent JT CSI contains multiple PMIs and each PMI is obtained based on the channel of a corresponding TRP.

The amplitude coefficients and/or the phase coefficients among TRPs may be needed.

The setting of candidate coefficient set and the use of the coefficient are the key issues.

It is simple for gNB to obtain the precoder for each TRP.

Compared to Scheme 2, Scheme 1 has performance gain.

TCI indication for DMRS

Regarding the TCI indication for DMRS of coherent JT, since DMRS comes from up to 4 TRPs, different TRPs may be associated with different QCL-TypeA properties. The performance of DMRS channel estimation may be affected if the UE follows only one TCI state related with certain TRP. Since the coherent JT can be considered as a special SFN transmission, some conclusions in the Rel-17 SFN enhancement can be reused. According to the HST-SFN enhancement in Rel-17, distributed TRS resource sets are transmitted from each TRP respectively. Thus, it is natural that the same DMRS port(s) could be QCLed with multiple TRS resource sets simultaneously. In HST-SFN enhancement, two TCI states in the DCI codepoint for PDSCH transmission is specified. It is natural that the similar solution can used for coherent JT, i.e., up to four TCI states in the DCI codepoint for PDSCH transmission.

SFN transmission with distributed TRS

Study possible enhancement on TCI state indication for DMRS ports of PDSCH coherent JT with up to 4 TRPs.

Conclusions

To summarize, we have following observations and proposals.

Observation 1:

Ideally, more significant gain can be obtained by JT in the Indoor Hotspot and intra-site CoMP scenarios.

Observation 2:

TRP recommendation causes marginal performance loss, but it reduces feedback overhead and UE complexity significantly because more than 50% of UEs do not need to report CSI for all TRPs in the measurement set.

Observation 3:

Some potential schemes for PMI acquisition are given below.

Scheme 1: A coherent JT CSI contains one PMI that is obtained based on the aggregated channels of multiple TRPs.

TRP-specific SD acquisition is supported.

Coefficient quantization enhancement can be considered.

It is hard for gNB to obtain the precoder for each TRP.

Scheme 2: A coherent JT CSI contains multiple PMIs and each PMI is obtained based on the channel of a corresponding TRP.

The amplitude coefficients and/or the phase coefficients among TRPs may be needed.

The setting of candidate coefficient set and the use of the coefficient are the key issues.

It is simple for gNB to obtain the precoder for each TRP.

Compared to Scheme 2, Scheme 1 has performance gain.

Proposal 1:

Support separate Doppler measurement and feedback based on TRS

A TRS port can be precoded with a SD-FD basis

FFS: whether/how TRS is enhanced to carry multiple SD-FD bases

Proposal 2:

Support UE reporting a predicted CSI in an indicated future slot.

Proposal 3:

Further study the following aspects.

RS configuration for Doppler measurement for CSI prediction.

Detailed feedback form of Doppler information

Potential specification impact on DL precoding prediction based on the CSI with Doppler information feedback

Proposal 4:

Assume 30km/h and 120 km/h for typical medium and high UE speeds.

Support introducing the effect of UE position change over time, which is close to the real-world case.

UE trajectory modelling is needed, FFS details

the spatial consistency modelling is needed to model spatial correlation.

Further discuss details of the following aspects for evaluation of high/medium-velocity CSI

Cell deployment scenario, e.g., whether rural scenarios should be assumed for high velocity

Number of UEs per cell and UE distribution, considering UEs with high and medium speeds can only appear in automobiles or trains.

Proposal 5:

Take the Indoor Hotspot and intra-site CoMP scenarios for evaluations.

Proposal 6:

Take STRP transmission with MU scheduling as the baseline for evaluating coherent JT performance.

Proposal 7:

Take full buffer and FTP1 traffic model with higher RU for evaluation of coherent JT.

Proposal 8:

Up to 32 total CSI-RS ports across 4 TRPs for simulation evaluation of coherent JT CSI measurement..

Proposal 9:

Determine the modelling of the overhead of RS and physical channels in coherent JT evaluation.

Proposal 10:

The actual propagation delay differences need to be considered in CJT evaluation.

Proposal 11:

Support a coherent JT measurement hypothesis measured on multiple CMRs where each CMR corresponds to a TRP.

Proposal 12:

Support UE recommendation of TRPs to reduce the feedback overhead and signaling overhead.

Proposal 13:

Following aspects should be considered for the acquisition of coherent JT PMI.

UE complexity

feedback overhead

Proposal 14:

Study possible enhancement on TCI state indication for DMRS ports of PDSCH coherent JT with up to 4 TRPs.

Appendix

The evaluation parameter configuration for UE velocity impact

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, OFDM |
| Multiple access | OFDMA |
| Scenario | Urban Macro |
| Frequency Range | FR1 only, 2.2GHz for downlink |
| Antenna setup and port layouts at gNB | (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power | 46 dBm for 10MHz |
| UE receiver noise figure | 7dB |
| Modulation | Up to 64QAM |
| Numerology | 14 OFDM symbol slot |
|  | SCS 15KHz, 52 PRB |
| Frame structure | Slot Format 0 (all downlink) for all slots |
| MIMO scheme | MU-MIMO with rank adaptation is assumed |
| Rank candidate | Rank 1/2 as a starting point |
| MIMO layers | The maximum MU layers 8 |
| CSI feedback |  |
|  | CSI feedback periodicity (full CSI feedback) : 10 ms, |
|  | Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead | Companies shall provide the downlink overhead assumption |
| Traffic model | Full buffer |
| UE distribution | 100%outdoor |
| UE receiver | MMSE-IRC as the baseline receiver |
| Codebook configuration | Rel-16 eTypeII Codebook, L=4, beta = 0.5, subband number = 13, M_v = 7 |

References

RP-213598, New WID: MIMO Evolution for Downlink and Uplink, RAN#94e.

3GPP TR 36.901: “Study on channel model for frequencies from 0.5 to 100 GHz”.

3GPP TR 36.814: “Further Advancements for E-UTRA, Physical Layer Aspects”.

3GPP TR 36.819: “Coordinated multi-point operation for LTE physical layer aspects”.

3GPP TR 36.910: “Study on self evaluation towards IMT-2020 submission”.