3GPP TSG RAN WG1 #109-e	R1-2203544

e-Meeting, May 9th – 20th, 2022

Source: 	vivo

Title:	Views on DMRS enhancements

Agenda Item:	9.1.3.1

Document for:	Discussion and Decision

Introduction

In the previous RAN #94 meeting, the following Rel-18 WIDs have been approved for DMRS enhancement, including DMRS ports enhancement for MU-MIMO and 8Tx UL DMRS [1].

Objective 3: Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM, - striving for a common design between DL and UL DMRS - Up to 24 orthogonal DM-RS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS Objective 6: Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In this contribution, some potential solutions for DMRS port enhancement for MU-MIMO would be discussed. Link-level simulation results are given to compare the performance differences between the potential solutions.  Besides, the DMRS ports indication for Rel-18 MU-MIMO and 8Tx DMRS are also analyzed.

Larger number of DMRS ports for MU-MIMO

In the current spec, there are two DMRS configuration types specified, i.e., DMRS type1 and DMRS type2. For DMRS type1, up to 4/8 orthogonal ports are supported with single/double-symbol, while for DMRS type2, up to 6/12 ports are supported. However, considering there would be more and more UEs in the current network. it is necessary to increase the number of UEs scheduled in MU-MIMO. One significant method is to increase the number of orthogonal ports of DMRS to support the higher order of MU-MIMO in Rel-18.

Moreover, current DMRS ports are multiplexed in FDM and CDM manner. For DMRS type 1/type 2, it is designed with two/three CDM groups, and in each group, there are up to 2/4 DMRS ports for single/double-symbol configurations. Therefore, to increase the number of DMRS ports, there are two possible ways that can be considered, i.e., increasing the number of DMRS ports in each CDM group and increasing the number of CDM groups.

Potential solutions for DMRS ports enhancement

Enhancement on code-domain

- Scheme 1: Introducing FD-OCC=4

For the DMRS ports in the same CDM group, FD-OCC=2 is used to multiplex two DMRS ports on the same set of REs on the OFDM symbol in the current NR system. Therefore, one natural way is to double the number of DMRS ports in each CDM group by introducing FD-OCC=4. For DMRS type2, FD-OCC=4 can be mapped directly on the 4 REs in each RB occupied by a DMRS port. However, for DMRS type 1, since there are 6 REs occupied by a DMRS port in one RB, FD-OCC=4 mapping on 6 REs is not straightforward.

In this case, two potential solutions can be considered for DMRS type 1 as follows.

- Scheme 1a: Define the scheduling granularity as 2 RBs and perform channel estimation based on FD-OCC=4 across 2 RBs.

- Scheme 1b: Perform two channel estimations (CE) based on FD-OCC=4 in one RB.

Two channel estimations based on FD-OCC=4 in one RB

As shown in Figure 1, it can be observed that two channel estimations are performed in two CE windows separately. In each CE window, the receiver can estimate the channel of DMRS ports based on the orthogonal codes of FD-OCC=4. In this case, the channel associated with sub-carrier #4 and #6 would be estimated twice. Then how to deal with these two results of channel estimation in sub-carrier #4 and #6 is up to implementation, e.g., to calculate an average result.

Two channel estimations based on FD-OCC=4 can be performed in one RB, due to the orthogonality of FD-OCC in each CE window.

One advantage of scheme 1b is that it can be used to address the issue when the number of scheduled RBs for PDSCH/PUSCH is odd. In this case, the receiver can perform two channel estimations in all RB or just in the orphan RB. Moreover, one key point needed to be mentioned is that, in scheme 1b, it doesn’t mean that FD-OCC=4 is as a mapping granularity for DMRS generation, but a granularity for FD-OCC based channel estimation. In other words, when mapping the FD-OCC on the DMRS ports in scheme 1b, the length of the FD-OCC codes can be either 2 or 4. If the length of the FD-OCC codes equals 2, the FD-OCC codes can be mapped on every pair of 2 REs in each CDM group, then two adjacent FD-OCC codes can be combined to perform channel estimation on the receiver side. However, if the length of the FD-OCC codes equals 4, when the number of scheduling RBs is odd, an additional mapping rule may be needed for how to map the FD-OCC codes in the orphan RB.

Performing two channel estimations based on FD-OCC=4 can address the issue when the number of scheduling RBs for PDSCH/PUSCH is odd, where UE can perform two channel estimations in all RB or only in the orphan RB.

Support scheme 1b with two CE windows to address the scheduled RB issue of DMRS type 1, if introducing FD-OCC=4 in Rel-18.

Another advantage of introducing FD-OCC=4 is that legacy DMRS ports can be reused in Rel-18, since the legacy FD-OCC=2 codes are [+1, +1] and [+1, -1], which can be regarded as a sub-sequence of FD-OCC=4 codes, i.e., [+1, +1, +1, +1], [+1, -1, +1, -1].

Legacy DMRS ports can be reused if introducing FD-OCC=4 in Rel-18.

- Scheme 2: Introducing FD-OCC=6

FD-OCC=6 can also be used to multiplex the ports of DMRS type 1, where the FD-OCC codes can be exactly mapped on the 6 REs per RB for each DMRS port. However, there are three key issues on FD-OCC=6 that should be noticed as follows.

- New OCC codes with a length of 6 should be designed. For instance,4 new codes such as DFT sequences or CGS with a length of 6 should be designed to double the number of DMRS ports and keep the orthogonality of DMRS ports in each CDM group.

- The performance of channel estimation may be degraded based on FD-OCC=6, especially when the delay spread is large.

- FD-OCC=6 can be only applied for DMRS type 1, which would lead to differentiated FD-OCC designs for two DMRS types.

4 new codes of FD-OCC=6 need to be designed to double the number of DMRS ports in each CDM group.

FD-OCC=6 could be only applied for DMRS type 1, which would lead to differentiated FD-OCC designs for two DMRS types.

Enhancement on FD-OCC to double the number of DMRS ports in each CDM group should be considered for Rel-18 DMRS enhancement.

Enhancement on frequency-domain

- Scheme 3: Reducing the frequency density based on a more sparse comb

To double the number of DMRS ports, another potential way is to further reduce the frequency density of DMRS ports, e.g. with a more sparse comb design. For DMRS type 1, its pattern can be regarded as a comb-2 design, since 6 REs are occupied at a regular interval of one RE in each RB by a DMRS port. Therefore, a comb-4 structure can be used to further reduce the frequency density for DMRS type 1. That means there are only 3 REs occupied uniformly by a port of DMRS type 1 in each RB. For DMRS type 2, its pattern does not follow the comb structure in the current design. However, by selecting one RE from two adjacent REs, a comb-6 structure can be achieved for DMRS type 2, where there are only 2 REs occupied uniformly by a port of DMRS type 2 in each RB.

In scheme 3, the number of CDM groups is doubled, and the number of DMRS ports multiplexed in each CDM group is the same as the current DMRS design. In Figure 2, examples of scheme3 are given to show the pattern of DMRS type1 and DMRS type 2.

| a) DMRS type 1 | b) DMRS type 2 |
| --- | --- |

The pattern of DMRS type 1 and DMRS type 2 in scheme 3

However, for the comb-4 design of DMRS type 1, how to map the FD-OCC would be an issue. One solution is to define the scheduling granularity as 2 RBs with FD-OCC=2. Similar to scheme 1b, two CE windows based on FD-OCC=2 can also be applied in one RB for DMRS type 1 in scheme 4. Moreover, introducing FD-OCC=3 is another possible way to address the issue. For the comb-6 design of DMRS type 2, FD-OCC=2 can be mapped on the two REs in each RB.

Two CE windows based on FD-OCC=2 can be applied to DMRS type 1 with comb-4 design.

- Scheme 4: Reducing the frequency density based on an occupying bundle

Another scheme to reduce the frequency density of DMRS ports is based on an occupying bundle. Each occupying bundle includes two adjacent REs of the total REs occupied by a DMRS port, which is associated with FD-OCC=2 mapping. In Figure 3, an example of scheme4 is given to show the pattern of DMRS type1in scheme 4. In Figure 3, it can be observed that a total of 6 REs associated with 3 occupying bundles are occupied across 2 RBs for each DMRS port of DMRS type 1. Regarding DMRS type2, it is obvious that occupied 4 REs in each RB by a DMRS port can be divided into two occupying bundles, each occupying bundle with two adjacent REs can be assigned to one specific CDM group.

The pattern of DMRS type 1 in scheme 4

From the perspective of the density of occupied REs, scheme 4 is similar to scheme 3. Scheme 4 can also achieve the target to double the number of DMRS CDM groups. Since scheme 4 can be used for DMRS type 1 and DMRS type 2 based on the same design principle with FD-OCC=2, it can lead to a unified design for both DMRS types with up to 4 CDM groups in scheme 4.

Scheme 4 can be applied to both DMRS type1 and DMRS type2 based on the same design principle with FD-OCC=2.

Enhancement on frequency-domain to double the number of CDM groups can be considered for Rel-18 DMRS enhancement.

Enhancement on time-domain

- Scheme 5: Introducing TD-OCC=4

In the current DMRS design, to multiplex more ports within one CDM group, DMRS can be configured with double-symbol, then TD-OCC=2 is applied to ensure the orthogonality among the DMRS ports within a CDM group. Therefore, to further increase the number of DMRS ports, introducing TD-OCC=4 is a possible way to double the number of DMRS ports in each CDM group. However, how to map TD-OCC=4 is an issue. One simpler way is to map TD-OCC=4 across the front-load and additional position of DMRS as shown in Figure 4. However, from the perspective of DMRS overhead, TD-OCC=4 may increase the overhead of DMRS compared with other solutions, e.g., FD-OCC=4.

The pattern of DMRS type 1 in scheme 5

- Scheme 6: Increase the CDM group in the additional position

In order to double the DMRS ports, another time-domain enhancement on DMRS is to increase the CDM group in the additional position. In scheme 6, new CDM groups are mapped only on the additional position of DMRS ports. The main difference is that scheme 5 is used to increase the number of DMRS ports in each CDM group, while scheme 6 is used to increase the number of CDM groups. Figure 5 shows an example of the pattern of DMRS type 1 in scheme 6. Similar to scheme 5, scheme 6 may also increase the total overhead of DMRS.

The pattern of DMRS type 1 in scheme 6

Enhancement on time-domain, e.g., TD-OCC=4 may increase the overhead of DMRS compared with other solutions, e.g., FD-OCC=4.

Enhancement on time-domain based on additional positions of DMRS would restrict the configurable number of additional positions.

Scheduling legacy UE and Rel-18 UE in MU-MIMO

When Rel-18 UEs and legacy UEs both exist in the network, for the overall efficiency of the network, scheduling of Rel-18 UEs and legacy UEs together in MU-MIMO should be supported. However, how to multiplex the DMRS ports for legacy UEs and Rel-18 UEs need to be further discussed, since it depends on how to design the DMRS pattern in Rel-18. For instance, if FD-OCC=4 is used to enhance the DMRS ports in Rel-18, it would be difficult to schedule legacy UEs and Rel-18 UEs in the same CDM group. The DMRS port with FD-OCC=4 would cause interference to the legacy DMRS ports with FD-OCC=2. In this case, legacy UEs and Rel-18 UEs should be scheduled in FDM manner, i.e., in different CDM groups.

How to multiplex the DMRS ports for legacy UEs and Rel-18 UEs need to be discussed.

Support scheduling of Rel-18 UEs and legacy UEs together in MU-MIMO.

Simulation assumptions and evaluation results

Link-level simulation assumption

To evaluate the performance of different potential schemes for DMRS enhancement, the following link-level simulation assumptions can be considered in Table 1.

Link-level simulation assumptions

| Parameter | Value |
| --- | --- |
| Duplexing | TDD |
| Carrier frequency | 4GHz |
| Subcarrier spacing | 30kHz |
| Channel model | CDL-A, DS=100/300ns |
| gNB antenna port | 8/16/32 ports |
| UE antenna port | 4/8 ports |
| DL precoding | Type I codebook, PRG=4 |
| PDSCH/PUSCH mapping | Type A, 14 OFDM symbols |
| Bandwidth | 20RB |
| MCS | 16QAM/64QAM, code rate=0.5 |
| Layer | 1/2/4/8 layer per UE |
| Channel estimation | 2D-MMSE filter |
| Receiver | MMSE |
| UE speed | 3km/h, 60km/h |
| TRS periodicity | 20ms/50ms, 2 slot TRS |

Evaluation results of potential solutions

The evaluation result is given to show the performance of different schemes for enhancement on DMRS type 1 in Figure 6. In the evaluation, the BLER per UE of PDSCH transmission is simulated in CDL-A channel with DS equal to 300, where the modulation order is 64QAM with the code rate equal to 0.5, and the number of the transmission layer per UE is assumed as 1. The number of antenna ports is set as 8 for gNB and 4 for UE with the moving speed as 3km/h. Besides, TRS with a period of 50ms is configured to track the time-frequency characteristics to provide the QCL reference for DMRS estimation with 2D-MMSE filter.

The evaluation results of schemes for enhancement on DMRS type 1

According to the above simulation results, it is observed that the performance of enhancement on code-domain (i.e., FD-OCC=4 and FD-OCC=6 scheme) outperforms enhancement on frequency-domain. Besides, two FD-OCC=4 schemes have almost the same performance, which shows using two CE windows in one RB is an effective solution for FD-OCC=4.

The performance of enhancement on code-domain outperforms enhancement on frequency-domain.

Enhancement on DMRS port indication

DMRS port indication for the larger number of DMRS ports

In the current spec, there are many specific tables for DMRS ports indication in TS 38.212. In Rel-18, there would be more DMRS ports available for MU-MIMO scheduling. Therefore, how to enhance the indication of DMRS port(s) is an important issue for the Rel-18 DMRS enhancement agenda.

When a Rel-18 UE is indicated with the Rel-18 DMRS ports for PDSCH/PUSCH transmission, the channel estimation performance would be degraded compared to channel estimation based on the legacy DMRS ports. Therefore, when the traffic load or the number of UEs in MU-MIMO is changed, it should be supported to switch back to DMRS ports between legacy DMRS ports and Rel-18 DMRS ports for different needs.

One solution is switching the table for DMRS ports indication by high-layer signaling such as RRC, and then indicating the DMRS port in DCI when scheduling the PDSCH/PUSCH. Another solution is to indicate the DMRS ports based on DCI without switching the table for DMRS ports.

Dynamic indication between legacy DMRS ports and Rel-18 DMRS ports should be supported.

DMRS port indication for 8TX

For uplink 8Tx transmission, there might be more than 4 layers of PUSCH transmission. In this case, more than 4 DMRS ports should be indicated to estimate the channel information for PUSCH transmission. In the current NR spec, downlink transmission already supports the DMRS ports indication for PDSCH transmission with more than four layers. Therefore, it is better to reuse the DMRS port indication mechanism of PDSCH for Rel-18 uplink 8Tx transmission.

For DMRS port indication for PUSCH with more than four layers, support to reuse DL DMRS indication principle.

Conclusion

In summary, the following observations and proposals are made for Rel-18 DMRS enhancement.

- Two channel estimations based on FD-OCC=4 can be performed in one RB, due to the orthogonality of FD-OCC in each CE window.

Performing two channel estimations based on FD-OCC=4 can address the issue when the number of scheduling RBs for PDSCH/PUSCH is odd, where UE can perform two channel estimations in all RB or only in the orphan RB.

Legacy DMRS ports can be reused if introducing FD-OCC=4 in Rel-18.

4 new codes of FD-OCC=6 need to be designed to double the number of DMRS ports in each CDM group.

FD-OCC=6 could be only applied for DMRS type 1, which would lead to differentiated FD-OCC designs for two DMRS types.

Two CE windows based on FD-OCC=2 can be applied to DMRS type 1 with comb-4 design.

Scheme 4 can be applied to both DMRS type1 and DMRS type2 based on the same design principle with FD-OCC=2.

Enhancement on time-domain, e.g., TD-OCC=4 may increase the overhead of DMRS compared with other solutions, e.g., FD-OCC=4.

Enhancement on time-domain based on additional positions of DMRS would restrict the configurable number of additional positions.

How to multiplex the DMRS ports for legacy UEs and Rel-18 UEs need to be discussed.

The performance of enhancement on code-domain outperforms enhancement on frequency-domain.

- Support scheme 1b with two CE windows to address the scheduled RB issue of DMRS type 1, if introducing FD-OCC=4 in Rel-18.

Enhancement on FD-OCC to double the number of DMRS ports in each CDM group can be considered for Rel-18 DMRS enhancement.

Enhancement on frequency-domain to double the number of CDM groups can be considered for Rel-18 DMRS enhancement.

Support scheduling of Rel-18 UEs and legacy UEs together in MU-MIMO.

Dynamic indication between legacy DMRS ports and Rel-18 DMRS ports should be supported.

For DMRS port indication for PUSCH with more than four layers, support to reuse DL DMRS indication principle.

Reference

RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung