3GPP TSG RAN WG1 #109-e	R1-2203547

e-Meeting, May 9th – 20th, 2022

Source: 	vivo

Title:	Views on enabling 8 TX UL transmission

Agenda Item:	9.1.4.2

Document for:	Discussion and Decision

Introduction

In RAN#94e, new WID on further enhancement of UL and DL MIMO technologies was approved [1], one of the objectives is to study necessary enhancements to support 8Tx UL operation, as copied below:

Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In this contribution, we discuss relevant scenarios and basic design principles, simulation assumptions, and also provide initial evaluation results.

Discussion

Rel-15/16/17 supports up to 4Tx transmission in UL, and there are 2 modes of operations supported in specification: codebook-based and non-codebook-based UL transmission. For codebook-based operation, considering there are many different UE implementations, non-coherent, partial-coherent and full-coherent codebook subsets are supported.

The WID [1] includes varieties of device types covering varieties of deployment scenarios, for example CPE and FWA type of devices are not moving however may have different capability in terms of antenna coherence. While vehicular devices are fast moving and industrial devices are with moderate mobile speed. Vehicular and industrial devices may have better capability than CPE type of devices. To ensure manageable workload, it is desired to limit the supported scenarios and antenna assumptions. The “Note” in the WID is meant to identify e.g. coherence assumptions, full power modes, hence in our views these should be settled in the very first meeting for the group to work in focused manner. CPE type of devices are economic devices, it is fair to assume that the antennas are non-coherent in this case. Similar to Rel-15/16/17 non-coherent assumption, a non-coherent codebook subset can be supported for 8Tx UL transmission, which is straight forward. Other devices such as FWA or vehicular or industrial type of devices, it can be assumed that the antennas are structured and fully-coherent as shown in Figure 1. For such type of devices, a separate codebook design can be considered such as Rel-15 DL Type 1 codebooks. Partial coherent codebook subset can also be supported. However, we should be careful not to cause large DCI overhead and avoid spending lots of time and effort. One straight forward way could be to simply reuse 4Tx codebook subsets to construct 8Tx partial-coherent codebooks.

Examples of 8Tx antenna structure

The supported scenarios and antenna assumptions should be settled in the very first meeting for the group to work in focused manner.

Design principle

From specification wise, both codebook-based and non-codebook-based schemes should be supported for 8Tx UL operation.

To support non-codebook-based transmission scheme, there are mainly two aspects to consider, one is SRS resource configuration and another is SRI indication design. Rel-15/16/17 framework can be extended to support 8Tx non-coherent transmission scheme however designing SRI indication tables could be time consuming and overhead could be large. One way could be to use 2 SRI fields as in Rel-17 mTRP PUSCH, in this case 2 sets of SRS resources can be configured, each with 4 resources. For SRI field mapping, first SRI field corresponds to first set and second SRI field corresponds to second set.

Two SRI fields corresponding to two SRS resource sets for non-codebook transmission can be considered to simplify the SRI indication.

To support codebook-based transmission scheme, 8Tx codebooks need to be specified. As discussed in previous section, non-coherent and partial-coherent codebook subsets can be supported by simply concatenating current 4Tx codebook subsets and UL 8Tx precoder is indicated using 2 TPMI fields in the DCI. For advanced devices with regular shaped cross-pole coherent antennas, Rel-15 DL single panel type 1 codebooks can be baseline, which is to be evaluated comparing system performance and DCI overhead. To support 2 TPMIs in DCI, a set of 2 SRS resources can be configured, or if 1 SRS is configured then the ports are grouped into 2 groups. Whereas 1 SRS resource with 8 ports can be configured to support Rel-15 DL single panel type 1 codebooks.

Codebook constructed by two 4Tx precoders indicated by two TPMI fields can be considered for partial and none coherent antenna assumption.

DL type1 codebook can be considered for fully coherent antenna assumption.

On the number of supported codewords (CWs), similar to DL, 2 CWs can be considered for rank>4, in this case separate HARQ ID, RV, NDI fields are needed in DCI.

Two CWs can be considered for rank>4.

Other issues to consider

Other related specification enhancement needed to support 8Tx UL are SRS, DMRS, PTRS and whether/how to support all UL full power transmission modes.

Details on SRS, DMRS configuration/indication are discussed in [3][2], for DMRS port indication the entries for DL 8Tx transmission can be taken as starting point.

In current spec, maximum of 2 ports PTRS is supported, which is configured by RRC. And, association of PTRS port to DMRS port is indicated in DCI. For the case of supported layers >4, PTRS to DMRS association table shall be extended.

Rel-16 supports UL full power transmission, there are three modes specified namely, fullpower mode (also known as mode0), fullpowerMode1 and fullpowerMode2.  For CPE/FWA type of devices it can be assumed that fullpower mode0 is supported, on the other hand for the devices supporting full-coherent precoders only (such as Rel-15 DL type 1 codebooks), full power modes are irrelevant. It can be discussed after the 8Tx codebook design becomes clear.

Following issues should be further discussed:

PTRS-DMRS association indication when rank>4

Impact on full power modes

Simulation assumptions and results

Simulation assumptions

To evaluate the performance of 8Tx UL transmission a set of simulation assumptions in Table1 can be considered. Assuming coherent cross-pole linear array for both 8Tx and 4Tx, where Rel-15 DL type-1 codebook is assumed for 8Tx and Rel-15 full coherent codebook subset is used for 4Tx evaluation. Other detailed assumptions can be found in Table 1.

Table 1: System level simulation assumptions

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | TDD, OFDM |
| Multiple access | OFDMA |
| Scenario | Urban Micro |
| Frequency Range | 4GHz |
| Antenna setup and port layouts at gNB | (2,8,2,1,1,2,8), (dH, dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 4TX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
|  | 8TX: (1,4,2,1,1,1,4), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power | 46 dBm |
| cell receiver noise figure | 5dB |
| Modulation | Up to 64QAM |
| Numerology | 14 OFDM symbol slot |
|  | SCS 15KHz, 50 PRB |
| Frame structure | All uplink |
| MIMO scheme | SU-MIMO with rank adaptation |
| Transmission rank | maxRank = 4 (single CW) maxRank = 8 (two CWs) |
| SRS configuration | Periodicity: 10 ms |
|  | Scheduling delay: 4 ms |
| Traffic model | Full buffer |
| UE distribution | 80%indoor, 20%outdoor |
| Receiver | MMSE-IRC |
| Codebook configurations | Non-codebook: eigenvector-based Rel-15 UL codebook: full-partial-non Rel-15 DL Type 1 codebook: L=1 |
| Precoder granularity | Full bandwidth |

For link level simulation, assumptions in Table 2 can be considered.

Table 2: Link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 4 GHz |
| Channel | CDL-A, DS=100ns |
| BWP size | 52 PRBs |
| Waveform | CP-OFDM |
| SCS | 15 kHz |
| BS antenna configuration | (Mg,Ng,M,N,P) = (1,1,2,2,2), dH=0.5 |
| UE antenna configuration | (Mg,Ng,M,N,P) = (1,1,2,2,2), dH=0.5 |
| gNB antenna port | 8 ports |
| UE antenna port | 8 ports |
| Codebook configurations | Non-codebook: eigenvector-based Rel-15 UL codebook: full-partial-non Rel-15 DL Type 1 codebook: L=1 |
| Precoder granularity | Full bandwidth |
| SRS periodicity | 50ms |
| PUSCH mapping | Type A, 14 symbols |
| MCS | 16QAM, code rate=0.5 |
| Layer | Adaptive Rank |
| Channel estimation | 2D-MMSE filter |
| Receiver | MMSE |

Evaluation results

According to the assumptions in Table 1, we provide initial evaluation results comparing the performance of eigenvector-based precoder for 4Tx and 8Tx. Performance of 4Tx based on Rel-15 codebook with full-partial-non-coherent codebook subset and 8Tx based on Rel-15 DL type1 codebook are also evaluated. In the simulation, both UE and gNB are equipped with dual-polarized linear array. Maximum rank of 4 with rank adaption is assumed.

Figure 1 shows the evaluation results, where 17.65% improvement of mean SE is observed with 8Tx compared to 4Tx eigenvector-based precoder, while 13.74% performance gain can be seen for 8Tx type 1 codebook against 4Tx Rel-15 codebook. Besides, compared to 8Tx eigenvector-based precoder, type 1 codebook has 7.3% performance loss. For cell-edge users, 21.74% performance gain can be seen for 8Tx eigenvector-based precoder compared to 4Tx. 8Tx type 1 codebook-based transmission has 20.93% performance gain against Rel-15 4Tx codebook. Compared to 8Tx eigenvector-based precoder, type 1 codebook has 8.56% performance loss.

Performance evaluation with 4Tx and 8Tx, maxRank=4, Umi

Considerable performance gain can be seen for 8Tx codebook-based and non-codebook-based transmission.

8Tx based on type 1 codebook has 7.3% and 8.56% performance loss compared to eigenvector-based transmission for mean SE and cell-edge users respectively.

Conclusions

In this contribution, we have the following observations and proposals:

- Considerable performance gain can be seen for 8Tx codebook-based and non-codebook-based transmission.

8Tx based on type 1 codebook has 7.3% and 8.56% performance loss compared to eigenvector-based transmission for mean SE and cell-edge users respectively.

- The supported scenarios and antenna assumptions should be settled in the very first meeting for the group to work in focused manner.

Two SRI fields corresponding to two SRS resource sets for non-codebook transmission can be considered to simplify the SRI indication.

Codebook constructed by two 4Tx precoders indicated by two TPMI fields can be considered for partial and none coherent antenna assumption.

DL type1 codebook can be considered for fully coherent antenna assumption.

Two CWs can be considered for rank>4.

Following issues should be further discussed:

PTRS-DMRS association indication when rank>4

Impact on full power modes

References

RP-213598, New WID: MIMO Evolution for Downlink and Uplink, RAN#94e

R1-2203544, “Views on DMRS enhancement”, RAN1#109-e, vivo

R1-2203545, “Views on SRS enhancement”, RAN1#109-e, vivo