3GPP TSG RAN WG1 #109-e                                                           R1-2203549

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	General discussions on AI/ML framework

Agenda Item:	9.2.1

Document for:	Discussion and Decision

Introduction

A new SI “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface” [1] was approved for Rel-18 and the objectives are as follow:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment, model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

- In this contribution, we provide some initial considerations on the general aspects of AI/ML framework.

Arrangement for AI/ML discussions over air interface

Focuses for each topic

Based on the SID and chair’s arrangement, discussion of AI/ML over air interface falls in the following aspects:

Framework for application of AI/ML over air interface

Evaluation for each use case

Spec impact and sub use case down-selection

For framework related discussion, as pointed out in the SID, some background information is needed for a common understanding on the conclusion of AI/ML for RAN. Based on common understanding from background discussion, specific aspects or considerations for AI/ML over air interface should also be discussed. Besides all these aspects, one additional issue may also need some discussion in the general aspects part: some common evaluation assumptions across multiple use cases, e.g., whether field data should be used, KPIs for evaluation of each use case, etc.

General evaluation methodology related aspects, e.g., whether field data should be used, or KPIs for all use cases should be clarified in a common agenda item.

For evaluation of each use case, the detailed evaluation assumption should be clarified based on progress on the other agenda item. Detailed KPIs should be discussed there, e.g., which KPI is used for performance evaluation, RSRP, spectral efficiency or some other intermediate results should be used.

For spec impact and sub use case down-selection, the discussion should focus on sub use case clarification considering different companies may have different understandings. Based on input from multiple companies, the sub use cases may need further consolidation and proper categorization. Comparison between different sub use cases for various KPIs should also be conducted in this agenda to do the sub use case down-selection. In the later phase of the study, the related specification impact for each sub use case is discussed in this agenda.

Discussion of each topic on AI/ML over air interface in RAN1 should be arranged and focused on the following aspects:

- For discussion over framework for application of AI/ML over air interface, focus on

  - Background information collection for terminology alignment

  - Framework considerations for AI/ML for Air interfaces

  - [General evaluation methodology discussion]

- For discussion over evaluation for each use case, focus on

  - Evaluation assumption for each use case/sub use case

  - Necessary KPIs for each use case

  - Evaluation results collection and comparison

- For discussion over spec impact and sub use case down-selection, focus on

  - Sub use case collection and categorization

  - Expected impact for each sub use cases

  - Down-selection of each sub case

Time budget and work plan

The time budget approved at RAN#95e is shown below [2]:

Table 2-1: Time budget allocation

| Date | 2Q'2022 | 3Q'2022 | 4Q'2022 |  | 1Q'2023 | 2Q'2023 |  | 3Q'2023 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Meeting | RAN1 #109 | RAN1 #110 | RAN1 #110bis | RAN1 #111 | RAN1 #112 | RAN1 #112bis | RAN1 #113 | RAN1 #114 |
| TUs | 2 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |

RAN1 #111 is an important checkpoint, since it has been determined to finalize representative subuse cases for each use case for characterization and baseline performance evaluations by RAN#98. To achieve consensus in RAN1 #111, the work for RAN1 #109, RAN1 #110, RAN1 #110b, RAN1 #111, should be arranged in the following way: evaluation assumptions for each use case/sub use case should be consolidated as much as possible before RAN1 #110. Starting from RAN1 #110, evaluation results should be collected and compared between companies for each use case/sub use case.  Framework discussion should be in parallel to help companies align on the conceptual framework of AI/ML application over air interface.

Consider the following arrangement for the work plan in 2022:

Evaluation assumptions for each use case/sub use case should be consolidated as much as possible before RAN1 #110;

Starting from RAN1 #110, evaluation results should be collected and compared between companies for each use case/sub use case;

Framework discussion should be in parallel to help companies align on the conceptual framework of AI/ML application over air interface.

An example time plan is shown in Figure 2-1.

Figure 2-1: An example time plan in 2022

High-level principles for AI/ML discussion over air interface

During the FS_NR_ENDC_data_collect SI phase discussion in RAN3, a set of high-level principles was formulated, which was applied for AI-enabled RAN intelligence [3]. However, some of them are not fully applicable to AI for air interface, the main reasons include:

The AI/ML functionalities (i.e., model training and model inference) are only deployed at the RAN node or OAM. No AI/ML functionality locates on the UE side, thus no definition of AI/ML capability.

The multi-vendor interoperability of details of model deployment and model update was not fully studied. As a consequence, the details of model deployment and model update were not studied.

Detailed machine learning algorithms and machine learning frameworks were also not fully discussed. But for current SI, as described in NR_AIML_Air SID, discussion on these issues would be inevitable.

Table 2-2: Consideration of high-level principles in RAN3

| High-level principles for FS_NR_ENDC_data_collect SI | Applicable to NR_AIML_Air SI discussion |
| --- | --- |
| The detailed AI/ML algorithms and models for use cases are implementation specific and out of RAN3 scope. | - Not fully applicable, RAN1 should study AI/ML related algorithms and associated complexity. - For performance evaluation, the evaluation details would need to be properly exposed to other companies. |
| The study focuses on AI/ML functionality and corresponding types of inputs/outputs. | Not fully applicable. The study would focus on the justification of performance gains in the initial phase before we can further study the specification impact. |
| The input/output and the location of the Model Training and Model Inference function should be studied case by case. | Yes |
| The study focuses on the analysis of data needed at the Model Training function from Data Collection, while the aspects of how the Model Training function uses inputs to train a model are out of RAN3 scope. | Not fully applicable. How the model is trained is within the scope of RAN1 discussion and should be well considered. |
| The study focuses on the analysis of data needed at the Model Inference function from Data Collection, while the aspects of how the Model Inference function uses inputs to derive outputs are out of RAN3 scope. | Not fully applicable. How model inference is done is related to RAN1 discussion on UE capability, complexity/latency/performance evaluation, etc. |
| Where AI/ML functionality resides within the current RAN architecture, depends on deployment and on the specific use cases. | Yes |
| The Model Training and Model Inference functions should be able to request, if needed, specific information to be used to train or execute the AI/ML algorithm and to avoid reception of unnecessary information. The nature of such information depends on the use case and on the AI/ML algorithm. | Yes |
| The Model Inference function should signal the outputs of the model only to nodes that have explicitly requested them (e.g., via subscription), or nodes that take actions based on the output from Model Inference. | Yes |
| An AI/ML model used in a Model Inference function has to be initially trained, validated and tested by the Model Training function before deployment. | Based on the deployment scenario, it is possible that the initial model can be further fine-tuning after deployment |
| NG-RAN SA is prioritized; EN-DC and MR-DC are down-prioritized, but not precluded from Rel.18. | Not relevant |
| Functional framework and high-level procedures defined in this TR should not prevent from “thinking beyond” them during normative phase if a use case requires so. | Yes |
| User data privacy and anonymization should be respected during AI/ML operation. | Yes, but how to ensure the user data privacy is out of RAN1 scope |

Some of the high-level principles introduced for FS_NR_ENDC_data_collect SI can also apply to current NR_AIML_Air SI while other principles are not applicable. The main reasons why they are not applicable include:

The AI/ML functionalities (i.e., model training and model inference) discussed during  FS_NR_ENDC_data_collect SI are only deployed at the RAN node or OAM. No AI/ML functionality locates on the UE side, thus no definition of AI/ML capability.

The multi-vendor interoperability of details of model deployment and model update was not fully studied.

Detailed machine learning algorithms and machine learning frameworks were not discussed. But for NR_AIML_Air SI discussion and evaluation, these would be inevitable

- In our understanding, RAN1 should further discuss and update the high-level principles, to be in line with the requirements of the use cases and model lifecycle management.

The following high-level principles from FS_NR_ENDC_data_collect SI can be used as starting point for RAN1 discussion. These high-level principles can be further updated in RAN1.

Table 2-3: High-level principles from FS_NR_ENDC_data_collect SI

| Index | High-level principles |
| --- | --- |
| Principle 1 | The input/output and the location of the Model Training and Model Inference function should be studied case by case. |
| Principle 2 | Where AI/ML functionality resides within the current RAN architecture, depends on deployment and on the specific use cases. |
| Principle 3 | The Model Training and Model Inference functions should be able to request, if needed, specific information to be used to train or execute the AI/ML algorithm and to avoid reception of unnecessary information. The nature of such information depends on the use case and on the AI/ML algorithm. |
| Principle 4 | The Model Inference function should signal the outputs of the model only to nodes that have explicitly requested them (e.g., via subscription), or nodes that take actions based on the output from Model Inference. |
| Principle 5 | Functional framework and high-level procedures defined in this TR should not prevent from “thinking beyond” them during normative phase if a use case requires so. |
| Principle 6 | User data privacy and anonymization should be respected during AI/ML operation. |
| Principle 7 | Terminologies related to machine learning algorithm and framework need to be aligned between companies for discussion for the use case of AI in air interface. |

Another important topic that is needed for RAN1 discussion is regarding what it means by “specific AI/ML models are not expected to be specified and are left to implementation”. The interpretation of the description means that model itself is not specified within 3GPP. But the ways for different sides to align the models used are within the scope of the study. The work could start from RAN1. Although signaling design is within RAN2 scope, it should also be considered in RAN1 since the understanding of how the model is transferred between multiple parties would be useful for the evaluation of the tradeoff between performance and overhead.

Ways of aligning models between multiple parties are within scope of 3GPP RAN study. The study can be started from RAN1 for evaluation of performance and overhead.

Terminologies and background information

To facilitate RAN1 discussion, some terminologies and background information should be aligned between companies. In the TR 37.817 of RAN3 [3], the terminologies of AI/ML functionality were captured as follows.

- Data collection: Data collected from the network nodes, management entity or UE, as a basis for AI/ML model training, data analytics and inference. - AI/ML Model: A data driven algorithm by applying machine learning techniques that generates a set of outputs consisting of predicted information and/or decision parameters, based on a set of inputs - AI/ML Training: An online or offline process to train an AI/ML model by learning features and patterns that best present data and get the trained AI/ML model for inference. - AI/ML Inference: A process of using a trained AI/ML model to make a prediction or guide the decision based on collected data and AI/ML model.

As analyzed above for high-level principles, RAN3 did not study the detailed machine learning algorithms and machine learning framework. However, algorithm and framework will impact evaluation assumptions, performance comparisons, KPI assessments, or even interactions between nodes and the model lifecycle management of AI in air interface. Therefore, RAN1 shall further study and capture the terminologies of machine learning algorithms and frameworks that are essential for the use case of AI in air interface.

To facilitate evaluation assumption, KPI assessment and specification impact discussion, RAN1 should further study and capture the essential terminologies of machine learning algorithm and framework for AI in air interface.

There are three typical machine learning algorithms, i.e., supervised learning, unsupervised learning, and reinforcement learning.

Supervised learning: supervised learning is a machine learning task that aims to learn a mapping function from the input to the output, given a labeled dataset. Input data is called training data and has a known label or result. Supervised learning can be further grouped into Regression and Classification problems. Classification is about predicting a label and Regression is about predicting a quantity.

Unsupervised Learning: unsupervised learning is a machine learning task that aims to learn a function to describe a hidden structure from unlabeled data. Input data is not labeled and does not have a known result. Some examples of unsupervised learning are: K-means clustering, principal component analysis (PCA) and nonlinear independent component analysis (ICA).

Reinforcement Learning: reinforcement learning is a machine learning which utilizes the environment's feedback and rewards to continue optimizing decisions and get maximum accumulated rewards.

Supervised learning is normally considered for the classification and regression of input data, where the label of input data is essential together with data to train the learning algorithm. Each data should be marked with its label and fed into ML training. Therefore, if supervised learning is considered, the model training shall collect the training data together with the label.

Unsupervised learning is usually integrated with either supervised learning or reinforcement learning in wireless communication networks. While it can also be used independently to understand the similarity across different network entities or scenarios based on the collected data.

For the air interface, the environment in reinforcement learning may refer to the measurements related to channel state, interference, throughput, latency, etc. Those measurements should be fed into the training node, as well as the rewards. Thus, the model training shall collect the training data together with the feedback and reward.

Based on where the training nodes are located, there are four types of machine learning frameworks, i.e., centralized learning, distributed learning, federated learning and transfer learning.

Centralized Learning: Centralized learning refers to an AI/ML framework which requires all training data collected by different nodes to be reported to a centralized node and the centralized node is responsible for model training.

Distributed Learning: Distributed learning refers to the concept in which machine learning processes have been scaled out and deployed across a cluster of nodes. The training model is split up and shared among multiple simultaneously working nodes to speed up model training.

Federated Learning: Federated learning is a distributed machine learning framework that allows a collective model to be constructed from data that is distributed across data owners. It brings AI/ML models to the data source, rather than bringing the data to the model, allowing the local nodes/individual devices to collect data and train their own copy of the model, thus no need to report source data to the centralized node. In federated learning, only parameters/weights of AI/ML model need to be sent back to the centralized node to assist generic model training.

Transfer Learning: Transfer Learning is a machine learning method that reuses a pre-trained model as the starting point for a model on a new task. A model trained on one task is repurposed on a second, related task as an optimization that allows rapid progress when modeling the second task.

In the centralized learning framework, training is managed by a centralized network node. The required information such as training data and performance feedback can be collected from various entities. For distributed learning, each UE or each NG-RAN node trains its own model. For distributed learning, ML models or related information are exchanged and integrated between distributed entities.

Federated learning is a framework in between, where both centralized training and distributed learning are included within the framework. One of the benefits of federated learning is that training information can be reported securely by distributed nodes without exposing distributed nodes’ privacy. The model parameter and related model update information need to be transmitted so that the central ML model can be updated based on the aggregated information and such central ML model can further be deployed into distributed UE or RAN node.

For transfer learning, the training functionality could use a pre-trained model as the input. This involves the model transfer between different functionalities.

The above only lists several high-level algorithms as a starting point. Based on discussion input from companies, more detailed algorithms can also be captured

Capture in TR the following terminologies of machine learning, including:

Supervised learning, unsupervised learning, reinforcement learning

Centralized learning, distributed learning, federated learning, transfer learning.

Discussion on collaboration levels

There are various levels of collaboration between UE and gNB and they have large impacts on the AI/ML algorithm design and also specification design. From system impact perspective, use cases can be categorized as follows.

Level 0: No collaboration. AI/ML related training or inference are all conducted at one side of network or UE and is transparent to the other side.

Level 1: Partial and implicit collaboration. AI/ML related training or inference are conducted at one side of network or UE, but requires additional signaling or procedure enhancements between two sides, potentially with existing signaling framework. Additional information is not directly related to training and inference, e.g., capability and new patterns. Training or inference is done by implementation and not explicitly seen in the specification.

Level 2: Coarse collaboration. AI/ML related training or inference is conducted at one side of network or UE, with assistance information exchanged between two sides. E.g., neural network models are provided to UE as a reference for fine-tuning or provided to the network for aggregation.

Level 3: Close collaboration. AI/ML related training or inference are conducted together at both sides of network and UE. Information related to inference or training need to be exchanged between both sides. The AI model at one side should match the AI model at other side. In other words, the training, updating, and inference of the AI model at one side need the assistance of the AI model at the other side.

Figure 4-1: Example levels of collaboration between UE and gNB

For the training stage and the inference stage, the collaboration levels need to be considered separately. There will be mismatch between inference collaboration level and training collaboration level. High inference collaboration level may be supported by a low training collaboration level, and the opposite is also true. For example, UE supports inference collaboration level 3 and uses the trained model at gNB for CSI prediction, but only supports training collaboration level 1, that is, UE will not do online training and the AI model is provided by gNB. In another example, also for CSI prediction, UE supports inference collaboration level 1 and detects the new pattern of CSI-RS with reduced overhead, but supports training collaboration level 2 and uses the suggested training hyper-parameters from gNB, which are suitable for the wireless environment nearby.

Based on listed collaboration levels, all the 3 sub cases have different sub use cases for each inference collaboration level, which can be seen in Table 4-1.

Table4-1: Representative sub use cases for each use case and each inference collaboration level

|  | CSI | Beam | Positioning |
| --- | --- | --- | --- |
| Level 0: No collaboration | CSI report with time prediction: based on the CSI report of UE, gNB could predict the future channel. | Beam prediction in time domain: similar to CSI prediction, gNB could predict the future beam quality based on the beam report of UE. | AI/ML based OTDOA estimate at the UE with training and inference transparent to the NW |
| Level 1: Partial and implicit collaboration | RS overhead reduction for channel estimation, e.g., CSI-RS or SRS. | Beam prediction in spatial/time domain: UE measures the qualities of small number of beam pairs and then estimates the qualities of all beam pairs or the possible best beam pairs. New format of beam report is sent by UE or UE knows the detailed information of beam pairs on the measured RSs. | Additional gNB antenna information/calibration information can be provided to UE to improve OTDOA estimate accuracy based on AL/ML. |
| Level 2: Coarse collaboration | CSI Prediction with AI model from gNB: gNB sends AI model to UE and UE uses this AI model to predict the future channel. | Beam prediction in spatial/time domain with AI model from gNB: gNB sends AI model that matches its beam pattern and wireless environment to UE. UE use this AI model to find the possible best beam pairs. | Neural network trained/aggregated at network for the environment are distributed to UE to expedite the training at the UE. |
| Level 3: Close collaboration | CSI Payload overhead reduction: That is, UE encodes the channel information by AI model to generate PMI. Then gNB uses the matched AI model to decode PMI. | Beam compression in spatial/time domain with AI based encoder and decoder: UE uses the AI model to compress the beam information, which can be decoded by gNB. | Inference at UE side to extract features and reported to the network that would further be used by NW for inference. |

Table 4-2 lists diferernt training collaboration levels, as well as possible sub use case for CSI for each training collaboration level.

Table 4-2: Different training collaboration levels

|  | Training stage | Sub use cases for CSI |
| --- | --- | --- |
| Level 0: No collaboration | The AI model is trained only by UE or gNB. | CSI report with time prediction or channel compression, where the AI model is trained only by UE or gNB. |
| Level 1: Partial and implicit collaboration | New signaling for UE capabilities, or new RS patterns. | New signaling for UE capabilities, or new RS patterns, for CSI prediction or CSI compression. |
| Level 2: Coarse collaboration | Training hyper-parameters exchange between UE and gNB. | Cell-specific CSI prediction or CSI compression, with training hyper-parameters exchange between UE and gNB. |
| Level 3: Close collaboration | Periodic training information exchange between UE and gNB, e.g. gradient or loss function results. | Continuing model updating of CSI prediction or CSI compression with periodic training information exchange between UE and gNB. |

All the listed collaborations are necessary and need to be studied in the SI. The main reasons are discussed in the following.

The requirements of specific sub use cases are different. For example, without the highest collaboration level, CSI Payload overhead reduction is not possible.

In some cases, only one side has valuable information. For example, for CSI prediction in FDD system, only UE has the original downlink channel information.

The requirements of different learning frameworks are different. Some learning frameworks can not work only in one side and then need a high-level collaboration level. One representative is federated learning, which needs the exchange of parameters/weights of AI/ML model from UE to gNB.

The specification impacts of these collaboration levels need extra consideration, since the AI algorithms, signaling procedure and report configuration of different training collaboration levels and inference collaboration levels will be different.

For the training stage and the inference stage, the collaboration levels need to be considered separately

The collaboration levels for inference can be categorized into the following 4 levels from system impact perspective.

Level 0: Inference with no collaboration. AI/ML related inference are all conducted at one side of network or UE and is transparent to the other side.

Level 1: Inference with partial and implicit collaboration. AI/ML related inference are conducted at one side of network or UE, but requires additional signaling or procedure enhancements between two sides, potentially with the existing signaling framework.

Level 2: Inference with coarse collaboration. AI/ML related inference is conducted at one side of network or UE, with assistance information exchanged between two sides, such as AI model information.

Level 3: Inference with close collaboration. AI/ML related inference are conducted together at both sides of network and UE, and the output of one side can be the input of the other side.

Training collaboration levels can be categorized into the following 4 levels from system impact perspective.

Level 0: Training with no collaboration. AI/ML related training are all conducted at one side of network or UE and is transparent to the other side.

Level 1: Training with partial and implicit collaboration. AI/ML related training are conducted at one side of network or UE, but requires additional signaling or procedure enhancements between two sides, potentially with the existing signaling framework.

Level 2: Training with coarse collaboration. AI/ML related training is conducted at one side of network or UE, with assistance information exchanged between two sides, such as training hyper-parameters.

Level 3: Training with close collaboration. AI/ML related training are conducted together at both sides of network and UE, with periodic training information exchange between two sides.

Conceptual framework of AI/ML application over air interface and lifecycle management

Almost every building block of the communication link can apply AI/ML to improve performance. The following examples from the initial study show performance gains:

CSI feedback with lower overhead to achieve higher system efficiency

Positioning performance improvement under various challenging conditions

RS overhead reduction and channel estimation performance improvement, including DMRS/CSI-RS/SRS

Intra-cell/Inter-cell beam management latency and accuracy improvement

Channel prediction to improve system performance in high mobility scenarios

Mobility enhancement to improve UE experience during handover

To facilitate AI/ML application over air interface, it is preferred to have a unified design for all these different use cases.

Study unified future-proof framework/procedure to facilitate AI/ML application over air interface.

RAN3 has already done some studies for the functionality framework of AI/ML in RAN. The functional framework for RAN intelligence is illustrated in Figure 5-1.

Figure 5-1: Functional Framework for RAN Intelligence in RAN3

Compared with this functional framework, some additional aspects need to be considered (which we also discussed previously):

- UE capabilities/status alignment between multiple parties: there are various devices in the market with vastly different computation power and implementation, UE capability/status needs to be indicated to the network appropriately to allow different kinds of implementation; the procedure should be designed in such a way that the two sides do not expose unnecessary implementation details to the other side;

- Different learning frameworks: AI/ML community has developed quite a few effective learning frameworks. All these learning frameworks deserve study for the use cases to be studied in Rel-18. The unified framework design should be flexible enough to allow free exchange of information between multiple parties, which are determined necessary to facilitate different learning frameworks, e.g., federated learning, reinforcement learning, transfer learning, etc. An example of model lifecycle management procedure is given for reference in appendix A. In the following figure, we also give one example lifecycle management procedure for Federated Learning, a typical scenario is that the RAN node acts as the server and the terminal as the client. In conclusion, All these different learning frameworks should be carefully studied based on different use cases.

Figure 5-2: An example lifecycle management procedure for Federated Learning

- Different network entities: for RAN3 study, the AI/ML functionalities (i.e., model training and model inference) are only deployed at RAN node or OAM. For air interface use cases, the functionalities may reside in UE side. Moreover, for positioning use cases, some core network entities are also involved. These would require the unified design applicable for these use cases with different entities involved. Example locations of the AI functionality are shown in Table 5-1.

Table 5-1: Location of AI functionality

| Use case | Data Collection | Model Training | Model Inference |
| --- | --- | --- | --- |
| CSI feedback enhancement | gNB | OAM/gNB | Decode: gNB Encode: UE |
| Beam management | gNB | OAM/gNB | gNB |
| Positioning | LMF | LMF | LMF or UE |

- Different collaboration levels: it has been widely discussed that there are different collaboration levels involved in the study, from purely implementation-based solutions to close collaboration between multiple parties. For different use cases, the appropriate collaboration level would be different. These different collaboration levels would also be different for training stage and inference stage. The unified procedure or framework should facilitate all the collaboration levels deemed necessary.

Figure 5-3: Additional aspects to be considered for the unified framework

Based on the above discussion, we have the following proposal.

Study procedures that allow UE to report its status for conducting computation resources and latencies.

Study interactions between different entities involved in different use cases for the unified framework/procedure for AI/ML over air interface. Send LS to RAN2/RAN3/SA2 on the high-level understanding from RAN1 discussion when the framework/procedure is stable.

Study lifecycle management for various learning frameworks deemed necessary from use case studies, e.g., the lifecycle management procedure for federated learning, transfer learning, etc.

Lifecycle management under different collaboration levels

As analyzed above, the characteristics of Model Training and Model Inference for the different collaboration levels are as follows:

Table 5-2: Model Training and Model Inference for the different collaboration levels

| Collaboration Levels | Model Training | Model Inference |
| --- | --- | --- |
| Level 0: No collaboration. | Conducted at one side only, i.e., network or UE, and is transparent to the other side. | Conducted at one side only, i.e., network or UE, and is transparent to the other side. |
| Level 1: Partial and implicit collaboration | Conducted at one side only, but requires additional signaling or procedure enhancements between two sides, potentially with the existing signaling framework. | Conducted at one side only, but requires additional signaling or procedure enhancements between two sides, potentially with the existing signaling framework. |
| Level 2: Coarse collaboration | Conducted at one side only, with assistance information exchanged between two sides, such as training hyper-parameters. | Conducted at one side only, with assistance information exchanged between two sides, such as AI model information. |
| Level 3: Close collaboration | Conducted at both sides of network and UE, with training information exchange between two sides. | Conducted at both sides of network and UE, and the output of one side can be the input of the other side. |

Taking Level 3 as an example, we will then briefly analyze the model lifecycle procedure, as shown in the following figure.

Figure 5-4: Model lifecycle management for Level 3 collaboration

At the very beginning, the network may send an initial model to the UE, upon which the UE can perform further fine-tuning. Afterward, the network and the UE collect training data separately and perform model training to generate feasible models. During the model inference phase, both sides need to collect data for model inference, Typically, the model inference output of the UE can be the input of the network side for model inference.

In general, the applicable collaboration levels and corresponding model lifecycle management procedures may be different for different use cases. Therefore, we propose:

Study lifecycle management for different collaboration levels case by case.

Lifecycle management with different model maintenance granularities

One of the key issues for lifecycle management is how often the model needs to be updated. In this section, we have some preliminary analysis on the granularities of model update.

In most cases, the parametric model defines a distribution p(y | x;w) and we simply use the principle of maximum likelihood. This means we use the cross-entropy between the training data and the model’s predictions as the cost function, as described

Jw=-E x,y~pdatalogpmodely | x;w

where y is the input data vector, x is the known data vector (or label), w is the coefficient vector or the weight vector, acquired by the training procedure, all in a given AI neural network.

It is worthwhile noting that, the training set associated with any input pair of (y,x) can be expressed as

(y,x)∈yn,xnn=1N

In such a procedure, accordingly, the AI model can be trained by means of the off-line training manner under the condition of the statistic wireless channel model and can be considered as a universal AI model for any UE or gNB uses.

However, the channel factors influenced by gNB are comparatively stable, while the channel factors influenced by UE are unpredictable, with respect to the antenna direction and location. In addition, the channel model utilized for performance evaluation mainly refers to TR 38.901 [4], where the long-term channel factors such as receive antenna field patterns (i.e., AoA and ZoA), receive antenna location vector, transmit antenna field patterns (i.e., AoD and ZoD), and transmit antenna location vector update statically, while the short-term channel factors such as Doppler frequency update dynamically. Consequently, therefore, a universal AI model purely trained by a statistic wireless channel model may be not feasible in terms of the complexity of neural network and the overall AI-based system performance. Somewhat UE assistance mechanism in addition to cell-based training model may be necessary.

Thanks to the unique wireless channel behaviors, we believe that the training set can be possibly divided into K training subsets relying on the long-term statistic channel parameters. If we assume that the k-th subset is associated with the parameter of zk, the training set can be represented as

yn,xnn=1N=k=1KYk,Xk | zk

where z can be seen as the assisted parameter vector, z=z1,z2,⋯,zK, and the k-th training subset can be expressed as

Yk,Xk=ynk,xnknk=1Nk;			for	  k=1KNk≥N.

If the subset Yk,Xk and subset Yi,Xi are completely independent, i.e., Yk,Xk∩Yi,Xi=∅, for k≠i, and the distribution p(y | x;w) associated with the parameter of zk is approximated as

p(y | x;z;w)≅k=1Kpyk | xk;zk;w

Then, the cross-entropy between the training data and the model’s predictions can be

Jw=-E x,y~pdatak=1Klogpmodelyk | xk;zk;w

If the parameter vector of z is given, the cross-entropy in the training procedure for the parametric model with the pre-known zk can be individually represented as

Jwk|zk=-E xk,yk~pdatalogpmodelyk | xk;zk;wk                                    Eq. 1

where k=1,2,⋯,K, and K is the total number of training models.

It is worthwhile noting that, the AI models can be trained by means of offline manner and utilized by each UE accordingly. This does imply that each AI-model can be seen as a sub model, and the K sub models form a cell-specific AI model which can be operated by all the UEs if connected with the corresponding gNB.

As one specific example, by geographically dividing the network area, AI models associated with different areas can be distinguished and the related tasks associated with AI models would be limited. This results in the improvement of the accuracy and effectiveness of the AI model, and the reduction of the complexity of AI neural networks. To achieve the above purpose, the network may perform regional division of geographic coordinates through a zone identification (i.e., Zone-ID). The network determines the network coverage area related to the maximum communication range Dmax according to the geographic location of the gNB, which is further divided into multi-zones represented by Zone-ID. As illustrated in Figure 5-5, the size of each zone with W×L is configurable according to the use-cases and the deployed scenarios, where L is the zone length and W is the zone width. During the AI model training procedure, the training dataset can be distinguished by the Zone-ID in the network coverage area. Therefore, the trained AI model behaves the characteristics of the zone indicated by Zone-ID.

Therefore, the training procedure for the parametric model with the pre-known parameter, zk, can be individually trained as formulated in Eq. 1, where zk is the Zone-ID within the maximum communication range Dmax.

It is worthwhile noting that this type of AI model training process can be completed by either the UE or by the network.

Figure 5-5: Schematic diagram of distinguishing AI models based on geographic information

By dividing different orientations of the network, alternatively, AI models associated with different orientations from gNB can be distinguished and the related tasks associated with AI models can be limited. This also results in the improvement of the accuracy and effectiveness of the AI model and the reduction of the complexity of AI neural networks.

As illustrated in Figure 5-5, the area covered by the network is divided into K orientations (or azimuths), and each orientation forms a pie-shaped directional sub area, denoted by zn,k, where n is the ID of the gNB and k is the subregion-orientated index. Optionally, the widths of the pie-shaped sub regions formed by the orientation of each sub region could be the same or different and determined by high-level configuration. More specifically, each sub region orientation can be regarded as an orientated beam (i.e., directional beamforming), where the orientated beam width is 2π/K. During the AI model training procedure, if the gNB or UE can roughly acquire the geographic location of the UE or the AoA/DoA associated with the gNB, the AI training dataset can be distinguished by the orientation of each sub region. In such a case, the gNB or UE only uses the data related to the orientation of the sub region to train the AI model, which behaves the orientation features.

Therefore, the training procedure for the parametric model with the pre-known parameter, zn,k, can be individually trained as formulated in Eq. 1, where zn,k is the subregion-orientated index within the maximum communication range Dmax.

It is worthwhile noting that this type of AI model training process can be completed by either the UE or by the network as well.

Figure 5-6: Schematic diagram of distinguishing AI models based on direction information

Based on the above preliminary analysis, we propose:

Study lifecycle management for different granularities of model training and update.

Discussion on UE capabilities

Computation power at the UE side is growing fast in recent years. Figure 6-1 shows the AI capability of NPU in mobile phones in recent years. The capacity of one typical NPU used in current mobile phone is 22.5T operations (OPs) per second. One OP denotes one addition or one multiplication. From 2017Q1, the capacity of typical NPU in mobile phone is growing very fast year by year. This trend is expected to continue for the coming years.

Figure 6-1: The growing capacity of NPU in mobile phone

In this section, we provide some tested results for typical models and UE chipsets. The following aspects are considered.

Latency. One part of latency comes from the inference of AI model, which is unavoidable. If AI models run in other hardware modules, such as NPU, GPU, or FPGA, there is extra latency of data exchange between the AI module and the non-AI module. In the initial test, we focus on the inference latency.

Complexity. Complexities that UE can handle within several milliseconds are another valid metric from UE implementation perspective. It should be noticed that there are quite a lot of different implementations for the same model thus the expected model complexity should be explicitly stated when under comparison.

Power consumption. Power consumption is relevant for good user experience. Different modules that are responsible for neural network related operations would have different power consumption, e.g., NPU typically consumes the least while CPU consumes the most. When compared, this should also be clarified.

Typical UE performance for latency and complexity

Typical physical layer modules have strict requirements for latency. The latency of AI/ML operation should be within several milliseconds, otherwise, the AI model would not be applicable for air interface use cases. Since latency is highly correlated with complexity, they are discussed together in this subsection. For this important issue, we have collected some latency information from the area of image processing. Figure 6-2 shows the inference performance of typical AI models for image and video in typical chipsets. The latency of the AI models in Figure 6-2 is about 0.9ms~5.1ms. The AI models for air interface would be much simpler than the listed AI models and the latency of AI models for air interface will be much smaller. Then AI models for air interface would be likely to meet the latency requirement.

Figure 6-2: The latency of typical AI models for image and video in typical chipsets.

Initial test of typical models for latency on typical chipsets in Figure x shows that the latency for neural network operation latency on UE are within the range of interest for air interface applications.

It is known that complex model would achieve better performance than simple model. There will be a tradeoff between complexity and performance. Using a very complex model for a simple task is not befitting. For the use case study, if companies could provide results for complexities and latencies for the models used in the contributions, it is very helpful for aligning the views of UE capabilities.

The complexities of AI models would be easily obtained by the API of AI/ML platform. Floating-point operations per second (FLOPs) could be used the KPI of complexity. The latency could be calculated using the capabilities of hardware and the complexities of AI models. Also, companies are encouraged to provide the latency information of model update and data transfer.

Companies are encouraged to provide results in the following table for complexities and expected latencies (under certain base chipset computation power assumption) for the models used for each use case.

Table 6-1: The complexities of AI models from companies

|  | Company 1 | Company 2 | Company 3 | … |
| --- | --- | --- | --- | --- |
| AI Model 1 |  |  |  |  |
| AI Model 2 |  |  |  |  |
| AI Model 3 |  |  |  |  |
| … |  |  |  |  |
| … |  |  |  |  |

Table 6-2: The latencies of AI models from companies

|  | Company 1 | Company 2 | Company 3 | … |
| --- | --- | --- | --- | --- |
| AI Model 1 |  |  |  |  |
| AI Model 2 |  |  |  |  |
| AI Model 3 |  |  |  |  |
| … |  |  |  |  |
| …. |  |  |  |  |

Power consumption

Power consumption is one of the key parameters in current chipsets and much work has been done for power saving. If AI models largely increase power consumption, the commercialization of AI algorithm in air interface would be not a good deal.

The actual power consumptions of typical AI models are listed in Table 6-3. From the discussion of power saving, the UE power consumption model for FR1 is shown in Table 6-4, in which the basic unit would be assumed as 5 mA. It is seen that the power consumptions of complex AI models are comparable with typical physical layer operations.

Table 6-3: The actual power consumptions of typical AI models in typical chipsets.

|  | Electron current (mA) |
| --- | --- |
| AI Model 2 (1.14 GOPs) | 291 |
| AI Model 4 (11.5 GOPs) | 420 |

Table 6-4: UE power consumption model for FR1 from the discussion of power saving

| Power state | Relative power |
| --- | --- |
| Deep sleep | 1*5 (Optional: 0.5) |
| Light sleep | 20*5 |
| Micro sleep | 45*5 |
| PDCCH-only | 100*5 |
| SSB or CSI-RS proc | 100*5 |
| PDCCH+PDSCH | 300*5 |
| UL | 250*5 (0dBm) 700*5 (23dBm) |

Power consumption for typical neural network operation on typical chipsets are at the same level of power consumption as for SSB or CSI-RS processing or PDCCH decoding.

Performance of model is not directly related to FLOPs, but specifically tuned for models. Even with the similar FLOPs, the performance may be drastically different. For example, as shown in Figure 6-3, the complexity of AI model 1 is 77.2% of AI model 2, and then the expected latency of AI model 1 is 77.2% of AI model 2. However, the actual latency of AI model 1 is 152% of AI model 2 in Chipset 2. For another example, the complexity of AI model 2 is 9.9% of AI model 4, while the power consumption of AI model 2 is 69.3% of AI model 4.

Figure 6-3: The complexity and latency comparison between AI models.

Even with the similar FLOPs, the performance of different models would be different for latency and power consumption.

Since power consumption is so important in this area, enough information of power consumptions for the AI models is very necessary. And then for the AI models used for each use case, companies are encouraged to provide results for power consumptions.

Companies are encouraged to provide results in the following table for power consumptions for the models used for each use case.

Table 6-5: The power consumptions of AI models from companies

|  | Company 1 | Company 2 | Company 3 | … |
| --- | --- | --- | --- | --- |
| AI Model 1 |  |  |  |  |
| AI Model 2 |  |  |  |  |
| AI Model 3 |  |  |  |  |
| … |  |  |  |  |
| … |  |  |  |  |

Quantization level

Float point is usually used in study and initial evaluation. Fixed point is usually used in implementation, where the parameters of AI model are transformed to integer value and the complexity could be reduced. Some kinds of hardware only support fixed point.

For float point or fixed point, there are also different levels of bits used for one number. It is clear that the overhead of 32 bits is twice of 16 bits, and 4 times of 8 bits. Considering the overhead reduction of model transfer, low quantization level would be better than high quantization level. The actual complexities of different quantization levels would be slightly different from the overhead, due to the practical hardware design and AI model structure.

Figure 6-4: The latency ratio of typical AI models of different quantization levels in one typical chipset, compared to CPU-FP32 of AI model 4

Figure 6-5: The power consumption ratio of typical AI models of different quantization levels in one typical chipset, compared to CPU-FP32 of AI model 4

Some evaluation results of different quantization levels are shown in Figure 6-4 and 6-5. INT8 denotes integer value with 8 bits, FP16 denotes float point value with 16 bits. The performance of NPU-INT8 is about 1.8 times of NPU-FP16, 4.8~17 times of GPU-FP32, and 4.5~17 times of CPU-FP32. INT8 is suitable for service with high complexity and power consumption, such as photographing and video. FP16 is suitable for service with high accuracy and low power consumption, such as pixel-wise image processing.

Quantization of the model has impacts on latency performance.

Quantization of the model has impacts on power consumption.

Since the quantization level has such large impact on the latency and power consumption of AI model, companies are expected to report the quantization level of the model used for better calibration.

Companies are encouraged to report the quantization level of the model used.

UE capability

As discussed above, there will be various kinds of UEs supporting different level of AI models with different latency performance and power consumption, as well as different collaboration levels as mentioned before. Different UEs with different capabilities should be considered in the same use case. Based on the discussion, we have the following proposal.

Study mechanisms of allowing different UEs with different implementations/capabilities to serve same use case, e.g., by defining flexible capability exchange mechanisms

Discussion on common evaluation methodologies for three use cases

In this section, common evaluation methodologies for all use cases are discussed, including field data, intermediate results, generalization performance, ONNX and simulation calibration.

Field data

Statistical models in 38.901 and 38.857 are designed based on the physical characteristics of wireless signals and the measurements in practical wireless environment. Statistical models have been verified and updated by 3GPP for recent years, and their performance and stability are satisfactory. Moreover, the simulation data from statistical models could be conveniently generated as many as use case evaluation demands.

Field data is collected from practical wireless systems and include the actual wireless features. Since AI models in commercial products should match the practical wireless environment, field data is necessary for the commercial stage. However, in 3GPP SI, field data has several disadvantages compared to simulation data. If field data is collected only in a few times and places, the variety of the samples could be less than simulation data and then the performance of the trained AI models could be decreased. Then it will take great effort to collect enough field data samples of high quality.

Field data should be evaluated before usage. That is, the features of field data should be analyzed, otherwise, the field data may not serve the purpose of justification for a specific proposal. This is also time-consuming and may not be fruitful.

From the above discussion, it is better to have the dataset for 3GPP evaluations constructed based on statistical models. If there are companies who are willing to provide field data for performance tests, it can also be used as a reference as long as the conditions for data collection are clearly documented and the statistics of the data are clearly analyzed.

Dataset for 3GPP evaluation is mainly constructed based on statistical models defined in 38.901 and 38.857. Besides, the field data test results can be used as a reference if they are justified and well documented.

Generalization performance

Generalization or overfitting is one of the key issues in AI/ML and should be seriously considered. With overfitting, the model is memorizing the features of the training set, rather than learning how to generalize to unseen samples. As a consequence, an AI model works well for training datasets but its performance on verification datasets would be unacceptable. The generalization performance (or test performance) of AI model is affected by the AI model structure, the variety of training dataset and the training strategy. It is better to keep the training loss to be an accurate approximation of the generalization loss uniformly for all hypotheses.

Generalization performance can be evaluated by constructing a suitable verification dataset. First, when generating verification dataset, large scale parameter perturbation could be used to increase the difference between training data and verification data. Second, since system level model contains more practical wireless features than link level model, system level models in 38.901 and 38.857 should mainly be considered for verification.

Different levels of generalization may need to be verified. For example, whether the performance maintains from one UE to another, from one cell to another, from one drop to another, or from one scenario to another.

Consider the following different levels of generalization performance for performance evaluation.

Generalization performance from one UE to another

Generalization performance from one cell to another

Generalization performance from one drop to another

Generalization performance from one scenario to another

Simulation calibration

To align the performance gain for each sub use case between different companies, simulation calibration is important and should be done in the first two RAN1 meetings.

Previously, MIMO evaluations are conducted typically without exposing absolute spectral efficiency. There may potentially be a large difference between companies on the baseline performance. It would be difficult for companies to align on whether AI/ML have gains if there are large differences on the baseline performance.

To alleviate such misaligned understanding on the baseline performance, some intermediate results can be considered for performance evaluation. For example, the results of spectral efficiency for CSI feedback enhancement depend significantly on the design of scheduler and MU pairing, which would make the performance results from different companies drastically different. If intermediate results such as cosine similarity are provided, the chances for companies to align the results would be much higher, which could be helpful for the group to achieve consensus on the final conclusions.

Some example intermediate results are given below.

Table 7-1: Example intermediate results for each use case

| Use case | Example intermediate results |
| --- | --- |
| CSI compression | Cosine similarity of reconstructed channels and original channels |
| CSI prediction | Cosine similarity of predicted channel and real channels |
| Positioning | MSE or MAE of TOAs |
| Beam | MSE or MAE of RSRPs |

Consider intermediate results for performance comparison between companies.

Dataset is another aspect that influences the aligned understanding on the performance of AI/ML over air interface. If the dataset for AI/ML model training is different for different companies, it would be impossible for companies to be aligned on the performance of AI/ML. There are two ways on the table for companies to be aligned on this issue:

Alt1: Provide details as much as possible for the generation of datasets.

Alt2: Directly provide publicly accessible dataset for training and testing.

It is preferable to go with Alt1+Alt2 since this would resolve the misalignment between companies to the largest extent.

For better simulation calibration, our datasets of each use case have been uploaded in [5]-[8].

It is encouraged for companies to provide publicly accessible datasets for training and testing for cross-checking purposes. Our datasets of each use case have been uploaded in [5]-[8].

Yet another aspect is model training and design aspect. If companies have different structures of models or different ways of training, the AI/ML performance would also be different. AI model should thus also be calibrated. Several levels of AI model alignment could be considered.

- At least for the baseline performance calibration, a simple and fixed model could be used for each sub use case. The full-connected network is one of the simplest AI models that could be used for this purpose. Companies could align on the hyper-parameters used for the fully connected models.

- For the second level, selected and recommended models for evaluations for applied areas could be collected from company inputs. The provided models include both model structures and ways of training.

- For the third level, companies could directly provide the model itself in a typical fileformat together with above mentioned publicly accessible dataset. Other companies could use the model and dataset together to cross check the performance. Currently, there are plenty of AI/ML frameworks, such as tensorflow, pytorch and caffe. Each AI/ML framework has its own model storage format and can not load the model storage format of other AI/ML frameworks, for example, ‘.h5’ format used for tensorflow and ‘.pth’ used for pytorch. How to align the models between different AI/ML frameworks should be considered if the model itself is published to the public. Similar model alignment procedure has been conducted in video coding community (MPEG and JVET) for evaluation purposes. Observations from JVET shows that conversion to/from ONNX (Open Neural Net Exchange) can be done from various framework. It was agreed to use ONNX format for model description for companies' evaluation of model performance( ‘https://jvet-experts.org/ ’).

Figure 7-1: The agreement of using ONNX format for model description in JVET

It is encouraged for companies to provide model description files in pre-defined file format together with the publicly accessible dataset for training and testing for cross-checking purposes.

AI model size

AI model size is also another important aspect that influences evaluation performances. An upper limit for the evaluation should be set thus companies can compare the performance in a fair way.

Consider setting up an upper limit for model size for a fair comparison between companies. 1~10Mega parameters size can be considered.

Discussion on datasets

One of the key elements for AI/ML evolution is large amount of data to extract useful features. To have an effective model trained for real-world problems, quite a few aspects need to be taken into account.

The quality of datasets needs to be considered. The only criteria to determine whether a specific dataset satisfies the expectations is whether the real-world problem can be effectively resolved. The following table shows some initial evaluation results for different scenarios for CSI compression. It can be seen that for a target scenario InH, if the training dataset is only constructed by the data collected from UMi, then the performance of the model would be poor. It is also true the other way around. If the dataset is properly adjusted to include more data from InH, then the performance would be much better for the target scenarios. If the representativeness is not good enough the performance of the trained model would not be useful for the target scenarios.

Table 8-1: The impact of different data constructions for CSI compression

Dataset construction would influence the performance of trained models.

Study how to construct a representative dataset (including matching between training and inference) for real-world problems for each use case/sub use case.

One of the possible ways to have a representative dataset is to collect real-world data and mix the data with pre-defined synthetic dataset. Such mixture of dataset can be used to train a model with good generalization performance. To collect data from real world, there are several ways as below. These different ways of data collection would have different overheads and latencies.

- Direct collection of data.

  - The data is collected at one entity and then exchanged between multiple entities.

  - Overhead of this collection would be large since size of the raw data would be huge.  Sometimes direct collection of data would be of very high cost, if possible. Rewarding in actors for reinforcement learning can also be viewed as direct collection of data. Overhead of such data collection would be relatively small.

  - Latency of this way of data collection can be small if the data is collected in real time very few samples interval.

- Collection of processed data or data characteristics.

  - For example, distribution information for the channel can be collected and exchanged between entities. Based on the characteristics, the data is re-generated at different entities.

  - Overhead of the collection can be small but may cause performance degradation if not properly designed.

  - Latency of this way of data collection would be dependent on how many samples are needed for the compression.

Study different ways of dataset construction from overhead and latency perspective.

Aligned understanding between two sides would be needed to use the collected data. For example whether the CSI-RS channel information is in frequency domain or time domain, or whether some de-noise function is needed or not. Dependent on how far the specification of AI/ML over air interface can go, it may be allowed to have several ways of pre-processing for the collected data for one sub use case, or only one fixed preprocessing is allowed. If fixed reference input is specified, the data collection may focus on those reference point where the specification is defined. But if for a specific use case, different preprocessings are allowed, it would be necessary to be aligned on how the data is preprocessed and where the reference point is for the data collection. The detailed formats for the reference point should be clarified.

Study how to align the reference point for data collection between different parties.

The interactions between data collection and data training would also need to be considered. It is possible that dataset is constructed first by synthetic data mixed with some real-world data. Then after validating the model in the real-world and based on the validation performance, more data can be collected. The model is finetuned further with the newly collected data. Based on further iterations, the models can be further finetuned.  Further data collection is also possible if more data is collected.

When deployed for use, the model performance can always be monitored, based on monitoring, more data/failure analytics can be ollected for learning from the analytics.

Figure 8-1: Iterative procedures for data collection and data training.

Study data collection requirements in different stages of the model generation and finetuning.

For different use cases, the entities involved in the data collection would be different. The collected data would need to be transferred to those entities responsible for training. Thus properly choosing the involved entities could save the effort of moving data from/to different entities. Currently, UEs, gNBs, LMF, NWDAF, OAM are all possible options. Some of the interactions between these entities may go beyond the scope of RAN1. But common understanding of what these interactions are like may well impact RAN1 discussion on the framework and use cases.

Study options for interactions between different entities for data collection, e.g., the interactions between UE, gNB, LMF, NWDAF, etc. Send LS to RAN3 and SA to ask the feasibility of these options.

Different UE capabilities would be needed for the expected pre-processing, data storage, feature extraction and report for data collection. Some UEs may have the ability to collect large amount of data while others would be limited. Depending on the reference point definition, some UEs may leave some interfaces for collection in the corresponding reference point while others may not. These should also be discussed in UE capability.

Study ways for UE to report its capabilityfor data collection.

Conclusions

- General evaluation methodology related aspects, e.g., whether field data should be used, or KPIs for all use cases should be clarified in a common agenda item.

Discussion of each topic on AI/ML over air interface in RAN1 should be arranged and focused on the following aspects:

- For discussion over framework for application of AI/ML over air interface, focus on

  - Background information collection for terminology alignment

  - Framework considerations for AI/ML for Air interfaces

  - [General evaluation methodology discussion]

- For discussion over evaluation for each use case, focus on

  - Evaluation assumption for each use case/sub use case

  - Necessary KPIs for each use case

  - Evaluation results collection and comparison

- For discussion over spec impact and sub use case down-selection, focus on

  - Sub use case collection and categorization

  - Expected impact for each sub use cases

  - Down-selection of each sub case

Consider the following arrangement for the work plan in 2022:

Evaluation assumptions for each use case/sub use case should be consolidated as much as possible before RAN1 #110;

Starting from RAN1 #110, evaluation results should be collected and compared between companies for each use case/sub use case;

Framework discussion should be in parallel to help companies align on the conceptual framework of AI/ML application over air interface.

An example time plan is shown in Figure 2-1.

- Some of the high-level principles introduced for FS_NR_ENDC_data_collect SI can also apply to current NR_AIML_Air SI while other principles are not applicable. The main reasons why they are not applicable include:

The AI/ML functionalities (i.e., model training and model inference) discussed during  FS_NR_ENDC_data_collect SI are only deployed at the RAN node or OAM. No AI/ML functionality locates on the UE side, thus no definition of AI/ML capability.

The multi-vendor interoperability of details of model deployment and model update was not fully studied.

Detailed machine learning algorithms and machine learning frameworks were not discussed. But for NR_AIML_Air SI discussion and evaluation, these would be inevitable

The following high-level principles from FS_NR_ENDC_data_collect SI can be used as starting point for RAN1 discussion. These high-level principles can be further updated in RAN1.

Ways of aligning models between multiple parties are within scope of 3GPP RAN study. The study can be started from RAN1 for evaluation of performance and overhead.

To facilitate evaluation assumption, KPI assessment and specification impact discussion, RAN1 should further study and capture the essential terminologies of machine learning algorithm and framework for AI in air interface.

Capture in TR the following terminologies of machine learning, including:

Supervised learning, unsupervised learning, reinforcement learning

Centralized learning, distributed learning, federated learning, transfer learning.

For the training stage and the inference stage, the collaboration levels need to be considered separately

The collaboration levels for inference collaboration levels can be categorized into the following 4 levels from system impact perspective.

Level 0: Inference with No collaboration. AI/ML related inference are all conducted at one side of network or UE and is transparent to the other side.

Level 1: Inference with Partial and implicit collaboration. AI/ML related inference are conducted at one side of network or UE, but requires additional signaling or procedure enhancements between two sides, potentially with the existing signaling framework.

Level 2: Inference with Coarse collaboration. AI/ML related inference is conducted at one side of network or UE, with assistance information exchanged between two sides, such as AI model information.

Level 3: Inference with Close collaboration. AI/ML related inference are conducted together at both sides of network and UE, and the output of one side can be the input of the other side.

Training collaboration levels can be categorized into the following 4 levels from system impact perspective.

Level 0: Training with No collaboration. AI/ML related training are all conducted at one side of network or UE and is transparent to the other side.

Level 1: Training with Partial and implicit collaboration. AI/ML related training are conducted at one side of network or UE, but requires additional signaling or procedure enhancements between two sides, potentially with the existing signaling framework.

Level 2: Training with Coarse collaboration. AI/ML related training is conducted at one side of network or UE, with assistance information exchanged between two sides, such as training hyper-parameters.

Level 3: Training with Close collaboration. AI/ML related training are conducted together at both sides of network and UE, with periodic training information exchange between two sides.

Study unified future-proof framework/procedure to facilitate AI/ML application over air interface.

Study procedures that allow UE to report its status for conducting computation resources and latencies.

Study interactions between different entities involved in different use cases for the unified framework/procedure for AI/ML over air interface. Send LS to RAN2/RAN3/SA2 on the high-level understanding from RAN1 discussion when the framework/procedure is stable.

Study lifecycle management for various learning frameworks deemed necessary from use case studies, e.g., the lifecycle management procedure for federated learning, transfer learning, etc.

Study lifecycle management for different collaboration levels case by case.

Study lifecycle management for different granularities of model training and update.

Initial test of typical models for latency on typical chipsets in Figure x shows that the latency for neural network operation latency on UE are within the range of interest for air interface applications.

Companies are encouraged to provide results in the following table for complexities and expected latencies (under certain base chipset computation power assumption) for the models used for each use case.

Power consumption for typical neural network operation on typical chipsets are at the same level of power consumption as for SSB or CSI-RS processing or PDCCH decoding.

Even with the similar FLOPs, the performance of different models would be different for latency and power consumption.

Companies are encouraged to provide results in the following table for power consumptions for the models used for each use case.

Quantization of the model has impacts on latency performance.

Quantization of the model has impacts on power consumption.

Companies are encouraged to report the quantization level of the model used.

Study mechanisms of allowing different UEs with different implementations/capabilities to serve same use case, e.g., by defining flexible capability exchange mechanisms

Dataset for 3GPP evaluation is mainly constructed based on statistical models defined in 38.901 and 38.857. Besides, the field data test results can be used as a reference if they are justified and well documented.

Consider the following different levels of generalization performance for performance evaluation.

Generalization performance from one UE to another

Generalization performance from one cell to another

Generalization performance from one drop to another

Generalization performance from one scenario to another

Consider intermediate results for performance comparison between companies.

It is encouraged for companies to provide publicly accessible datasets for training and testing for cross-checking purposes. Our datasets of each use case have been uploaded in [5]-[8].

It is encouraged for companies to provide model description files in pre-defined file format together with the publicly accessible dataset for training and testing for cross-checking purposes.

Consider setting up an upper limit for model size for a fair comparison between companies. 1~10Mega parameters size can be set as the upper limit for load and performance.

Dataset construction would influence the performance of trained models.

Study how to construct a representative dataset (including matching between training and inference) for real-world problems for each use case/sub use case.

Study different ways of dataset construction from overhead and latency perspective.

Study how to align the reference point for data collection between different parties.

Study data collection requirements in different stages of the model generation and finetuning.

Study options for interactions between different entities for data collection, e.g., the interactions between UE, gNB, LMF, NWDAF, etc. Send LS to RAN3 and SA to ask the feasibility of these options.

Study ways for UE to report its capabilityfor data collection.

References

RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

RP-221019, Status of time budgets for RAN1/2/3/4 ongoing and new WIs/SIs after RAN #95e.

TR 37.817, Study on enhancement for Data Collection for NR and EN-DC.

3GPP TR 38.901 V17.0.0.

vivo, “Dataset for AI CSI feedback”, https://commonbox.vivo.xyz/s/VkhgUFG2hhd.

vivo, “Dataset For AI CSI Prediction”, https://commonbox.vivo.xyz/s/1qv4tjQ5efk.

vivo, “Dataset for beam management”, https://commonbox.vivo.xyz/s/gMEadbdyFtd.

vivo, “Dataset for AI Positioning”, https://commonbox.vivo.xyz/s/UQnWAcqp2DL.

Appendix A: Exemplary model lifecycle management procedure

An example of the model lifecycle management procedure is shown in Figure A-1.

Figure A-1: model lifecycle management procedure

Step 1: The Data Collection function provide essential input data to Model Training;

Step 2: The Model Training function performs the AI/ML model training, validation, and testing after data preparation;

Step 3: If the Model Inference locates at UE side, the UE shall provide the AI related capability to Model Training;

Step 4: The Model Training function sends the trained, validated, and tested AI/ML model to the Model Inference function;

Step 5: The Data Collection function provide essential input data to Model Inference;

Step 6: The Model Inference performs data preparation and provides AI/ML model inference output;

Step 7: If the Actor and the Model Inference are not in the same location, the Model Inference sends the output to Actor;

Step 8: The Actor may feedback the essential information that can be used by the Model Training to monitor the performance of the AI/ML Model and the feedback can be transferred via Data Collection;

Step 9: The Model Inference may feedback model performance that can be used by the Model Training to monitor the performance of the AI/ML Model and the feedback can be transferred via Data Collection;

Note: Steps 8 and 9 are not strictly in order of sequence.

Step 10: The Data Collection function provide essential input data to Model Training for model fine-tuning;

Step 11: The Model Training function performs the model fine-tuning, validation, and testing after data preparation;

Step 12: The Model Training function sends the updated, validated, and tested AI/ML model to the Model Inference function. The Model Training may also release the deployed model for some reasons, e.g., the model fine-tuning failed.

Appendix B: Introduction of ONNX

ONNX (Open Neural Net Exchange, ‘https://onnx.ai/’) aims to support a common intermediate representation for AI models transmission for both deep learning and machine learning. It is developed as a common data format but not an AI framework or a development tool, so it can be compatible with any AI framework including tensorFlow, pytorch and so on. In the design of ONNX, model structure and weights are sequenced by Protobuf. It defines an extensible computation graph with nodes with operators and handles all weights as inputs or outputs. It also defines the standard data types.

In ONNX, the computation graph is composed of some nodes and each node has several inputs and outputs. All the tensors are identified by its name. The same input name of node A and output name of node B means node A and B are connected. All weights are also identified by their names and corresponding to some nodes as inputs or outputs. Then the computation graph is constructed with the input name and the output name of each node.

With the help of ONNX, All developers can choose their own tools to develop their models and load other models in different framework. And now, ONNX is supported in many frameworks, tools and even some hardwares officially. Since ONNX does not impose restriction on operators, the same construct or function can be transformed to different combinations of nodes for different developers. All developers can have their specific transition code, which means it can be enhanced further to support other destinations like security.