3GPP TSG RAN WG1 #109-e                                                           R1-2203551

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Other aspects on AI/ML for CSI feedback enhancement

Agenda Item:	9.2.2.2

Document for:	Discussion and Decision

Introduction

It has been agreed that a new study item will be launched in Release 18 towards artificial intelligence (AI)/machine learning (ML) for NR air interface. Specifically, the initial set of use cases are identified to include: 1) CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction; 2) beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement; 3) positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions. Furthermore, representative sub use cases within each use case will be finalized for characterization and baseline performance evaluations. Details could be referred as follows [1]:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project.

Therefore, in this contribution, we would present our views on the aforementioned issues, with a focus on representative sub use cases and the corresponding specification impacts for CSI feedback enhancement.

Finalize representative sub use cases for CSI feedback enhancement

In this section, we first overview the possible sub use cases for CSI feedback enhancement, and then present our suggestions on the sub use case list together with the reasons.

Overview of possible sub use cases for CSI feedback enhancement

Following possible sub use cases for CSI feedback enhancement are provided.

    - AI/ML for spatial-frequency domain CSI compression

Spatial-frequency domain CSI compression is one of the most typical CSI feedback use cases for FDD DL MIMO channels, and Rel-16 Type-II CSI feedback scheme could be considered as a general baseline. For AI/ML based CSI compression, though AI/ML could be in either UE or gNB (but not both), the dual-sided autoencoder-like architecture is more common and typical, where one encoder model deployed at UE and one decoder model deployed at gNB are jointly trained to compress and reconstruct CSI as presented in Figure 1.

Figure 1: The block diagram of the AI/ML for spatial-frequency domain CSI compression.

Note that despite we refer this use case to “spatial-frequency domain CSI compression”, the input of the ML models could be either the spatial-frequency domain CSI or its equivalent transformation, such as angle-delay domain CSI. A brief introduction to various channel representations could be referred to Appendix A.

The benefits of AI/ML for spatial-frequency domain CSI compression are the potentially improved accuracy and/or reduced overhead. Various evaluation results have demonstrated that ML-based spatial-frequency CSI compression outperforms those based on Type-I and eType-II codebook in terms of tradeoff between accuracy and overhead (More detailed evaluation results as well as the evaluation assumptions could be referred to our companion contributions in [2]). The disadvantages of AI/ML based CSI compression include the introduced signalling overhead for data collection, model transmission, and model alignment between gNBs and UEs. Meanwhile, the additional UE power consumption and the potential latency caused by model inference should also be considered.

Observation 1: AI/ML for spatial-frequency domain CSI compression is the most typical sub use case, where the autoencoder architecture (AI/ML models are deployed at both gNB and UE side and operate jointly) could be considered as a baseline for study.

    - AI/ML for temporal-spatial-frequency domain CSI compression

Temporal-spatial-frequency domain CSI feedback takes the temporal correlation across multiple CSI-RS instances into consideration in addition to the conventional spatial-frequency domain CSI compression. Due to the need to joint process multiple CSI-RS instances, the basic CSI feedback framework has to be re-considered in temporal-spatial-frequency domain CSI feedback, and the typical eType-II CSI feedback could no longer serve as the baseline. One potential candidate scheme is to feedback the difference of spatial-frequency domain CSI on two slots and the gNB reconstructs the current CSI based on both the received CSI difference and the previously obtained reference CSI, as shown in the following Figure 2.

Figure 2. An illustration of temporal-spatial-frequency domain CSI feedback

One of the major benefits of AI/ML for temporal-spatial-frequency domain CSI compression is to further reduce the overhead compared with spatial-frequency domain CSI feedback. However, there are a series of challenges in the implementation: 1) higher data collection overhead, as the temporal relationship among multiple CSI instances should be labelled; 2) requiring more complex models to learn the latent correlation in temporal-spatial-frequency domain; 3) more efforts in 3GPP, since there is no common baseline for temporal domain CSI compression.

Observation 2: Due to the exploitation of correlations in temporal domain CSI, AI/ML for temporal-spatial-frequency domain CSI compression offers more performance gain at the cost of more complex CSI feedback framework and AI/ML models.

2.1.3 AI/ML for CSI prediction

In AI-based CSI prediction, the AI model is designed to derive the prediction of future CSIs as the output of model when using the historical CSIs as the input. The block diagram of AI-based CSI prediction is illustrated in Figure 3.

Figure 3. The block diagram of AI-based CSI prediction

As shown in our evaluation contributions [2], the AI-based prediction achieves significant gain over the non-prediction case (i.e., the case using the nearest historical CSI as the prediction). The more accurate prediction will lead to better precoding, which is promising to result in higher spectral efficiency and throughput. Therefore, the AI-based CSI prediction will introduce significant improvement for the wireless communications.

The AI-based CSI prediction can be conducted at both the gNB and UE. As summarized in Table 1, if the AI-based CSI prediction is conducted at gNB, the input of the CSI model is the CSI feedback information, and the output of AI model can be directly used for precoding. If the AI-based CSI prediction is conducted at UE, the input of the AI model can be the estimated channel or its eigen vector (or singular vector), the output of AI model should be further feedback to gNB for precoding.

Table 1. Comparisons of potential implementations of AI-based CSI prediction.

| Network location for conducting the AI-based CSI prediction | Benefits | Disadvantages |
| --- | --- | --- |
| gNB | The predicted CSI can be directly used for precoding. | The input of AI model is with distortion. High-precision label is hard to be acquired. |
| UE | There is no information loss for the input of AI model, so that the prediction accuracy is high. It is easy to collect the labeled data for training or finetuning. | The feedback of predicted CSI will reduce the prediction accuracy. The feedback of predicted CSI may require additional feedback overhead. |

Furthermore, CSI prediction could serve as a typical sub use case to study the performance and specification impact of finetuning. For AI-based CSI prediction, the label of data can be collected in real time by the CSI measurement. Therefore, if the predicted future CSI is on the time occasion of a CSI measurement, finetuning is available for the AI-based CSI prediction. By finetuning, scenario-specific model can be derived using a few online collected data, which can further improve the prediction performance in the real system.

The following details should be considered for the AI-based CSI prediction:

- The procedure of gNB-based case and the UE-based case;

- The impact of the type, number and the time indices of historical CSI inputs;

- The impact of the number of predicted future CSIs;

- The generalization aspects.

Observation 3: AI/ML for CSI prediction could achieve more than 9dB NMSE gain over the non-prediction case (i.e., the case using the nearest historical CSI as the prediction).

Suggestions on the sub use case list

Based on the above overview, our suggested list of sub use cases for study are as follows:

- AI/ML for spatial-frequency domain CSI compression

- AI/ML for Temporal-spatial-frequency domain CSI compression

- AI/ML for CSI prediction

Furthermore, there are following proposals on the priorities of each item:

- Proposal 1: Study spatial-frequency domain CSI compression with high priority.

- Proposal 2: Study AI/ML for CSI prediction with high priority.

Our reasons are presented below:

1) AI/ML for spatial-frequency domain CSI compression is one of the most typical use cases with clear baseline and potential performance gain. In addition, the representative auto-encoder architecture of CSI feedback models requires gNB and UE to interact information and align configurations through air interface, which incurs explicit specification impacts (the highest gNB-UE calibration level). Therefore, it is necessary for 3GPP to study this sub use case with high priority.

2) Although temporal-spatial-frequency domain CSI compression could further reduce the overhead by exploiting the correlations in temporal CSI instances, it is challenging to finish the study within this study item, as reaching an agreement on temporal domain CSI compression baseline would consume too much efforts. From our perspective, it is better to focus on the most typical spatial-frequency domain CSI feedback and leave the exploitation of temporal domain correlations to future work.

3) Various evaluations show that AI-based prediction outperforms the non-prediction case. While AI-based CSI prediction can be conducted at both the gNB and UE, it is worth noting that UE-based CSI prediction is more beneficial than the gNB-based one due to the availability to the labeled data and less information loss in the historical CSIs. Besides, CSI prediction could serve as a typical sub use case to study the performance and specification impact of finetuning in wireless communications. Therefore, we propose to study the sub use case of AI/ML for CSI prediction with high priority.

Potential specification impact

In this section, potential specification impacts of AI/ML for CSI feedback enhancement are presented, where general issues are firstly investigated and then followed by case by case discussions.

General specification impact for the use case

Firstly, we present our opinions on the general specification impacts of CSI feedback enhancement.

1. Introduction of channel domain transformation in CSI feedback enhancement.

As presented in Appendix A, the channel in either angle-delay domain or delay-Doppler domain, in general, owns its unique sparsity or stability, then it is expected to represent a low-dimensional channel input and efficiently compress the channel so as to realize the CSI feedback with a small number of bits. With the transformation, however, an additional specification impact may occur between the transmitter (say UE) and the receiver (say gNB). This is because the AI-model dataset constructed for the training, validation and testing is dependent on what domain type of the input and output data is, and the dataset alignment between the UE and the gNB is necessary. In addition, somewhat complexity in both UE and gNB is required after all.

Furthermore, the time varying characteristic of the channel is important for channel prediction, which is most straightforward to be extracted from the time frequency representation. However, the time varying characteristics of channel are mainly caused by the Doppler effect. Thus, the delay-Doppler representation may bring some benefits for reducing the difficulty of feature extraction.

In this AI/ML SI, therefore, we need to carefully confirm whether the channel domain transformation can bring the additional performance gains to the AI-based CSI feedback system. Without the clear evidence, we believe that it had better keep using the legacy channel representation in time-frequency domain as much as possible.

Observation 4:	Introducing the channel domain transformation requires an additional specification impact and complexity.

2. Potential scenario-specific AI models.

Due to the vastly different channel characteristics in different scenarios, it could be challenging for a unified CSI feedback model to achieve good performance in all scenarios, where by “scenario” we include both the typical channel scenarios regularized in TR 38.901 (e.g., UMi, UMa, Indoor, etc.) and special channel scenarios such as the channel within one specific cell. In spite of the fact that initial evaluation results demonstrate that one unified model could achieve an acceptable performance in diversified scenarios (e.g., UMi and Indoor) if trained with proper data (i.e., collected in all these scenarios), cases would occur where qualified data is insufficient or even not available, which facilitates the necessity of scenario-specific models. If scenario-specific models are required in this sub-use-case, the potential specification impacts include the new signalling, assistance information, and procedure to collect scenario-specific data, select proper models, etc.

3. Adaption to multi-configuration, including bandwidth, number of ports, feedback payloads, antenna configurations, etc.

Since the input and output shape of a trained AI/ML model are usually fixed, it is challenging for one unified model to adapt to the multi-configurations of CSI, such as various bandwidth, number of ports, feedback payloads, antenna configurations, etc. The following approaches can be considered to address the above issue: 1) adopting multiple models, each of which adapts to one specific configuration; 2) applying pre-processing and/or post-processing to the input and/or the output of the model to enable one model adaptive to multi-configurations; 3) the integration of (1) and (2). Adopting multiple models requires to identify how to train, manage, transfer, and update these models in specifications, which is similar to the management of multiple scenario-specific models. Applying pre- and post-processing to the inputs and/or the output of the model requires to align the processing method (either explicitly describe the processing methods or identify corresponding signalling to align them).

Example 1: To enable one model adaptive to multiple CSI formats with different bandwidth and/or number of ports configurations in CSI feedback, one could transform the input CSI from spatial-frequency domain to angle-delay domain and then tailor CSI with different configurations into the same shape just as the pre-processing procedure in eType-II codebook. Given a proper designed training dataset (e.g., containing data samples collected from different bandwidth and/or number of ports configurations), it is possible to train one model adaptive to different bandwidth and/or number of ports configurations. The potential specification impact of the above solution is that the CSI transformation and tailoring method should be explicitly specified.

Example 2: For the adaption to different feedback payloads in CSI feedback, one could integrate post-processing on the output of one encoder (i.e., the codeword) and multiple decoders to address the issue, as presented in Figure 4. Specifically, there are only one encoder and multiple decoders in Figure 4, each of which receives only a proportion of the same feedback payloads (i.e., the first 177bits, 199bits, 223bits of total 223bits) to reconstruct the CSI. Note that multiple decoders together with the encoder could be jointly trained to achieve better performance. Since only the encoder is essential at UE side to generate the feedback payload, the above solution could significantly reduce the overhead of model transfer. To enable such approach, the corresponding signalling procedure indicating how to process the output of the encoder and which decoder to use should be identified.

Figure 4. Illustration of utilizing post-processing on the output of one encoder (i.e., the codeword) and multiple decoders to address the adaption to different payloads.

Proposal 3: Study the specification impact of utilizing multiple AI models to address issues of scenario (e.g., UMi, UMa, Indoor, etc.) and configuration adaption (e.g., bandwidth, number of ports, feedback payloads, antenna configurations, etc.) in AI/ML for CSI feedback enhancement, including:

- Procedure and signalling for scenario- and configuration-specific data collection;

- Procedure and signalling for model selection;

- Signalling to indicate the application scope of each model.

Proposal 4: Study the specification impact of pre- and post-processing approaches for model input/output to address the issue of adaption to multi-configurations (e.g., bandwidth, number of ports, feedback payloads, antenna configurations, etc.).

4. How to exchange models between UE and gNB.

Considering the very different capabilities of UEs and gNBs in computation and data collection, it is challenging for all entities to obtain the required model based on local resources. In addition, the encoder and decoder for CSI compression need to be trained together but deployed separately. Consequently, it is necessary for CSI feedback enhancement to exchange models between UE and gNB.

Basically, following issues should be addressed to fully support model exchange: 1) whether to come up with an official model exchange format; 2) where and how to allocate the required air interface resource for model exchange; 3) the corresponding control signalling. As model exchange is a common issue for all use cases in this study item (including AI/ML for beam management and positioning), we leave further detailed discussions on this topic to our contributions in [3].

Proposal 5: Study the specification impact of model exchange in AI/ML for CSI feedback enhancement.

5. UE capability reporting in AI/ML for CSI feedback enhancement

When UEs need to request AI/ML models from gNB side (e.g., request encoder in CSI compression or request models for CSI prediction), they are supposed to report their capabilities/status towards AI/ML models to help gNB select suitable models to meet the latency requirement of CSI feedback. As presented in our companion contributions [3], the capabilities/status of various UEs to run AI/ML models could be dramatically different. Therefore, a flexible UE capability reporting framework should be identified to be compatible with all possible UE types. Generally speaking, UE capabilities towards AI/ML models could be depicted via available computation resource, storage resource, power budget constraints, etc. For the convenience of reporting, UE capabilities/status could be assessed and rated via a series of reference models.

Proposal 6: Study the specification impact of reporting various UE capabilities/status towards AI/ML models for CSI feedback enhancement.

Detailed specification impact for AI/ML for spatial-frequency domain CSI compression

Towards AI/ML for spatial-frequency domain CSI compression, the following issues could have potential specification impacts.

1. Whether the whole model (encoder and decoder) or only the encoder is available at the UE side?

Basically, only the encoder is required at UE side to compress the CSI. Meanwhile, if the whole model (encoder and decoder) is available for UE, it would be more convenient to monitor the model performance (It is possible for UE with the whole model to report the real-time CSI reconstruction accuracy, which, however, is impossible for UE with only the encoder) and/or finetune the model based on newly-acquired data samples. Whether to deploy the whole model for CSI compression at the UE side depends on the specific UE capabilities (e.g., computation and storage resources). Therefore, it is suggested to additionally identify the following cases specific for CSI compression in UE capability/status reporting apart from the regular UE capability discussed in Section 3.1: 1) the whole model (encoder and decoder) is available at UE; 2) only the encoder is available at UE. In addition, approaches for model performance monitoring as well as the corresponding specification impacts regarding each case should also be studied. For example, consider utilizing both direct performance indicators (e.g., the cosine similarity between reconstructed channel and original channel) and intermediate indicators (e.g., whether the statistical characteristics of channels have dramatically changed) to monitor the model performance.

Proposal 7: Study the impact of the following cases in UE capability/status reporting for CSI compression: 1) the whole model (encoder and decoder) is available at UE side; 2) only the encoder is available at UE side.

Proposal 8: Study the specification impact of different model performance monitoring approaches in AI/ML for CSI compression.

2. AI-based quantizer vs. non-AI-based quantizer

How to design the quantizer (or quantization unit) from both performance and specification impact perspectives is an important issue in CSI compression. The main function of quantizer is to normalize the CSI feedback bits to be a realistic level so that the UE as a transmitter can directly reuse the legacy UCI format to convey the compressed CSI relative codebook to the gNB on the PUCCH or PUSCH.

The quantizer, nevertheless, can be plugged into the auto-encoder to realize so called end-to-end AI-model learning. This enables the AI neural network to learn all the features with respect to CSI compression and harvest the overall AI gain together. Here, we denote it as AI-based quantizer, which is illustrated in Figure 5.

Figure 5. The block diagram of AI-based CSI feedback neural network with AI-based quantizer.

The quantizer, alternatively, can be placed out from the auto-encoder so that the UE can offer a universal AI-model for CSI compression, and isolate the quantization function with the flexibility in feedback overhead design. This flexibility, however, is possibly at the cost of the degradation from overall AI performance. Here, we denote it as non-AI-based quantizer, which is illustrated in Figure 6.

Figure 6. The block diagram of AI-based CSI feedback neural network with non-AI-based quantizer.

Observation 5: The quantizer can be either plugged into the auto-encoder beneficially with end-to-end AI model training feature or placed out from the auto-encoder beneficially with the flexibility in feedback overhead design.

Proposal 9: Study the specification impact of both AI-based quantizer and non-AI-based quantizer for down-selection.

Detailed specification impact for AI/ML for temporal-spatial-frequency domain CSI compression

Towards AI/ML for spatial-frequency domain CSI compression, the following potential specification impact could be considered. Firstly, the specification impacts for AI/ML based spatial-frequency domain CSI compression described in section 3.2 still hold for the temporal compression sub use-case. In addition, due to the joint feedback of multiple temporal domain CSI instances, specifications need to identify the temporal domain window size, reference slot for CSI difference, pre- and post-processing approaches for CSI in each slot, compatibility mechanism towards conventional spatial-frequency domain CSI feedback, etc. Last but not least, the data collection framework for CSI feedback is also required to be extended, i.e., the temporal relations of collected CSI samples should also be reported.

Proposal 10: If time permits and the sub-use is justified, it can be allowed to study the specification impact of AI/ML for temporal-spatial-frequency domain CSI compression with priorities on the potential modifications on CSI feedback framework and data collection.

Proposal 11: If time permits and the sub-use is justified, it can be allowed to discuss temporal-spatial-frequency domain CSI compression based on the current CSI-RS framework.

Detailed specification impact for AI/ML for CSI prediction

For the gNB-based CSI prediction, the CSI feedback enhancement should be carefully designed to reserve Doppler information or time varying information as much as possible.

For the UE-based CSI prediction, the gNB may indicate the time-domain indices of the desired future CSIs to UE or the feedback information should contain the predicted time-domain indices.

- Proposal 12: Study the specification impact of both gNB- and UE-based CSI prediction.

The choice of the input (historical CSIs) and output (future CSIs to be predicted) of the CSI prediction should be studied. The candidate set of the type of input for AI-based CSI prediction can be: the raw channel, the eigenvector (or singular vector), and the CSI feedback information. The benefits and disadvantages of these candidate input are given in Table 2. Similarly, the type of output can also be chosen from raw channel and eigenvector. Furthermore, the number and time ID of historical CSIs and future CSIs to be predicted is important for the prediction.

Table 2 The benefits and disadvantages of candidate inputs

| candidate input | benefits | disadvantages |
| --- | --- | --- |
| Raw channel | Contains complete channel information | Only available at UE, not available by gNB; Overhead is large if the channel is feedback to the gNB. |
| eigenvector (or singular vector) | The time varying characteristic is still (maybe partially) reserved | The eigenvalue (or singular value) decomposition leads to irregular phase reversal; May lose some time varying characteristic |
| CSI feedback information | Available at gNB | The feedback procedure introduces heavy information loss and distortion. |

The input and output of CSI prediction should be aligned between the gNB and UE. This alignment includes the type (raw channel, eigen vector, CSI feedback), the number and the time ID of historical CSIs and future CSIs to be predicted. This may have further impacts on the data collection and CSI feedback.

Proposal 13: The input and output of CSI prediction should be aligned between the gNB and UE.

The choice of Per-UE model or Per-cell model have different impacts on the model training, model delivering and model validation process. Per-UE model achieves better performance at the cost of heavier overhead of model training and storage. The application of the Per-cell or Per-UE model should be decided by the real time validation. Furthermore, if the model needed to be delivered between gNB and UEs, Per-UE model should be delivered by a unicast way while the Per-cell model should be delivered by a broadcast way.

Proposal 14: The specification impacts of different levels of scenario specific models should be studied.

If the predicted future CSI is on the time occasion of a CSI measurement, finetuning is available for the AI-based CSI prediction. The finetuning parameters, e.g., the base model, learning rate, batch size and update number, should be specified or indicated.

Proposal 15: Study the finetuning process of AI-based CSI prediction.

Conclusions

The conclusions are summarized as follows:

Observation 1: AI/ML for spatial-frequency domain CSI compression is the most typical sub use case, where the autoencoder architecture (AI/ML models are deployed at both gNB and UE side and operate jointly) could be considered as a baseline for study.

Observation 2: Due to the exploitation of correlations in temporal domain CSI, AI/ML for temporal-spatial-frequency domain CSI compression offers more performance gain at the cost of more complex CSI feedback framework and AI/ML models.

- Observation 3: AI/ML for CSI prediction could achieve more than 9dB NMSE gain over the non-prediction case (i.e., the case using the nearest historical CSI as the prediction).

- Proposal 1: Study spatial-frequency domain CSI compression with high priority.

- Proposal 2: Study AI/ML for CSI prediction with high priority.

Observation 4:	Introducing the channel domain transformation requires an additional specification impact and complexity.

Proposal 3: Study the specification impact of utilizing multiple AI models to address issues of scenario (e.g., UMi, UMa, Indoor, etc.) and configuration adaption (e.g., bandwidth, number of ports, feedback payloads, antenna configurations, etc.) in AI/ML for CSI feedback enhancement, including:

- Procedure and signalling for scenario- and configuration-specific data collection;

- Procedure and signalling for model selection;

- Signalling to indicate the application scope of each model.

Proposal 4: Study the specification impact of pre- and post-processing approaches for model input/output to address the issue of adaption to multi-configurations (e.g., bandwidth, number of ports, feedback payloads, antenna configurations, etc.).

Proposal 5: Study the specification impact of model exchange in AI/ML for CSI feedback enhancement.

Proposal 6: Study the specification impact of reporting various UE capabilities/status towards AI/ML models for CSI feedback enhancement.

Proposal 7: Study the impact of the following cases in UE capability/status reporting for CSI compression: 1) the whole model (encoder and decoder) is available at UE side; 2) only the encoder is available at UE side.

Proposal 8: Study the specification impact of different model performance monitoring approaches in AI/ML for CSI compression.

Observation 5: The quantizer can be either plugged into the auto-encoder beneficially with end-to-end AI model training feature or placed out from the auto-encoder beneficially with the flexibility in feedback overhead design.

Proposal 9: Study the specification impact of both AI-based quantizer and non-AI-based quantizer for down-selection.

Proposal 10: If time permits and the sub-use is justified, it can be allowed to study the specification impact of AI/ML for temporal-spatial-frequency domain CSI compression with priorities on the potential modifications on CSI feedback framework and data collection.

Proposal 11: If time permits and the sub-use is justified, it can be allowed to discuss temporal-spatial-frequency domain CSI compression based on the current CSI-RS framework.

- Proposal 12: Study the specification impact of both gNB- and UE-based CSI prediction.

Proposal 13: The input and output of CSI prediction should be aligned between the gNB and UE.

Proposal 14: The specification impacts of different levels of scenario specific models should be studied.

Proposal 15: Study the finetuning process of AI-based CSI prediction.

Appendix A: A brief introduction to various channel representations

The representation of the channel, in general, can be described in various domains, such as spatial, time/Doppler, and frequency. The channel transformed from one to other domain can rely on, for instance, unitary matrix and DFT/IDFT operations. The channel represented in different domains may behave different characteristics, in terms of stability, sparsity, and spatiality. The represented channel can be mathematically classified into the channels in spatial, time/Doppler and frequency domains. In what follows, we describe the represented channel behaviors.

It is assumed that the OFDM channel consists of Nc sub-carriers in the OFDM symbol of T, in a MIMO channel, containing Nt transmit antenna ports and Nr receive antenna ports. Thus, the MIMO OFDM channel in spatial-time-frequency (S-TF) domain can be represented as

H=H0H1⋯HNc-1T

where Hc the Nr×Nt matrix channel in spatial domain, and c=0,1,⋯,Nc-1.

Mathematically, the channel in spatial domain can be transformed into the channel in angular domain, by means of the nt×nt unitary matrix of Ut on the transmit antenna ports, and the nr×nr unitary matrix of Ur on the receive antenna ports. If Hca denotes the channel in angle domain at the c-th sub-carrier, the transformation of channel in frequency-angle domain is expressed by

Ha=H0aH1a⋯HNc-1aT 		and		 Hca=Ur*HcUt

where Ux is the Nx×Nx unitary matrix defined as

Uxk,l=1Nxexp-j2πklNx

By operating the IDFT over Nc OFDM sub-carriers, simply formulated by the Nc×Nc matrix of FNc-1, the channel in angle-delay domain can be represented as

Had=FNc-1Ha

It is worthwhile noting that the channel Had in angle-delay domain is expected to offer a channel sparsity, which can be efficiently considered for the channel compression in AI-based CSI feedback system.

Furthermore, if we define that Hrt is the Nc×Nd matrix of the channel in time-frequency domain, on the t-th transmit antenna port, and the r-th receive antenna port, the channel in spatial-delay-Doppler domain can be represented as

Hdd=H11dd⋯H1Ntdd⋮⋱⋮HNr1dd⋯HNrNtdd			and			Hrtdd=FNc-1HrtFNd

where FNd is the Nd×Nd DFT matrix, and Nd is the number of samples in Doppler domain within the time interval of NdT, and t=0,1,⋯,Nt-1 and r=0,1,⋯,Nr-1.

It is worthwhile noting that the channel Hdd in spatial-delay-Doppler domain is formed by a block of fading channel with the time interval of NdT, in general, where the data symbols can be conveyed. It is expected to offer a channel stability within the time interval of NdT, but at the cost of the high latency of the CSI feedback. In such as case, the feedback CSI may become stale prior to its uses. The channel H in spatial-time-Frequency, Had in angle-delay domain, and Hdd in spatial-delay-Doppler domain are summarized in Table 3.

Table 3 MIMO OFDM channel in time-frequency domain, angle-delay domain, and spatial-delay-Doppler domain.

| Domain | Formula for Feedback Channel | Descriptions |
| --- | --- | --- |
| Spatial-Time- Frequency | H=H0H1⋯HNc-1T | Hc is the Nr×Nt matrix channel in spatial domain, where c=0,1,⋯,Nc-1. |
| Angle-Delay | Hca=Ur*HcUt Ha=H0aH1a⋯HNc-1aT Had=FNc-1Ha | Hca is the Nr×Nt channel matrix in angle-domain. Note: it requires the transformation. |
| Spatial-Delay-Doppler | Hrtdd=FNc-1HrtFNd Hdd=H11dd⋯H1Ntdd⋮⋱⋮HNr1dd⋯HNrNtdd | Hrt is the Nc×Nd matrix of the channel in time-frequency domain, on the t-th transmit antenna port and the r-th receive antenna port. Note1: It requires the transformation. Note2: UE is required to feed back a block fading channel (a kind of temporal compression). |

References

RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, #94e, 2021.

R1-2203550, Evaluation on AI/ML for CSI feedback enhancement, vivo, RAN1 #109e, 2022.

R1-2203549, General discussions on AI/ML framework, vivo, RAN1 #109e, 2022.