3GPP TSG RAN WG1 #109-e                                                           R1-2203553

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Other aspects on AI/ML for beam management

Agenda Item:	9.2.3.2

Document for:	Discussion and Decision

Introduction

- In RAN 94-e meeting [1], study on artificial intelligence (AI) / machine learning (ML) for multiple use cases were approved regarding aspects such as performance, complexity, and potential specification impact.

- Regarding multiple use cases:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project.

- For the use cases under consideration:

- Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set:

  - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations.

    - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed.

    - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study.

    - Need for common assumptions in dataset construction for training, validation and test for the selected use cases.

    - Consider adequate model training strategy, collaboration levels and associated implications

    - Consider agreed-upon base AI model(s) for calibration

    - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes

  - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases.

    - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline

    - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered.

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signaling, means for training and validation data assistance, assistance information, measurement, and feedback

  - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input

    - Collaboration level specific specification impact per use case

  - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2

    - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable

    - Consider the need and implications for AI/ML processing capabilities definition

Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved.

Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this contribution, we focus on the possible use cases of beam management, including spatial domain beam prediction and time domain beam prediction, and simulation results, corresponding comparisons, and observations will be provided to verify the rationality and validity of our beam management algorithm by artificial intelligence (AI) / machine learning (ML).

Discussion on sub use cases

In this section, we first analyze potential sub use cases for beam management, and then we present our preference based on performance evaluation and flexibility of model deployment. Due to two use cases, i.e. beam prediction in the spatial domain and beam prediction in the time domain, were agreed to study in [1], we will discuss potential sub use cases separately in the two agreed use cases in the following sections.

Potential sub use cases and suggestions in spatial domain beam prediction

Without considering AL/ML output types, such as predicting optimal beam ID, all beams’ RSRP, etc., two main sub use cases can be used in spatial domain beam prediction to reduce overhead and/or improve beam searching accuracy. One is fine beam prediction based on coarse beam measurement and another is super-resolution beam prediction based on partial beam measurement.

- Sub use case 1a: fine beam prediction based on coarse beam measurement.

- Sub use case 1b: super-resolution beam prediction based on partial beam measurement.

Two main sub use cases can be considered for spatial domain beam prediction to reduce overhead and/or improve beam searching accuracy.

- Fine beam prediction based on coarse beam measurement

- Super-resolution beam prediction based on partial beam measurement.

From initial observation, both two sub use cases have the same core issue: finding an AI algorithm with good prediction performance gain and also provide sufficient flexibility on AI model deployment. Thus, we will focus on how to find a suitable AI algorithm based on the sub use case of super-resolution beam prediction. Besides, for considering the importance of RSRP accuracy and selected optimal beam accuracy, the output of the AI model should be L1-RSRP of all beam pairs and the relative optimal beam set can be acquired from predicted L1-RSRP, as shown in figure 1.

Figure 1 beam prediction with fixed pattern RSRP

- For the super resolution beam prediction sub-case, there are some variations considering beams used.

- One beam prediction algorithm is to use RSRP of beam subset with fixed beam pattern to predict all RSRPs. From our evaluation in [2], compared with baseline/traditional beam management, a significant gain can be obtained by AI algorithm with the same fixed pattern training and validation, but unacceptable performance deterioration appears in AI algorithm with different fixed pattern for model training and validation.

- Another beam prediction algorithm is to use RSRP of beam subset with random beam pattern to predict all RSRPs. There is performance degradation compared to fixed pattern scheme. To combat such performance degradation, assistance information e.g., beam ID or angle information can be considered. In [2], the performance improved by assistance information for random beam pattern is notable.

- For more flexibility for various implementations, the output of an AI beam prediction algorithm can be decoupled with the number of Rx beams as below. The output is predicted partial RSRPs corresponding to expected Rx angle in AI input. Based on our evaluation, the performance is quite good.

Figure 2 beam prediction with random pattern, assistance information and expected information

- Based on above discussion, all the different variation of the two use cases should be further studied.

Study different variations for each sub-use case, considering generalization performance for different number of Tx and Rx beams.

Potential sub use cases and suggestions for time domain beam prediction

Time domain prediction are used to reduce beam sweeping overhead and improve beam switching time.

The initial principle for both the two purposes is to use a limited number of historical information of measured beams to predict partial or full future beam information, which is similar with spatial domain beam prediction with partial beam input. Besides, both of performance improvement and AI model generalization should be considered. As a consequence, AI algorithm for spatial domain beam prediction is an essential precondition for time domain beam prediction study.

The sub use case of time domain prediction is to predict future beam RSRP with historical RSRPs for both of the following purposes,

- Time domain beam prediction for overhead reduction

- Time domain beam prediction for accurate beam switching time

Spatial domain beam prediction algorithm is an essential precondition for time domain beam prediction study.

Figure 3 Time domain beam prediction schematic diagram

Based on our performance comparison in [2] for spatial domain beam prediction, sub use case 2a for overhead reduction is selected for preliminary study. Similar to spatial time domain prediction, the output of AI model is L1-RSRP of all beam pairs at multiple time periods, and relative best beam sets for different time instants can be obtain from all predicted RSRPs of each time period as shown in figure 3. For AI input, a target number of beam pairs are selected from each time period, and then historical beam pairs with multiple time periods as one sample can be used for input parameters to predict future beams.

For temporal beam prediction with overhead reduction purpose, two beam pattern selection method can be considered.

- Half-fixed beam pattern selection: Half-fixed beam pattern is improved from fixed beam pattern scheme in spatial beam prediction. Beam pattern is fixed from the perspective of each sample used for AI model input, but the pattern for half-fixed scheme is different across all time periods included in a sample.

- Random beam pattern selection: It is very similar to that method in spatial domain beam prediction with randomly selecting a target number of beams from total beam pairs for each historical time period across all input samples.

Refer to the performance comparisons in spatial domain beam prediction, half-fixed pattern scheme is used to obtain optimal performance gain for temporal beam prediction, while random pattern scheme with assistance information is used to bring satisfactory gains with sufficient flexibility.

In [2], a significant gain is obtained by AI algorithm with both of two beam pattern selection schemes in comparison with traditional beam management, and almost same performance gain can be achieved between half-fixed pattern scheme and random pattern scheme with assistance information which random pattern scheme brings more flexibilities for AI deployment.

Study impact of different beam sweeping patterns for time domain beam prediction.

Potential specification impact

As pointed out in our companion paper [3], at least 6 stages may be included in lifecycle management for beam prediction, which are data collection, model training, model validation, model deployment, performance monitoring and model updating. In this section, we will analyze the potential specification impact on aspects of model deployment with collaboration level, data collection and performance monitoring for beam management use case.

Discussion on collaboration levels

Different collaboration levels have different impact on lifecycle management. Dependent on which side is responsible model training and inference, the specification impact would be different.

For beam management use case, if AI model is deployed at gNB side, level 0 can be achieved as long as enough data is collected. To further improve performance and collect more data, level 1 can be supported with more RSRP measurement results collected. There is also some potential performance deterioration due to quantization error for quantizing received L1-RSRP.

For collaboration level 2, additional singling can be exchanged between two sides for model-related information especially considering model deployment at UE side. Based on our evaluation results in [2], with fixed or random beam pattern plus assistance information, performance improvement and overhead reduction can be observed. Such beam pattern or assistance information also needs signaling exchange between two sides.

For collaboration level 3, there may also be some use cases that if the model is jointly trained by the two sides, more local information can be used. Companies are encouraged to study sub-use cases for this collaboration level.

Study model deployment procedure and specification impact for both cases that beam prediction functionality resides in UE side and the functionality resides in gNB side.

Study sub-use cases from collaboration level 0~ level 3 for beam management cases.

Discussion on assistance information

For assistance information exchange, as our evaluation in [2], this information provides considerable gains and flexibilities on AI model deployment at least for spatial domain beam prediction. Thus, interactions of assistance information would be necessary between the network and UE which will impact relative resource configurations for beam sweeping.

4 types of assistance information are provided with performance improvement in section 2.1, such as {total beam ID}, {Tx beam ID, Rx beam ID}, {Tx vertical beam ID, Tx horizontal beam ID, Rx vertical beam ID, RX horizontal beam ID} and {Tx vertical beam angle, Tx horizontal beam angle, Rx vertical beam angle, RX horizontal beam angle}.  As the number of total beams related to {Total beam ID} varies dramatically for different UE capabilities, the type of {Total beam ID} should not be used as assistance information.

It is difficult to specify any types of beam ID in which the maximum value of beam ID is theoretically infinity. If the maximum value of beam ID is restricted, we can find that the meaning of angle and beam ID naturally provide the same functions in AI algorithm. For example, angle information with a resolution of 1 degree is identical to one type of beam ID information in which the maximum value of the beam ID information is limited to 360.

Although beam information can help to improve prediction performance, the exchanged beam angle information may expose some implementations on the design of physical antennas and corresponding phase shifters. We thus propose:

Study configuration method of beam angle with minimum exposures of implementation details.

Discussion on input orders

As discussed previously, random pattern provides the most flexibility. The corresponding issue is how to select random pattern from all samples. Different ways of constructing dataset for training may have performance impact to the inference. In the following we give two ways of constructing data set: permutation method and sorted-combination method. For example, if beam ID of {1, 8, 4, 6} is randomly selected for AI model input, sorted-combination method means {1,4,6,8} is the only one beam pattern for training, while for perturbation method there are a total of 24 beam patterns for {1,8,4,6}.

Table 1: performance comparison between permutation and combination with 8 random pattern RSRPs

| Scheme | Mean BM deterioration |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| random selection method | permutation |  |  |  | sorted-combination |  |  |  |
| Number of beams in predicted best beam set | 1 | 2 | 4 | 8 | 1 | 2 | 4 | 8 |
| RSRP only | 20.16 | 16.42 | 13.07 | 10.12 | 16.87 | 14.02 | 11.02 | 8.05 |
| RSRP + angle | 8.71 | 6.34 | 4.02 | 2.45 | 7.99 | 5.81 | 3.82 | 2.24 |

In table 1, 0.3 dB ~ 0.7 dB gain is obtained from sorted-combination method without increasing beam sweeping overhead and computational cost. There are more performance comparisons between the permutation method and combination method for random pattern selection in [2]. Based on these evaluations, we have the following proposal,

Study the gains and impact of different beam input orders.

Discussion on performance monitoring

In the model performance monitoring stage for beam management use case, performance of beam prediction models would need to be monitored and corresponding metrics for the performance would need to be reported to the network. For the case with random input patterns, the input beam pattern would influence the performance of the model. Thus, the two parties involved in model monitoring would need to be aligned on how the model is monitored.

Study the impact of input beam patterns to model performance monitoring for both spatial and time predictions.

Conclusions

In this contribution, we discuss some issues on AL/ML for beam management and have the following proposals:

- Two main sub use cases can be considered for spatial domain beam prediction to reduce overhead and/or improve beam searching accuracy.

- Fine beam prediction based on coarse beam measurement

- Super-resolution beam prediction based on partial beam measurement.

Study different variations for each sub-use case, considering generalization performance for different number of Tx and Rx beams.

The sub use case of time domain prediction is to predict future beam RSRP with historical RSRPs for both of the following purposes,

- Time domain beam prediction for overhead reduction

- Time domain beam prediction for accurate beam switching time

Spatial domain beam prediction algorithm is an essential precondition for time domain beam prediction study.

Study impact of different beam sweeping patterns for time domain beam prediction.

Study model deployment procedure and specification impact for both cases that beam prediction functionality resides in UE side and the functionality resides in gNB side.

Study sub-use cases from collaboration level 0~ level 3 for beam management cases.

Study configuration method of beam angle with minimum exposures of implementation details.

Study the gains and impact of different beam input orders.

Study the impact of input beam patterns to model performance monitoring for both spatial and time predictions.

References

Chairman’s notes for 3GPP TSG RAN WG #94-e Meeting.

R1-2203552, Evaluation on AL/ML for beam management, RAN WG1 #109-e

R1-2203549, General discussions on AI/ML framework, RAN WG1 #109-e