3GPP TSG RAN WG1 #109-e                                                           R1-2203554

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Evaluation on AI/ML for positioning accuracy enhancement

Agenda Item:	9.2.4.1

Document for:	Discussion and Decision

Introduction

Many positioning methods have been specified in Rel-16 and Rel-17 NR positioning, to obtain position estimation with target horizontal positioning accuracy <0.2 m (90%) for IIoT use cases and <1 m (90%) for commercial use cases. However, the performance of these positioning methods, highly relies on the existence of multiple LOS (line-of -sight) paths between the target terminal and multiple TRPs (Transmission-Reception Points). In the scenarios with extremely low LOS (i.e. high NLOS) probability, positioning accuracy would decrease dramatically, which may be not enough to satisfy the high-accuracy positioning requirements stemming from new applications and industry verticals.

Target positioning requirements for commercial and IIoT use cases are defined in Rel-17 as follows:

- In Rel-17 target positioning requirements for commercial use cases are defined as follows: - Horizontal position accuracy (< 1 m) for [90%] of UEs - Vertical position accuracy (< [2 or 3] m) for [90%] of UEs - End-to-end latency for position estimation of UE (< [100 ms]) - FFS: Physical layer latency for position estimation of UE (< [10 ms]) - In Rel-17 target positioning requirements for IIoT use cases are defined as follows: - Horizontal position accuracy (< X m) for [90%] of UEs - X = [0.2 or 0.5] m - Vertical position accuracy (< Y m) for [90%] of UEs - Y = [0.2 or 1] m - End-to-end latency for position estimation of UE (< [10ms, 20ms, or 100ms]) - FFS: Physical layer latency for position estimation of UE (< [10ms]) Note: Target positioning requirements may not necessarily be reached for all scenarios

AI/ML technology has powerful abilities in feature extraction, environment awareness, complex problem modeling and processing. In recent years, applying AI/ML into air-interface has attracted great attentions from academics to industries, and a lot of meaningful exploration has been made to verify the performance gain as compared to conventional non-AL/ML schemes. Related research has also verified that AI/ML technology has the potential to significantly improve the performance of wireless communications in general.

Under this background, a new SI on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface has been agreed at RAN #94e [1], including three sub use cases to assess the applications of AI/ML in air-interface. Among them, AI/ML based positioning accuracy enhancement is included, with the goal to improve the positioning accuracy for different scenarios, especially for some challenging scenarios with heavy NLOS (non-line-of -sight) conditions.

The objective of the new SI for AI/ML based positioning includes the following [1]:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

In this contribution, we present our views about the simulation scenarios and assumptions, key performance indicator (KPI) and evaluation methodology. Moreover, some initial simulation results and observations are also presented to demonstrate the performance gain of applying AI/ML technology onto positioning for InF-DH scenarios.

Evaluation methodology

In this section, evaluation methodology of AI/ML based positioning is discussed, including simulation scenario, dataset construction, model evaluation and other calibration.

Simulation scenario

According to the SID [1],	the evaluation methodology should be based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations.

In TR38.901, multiple InF scenarios are defined, focusing on factory halls with varying sizes and varying levels of clutter density. The InF scenarios include:

- InF-SL	Indoor Factory with Sparse clutter and Low base station height (both Tx and Rx are below the average height of the clutter)

- InF-DL	Indoor Factory with Dense clutter and Low base station height (both Tx and Rx are below the average height of the clutter)

- InF-SH	Indoor Factory with Sparse clutter and High base station height (Tx or Rx elevated above the clutter)

- InF-DH	Indoor Factory with Dense clutter and High base station height (Tx or Rx elevated above the clutter)

- InF-HH	Indoor Factory with High Tx and High Rx (both elevated above the clutter)

Among them, the DH scenario with clutter parameter {density: 60%, height: 6m, size: 2m} have extremely low LOS probability (95% NLOS links, as shown in Figure 1) and it is challenging to achieve accurate position estimation by utilizing the conventional RAT-dependent positioning methods, such as TDoA, RTT and so on.  To evaluate AI based positioning performance in heavy NLOS conditions, we suggest to select the DH scenario with clutter parameter {density 60%, height 6m, size 2m} as a typical scenario for evaluation.

LOS probability of 4 InF scenarios (SL, DL, SH, DH)

Select the InF-DH scenario with clutter parameter {density 60%, height 6m, size 2m} as a typical scenario for positioning accuracy enhancement evaluation.

Dataset and model Calibration

ML technology mainly consists of three components, including data, model and algorithm. For deep learning represented by neural networks, stochastic gradient descent algorithm has been widely used to optimize parameters of neural networks, and can quickly converge to a suboptimal solution by appropriate hyperparameter settings. To conclude, model performance highly relies on the architecture of AI model, quantity and quality of dataset, hyperparameters related to model training. In this sense, various AI model and dataset construction can result in misunderstanding on the performance of AI/ML over air interface.

To evaluate the performance of AI/ML technology for various sub use cases and potentially to calibrate the performance among different companies, it would be better to have a common dataset and AI model constructed for this study. If that’s not possible, then another option is to at least align how datasets are generated in system-level simulation among all companies. In particular, specific simulation parameters should be identified and agreed to assure a common methodology and distribution of dataset construction. Moreover, some details about AI model, including model architecture and hyperparameters related model training, should be provided. For research purpose, we share our dataset for AI based positioning through the link [2], and provide some details about AI model in Appendix B.

Dataset and AI model sharing among different companies should be encouraged.

Dataset construction and model validation

When datasets are available, the next step is to discuss how to train an AI model with these datasets. Considering the lifecycle of AI model [3], we think the process from model training to model inference should be divided into three phases: training, validation and testing. Accordingly, dataset should also be divided into training dataset, validation dataset and test dataset. Training dataset is used to train an AI model, and validation dataset is used to validate the model performance. It is noted that training dataset and validation dataset should obey the same statistical distribution, or in other words, the samples of these two datasets are taken from the same environment or generated with the same simulation parameters in the same scenario.

According to the SID [1],	the evaluation methodology should be based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. We propose to follow the SID and take the same statistical mode as in TR 38.901 and TR 38.857 to generate dataset for this SI. Note that, AI model training and validation can be performed either offline or online. However, considering the nature of AI/ML technology where training and validation would require a large amount of data for processing, it seems cost (e.g., power and time consumption, memory requirement etc.) prohibitive especially for UE to conduct online AI model training and/or validation. Therefore, we assume the focus of this SI will be offline AI model training and validation at least for UE.

Testing dataset is used to test AI model performance in actual deployment. Ideally, testing dataset should be based on field data sampled in the deployment scenarios. However, it may not be practical for companies to measure field data for all potential deployment scenarios. For the purpose of study AI model performance in this SI, we think testing data samples can be considered using the same methodology as that of AI model training/validation in one or more scenario(s) and/or a combination of a set of scenarios. Additionally, we think field data measured in actual deployment for AI model performance testing should be allowed and encouraged.

For the purpose of link level and system level evaluation, statistical models (from TR 38.901 and TR 38.857) are utilized to generate dataset for AI/ML based positioning for model training/validation and testing.

  - Field data measured in actual deployment for AI/ML model performance testing should be allowed and encouraged

Intuitively, when the distribution of test dataset and training/validation set is not the same, the positioning accuracy of AI/ML model based positioning may be adversely impacted. This is observed in the following evaluation.

From simulation results listed in Table 1 and Figure 2, we can observe that AI technology can achieve high-accuracy positioning when training dataset and test dataset are consistent (generated in the same scenario). When the model trained with dataset of an InF-DH scenario is directly transferred to other scenarios, such as InF-HH and InF-SH scenarios, different dataset distribution with training dataset will severely deteriorate the positioning accuracy due to limited generalization ability of AI model. As we can see, high positioning accuracy (<1m @90%) could be achieved when training dataset and test dataset are sampled from the same scenario. Otherwise, the performance will deteriorate severely (>10m @90%).

CDF of positioning accuracy (m) when the scenarios of training dataset and test dataset are different

| Training dataset | Test dataset | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| InF-DH | InF-DH | 0.35 | 0.49 | 0.70 | 0.99 |
| InF-HH | InF-HH | 0.29 | 0.40 | 0.48 | 0.63 |
| InF-SH | InF-SH | 0.36 | 0.46 | 0.60 | 0.87 |
| InF-DH | InF-HH | >10 | >10 | >10 | >10 |
| InF-DH | InF-SH | >10 | >10 | >10 | >10 |

CDF of positioning accuracy when training dataset and test dataset are not matched

When the distribution of test dataset is different from that of training dataset, the positioning accuracy of AI/ML based positioning can be seriously degraded.

The positioning accuracy performance of AI/ML based positioning should be evaluated under all scenarios.

Spatial consistency

The scenarios with non-spatial consistency assumes that channels are random and not related to UEs’ locations. While in real environment, it is intuitive that geographically close UEs share similar channels including LOS condition, multipath, path loss, channel condition and so on. In order to make simulation assumptions closer to real environment, spatial consistency procedure is naturally introduced in TR 38.901.  In this section, we further evaluate the impact of spatial consistency assumption on positioning performance for AI/ML based positioning schemes.

CDF of positioning accuracy (m) with and without spatial consistency assumption

| Scenario | Spatial consistency | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| InF-DH {0.6, 6, 2} | Without | 2.49 | 3.24 | 4.27 | 5.98 |
|  | With | 0.35 | 0.49 | 0.70 | 0.99 |

CDF of positioning accuracy with and without spatial consistency assumption

As shown in Table 2, positioning performance with spatial consistency is noticeably better than that of without spatial consistency. Thus, we think that the assumption of spatial consistency can facilitate AI model find the relation between CIR and position.

Performance under different spatial consistency assumption can be vastly different, and positioning performance with spatial consistency (0.99m@90%) is noticeably better than that of without spatial consistency (5.89m@90%).

Spatial consistency assumption should be adopted for performance evaluation.

KPIs

While AI/ML technology may improve positioning accuracy, the introduction of AI/ML requires powerful computation capability for model training and inference, large memory for model and data storage, and so on. Thus, we suggest that KPIs of AI/ML based positioning should consist of performance related KPIs for positioning accuracy evaluation, and cost related KPIs for cost evaluation.

Performance related KPIs

It is necessary to align performance related KPIs among different companies so that potential observation/conclusion can be drawn upon different companies’ evaluation results. In our opinion, performance related KPIs, such as @50%, @90% positioning accuracy as defined in TR 38.857, can be used directly to evaluate the performance gain of AI/ML based positioning. Besides, conventional RAT-dependent positioning methods specified in previous releases can be considered as baseline schemes for performance comparison.

Generalization is one of the key issues for all AI/ML applications, and AI based positioning is of no exception. The generalization performance (or test performance) of AI model is affected by the AI model structure, the variety of training data set and the training strategy. It is better to keep the training loss to be an accurate approximation of the generalization loss uniformly for all hypotheses. When performing evaluation of performance related KPIs, generalization performance should be seriously considered, and different levels of generalization may need to be verified. For example, whether the performance maintains when AI model transfers from one cell to another, from one drop to another, or from one scenario to another.

Performance related KPIs, such as @50%, @90% positioning accuracy defined in TR 38.857, can be used directly to evaluate the performance gain of AI/ML based positioning.

Consider the following different levels of generalization performance for performance evaluation.

  - Generalization performance form one cell to another

  - Generalization performance from one drop to another

  - Generalization performance from one scenario to another

Cost related KPIs

Signaling overhead, power consumption, computational complexity, parameter quantity, training data requirement and hardware requirement (including for given processing delays) associated with enabling respective AI/ML scheme should also be carefully considered. Among the above, computational complexity, parameter quantity and training data requirement are of importance.

There is a tradeoff between computational complexity, parameter quantity and capability of AI/ML models. While AI/ML models with heavy computational complexity and large-scale parameter usually enjoy better model capability and inference accuracy, model operations, including model training, validation and testing, require more powerful support of power and hardware capability. These requirements may restrict the users with low device capability from accessing the service of AI/ML based positioning. In practice, computational complexity of AI models would be easily obtained by the API of AI/ML platform, and floating-point operations per second (FLOPs) could be adopted for evaluation. Moreover, when AI/ML models need to interact between the network entity and the terminal, large-scale parameter interaction will consume plenty of air-interface resources. Thus, the evaluation of parameter quantity is necessary, and will provide great guidance for real deployment of AI model in wireless network. For example, large-scale AI model should not be interacted frequently over air interface, and thus, AI model interactive mechanism should be carefully designed to reduce signaling overhead without loss of performance gain.

For AI/ML based positioning, while model performance can be benefit from larger dataset, it is an expensive and privacy sensitive task to sample large amounts of field data with ground truth labels to support large-scale model training. Thus, training data requirement should be also considered as a cost-related KPI.

Moreover, other cost related KPIs, such as power consumption and processing delay, which are strongly related to specific hardware, should be also considered, but are not necessary to be touched at the beginning of this study. As hardware capabilities improve, these factors will not pose a major challenge for applications of AI in air interface.

Computational complexity, parameter quantity and training data requirement are three crucial cost-related KPIs for AI/ML based positioning, and should be considered with high priority at the beginning of this study.

Preliminary evaluation results

In this section, we provide our preliminary simulation results for performance evaluation of AI/ML based positioning. The heavy NLOS scenario of InF-DH {0.6, 6, 2} is adopted for evaluations, and datasets with spatial consistency, including training dataset, validation dataset and test dataset, are generated with system-level simulation platform to train, validate and test AI model, respectively. The detailed parameter assumptions for scenarios are provided in Appendix A, and the details about AI model training/validation and testing parameters are provided in Appendix B.

Positioning with multi-TRPs’ CIRs

For AI/ML based positioning, UE position can be estimated according to multiple TRPs’ Channel Impulse Response (CIR) vectors, as shown in Figure 4. Note that, AI model can be deployed at the UE side or network side.

UE position estimation with multiple TRPs’ CIRs

The InF-DH scenario with size 120m60m and cluster density {0.6, 6, 2} is adopted for evaluation. For each UE, we generate time-domain channel response data points (with the dimension of ) labeled with associated location by system level simulation platform. Then, we sample by truncating the first 256 time-domain points based on the 1st Tx antenna element  and the 1st Rx antenna element from CIR. Finally, the sampled CIR is reshaped into the dimension of  as the input of AI model. Moreover, 23000 samples are used to train the adopted Vision Transformer model [4], and 3000 samples are used for validation.

Performance comparison

The conventional positioning methods in previous releases are considered as baselines. From the simulation results in Table 3, it is observed that the positioning errors of baselines are larger than 20m due to the low probability of LOS path, which is not able to satisfy the requirements of high accuracy positioning in heavy NLOS scenarios. While AI technology can significantly improve positioning accuracy and reaps a conspicuous performance gain (<1m @90%). Thus, we observe that AI technology can improve the positioning accuracy in heavy NLOS scenarios.

CDF of positioning accuracy (m) of different positioning methods

| Scenario | Positioning methods | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| InF-DH {0.6,6,2} | DL-TDOA | 8.38 | 11.09 | 15.95 | 32.12 |
|  | UL-TDOA | 8.60 | 11.52 | 16.33 | 32.81 |
|  | RTT | 8.32 | 11.42 | 15.72 | 32.41 |
|  | AOA | 8.13 | 10.36 | 14.09 | 20.16 |
|  | Machine learning | 0.35 | 0.49 | 0.70 | 0.99 |

CDF of positioning accuracy (m) of different positioning methods

AI/ML based positioning can significantly improve the positioning accuracy compared to existing RAT-dependent positioning methods in heavy NLOS scenarios.

Model input

Simulation comparison of different input selection for AI/ML based positioning is shown in Table 4. We can see that time domain channel CIR as the input of AI/ML model can obtain the best positioning accuracy compared to other inputs, such as power, delay and angle of the first path. The reason for this we believe is that original CIR contains richer features which may be strongly related to the target UE’s location. In this sense, AI/ML model can be regarded as a feature extractor, capturing location related features from CIR in an implicit manner, and then determining the location according to these features.

CDF of positioning accuracy (m) of different model inputs

| Scenario | Model inputs | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| InF-DH {0.6,6,2} | CIR | 0.35 | 0.49 | 0.70 | 0.99 |
|  | Power + delay + angle of the first path | 0.51 | 0.70 | 0.90 | 1.19 |
|  | Power + delay of the first path | 0.52 | 0.75 | 1.00 | 1.31 |
|  | Delay + angle of the first path | 0.65 | 0.84 | 1.11 | 1.43 |
|  | Angle + power of the first path | 0.83 | 1.14 | 1.45 | 1.79 |

CDF of positioning accuracy (m) of different measurements

Different input of AI model will affect the positioning performance for AI/ML based positioning. Time domain channel CIR as the input of AI model obtains the best positioning accuracy.

Support time domain CIR as the model input for AI/ML based positioning.

AI model generalization

We perform some simulations to evaluate the generalization capability of AI based positioning with multi-TRPs’ CIRs. As shown in Table 5, while the model trained with dataset of drop 1 perform well with test dataset of drop 1, the performance will deteriorate severely when the model (without any modification on parameters) is tested on dataset from other drops. Here different drops mean different distributions of large-scale parameters in system level simulation, and these large-scale parameters contain absolute time, angle of arrival, angle of departure, power of LOS/NLOS paths, initial phase of LOS/NLOS paths, delay of LOS/NLOS paths, and so on. For the case of InF scenario, different drops can be intuitively viewed as different factories with different interiors.

CDF of positioning accuracy when AI model is transferred to other drops

| Scenario | Training dataset | Test dataset | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| InF-DH {0.6,6,2} | Drop 1 | Drop 1 | 0.35 | 0.49 | 0.70 | 0.99 |
|  | Drop 1 | Drop 2 | 2.76 | 3.68 | 4.58 | 6.00 |
|  | Drop 1 | Drop 3 | 2.78 | 3.70 | 4.62 | 5.81 |

CDF of positioning accuracy when AI model is tested on other drops

Positioning performance of AI/ML based positioning degrades when the model trained with dataset of one drop is tested with dataset of other drops.

Positioning with single-TRP’s CIR

As shown in Figure 8, instead of constructing an AI model with 18 TRPs’ CIRs as input and the target UE’s location as output, we consider a more general framework with one TRP’s CIR as the input and an intermediate feature (such as TOA of that TRP at the target UE) as the output for each TRP, respectively. Based on the intermediate feature extracted from CIR of each TRP, location of the target UE can be further derived by utilizing other positioning algorithms, including AI-based or non-AI based algorithms. In order to distinguish from aforementioned positioning method based on multi-TPRs’ CIRs, we call it two-step positioning method, i.e., CIR-intermediate feature-positioning.

The main motivations come from our considerations about AI model generalization and practical deployment in real environment for AI based positioning. The AI model related with multiple TRPs is strongly correlated with TRPs’ distribution, and may not work well once TPRs’ distribution changes, such as the number of TPRs, the location of each TRP. Apparently, an AI model trained with multiple TRPs’ CIRs works the best in those trained scenarios with multiple TRPs, which in turn means that large number of field data needs to be collected from real deployment and computation & time-consuming model training/validation process needs to be conducted from scratch for each scenario. However, the two-step positioning method estimating intermediate feature from single-TRP’s CIR is independent of these factors, and can be largely compatible with existing positioning protocol framework (i.e. LPP) specified in previous releases.

The framework of two-step positioning method

We evaluate the performance of two-step positioning method by simulations where TOA from a TRP to a target UE is taken as the intermediate feature. The specific procedures are presented as follows. Firstly, we obtain the input of the AI model with the dimension of  in a similar manner as described in the section 4.1. Note that full CIR points are taken without truncation. It is further divided into 18 vectors each with dimension of as the input of single-TRP’s model. Based on the relative position of UEs and each TRP, AI models for TOA label associated with straight-line (LOS) distance can be trained. Then TOA associated with each TRP can be estimated according to the trained AI/ML models with UE’s CIR as the input. To estimate UE’s location according to TOAs of multiple TRPs, we adopt a Non-AI algorithm by combining Least Square algorithm with Taylor algorithm. Specifically, we firstly select four TOA estimation with the highest accuracy from multiple TOA estimations when assuming that TOA errors can be obtained. Moreover, conventional TRP selection algorithms can be used to select the TOAs with minimal errors, such as Receive Autonomous Integrity Monitoring algorithm. Utilizing the selected TOAs and prior locations of TRPs, Least Square algorithm is used to estimate a rough location as one of the inputs of Taylor algorithm, and then the final location can be estimated by Taylor algorithm.

As shown in Table 6, the two-step positioning method (0.60m@90%) achieves remarkable performance gain compared to one-step positioning method (0.99m@90%). Besides, the two-step method is less sensitive to environmental change compared to the one-step method. Even if the model trained with dataset of a fixed drop is tested with a new drop without any modification, the performance of two-step positioning method can still be maintained, which is significantly better than that of one-step positioning method. This method also enjoys better generalization capability. Thus, we can conclude that the two-step method has great advantages in positioning performance, deployment flexibility, compatibility with existing positioning protocol framework, and generalization capability.

CDF of positioning accuracy of different positioning methods

| Scenario | Methods | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| InF-DH {0.6,6,2} | One-step (CIR-pos) training dataset and testing dataset from same drop | 0.35 | 0.49 | 0.70 | 0.99 |
|  | Two-step (CIR-TOA-pos) training dataset and testing dataset from same drop | 0.19 | 0.28 | 0.40 | 0.60 |
|  | One-step (CIR-pos) training dataset and testing dataset from different drops | 2.76 | 3.68 | 4.58 | 6.00 |
|  | Two-step (CIR-TOA-pos) training dataset and testing dataset from different drops | 0.93 | 1.27 | 1.73 | 2.51 |

CDF of positioning accuracy of different positioning methods

AI/ML based two-step positioning framework has better positioning accuracy, deployment flexibility, compatibility with existing positioning protocol framework and generalization capability than one-step positioning method.

Study further on the benefits of two-step positioning for AI/ML based positioning in terms of positioning accuracy and AI model generalization.

fine-tuning for model generalization

When AI model trained offline is transferred to a new scenario, performance degradation may be inevitable due to the mismatch between training data and field data. In general, there are two solutions to deal with this generalization problem:

The first is to ensure training data and field data are sampled from the same scenario. In this way, the network entity or UE needs to collect large amounts of data for model training and validation, and considerable computational and time resource are also required to train these scenario-specific models.

The second is fine-tuning. Specifically, AI model is pre-trained by offline data which may be from simulation data, field data collected by different TRPs, or both. When the pre-trained model is transferred to real environment, a retraining process, named fine-tuning, should be triggered to fine-tune the pre-trained model with field data collected from real environment. In this way, a scenario-specific model is obtained with a small amount of field data and computation & time resource consumption.

- The first solution is obvious and intuitive, which can achieve good performance at the cost of heavy data collection and model training. In this section, we mainly focus on the second solution utilizing fine-tuning to enhance the model generalization performance.

- In general, fine-tuning procedure consists of two steps. The first step is to pre-train a model based on some offline data. The second step is to fine-tune the pre-trained model based on collected field data. Here, a meta-learning framework is adopted to pre-train a meta-model [5]. For comparison, AI based two-step positioning method without fine-tuning serves as the baseline. As shown in Table 7, fine-tuning the model with only 1000 samples, can achieve obvious positioning accuracy gain when the pre-trained model trained with drop 1 is transferred to other drops compared to without fine-tuning.

CDF of positioning accuracy of fine-tuning in different drops

| Scenario | Fine-tuning | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| InF-DH {0.6,6,2} | With | 0.73 | 1.07 | 1.49 | 1.97 |
|  | Without | 1.00 | 1.38 | 1.87 | 2.76 |

CDF of positioning accuracy of fine-tuning in different drops

We further evaluate the positioning performance when the pre-trained model trained with InF-DH {0.6, 6, 2} data is transferred to an InF-HH scenario. As listed in Table 8, we can observe that the AI model trained with InF-DH data will not work in an InF-HH scenario, and the positioning error is unacceptable. However, when the pre-trained model trained with InF-DH data is fine-tuned with only small amounts of InF-HH data, we observe an obvious performance improvement. This result is even better than the first solution where training an InF-HH model with large amounts of InF-HH data as listed in Table 1.

CDF of positioning accuracy of fine-tuning in different scenarios

| Fine-tuning | Training dataset | Testing dataset | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- | --- |
| Without | InF-DH | InF-HH | >10 | >10 | >10 | >10 |
|  | InF-HH | InF-DH | >10 | >10 | >10 | >10 |
| With | InF-DH | InF-HH | 0.09 | 0.14 | 0.19 | 0.28 |

CDF of positioning accuracy of fine-tuning in different scenarios

Fine-tuning the model with small amounts of samples can achieve significantly positioning accuracy improvement when the pre-trained model is transferred to a new scenario.

Study further on the benefits of fine-tuning for AI/ML based positioning in terms of positioning accuracy and AI model generalization.

Cost evaluation

In the previous sections, we mainly evaluate the positioning accuracy performance and generalization capability for AI/ML based positioning, and observe that AI technology has great potential to improve positioning accuracy. On the other hand, power consumption, computational complexity, parameter quantity, training data requirements and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme are essential for practical deployment of AI based positioning. Here, we mainly focus on computational complexity, parameter quantity, training data requirements in this section.

Model assumption

We adopt two different AI models, vision transformer and convolutional neural network (CNN), to evaluate the positioning performance of multi-TRPs and single-TRP based positioning methods, respectively. Basically, CNN has become the most popular AI model in computer vision due to its inherent properties, including inductive biases, translation equivariance and locality. These inherent properties are suitable for most of computer vision tasks since there is a strong correlation between adjacent pixels. Vision transformer lacks these properties inherent to CNN, while utilizing self-attention to build the global correlation every two pixels.  With the same amount of training data (23000 samples), we observe that vision transformer performs better than CNN for multi-TRPs based positioning method, but in turn, CNN performs better for single-TRP based positioning method.

All models are wrong, but some models are useful, George Box. We think the selection of AI model may be strongly related to specific tasks, and a suitable model can facilitate better evaluation of performance gain for AI/ML based positioning.

The selection of AI model may be strongly related to specific tasks, and a suitable model can facilitate better evaluation of performance gain for AI/ML based positioning

Cost related KPIs

The cost related KPIs of these two models are listed in Table 9.

Cost evaluation of AI models

| AI models | CNN | Vision Transformer |
| --- | --- | --- |
| Computational complexity | 1.45G FLOPs | 22.30M FLOPs |
| Parameter quantity | 44M | 1.65M |
| Training data requirements | 23000 | 23000 |

The computational complexity and parameter quantity can be further reduced by model optimization, and some other models may get better performance with lower cost.

Conclusions

- In this contribution, we have the following observations based on our evaluation results for AI/ML based positioning accuracy enhancement.

- When the distribution of test dataset is different from that of training dataset, the positioning accuracy of AI/ML based positioning can be seriously degraded.

Performance under different spatial consistency assumption can be vastly different, and positioning performance with spatial consistency (0.99m@90%) is noticeably better than that of without spatial consistency (5.89m@90%).

AI/ML based positioning can significantly improve the positioning accuracy compared to existing RAT-dependent positioning methods in heavy NLOS scenarios.

Different input of AI model will affect the positioning performance for AI/ML based positioning. Time domain channel CIR as the input of AI model obtains the best positioning accuracy

Positioning performance of AI/ML based positioning degrades when the model trained with dataset of one drop is tested with dataset of other drops.

AI/ML based two-step positioning framework has better positioning accuracy, deployment flexibility, compatibility with existing positioning protocol framework and generalization capability than one-step positioning method.

Fine-tuning the model with small amounts of samples can achieve significantly positioning accuracy improvement when the pre-trained model is transferred to a new scenario.

The selection of AI model may be strongly related to specific tasks, and a suitable model can facilitate better evaluation of performance gain for AI/ML based positioning

We  have made the following proposals for AI/ML based positioning accuracy enhancement.

- Select the InF-DH scenario with clutter parameter {density 60%, height 6m, size 2m} as a typical scenario for positioning accuracy enhancement evaluation.

Dataset and AI model sharing among different companies should be encouraged.

For the purpose of link level and system level evaluation, statistical models (from TR 38.901 and TR 38.857) are utilized to generate dataset for AI/ML based positioning for model training/validation and testing.

  - Field data measured in actual deployment for AI/ML model performance testing should be allowed and encouraged

The positioning accuracy performance of AI/ML based positioning should be evaluated under all scenarios.

Spatial consistency assumption should be adopted for performance evaluation.

Performance related KPIs, such as @50%, @90% positioning accuracy defined in TR 38.857, can be used directly to evaluate the performance gain of AI/ML based positioning.

Consider the following different levels of generalization performance for performance evaluation.

  - Generalization performance form one cell to another

  - Generalization performance from one one drop to another

  - Generalization performance from one scenario to another

Computational complexity, parameter quantity and training data requirement are three crucial cost-related KPIs for AI/ML based positioning, and should be considered with high priority at the beginning of this study .

Support time domain CIR as the model input for AI/ML based positioning.

Study further on the benefits of two-step positioning for AI/ML based positioning in terms of positioning accuracy and AI model generalization.

Study further on the benefits of fine-tuning for AI/ML based positioning in terms of positioning accuracy and AI model generalization.

References

3GPP RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”,  #94e, Dec.6-17, 2021.

vivo, “Dataset for AI Positioning”, https://commonbox.vivo.xyz/s/UQnWAcqp2DL.

Nokia, R3-222653, “(TR for TR 37.817)  General Framework”, #115-e, Feb.21 –  Mar.3, 2021.

Dosovitskiy, Alexey, et al. "An image is worth 16x16 words: Transformers for image recognition at scale." arXiv preprint arXiv:2010.11929, 2021.

Finn, Chelsea, Pieter Abbeel, and Sergey Levine. "Model-agnostic meta-learning for fast adaptation of deep networks." International conference on machine learning. PMLR, 2017.

Appendix A

- The common simulation parameters of scenarios can be found in Table A.1.

Table A.1 Parameters of InF scenario(s)

| Parameter | Values |
| --- | --- |
| Scenario | InF-HH (High Tx, High Rx), InF-SL (Sparse-clutter, Low BS), InF-DL (Dense-clutter, Low BS), InF-SH (Sparse-clutter, High BS), InF-DH (Dense-clutter, High BS) – Note 1 |
| Hall size | InF-HH: 300x150 m InF-SL: 120x60 m InF-DL: 300x150 m InF-SH: 300x150 m InF-DH: 120x60 m |
| Room height | 10 m |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ for FR1. |
| UE antenna configuration | (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1). dH=0.5λ for FR1. |
| Penetration loss | 0dB |
| Number of floors | 1 |
| UE horizontal drop procedure | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy. The evaluation area should be at least the convex hull of the horizontal BS deployment. It can also be the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. |
| BS deployment | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |
| UE distribution | uniform dropping for indoor with minimum 2D distance of 0 m |
| UE antenna height | 1.5m |
| gNB antenna height | BS height = 1.5 m for InF-SL and InF-DL BS height = 8 m for for InF-SH, InF-DH and InF-HH |
| Carrier frequency | 3.5G Hz |
| Bandwidth | 100M Hz |
| Clutter density: r | Low clutter density: 20% High clutter density: 60% |
| Clutter height: hc | Low clutter density: 2 m High clutter density: 6 m |
| Clutter size: dclutter | Low clutter density: 10 m High clutter density: 2 m |
| Note 1: According to Table A.2.1-7 in 3GPP TR 38.802 |  |

Appendix B

- The simulation parameters related to AI model training can be found in Table B.1.

Table B.1 Parameters of AI model training

| Parameter | Value 1 | Value 2 |
| --- | --- | --- |
| methods | - One-step positioning | - Two-step positioning |
| AI model | - Vision Transformer | - CNN |
| BS number | - 18 | - 18 |
| CIR length | - 256 | - 4096 |
| Input | - CIR 256x1x18 | - CIR 4096x1 |
| Output | - Location (x, y) | - TOA |
| Synchronization | - Ideal | - Ideal |
| Channel estimation | - Ideal | - Ideal |
| Learning rate | - 0.002 | - 0.002 |
| Batch size | - 100 | - 100 |
| Epoch | - 1000 | - 1000 |
| Loss function | - Mean absolute error | - Mean absolute error |
| Opimizer | - Adam | - Adam |
| Training dataset | - 23000 | - 23000 |
| Validation dataset | - 3000 | - 3000 |
| Test dataset | - 1000 | - 1000 |
| Framework for finetuning | - / | - Model agnostic meta learning |

- The CNN is mainly made up by two blocks stacked alternately, including Inception block and Squeeze-and-Extraction block. Specifically, Inception block is used to extract features of different scales, and the following Squeeze-and-Extraction block is used to add attention in the Channel dimension.  The Vision Transformer evolves from typical Transformer model widely used in natural language processing, and consists of an encoder of typical Transformer model and an additional Embedding block.