3GPP TSG RAN WG1 #109-e                                                           R1-2203555

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Other aspects on AI/ML for positioning accuracy enhancement

Agenda Item:	9.2.4.2

Document for:	Discussion and Decision

Introduction

Many positioning methods have been specified in Rel-16 and Rel-17 NR positioning, to obtain position estimation with target horizontal positioning accuracies <0.2 m (90%) for IIoT use cases and <1 m (90%) for commercial use cases. However, the performance of these positioning methods, highly relies on the existence of multiple LOS (line-of-sight) paths between the target terminal and multiple TRPs (Transmission-Reception Points). In the scenarios with extremely low LOS probability, positioning accuracy would decrease dramatically, which may be not able to satisfy the high-accuracy positioning requirements coming from new applications and industry verticals.

Target positioning requirement for commercial and IIoT use cases in Rel-17 are defined as follows:

- In Rel-17 target positioning requirements for commercial use cases are defined as follows: - Horizontal position accuracy (< 1 m) for [90%] of UEs - Vertical position accuracy (< [2 or 3] m) for [90%] of UEs - End-to-end latency for position estimation of UE (< [100 ms]) - FFS: Physical layer latency for position estimation of UE (< [10 ms]) - In Rel-17 target positioning requirements for IIoT use cases are defined as follows: - Horizontal position accuracy (< X m) for [90%] of UEs - X = [0.2 or 0.5] m - Vertical position accuracy (< Y m) for [90%] of UEs - Y = [0.2 or 1] m - End-to-end latency for position estimation of UE (< [10ms, 20ms, or 100ms]) - FFS: Physical layer latency for position estimation of UE (< [10ms]) Note: Target positioning requirements may not necessarily be reached for all scenarios

AI/ML technology has the powerful abilities in feature extraction, environment awareness, complex problem modeling and processing. In recent years, applying AI/ML into air-interface has attracted great attentions from academics to industries, and a lot of meaningful exploration has been made to verify the performance gain compared to conventional non-AL/ML schemes. Related research has also verified that AI/ML technology has the potential to significantly improve the performance of wireless communications.

Under this background, a new SI on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface has been agreed at RAN #94e [1], including three use cases to assess the applications of AI/ML in air-interface. Among them, AI/ML based positioning accuracy enhancement is included, with the target to improve the positioning accuracy for different scenarios, especially for some challenging scenarios with heavy NLOS (non-line-of-sight) conditions.

The objective of the new SI for RAN1 AI/ML based positioning includes the following:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

In this contribution, we present our views on possible sub use cases and analyze the potential specification impact for AI/ML based positioning accuracy enhancement according to the simulation results and observations in our companion contribution [2].

Sub use cases

For AI/ML based positioning, we present our views about possible sub use cases based on our evaluation, and also give some suggestions on other sub use case for future study.

Direct AI/ML and AI/ML assisted positioning

Depending on the role where AI/ML technology plays in positioning, the use cases for AI/ML based positioning accuracy enhancement can be divided into two types of sub use cases, including direct AI/ML positioning and AI/ML assisted positioning, as shown in Figure 1.

The first is to utilize AI/ML to estimate UE location with reference to some measurements and/or measurement reports directly, such as estimating location according to multiple TRPs’ CIRs.  This type of method may achieve relatively high positioning accuracy, while suffering from poor generalization capability since the AI model is strongly related to TRP’s geographical distribution.

The second is to utilize AI to extract intermediate feature from some measurements and/or measurement reports, and then estimate position according to the intermediate feature. For positioning accuracy enhancement, multiple intermediate features can be considered, such as time of arrival (TOA), Angle of Arrival (AoA), light-of-sight (LoS)/ NLoS condition, and so on. This type of method has advantages of better generalization capability, deployment flexibility, compatibility with existing positioning protocol framework, and possible positioning accuracy enhancement. In our companion contribution [2], we consider TOA as the intermediate feature, and showed that the positioning accuracy and generalization capability are better than direct AI/ML based positioning method. The results are also shown in Table 1.

Two possible sub use cases for AI/ML based positioning accuracy enhancement

CDF of positioning accuracy (m) of different positioning methods

| Scenario | Methods | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| InF-DH {0.6,6,2} | One-step (CIR-pos) training dataset and testing dataset from same drop | 0.35 | 0.49 | 0.70 | 0.99 |
|  | Two-step (CIR-TOA-pos) training dataset and testing dataset from same drop | 0.19 | 0.28 | 0.40 | 0.60 |
|  | One-step (CIR-pos) training dataset and testing dataset from different drops | 2.76 | 3.68 | 4.58 | 6.00 |
|  | Two-step (CIR-TOA-pos) training dataset and testing dataset from different drops | 0.93 | 1.27 | 1.73 | 2.51 |

CDF of positioning accuracy of different positioning methods

Depending on the role where AI technology plays in positioning, the use cases for AI/ML based positioning accuracy enhancement can be divided into two types of sub use cases, i.e., direct AI/ML positioning and AI/ML assisted positioning.

AI/ML assisted positioning has advantages in generalization capability, deployment flexibility, compatibility with existing positioning protocol framework, and possible positioning accuracy enhancement.

Other sub use cases

In addition to the above sub use cases of AI/ML for positioning accuracy enhancement, other sub use cases in positioning should be also considered for further study from the perspective of AI. Our suggested other sub use cases for consideration are listed as follows.

AI/ML for synchronization error: for timing based positioning methods, imperfect synchronization can result in considerable positioning error. This issue has been identified in study of RAT-dependent positioning methods in previous releases and we believe it also applies for AI/ML based positioning. For example, if CIR is adopted as the measurement and/or input for AI/ML model, imperfect synchronization will result in the offset of CIR pattern, which may further affect the positioning accuracy.

A unified AI/ML framework covering different use cases of this SI: there are three use cases included in the SID. Given they are related to each other, a unified AI/ML model and/or framework covering them all may be preferred. For example, whether predicted CSI or compressed CSI can be used for AI/ML based positioning? Whether positioning information can be used to facilitate CSI prediction or beam management? Whether AI/ML model can be shared among different use cases. A further study to these problems may help to improve overall efficiency of AI/ML for NR air-interface.

Potential specification impact

In this section, we analyze potential specification impact of AI/ML based positioning accuracy enhancement from the perspectives of model management and deployment.

General specification impact

According to the common functional framework for RAN intelligence illustrated in Figure 3 [3], we analyze the general specification impact from the perspective of lifecycle management for AI/ML based positioning accuracy enhancement.

Data Collection is a function that provides training dataset to model training and input data to model inference functions. An example of training data format is {CIR, location}. And accordingly, the input data to model inference functions is CIR. Specially, a data pre-processing procedure is necessary after data collection, such as CIR truncation and translation. Through data normalization, overhead for data collection may be able to decrease and model performance can be improved.

Model Training is a function that performs the AI/ML model training, validation, and testing which may generate model performance metrics as part of the model testing procedure. AI/ML model training can be further divided into two phases, i.e., model pre-training according to collected data and model update according to collect field data if the model is not working well.

Model Inference is a function that provides AI/ML model inference output (e.g. predictions or decisions). In different sub use cases, the output of model inference could be different, such as UE’s location and an intermediate feature.

Actor is a function that receives the output from the model inference function and triggers or performs corresponding actions. For AI/ML assisted positioning, the output from the Model Inference function is the intermediate feature, and then Actor performs position estimation according to the intermediate feature.

A common functional framework for RAN intelligence [3]

The model training usually requires large amounts of training data and computational resources. Considering limited battery capacity and available training data of mobile terminals, we think that model training with large scale of dataset should be avoided at UE side. Once trained, the model inference can be conducted at UE and/or network side though.

Model training with large scale of dataset should be avoided at UE side. The model inference can be conducted at UE and/or network side.

Direct AI/ML positioning

From the perspective of model deployment, this section discusses the detail specification impact for direct AI/ML positioning.

Model inference at UE side

When UE side performs model inference function, the detail specification impacts are listed as follows.

UE side needs to inform the network of device capability in advance. The device capabilities may consist of computation capability, available computation resource, available storage resource, battery status, and other capabilities related to model inference.

Based on the received device capability from the target UE, if network side identifies that the device can support model inference for an AI/ML model, network side delivers information of that model to the target UE. If not, network side can configure some non-AI positioning method for the target UE. The model information may consist of the input and output of model, the architecture of model,  the weight of model, the configuration of model, the state of optimizer, and so on. Among them, the input of model specifies which measurement is utilized for positioning, and the output of model is the location of the target UE for direct AI/ML positioning.

Information exchange procedure for direct AI/ML positioning when model inference is at UE side

For direct AI/ML positioning, UE capability corresponding to AI/ML model(s) is required when model inference is at UE side.

For direct AI/ML positioning, assistance information to the target UE about AI/ML model for inference is beneficial and necessary when model inference is at UE side.

  - Study further on the details of assistance information, which may consist of the input and output of model, the architecture of model, the weight of model, the configuration of model, the state of optimizer, and so on.

Model inference at network side

In this case, UE side does not need to inform network side of device capability, and network side does not need to deliver model information to the target UE. To enable direct AI/ML positioning, information exchange is still necessary. The detail specification impacts are listed as follows.

Network side should inform UE side of the required measurement type (e.g., CIR as input to AI/ML model for inference).

UE side reports requested measurement feedback.

Network side transfers positioning results to the target UE if the positioning application is originated from UE side.

Information exchange procedure for direct AI/ML positioning when model inference is at network side

For direct AI/ML positioning, when model inference is at network side, request to and feedback from the target UE of the necessary measurement (e.g., as the input to AI/ML model) for model inference is needed.

AI/ML assisted positioning

From the perspective of model deployment, this section discusses the detail specification impact for AI/ML assisted positioning.

Model inference at UE side

When UE side performs Model Inference function, the detail specification impacts are listed as follows.

UE side needs to inform network side of device capability in advance. The device capabilities may consist of computation capability, available computation resource, available storage resource, battery status, and other capabilities related to model inference.

Based on the received device capability from the target UE, if the network identifies that the device can support model inference for an AI/ML model, the network delivers information related to that model to the target UE. If not, the network can configure some non-AI positioning method for the target UE. The model information may consist of the input and output of model, the architecture of model,  the weight of model, the configuration of model, the state of optimizer, and so on. Among them, the input of model specifies which measurement type is utilized for model inference, and the output of model is the intermediate feature of the target UE for AI/ML assisted positioning.

For UE based positioning, the target UE acts as Actor computing position according to the intermediate feature related to multiple TPRs.  For UE assisted positioning, the target UE needs to report the intermediate feature to network side, and network side acts as Actor computing position according to the received intermediate feature. Then, network side may send positioning results to the target UE if positioning application originated from UE side.

Information exchange procedure for AI/ML assisted positioning when model inference is at UE side

For AI/ML assisted positioning, UE capability corresponding to AI/ML model(s) is required when model inference is at UE side.

For AI/ML assisted positioning, assistance information to the target UE about AI/ML model for inference is beneficial and necessary when model inference is at UE side.

  - Study further on the details of assistance information, which may consist of the input and output of model, the architecture of model, the weight of model, the configuration of model, the state of optimizer, and so on.

For AI/ML assisted & UE assisted positioning, support the target UE to report the output of AI/ML model inference (intermediate feature for positioning) when model inference is at UE side.

Model inference at network side

In this case, UE side does not need to inform network side of device capability, and network side does not need to deliver model information to the target UE. To enable AI/ML assisted positioning, interaction of assistance information is still necessary. The detail specification impacts are listed as follows.

Network side should inform UE side of the required measurement type (e.g., CIR as input to AI/ML model for inference).

UE side reports requested measurement.

Network side transfers positioning results to the target UE if the positioning application is originated from UE side.

Information exchange procedure for AI/ML assisted positioning when model inference is at network side

For AI/ML assisted positioning, when model inference is at network side, request to and feedback from the target UE of the necessary measurement (e.g., as the input to AI/ML model) for model inference is needed.

Fine-tuning

In our companion contribution [2], we have shown that fine-tuning can significantly improve model generalization performance. In this section, we further analyze detailed specification impact when fine-tuning is included.

Fine-tuning at UE side

For the case in which AI model inference is at UE side, fine-tuning can be conducted at UE side based on the pre-trained model. The detailed specification impacts are listed as follows.

UE side needs to inform the network of device capability in advance. The device capabilities may consist of computation capability, available computation resource, available storage resource, battery status, data collection, and other capabilities related to fine-tuning.

Based on the received device capability from the target UE, if the network identifies that the device can support model fine-tuning, network side delivers assistance information including pre-trained model information and training configuration to the target UE to support model fine-tuning. The model information may consist of the input and output of model, the architecture of model,  the weight of model, the configuration of model, the state of optimizer, and so on. The training configuration consists of learning rate, training epoch,  batch size,  end condition, and so on.

Information exchange procedure when fine-tuning is conducted at UE side

When fine-tuning is conducted at UE side, UE capability corresponding to fine-tuning is required.

To assist UE performing model fine-tuning, assistance information to the target UE about pre-trained model and training configuration is beneficial.

Fine-tuning at network side

- For the case that AI model inference is at UE or network side, fine-tuning can be conducted at network side based on the pre-trained model. In this case, UE side does not need to inform network side of device capability, and network side does not need to deliver model information and training configuration to the target UE. To support model fine-tuning at network side, UE should feedback some requested training data to network side. The detailed specification impacts are listed as follows.

Network side should send data collection request to the target UEs in advance. The data collection request may consist of data format, privacy permission, training data collection configuration, training data reporting configuration, and so on.

UE should provide training data to network side to support model fine-tuning.

Information exchange procedure when fine-tuning is conducted at network side

Training data collection request to and feedback from the target UE is required to support model fine-tuning at network side.

Conclusions

In this contribution, we discuss AI/ML based positioning accuracy enhancement with the following observations and proposals .

- AI/ML assisted positioning has advantages in generalization capability, deployment flexibility, compatibility with existing positioning protocol framework, and possible positioning accuracy enhancement.

- Depending on the role where AI technology plays in positioning, the use cases for AI/ML based positioning accuracy enhancement can be divided into two types of sub use cases, i.e., direct AI/ML positioning and AI/ML assisted positioning.

Model training with large scale of dataset should be avoided at UE side. The model inference can be conducted at UE and/or network side.

For direct AI/ML positioning, UE capability corresponding to AI/ML model(s) is required when model inference is at UE side.

For direct AI/ML positioning, assistance information to the target UE about AI/ML model for inference is beneficial and necessary when model inference is at UE side.

  - Study further on the details of assistance information, which may consist of the input and output of model, the architecture of model, the weight of model, the configuration of model, the state of optimizer, and so on.

For direct AI/ML positioning, when model inference is at network side, request to and feedback from the target UE of the necessary measurement (e.g., as the input to AI/ML model) for model inference is needed.

For AI/ML assisted positioning, UE capability corresponding to AI/ML model(s) is required when model inference is at UE side.

For AI/ML assisted positioning, assistance information to the target UE about AI/ML model for inference is beneficial and necessary when model inference is at UE side.

  - Study further on the details of assistance information, which may consist of the input and output of model, the architecture of model, the weight of model, the configuration of model, the state of optimizer, and so on.

For AI/ML assisted & UE assisted positioning, support the target UE to report the output of AI/ML model inference (intermediate feature for positioning) when model inference is at UE side.

For AI/ML assisted positioning, when model inference is at network side, request to and feedback from the target UE of the necessary measurement (e.g., as the input to AI/ML model) for model inference is needed.

When fine-tuning is conducted at UE side, UE capability corresponding to fine-tuning is required.

To assist UE performing model fine-tuning, assistance information to the target UE about pre-trained model and training configuration is beneficial.

Training data collection request to and feedback from the target UE is required to support model fine-tuning at network side.

References

3GPP RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”,  #94e, Dec.6-17, 2021.

vivo, R1-2203554, “Evaluation on AI/ML for positioning accuracy enhancement”,  RAN1 #109e, May 9-20, 2022.

Nokia, R3-222653, “(TR for TR 37.817)  General Framework”, #115-e, Feb.21– Mar.3, 2021.