3GPP TSG RAN WG1 #109-e		R1- 2203566

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Discussion on potential solutions for sidelink positioning

Agenda Item:	9.5.1.3

Document for:	Discussion and Decision

Introduction

In RAN #94e e-meeting, study solutions for sidelink positioning has been captured in FS_NR_pos_enh2 [1]. In this contribution, we present our views on the potential solutions for SL positioning.

- Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1]

Positioning techniques

There are many RAT-dependent positioning techniques introduced in Rel-16, such as DL-TDOA, UL-TDOA, DL-AOD, UL-AOA, and multi-RTT techniques.

In the section, we present our views on reusing those techniques or introducing some enhancement for SL positioning.

Rel-16 positioning techniques

In this section, we evaluate whether those methodologies can be solutions for SL positioning of relative positioning, ranging, and absolute positioning.

Table 1 The positioning techniques for achieving relative positioning, ranging, and absolute positioning

|  | relative positioning | ranging for distance | ranging for angle | absolute positioning |
| --- | --- | --- | --- | --- |
| RTT | × |  | × | × |
| Multi-RTT |  |  | × | when multiple nodes’ locations (i.e., RSU) can be known |
| TDOA |  |  | × | when multiple nodes’ locations (i.e., RSU) can be known |
| AoA |  |  |  |  |
| AoD |  |  |  |  |
| RTT+AoA |  |  |  | when one UE’s location is known |
| RTT with Multi-panel |  |  |  |  |

Based on the analysis in Table 1, TDOA is difficult to achieve the relative positioning or ranging for distance between two UEs unless under perfect synchronization conditions. So, in our view, RTT should be supported at least for distance ranging and relative positioning. In addition, AoA also need to be supported for the angle ranging. It is observed in our companon contribution [2],with limited number of antenna on the UE side, the measured angle performance is only 14 degrees for 2 antenna case. Therefore, in this case, we suggest

- Prioritize RTT and AoA for sidelink positioning considering to achieve relative positioning and ranging (e.g., ranging for distance and ranging for angle).

In addition, we prefer to prioritize sidelink-based positioning. The combination of SL positioning with Uu positioning can be seen as a low priority.

- Prioritize sidelink-based positioning. The combination  of SL positioning with Uu positioning is of low priority.

RTT enhancement for sidelink positioning

In Rel 16, the UE Rx – Tx time difference is defined as TUE-RX – TUE-TX  for Multi-RTT.

Where:

- TUE-RX is the UE received timing of downlink subframe #i from a Transmission Point (TP), defined by the first detected path in time.

- TUE-TX is the UE transmit timing of uplink subframe #j that is closest in time to the subframe #i received from the TP.

Based on the discussion of Rel-17, the UE Rx – Tx time difference performance will be impacted by the timing changes/timing adjustment between UE received timing and the UE transmit timing. So, many companies suggest modifying the definition of UE Rx – Tx time difference for reducing the impact of TA.

Considering the synchronization structure of SL positioning,  we prefer the UE Rx – Tx time difference can be defined as the difference between UE received timing of SL-PRS and UE transmitted timing of SL-PRS (e.g., Tround in Figure 1).

Figure 1 Sidelink RTT positioning

So, the propagation delay T can be estimated as:

T=12(Tround-Treply)

The clock error for RTT for SL positioning

TS 38101-1-h10 6.4E.1.1 The UE modulated carrier frequency for NR V2X sidelink transmissions in Table 5.2E.1-1, shall be accurate to within ±0.1 PPM observed over a period of 1 ms compared to the absolute frequency in case of using GNSS synchronization source. The same requirements applied over a period of 1 ms compared to the carrier frequency received from the gNB or V2X synchronization reference UE in case of using the gNB or V2X synchronization reference UE sidelink synchronization signals.

Based on TS 38101-1, if two UE has the same synchronization source, the clock error between 2 UEs will be within ±0.1 PPM. But if the synchronization source is different, the clock error will be larger than ±0.1 PPM. With the clock error between two UEs, the potential error will be introduced in the estimated propagation delay as follows:

error=T-T=12()≈12(eUE1-eUE2)×Treply

Where eUE1 and eUE2 is the clock offset of UE 1 and UE 2.

Based on the above equation, the error will increase as the increasing of Treply. For example, even though the clock error between two UEs is 0.2 PPM ( that is eUE1-eUE2=0.2 PPM), the error can be 50ns if the Treplyassumed as 1s.

Table 2 The estimated propagation delay errors with different clock errors and Treply

| Clock error Treply | 0.2 ppm | 2 ppm | 5 ppm | 10 ppm | 20 ppm |
| --- | --- | --- | --- | --- | --- |
| 1ms | 0.05ns | 0.5ns | 1.25 ns | 2.5 ns | 5ns |
| 10ms | 0.5ns | 5ns | 12.5 ns | 25 ns | 25 ns |
| 100ms | 5ns | 50ns | 125 ns | 250 ns | 250 ns |
| 1s | 50ns | 500ns | 1250 ns | 2500 ns | 2500 ns |

- The estimated propagation delay errors for RTT will be increased with the increase of Treply, which is the difference between UE transmitted timing of SL-PRS and UE received timing of SL-PRS.

- 50ns estimated errors will be introduced in RTT if the clock error between two UEs is assumed as 0.2 PPM and Treply is assumed as 1s.

Double-side RTT

Double-side RTT is widely used for UWB positioning and can reduce the impact of clock error. Take the following figure as an example, the propagation delay T can be estimated by two measurements (i.e., Tround1 , Tround2, Treply1, Treply2), that is:

- T=12(Tround1-Treply1)

- T=12(Tround2-Treply2)

And then, we can found Tround1×Tround2-Treply1×Treply2 can be

- Tround1×Tround2-Treply1×Treply2=4T2+2TTreply1+Treply2=TTround1+Tround2+Treply1+Treply2

Where

Tround1×Tround2=2T+Treply12T+Treply2=4T2+2TTreply1+Treply2+Treply1×Treply2

So,  the propagation delay T can be estimated as:

T=Tround1×Tround2-Treply1×Treply2Tround1+Tround2+Treply1+Treply2

Figure 2 double side RTT

In this case, the error of propagation delay estimation due to clock error can be :

error=T-T≈(eUE1+eUE2)2T

Where

eUE1 and eUE2 is the clock offset of UE 1 and UE 2;

T is estimated propagation delay between UE1 and UE 2.

- The error of propagation delay estimation due to clock error for double-side RTT will be significantly reduced compared with that of RTT method considering the estimated propagation delay will be smaller than the CP length and the error of propagation delay estimation for double-side RTT  is the multiple of clock error and the estimated propagation delay.

- Around 0.02ns estimated errors will be introduced in double-side RTT if the clock error between two UEs is assumed as 0.2 PPM.

- Double-side RTT should be considered for SL positioning.

Positioning enhancement with multi-panel

In NR positioning enhancement, ARP and TEG ID have been introduced for multi-panels to achieve high accuracy positioning.

The current antenna configuration in TR37.885 for V2X is placed as option 2 shown below which is a distributed antenna system. Besides, according to the analysis in the 5GAA LS, the distributed antennas on the vehicle may affect the positioning performance and reduce the number of gNB/RSU.

TR 37.885 6.1.4[6] - Option 2: Two panels are placed in the vehicle as follows and the antenna pattern for each location is given by Tables 6.1.4-10 and, 6.1.4-11. The antenna array configuration is given by Table 6.1.4-12. - For vehicle type 1, one panel at the front bumper and one panel at the rear bumper - For vehicle type 2, one panel at the front rooftop and one panel at the rear rooftop - For vehicle type 3, one panel at the front rooftop and one panel at the rear rooftop

In addition, we evaluate positioning metrics based on the existing antenna configuration and panel distribution (that is option 2 for the vehicle) in V2X positioning. And then, we can find the performance gain with the distributed antenna systems for V2X positioning [2].

Therefore, we propose

- With distributed antenna systems, the positioning performance will be improved compared to single-panel antenna systems.

- Two antenna panels as the distributed antenna system can be considered for V2X positioning.

SL PRS design

In Rel-16, DL-PRS has been introduced for NR positioning. In this section, we present our views on SL positioning signal design for SL positioning.

Candidates for SL-PRS signal

Table 3 Summary of candidates for SL-PRS signal

|  | Bandwidth | Port | Application scope | Sequence ID | Pattern |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Combsize N | Symbol number | stagger |
| SL SSB | 11 RBs |  | / | (0..671) | / | 2 symbols for PSS and SSS | / |
| SL PSCCH DMRS | Within PSCCH | One port | / | Pre-configured ID {0, …, 65536} Per resource pool | Comb 4 | Every symbol of PSCCH | / |
| SL PSSCH DMRS | Within PSSCH | One or Two-port | / | 16-bit LSB of CRC of 1st SCI | Comb 2 | {2,3,4} | / |
| SL CSI-RS | Within PSSCH | One or Two-port | Only for unicast | 10-bit LSB of CRC of 1st SCI |  |  | / |
| SL PTRS | Within PSSCH | One or Two-port | Only for FR2 Only for unicast | 16-bit LSB of CRC of 1st SCI |  | 1 | / |
| DL-PRS | 24:4:272 RB | One | NR | {0, …, 4095} | {2, 4, 6, 12} | 1~14 | Full stagger |
| UL-SRS | Within active BWP | One | NR | {0, …, 65536} | {2, 4, 8} | {1 2 4 8 12} | Full stagger |

Based on the summary in Table 3, SSB, SL-DMRS, and SL-PTRS are not suitable as SL positioning signal directly considering those signal limitations, such as 11 RB bandwidth of SSB, CDM group architecture for DMRS, only applicable for FR2 for PTRS.

In addition, considering SL CSI-RS only can be transmitted within PSSCH, it is difficult to guarantee the bandwidth of PSSCH can satisfy the positioning requirement (e.g., 1-3m positioning accuracy).

We prefer to refer to the principle of both NR positioning and sidelink signal (e.g., CSI-RS) to design the new SL positioning reference signal (i.e., SL-PRS) for SL positioning.

- Introduce a new SL reference signal (i.e., SL PRS) for SL positioning.

SL-PRS sequence generation

In Rel-16, the rm of DL-PRS is defined by

rm=121-2c2m+j121-2c2m+1

where the pseudo-random sequence ci  shall be initialized with

cinit=222nID,seqPRS1024+210Nsymbslotns,fμ+l+12nID,seqPRS mod 1024+1+nID,seqPRS mod 1024

And the nID,seqPRS can be used to identify the PRS sequence (e.g to identify the  PRS from different TRP).

But the current SL signal NID is 10/16 bit of CRC of corresponding 1st SCI, which is different from the Rel-16 positioning signal. So we have some concerns about directly reusing the NID for RTT positioning for SL PRS, for example

- Any UE can get  NID based on the 1st SCI and measure the positioning signal, so that it may expose some UE information.

- For RTT positioning, maybe some relationships are needed between SL-PRS1 from UE1 and SL-PRS2 from UE2 so that UE1 knows SL-PRS2 is a feedback signal.

Therefore, in our view, we prefer the NID of SL-PRS can be associated with some UE information (e.g pre-configured sequence ID, source ID). In this case, if other UEs cannot obtain those information in advance, other UEs cannot measure the corresponding SL-PRS successfully.

- The NID of SL PRS can be associated with some UE information (e.g., pre-configured sequence ID, source ID).

SL-PRS pattern

Firstly, considering the principle of  SL design, AGC and GP symbol for SL PRS pattern are needed. For AGC symbol, the receiver collects the energy around and adjusts the gain of power amplifier accordingly to make the power amplifier operate in a linear operating range, which keeps the signal from being distorted. For GP symbol, the UE can switch between transmission and reception during the symbol. And a SL UE is not expected to receive any signal during the GP symbol.

Secondly, in Rel-16, the pattern of DL PRS can be referred, including the comb size KcombPRS∈2, 4, 6,12 as the following Figure 3, and the combination LPRS,KcombPRS is one of {2, 2},{4, 2}, {6, 2}, {12, 2}, {4, 4}, {12, 4}, {6, 6}, {12, 6} and {12, 12}.

Figure 3 The DL-PRS pattern

For us, reusing one or more comb sizes of DL-PRS for SL-PRS is reasonable. In addition, considering SL structure (e.g, excluding the PSCCH symbol, AGC, or GP symbol for SL-PRS transmission), the number of PRS  (ie.LPRS∈2,4,6,12) may need to be enhanced for SL-PRS transmission. For example, 12 symbols are impossible for SL-PRS transmission if 2 or more symbols are assumed to be PSCCH, AGC and GP. And given the limited communication range between UEs for SL positioning, we think partial staggered pattern can be supported in SL-PRS.

- AGC and GP symbol are needed for the SL PRS pattern.

- Support reuse of one or more comb sizes of DL-PRS for the SL-PRS pattern.

- Partial staggered pattern can be considered for SL-PRS pattern considering SL structure (e.g, excluding the PSCCH symbol, AGC, or GP symbol for SL-PRS transmission)

SL-PRS format

With the analysis of SL-PRS, multiplexing SL-PRS onto PSSCH will reduce the positioning performance. Transmitting SL-PRS within PSSCH bandwidth is also difficult to guarantee the positioning accuracy. So, we prefer to transmit SL-PRS without PSSCH  transmission in the resource pool.

In this case, we provide a potential slot format for SL-PRS design as Figure 4. Considering autonomous resource allocation, the SCI at least for sensing should be transmitted with SL-PRS, which would reduce the probability of collision with other transmissions. And the receiver can receive the PRS according to the scheduling of the SCI.

Figure 4 the slot format of SL-PRS

In legacy NR sidelink, only slot level transmission is supported. In our opinion, the slot level SL PRS transmission should be supported at least as the baseline. Therefore, we propose

- SL PRS can be transmitted without PSSCH transmission in the resource pool.

- SCI should be transmitted with the associated SL PRS.

- Slot level SL PRS transmission should be supported.

Resource pool for SL-PRS

For the resource pool or BWP of SL PRS, we think it should be discussed. In NR SL, only one SL BWP is supported on a carrier. And resource pool is configured in the BWP. In our opinion, the SL PRS is transmitted in the BWP, and restricted in a resource pool. There are the following alternatives  for SL PRS transmission in a resource pool.

- Alt 1. Dedicated resource pool for SL PRS and potential associated PSCCH.

- Alt 2. Shared resource pool for SL PRS and legacy PSCCH/PSSCH/PSFCH, etc.

For Alt 1, it would be straightforward for SL PRS transmission, and the impact on the R16/R17 SL transmission is small. However, it might decrease the frequency efficiency on that SL carrier. For Alt 2, the backward compatibility for legacy SL transmission should be considered. The bandwidth of SL PRS may exceed the bandwidth of the PSSCH in order to satisfy the positioning accuracy, which would have an impact on the legacy SL transmission. Considering both alternatives having pros and cons, we slightly prefer alt 1 for SL PRS transmission.

- A dedicated resource pool should be studied for SL PRS transmission in Rel-18.

Resource allocation for SL-PRS

According to the SID, the resource allocation mechanism for sidelink reference signals for positioning purposes should be studied. In NR sidelink, mode 1 and mode 2 resource allocation are specified. SL mode 2 resource allocation is UE autonomous resource allocation, for which the UE performs sensing and selects resources in the candidate resource set. For SL mode 1 resource allocation, the resource is based on the scheduling of gNB. In our opinion, both mode 1 and mode 2 resource allocation should be studied for SL-PRS.

- SL mode 1 and mode 2 resource allocation mechanisms should be studied.

The mode 2 resource allocation should be the baseline, which can work in in-coverage, out-of-coverage and partial coverage scenarios, and should be studied first.

In mode 2 resource allocation, UE performs sensing in the sensing window and operates resource exclusion in the resource selection window. Then, UE selects resource in the remaining resource in the resource selection window after resource exclusion. Therefore, the sensing procedure and resource selection should be defined for SL PRS in a resource pool.

For the sensing procedure, in NR sidelink, the UE performs SCI decoding and RSRP measurement. The RSRP measurement can be based on PSCCH DMRS and PSSCH DMRS which are determined by configuration. For SL positioning, if a dedicated resource pool is used for SL PRS and the associated SCI, the UE performs SCI decoding to recognize the resource reserved by other UEs. The RSRP measurement can be derived based on SL PRS, considering that PSSCH DMRS is not available and SL PRS can be measured in that resource pool. Then, UE performs resource exclusion based on SCI decoding and RSRP measurement in the resource selection window.

For the resource selection procedure, UE reports some candidate resources remaining in the resource selection window to the higher layer. Then, the higher layer would select resources in the reported candidate resources.

One example of mode 2 resource selection for SL PRS is illustrated in Figure 5.

Figure 5 An example for mode 2 resource selection for SL PRS

- SL mode 2 resource allocation can be used for SL PRS resource allocation with some modification (e.g., the SL PRS is used for RSRP measurement).

The measurement and reporting for SL-PRS

In this section, some enhancements and considerations for measurement and report in SL Positioning are discussed.

Based on the discussion of Rel-17, the combination of multiple measurement results is suggested under different positioning methods. For example, timing information is suggested to be reported in AoD/AoA positioning. So, in sidelink positioning, we propose to refer to the UL positioning method to choose one or more measurement results reporting for SL positioning other than reporting hybrid measurement for each method.

- The unified SL Positioning method can be introduced to support reporting one or more measurement results, similar to the TRP measurement result in TS 38.455.

Rx – Tx time difference for SL positioning

Based on the explanation in 2.2, we propose the UE Rx – Tx time difference for SL positioning can be defined as the timing gap between transmission of SL-PRS and receiving SL-PRS from other UE.

That is, UE Rx – Tx time difference is the timing gap between the UE received timing of SL -PRS 1 from UE #i, defined by the first detected path in time, and the UE transmit timing of SL -PRS 2 from the UE.

- Suggest UE Rx – Tx time difference for SL positioning can be defined as T, which is the timing gap between transmission of SL-PRS and receiving SL-PRS to/from other UE.

SL-PRS RSRP and SL-PRS RSRPP

DL-PRS RSRP has been supported in Rel-16, and DL-PRS RSRPP has been supported in Rel-17. For us, with limited number of antennas on the UE side, the path RSRP may not be needed for SL positioning. In addition, with the SL PRS design in section 3, the SL-PRS RSRP may be helpful for resource selection.

Therefore, we propose

- SL-PRS RSRP is needed to be introduced for SL-PRS measurement and reporting.

Angle of Arrival for SL positioning

Even though the angle estimation is limited based on the UE antenna number, the AoA measurement and reporting are needed for angle ranging.

- SL-AoA is needed to be introduced for SL-PRS measurement and reporting.

Enhancements of physical-layer procedure

Physical procedures for positioning enhancement should also be considered for objectives such as accuracy, latency and network/UE efficiency.  Some enhancements and considerations are discussed below.

SL-PRS design for unicast, groupcast and broadcast

Based on the SID, potential solutions for SL positioning of relative positioning, ranging, and absolute positioning, are needed to be studied.

Firstly, for SL positioning, unicast can be considered for one-to-one relative positioning.

Besides, for groupcast  with preconfigured group information,  if UE is within the group, and needs to know the relative positioning with different members, we think groupcast SL-PRS can be considered for the relative positioning within the group.

In addition, given RSU has been supported in V2X, using RSU for absolute positioning is logical for SL positioning. In this case, RSU can be seen as TRP, which can broadcast SL-PRS for other UEs to achieve absolute positioning,

Therefore, for us, unicast, groupcast and broadcast should be studied in Rel-18.

- Unicast, groupcast and broadcast should be studied for SL positioning in Rel-18.

Power control of SL-PRS for positioning

In NR sidelink, open loop power control is supported for PSCCH/PSSCH, and the power control of SL is based on DL pathloss or SL pathloss (only for unicast). The DL pathloss should be taken into account to avoid seriously interference to the UL transmission performance. Additionally, if distance between TX UE and RX UE is small, the power control based on SL pathloss can be considered. It would decrease the power of TX UE which satisfies the performance of the SL transmission and can reduce the interference to other UEs.

Figure 6 Open-loop power control for SL PRS

- Open-loop power control scheme should be studied for SL PRS.

Conclusion

In this contribution, we discuss potential positioning enhancements with the following observations and proposals.

- The estimated propagation delay errors for RTT will be increased with the increase of Treply, which is the difference between UE transmitted timing of SL-PRS and UE received timing of SL-PRS.

- 50ns estimated errors will be introduced in RTT if the clock error between two UEs is assumed as 0.2 PPM and Treply is assumed as 1s.

- The error of propagation delay estimation due to clock error for double-side RTT will be significantly reduced compared with that of RTT method considering the estimated propagation delay will be smaller than the CP length and the error of propagation delay estimation for double-side RTT  is the multiple of clock error and the estimated propagation delay.

- Around 0.02ns estimated errors will be introduced in double-side RTT if the clock error between two UEs is assumed as 0.2 PPM.

- With distributed antenna systems, the positioning performance will be improved compared to single-panel antenna systems.

- Prioritize RTT and AoA for sidelink positioning considering to achieve relative positioning and ranging (e.g., ranging for distance and ranging for angle).

- Prioritize sidelink-based positioning. The combination  of SL positioning with Uu positioning is of low priority.

- Double-side RTT should be considered for SL positioning.

- Two antenna panels as the distributed antenna system can be considered for V2X positioning.

- Introduce a new SL reference signal (i.e., SL PRS) for SL positioning.

- The NID of SL PRS can be associated with some UE information (e.g., pre-configured sequence ID, source ID).

- AGC and GP symbol are needed for the SL PRS pattern.

- Support reuse of one or more comb sizes of DL-PRS for the SL-PRS pattern.

- Partial staggered pattern can be considered for SL-PRS pattern considering SL structure (e.g, excluding the PSCCH symbol, AGC, or GP symbol for SL-PRS transmission).

- SL PRS can be transmitted without PSSCH transmission in the resource pool.

- SCI should be transmitted with the associated SL PRS.

- Slot level SL PRS transmission should be supported.

- A dedicated resource pool should be studied for SL PRS transmission in Rel-18.

- SL mode 1 and mode 2 resource allocation mechanisms should be studied.

- SL mode 2 resource allocation can be used for SL PRS resource allocation with some modification (e.g., the SL PRS is used for RSRP measurement).

- The unified SL Positioning method can be introduced to support reporting one or more measurement results, similar to the TRP measurement result in TS 38.455.

- Suggest UE Rx – Tx time difference for SL positioning can be defined as T, which is the timing gap between transmission of SL-PRS and receiving SL-PRS to/from other UE.

- SL-PRS RSRP is needed to be introduced for SL-PRS measurement and reporting.

- SL-AoA is needed to be introduced for SL-PRS measurement and reporting.

- Unicast, groupcast and broadcast should be studied for SL positioning in Rel-18.

- Open-loop power control scheme should be studied for SL PRS.

- References

- RP-213588 Study on expanded and improved NR positioning, Dec. 6 – 17, 2021

- R1-2203565 Discussion on evaluation methodology in sidelink positioning, vivo, May 9th – 20th, 2022.