3GPP TSG RAN WG1 #109-e		R1-2203568

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Discussion on carrier phase measurement enhancements

Agenda Item:	9.5.2.2

Document for:	Discussion and Decision

Introduction

In RAN #94e Meeting, the SID on study on expanded and improved NR positioning RP-213588[1] was approved including the following objective related to carrier phase measurement:

- Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4] - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1] - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary

Carrier phase-based positioning

In GNSS, carrier phase-based positioning as a high-precision positioning method is relevant to many issues, for example:

- Carrier phase measurement

- Integer ambiguity

- Cycle slipes

- The time/frequency error

- The reference node

- The joint positioning method

But, based on the scope of SID, we would like to focus on the NR carrier phase measurements first. In addition, considering carrier phase measurement is a totally new measurement, to better align companies’ understanding, the selected measurement method and corresponding phase measurement performance should be provided by companies.

- Focus on NR carrier phase measurements in the study of carrier phase positioning.

- The selected measurement method and corresponding phase measurement performance should be provided by companies.

Besides, there are many issues that will impact the accuracy of carrier phase measurement, including:

- Multi-path

- Frequency error (e.g., dropper frequency offset)

- Clock offset

- Phase error

- Cycle slipes

So, we think the phase measurement performance shall be evaluated with errors (e.g., multi-path, frequency error, clock offset).

- The phase measurement performance should be evaluated with errors(e.g., multi-path, frequency error, clock offset).

The methodology of case phase measurement

In this section, we present our views on carrier phase measurement methodology.

Figure 1. Illustration of UE carrier phase positioning

The carrier phase measurement for existing signal

The carrier phase measurement in the frequency domain

The channel frequency response:

Hk=alose-j2π(f0+k∙∆f)τlos+l=1male-j2πk(f0+k∙∆f)τl

given, 
λ=νf=vf0                 τ=rc=rλf0

where r is the distance between BS and UE

The channel frequency response can be described as

Hk=alose-j2π(f0+k∙∆f)τlos+l=1male-j2π(f0+k∙∆f)τl
=alose-j2π(f0+k∙∆f)rlosλf0+l=1male-j2π(f0+k∙∆f)rlλf0=alose-j2π1+k∆ff0Nlos+φlos+l=1male-j2π1+k∆ff0Nl+φl

Where Nlos is the number of Integer cycles, and φlos is a fractional phase in a cycle

Based on the above formulate, and in LoS case, if the alos≫al, h0 can be described as

H0=alose-j2π1+0∙∆ff0Nlos+φlos
Option a: the carrier phase φ can be calculated as follows based on single carrier

φ=arctanQH(0)IH(0)

In addition, the PRS is transmitted in multiple carriers, and the LoS component in channel frequency response can be described as

Hlosk=alose-j2π1+k∆ff0Nlos+φlos=alose-j2πNlos+φlos+k∆ff0Nlos+φlos

Then, the carrier phase φk in the carrier k can be

φk=Nlos+φlos+k∆ff0Nlos+φlos

So, the carrier phase in multiple carriers can be equivalent to a linear function, which is the slope (Nlos+φlos) and  intercept(Nlos+φlos), and we can calculate the intercept φlos based on multiple carriers.

Option b: the carrier phase φ (that is the intercept/slope )can be calculated according to the (k, φk ) of multiple carriers, for example:

argmin φφk-(a∙k+φ)2

The carrier phase measurement in the time domain

Figure 2.  the example of phase calculation in the time domain

Option C: the phase φ can be calculated in the time domain as

φ=arctanQCIR(τlos)ICIR(τlos)

But we wonder the φ can be seen as the carrier phase, since it will introduce errors due to the ∆f in multiple carriers.

The carrier phase measurement with high-resolution algorithm

Option d: the carrier phase measurement with a high-resolution algorithm in the time domain

To achieve the carrier phase of the first path, we can analogy with a phase detector and calculate it in the time domain with different θi as following steps

Step 1: Give a set θi(rad):{-π:0.01:π}

Step 2: Generate a set of phase-detection signals uotaccording to the aboveθi

uot=cosωot+θi

Step 3: The received I/Q signal can be assumed as

yt=sinωot+φ

Step 4: multiple uot and yt can get

yt∙uot=sinωot+φ∙cosωot+θi=(sin2ωot+φ+θi+sin⁡(ωo-ωot+φ-θi))/2

Step 5: After a low-pass filter,   yt∙uot will be

LPF(yt∙uot)=sin⁡(ωo-ωot+φ-θi)

Step 6: Search an θ from θi(rad):{-π:0.01:π} that maximizes LPF(yt∙uot)

Option f: the carrier phase measurement with a high-resolution algorithm in the frequency domain

To achieve the carrier phase of the first path, we can do a high-resolution search algorithm in the frequency domain with different θi .

Step 1: Give a set θi(rad):{-π:0.01:π}

Step 2: Generate a set of phase-detection signals u(θi) in the frequency domain according to the above θi

uk,θi=ejwkt+θi

uθi=ejw0t+θiejw1t+θi⋮ejwnt+θi

Step 3: The received signal in the frequency domain can be assumed as Y(k)

Step 4: Search an θ from θi(rad):{-π:0.01:π} that maximizes xθi

- xθi=1NkYkuk,θi

- argmaxθi(xθi)

The carrier phase measurement for CDMA signal

Figure 3.  An example of PLL in GNSS

In addition to methodologies above, in GNSS, Phase-locked loop (PLL)-based method is used to lock the phase and is widely used for phase tracking and locking, which can generate an output signal whose phase is related to the phase of an input signal. It mainly includes a Phase detector, Low-pass filter, Voltage controlled oscillator, and Feedback path (which may include a frequency divider). The oscillator generates a periodic signal, and the phase detector compares the phase of that signal with the phase of the input periodic signal, adjusting the oscillator to keep the phases matched.

The summary of the methodology for carrier phase measurement

Table 1 The analysis of candidate SL-PRS signal

| methodology | Pros | Cons |
| --- | --- | --- |
| The carrier phase measurement based on PLL | The mature solution in the GNSS field on phase measurement can be reused | A new signal is needed, and an additional electronic circuit for PLL may be needed |
| The carrier phase measurement in the time/ frequency domain | Simple and easy to implement | Potential performance loss |
| The carrier phase measurement with high-resolution algorithm | Medium complexity | / |

Based on the SID “Focus on reuse of existing PRS and SRS”, we suggest evaluating the carrier phase performance with the existing signal first, and evaluating the performance gain.

- Support evaluating the carrier phase performance with the existing signal first, and comparing the performance gain with the existing NR method.

- Don’t introduce a new signal for the carrier-phase measurement without the necessary reason.

Priliminary evaluation for carrier phase measurement

In this section, we focus on evaluation of existing signal with frequency domain measurement. The preliminary evaluation results are captured as the following.

Figure 4. The performance of carrier phase measurement (radian) in the frequency domain

Table 2 The performance of carrier phase measurement (radian) with Option C

| Assumptions | scenario | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 1], [phase carrier measurement], [FR1] [Option C in 3.1.2] (radian) | SH | 0.16 | 0.24 | 0.31 | 0.43 |
|  | DH | 0.19 | 0.29 | 0.44 | 1.36 |

Based on the evaluation results, we make the following observation.

Observation 1

- For the carrier phase measurement in the frequency domain with option c, the performance accuracy is 0.43 radian @90% in InF- SH.

- For the carrier phase measurement in the frequency domain with option c, the performance accuracy is 1.36 radian @90% in InF- DH.

Observation 2:

- The positioning performance of carrier phase measurement degraded significantly with the multiple path/NloS scenario.

Conclusion

In this contribution, we discuss potential enhancement for carrier phase measurement with the following observations and proposals.

Observation 1

- For the carrier phase measurement in the frequency domain with option c, the performance accuracy is 0.43 radian @90% in InF- SH.

- For the carrier phase measurement in the frequency domain with option c, the performance accuracy is 1.36 radian @90% in InF- DH.

Observation 2:

- The positioning performance of carrier phase measurement degraded significantly with the multiple path/NloS scenario.

- Focus on NR carrier phase measurements in the study of carrier phase positioning.

- The selected measurement method and corresponding phase measurement performance should be provided by companies .

- The phase measurement performance should be evaluated with errors(e.g., multi-path, frequency error, clock offset).

- Support evaluating the carrier phase performance with the existing signal first, and comparing the performance gain with the existing NR method.

- Don’t introduce a new signal for the carrier-phase measurement without the necessary reason.

- References

- RP-213588 Study on expanded and improved NR positioning, Dec. 6 – 17, 2021