3GPP TSG RAN WG1 #109-e		R1-2203569

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Discussion on Low Power High Accuracy Positioning

Agenda Item:	9.5.2.3

Document for:	Discussion and Decision

Introduction

At the RAN#94e meeting, the SID on Study on expanded and improved NR positioning RP-213588[1] was approved including the following objective related to Low power and high accuracy positioning (LPHAP):

- Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1] - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study. - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state

In this contribution, we will present our views and preliminary evaluation results of LPHAP.

Use cases of LPHAP

SA1 has introduced use cases and requirements for LPHAP (Low Power High Accuracy Positioning) for industrial IoT scenarios. As mentioned in [2], low power high accuracy positioning is an integral part of a considerable number of industrial applications. The total energy needed for a specific operation time for such a low power high accuracy positioning optimized IoT-device is a combination of energy for positioning (varies depending on the used positioning method), energy for communication/‌synchronization and a difficult to predict factor to take additional losses through e.g. security, power management, microcontroller, and self-discharge of batteries into account. Examples of target applications for low power high accuracy positioning are asset tracking in process automation, tracking of vehicles, and tool tracking. Table 1 gives an indication of the required operation time of the 5G enabled IoT device and duty cycle of the updated position information for different use cases.

Table 1: Low power high accuracy positioning use cases

| Use Case # | Horizontal accuracy | Corresponding service level (22.261) | Positioning interval/ duty cycle | battery life time/ minimum operation time |
| --- | --- | --- | --- | --- |
| 1 | 10 m | Service Level 1 | on request | 24 months |
| 2 | 2 m to 3 m | Service Level 2 | < 4 seconds | > 6 months |
| 3 | < 1 m | Service Level 3 | no indication | 1 work shift - 8 hours (up to 3 days, 1 month for inventory purposes) |
| 4 | < 1 m | Service Level 3 | 1 second | 6 - 8 years |
| 5 | < 1 m | Service Level 3 | 5 seconds - 15 minutes | 18 months |
| 6 | < 1 m | Service Level 3 | 15 s to 30 s | 6 - 12 months |
| 7 | 30 cm | Service Level 5 | 250 ms | 18 months |
| 8 | 30 cm | Service Level 5 | 1 second | 6 - 8 years (no strong limitation in battery size) |
| 9 | 10 m | Service Level 1 | 20 minutes | 12 years (@20mJ/position fix) |

The specific use cases are as follows

- Process automation: Dolly tracking (outdoor).

- Process automation: Asset tracking.

- Flexible modular assembly area: Tool tracking in flexible, modular assembly areas in smart factories.

- Process automation: Sequence container (Intralogistics).

- Process automation: Palette tracking (e.g. in turbine construction).

- Flexible modular assembly area: Tracking of workpiece (in- and outdoor) in assembly area and warehouse.

- Flexible modular assembly area: Tool assignment (assign tool to vehicles in a production line, left/right) in flexible, modular assembly area in smart factories.

- Flexible modular assembly area: Positioning of autonomous vehicles for monitoring purposes (vehicles in line, distance 1.5 meter).

- (Intra-)logistics: Asset tracking

Based on one of objectives of LPHAP, companies are required to review use case 6 at the start of the study. From our point of view, use case 6 is a more reasonable choice. On the one hand, it is a typical tool tracking use case in industrial IoT scenario, and on the other hand, it does not bring extremely strict power consumption requrement (more than 1 year of battery life) as other use cases.

- Confirm that study is limited to a single representative use case (use case 6 as defined TS 22.104) for LPHAP.

Regarding the evaluation of LPHAP, we tend to only evaluate the power consumption, because the evaluation of accuracy has been evaluated in R16/17, and duplication of work should be avoided.

- Accuracy evaluation is not considered for LPHAP.

Power consumption evaluation of LPHAP

According to the objectives, quantitative power consumption evaluation is needed. It may study the power model in inactive and/or idle state. We need to evaluate whether the current solution in inactive state and/or idle state can meet the power consumption requirements of LPHAP, and identify feasible solutions to reduce power consumption. Considering UE power saving study has been completed in Rel-16, and the conclusions and methods of that study have been captured in TR38.840 [3]; we can largely reuse their models and the methods when evaluating positioning power consumption, which greatly reduces the complexity of quantitative evaluation.

It should be noted that the baseline power model has no difference for inactive state and idle state. So, in this section, we provide the related model and evaluation in inactive state.

Power consumption evaluation model

Power consumption model for DL PRS measurement

PRS measurement is similar to RRM measurement. There are some common points as below:

- Both PRS measurement and RRM measurement are RS-based measurement, one of which is PRS and the other is SSB or CSI-RS for mobility

- RSs to be measured can be from serving cell and neighboring cells

- Both support intra-frequency measurement and inter-frequency measurement. More specifically, in these two kinds of measurement, the concept of frequency layer is defined. In each frequency layers, UE measures RSs from multiple cells.

The power consumption model for RRM in clause 8.1.4 of TR38.840 can be referenced. In that model, the target signal is SSB. Therefore, for PRS measurement power consumption, we make the following assumptions.

General PRS measurements assumptions

- Frequency: FR1 has high priority; 1 positioning frequency layer in FR1

- Bandwidth: 100MHz/30kHz, 20MHz/30kHz

- RE mapping: Comb-4/4 symbols for a PRS resource

- Repetition: repetition factor=8; time gap=1 slot

- 8 symbols per slot

- 8 TRPs per frequency layer

Slot-averaged power for single positioning frequency layer measurements

The following table is for RRM measurement in connected state based on the baseline assumption of SSB captured in TR38.840 clause 8.1.4.1.

Table 2: UE power consumption for the RRM measurements in TR38.840 [3]

| N: Number of cells for intra-frequency measurement | Synchronous case |  | Asynchronous case |  |
| --- | --- | --- | --- | --- |
|  | FR1 | FR2 | FR1 | FR2 |
| N=8 | 150 | 225 | 170 | 285 |
| N=4 | 120 | 195 | 140 | 255 |

Then, for PRS measurement in connected state, considering that the assumption of PRS is similar to SSB, we can reuse the SSB power value for PRS measurement.

Table 3: UE power consumption for PRS measurements in connected state (100MHz, FR1)

| N: Number of TRPs for intra-frequency measurement | Synchronous case | Asynchronous case |
| --- | --- | --- |
| N=8 | 150 | 170 |
| N=4 | 120 | 140 |

For the operation in inactive state, PRS can be measured inside or outside initial DL BWP. Thus, for PRS outside initial DL BWP, bandwidth of 100MHz is assumed and the same power value as connected state is considered; while for PRS inside initial DL BWP, bandwidth of 20MHz is assumed, so that 0.4x power scaling is considered.

Table 4: UE power consumption for PRS measurements in inactive state (100MHz, FR1)

| N: Number of TRPs for intra-frequency measurement | Synchronous case | Asynchronous case |
| --- | --- | --- |
| N=8 | 150 | 170 |
| N=4 | 120 | 140 |

Table 5: UE power consumption for PRS measurements in inactive state (20MHz, FR1)

| N: Number of TRPs for intra-frequency measurement | Synchronous case | Asynchronous case |
| --- | --- | --- |
| N=8 | 60 | 68 |
| N=4 | 50 | 56 |

Note: in the above tables, all above values are slot-averaged power (PPRS ) in FR1. The synchronous case means actual PRS transmissions from TRPs are time-aligned, while the asynchronous case means actual PRS transmissions from TRPs are not time-aligned.

Total power for single and multiple positioning frequency layers measurements

We can also refer to the model for intra/inter-frequency RRM measurements in TR38.840 and make some minor changes.

According to RAN4 requirement, a UE is expected to process one frequency layer at a time. Therefore, we propose that, for frequency layer i, the power of PRS measurement is represented as:

Ei=PPRS*NPRS+Eswitch

where

-    Ei is total power over slots over which measurements are carried out in frequency layer i

-    PPRS is the slot average power for PRS measurements in frequency layer i

-    NPRS is the number of slot over which measurements are carried out

-    Eswitch is the power of switching from initial DL BWP to positioning frequency layer i, where Eswitch= Pt * Tt

-	Pt is the switching power consumption

-	Assume micro sleep power for Pt which equals to 45 power units

-    If PRS is inside initial DL BWP, switching power is 0

-	Tt is switching time (including switching on and off) for FR1 (similar to measurement gap switching), which is assumed to be 1ms (0.5ms*2)

For Nf frequency layers, the total power is

ENf=i=0Nf-1Ei

It can be simplified to the following if Ei is the same across frequency layers (i.e. Ei = E for different frequency layers ).

ENf = E*Nf

In inactive state, for simplicity, we assume that the UE only measures the PRS of one frequency layer. Therefore, we propose,

- Support quantitative evaluation of power consumption for LPHAP.

- Support power consumption evaluation model for PRS measurement in inactive state as the following.

- Slot-averaged power for PRS measurement in inactive state

UE power consumption for PRS measurements in inactive state (100MHz, FR1)

| N: Number of TRPs for intra-frequency measurement | Synchronous case | Asynchronous case |
| --- | --- | --- |
| N=8 | 150 | 170 |
| N=4 | 120 | 140 |

UE power consumption for PRS measurements in inactive state (20MHz, FR1)

| N: Number of TRPs for intra-frequency measurement | Synchronous case | Asynchronous case |
| --- | --- | --- |
| N=8 | 60 | 68 |
| N=4 | 50 | 56 |

- Power calculation formula for PRS measurement in inactive state

For single frequency layer measurement of frequency layer i, the power of PRS measurement is represented as: Ei=PPRS*NPRS+EMG_switch where - Ei is total power over slots over which measurements are carried out in frequency layer i - PPRS is the slot average power for PRS measurements in frequency layer i - NPRS is the number of slot over which measurements are carried out - Eswitch is the power of switching from initial DL BWP to positioning frequency layer i, where Eswitch= Pt * Tt - Pt is the switching power consumption - Assume micro sleep power for Pt which equals to 45 power units - If PRS is inside initial DL BWP, switching power is 0 - Tt is switching time (including switching on and off) for FR1, which is assumed to be 1ms (0.5ms*2)

Power consumption model for SRS transmission

General SRS transmission assumptions

- Frequency: FR1

- Bandwidth:100MHz/30kHz, 20MHz/30kHz

- 8 symbols in a slot (same as PRS), 1ms length (2 slots)

In TR38.840, the following assumption for UL transmission is captured.

| Power State | Characteristics | Relative Power |  |
| --- | --- | --- | --- |
|  |  | FR1 | FR2 |
| UL | Long PUCCH or PUSCH. | 250 (0 dBm) 700 (23 dBm) | 350 (FFS Tx power level) |

In addition, for UE power consumption scaling for adaptation, the following table is considered.

| Scaling for FR1 | Proposal | Comment |
| --- | --- | --- |
| BWP Bandwidth (UL) | No scaling at 0dBm or 23dBm Above scaling is applicable for FR1 only. In case scaling is needed for FR2, companies can report the assumed scaling factor. |  |
| SRS | SRS power = 0.3 x uplink power | Applicable for FR1 and FR2. |

So, we can assume that SRS transmission power in connected state is equal to 210 (700(23dBm)*0.3) power units.

Furthermore, since there is no bandwidth scaling for uplink transmission, SRS transmission power in inactive state can be assumed as 210 regardless of the SRS bandwidth is within the initial UL BWP or not.

In addition, considering SRS can be transmitted outside initial UL BWP, SRS switching power should also be added. Switching power similar to DL PRS measurement can be assumed in inactive state.

Therefore, the power of SRS transmission is represented as:

Esrs=PSRS*NSRS+Eswitch

where

-    ESRS is total power over slots over which SRS is transmitted

-    PSRS is the slot average power for SRS transmission

-    NSRS is the number of slots over which SRS is transmitted

-    Eswitch is the power of switching from initial UL BWP to ‘SRS only BWP’, where Eswitch= Pt * Tt

-	Pt is the switching power consumption

-	Assume micro sleep power for Pt which equals to 45 power units

-    If SRS is inside initial UL BWP, switching power is 0

-	Tt is switching time (including switching on and off) for FR1 (similar to measurement gap switching), which is assumed to be 1ms (0.5ms*2)

- Support power consumption evaluation model for SRS transmisson in inactive state as the following.

- Slot-averaged power for SRS transmission in inactive state is assumed as 210 power units

- Power calculation formula for SRS transmission in inactive state

The power of SRS transmission is represented as: Esrs=PSRS*NSRS+Eswitch where - ESRS is total power over slots over which SRS is transmitted - PSRS is the slot average power for SRS transmission - NSRS is the number of slots over which SRS is transmitted - Eswitch is the power of switching from initial UL BWP to ‘SRS-only BWP’, where Eswitch= Pt * Tt - Pt is the switching power consumption - Assume micro sleep power for Pt which equals to 45 power units - If SRS is inside initial UL BWP, switching power is 0 - Tt is switching time (including switching on and off) for FR1, which is assumed to be 1ms (0.5ms*2)

Power consumption evaluation assumptions

In this section, power consumption evaluation assumptions are discussed. In addition to TR38.840, the following assumptions regarding Rel-17 power saving reached in RAN1#102e meeting[4] can also be considered. It should be noted that for DL reception in inactive/idle state, corresponding relative power is scaled compared to the power in connected state.

Agreements: The following power consumption model for FR1 is utilized for the evaluations of Rel-17 UE power saving enhancements in idle/inactive mode - FFS: FR2 power consumption model for idle/inactive mode operations - Power State; Relative Power (FR1 reference from TR 8438.840); Relative Power (Idle/inactive-mode operation with reception bandwidth 20 MHz). Deep Sleep (PDS); 1; 1. Light Sleep (PLS); 20; 20. Micro sleep (PMS); 45; 45. PDCCH-only (PPDCCH); 100; 50Note. PDCCH + PDSCH (PPDCCH+PDSCH); 300; 120. PDSCH-only (PPDSCH); 280; 112. SSB/CSI-RS proc. (PSSB); 100 (synchronization or serving cell measurement); 50. Intra-frequency RRM measurement (Pintra);  150 (synchronous case, N=8, measurement only; Pintra, meas-only)  200 (combined search and measurement; Pintra, search+meas);  [60] (synchronous case, N=8, measurement only; Pintra, meas-only)  [80] (combined search and measurement; Pintra, search+meas). Inter-frequency RRM measurement (Pinter); · 150 (measurement only per freq. layer; Pinter, meas-only) · 150 (neighbor cell search power per freq. layer; Pinter, search-only) · Micro sleep power assumed for switch in/out a freq. layer; · [60] (measurement only per freq. layer; Pinter, meas-only) · [150] (neighbor cell search power per freq. layer; Pinter, search-only) · Micro sleep power assumed for switch in/out a freq. layer. Note: Power scaling to 20MHz reception bandwidth follows the rule in Section 8.1.3 of TR 38.840, i.e., max{reference power * 0.4, 50}.. Agreements: Group paging rate of 10% is assumed for the evaluation of Rel-17 paging enhancement - FFS: Another group paging rate > 10% - Note: If UE sub-grouping is applied, the sub-group paging rate can be reduced w.r.t. the total sub-group number for a PO

Quantification of power consumption requirement

For LPHAP, companies should evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the use case of 6 for LPHAP, the battery life time of 6~12 months is required. However, the evaluation of power saving is generally based on relative power unit. For example, for power consumption of deep sleep, 1 power unit is assumed. Therefore, battery life time of 6~12 months should be converted to relative power unit.

Taking a mobile phone as an example, we assume that the battery capacity of the mobile phone is 4500mAh, which can support 10 hours under normal use. In addition, we assume that 20% of the total power consumption is consumed by 5G communication services. Furthermore, the relative power unit for communication services can also refer to the evaluation results in TR38.840. Here, we extract 73.65 power units as a typical slot-average relative power based on the following table with corresponding assumptions marked by yellow.

Table 6: Evaluation of relaxed RRM measurement in time-domain in TR38.840[3]

|  | (0) | (1) | (2) | (3) | (4) | (5) | (6) | (7) | (8) | (9) | (10) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (0) Power saving schemes description (1) Average power consumption (2) Power reduction compared to baseline [%] |  |  | (3) RRC Idle/RRC Inactive (i) or RRC Connected (c) (4)Measurement period [ms] (5)number of samples(e.g., SSB bursts) in each MP (6) UE speed and channel model (7) system impact, e.g., overhead (8) Performance impact (9)Additional assumption, e.g., PO and SSB offset, |  |  |  |  |  |  | Note |
| Source 2 Vivo [89] | Only SSB is used for RRM (2 samples per DRX cycle) | 4.17 | Baseline1 | Idle | 320 | 2 | TDL-C 3km/h 30km/h | Low | RSRP accuracy impact are captured in Figure 2-3 in [89] | The power saving gain is average gain of [0-10] ms SSB/CSI-RS and PO offset. Additional assumptions: 1. The duration of SSB/CSI-RS measurement occasion per paging cycle is 2ms. 2. Periodicity of SSB is 20ms and SSB is transmitted in the 1st slot of each 20ms period. 3. Periodicity of CSI-RS is 10ms and CSI-RS is transmitted in the 6th slot of each radio frame. 4. Paging cycle (DRX cycle) is 320ms, and the paging is monitored in one slot in first 2 radio frames of each paging cycle by different UEs with equal probability (random paging occasion). 5. Assuming no cell reselection. Thus no SIB1 energy consumption is included. 6. Assuming no additional resource for AGC tuning. The slot duration is based on 30 kHz SCS for FR1 and 120kHz for FR2. | FR1 |
|  | The number of measurement samples is reduced to 1 per paging cycle in idle state. Measurement is still based on SSB | 2.84 | 31.9% | Idle | 320 | 1 |  | Low |  |  |  |
|  | The measurement periodicity is extended to 2 paging cycle, 2 measurements sample per paging cycle. | 3.02 | 27.6% | Idle | 640 | 2 |  | Low |  |  |  |
|  | Only SSB is used for RRM (2 samples per DRX cycle) | 2.97 | Baseline2 | Idle | 320 | 2 |  | Low |  |  | FR2 |
|  | The number of measurement samples is reduced to 1 per paging cycle in idle state. Measurement is still based on SSB | 2.173 | 26.8% | Idle | 320 | 1 |  | Low |  |  |  |
|  | The measurement periodicity is extended to 2 paging cycle, 2 measurements sample per paging cycle. | 2.11 | 29% | Idle | 640 | 2 |  | Low |  |  |  |
|  | Reduce RRM measurement in RRC connected state. with short DRX InactivityTimer | 40.45 | Baseline1 | Con | 200 | 5 |  | Low |  | Long DRXCycle = 160ms, onDurationTimer = 8ms, data_rate=300Mbps FTP Model 3, packet size =0.5MByte Periodic activities is not modeled, e.g., AGC | FR1, 200 ms measurement period is considered as the baseline. DRX InactivityTimer = 40ms |
|  |  | 35.80 | 11.5% | Con | 400 | 5 |  | Low |  |  |  |
|  |  | 31.38 | 22.4% | Con | 800 | 5 |  | Low |  |  |  |
|  | Reduce RRM measurement in RRC connected state. With long DRX InactivityTimer | 73.65 | Baseline2 | Con | 200 | 5 |  | Low |  |  | FR1, 200 ms measurement period is considered as the baseline. DRX InactivityTimer = 100ms |

For LPHAP devices that can support 6 months to one year, the only job can be communication and positioning. So, we assume that the battery capacity is 4500mAh, and all of the power is consumed by the communication and positioning services. After comparing with the cell phone, we scale the ‘slot-average relative power unit’ of mobile phone, then obtain the following table and the target ‘slot-average relative power unit’ of the LPHAP device.

Table 7: Battery usage and slot-average relative power unit for different devices

| Device type | Battery capacity | Battery life | Voltage | Slot-average relative power unit |
| --- | --- | --- | --- | --- |
| Mobile phone | 4500mAh*20% | 10h | 3.7V | 73.65 |
| LPHAP device | 4500mAh | 6 months (180*24h) | 3.7V | [0.85] |

- For LPHAP device, the power consumption requirement should be converted to slot-average relative power unit.

- Consider 0.85 as target slot-average relative power unit for LPHAP .

Assumptions for DL positioning in inactive state

In inactive state, in addition to positioning related DL PRS measurement/report and SRS transmission, UE also needs to perform paging monitoring, RRM measurement, AGC and T/F tracking based on SSB. Therefore, all of the above power components need to be considered in the evaluation.

PRS measurement only

Firstly, we model the positioning measurements in inactive state with following power components:

Table 8: Power components for PRS measurement in inactive state

| Power components | Assumptions/characteristics | Slot-average relative Power |
| --- | --- | --- |
| PRS occasion | - 4ms (8 slots)PRS length between SSB burst and paging occasion - For 100MHz PRS, additional 1ms switching time/power should be considered; for 20MHz PRS, no switching time/power is considered - 100MHz or 20MHz - for simplicity, 100MHz PRS is preferred in baseline evaluation - 8 TRPs in a positioning frequency layer - UE only measures 1 positioning frequency layer, 1 PRS occasion per I-DRX cycle | 150/60 (100MHz/20MHz) Switching power (if needed): 45 |
| Paging occasion | - High SINR condition: 1ms (2 slots) length; Low SINR condition: 4ms (8slot) length; - Period: I-DRX cycle(or paging cycle) 1280ms - Equivalent to PDCCH-only and PDCCH+PDSCH (with 10% paging rate) Note: for PDCCH-only, there is no indication to schedule paging PDSCH; for PDCCH+PDSCH, it is used when the UE decodes the PDCCH for paging and obtains the need to decode the corresponding PDSCH | 50/120 (without / with paging) |
| SSB burst for loop convergence and T/F tracking; RSRP measurement of serving cell; (SSB Proc.) | - 2ms SMTC; - High SINR condition: 1 SSB bursts before paging occasion to be measured; Low SINR condition: 3 SSB bursts before paging occasion to be measured; - SSB burst period: 20ms | 50 |
| SSB burst for intra-frequency RRM measurement | - 2ms SMTC; - High SINR condition: no intra-frequency RRM measurement (according to S-measurement criterion); Low SINR condition: 1SSB burst before paging occasion - SSB burst period: 20ms Note: One of SSB bursts above (for loop convergence and T/F tracking; RSRP measurement of serving cell) can be reused. | 60 |
| SSB burst for inter-frequency RRM measurement | - 5ms SMTC; - High SINR condition: no inter-frequency RRM measurement (according to S-measurement criterion); Low SINR condition: 1SSB burst after paging occasion - SSB burst period: 20ms | 60 |
| Sleep | - Deep sleep - Relative power: 1 - Transition energy: 450 - Total transition time: 20ms - Light sleep - Relative power: 20 - Transition energy: 100 - Total transition time: 6ms - Micro sleep - Relative power: 45 - Transition energy: 0 - Total transition time: 0ms |  |

Then, the procedure for PRS measurement in inactive state is shown in Figure 1 and Figure 2 below.

Figure1 Procedure of PRS measurements in inactive state (low SINR)

Figure2 Procedure of PRS measurements in inactive state (high SINR)

- For LPHAP power consumption evaluation, PRS measurement assumptions in Table 8 should be adopted.

PRS measurement and report

In addition to power components for PRS measurements, for UE-assisted DL positioning, power components for PRS report should be considered.

Table 9: Power components for PRS report in inactive state

| Power components | Assumptions/characteristics | Slot-average relative Power |
| --- | --- | --- |
| Positioning report via CG-SDT | - CG SDT is equivalent to PUSCH.(in this PUSCH, UE send RRCresumerequest and positioning report information) - 1ms length - After positioning report, UE receives RRC release and remain in inactive state - equivalent to PDCCH+PDSCH - 1ms length - 1 positioning report per I-DRX cycle | Positioning report: 250 RRC release: 120 |
| Positioning report via RA-SDT | For RA-SDT, the following components may be considered: - Coreset0+SIB1 - Equivalent to PDCCH + PDSCH. - 1ms length - PRACH - Equivalent to SRS - 1ms length - RAR - Equivalent to PDCCH + PDSCH - 1ms length - Msg3 - It carries RRCresumerequest and positioning report information - Equivalent to PUSCH (0dBm) - 1ms length - Msg4 - It carries RRC release message and/or contention resolution - Equivalent to PDCCH + PDSCH - 1ms length Note: An RA-SDT window is assumed including the above components and 4 light sleep transitions | Coreset0+SIB1:120 PRACH: 210 RAR:120 Msg3:250 Msg4:120 |

Then, the procedure for PRS measurement and report in inactive state is shown in Figure 3~6 below.

Figure3 Procedure of positioning report via CG-SDT in inactive state (low SINR)

Figure4 Procedure of positioning report via CG-SDT in inactive state (high SINR)

Figure 5 Procedure of positioning report via RA-SDT in inactive state (low SINR)

Figure 6 Procedure of positioning report via RA-SDT in inactive state (high SINR)

- For LPHAP power consumption evaluation, PRS measurement report assumptions in Table 9 should be adopted.

Assumptions for UL positioning in inactive state

For UL positioning, we focus on SRS transmission procedures in inactive state. Firstly, we model SRS transmission in inactive state with following power components.

Table 10: Power components for SRS transmission in inactive state

| Power components | Assumptions/characteristics | Slot-average relative Power |
| --- | --- | --- |
| SRS occasion | - 1ms SRS length between SSB burst and paging occasion - For SRS outside initial UL BWP, additional 1ms switching time/power should be considered; for SRS inside initial UL BWP, no switching time/power is considered - for simplicity, SRS outside intial UL BWP is preferred in baseline evaluation - No bandwidth scaling for UL(according to TR38.840) - UE only transmits 1 SRS occasion per I-DRX cycle | 210 Switching power (if needed): 45 |

Then, the procedure for SRS transmission in inactive state is shown in the following figure.

Figure7 Procedure of SRS measurements in inactive state (low SINR)

Figure8 Procedure of SRS measurements in inactive state (high SINR)

- For LPHAP power consumption evaluation, SRS transmission assumptions in Table 10 should be adopted.

Preliminary evaluation results

In this section, we analyze the time of multiple power components and calculate the average power consumption. under the assumptions in section 3.2.

Baseline evaluation results for DL positioning in inactive state

PRS measurement only

We analyze the time of multiple power components of  Figure 1 and Figure 2 in section 3.2.2.1, and calculate the average energy consumption, the specific content can refer to the following Tables. It should be noted that the slot average power is calculated from the total power/duration(e.g. 1280ms) in a I-DRX cycle

Table 11 Power components analysis for positioning measurement only in inactive state(low SINR, Figure 1)

| Power state | Relative power | Duration(ms) |
| --- | --- | --- |
| Sleep |  |  |
| Deep sleep | 1 | 1215 |
| Light sleep | 20 | 41 |
| Micro sleep | 45 | 4 |
| SSB measurment |  |  |
| SSB for Inter-frequency measurement | 60 | 5 |
| SSB for Intra-frequency measurement | 60 | 2 |
| SSB Proc. | 50 | 2+2 |
| PRS measurement |  |  |
| PRS measurement | 150 | 4 |
| PRS switching time | 45 | 1 |
| Paging monitoring |  |  |
| Paging Occasion | 50/120 | 4 |
| Sleep transition |  |  |
| Sleep transition type | Transition energy | Transition times |
| Deep sleep transition | 450 | 1 |
| Light sleep transition | 100 | 4 |
| Average power | 3.539/3.758(without/with paging) |  |
|  | weighted average: 3.561 |  |

Table 12 Power components analysis for positioning measurement only in inactive state(high SINR, Figure 2)

| Power state | Relative power | Duration(ms) |
| --- | --- | --- |
| Sleep |  |  |
| Deep sleep | 1 | 1260 |
| Light sleep | 20 | 12 |
| Micro sleep | 45 | 0 |
| SSB measurment |  |  |
| SSB for Inter-frequency measurement | 60 | 0 |
| SSB for Intra-frequency measurement | 60 | 0 |
| SSB Proc. | 50 | 2 |
| PRS measurement |  |  |
| PRS measurement | 150 | 4 |
| PRS switching time | 45 | 1 |
| Paging monitoring |  |  |
| Paging Occasion | 50/120 | 1 |
| Sleep transition |  |  |
| Sleep transition type | Transition energy | Transition times |
| Deep sleep transition | 450 | 1 |
| Light sleep transition | 100 | 2 |
| Average power | 2.300/2.355(without/with paging) |  |
|  | weighted average:2.306 |  |

PRS measurement and report

We analyze the time of multiple power components of  Figure 3~6 in section 3.2.2.2, and calculate the average energy consumption, the specific content can refer to the following Tables.

Table 13 Power components analysis for positioning measurement and report in inactive state(low SINR, CG-SDT, Figure 3)

| Power state | Relative power | Duration(ms) |
| --- | --- | --- |
| Sleep |  |  |
| Deep sleep | 1 | 1203 |
| Light sleep | 20 | 47 |
| Micro sleep | 45 | 8 |
| SSB measurment |  |  |
| SSB for Inter-frequency measurement | 60 | 5 |
| SSB for Intra-frequency measurement | 60 | 2 |
| SSB Proc. | 50 | 2+2 |
| PRS measurement |  |  |
| PRS measurement | 150 | 4 |
| PRS switching time | 45 | 1 |
| Paging monitoring |  |  |
| Paging Occasion | 50/120 | 4 |
| Positioning report |  |  |
| CG-SDT | 250 | 1 |
| RRC-release | 120 | 1 |
| Sleep transition |  |  |
| Sleep transition type | Transition energy | Transition times |
| Deep sleep transition | 450 | 1 |
| Light sleep transition | 100 | 5 |
| Average power | 4.131/4.35(without/with paging) |  |
|  | weighted average:4.153 |  |

Table 14 Power components analysis for positioning measurement and report in inactive state(high SINR, CG-SDT, Figure 4)

| Power state | Relative power | Duration(ms) |
| --- | --- | --- |
| Sleep |  |  |
| Deep sleep | 1 | 1244 |
| Light sleep | 20 | 26 |
| Micro sleep | 45 | 0 |
| SSB measurment |  |  |
| SSB for Inter-frequency measurement | 60 | 0 |
| SSB for Intra-frequency measurement | 60 | 0 |
| SSB Proc. | 50 | 2 |
| PRS measurement |  |  |
| PRS measurement | 150 | 4 |
| PRS switching time | 45 | 1 |
| Paging monitoring |  |  |
| Paging Occasion | 50/120 | 1 |
| Positioning report |  |  |
| CG-SDT | 250 | 1 |
| RRC-release | 120 | 1 |
| Sleep transition |  |  |
| Sleep transition type | Transition energy | Transition times |
| Deep sleep transition | 450 | 1 |
| Light sleep transition | 100 | 4 |
| Average power | 2.952/3.007(without/with paging) |  |
|  | weighted average:2.958 |  |

Table 15 Power components analysis for positioning measurement and report in inactive state(low SINR, RA-SDT, Figure 5)

| Power state | Relative power | Duration(ms) |
| --- | --- | --- |
| Sleep |  |  |
| Deep sleep | 1 | 1182 |
| Light sleep | 20 | 65 |
| Micro sleep | 45 | 8 |
| SSB measurment |  |  |
| SSB for Inter-frequency measurement | 60 | 5 |
| SSB for Intra-frequency measurement | 60 | 2 |
| SSB Proc. | 50 | 2+2 |
| PRS measurement |  |  |
| PRS measurement | 150 | 4 |
| PRS switching time | 45 | 1 |
| Paging monitoring |  |  |
| Paging Occasion | 50/120 | 4 |
| Positioning report |  |  |
| Coreset0+SIB1 | 120 | 1 |
| PRACH | 210 | 1 |
| RAR | 120 | 1 |
| Msg3 | 250 | 1 |
| Msg4 | 120 | 1 |
| Sleep transition |  |  |
| Sleep transition type | Transition energy | Transition times |
| Deep sleep transition | 450 | 1 |
| Light sleep transition | 100 | 8 |
| Average power | 4.982/5.200(without/with paging) |  |
|  | weighted average:5.004 |  |

Table 16 Power components analysis for positioning measurement and report in inactive state(high SINR, RA-SDT, Figure 6)

| Power state | Relative power | Duration(ms) |
| --- | --- | --- |
| Sleep |  |  |
| Deep sleep | 1 | 1237 |
| Light sleep | 20 | 36 |
| Micro sleep | 45 | 4 |
| SSB measurment |  |  |
| SSB for Inter-frequency measurement | 60 | 0 |
| SSB for Intra-frequency measurement | 60 | 0 |
| SSB Proc. | 50 | 2 |
| PRS measurement |  |  |
| PRS measurement | 150 | 4 |
| PRS switching time | 45 | 1 |
| Paging monitoring |  |  |
| Paging Occasion | 50/120 | 1 |
| Positioning report |  |  |
| Coreset0+SIB1 | 120 | 1 |
| PRACH | 210 | 1 |
| RAR | 120 | 1 |
| Msg3 | 250 | 1 |
| Msg4 | 120 | 1 |
| Sleep transition |  |  |
| Sleep transition type | Transition energy | Transition times |
| Deep sleep transition | 450 | 1 |
| Light sleep transition | 100 | 6 |
| Average power | 3.752/3.806(without/with paging) |  |
|  | weighted average:3.757 |  |

Baseline evaluation for UL positioning in inactive state

We analyze the time of multiple power components of  Figure 7~8 in section 3.2.3, and calculate the average energy consumption, the specific content can refer to the following Tables.

Table 17 Power components analysis for SRS transmission in inactive state(low SINR, Figure 7)

| Power state | Relative power | Duration(ms) |
| --- | --- | --- |
| Sleep |  |  |
| Deep sleep | 1 | 1213 |
| Light sleep | 20 | 46 |
| Micro sleep | 45 | 4 |
| SSB measurment |  |  |
| SSB for Inter-frequency measurement | 60 | 5 |
| SSB for Intra-frequency measurement | 60 | 2 |
| SSB Proc. | 50 | 2+2 |
| SRS transmission |  |  |
| SRS transmission | 210 | 1 |
| SRS switching time | 45 | 1 |
| Paging monitoring |  |  |
| Paging Occasion | 50/120 | 4 |
| Sleep transition |  |  |
| Sleep transition type | Transition energy | Transition times |
| Deep sleep transition | 450 | 1 |
| Light sleep transition | 100 | 4 |
| Average power | 3.194/3.413(without/with paging) |  |
|  | weighted average:3.216 |  |

Table 18 Power components analysis for SRS transmission in inactive state(high SINR, Figure 8)

| Power state | Relative power | Duration(ms) |
| --- | --- | --- |
| Sleep |  |  |
| Deep sleep | 1 | 1263 |
| Light sleep | 20 | 12 |
| Micro sleep | 45 | 0 |
| SSB measurment |  |  |
| SSB for Inter-frequency measurement | 60 | 0 |
| SSB for Intra-frequency measurement | 60 | 0 |
| SSB Proc. | 50 | 2 |
| SRS transmission |  |  |
| SRS transmission | 210 | 1 |
| SRS switching time | 45 | 1 |
| Paging monitoring |  |  |
| Paging Occasion | 50/120 | 1 |
| Sleep transition |  |  |
| Sleep transition type | Transition energy | Transition times |
| Deep sleep transition | 450 | 1 |
| Light sleep transition | 100 | 2 |
| Average power | 1.9984/2.053 (without/with paging) |  |
|  | weighted average:2.004 |  |

Summary of baseline evaluation results

In this section, we summarize the power evaluation results of the above baseline assumptions. On this basis, we further add the result of I-DRX cycle of 2560ms (largest I-DRX cycle). Then, we compare these evaluation results with the target power consumption requirement of LPHAP in the following table.

Table 19 Summary of baseline evaluation results

| Baseline scenarios/assumptions | 1280ms I-DRX cycle |  | 2560ms I-DRX cycle |  | LPHAP requirement |
| --- | --- | --- | --- | --- | --- |
|  | Average relative power | Power consumption gap | Average relative power | Power consumption gap |  |
| PRS measurement only (low SINR) | 3.561 | +318.94% | 2.281 | +168.35% | [0.85] |
| PRS measurement only (high SINR) | 2.306 | +171.29% | 1.653 | +94.47% |  |
| PRS measurement/report(low SINR, CG-SDT) | 4.153 | +388.59% | 2.577 | +203.17% |  |
| PRS measurement/report(high SINR, CG-SDT) | 2.958 | +248% | 1.979 | +132.82% |  |
| PRS measurement/report(low SINR, RA-SDT) | 5.004 | +488.71% | 3.002 | +253.17% |  |
| PRS measurement/report(high SINR, RA-SDT) | 3.757 | +342% | 2.379 | +179.88% |  |
| SRS transmission(low SINR) | 3.216 | +278.35% | 2.108 | +148% |  |
| SRS transmission(high SINR) | 2.004 | +135.76% | 1.502 | +76.71% |  |

It can be observed that when I-DRX cycle (or paging cycle) is selected as a typical value of 1280ms or the maximum value of 2560ms, the power consumption cannot meet the requirement. Even for the lowest power consumption in the case of SRS transmission under high SINR and with 2560ms IDRX cycle, the power consumption is 76.71% higher than the target.

Then, PRS measurement only or SRS transmission only saves more power than PRS measurement and report due to the reduced positioning report. In addition, for positioning report, CG-SDT is more power efficient than RA- SDT. Besides, with better channel conditions (such as high SINR), power saving gain can also be obtained.

- Regardless of I-DRX cycle (or paging cycle) is selected as a typical value of 1280ms or the maximum value of 2560ms, the power consumption in inactive state for all the cases cannot meet the requirement.

- e.g., even for the lowest power consumption in the case of SRS transmission under high SINR and with 2560ms IDRX cycle, the power consumption is 76.71% higher than the target.

Considering that power consumption cannot reach the requirement based on existing operation in inactive state, we propose to further study power saving mechanism for LPHAP.

- Power saving mechanism should be studied for LPHAP to meet the power consumption requirement.

Potential solutions for LPHAP

Enhancement with eDRX

In NB-IoT, considering traditional paging cycle consumes a lot of power for the UE, eDRX mode was introduced to support a longer period of paging monitoring, so as to save power. The figure below shows a typical eDRX configuration. As shown in the figure, UE monitors paging in the PTW (paging time window), and each PTW can contain multiple paging cycles within a paging Hyperframe (1024 SFN). PTW is periodic, and the unit is Hyper SFN. To achieve power comsumption requirement for LPHAP, in this section, we further evaluate the power consumption performance for positioning in eDRX mode.

Figure9 an eDRX configuration

Then, we make the following assumptions for power evaluation in eDRX mode.

Table 20 Assumptions for power evaluation in eDRX mode

| Assuptions | Characteristics | Note |
| --- | --- | --- |
| eDRX cycle | 30.72s (3 Hyper SFNs) | Select 3 Hyper-SFN as typical eDRX cycle, which is close to 30s positioning interval in requirement. |
| PTW length | 2.56s (2 paging cycles) | Select PTW length of 2 paging cycles as typical PTW length. UE is not expected to monitor paging outside PTW. |
| Paging cycle | 1.28s | / |
| Paging rate | 10% | / |
| Sleep assumption | Within a PTW, same sleep assumption in TR38.840 can be reused. - Deep sleep - Relative power: 1 - Transition energy: 450 - Total transition time: 20ms - Light sleep - Relative power: 20 - Transition energy: 100 - Total transition time: 6ms - Micro sleep - Relative power: 45 - Transition energy: 0 - Total transition time: 0ms For sleep between 2 adjacent PTWs, the following is assumed - Ultra deep sleep - Relative power: 0.015 - Transition power unit: 50 per ms - Total transition time: 200ms | For sleep between 2 adjacent PTWs in eDRX mode, the UE only needs to maintain a very low current and maintain a relatively low relative power (such as 0.015, which is lower than deep sleep) |

Next, we take PRS Measurement only under low SINR (Figure 1) as an example, analyze the time of multiple power components in an eDRX cycle, and calculate the average energy consumption, the specific content can refer to the following Table.

It should be noted that a complete cycle is 3 Hyperframe (3*1024*10ms). Within a PTW, same sleep assumption in TR38.840 can be reused, so that the slot average power previously calculated based on a I-DRX cycle (or paging cycle) can also be reused. Therefore, in this case, the total power can be calculated based on the power of 2 equivalent I-DRX cycles and ultra deep sleep. In addition, the overcounted power should be excluded.

Table 21 Power components analysis for PRS measurement only under low SINR (Figure 1) in eDRX mode

| Power state | Relative power | Duration(ms) |
| --- | --- | --- |
| Ultra deep sleep | 0.015 | 896*10+2*1024*10 |
| Equivalent I-DRX cycle | 3.561 | 1280*2 |
| Ultra deep sleep transition | 50 | 200 |
| Overcounted components total power |  |  |
| Deep sleep transition | 450 |  |
| Deep sleep | 1215 |  |
| Average power |  |  |
| 0.582 |  |  |

Similarly, we calculate the power consumption under the other assumptions/scenarios and summarized them as the following table.

Table 22 Summary of evaluation results in eDRX mode

| Baseline scenarios/assumptions | I-DRX mode (1280ms cycle) |  | eDRX mode |  | LPHAP requirement |
| --- | --- | --- | --- | --- | --- |
|  | Average relative power | Power consumption gap | Average relative power | Power consumption gap |  |
| PRS measurement only (low SINR) | 3.561 | +318.94% | 0.582 | -31.53% | [0.85] |
| PRS measurement only (high SINR) | 2.306 | +171.29% | 0.476 | -44% |  |
| PRS measurement/report(low SINR, CG-SDT) | 4.153 | +388.59% | 0.632 | -25.64% |  |
| PRS measurement/report(high SINR, CG-SDT) | 2.958 | +248% | 0.531 | -37.52% |  |
| PRS measurement/report(low SINR, RA-SDT) | 5.004 | +488.71% | 0.704 | -17.17% |  |
| PRS measurement/report(high SINR, RA-SDT) | 3.757 | +342% | 0.598 | -29.64% |  |
| SRS transmission(low SINR) | 3.216 | +278.35% | 0.554 | -34.82% |  |
| SRS transmission(high SINR) | 2.004 | +135.76% | 0.451 | -46.94% |  |

It can be observed that when eDRX mode is applied to LPHAP devices, the power consumption under all scenarios/assumptions can meet the requirement.

- When eDRX mode is applied to LPHAP devices, the power consumption under all scenarios/assumptions can meet the requirement

- e.g., even for the case with the highest power consumption (PRS measurement/report (low SINR, RA-SDT)),  power consumption is 17.17% below target.

Therefore, we propose

- eDRX mode can be considered for LPHAP device for power saving .

Mobility for positioning in inactive state

Uplink positioning (or positioning SRS transmission) in inactive state is supported in Rel-17. However, SRS transmission is only valid in the serving cell where RRC release is received. Due to the mobility of the UE, potential cell reselection will occur, which will cause the SRS configuration to no longer be valid and the SRS transmission to be terminated. If positioning service is required, the UE can only enter the connected state. Especially in the scenario where the cell range is relatively small and/or the UE moves quickly, the continuous positioning service will cause the UE to perform state transitions frequently. This will result in more power consumption, which matters for LPHAP devices.

Therefore, mobility support for SRS transmission in inactive state is important to save more power. For example, SRS transmission is valid in a predefined area, which includes a list of cells. The potential solutions may include:

- Pre-configured SRS. When UE access a new cell, it can select a new configuration of SRS without entering connected state and continue SRS transmission.

- UE initiated SRS configuration update request. For instance, when UE leaves the SRS valid cell, it can remain in the inactive state and use the similar method as the positioning report (e.g., via SDT) to require updated SRS configuration; and then the new gNB can deliver the updated SRS configuration to the UE without entering connected state.

In addition, even if UE is within a cell, the movement of UE can also cause the SRS validity criteria to be unsatisfied and the transmission will be terminated. This makes the UE to stop SRS transmission and potentially enter the connected state. In Rel-17, SRS validity criteria includes validity criteria of spatial relationship, pathloss RS and TA. If we can reduce unnecessary validity judgments and related measurements, we can reduce the probability that the UE stops SRS transmission and enters the connected state, which is ultimately beneficial to power saving. A potential solution is to enable SRS beam sweeping in inactive state. In this way, there is no need to consider maintaining the spatial relationship between the UE and gNBs, correspondingly, the validation issues of spatial relation RS no longer exists. The reliability of SRS transmission in inactive state will be increased since the problem of spatial relationship failure that causes SRS transmission to stop will not exist; the additional measurement for validation determination will be no longer needed, which is beneficial to power consumption and complexity

- Mobility for SRS transmission in active state can be considered for LPHAP, including

- Pre-configured SRS

- UE initiated SRS configuration update request

- SRS beam sweeping enabling

Consideration of inactive DRX related issue

In Rel-17, RAN4 has discussed the positioning measurement delay requirement in inactive state, and the following conclusion regarding inactive DRX was achieved based on the LS[5].

RAN4 would like to inform RAN2 and RAN3 about the following agreements that it has reached regarding UE requirements for NR positioning measurements performed while the UE is in RRC_INACTIVE state. Agreement - DRX cycle should be considered in the positioning measurement delay requirements in RRC_INACTIVE state.. RAN4 also observes that DRX cycle may be different in different cells. RAN4 is further discussing the impact of DRX cycle on positioning measurement delay requirements under cell reselection in RRC_INACTIVE state.

It can be seen that DRX cycle is considered in the positioning measurement delay requirement, e.g. DRX cycle is used to calculate available PRS period via LCM(DRX cycle, PRS period). Considering that the value of the DRX cycle is relatively large, such as a typical value of 1280ms, this will cause a large delay in the PRS measurement in inactive state. Then, for some positioning services with specific power requirements which requires UE to stay in inactive state and measure based on DRX cycle, the influence of inactive DRX cycle on the latency should be considered by the LMF. For instance, the LMF may obtain inactive DRX configurations(e.g. DRX cycle, etc.) from cells in advance; then, considering these DRX configurations and other factors (e.g. QoS requirement, UE capabilities, etc.), it may indicate some assistance information to the serving gNB (e.g. assistance information to keep UE in connected state for positioning), or set a looser response time. It is also noted that, DRX cycle may be different in different cells. Therefore, the cells that provide DRX configuration can be not only the serving cell, but neighboring cells around the UE which may be reselected in inactive state, such as cells in the RNA area.

In addition, for power saving, we support PRS measurement or SRS transmission can be in the vicinity of paging monitoring. On the one hand, this can reduce the transition power of the UE to receive PRS when it wakes up from deep sleep; on the other hand, it reduces the power of the UE to additionally measure the reference signal (e.g., when the UE transmits SRS, it can reuse the SSB used for paging monitoring, so as to get synced).

- The following solutions related to inactive DRX can be considered for LPHAP, including

- LMF requesting inactive DRX configurations (e.g. DRX cycle, etc.) from the cells including UE serving cell and neighboring cells that may be reselected can be considered for LPHAP

- PRS measurement/SRS transmission in the vicinity of paging monitoring

Consideration of positioning in idle state

In the objective of LPHAP, it can be seen that the study is not only limited to the enhancements to inactive state, but also idle state.

In Rel-17 SI, we studied NR positioning for UEs in inactive state and idle state, including the benefits on latency, network/UE efficiency and UE power consumption. Then, we identified the following impact for positioning in idle state.

From a physical layer perspective, it is feasible for a UE to perform DL positioning measurement in RRC_IDLE state. - Note: This does not imply that measurements have to be reported in RRC_IDLE state. The following procedures are considered as feasible for DL positioning methods in RRC_IDLE: - Reporting of DL-PRS measurement and/or location estimate performed in RRC_IDLE when the UE is in RRC_INACTIVE/RRC_CONNETED.

Both RAN1 and RAN2 consider it is feasible for a UE to perform DL positioning measurement in idle state. In addition, reporting of DL-PRS measurement and/or location estimate performed by idle state in inactive state or or connected state is also feasible. However, for some reasons, corresponding normative work was not supported in Rel-17. Therefore, from our point of view, they should be supported in Rel-18.

Then, regarding positioning report in idle state, we are also open to discuss related issues in Rel-18. However, due to the lack of support for idle state report from other projects (such as Rel-17 SDT), more RAN2 time may be required to study related issues separately. Considering limited time and large work for other agendas, the study on idle state report may not set as a high priority.

- For LPHAP, the following enhancements related to idle state positioning should be supported at least

- DL-PRS measurement in idle state

- Reporting of DL-PRS measurement and/or location estimate performed in idle state when the UE is in inactive/connected state.

Conclusion

In this contribution, we discuss LPHAP with the following observations and proposals.

- Regardless of I-DRX cycle (or paging cycle) is selected as a typical value of 1280ms or the maximum value of 2560ms, the power consumption in inactive state for all the cases cannot meet the requirement.

- e.g., even for the lowest power consumption in the case of SRS transmission under high SINR and with 2560ms IDRX cycle, the power consumption is 76.71% higher than the target.

- When eDRX mode is applied to LPHAP devices, the power consumption under all scenarios/assumptions can meet the requirement

- e.g., even for the case with the highest power consumption (PRS measurement/report (low SINR, RA-SDT)),  power consumption is 17.17% below target.

- Confirm that study is limited to a single representative use case (use case 6 as defined TS 22.104) for LPHAP.

- Accuracy evaluation is not considered for LPHAP.

- Support quantitative evaluation of power consumption for LPHAP.

- Support power consumption evaluation model for PRS measurement in inactive state as the following.

- Slot-averaged power for PRS measurement in inactive state

UE power consumption for PRS measurements in inactive state (100MHz, FR1)

| N: Number of TRPs for intra-frequency measurement | Synchronous case | Asynchronous case |
| --- | --- | --- |
| N=8 | 150 | 170 |
| N=4 | 120 | 140 |

UE power consumption for PRS measurements in inactive state (20MHz, FR1)

| N: Number of TRPs for intra-frequency measurement | Synchronous case | Asynchronous case |
| --- | --- | --- |
| N=8 | 60 | 68 |
| N=4 | 50 | 56 |

- Power calculation formula for PRS measurement in inactive state

For single frequency layer measurement of frequency layer i, the power of PRS measurement is represented as: Ei=PPRS*NPRS+EMG_switch where - Ei is total power over slots over which measurements are carried out in frequency layer i - PPRS is the slot average power for PRS measurements in frequency layer i - NPRS is the number of slot over which measurements are carried out - Eswitch is the power of switching from initial DL BWP to positioning frequency layer i, where Eswitch= Pt * Tt - Pt is the switching power consumption - Assume micro sleep power for Pt which equals to 45 power units - If PRS is inside initial DL BWP, switching power is 0 - Tt is switching time (including switching on and off) for FR1, which is assumed to be 1ms (0.5ms*2)

- Support power consumption evaluation model for SRS transmisson in inactive state as the following.

- Slot-averaged power for SRS transmission in inactive state is assumed as 210 power units

- Power calculation formula for SRS transmission in inactive state

The power of SRS transmission is represented as: Esrs=PSRS*NSRS+Eswitch where - ESRS is total power over slots over which SRS is transmitted - PSRS is the slot average power for SRS transmission - NSRS is the number of slots over which SRS is transmitted - Eswitch is the power of switching from initial UL BWP to ‘SRS-only BWP’, where Eswitch= Pt * Tt - Pt is the switching power consumption - Assume micro sleep power for Pt which equals to 45 power units - If SRS is inside initial UL BWP, switching power is 0 - Tt is switching time (including switching on and off) for FR1, which is assumed to be 1ms (0.5ms*2)

- For LPHAP device, the power consumption requirement should be converted to slot-average relative power unit.

- Consider 0.85 as target slot-average relative power unit for LPHAP .

- For LPHAP power consumption evaluation, PRS measurement assumptions in Table 8 should be adopted.

- For LPHAP power consumption evaluation, PRS measurement report assumptions in Table 9 should be adopted.

- For LPHAP power consumption evaluation, SRS transmission assumptions in Table 10 should be adopted.

- Power saving mechanism should be studied for LPHAP to meet the power consumption requirement.

- eDRX mode can be considered for LPHAP device for power saving .

- Mobility for SRS transmission in active state can be considered for LPHAP, including

- Pre-configured SRS

- UE initiated SRS configuration update request

- SRS beam sweeping enabling

- The following solutions related to inactive DRX can be considered for LPHAP, including

- LMF requesting inactive DRX configurations (e.g. DRX cycle, etc.) from the cells including UE serving cell and neighboring cells that may be reselected can be considered for LPHAP

- PRS measurement/SRS transmission in the vicinity of paging monitoring

- For LPHAP, the following enhancements related to idle state positioning should be supported at least

- DL-PRS measurement in idle state

- Reporting of DL-PRS measurement and/or location estimate performed in idle state when the UE is in inactive/connected state.

- References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning ”, December 6-17, 2021

- TS 22.104, “Service requirements for cyber-physical control applications in vertical domains”

- TR38.840, “Study on User Equipment (UE) power saving in NR”

- Draft Report of 3GPP TSG RAN WG1 #102-e

- R4-2202686, LS on DRX cycle used in PRS measurement in RRC_INACTIVE state