3GPP TSG RAN WG1 #109-e		R1-2203571

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Discussion on PRS measurement in idle state

Agenda Item:	9.5.4

Document for:	Discussion and Decision

Introduction

At the RAN#94e meeting, the SID on Study on expanded and improved NR positioning RP-213588[1] was approved. In this contribution, we will present our views on other issues related to PRS measurement in idle state.

Discussion

In Rel-17 WI, positioning measurement and report in inactive state was specified. In inactive state, any uplink LCS or LPP message can be transported. And if the UE initiated data transmission using UL SDT, the network can send DL LCS, LPP and RRC message (e.g. to configure SRS for UL positioning, if it is supported) to the UE. Meanwhile, UE may also receive PRS or transmit SRS in inactive state. In addition, support of all NR positioning measurements and support of NR positioning methods such as NR E-CID, DL-TDOA, DL-AOD, UL-AOA, UL-TDOA, Multi-RTT and RAT-independent positioning methods are extended for UEs in inactive state.

However, for positioning in idle state, due to some reasons, the normative work was not supported. From our point of view, DL PRS measurement in idle state should be supported at least in Rel-18 with the following reasons.

- Supporting PRS measurement in inactive state is an extremely power-saving feature, especially for DL only UE-based positioning. For example, if UE support PRS measurement in idle state under UE-based mode, it only needs to wake up at a specific time to receive PRS to complete the positioning without measurement report. We capture the power consumption evaluation results for PRS measurement only and PRS measurent and report in idle state as the following table based on our companion contribution [2]. It can be observed that PRS measurement only can obtain 53.9% power saving gain compared with PRS measurement and report. Therefore, supporting the PRS measurement in idle state gives UE an opportunity to complete positioning with minimal power cost.

Table 1 Power consumption comparison in idle state

| Baseline scenarios/assumptions | Average relative power (with 1280ms I-DRX cycle) | Power saving gain |
| --- | --- | --- |
| PRS measurement only (high SINR) | 2.306 | 53.9% |
| PRS measurement/report (low SINR, RA-SDT) | 5.004 | Baseline |

- Supporting PRS measurement in idle state has minimal impact on existing specification. Currently, other than adding a sentence to 38.215, we have not yet identified any other specification impacts. An example for DL RSTD measurement in idle state is shown as the following.

| Definition | DL reference signal time difference (DL RSTD) is the DL relative timing difference between the Transmission Point (TP) [18] j and the reference TP i, defined as TSubframeRxj – TSubframeRxi, Where: TSubframeRxj is the time when the UE receives the start of one subframe from TP j. TSubframeRxi is the time when the UE receives the corresponding start of one subframe from TP i that is closest in time to the subframe received from TP j. Multiple DL PRS resources can be used to determine the start of one subframe from a TP. For frequency range 1, the reference point for the DL RSTD shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSTD shall be the antenna of the UE. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE RRC_IDLE |

- In Rel-17 SI, we also studied NR positioning for UEs in idle state, including the benefits on latency, network/UE efficiency and UE power consumption. Then, we identified the following impact for positioning in idle state [3]. And both RAN1 and RAN2 consider it is feasible for a UE to perform DL positioning measurement in idle state. In addition, reporting of DL-PRS measurement and/or location estimate performed by idle state in inactive or connected state is also feasible.

From a physical layer perspective, it is feasible for a UE to perform DL positioning measurement in RRC_IDLE state. - Note: This does not imply that measurements have to be reported in RRC_IDLE state. The following procedures are considered as feasible for DL positioning methods in RRC_IDLE: - Reporting of DL-PRS measurement and/or location estimate performed in RRC_IDLE when the UE is in RRC_INACTIVE/RRC_CONNETED.

Then, regarding positioning report in idle state, we are open to discuss related issues in Rel-18. However, considering the lack of support for idle state report from other projects (such as Rel-17 SDT) and limited time for positioning agendas, it may be hard for us to study positioning report in idle state. Therefore, PRS measurement in idle state can be supported in Rel-18 at least.

- For positioning in idle state in Rel-18, PRS measurement should be supported at least.

Conclusion

In this contribution, we discuss PRS measurement in idle state with the following proposal.

- For positioning in idle state in Rel-18, PRS measurement should be supported at least.

- References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”

- R1-2203569, “Discussion on Low Power High Accuracy Positioning”, vivo, RAN1#109e meeting

- TR38.867, “Study on NR Positioning Enhancement”