3GPP TSG RAN WG1 #109-e		R1-2203573

e-Meeting, May 9th – 20th, 2022

Source:	vivo, Guangdong Genius

Title:                     Discussion on potential simulation for eRedCap

Agenda Item:	9.6.2

Document for:	Discussion and Decision

- Introduction

In [1], following objectives are agreed to be studied:

- Study further UE complexity reduction techniques based on Rel-17 evaluation methodology in TR 38.875 [RAN1] - Consider network impact, coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs in a cell, UE impact, specification impact - Potential solutions, which may complement each other, for reducing device complexity are focusing on: - UE bandwidth reduction to 5MHz in FR1, - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - reduced UE peak data rate in FR1, - Possibly including restricted bandwidth for PDSCH and/or PUSCH - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - Notes: - Rel-15 SSB should be reused and L1 changes minimized. - Operation in BWP with/without SSB and without/with RF retuning should be considered. - It is not precluded that some solutions for FR1 can be applied to FR2 in WI stage. - Aim to define a single Rel-18 RedCap UE type for further UE complexity reduction.

In this contribution, we provide our views on the potential simulation need based on the initial simulation results and analysis on performance impacts from the UE bandwidth reduction to 5MHz for both RF and baseband.

- UE bandwidth reduction

The maximum bandwidth supported by Rel-17 RedCap is 20MHz in FR1 and the 20MHz is applied to both RF and baseband. For Rel-18 RedCap, the maximum bandwidth to 5MHz will be studied to further lower the UE cost. Depending on whether the reduction of the maximum bandwidth is applied to the RF and/or baseband components, and/or applied to the data and/or control channels, following options can be considered:

- Option 1: Reduced bandwidth for both RF and Baseband (BB)

- For Option 1, the maximum bandwidth for RF and BB is 5MHz. RF is tuned to the interested 5MHz bandwidth. If the location of the 5MHz BW changes for RF, RF retuning is required. The maximum bandwidth for processing the SSB and CORESET#0 is assumed to be 5MHz.

- Option 2: Reduced bandwidth for BB only and for both data channel and control channels

- For Option 2, the maximum bandwidth for RF is 20MHz. For baseband control and data channels, it is 5MHz. Compared to Option 1, the bandwidth for ADC/FFT is assumed to be the same as RF bandwidth since the signal enters the ADC can be wideband and it is easier for Option 2 to change the received 5MHz bandwidth in the digital domain without RF retuning. Given the buffering for control channel is 5MHz, the maximum bandwidth for processing the SSB and CORESET#0 is assumed to be 5MHz.

- Option 3: Reduced bandwidth for data channel in BB only

- For Option 3, the maximum bandwidth for RF and for control channel is 20MHz, for data channel, it is 5MHz. The main difference from Option 2 is Option 3 allows the control channel transmitted across the 20MHz bandwidth and the buffering for control channel is 20MHz. Therefore, the maximum bandwidth for processing the SSB and CORESET#0 is assumed to be 20MHz.

For different options, the relative cost savings, performance and the specification impact can be different. Detailed analysis can be found in our companion contribution [2]. As shown in [2], Option 1 that the 5MHz bandwidth is applied to both RF and BB, the most cost saving can be obtained, but the configuration/scheduling flexibility for both control and data channel is largely reduced, and there is also some degradation for the coverage and spectral efficiency due to the loss in the frequency selective and frequency hopping gain. Therefore, the justification to support the fast RF retuning for UE bandwidth reduction Option 1 (i.e., RF is 5MHz) might increased compared to Rel-17 RedCap supporting maximum 20MHz bandwidth. However, it was observed in Rel-17 RedCap that for some cases, the performance for frequency hopping over 100MHz with 2-/4-symbol RF retuning gap is worse than the frequency hopping over 20MHz without RF retuning gap [3].  Therefore, if UE bandwidth reduction Option 1 is adopted, simulation is needed to compare the performance for frequency hopping over 100MHz or 20MHz with 2-/4-symbol RF retuning gap vs. the performance for the frequency hopping over 5MHz without RF retuning gap.

To check the impacts of RF retuning time for intra-slot frequency hopping, we conducted the LLS for Msg.3 and PUCCH format 1 (PF1) and PUCCH format 3 (PF3). According to the maximum retuning time between narrowband regions in the LS [4], 4-symbol and 2-symbol RF retuning time are assumed for the Msg.3 and PUCCH with 30KHz SCS. The simulation assumes legacy MSG3 and PUCCH format. Due to the 4-symbol (or 2-symbol) RF retuning gap,  the last 2 symbols (or 1 symbol) of the first hop and the first 2 symbols (or 1 symbol) of the second hop of the UL transmission are punctured. Other detailed simulation assumptions can be found in Table 1 in Appendix.

The corresponding simulation results are shown in the figure 1~3 below. In all the simulated cases, the UE RF BW capability is 5MHz and frequency hopping is enabled.

- 5MHz w/o RF retuning: UE performs frequency hopping within 5MHz BW for PUCCH/MSG3 transmission

- 20MHz w 2-OS/4-OS RF retuning: UE performs frequency hopping over 20MHz while each hop is confined within the UE RF BW, i.e. 5MHz, 2-OS/4-OS are punctured (1-OS/2-OS for each hop) due to RF retuning.

- 20MHz w 2-OS/4-OS RF retuning: UE performs frequency hopping over 100MHz while each hop is confined within the UE RF BW, i.e. 5MHz, 2-OS/4-OS are punctured (1-OS/2-OS for each hop) due to RF retuning.

Figure 1: Performance impacts of RF retuning time for Msg.3

Figure 2: Performance impacts of RF retuning time for PUCCH format 1

Figure 3: Performance impacts of RF retuning time for PUCCH format 3

As shown in Figure 1, it is observed that for Msg.3 transmission with 10% BLER, compared with the case of FH over 5MHz BW without RF retuning, there is about 1.4dB and 1.8dB performance loss for the case of FH over 100MHz BW with 2-symbol RF retuning and the case of FH over 20MHz BW with 2-symbol RF retuning.

Similarly, compared with the case of FH over 5MHz BW without RF retuning, performance loss for the case of FH over 100MHz BW with 4-symbol RF retuning and the case of FH over 20MHz BW with 4-symbol RF retuning is around 2.3dB and 2.8dB respectively.

As shown in Figure 2 and Figure 3, following are observed for PUCCH format 1 and format 3:

- For 14-symbol PF1 with 4-symbol RF retuning,

  - To achieve 0.1% NACK-to-ACK, the required SNR for the case of FH cross 100MHz or the case of FH cross 20MHz is 1.7dB worse than the case of FH cross 5MHz without RF retuning respectively;

- For 14-symbol PF1 with 2-symbol RF retuning,

  - To achieve 0.1% NACK-to-ACK, the required SNR for the case of FH cross 100MHz or the case of FH cross 20MHz is 0.5dB worse than the case of FH cross 5MHz without RF retuning respectively;

- For 14-symbol PF3 with 4-symbol RF retuning,

  - To achieve 1% BER, the required SNR for the case of FH cross 100MHz and the case of FH cross 20MHz is 1.9dB and 2dB worse than the case of FH cross 5MHz without RF retuning respectively.

- For 14-symbol PF3 with 2-symbol RF retuning,

  - To achieve 1% BER, the required SNR for the case of FH cross 100MHz or the case of FH cross 20MHz is almost the same as for the case of FH cross 5MHz without RF retuning respectively.

Observation 1: For Msg.3 with 10% BLER, compared with the case of FH cross 5MHz without RF retuning,

- FH over 100MHz and 20MHz with 2-symbol puncturing due to RF retuning time has 1.4dB and 1.8dB performance loss respectively;

- FH over 100MHz and 20MHz with 4-symbol puncturing due to RF retuning time has 2.3dB and 2.8dB performance loss respectively.

Observation 2: For PUCCH format 1 with 0.1% NACK-to-ACK,

- the required SNR for the case FH cross 20MHz or 100MHz with 2-symbol puncturing due to RF retuning time is 0.5dB worse than the case of FH cross 5MHz without RF retuning;

- the required SNR for the case FH cross 20MHz with 2-symbol puncturing due to RF retuning time is similar as the case FH cross 100MHz with 2-symbol RF retuning time.

Observation 3: For PUCCH format 3 with 1% BER,

- comparing to the case of FH cross 5MHz without RF retuning, there is no performance loss for the case of FH cross 20MHz or 100MHz with 2-symbol RF retuning time;

- but with higher reliability requirement, like to achieve 0.1% BER, the required SNR for the case FH cross 20MHz and 100MHz with 2-symbol puncturing due to RF retuning time is 0.6dB and 0.3dB worse than the case of FH cross 5MHz without RF retuning.

In general, due to RF retuning, some transmission symbols has to be punctured, the performance loss caused by 4-symbol RF retuning is more than the gain from frequency hopping for all cases in this simulation. While for some cases, e.g., when RF retuning occupies two symbols, for 14-symbol PUCCH format 1 with 2 bits and 14-symbol PUCCH format 3 with 11 bits, the performance loss is neglectable, but more configuration/scheduling flexibility can be achieved. Besides, it is expected that more gains can be obtained by FH over a larger bandwidth compared to FH over a smaller bandwidth. However, the amount of the gain (slight or large) may also depend on factors like payload size, transmission length, number of symbols and symbol positions used by RF retuning. In our current simulation, Msg.3 and PF1, PF3 with 14 symbol duration and small payload are evaluated with the assumption that the minimum RF retuning gap is two symbols, and the two-symbols (or four-symbols) RF retuning gap occupies the last one symbol (or two symbols) of the first hop and the first one symbol (or two symbols) of the second hop of the UL transmission. On the other hand, the gain by FH over larger bandwidth may be improved if the RF retuning gap does not cause puncturing of PUCCH/PUSCH transmission symbol. Hence, we also simulate one case for PUCCH format 3 as following:

- 7-symbol subslot configuration, 5-symbol PUCCH format 3 with two subslot repetitions as shown in Figure 4; The payload size for PF3 is 11bits; Inter-repetition FH is performed over 5MHz, 20MHz and 100MHz.

- Assuming the RF retuning occupies two symbols for inter-repetition FH over 20MHz and 100MHz, then for this case, the RF retuning gap does not cause puncturing of the PF3 transmission symbol.

Figure 4: Illustration for 5-symbol PF3 with two repetitions based on 7-symbol subslot

The simulation result is provided in Figure 5. As shown in Figure 5, to achieve 1% BER, compared with the case of inter-repetition FH over 5MHz, there is 0.25dB and 0.5dB performance gain for the case of inter-repetition FH over 20MHz and the case of inter-FH over 100MHz, respectively.

Figure 5: Performance for 5-symbol PUCCH format 3 based on 7-symbol subslot with inter-repetition FH over 5MHz, 20MHz and 100MHz BW

Observation 4: For 5-symbol PUCCH format 3 with two repetitions based on 7-symbol subslot, when inter-repetition FH is applied, to achieve 1% BER,

- Comparing to the case of FH cross 5MHz, there is 0.25dB and 0.5dB performance gain for the case of FH cross 20MHz and 100MHz respectively.

As observed, for the evaluated case, if he RF retuning gap does not cause puncturing of PUCCH transmission symbol, the performance by FH over larger bandwidth is improved, but the overall gain is very limited. It is would be necessary to have further simulations to check the performance gain (if any) for other cases, e.g., PUSCH with larger TBS, e.g., 5000bits, inter-slot FH for PUSCH etc.

Proposal 1: If fast RF retuning is supported for UE bandwidth reduction Option 1 (i.e., RF is 5MHz), simulations with aligned assumptions are needed to check under what conditions (e.g., PUSCH and PUCCH with larger payload combining with inter-repetition FH, number of symbols needed for RF retuning, and whether PF retuning time occupies the valid UL transmission symbol(s) etc.), the performance gain brought by fast RF retuning would be obtained.

- Conclusion

This contribution discusses potential simulation of reduced bandwidth for eRedCap devices. The observations and proposals are summarized as following:

Observation 1: For Msg.3 with 10% BLER, compared with the case of FH cross 5MHz without RF retuning,

- FH over 100MHz and 20MHz with 2-symbol RF retuning time has 1.4dB and 1.8dB performance loss respectively;

- FH over 100MHz and 20MHz with 4-symbol RF retuning time has 2.3dB and 2.8dB performance loss respectively.

Observation 2: For PUCCH format 1 with 0.1% NACK-to-ACK,

- the required SNR for the case FH cross 20MHz or 100MHz with 2-symbol RF retuning time is 0.5dB worse than the case of FH cross 5MHz without RF retuning;

- the required SNR for the case FH cross 20MHz with 2-symbol RF retuning time is similar as the case FH cross 100MHz with 2-symbol RF retuning time.

Observation 3: For PUCCH format 3 with 1% BER,

- comparing to the case of FH cross 5MHz without RF retuning, there is no performance loss for the case of FH cross 20MHz or 100MHz with 2-symbol RF retuning time;

- but with higher reliability requirement, like to achieve 0.1% BER, the required SNR for the case FH cross 20MHz and 100MHz with 2-symbol RF retuning time is 0.6dB and 0.3dB worse than the case of FH cross 5MHz without RF retuning.

Observation 4: For 5-symbol PUCCH format 3 with two repetitions based on 7-symbol subslot, when inter-repetition FH is applied, to achieve 1% BER,

- Comparing to the case of FH cross 5MHz, there is 0.25dB and 0.5dB performance gain for the case of FH cross 20MHz and 100MHz respectively.

Proposal 1: If fast RF retuning is supported for UE bandwidth reduction Option 1 (i.e., RF is 5MHz), simulations with aligned assumptions are needed to check under what conditions (e.g., PUSCH and PUCCH with larger payload combining with inter-repetition FH, number of symbols needed for RF retuning, and whether PF retuning time occupies the valid UL transmission symbol(s) etc.), the performance gain brought by fast RF retuning would be obtained.

- References

- RP-213661, New SID on Study on further NR RedCap UE complexity reduction, Ericsson.

- R1-2203572, Techniques to further reduce the complexity of RedCap devices, vivo.

- R1-2102529, Discussion on reduced maximum UE bandwidth, vivo.

- R1-155051, Reply LS on retuning time between narrowband regions for MTC,’ RAN4.

Appendix

Table 1. Simulation assumptions

Main purpose to check when RF and baseband BW is reduced to 5MHz, with fast RF retuning, what is the gain for FH across 20MHz, FH across 100MHz compared to the FH across 5MHz without RF retuning

| Parameter | Evaluation assumptions |
| --- | --- |
| Scenario | Urban: 4GHz, SCS=30kHz - 5MHz@30Khz = 12 PRBs - 20Mhz@30KHz = 51 PRB - 100Mhz @30KHz =273 PRB |
| Channel model | TDL-C 300ns for PUCCH format 1, format 3and Msg.3 |
| UE velocity | 3km/h |
| Number of gNB antenna port in LLS | 2Rx for 4GHz |
| Number of UE antenna port in LLS | 1Tx/1Rx |
| Msg.3 | Waveform: DFT-s-OFDM |
|  | TBS: 56 bits MCS: 1 |
|  | RB allocation: 2PRBs |
|  | Hopping offset: - 6RB for 5MHz bandwidth; - 25RB for 20MHz bandwidth; - 136RB for 100MHz bandwidth. |
|  | DMRS configurations: PUSCH mapping type A, single symbol DMRS, dmrs-AdditionalPosition equals to 'pos1'; - For FH over 5MHz, - [D D R D D D R], [R D D D R D D]; D: Data symbol; R: DMRS symbol; - For FH over 20MHz or 100MHz with 2-symbol retuning time with puncturing Msg.3, - [D D R D D D X], [X D D D R D D]; X: RF retuning symbol; - For FH over 20MHz or 100MHz with 4-symbol RF retuning time with puncturing Msg.3, - [D D R D D X X], [X X D D R D D]; X: RF retuning symbol. |
| PUCCH | PUCCH Format 1 (PF1), 2 bits, 0.1% NACK to ACK probability, 1% DTX to ACK; - For 14-symbol PF1 FH over 5 MHz, - [R D R D R D R], [D R D R D R D]; D: Data symbol; R: DMRS symbol; - For 14-symbol PF1 FH over 20MHz or 100MHz with 4-symbol RF retuning time with puncturing PF1, - [R D R D R X X], [X X D R D R D]; X: RF retuning symbol; - For 14-symbol symbol PF1 FH over 20MHz or 100MHz with 2-symbol RF retuning time with puncturing PF1, - [R D R D R D X], [X R D R D R D]; X: RF retuning symbol. |
|  | PUCCH Format 3 (PF3) with additional DMRS, 11 bits, 1% BER; - For 14-symbol PF3 FH over 5 MHz, - [D R D D D R D] [D R D D D R D]; D: Data symbol; R: DMRS symbol; - For 14-symbol PF3 FH over 20MHz or 100MHz with 4-symbol RF retuning time with puncturing PF3, - [D R D D D X X], [X X D D D R D]; X: RF retuning symbol; - For 14-symbol symbol PF3 FH over 20MHz or 100MHz with 2-symbol RF retuning time with puncturing PF3, - [D R D D D R X], [X R D D D R D]; X: RF retuning symbol. |
|  | PUCCH Format 3 (PF3) with additional DMRS, 11 bits, 1% BER, 7-symbol sub-slot repetitions; - For 5-symbol PF3 FH over 5 MHz, - [R D D R D X X] [R D D R D X X]; D: Data symbol; R: DMRS symbol; X: No transmission; - For 5-symbol PF3 FH over 20MHz or 100MHz with 2-symbol RF retuning time without puncturing, - [R D D R D X X], [R D D R D X X]; X: RF retuning symbol; |