3GPP TSG RAN WG1 #109-e	                                              R1-2203575

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Discussions on NW energy savings performance evaluation

Agenda Item:	9.7.1

Document for:	Discussion and Decision

- Introduction

In RAN#94e meeting, a new study item: “Study on network energy savings for NR” was approved with the following objectives [1].

The objectives of the study are the following:

- Definition of a base station energy consumption model [RAN1]

- Adapt the framework of the power consumption modelling and evaluation methodology of TR38.840 to the base station side, including relative energy consumption for DL and UL (considering factors like PA efficiency, number of TxRU, base station load, etc), sleep states and the associated transition times, and one or more reference parameters/configurations.

- Definition of an evaluation methodology and KPIs [RAN1]

- The evaluation methodology should target for evaluating system-level network energy consumption and energy savings gains, as well as assessing/balancing impact to network and user performance (e.g. spectral efficiency, capacity, UPT, latency, handover performance, call drop rate, initial access performance, SLA assurance related KPIs), energy efficiency, and UE power consumption, complexity. The evaluation methodology should not focus on a single KPI, and should reuse existing KPIs whenever applicable; where existing KPIs are found to be insufficient new KPIs may be developed as needed.

Note: WGs will decide KPIs to evaluate and how.

- Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include:

- How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2]

- Information exchange/coordination over network interfaces [RAN3]

Note: Other techniques are not precluded

The study should prioritize idle/empty and low/medium load scenarios (the exact definition of such loads is left to the study), and different loads among carriers and neighbor cells are allowed.

The following example scenarios (mapping between scenarios and network loads is left to the study) including single-carrier and multi-carrier deployments are used as the starting point for discussion on prioritized scenarios for the study.

The following example scenarios are listed in no particular order.

- Urban micro in FR1, including TDD massive MIMO (note: this scenario can also model small cells)

- FR2 beam-based scenarios (note: this scenario can also model small cells)

- Urban/Rural macro in FR1 with/without DSS (no impact to LTE expected in case of DSS)

- EN-DC/NR-DC macro with FDD PCell and TDD/Massive MIMO on higher FR1/FR2 frequency

Note 1: legacy UEs should be able to continue accessing a network implementing Rel-18 network energy savings techniques, with the possible exception of techniques developed specifically for greenfield deployments.

Note 2: the study of energy savings specifically for IAB is not part of the scope.

The study should coordinate with RAN4 as needed.

For NW energy savings performance evaluation, the UE power consumption model and evaluation methodology captured in TR38.840 [2] for Rel-17 UE power saving could be the starting point. However, considering the characteristics of base station, more details on the base station energy consumption model, and evaluation methodologies need to be discussed. In this contribution, we provide our views on base station energy consumption model, evaluation methodologies and KPIs for NW energy saving.

- Base station (BS) energy consumption model

According to the SID, the framework of BS energy consumption model will adopt UE power consumption modelling in TR 38.840 [2] as the baseline with proper adaptation. By going through the UE power consumption model as provided in Table 1, it involves the following three kinds of power states:

- Sleep mode includes power states for Deep sleep, Light sleep and Micro sleep;

- Rx mode including power states for PDCCH-only, SSB or CSI-RS proc., and PDCCH+PDSCH;

- Tx mode include a unified UL power state.

Table 1: UE power consumption model for FR1 in TR 38.840

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | 1 (Optional: 0.5) |
| Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 |
| Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. | 300 |
| UL | Long PUCCH or PUSCH. | 250 (0 dBm) 700 (23 dBm) |

Following a similar design principle, BS energy consumption model is discussed in the following sub-sections from three aspects: sleep mode, Tx mode, and Rx mode.

  - Discussion on sleep mode

Power states for sleep mode

When the base station is operating in an idle state with no data load, the energy consumption of the base station is low, but the hardware components of base station are still completely active at this time. Further deactivation of the hardware components of the base station can reduce some additional energy consumption. The transition time of these hardware components, i.e., the time it takes to shut down and wake up again, differs significantly. For example, the power amplifier (PA) requires only one OFDM symbol (71 µs) to complete this transition, while other components such as the digital baseband require more time, to reach the millisecond-level.

As described in [3], four BS sleep modes are defined by grouping components with similar transition times, corresponding to a total transition (deactivation plus reactivation) of 71 μs (OFDM symbols), 1 ms (subframe or TTI), 10 ms (frame) and 1 s (long-term sleep). A deeper level of sleep mode saves more energy as more hardware components are deactivated, but a deeper level of sleep mode means it takes longer to deactivate or reactivate them. In addition, the process of deactivating and reactivating components consumes additional energy. As shown in Table 2 and Table 3, similar to the framework of UE sleep modes, for each sleep mode on the base station side, it is necessary to define the relative power of the sleep state, the total transition time (activation plus de-activation) and the transition energy consumption for the corresponding transition state.

Table 2. The framework of sleep modes on BS side

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| Quasi-off Sleep (SM4 in [3]) | Represents the longest sleep mode where the BS retains its wake-up functionalities but almost all its components are deactivated | X0 |
| Deep Sleep (SM3 in [3]) | corresponds to a longer sleep mode in which more components of the analog front-end and digital baseband are disabled compared to Light Sleep. | X1 |
| Light Sleep (SM2 in [3]) | corresponds to a longer sleep mode in which more components of the analog front-end are disabled compared to Micro sleep. | X2 |
| Micro sleep (SM1 in [3]) | represents the shortest sleep mode where the PA and some components of the digital baseband and the analog front-end are deactivated. | X3 |

Table 3. The framework of transition state on BS side

| Sleep type | Additional transition energy: (Relative power x ms) | Total transition time |
| --- | --- | --- |
| Quasi-off sleep | Y0 | 1s |
| Deep sleep | Y1 | 10 ms |
| Light sleep | Y2 | 1 ms |
| Micro sleep | 0 | 0 ms* |
| *Immediate transition is assumed for BS energy saving study purpose from or to a non-sleep state |  |  |

Proposal 1: Adopt the framework with four sleep modes and transition states shown in Table 2 and Table 3, at least including sleep types with second-level, millisecond-level and microsecond-level (~0 ms) transition time.

Proposal 2: Relative power value for different sleep modes and transition states (i.e. the value of X0, X1, X2, X3 and Y0, Y1, Y2) should be determined based on input from BS implementations.

Handling of Rx in sleep mode

The handling of Rx for the base station in sleep mode is an issue to be considered, e.g., whether Rx components are deactivated in the above-mentioned sleep mode. For the base station itself, the PA in the transmitter components, which has been identified as the largest energy consumer, typically accounts for about 60% of the total BS energy. On the other hand, the energy consumption of the low noise amplifier (LNA) in the receiver components, is relatively small. In our opinion, there are three options to consider for the handling of Rx for the base station in sleep mode, as shown in Figure 1.

In Option 1, the sleep mode is defined for both Tx and Rx jointly, i.e. the base station needs to be woken up for any Tx or Rx. This way is relatively simple to implement, but it may be difficult to enter deeper sleep mode due to the frequent wake-up of Rx (usually a base station would serve many UEs).

In Option 2, the sleep mode is only defined for Tx, i.e. there is no need to wake up the BS due to Rx. Under this approach, it assumes almost zero-transition Rx switching or always-on Rx since the energy consumption of Rx is much smaller than that of Tx and thus can be omitted. In this way, it is easier to enter deeper sleep mode.

In Option 3, separate sleep modes are defined for Tx and Rx, i.e. wake up the BS Tx sleep or Rx sleep due to the Tx or Rx needs respectively. This approach is the most flexible and conducive to network energy saving, but it is also the most complex implementation with more diverse combinations.

| (a) Option 1 | (b) Option 2 |
| --- | --- |
| (c) Option 3 |  |

Figure 1. Three options for the handling of Rx for the base station in sleep mode

Proposal 3: The following options for handling of Rx for the base station in sleep mode should be discussed and determined.

- Option 1: Sleep mode is defined for Tx and Rx jointly, i.e. there is need to wake up the BS due to any Tx or Rx.

- Option 2: Sleep mode is defined for Tx only, i.e. there is no need to wake up the BS due to Rx.

- Option 3: Sleep mode is defined for Tx and Rx separately, i.e. there is need to wake up BS Tx sleep/Rx sleep due to Tx/Rx respectively.

Handling of wake-up in different phases

For the base station in sleep mode, the handling of wake-up in different phases is another issue to be determined. Each sleep mode is characterized by 3 different phases, deactivation phase, sleep phase and activation phase. When the base station is in the deactivation phase, if there is a waking request during this phase, the base station would continue to deactivate the relevant hardware components until deactivation is complete, and then start activating immediately. When the base station is in the sleep phase, if there is a waking request during this phase, the base station would start activating immediately. When the base station is in the activation phase, if there is a waking request during this phase, the base station would continue to activate until the activation is complete. The new waking request would have no additional impact on the activation of the base station.

Figure 2. The handling of wake-up in different phases of sleep mode

Proposal 4: Adopt the following handling of wake up in different phases from a sleep mode:

- Waking request in deactivation phase:  Continue deactivation until finished and then start activation immediately.

- Waking request in sleep phase: Start activation immediately after the request.

- Waking request in activation phase: Continue activation until finished.

  - Discussion on Tx mode

Power state for Tx mode

In UE power consumption model, the following one power state is defined for UE Tx mode:

| UL | Long PUCCH or PUSCH. | 250 (0 dBm) 700 (23 dBm) |
| --- | --- | --- |

Similarly, for BS Tx mode, a unified power state could be defined for all downlink channels based on the assumption that the transmission of different DL signals/channels (e.g. PDCCH, PDSCH, CSI-RS and SSB) consume almost the same energy if they occupy the same amount of time-frequency resource.

On the other aspect, different types of BSs may result in various energy consumption values. BS generally consists of base band unit (BBU), active antenna unit (AAU) and power supply system. The BBU is responsible for NR baseband protocol processing, and AAU mainly completes the conversion of NR baseband signal to RF signal and the transceiver processing function of NR RF signal. The energy consumption level of each hardware component is different, and in general, the energy consumption of AAU is the main component of the energy consumption of base station. Considering the effects of hardware integration level, PA efficiency, active cooling and other factors, the energy consumption of different base station types varies relatively widely. Based on the SID, the following three types of scenarios are mainly considered, i.e. Indoor Hotspot, Dense Urban and Rural/Urban Macro. To reflect different types of BSs in the power state definition, relative power is defined respectively for different power levels in different scenarios as referred to TR 38.802 [4], i.e. 31dBm for indoor hotspot, 51dBm for dense urban and 56dBm for Rural/Urban Macro. Note that the Tx power values are calculated assuming the 100MHz Tx bandwidth mentioned in reference configuration below.

Based on the above discussions, the framework of BS energy consumption model for FR1 Tx mode is proposed in Table 4.

Table 4. The framework of BS energy consumption model in FR1 Tx mode

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| DL | Include PDCCH and/or PDSCH and/or SSB and/or CSI-RS | Z1 (31dBm for Indoor Hotspot) Z2 (51dBm for Dense Urban) Z3 (56dBm for Rural/Urban Macro) |

Proposal 5: Adopt a unified DL power state in Table 4 for all DL signals/channels as BS energy consumption model of Tx mode, including the definition of different relative power in 3 Tx power levels.

Reference configuration in Tx mode

To determine the value of Z1, Z2 and Z3, reference configurations such as SCS, bandwidth, antenna, etc. need to be defined. After the definition, Z1, Z2 and Z3 is the power value under the reference configuration. Particularly, Z1, Z2 and Z3 are the relative power values for DL transmissions occupying all the system bandwidth and the whole slot under corresponding reference antenna configurations. Reference configuration for UE power model could be reused with adaptation of Tx antenna configuration and power level as provided in Table 5.

Table 5. The reference configuration of base station in FR1 Tx mode

|  | UE power model (UE Tx, TDD UL) | BS energy consumption model (BS Tx, TDD DL) |
| --- | --- | --- |
| Subcarrier Spacing | 30KHz | 30KHz |
| Number of Carriers | 1 | 1 |
| System bandwidth | 100MHz | 100MHz |
| Tx antenna configuration | 1 TX | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1) for Indoor Hotspot (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) for Dense Urban and Rural/Urban Macro |
| Power level | 0dBm and 23dBm | 31dBm for Indoor Hotspot 51dBm for Dense Urban 56dBm for Rural/Urban Macro |

Proposal 6: Adopt Table 5 as a starting point for definition of BS reference configuration in FR1 TDD DL (i.e. Tx mode), including SCS, number of Carriers, system bandwidth, Tx antenna configuration and power levels.

Scaling model in Tx mode

As discussed before, the relative power level of the base station in Tx mode is related to many factors, such as resource usage, the number of antennas, etc. Firstly, for the resource usage, the relative power values in Table 4 are for the case where the DL transmissions occupy all the system bandwidth and the whole slot. However, from a practical point of view, neither SSB nor CSI-RS occupies the whole system bandwidth or the whole slot, even for PDCCH and PDSCH, there are instances where the entire system bandwidth or the whole slot is not fully occupied. Besides, the antenna configuration is not fixed for different base stations. Therefore, for BS Tx mode, the actual relative power consumption of each DL slot needs to be scaled according to the actual antenna configuration, resource usage, etc. Referring to the scaling framework of UE power model, the scaling framework in Table 6 for BS Tx mode could as a starting point, and the value of alpha, beta, gamma and sigma should be determined based on input from BS implementations.

Table 6. The scaling framework for BS Tx mode

| Scaling for FR1 | Proposal | Comment |
| --- | --- | --- |
| Carrier/Tx Bandwidth (DL) | Scaling of X MHz = alpha + (1-alpha) * (X - 20) / 80. Linear interpolation for intermediate bandwidths. Valid only for X = 10, 20, 40, 80, and 100. | The range of resulting power value from scaling is between Micro sleep power X3 and corresponding DL power Z1/Z2/Z3 0<alpha<1 |
| CA (DL) | 2CC is beta x 1CC 4CC is 2*beta x 1CC | 1<beta<2 |
| Antenna scaling (DL) | 64Tx power is gamma x 32Tx power for FR1 128Tx power is gamma x 64Tx power for FR1 | 1<gamma<2 |
| Time-domain scaling (DL) | Scaling factor sigma * Z1/Z2/Z3 | sigma is ratio of the DL channel in the slot, e.g. sigma=4/14 for 4-symbol SSB |

Proposal 7: Adopt scaling adaptation for BS Tx mode compared to reference configuration in frequency domain (Tx bandwidth, CA), antenna domain and time domain, where Table 6 could serve as a starting point.

  - Discussion on Rx mode

Power state for Rx mode

In UE power consumption model, the following power states are defined for UE Rx mode:

| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 |
| --- | --- | --- |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. | 300 |

Similarly, for BS Rx mode, one way is to define different power states and relative power values for different uplink channels or signals (e.g., PUCCH, PUSCH and SRS) respectively, where the framework is provided in the following Table 7:

Table 7. Per-channel framework of BS energy consumption model in Rx mode

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| PUSCH + PUCCH | PUSCH + PUCCH occupying a slot | P1 |
| RACH proc | RACH is used for initial access or acquiring UL sync | P2 |
| SRS proc | SRS is used for channel estimation | P3 |
| PUCCH-only | PUCCH is used for SR or HARQ feedback or CSI report | P4 |

On the other hand, since the energy consumption of the base station receiver components is relatively small compared to the transmitter components and there may not be much difference in reception processing for different UL channels or signals, the main energy consumption is all in the BBU processing part. Therefore, another way is to define a unified power state and relative power value for all uplink channels based on the assumption that the reception of different UL signals/channels (e.g., PUCCH, PUSCH and SRS) consume almost the same energy if they occupy the same amount of time-frequency resource. Then the unified power state is provided in the following Table 8.

Table 8. Unified framework of BS energy consumption model in Rx mode

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| UL | PUSCH and/or PUCCH and/or SRS and/or RACH | P |

Taking into account that Tx power is the main focus of network energy saving and Rx power is much smaller compared to Tx power, the above unified framework is preferred for simplicity to avoid further complexity.

Proposal 8: Adopt a unified UL power state in Table 8 for all UL signals/channels as BS energy consumption model of Rx mode.

Reference configuration in Rx mode

Determining the relative power value of the base station in Rx mode is similar to Tx mode, where a set of reference configurations needs to be determined first. Referring to the TDD DL reference configuration in UE Rx mode, the reference configurations in Table 9 can be a starting point for the base station in Rx mode.

Table 9. The reference configuration of base station in Rx mode

|  | UE power model (UE Rx, TDD DL) | BS energy consumption model (BS Rx, TDD DL) |
| --- | --- | --- |
| Subcarrier Spacing | 30KHz | 30KHz |
| Number of Carriers | 1 | 1 |
| System bandwidth | 100MHz | 100MHz |
| Rx antenna configuration | 4 Rx | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1) for Indoor Hotspot (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) for Dense Urban and Rural/Urban Macro |

Proposal 9: Adopt Table 9 as a starting point for definition of BS reference configuration in FR1 TDD UL (i.e. Rx mode), including SCS, number of Carriers, system bandwidth and Rx antenna configuration.

Scaling model in Rx mode

Similar to the BS Tx mode, for the BS RX mode, the actual relative power consumption of each UL slot also needs to be scaled according to the actual antenna configuration, resource usage, etc. The scaling framework used for the BS Rx mode in Table 10 can be used as a starting point, while the values of alpha’, beta’, gamma’ and sigma’ should be determined based on the inputs of the BS implementation.

Table 10. The scaling framework for BS Rx mode

| Scaling for FR1 | Proposal | Comment |
| --- | --- | --- |
| Carrier/Tx Bandwidth (UL) | Scaling of Y MHz = alpha’ + (1-alpha’) * (Y - 20) / 80. Linear interpolation for intermediate bandwidths. Valid only for Y= 10, 20, 40, 80, and 100. | The range of resulting power value from scaling is between Micro sleep power X3 and corresponding DL power Z1/Z2/Z3 0<alpha’<1 |
| CA (UL) | 2CC is beta’ x 1CC 4CC is 2*beta’ x 1CC | beta’>1 |
| Antenna scaling (UL) | 64Tx power is gamma’ x 32Tx power for FR1 128Tx power is gamma’ x 64Tx power for FR1 | gamma’>1 |
| Time-domain scaling (UL) | Scaling factor sigma’ * Z1/Z2/Z3 | sigma’ is ratio of the UL channel in the slot, e.g. sigma=4/14 for 4-symbol SSB |

Proposal 10: Adopt scaling adaptation for BS Rx mode compared to reference configuration in frequency domain (Tx bandwidth, CA), antenna domain and time domain, where Table 10 could serve as a starting point.

- Evaluation methodology and KPIs

  - Evaluation methodology

The use of energy consumption model on the base station side will not only have an impact on the performance of the base station but also on the performance of the user, so the evaluation of the NW energy saving study should consider using the system-level simulation, which can obtain more comprehensive evaluation results. For system-level simulation, deployment scenarios, traffic models, and modeling of power consumption model could be considered.

Deployment scenarios

In NW energy saving SID, the following example scenarios are listed in no particular order.

- Urban micro in FR1, including TDD massive MIMO (note: this scenario can also model small cells)

- FR2 beam-based scenarios (note: this scenario can also model small cells)

- Urban/Rural macro in FR1 with/without DSS (no impact to LTE expected in case of DSS)

- EN-DC/NR-DC macro with FDD PCell and TDD/Massive MIMO on higher FR1/FR2 frequency

Due to the limited TUs for NW energy saving study and the numerous evaluation workload, the priority of deployment scenarios for evaluation needs to be determined. For the prioritization of deployment scenarios, we propose to prioritize the evaluation of Urban micro in FR1, FR2 beam-based scenarios and Urban/Rural macro in FR1 without DSS. In addition, from the evaluation perspective, it is possible to consider mapping the scenarios described in SID to those in TR 38.802 [4], as shown in the table below and reusing the simulation assumption setting in TR 38.802 [4].

Table 11. The mapping between scenarios in SID and scenarios in TR 38.802

| Scenarios in SID | Scenarios in TR 38.802 |
| --- | --- |
| Urban micro in FR1, including TDD massive MIMO (note: this scenario can also model small cells) | Dense Urban single layer/dual layer on 4GHz |
| FR2 beam-based scenarios (note: this scenario can also model small cells) | Dense Urban single layer/dual layer on 30GHz |
| Urban/Rural macro in FR1 with/without DSS (no impact to LTE expected in case of DSS) | Urban/Rural macro on 4GHz |
| EN-DC/NR-DC macro with FDD PCell and TDD/Massive MIMO on higher FR1/[FR2] frequency | Urban/Rural macro on 4GHz/30GHz |

Proposal 11: Evaluation scenarios could reuse the simulation assumption setting in TR 38.802.

Traffic models

The evaluation should assess the impact on user performance, UE power consumption, and complexity, in addition to system-level network energy consumption and energy savings benefits. Therefore, the traffic models considered in the evaluation of UE power saving study in TR 38.840 [2] should be reused for the evaluation of the network energy saving. The specific traffic models are shown in Table 12.

Table 12. The traffic model used in network energy saving evaluation

|  | FTP traffic | Instant messaging | VoIP |
| --- | --- | --- | --- |
| Model | FTP model 3 | FTP model 3 | As defined in R1-070674 [5]. Assume max two packets bundled. |
| Packet size | 0.5 Mbytes | 0.1 Mbytes |  |
| Mean inter-arrival time | 200 ms | 2 sec | Period = 40 ms Inactivity timer = 10 ms |

Proposal 12: Traffic models could reuse the setting in TR 38.840 for UE power saving study.

System load

As described in SID, the study should prioritize idle/empty and low/medium load scenarios (the exact definition of such loads is left to the study), and different loads among carriers and neighbor cells are allowed.

On one aspect, different load scenarios should be exactly defined. For idle/empty load scenarios, there will be no traffic arrival and connected UEs in the concerning cells. For definition of low/medium load scenario, many factors could be considered such as traffic density, number of UEs per cell, resource utilization, etc. For example, 5 UEs per cell and an average packet interval of 200ms for FTP3 is defined as a load level, or, 20% ~ 30% of RU is defined as a load level.

Proposal 13: The definition of load scenarios (e.g., low/medium load) should at least include traffic density, number of UEs per cell and/or resource utilization ratio.

On the other hand, to achieve different loads among carriers and/or neighbor cells, the non-uniform UE distribution should be considered. For example, in an evaluation scenario with multiple cells, the UE distribution is only a concentrated partial number of cells, e.g. 14 cells out of 21 cells in dense urban scenario.

Proposal 14: Non-uniform UE distribution should be considered in network energy saving evaluation.

How to model the BS energy consumption model

To model the BS power consumption model in system-level simulation, the following algorithm can be considered as a starting point. Firstly, periodical signaling (e.g. periodical SSB, CSI-RS, etc.) is used to determine the sleep mode. If sleep mode is defined for Tx and Rx jointly (i.e., Option 1 in section 2.1), periodical signaling includes UL signaling (e.g., periodical PUCCH for SR, SRS) besides periodical SSB or CSI-RS. Secondly, if the BS reception is always on in sleep mode, all DL and UL data requests wake up the base station from sleep mode immediately. However, if sleep mode is defined for Tx and Rx jointly, UL data requests cannot wake up the base station immediately, the base station will start uplink scheduling only after periodical wake-up. Considering the time of activation and deactivation of the base station components, the time interval between periodical signaling or between the end of active time and periodical signaling determines the sleep state that the base station could enter, which is depicted in Figure 3. The results in our companion’s contribution [6] show that the BS with the energy consumption model can obtain about 51.44% of the BS power saving gain at low load with 2 UEs per cell.

Figure 3. The schematic diagram of sleep pattern determination

Proposal 15: Algorithm for determination of sleep mode should be aligned/reported since it will impact the final evaluation of benefit for network energy saving schemes.

  - Performance metrics

For NW energy consumption evaluation, both base station power performance and throughput performance metrics should be considered, and the impact of network energy saving on UE performance should also be considered. The following BS side and UE side KPIs can be taken into consideration.

BS side:

- BS average power consumption: the power values averaged over a slot, calculated per cell.

- BS power saving gain: percentage of power consumption reduction of the proposed power saving scheme from the non-power saving scheme.

- Resource utilization: resource utilization is defined as the ratio between the number of REs used for packet transmission and the total number of available REs during a simulation for a given transmission direction.

UE side:

- UE average power consumption: the power values averaged over a slot, calculated per user.

- UE average UPT: the size of a burst divided by the time between the arrival of the first packet of the burst and the reception of the last packet of the burst, the value is calculated per user.

- Packet average transmit latency: the time duration between the packet generation and when the packet is correctly decoded or dropped, the value could be calculated per packet or user.

- UE average PER: a packet is deemed to be delivered if it is received successfully by a UE over the air interface within PDB. The packet delivered rate PD is derived by the number of successfully delivered packets and the number of generated packets for a user, while the packet error rate (PER) would be PE = 1 - PD.

In addition to the average values of the above metrics, their CDF values can also be considered, such as the values taken at the 5%, 50%, and 95% points of the CDF curve.

As described in SID, the study should prioritize idle/empty and low/medium load scenarios. However, for idle/empty load scenarios, since there is no packet generation and transmission, the throughput/latency related performance metrics mentioned above may not be meaningful and new metrics need to be introduced to reflect performance other than energy consumption.

Proposal 16: It is not only the network-side performance metrics that should be considered but also the impact on user performance. Both network/user power saving gains and the impact on network/user performance should be considered for each network energy saving scheme.

- Conclusion

In this contribution, we discuss the BS energy consumption model, evaluation methodologies and KPIs, and have the following observations and proposals:

Proposal 1: Adopt the framework with four sleep modes and transition states shown in Table 2 and Table 3, at least including sleep types with second-level, millisecond-level and microsecond-level (~0 ms) transition time.

Proposal 2: Relative power value for different sleep modes and transition states (i.e. the value of X0, X1, X2, X3 and Y0, Y1, Y2) should be determined based on input from BS implementations.

Proposal 3: The following options for handling of Rx for the base station in sleep mode should be discussed and determined.

- Option 1: Sleep mode is defined for Tx and Rx jointly, i.e. there is need to wake up the BS due to any Tx or Rx.

- Option 2: Sleep mode is defined for Tx only, i.e. there is no need to wake up the BS due to Rx.

- Option 3: Sleep mode is defined for Tx and Rx separately, i.e. there is need to wake up BS Tx sleep/Rx sleep due to Tx/Rx respectively.

Proposal 4: Adopt the following handling of wake up in different phases from a sleep mode:

- Waking request in deactivation phase:  Continue deactivation until finished and then start activation immediately.

- Waking request in sleep phase: Start activation immediately after the request.

- Waking request in activation phase: Continue activation until finished.

Proposal 5: Adopt a unified DL power state in Table 4 for all DL signals/channels as BS energy consumption model of Tx mode, including the definition of different relative power in 3 Tx power levels

Proposal 6: Adopt Table 5 as a starting point for definition of BS reference configuration in FR1 TDD DL (i.e. Tx mode), including SCS, number of Carriers, system bandwidth, Tx antenna configuration and power levels.

Proposal 7: Adopt scaling adaptation for BS Tx mode compared to reference configuration in frequency domain (Tx bandwidth, CA), antenna domain and time domain, where Table 6 could serve as a starting point.

Proposal 8: Adopt a unified UL power state in Table 8 for all UL signals/channels as BS energy consumption model of Rx mode.

Proposal 9: Adopt Table 9 as a starting point for definition of BS reference configuration in FR1 TDD UL (i.e. Rx mode), including SCS, number of Carriers, system bandwidth and Rx antenna configuration.

Proposal 10: Adopt scaling adaptation for BS Rx mode compared to reference configuration in frequency domain (Tx bandwidth, CA), antenna domain and time domain, where Table 10 could serve as a starting point.

Proposal 11: Evaluation scenarios could reuse the simulation assumption setting in TR 38.802.

Proposal 12: Traffic models could reuse the setting in TR 38.840 for UE power saving study.

Proposal 13: The definition of load scenarios (e.g., low/medium load) should at least include traffic density, number of UEs per cell and/or resource utilization ratio.

Proposal 14: Non-uniform UE distribution should be considered in network energy saving evaluation.

Proposal 15: Algorithm for determination of sleep mode should be aligned/reported since it will impact the final evaluation of benefit for network energy saving schemes.

Proposal 16: It is not only the network-side performance metrics that should be considered but also the impact on user performance. Both network/user power saving gains and the impact on network/user performance should be considered for each network energy saving scheme.

References

- RP-213554, RAN #94e, Electronic Meeting, Dec 6-17, 2021.

- 3GPP TR 38.840, Study on User Equipment (UE) power saving in NR (Release 16).

- B. Debaillie, C. Desset and F. Louagie, “A Flexible and Future-Proof PowerModel for Cellular Base Stations”, 2015 IEEE 81st VTC, Glasgow, 2015, pp. 1-7.

- 3GPP TR 38.802, Study on New Radio Access Technology Physical Layer Aspects (Release 14).

- R1-070674, RAN1#48, 12th-16th February, 2007, St. Louis, MI, USA.

- R1-2203577, Initial evaluation results for network energy saving scheme, RAN1#109-e, May 2022, vivo.