3GPP TSG RAN WG1 #109-e		R1-2203579

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Discussion on L1/L2 signaling for side control information

Agenda Item:	9.8.2

Document for:	Discussion and Decision

- Introduction

New SID [1] on network-controlled repeaters has been approved, and multiple types of side control information need to be justified as following.

The study on NR network-controlled repeaters is to focus on the following scenarios and assumptions: - Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios. - For only single hop stationary network-controlled repeaters - Network-controlled repeaters are transparent to UEs - Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously NOTE1: Cost efficiency is a key consideration point for network-controlled repeaters. Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1] - Beamforming information - Timing information to align transmission / reception boundaries of network-controlled repeater - Information on UL-DL TDD configuration - ON-OFF information for efficient interference management and improved energy efficiency - Power control information for efficient interference management (as the 2nd priority) Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1] Study the following aspects of network-controlled repeater management - Identification and authorization of network-controlled repeaters [RAN2, RAN3] NOTE2: Coordination with SA3 may be needed.

In this contribution, we provide our view on L1/L2 signaling to convey the side control information.

- Discussion

  - MT and RU

Figure 1 Structure of network-controlled repeater

Legacy repeater has only RF unit that is used to forward the DL/UL signal. Different from the legacy repeater, the network-controlled repeater is comprised of repeater unit (RU) and mobile terminal (MT). The MT is used to receive/transmit side control information from/to gNB, while the RU is the same as the RF unit for the legacy repeater.

Proposal 1: Network controlled repeater is comprised of MT and RU.

  - L1/L2 signaling

The functionality of MT unit is the same as a normal UE, so the L1/L2 signaling defined for UE which is received by MT can be used to control the repeater, e.g., RRC, MAC CE, DCI, UCI. Since the latency and capacity of the signaling types are different, RAN1 needs to decide which signaling type(s) is used for the respective side control information delivery.

Proposal 2: Signaling types used to control legacy UE are considered to carry side control information, e.g., RRC, MAC CE, DCI, UCI.

As discussion in companion paper [2], when gNB controls the repeater, both semi-static and dynamic side control information are beneficial considering the different information types.

Regarding beam control, both semi-static signaling and dynamic signaling are beneficial considering different repeater deployment scenarios. Especially for the per-UE beam control, dynamic signaling is requisite for the fast control information transmission.

Regarding timing and TDD, RAN1 should decide whether the repeater needs to know the dynamic TDD information or not. If not, legacy signaling (e.g., SIB) or a new L2 signaling is sufficient for the control.

Regarding the on-off, the repeater is “ON” only when UE served by repeater is scheduled or when the repeater is searching for new UE(s) under the coverage of the repeater. Since the on-off operation is similar as UE DRX operation, at least semi-static on-off mechanism should be studied.

Regarding power control, since the RF power amplifier adjustment may be not so dynamic, e.g., similar as gNB power amplifier, so it is preferred to support semi-static power control.

Table 1 signaling type for side control information

|  | Semi-static signaling | Dynamic signaling |
| --- | --- | --- |
| Beam control information | Yes | Yes |
| Timing/TDD | Yes | TBD |
| On-off | Yes | TBD |
| Power control | Yes | - |

Proposal 3: To convey beam control information, both semi-static signaling and dynamic signaling are used.

Proposal 4: To convey timing/TDD information, at least semi-static signaling is used.

Proposal 5: To convey on-off information, at least semi-static signaling is used.

Proposal 6: To convey power control information, semi-static signaling is used.

- Conclusion

This contribution focus on L1/L2 signaling to convey the side control information and the following are proposed,

Proposal 1: Network controlled repeater is comprised of MT and RU.

Proposal 2: Signaling types used to control legacy UE are considered to convey side control information, e.g., RRC, MAC CE, DCI, UCI.

Proposal 3: To convey beam control information, both semi-static signaling and dynamic signaling are used.

Proposal 4: To convey timing/TDD information, at least semi-static signaling is used.

Proposal 5: To convey on-off information, at least semi-static signaling is used.

Proposal 6: To convey power control information, semi-static signaling is used.

- Reference

- RP-213700, “New SI: Study on NR Network-controlled Repeaters”, ZTE, RAN#94e.