3GPP TSG RAN WG1 #109-e		R1-2203580

e-Meeting, May 9th – 20th, 2022

Source:	vivo

Title:	Discussion on in-band repeater

Agenda Item:	9.8.3

Document for:	Discussion and Decision

- Introduction

New SID [1] on network-controlled repeaters has been approved, and multiple types of side control information need to be justified as following.

The study on NR network-controlled repeaters is to focus on the following scenarios and assumptions: - Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios. - For only single hop stationary network-controlled repeaters - Network-controlled repeaters are transparent to UEs - Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously NOTE1: Cost efficiency is a key consideration point for network-controlled repeaters. Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1] - Beamforming information - Timing information to align transmission / reception boundaries of network-controlled repeater - Information on UL-DL TDD configuration - ON-OFF information for efficient interference management and improved energy efficiency - Power control information for efficient interference management (as the 2nd priority) Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1] Study the following aspects of network-controlled repeater management - Identification and authorization of network-controlled repeaters [RAN2, RAN3] NOTE2: Coordination with SA3 may be needed.

In this contribution, we provide our view on some critical points which is relevant but not explicitly included in the WID, e.g., MT and RU resource multiplexing and auto-exitation issues.

- Discussion

  - MT and RU resource multiplexing

The signal generated by MT is baseband signal, while the signal forwarded by RU is RF signal. From hardware implementation perspective, if RU and MT share the RF components, it may be challenging to merge the baseband signal and RF signal at the same time considering the interference and timing alignment, thus it should be considered how to multiplex MT’s UL signal and RU’s UL signal.

To address the above issue, the following UL resource multiplexing can be taken into consideration between MT and DU.

TDM

The transmissions of MT and RU are always TDMed, which means that the MT UL and RU UL are not required to multiplex at the same time. This method is beneficial for reducing the handling complexity of the repeater. However, it may not friendly for the efficiency of the transmission, especially for the spectrum efficiency.

Figure 1 MT and RU operate in TDM

Out-of-band FDM

If the MT and RU operate in out-of-band FDM manner, the MT UL and RU UL could operate at the same time, since different RF components can be assumed for operation on different bands. Such multiplexing method can avoid baseband signal and RF signal multiplexing issue, however, it may increase the hardware cost of the repeater and may increase the deployment complexity since operation on multiple carriers is required.

In-band FDM

The in-band FDM is supported only when the MT UL signal and RU UL signal can multiplex together to transmit to gNB. RAN1 should discuss whether repeater’s capability allows such operation or not.

Based on the above discussion, we have the following proposal:

Proposal 1: Study how to multiplex MT UL signal and RU UL signal, i.e., TDM, out-of-band FDM or in-band FDM, and the associated specification impact if any.

  - Auto-exitation

As required in SID [1], the network-controlled repeater can maintain the gNB-repeater link (e.g., backhaul link as illustrated in Figure 2) and repeater-UE link (e.g., access link as illustrated in Figure 2) simultaneously, since the two links operate in the same band, the signal from one of the link may impact the other link. For example, for DL/UL signaling forwarding, the output signal of the RU may return to the RU’s input antenna, this will incur the autoexitation of the repeater.

Since the autoexitation phenomenon may destroy the hardware of the repeater, it should be avoided. The autoexitation phenomenon appears only if the output power cannot be compensated by the pathloss between the output side to the input side. Hence, to avoid autoexitation phenomenon, either the output signaling strength is reduced, or the pathloss between output side and input side is enlarged, e.g., by isolating the input and output antenna or by selecting proper output beam and input beam.

Figure 2 Interferred links

Proposal 2: Study whether specification effort is requisite or not to avoid autoexitation phenomenon of the repeater.

- Conclusion

This contribution focus on resource multiplexing between child and parent links, and give the following proposals,

Proposal 1: Study how to multiplex MT UL signal and RU UL signal, i.e., TDM, out-of-band FDM or in-band FDM, and the associated specification impact if any.

Proposal 2: Study whether specification effort is requisite or not to avoid autoexitation phenomenon of the repeater.

- Reference

- RP-213700, “New SI: Study on NR Network-controlled Repeaters”, ZTE, RAN#94e.